3GPP TSG RAN WG1 #120                                                                             R1- 2500367

Athens, Greece, February 17th – 21st, 2025

Source:	vivo

Title:	Discussion on support of RedCap and eRedCap UEs with NR-NTN

Agenda Item:	9.11.2

Document for:	Discussion and Decision

- Introduction

The Rel-19 NR NTN objectives are revised and provided in [1]. One of the objectives is to specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap and eRedCap UEs with NR NTN operating in the FR1-NTN band as follows.

Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In this contribution, we discuss the following aspects about half duplex in NTN:

- Remaining issues on collision Case 3

- Remaining issues on collision Case 4

- Remaining issues on TA report enhancement

- Discussion

  - Remaining issues on collision Case 3

Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above. - This is signaled by RRC configuration. Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

According to agreement of the RAN1#119 meeting, the RRC configuration can be configured as UL overriding DL for all the other use cases. However, the detailed RRC configuration is not very clear, e.g., it is configured case by case or it is only 1 bit for all the other use cases.

From our view, 1 bit configuration by RRC for all the other cases is sufficient. The detailed RRC signaling design, including the default configuration, is up to RAN2.

Proposal 1: Support one bit RRC configuration to indicate UL overriding DL for all the collision cases except for collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode.

  - Remaining issues on collision Case 4

Agreement The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode. - FFS whether to prioritize UL or prioritize DL.

According to the agreement in RAN1#119 meeting, the collision case 4 is not considered as an error case. Thus, the details of collision handling rule should be defined, e.g., whether to prioritize UL or prioritize DL.

In the collision case 4, the DL reception may include dynamic scheduled PDSCH, and CSI-RS, and the UL transmission may include dynamic scheduled PUSCH, PUCCH, SRS, and PDCCH order triggered PRACH. The potential collision cases are listed in the following table.

Table 2 Prioritized rule in collision case 4

| Prioritized direction | Dynamic scheduled PUSCH | Dynamic scheduled PUCCH | Dynamic scheduled SRS | PDCCH order triggered PRACH |
| --- | --- | --- | --- | --- |
| Dynamic scheduled PDSCH | - Dependent - Or, DL is prioritized. | - Dependent - Or, DL is prioritized. | - PDSCH is prioritized. | - Dependent - Or, DL is prioritized. |
| Dynamic scheduled CSI-RS | - PUSCH is prioritized | - CSI-RS is prioritized | - CSI-RS is prioritized. | - PRACH is prioritized. |

Some analyses of the use cases are listed as follows:

- When a dynamic scheduled PDSCH collides with another dynamic scheduled PUSCH, which one should be prioritized is depending on many aspects. For example, if the PDCSH is important (e.g., carrying high priority data or SIB 19), or if the HARQ is disabled, the PDSCH can be prioritized. On the other hand, if HARQ is enabled, the PUSCH may be prioritized.

- When the PDSCH collides with a PDCCH order triggered PRACH, generally, the PRACH should be prioritized. However, if the PDSCH carries SIB 19 to determine the pre-compensated TA for message 3 or PRACH for UL sync, for a UE without valid SIB 19, the PDSCH may need to be prioritized. As discussed in collision Case 3, left to UE implementation to perform the prioritization is more efficient.

Based on the above, in collision case 4, it would be more efficient for the following cases up to UE implementation, at least for collision for SIB PDSCH reception.

For the collision between dynamic scheduled CSI-RS and PDCCH order triggered PRACH, the PRACH can be prioritized.

In order to simplify the spec and implementation, the collision handling rule of collision case 3 can be reused. Specifically, for all other use cases, default priority rule can be defined as that DL is prioritized, while Network is allowed to indicate UL overriding by RRC.

Proposal 2: In collision case 4 (Dynamic scheduled DL reception collides with Dynamic scheduled UL transmission), the following handling rules should be taken into consideration:

- The collision between Dynamic scheduled PDSCH and the dynamic scheduled PUSCH, PUCCH, and PDCCH order triggered PRACH should be left to UE implementation to prioritize UL transmission or DL reception,

- If dynamic scheduled CSI-RS collide to PDCCH order triggered PRACH or PUSCH, the PRACH or PUSCH should be prioritized

- For all other use cases, the DL should be prioritized

  - Network is allowed to indicate UL overriding DL by RRC configuration.

  - Remaining issues on TA report enhancement

The mismatch between UE and gNB is caused by the granularity of TA reporting, infrequently TA reports and the TA drifting of the TA caused by the movement of the satellite. In the RAN1 118 meeting, the following agreement about TA reporting enhancements was approved to study to reduce the TA mismatch.

Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied

In the current spec, the granularity of TA reporting is 1ms and the TA offset threshold can be configured with a smaller value (e.g., 0.5ms). If a smaller TA offset threshold is configured, it would trigger UE frequently reporting TA which would lead to more power consumption of UE and would increase the signaling overhead of the system. The maximum TA mismatch caused by the limited value of the TA threshold can be controlled by the configuration of gNB. If finer granularity is introduced for the TA report, at most 1ms resource can be additional used by the UE in a switching duration. The benefit of the enhancement is very limited.

Another factor is the TA drifting due to the propagation delay of the TA report. The rough TA drift rate can be determined by the height of the orbit. In the scenario of LEO with 600km height, 3dB beam width, and 60-degree elevation angle, the propagation delay for a UE in the center of the cell in the UE-satellite-gNB link can be 600km3*108ms*sin60°=4.6ms. The timing drift rate is equal to 23.31 us/s. Therefore, the TA mismatch caused by propagation delay is 23.31 us/s*4.6ms = 0.1107226us. It can be covered by the CP which is equal to 4.7us with 15kHz SCS. In this case, the difference would not introduce symbol and slot level mismatch.

One more factor mentioned was whether TA reporting is supported by the UE. In our opinion, the maximum TA variation can be estimated by the gNB based on the maximum propagation delay, and the timing drift caused by the movement of satellite and the movement of the UE. It can be up to gNB implementation based on the conservation schedule for that kind of low-capability UE.

Observation 1: If the TA report is supported by a UE, a smaller TA offset threshold leads to more power consumption of UE and increases the signaling overhead of the system, and the maximum TA mismatch caused by the limited value of the TA threshold can be controlled by the configuration of gNB. The benefit of smaller granularity is limited. The TA mismatch caused by TA drifting can be covered by CP.

Observation 2: If the TA report is not supported by the UE, the maximum TA variation can be estimated by the gNB based on the maximum differential delay and the timing drift. The maximum differential delay in the cell can be calculated by the gNB. The gNB can perform scheduling based on the estimated maximum TA variation to avoid UL and DL collision.

Proposal 3: No enhancement on TA report in Rel-19.

- Conclusion

In this contribution, we provide our views on the support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN band. According to the discussions, we have the following observations and proposals:

Observation 1: If the TA report is supported by a UE, a smaller TA offset threshold leads to more power consumption of UE and increases the signaling overhead of the system, and the maximum TA mismatch caused by the limited value of the TA threshold can be controlled by the configuration of gNB. The benefit of smaller granularity is limited. The TA mismatch caused by TA drifting can be covered by CP.

Observation 2: If the TA report is not supported by the UE, the maximum TA variation can be estimated by the gNB based on the maximum differential delay and the timing drift. The maximum differential delay in the cell can be calculated by the gNB. The gNB can perform scheduling based on the estimated maximum TA variation to avoid UL and DL collision.

Proposal 1: Support one bit RRC configuration to indicate UL overriding DL for all the collision cases except for collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode.

Proposal 2: In collision case 4 (Dynamic scheduled DL reception collides with Dynamic scheduled UL transmission), the following handling rules should be taken into consideration:

- The collision between Dynamic scheduled PDSCH and the dynamic scheduled PUSCH, PUCCH, and PDCCH order triggered PRACH should be left to UE implementation to prioritize UL transmission or DL reception,

- If dynamic scheduled CSI-RS collide to PDCCH order triggered PRACH or PUSCH, the PRACH or PUSCH should be prioritized

- For all other use cases, the DL should be prioritized

  - Network is allowed to indicate UL overriding DL by RRC configuration.

Proposal 3: No enhancement on TA report in Rel-19.

- Reference

- RP- 243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT.