3GPP TSG RAN WG1 #120		R1-2500436

Athens, Greece, February 17th – 21st, 2025

Title:	[Draft] reply LS on soft satellite switch with re-sync

Response to:	R2-2410978

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	OPPO [to be RAN1]

To:	RAN2

Cc:

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN1 would like to thank RAN2’s LS on DL coverage enhancement, and RAN2’s LS provides the following questions to ask for RAN1’s feedback.

# 1. Overall Description: For soft satellite switching with re-synchronization, to ensure non-colliding SSB with same PCI at the UE side, RAN2 has discussed one possible solution to introduce a temporarily shifted SSB pattern during [t-ServiceStart, t-Service]. To be specific, the target satellite transmits SSB during [t-ServiceStart, t-Service] which is not overlapping with SSB from the source satellite. At t-Service, the new serving satellite SSB pattern (e.g., SSB offset) is restored to the same as the previous source satellite. RAN2 wonders whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. # 2. Actions: To RAN4, RAN1 ACTION: RAN2 kindly asks RAN4 and RAN1 whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

RAN1 response

From RAN1 understanding, as long as the offset value (ssb-TimeOffset) is fixed and known to the UE, it will not impact the DL synchronization. The UE obtains the DL sync within [t-ServiceStart, t-Service] and the UE will continue to track the SSB by removing the offset after t-Service.

# 2	Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above responses into consideration in the future work.

# 3	Date of next TSG-RAN WG1 meeting

TSG-RAN WG1 Meeting #120bis        April 7-11        Wuhan, China