3GPP TSG RAN WG1 #120		R1-2500458

Athens, Greece, February 17th – 21st, 2025

Source:	OPPO

Title:	Discussion on RAN4 LS for CSI-RS measurement with SBFD operation

Agenda Item:	5

Document for:	Discussion and Decision

# Introduction

This contribution elaborates our views on the responses to RAN4 LS in [1].

# Discussion

Issue #1: Whether the “semi-statically configured DL reception” in the Case-2 collision 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol includes CSI-RS resources? Specifically, if semi-statically configured CSI-RS resources for above mentioned measurements collide with dynamically scheduled UL transmission in the SBFD slots or symbols, which action should the UE prioritize: performing measurements on the semi-statically configured CSI-RS or executing the dynamically scheduled UL transmission?

The RAN1 #116 discussion for the Case-2 collision leaves the following moderator comments in the FL summary, which shows that CSI-RS is considered as an example for semi-statically configured DL reception.

| Company | Comments |
| --- | --- |
| Moderator | The proposal is to follow the existing rules as for Rel-17 HD-FDD RedCap UEs. Agreements: (RAN1#104bis-e) […] - For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier/single cell in unpaired spectrum - The semi-statically configured DL reception may include PDCCH (excluding ULCI), SPS PDSCH, CSI-RS or PRS. - FFS on PDCCH carrying ULCI, including whether or not it is supported by RedCap UEs (including potential difference between HD vs. FD RedCap UEs) - The dynamically scheduled UL transmission may include PUSCH, PUCCH, SRS or PRACH triggered by PDCCH order Agreements: (RAN1#105) - For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), a HD-FDD RedCap UE is not required to monitor ULCI - No special handling on the priority rule for PDCCH carrying ULCI. |

Proposal 1: For Issue #1 in R4-2420165, RAN1 responds as following.

- The “semi-statically configured DL reception” in the Case-2 collision 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol includes CSI-RS resources.

- Specifically, if semi-statically configured CSI-RS resources collide with dynamically scheduled UL transmission in the SBFD slots or symbols, the UE prioritizes the dynamically scheduled UL transmission over the CSI-RS reception.

Issue #2: Whether the following agreements on resource configuration and/or reporting configuration apply to CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement?

Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. · Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only · Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols · FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configuration 2 via RRC configuration. Select from the following options - Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis - Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded. - Option 3: Configuration 1 or Configuration 2 is on a per cell basis. - No separate UE capabilities for Configuration 1 and 2. Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

In our understanding, the above RAN1 agreements were made in the assumption of being agnostic to different purposes of measurements based on CSI-RS. We believe such assumption to be continuously valid for purpose of simplicity. Therefore, the answer to RAN4 question in Issue #2 should be yes.

Proposal 2: For Issue #2 in R4-2420165, RAN1 responds as following.

- The concerned agreements provided within Issue #2 on resource configuration and/or reporting configuration apply to CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.

# Conclusion

This contribution concludes with following proposals:

Proposal 1: For Issue #1 in R4-2420165, RAN1 responds as following.

- The “semi-statically configured DL reception” in the Case-2 collision 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol includes CSI-RS resources.

- Specifically, if semi-statically configured CSI-RS resources collide with dynamically scheduled UL transmission in the SBFD slots or symbols, the UE prioritizes the dynamically scheduled UL transmission over the CSI-RS reception.

Proposal 2: For Issue #2 in R4-2420165, RAN1 responds as following.

- The concerned agreements provided within Issue #2 on resource configuration and/or reporting configuration apply to CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.

# Reference

- R1-2500019(R4-2420165), LS on CSI-RS measurement with SBFD operation, RAN4