3GPP TSG RAN WG1 #120                         	R1-2500493

Athens, GR, February 17th – 21st, 2025

Agenda Item:	9.4.3

Source:	InterDigital, Inc.

Title:	Discussion on timing acquisition and synchronization

Document for:	Discussion and Decision

# Introduction

RAN approved a work item on solutions for Ambient IoT in NR in Rel-19 [2]. Ambient IoT (AIoT) is a new 3GPP IoT technology which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications[2]. The scope for this work item in Rel-19 is as follows:

This WI is to standardize RAN aspects of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The work shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power.

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- The overall objective shall be to standardize the following Ambient IoT device:

Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Deployment scenario 1 with Topology 1, according to D1T1-B.

- FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

- Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

- Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command).

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769:

  - Case 1-4 for D1T1-B

- Proximity determination via Solution 1 in TR 38.769 only.

- Device (un)availability via Direction 1 in TR 38.769 only.

Specific RAN WG1 objectives are included below [2]:

- PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively.

- R2D and D2R signal(s)

- Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA.

- R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations.

- R2D transmission supports only the Manchester line code in TR 38.769

- D2R transmission supports:

  - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and

  - A corresponding small frequency shift method according to the options in TR 38.769.

- R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis).

- PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1.

- D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

This contribution discusses InterDigital Inc.’s views on design aspects related to timing acquisition and synchronization

# Discussion

## R2D Physical Signals

### R2D Preamble Design

In the Rel-19 SI phase [1], a timing acquisition signal was studied for PRDCH transmissions with the understanding that it would be beneficial to include a separate signal identifiable for:

- Performing timing synchronization and identifying the beginning of a PRDCH transmission

- Determining the OOK chip duration for the PRDCH following the preamble

As no scenarios have been identified where it would be beneficial to transmit PRDCH without preamble, it should be agreed that every PRDCH is immediately preceded with a preamble sequency including a start indicator part (SIP) for identifying the beginning of the PRDCH transmission, and a clock acquisition part (CAP) for determining the chip duration used for the OOK modulation of the PRDCH.

Proposal 1:	A preamble immediately precedes every PRDCH transmission including:

- A start indicator part (SIP) used for a device receiving the PRDCH transmission to perform timing synchronization and identify the beginning of the PRDCH transmission

- A clock acquisition part (CAP) used for a device receiving the PRDCH transmission to determine the chip duration used to modulate/encode the PRDCH transmission

It was further agreed that for OOK-1 and OOK-4 transmission modulation, the if the reader assumes the PRDCH transmission is aligned with the boundary of an NR OFDM symbol, then CP-OFDM can be feasibly implemented for OOK-1 and DFT-s-OFDM and CP-OFDM can be feasible implemented for OOK-4 [1]:

For this waveform, the start of R2D transmission from the reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation. For the generation of this waveform, CP-OFDM and DFT-s-OFDM are both feasible. For M=1, i.e. use of OOK-1 or OOK-4 for single-chip per OFDM symbol transmission, CP-OFDM and DFT-s-OFDM are both feasible. For M>1, i.e. use of OOK-4 for M-chip per OFDM symbol transmission, DFT-s-OFDM is feasible.

Additionally, two methods were studied for the handling of CP insertion as part of the NR waveform for both DFT-s-OFDM and CP-OFDM [1]:

Method Type 1:	Removal of CP at device without specified transmit-side.

Method Type 2:	Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n.

In both cases, there is a benefit in aligning PRDCH transmissions with OFDM symbol boundaries to ease device implementation in handling CP insertion.  Even in situations where the reader ensures that the CP does not produce unintended rising/falling edges in the PRDCH transmission, if the start of the PRDCH transmission aligns with the NR OFDM symbol boundary the device does not need to first identify the location of the NR CP for handling variation in the PRDCH chip duration caused by CP insertion.

Proposal 2: The start and end of a SIP transmission by a reader should align with the start and end of an OFDM symbol in carrier in which the reader transmission is transmitted.

The CAP of the preamble is intended to enable the receiving device to determine the chip duration for OOK modulation used by the reader since it cannot be assumed that the receiving device will be capable of maintaining sampling frequency synchronization with the reader or maintain prior knowledge of the configuration used by the reader initiating the service procedure. To ease device implementation in detecting the CAP and estimating PRDCH chip rate, the CAP should also align with OFDM symbol boundaries.

Proposal 3: The start and end of a CAP transmission by a reader should align with the start and end of an OFDM symbol in carrier in which the reader transmission is transmitted.

There has not been agreement on whether a PRDCH will multiplex L1 control information with relevant R2D with a fixed format L1 control signal as an example, and it is not in scope for this AI, but whether L1 control is multiplexed with R2D data or not may have design implications for the CAP [1].  If a single fixed preamble design is used regardless of modulation chip rate, it may be necessary to signal the R2D chip rate relative to the CAP chip rate; however, this would require the device to know how to recover the scaling rate prior to decoding R2D data.  Alternatively, if the CAP chip duration scales with PRDCH chip duration, then the device can estimate the PRDCH chip rate without any further L1 control information.  For this reason, the chip duration of the CAP should scale with either the chip duration of the L1 control data configuring R2D data, or the entirety of the PRDCH.

Observation 1:	If L1 control, including PRDCH chip duration for R2D transmission, is not included in R2D transmission, the device must determine the chip duration explicitly from the CAP.

Proposal 4:	Unless a L1 control signal with a fixed format and dedicated CRC is included in PRDCH, the CAP is transmitted with the same chip rate as the PRDCH.

Proposal 5:	The chip duration of the CAP sequence in the R2D preamble should be equivalent to either the chip duration of L1 control information for configuring R2D data transmission or the entire PRDCH transmission.

Additionally, the number of chips in the CAP should be constant regardless of the modulation order configured for the PRDCH.  This can avoid complex device implementation requiring the joint estimation of chip duration and number of chips in the CAP.  To ensure the CAP aligns with OFDM symbol boundaries, the number of chips in the CAP should be selected such that the number of chips in the CAP have duration equal to at least 1 OFDM symbol or a number that equates to the CAP having duration equal to an integer multiple of the OFDM symbol duration.

Proposal 6:	CAP should use a fixed number of chips regardless of chip duration.

- The number of chips in the CAP should be such that CAP duration is always at least 1 NR OFDM symbol or integer multiples of an NR OFDM symbol in the carrier in which the PRDCH is being transmitted.

### R2D Post-Amble Design

Similar to discussions regarding preamble design for the PRDCH, the need for a R2D post-amble will be dependent on whether there is agreement to support L1 R2D control information multiplexed in the PRDCH.  Two options were identified during the SI phase for determining the end of PRDCH transmission [1]:

To determine or derive the end of PRDCH transmission, the following options are studied:

Option 1: TBS information via implicit/explicit L1 R2D control information.

Option 2: Postamble at the end of PRDCH.

As with chip duration, if the TBS for the R2D is not explicitly indicated, a preamble may be necessary for determining the end of the PRDCH transmission.

Observation 2:	If L1 control, including TB size for R2D transmission. is not included in R2D transmission, a post-amble is necessary for device to identify the end of the R2D transmission.

Decision on whether an R2D post-amble should immediately follow PRDCH transmission should be decided after there is consensus on whether L1 control information will be multiplexed in the PRDCH.

Proposal 7:	Support a post-amble appended to the end of every PRDCH transmission if RAN WG1 agrees that a fixed format L1 control signal with dedicated CRC is not included at the beginning of every PRDCH.

## D2R Physical signals

During the study item phase, the following observation was agreed regarding the design of D2R preamble immediately preceding PDRCH transmission [1]:

A D2R timing acquisition signal (D-TAS), preceding each PDRCH, is included at least for timing acquisition, indicating the start of the D2R transmission in the time domain, and studied potentially for SFO estimation, CFO estimation, channel estimation, and interference estimation. A D-TAS structure using a preamble is studied, for which a binary signal is considered. The preamble is not part of PDRCH.

In order to reduce reader implementation, it would be beneficial if in addition to timing acquisition, the preamble enabled the reader to estimate the channel and synchronize with the device, since the schedule for the PDRCH transmission may be variable due to TDMA and scheduling load. Poor clock stability may result in the device sampling frequency not being adequately synchronized by the time the PDRCH is transmitted.

Proposal 8:	A preamble immediately precedes every PDRCH transmission to enable at least:

- Detection of start of D2R transmission

- D2R timing synchronization

- Channel estimation

Additionally, configuration of the D2R preamble, mid-amble, and post-amble, if supported, should be fully under control of the reader to avoid ambiguity in proper reception of the PDRCH.

Proposal 9:	D2R preamble, mid-amble, and post-amble, if supported, for any PDRCH transmission can be configured by prior PRDCH transmission to the device.

# Conclusion

The following observations and proposals are provided for discussion in RAN1#120:

Observation 1:	If L1 control, including PRDCH chip duration for R2D transmission, is not included in R2D transmission, the device must determine the chip duration explicitly from the CAP.

Observation 2:	If L1 control, including TB size for R2D transmission. is not included in R2D transmission, a post-amble is necessary for device to identify the end of the R2D transmission.

Proposal 1:	A preamble immediately precedes every PRDCH transmission including:

- A start indicator part (SIP) used for a device receiving the PRDCH transmission to perform timing synchronization and identify the beginning of the PRDCH transmission

- A clock acquisition part (CAP) used for a device receiving the PRDCH transmission to determine the chip duration used to modulate/encode the PRDCH transmission

Proposal 2: The start and end of a SIP transmission by a reader should align with the start and end of an OFDM symbol in carrier in which the reader transmission is transmitted.

Proposal 3: The start and end of a CAP transmission by a reader should align with the start and end of an OFDM symbol in carrier in which the reader transmission is transmitted.

Proposal 4:	Unless a L1 control signal with a fixed format and dedicated CRC is included in PRDCH, the CAP is transmitted with the same chip rate as the PRDCH.

Proposal 5:	The chip duration of the CAP sequence in the R2D preamble should be equivalent to either the chip duration of L1 control information for configuring R2D data transmission or the entire PRDCH transmission.

Proposal 6:	CAP should use a fixed number of chips regardless of chip duration.

- The number of chips in the CAP should be such that CAP duration is always at least 1 NR OFDM symbol or integer multiples of an NR OFDM symbol in the carrier in which the PRDCH is being transmitted.

Proposal 7:	Support a post-amble appended to the end of every PRDCH transmission if RAN WG1 agrees that a fixed format L1 control signal with dedicated CRC is not included at the beginning of every PRDCH.

Proposal 8:	A preamble immediately precedes every PDRCH transmission to enable at least:

- Detection of start of D2R transmission

- D2R timing synchronization

- Channel estimation

Proposal 9:	D2R preamble, mid-amble, and post-amble, if supported, for any PDRCH transmission can be configured by prior PRDCH transmission to the device.

# References

R1-2410929,	“Study on solutions for ambient IoT (Internet of Things) (Rel-19)” v2.0.0

RP-243327,	“New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei)

RP-240826, 	“Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA.

3GPP TR 38.848, 	“Study of Ambient IoT (Internet of Things) in RAN”, v18.0.0.

3GPP TR 22.840, 	“Study on Ambient power-enabled Internet of Things”, v19.0.0.

EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID – Specification for RFID Air Interface – Version 2.0.1.

R1-2404520, 	“Downlink and uplink channels aspects of Ambient IoT”, InterDigital.

R2-2409412	“LS on assistance information from the CN to the Reader for A-IoT”	RAN2[1]