3GPP TSG RAN WG1 #120                                  			               R1-2500501

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.11.1

Source:	InterDigital, Inc.

Title:	NR-NTN downlink coverage enhancement

Document for:	Discussion

# Introduction

In RAN #102 meeting, the WI for R19 NR NTN for Phase 3 [1] is approved and revised in RAN #106 meeting as [2]. For DL coverage enhancement, NR NTN phase 3 has the following objective to improve the DL coverage through link level and system level enhancements:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

In RAN1 #119 [3] meeting, following agreements were reached for NR NTN DL coverage enhancement:

Agreement For PDSCH with Msg4 Link level enhancement: - Support PDSCH repetition - FFS: signalling design including number of repetitions - FFS: impact on UE capability - Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms. Agreement For link level enhancement of PDSCH with SIB1: - Support PDSCH repetitions within 20 ms duration - The number of repetitions is fixed to 2 repetitions - Further discuss the specification impact for at least the following: - Procedure and signaling (enabling repetitions, associated time resource determination, etc.) - Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size. - Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern. Agreement For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN. - FFS: intra-slot and/or inter-slot

In this contribution, we discuss some aspects of link and system level enhancements to improve DL coverage of NR NTN.

# System Level Enhancements for NTN

During the Rel-19 scoping for NR NTN WID [1], many satellite operators brought the transmission power constraints issue for the satellites and the need to do dynamic power adaptation in a flexible manner. The power adaptation can be fractional or binary (ON/OFF). Similarly, the WID notes the possibility to change the spatial footprint of the satellite beams.

3GPP specified a number of network energy saving techniques in Rel-18 NES [4] work. It was noted that in NTN Rel-19 WID that energy saving mechanisms standardized in Rel-18 NES WID [4] may be considered as baseline. These mechanisms may be considered as candidates for power sharing among satellite beams and to save energy at the satellite payloads. In this section, we study their potential use for NTN networks, both for power sharing among satellite beams and for energy saving purpose.

## Satellite Beam Hopping and SSB Periodicity

The beams transmitted by the satellites may be turned on/off in a dynamic manner. This may follow a periodic pattern. Cell DTx and cell DRx mechanisms standardized in Rel-18 NES [5] are applicable at cell level. Cell DTx/DRx may provide a baseline to perform power sharing among satellite beams and also to activate and de-activate a sub-set of beams in a periodic manner.

Observation 1: NES Cell DTx and cell DRx mechanisms may provide a baseline for NTN scenarios to enable beam hopping among satellite beams.

Cell DTx and cell DRx mechanisms standardized in Rel-18 NES have their usage limited to RRC CONNECTED UEs. The impact of NES cell DTx/DRx procedures is limited to UE specific channels/signals. NTN systems with beam hopping need DTx/DRx mechanisms where the whole HW/RF chains can be switched from one satellite beam to another, this will require DTx/DRx mechanisms impacting the transmission/reception of all channels/signals, i.e., common channels and UE specific channels.

Proposal 1: Support NTN DTx/DRx mechanisms which determine the transmission/reception of common and UE specific channels/signals.

The configuration and signaling for cell DTx/DRx mechanisms in Rel-18 NES is also limited to RRC connected UEs. In addition to RRC based configuration/activation, a group common DCI can be used for activation purpose if the RRC configurations have been provided. This is not compatible to NTN beam hopping scenarios where idle UEs need to be supported for beam hopping.

Observation 2: The configuration and signaling mechanisms for NES cell DTx/DRx are not applicable in NTN scenarios.

To enable satellite beam hopping, where both idle and connected mode UEs can receive information about the NTN DTx/DRx mechansims, common signaling can be used to transmit this information. This can be achieved by providing DTx/DRx configuration and/or activation as part of the system information.

Proposal 2: Support common signaling for configuration/activation of NTN DTx/DRx mechanisms.

A single DTx/DRx pattern may not be suitable for all the UEs/devices present in the large coverage area of a satellite beam. The idle mode UEs present in the coverage of a satellite beam may need to listen only paging and system information while connected mode UEs may have different activity levels and different transmit/receive patterns. As an example, the connected mode UEs in active DL/UL communication may need regular activity intervals for transmission/reception, and if the DTx/DRx periodicity is larger, that may have adverse effect on the QoS for such UEs.

Observation 3: A single DTx/DRx pattern may not be suitable for NTN due to wide coverage of satellite beams supporting UEs in different RRC states and with different activity levels.

To accommodate different suitable periodicities and active duration for idle and connected mode UEs, the network can provide a first DTx/DRx pattern which all UEs follow, i.e., both idle and connected mode UEs. The network can use the first pattern for common channels/signals. Then for the connected mode UEs, the network can provide additional active durations for DTx and/or DRx. In one approach, the network can activate an additional DTx/DRx configuration which may be applicable only for connected mode UEs. Another approach can be to increase the active duration for the DTx/DRx configuration where the additional increased duration can be indicated only to connected mode UEs. No matter which approach is used, given that the connected mode UEs are in active communication, it’s easier to have the dynamic updates for the DTx/DRx configuration or active duration destined only to connected mode UEs.

Proposal 3: Support network indication to connected mode UEs an additional active duration for DTx/DRx through the following mechanisms:

- Simultaneous activation of multiple DTx/DRx patterns.

- Additional active time for the active DTx/DRx pattern.

Due to large number of beams employed by a satellite and very large variations in number of users and traffic load per beam, the real benefit of beams turning on/off or beam hopping can be harnessed by allowing an increased SSB periodicity. To that end, RAN1 #118Bis [3] agreed to support at least 160 ms for SSB transmission, in addition to existing value of 20 ms). The agreement has an “FFS” whether 320 ms can be supported as the maximum of the additional default value. The updated NTN WID mentions the max value of SSB periodicity to be 160 ms. Based upon the updated WID for NR NTN, RAN1 does not need to study SSB periodicities higher than 160 ms.

Proposal 4: RAN1 updates the agreement from R1 #118Bis as in the following:

For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

- The maximum of the additional default value (apart from the existing 20ms value) is at least most 160 ms.

An increased SSB periodicity allows revisiting a beam coverage area after longer gap, thus may contribute to additional power saving and enable coverage of large satellite footprint. RAN1 needs to investigate the impact of larger SSB periodicity on other UE procedures, e.g., system information transmission/acquisition, paging, random access.

Proposal 5: RAN1 investigates the impact of increased SSB periodicity on the following:

- System information transmission/acquisition

- Paging

- Random access

## Wide and Narrow Satellite Beams

In one method of coverage enhancement, the satellite could adapt its beam footprint, e.g., wide vs narrow footprint. Spatial footprint adaptation may enable control on how many beams are required to serve a given coverage area, and eventually help handling the power constraints at the satellite. Adaptation of spatial beam footprint may have some relevance to spatial domain adaptations standardized in Rel-18 NES. The basic framework for spatial domain adaptations hinges upon the network getting per UE CSI reports for a number of sub-configurations and then adapting the UE dedicated transmissions either for Type 1 spatial adaptation (antenna port muting) or Type 2 spatial adaptations (same number of ports but port spatial footprint update) based upon the feedback received from the UE. It would be important to highlight that adaptations based upon UE feedback will be primarily targeting connected mode UEs.

From this perspective, spatial domain adaptations as standardized in NES may have very large operating overhead as it will require multiple CSI sub-configurations reported to the network from each UE. Given the fact that typically NTN UEs have medium to small data rate requirements, the feedback overhead may largely exceed the potential benefit if any.

Observation 4: Spatial domain adaptation techniques standardized in Rel-18 NES work may have very large overhead for NTN scenarios and use cases to adapt the beam footprint.

The satellite beams may be transmitted applying spatial adaptations to the antennas and antenna elements to choose wider or narrower footprint to suit the coverage requirements and active traffic with minimal feedback from the UEs. The simpler spatial domain adaptation without any dynamic feedback from the UEs will be the key given the large number of UEs in the satellite coverage areas.

Proposal 6: Satellite beams may adapt spatial footprint based upon coverage requirements and active traffic without UE feedback.

A LEO-600 satellite may have coverage footprint of ~1.8 million km2, equivalent to 1058 beams for a reference beam diameter of 50 km [RAN1 R-19 assumptions]. The payload constraints in terms of (i) available power, (ii) hardware (RF chains / antenna arrays) and (iii) feeder link capacity, may limit the number of active beams to 10%, 5%, or even 2.5%. With limited number of active beams, the satellite has to illuminate several beam footprints for the transmission of SSBs and common channels in a time domain multiplexed manner, resulting in very fast hopping of the active beams. The fast beam hopping in narrow beams results in significant time/resource overhead for DL common channels transmission, given that the network has to do the periodic transmissions of SSB/SIBs throughout the satellite coverage.

Observation 5: Transmission of SSBs and common channels through narrow beams results in large power and communication resource (T-F) overhead for satellite systems.

Satellite operators can use wide beams for common DL channels transmission (SSB, SIBs), and narrow beams for UE dedicated channels. With wide beams, the overhead of common channel transmissions reduces. This also alleviates the fast beam hopping requirement at the satellite.

Observation 6: Wide satellite beams may be helpful to reduce the common channel overhead and alleviate the beam hopping requirements.

Based upon the discussion on wide and narrow beams, we support the use of wide beams to transmit SSBs and common channels.

Proposal 7: Wide satellite beams are supported for the transmission of SSBs and system information.

If the UEs performing initial access transmit RACH based upon wide beam and the gNB has to detect/decode RACH through wide beam based receiver, RACH reception will suffer significant coverage degradation. This may be not desirable for the operator as the RACH degradation will result in UEs unable to attach and camp on the NTN cells.

Observation 7: RACH transmissions against wide beams may lead to uplink coverage degradation due to gNB receiving RACH over wide beam, resulting in some UEs unable to camp on NTN cells.

If the UEs transmit RACH based only upon the wide beam SSB and common channel reception, one additional issue is the UE Tx filter used to transmit RACH. If the UE’s have not received/detected any signals through their relevant narrow satellite beam, they may not be able to choose a suitable Tx filter.

Observation 8: The UEs may not be able to use a suitable Tx filter for their RACH transmission without any signals received through narrow beams.

To avoid the RACH coverage degradation due to RACH transmission over the wide satellite beam, the UEs should be transmitting RACH over the narrow beams. The network can then receive the UE transmitted RACH through its suitable narrow beam based receiver, avoiding the degradation.

Proposal 8: The UEs perform RACH transmission on their relevant narrow satellite beams.

For UEs to transmit RACH on a suitable narrow satellite beam, the UEs need to be able to identify the narrow beam, where the network may be operating several narrow satellite beams within the coverage of one wide beam.

Observation 9: The UEs need to identify a suitable narrow satellite beam to transmit RACH against the narrow beam.

The UEs can measure some reference signals, e.g., CSI-RS, transmitted through narrow beams to identify a suitable narrow beam. In addition to selecting a suitable narrow satellite beam, the UEs can choose a Tx filter for RACH transmission which is matched to Rx filter with which they performed measurements on narrow beam RSs.

Proposal 9:  The UEs identify narrow beams for RACH transmission based upon measurements of reference signals transmitted through narrow beams.

The UEs may receive the measurement configurations for narrow beam CSI-RS through the system information transmitted by wide satellite beams. To provide an association of narrow beams and RACH occasions (RO), and to minimize the specification impact, the existing RACH mappings for SSB-RO can be reused for wide/narrow beam based systems, where the CSI-RS of narrow beams are indicated instead of legacy SSBs.

Proposal 10: The UEs identify RACH resource based on measurement of CSI-RSs associated with narrow beams, where the CSI-RSs and CSI-RS to RO mapping are configured by wide beam system information.

# Link Level Enhancements for NTN

After discussing some system level enhancements to save energy and improve DL coverage, this section focuses on link level enhancements to improve DL coverage. The link level coverage enhancements are needed to compensate the coverage loss resulting from power sharing among the beams. The power sharing among the satellite beams may be binary (ON/OFF) or flexible as discussed in the previous section. In Rel-19 NTN WID, SSB enhancements are not considered. So the link level enhancements should be applied to other DL channels, e.g., PDCCH and PDSCH. The link level enhancements are discussed for PDCCH and PDSCH in the following sub-sections.

## Link Level Enhancements for PDCCH

RAN1 study found that there is coverage gap for common PDCCH, which is used for system information transmission and during initial access. In addition, if the NTN system uses wide beams, the UE received power may further reduce due to loss of beamforming gain. In one approach, the network can use higher aggregation factor for PDCCH transmission to compensate the PDCCH coverage gap. Another method can be to do the PDCCH repetition. A single PDCCH can be repeated in a single occasion or across multiple occasion, providing the additional energy to allow the UEs decode their relevant PDCCH. This needs to be designed carefully to keep the blind decode efforts within reasonable limits.

RAN1 #118Bis [4] agreed to study PDCCH repetition where two options were noted:

- Option 1: Intra-slot PDCCH repetition

- Option 2: Inter-slot PDCCH repetition

PDCCH repetitions can use the search space linkage from multi-TRP as baseline.

Proposal 11: PDCCH repetitions use linked search spaces from multi-TRP framework as baseline.

In one approach, the network can transmit PDCCH to a UE through two different satellite beams. This can for example be of interest for the UEs who are in the overlapping coverage area of more than one satellite beam. The network can configure the UE to report the beams it detects and based upon UE reporting, the network can configure the UE with CORESET/search-space through two satellite beams.

Proposal 12: Support PDCCH repetitions over two or more satellite beams based upon UE reporting.

## Link Level Enhancements for PDSCH

When the satellite beams are undergoing cell DTx mechanism, the network will not schedule any PDSCH. The UEs may though be performing blind decodes on the configured DL transmission occasions. There is no risk of increased UE power consumption if the UEs are informed of the active DTx procedure as discussed in Section 2.1.

RAN1 #118 Agreement

As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider:

- Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

  - Default value[s] with extended periodicity assumed by NTN UE for initial access can be:

    - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms}

- Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint

- Link-level enhancements for the following channels:

- PDCCH

- PDSCH with Msg 4

- PDSCH with SIB1/SIB19.

  - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact.

Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

As simulation campaign showed coverage gap for PDSCH Msg 4/SIB1/SIB19, these channels were noted as candidates for link level enhancements. If wide beam transmissions are supported for common channels, the received power at the UE for some of these channels may further reduce. The simplest solution to improve the PDSCH coverage for these channels can be to introduce PDSCH repetitions. A single PDSCH can be repeated multiple times and the UEs can be instructed to combine appropriately improving the PDSCH coverage. For PDSCH with Msg 4, the repetitions can be indicated as part of the system information, or as part of Msg 2.

Proposal 13: The repetitions for PDSCH with Msg 4 can be indicated as part of the system information or in Msg 2.

# Conclusions

In this contribution, the following observations are made:

Observation 1: NES Cell DTx and cell DRx mechanisms may provide a baseline for NTN scenarios to enable beam hopping among satellite beams.

Observation 2: The configuration and signaling mechanisms for NES cell DTx/DRx are not applicable in NTN scenarios.

Observation 3: A single DTx/DRx pattern may not be suitable for NTN due to wide coverage of satellite beams supporting UEs in different RRC states and with different activity levels.

Observation 4: Spatial domain adaptation techniques standardized in Rel-18 NES work may have very large overhead for NTN scenarios and use cases to adapt the beam footprint.

Observation 5: Transmission of SSBs and common channels through narrow beams results in large power and communication resource (T-F) overhead for satellite systems.

Observation 6: Wide satellite beams may be helpful to reduce the common channel overhead and alleviate the beam hopping requirements.

Observation 7: RACH transmissions against wide beams may lead to uplink coverage degradation due to gNB receiving RACH over wide beam, resulting in some UEs unable to camp on NTN cells.

Observation 8: The UEs may not be able to use a suitable Tx filter for their RACH transmission without any signals received through narrow beams.

Observation 9: The UEs need to identify a suitable narrow satellite beam to transmit RACH against the narrow beam.

The observations and the discussion have led to the following proposals:

Proposal 1: Support NTN DTx/DRx mechanisms which determine the transmission/reception of common and UE specific channels/signals.

Proposal 2: Support common signaling for configuration/activation of NTN DTx/DRx mechanisms.

Proposal 3: Support network indication to connected mode UEs an additional active duration for DTx/DRx through the following mechanisms:

- Simultaneous activation of multiple DTx/DRx patterns.

- Additional active time for the active DTx/DRx pattern.

Proposal 4: RAN1 updates the agreement from R1 #118Bis as in the following:

Proposal 5: RAN1 investigates the impact of increased SSB periodicity on the following:

- System information transmission/acquisition

- Paging

- Random access

Proposal 6: Satellite beams may adapt spatial footprint based upon coverage requirements and active traffic without UE feedback.

Proposal 7: Wide satellite beams are supported for the transmission of SSBs and system information.

Proposal 8: The UEs perform RACH transmission on their relevant narrow satellite beams.

Proposal 9:  The UEs identify narrow beams for RACH transmission based upon measurements of reference signals transmitted through narrow beams.

Proposal 10: The UEs identify RACH resource based on measurement of CSI-RSs associated with narrow beams, where the CSI-RSs and CSI-RS to RO mapping are configured by wide beam system information.

Proposal 11: PDCCH repetitions use linked search spaces from multi-TRP framework as baseline.

Proposal 12: Support PDCCH repetitions over two or more satellite beams based upon UE reporting.

Proposal 13: The repetitions for PDSCH with Msg 4 can be indicated as part of the system information or in Msg 2.

# Reference

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, RAN #102, December 2023.

- RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, RAN #106, December 2024

- RAN1 Chairman’s note, RAN1 #119, November 2024.

- RAN1 Chairman’s note, RAN1 #118Bis, October 2024.

- RAN1 Chairman’s note, RAN1 #118, August 2024.

- RP-223540, NES Rel-18 WID, “New WID: Network energy savings for NR”, Huawei