3GPP TSG RAN WG1 #120 	R1- 2500510

Athens, Greece, 17 – 21 February 2025

Agenda item:		5

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on LS from RAN2 on PWS reception in RRC_CONNECTED

Document for:		Discussion and Decision

# Introduction

In RAN2 #128 meeting, there is one LS from RAN2 [1] to RAN1 on PWS reception in RRC CONNECTED mode.

1. Overall Description:

A revised WID was approved in RAN#105 (Sept 2024) to further enhance the support of NB-IoT for NTN in RP-242397, with the following new objective added:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

RAN2 discussed whether to introduce support for reception of PWS (ETWS and CMAS) notifications in RRC_CONNECTED for NB-IoT but were not able to conclude.

PWS notifications are delivered via paging or direct indication to trigger acquiring ETWS system information and CMAS system information.

Currently NB-IoT UEs are not required to monitor for paging or receive system information triggered by the paging in RRC_CONNECTED, and RAN2 would like to understand from RAN1 perspective whether it is feasible to introduce this in Rel-19.

2. Actions:

To RAN1

ACTION:	 Please respond with an indication of whether it is feasible from RAN1 perspective to introduce support in Rel-19 for NB-IoT UEs reception of PWS notifications (in paging, direct indication, and system information) while in RRC_CONNECTED.

RAN2 ask RAN1 to provide views on whether it is feasible to support it. We will provide our view on this in the document.

# Discussion

As RAN2 mentioned for current specification status, NB-IoT UEs are not required to monitor for paging and SIB triggered by paging in RRC CONNECTED mode but only in RRC IDLE mode, for both TN and NTN.

This is for the requirement from NB-IoT UE on power saving and network will provide enough SIB information when NB-IoT UE access into network. After a short communication where NB-IoT UE completes the data transmission/reception, then it will go back to RRC IDLE mode for power saving.

Observation 1: Currently NB-IoT UE does not support monitoring for paging and SIB triggered by paging in RRC CONNECTED mode for both TN and NTN.

Observation 2: The reason NB-IoT UE not support paging/SIB triggered by paging is power saving and short communication time between UE and network.

In RRC CONNECTED mode, NB-IoT UE will focus on data transmission/reception with repetitions, where the repetition may last for long time considering the link budget status. Additionally, for more efficient power saving and complexity reduction, most NB-IoT UE will be half duplex device, i.e. while UE make the UL transmission repetition, the UE will not simultaneously perform DL receiving/monitoring, including NPDCCH/Paging/SIB, etc.

While different NB-IoT UE may have different link status and request different number of repetitions with different traffic arriving/active time, it is very difficult to dynamically configure one common time period for all NB-IoT UE in RRC CONNECTED mode to receive SIB and corresponding NPDCCH/Paging.

Observation 3: Half-Duplex NB-IoT UE can not monitor NPDCCH/Paging/SIB when transmitting UL with long repetitions and it is very difficult to dynamically configure one common time for all NB-IoT UEs in RRC CONNECTED mode to receive SIB and corresponding NPDCCH/Paging.

Additionally, NB-IoT UE can not monitor CSS and USS simultaneously as specified in TS 36.213, which also means legacy NB-IoT UE will not receive SIBs or paging in RRC CONNECTED mode.

So to handle all of the above aspects, maybe one simple way is to introduce a new predefined gap that is specially for PWS and reuse the most of legacy processing for UE monitoring PWS notifications delivered via paging or direct indication to trigger acquiring ETWS system information and CMAS system information. But this seems to have no impact on RAN1 work and should be discussed and determined by RAN2.

Considering the limited remaining time in Release 19 for RAN1, it is almost impossible to introduce additional enhancements in RAN1 in order to support PWS for NB-IoT NTN UE. In our view, it is up to RAN2 to decide if support of PWS for NB-IoT NTN UE can be specified without impact on RAN1 work.

Proposal 1: Considering limited remaining time in Rel19, RAN1 do not further discuss PWS for RRC Connected NB-IoT NTN UE in Rel19.

# Conclusion

In this contribution, we discussed our view on PWS reception in RRC CONNECTED mode. Our observations and proposals are as follows:

Observation 1: Currently NB-IoT UE does not support monitoring for paging and SIB triggered by paging in RRC CONNECTED mode for both TN and NTN.

Observation 2: The reason NB-IoT UE not support paging/SIB triggered by paging is power saving and short communication time between UE and network.

Observation 3: Half-Duplex NB-IoT UE can not monitor NPDCCH/Paging/SIB when transmitting UL with long repetitions and it is very difficult to dynamically configure one common time for all NB-IoT UEs in RRC CONNECTED mode to receive SIB and corresponding NPDCCH/Paging.

Proposal 1: Considering limited remaining time in Rel19, RAN1 do not further discuss PWS for RRC Connected NB-IoT NTN UE in Rel19.

# References

- R2-2411194, “LS on PWS support in RRC_CONNECTED for NB-IoT NTN”, November 2024