3GPP TSG RAN WG1 #120		                                    R1-2500512

Athens, Greece, February 17th – 21st, 2025

Source:	Ofinno

Title:	Discussion on AI/ML for beam management

Agenda Item:	9.1.1

Document for:	Discussion and Decision

# Introduction

The WID [1] of “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” has been endorsed in RAN#102 meeting. The objectives related to this agenda item are listed below:

Provide specification support for the following aspects: - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, we present our views on Release 19 AI/ML for beam management.

# Discussions

During the RAN#102 meeting, it has been agreed to specify enhancements for downlink Tx beam prediction for both UE-sided models and NW-sided models. The objective of the WID [1] also includes specifying necessary signaling to facilitate model training, model inference, and performance monitoring of an AI/ML model for beam prediction. Figure 1 illustrates an example signaling procedure of a UE-sided model for beam prediction. For the model training of the UE-sided model, the gNB configures and transmits reference signals for data collection of the model training. The UE receives the reference signals, where measurements on the reference signals are used for the model training performed by the UE or by a model training server. For the model inference, the UE indicates to the gNB on UE capability on AI/ML-enabled Features/FGs. The gNB configures and transmits Set B reference signals. The UE performs the model inference using measurements on the Set B reference signals as input. The UE reports, to the gNB, results of the AI/ML model inference. The UE determines results of performance monitoring by using measurements on the Set B of reference signals and the Set A of reference signals (or Set C that is a subset of Set A). The UE reports, to the gNB, the results of performance monitoring.

- : Signaling procedure of an AI/ML model for beam management

## Overall procedure and configurations

In RAN1#116bis and RAN1 #118 meetings, a high-level procedure for UE-sided model training and support of associated ID for training-inference consistency have been agreed.

Agreement Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded. Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification. Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

RAN2 wants to clarify the following for the discussion of the UE side data collection for UE side model training:

- Data collection refers to the UE collecting data that involves the UE performing measurements (e.g., radio measurements).

- Data transfer refers to the sending/transfer of the collected data from the UE to Server for data collection for UE-side model training/OTT server (as capture in Table 7.2.1.3.2-1) in TR38.843.

SA2 can assume that the NG-RAN is involved in providing radio measurement configuration (if needed) for the UE side data collection at least for the beam management use case, as captured in the following agreement in RAN2-127bis:

Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g., via UE requests (UE directly or UE server))

In RAN2 #128 meeting, RAN2 replied SA2 regarding AI/ML data collection for a UE-sided model [6] including the following RAN2 understanding:

Currently, it is FFS how the NW determines whether data collection should be initiated (e.g., via UE requests (UE directly or UE server)). For example, when data collection is started for UE-sided model, Set A of RSs would be transmitted. Without UE request to initiate and/or stop data collection (e.g., transmission of Set A of RSs), NW may need to transmit Set A of RSs periodically. This could potentially reduce benefits for both BM-Case1 and BM-Case2. To avoid unnecessary transmission of Set A of RSs, we propose UE requests to initiate and stop data collection for UE-sided model. As training may be done at the UE side (e.g., UE local refining), it would be desirable for the UE to trigger initiation and also inform when UE does not require data collection any longer.

For example, UE may need to start a training on a new model e.g., when the current model performance is not reliable. In such a case, UE may need to request to initiate data collection for the new model training. In another example, when either UE finishes training e.g., locally or UE does not perform measurement on the Set A of RSs any longer, UE may request/inform NW to stop data collection.

In both cases, we see benefits of UE request on initiation/termination data collection for a UE-sided model.

- Support UE requests to initiate/terminate data collection for a UE-sided model:

- RAN1 to discuss conditions to trigger UE requests to initiate data collection (e.g., for training a new UE-sided model to potentially replace the current UE-sided model deployed for inference).

- RAN1 to discuss events to trigger UE requests stopping data collection (e.g., indicating the model training is completed in case UE performs the training (local refining)).

## Inference result report

### BM-Case 1

RAN1 agreements related to BM-Case 1 are as follows:

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. Where the set of beams is Set A, i.e., the beams for UE prediction. Agreement - For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A. Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Agreement For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output Agreement For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details

For contents in the report of inference results (e.g., at least for BM-Case 1), support of predicted Top K beam(s) (e.g., Opt. 1) and predicted Top K beam(s) and RSRPs (e.g., Opt. 2) are agreed and support of reporting probability information (e.g., Opt. 3) and confidence information (e.g., Opt. 4) are still FFS. Reporting the probability information generated by a classification model and reporting the confidence information generated by a regression model could be useful for the gNB to know the “good” candidate beams among the reported Top-K beams.

Depending on the type of an AI/ML model (e.g., classification model or regression model), the output type of the model inference can be predicted probabilities of the beams in Set A or predicted RSRPs of the beams in Set A. For example, when a classification model is used, currently Opt. 1 is the only option adopted. Opt. 1 however does not provide quality of predicted Top K beams, where potentially predicted Top K beams include beam(s) with poor probability values. One way to address this is to adopt Opt. 3. However, this may add overhead of including probabilities for each predicted beam. Another approach is to configure a threshold of a probability such that UE report predicted beam(s) only if such predicted beam(s)’ probability(s) exceed the threshold. This may not provide exact probability information, however, gNB could obtain sufficient information to utilize the predicted Top K beam(s).

Similarly, when a regression model is used, a predicted beam with good predicted RSRP with low confidence may not be a good candidate beam to use. Confidence level information could be useful for the gNB to determine whether to use the predicted beam(s). Even when Opt. 2 is used for the regression model, still the gNB may not obtain information what is confidence information of the predicted beams. Opt. 4 can be considered, though it brings up more overhead. For the regression model, we can consider similar approach. For example, UE may be configured with a threshold of the confidence information and may report predicted beam(s) only if such predicted beam(s) exceed the threshold.

- For contents in the report of inference results, consider enhancing Opt.1 and Opt. 2 to allow informing prediction quality (e.g., probability information, confidence information).

- Specify RRC configuration of a threshold for prediction quality and discuss report options based on the threshold.

- In terms of reporting, consider options (1) to include predicted beams, satisfying the threshold, from top K predicted beams; (2) to indicate whether each predicted beam satisfies the threshold.

### BM-Case 2

Agreement

For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable

- FFS: how to determinate reference time for the time instance(s)

- FFS: duration values of the N time instance(s) that can be predicted.

Agreement

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection:

- Option 1: Based on the uplink slot for the report

- Option 2: Based on the CSI reference resource corresponding to the report

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

In RAN1 #118 [3] and #118b [4], agreements below were made for BM-Case 2 inference results.

For BM-Case 2, it was agreed that one report may comprise inference results for N (e.g., N>1) future time instances and FFS on duration values. Based on company proposals on duration values and evaluation results, duration values could be large values such as 40 msec (to a few hundred msec). A duration value used in a report of inference results may have performance impact depending on UE’s mobility. For example, for a low mobility UE, longer duration may be beneficial to cover steady channel conditions with low reporting overhead. For a medium/high mobility UE, shorter duration would be necessary to report sufficiently to reflect varying channel conditions. Some other aspects than mobility may also impact on determining an optimal duration size.

As such an optimal duration value may change dynamically based on UE mobility and potential other aspects. Relying on the gNB configuration based on the gNB prediction/awareness may not work effectively. Such information would be more available to the UE than the gNB. For example, UE may be able to estimate its mobility based on measuring RSs. One way to address this is to allow configuration of multiple candidate duration values, and UE may select one duration value in reporting. For example, in reports, UE can also inform a duration value used for the prediction such that the gNB applies the prediction window properly.

- Support multiple duration values for BM-Case 2 inference report:

- UE select one value among the multiple duration values and inform the value in the report.

When N > 1 is used, another potential issue is about misalignment or a non-negligible gap between reported beam quality and actual beam quality. For example, when N is large (e.g., N = 4) with a duration value of 160 msec, a prediction window becomes 640 msec. During the prediction window, actual channel quality may change considerably (e.g., blockage), and the gNB needs to wait until the next report to acquire updated prediction, which could take minimum the prediction window delay.

To determine whether and how to address this case, firstly, it needs to be clarified that whether the UE can detect this case. This may depend on whether UE can perform prediction whenever necessary e.g., based on Set B of RSs. If the Set B of RSs are periodically transmitted, the UE may continue measuring the periodically transmitted Set B of RSs and obtaining corresponding inference results. In this case, the UE could detect this case e.g., by comparing updated interference results to the reported inference results. If the Set B of RSs are transmitted in an ON/OFF pattern (which still requires standard efforts to support), where the ON period applies to the measurement window of the Set B of RSs and the OFF period applies to the N future time instances, UE may not be able to update inference results during the OFF period. UE may not be able to detect the channel quality change in this case. Depending on the total length of N time instances/durations, this may impact the performance with a sudden channel condition change. We have a proposal as below:

- Specify UE-initiated inference results report mechanism, to override previously reported inference results.

It was agreed to down-select one option from three options for determining the reference time of the earliest time instance. We consider that Option 1 or Option 2 is simpler. On the contrary, Option 3 is a new mechanism for determining the reference time and thus requires further justifications for the potential high volume of standard impacts.

With Option 1, the reference time of the earliest time instance is based on the uplink slot for the report. With Option 2, since the reference time of the earliest time instance is based on the CSI reference signal resource corresponding to the CSI report, timing relationship between the CSI report and the earliest time instance of the prediction window should be clarified. For example, it is undesired that the CSI report carries a time instance (e.g., the earliest time instance) occurring before the CSI report timing, which makes the time instance become obsolete at the reception.

- If Option 2 is considered, the CSI report timing should occur before the earliest time instance of the N time instances.

## Beam indication

For beam indication, it was agreed in RAN1 #116 [2] that the legacy unified TCI state framework will be a starting point for the beam indication of NW-sided model and UE-sided model:

Agreement - For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework - FFS on whether/how potential enhancement is needed

It has been noted by different companies that the potential enhancement for beam indication of a predicted beam is the assumption of QCL properties of a purely predicted TCI state (e.g., unmeasured TCI state). A conservative approach is to assume that UE has no pre-knowledge about QCL properties of an unmeasured TCI state. As such, a second round of beam sweeping of top K TCI states are needed if the top K TCI states include at least one unmeasured TCI state.

This approach may simplify UE implementation but may lead to large overhead and delay to update QCL information to switch active TCI state(s). Not to increase UE complexity significantly, we are open to this approach. Some companies mention this sweeping should be limited to sweep the reported predicted top K beams of Set A. In our view, such a restriction is not necessary. For example, the gNB should be able to activate different beams from the reported predicted top K beams or may activate a subset of top K beams. Either case, beam sweeping may not strictly follow the predicted top K beams.

Observation 1: Restricting a beam sweeping, after predicted top K beam reporting, to the predicted top K beams may not be necessary.

- Further discuss beam sweeping to address QCL assumption for predicted beams. If adopted, consider flexible indication of beams to be swept based on the gNB configuration/indication.

Another approach for addressing QCL assumption is to rely on UE prediction on QCL properties/information for the predicted beam(s). Given complexity and potential need of sharing proprietary information such as Rx beam implementation details of each UE vendor, we consider this approach is not desirable.

To address some potential overhead of beam sweeping with reasonable training complexity and data collection feasibility, we propose to consider a synthesized QCL property of a predicted beam using one or more measured beams/known TCI states. For example, the gNB may indicate a relationship between a Tx beam of Set A (not of Set B) is spatially correlated to one or more Tx beams of Set B. Based on the information, the UE could synthesize a QCL property for the Tx beam based on the QCL properties of the one or more Tx beams of Set B. This would require minimum disclosure of spatial correlations between Set B of Tx beams and Set A of Tx beams. For example, a gNB vendor may be concerned that such indication may disclose the implementation details of Tx beams, but with the agreed Associated ID, UE may already be able to learn such details. As such, there seems to be contradictory trying to protect information that can be obtained. On the other hand, providing such information may be helpful for the gNB/UE to collaboratively obtain the QCL properties without always going through the second round of top K beam sweeping.

- Consider an approach where gNB indicates spatially correlated Tx beams from Set B for each Tx beam of Set A:

- UE may synthesize QCL properties of an unmeasured beam from the QCL properties of its spatially correlated measured beams.

- FFS how to indicate the spatially correlated measured beams of an unmeasured beam.

Another issue of beam indication is specific to BM Case-2. As it has already been agreed that the inference results of BM Case-2 will include predicted beams for N future time instances. Some companies want to keep the current unified TCI framework to indicate a single TCI state for one time instance at a time. This approach not only requires more DCI overhead to change TCI states but also requires TCI application delay every time the TCI state is changed. For example, in case of an uplink heavy scenario, additional DCI overhead may not be negligible. Also, some scenarios such as high speed train, applying different beams in different time instances e.g., based on trajectory/prediction, could improve reception/transmission quality.

- Support to extend unified TCI framework to allow single beam indication for N future time instances.

## Performance monitoring

For performance monitoring, the following agreements were achieved:

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring. Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives FL proposal during RAN1 #119 meeting At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the configuration for monitoring, - The CSI-ReportConfigID of a inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - The resource set(s) for monitoring can be configured for full set of Set A, or a subset of Set A, [or different from Set A]. - For the case that a subset of Set A is configured as the resource set(s) for monitoring, further stuy the following: - Alt 1: Performance metric is calculated (i.e., as valid instance for metric calculation) only if Top-1 or Top-K (FFS all or at least one of Top-K) predicted beam(s) are included in the subset of Set A - Alt 2: For performance metric calculation, with predefined mapping between the full resources for Set A to the subset of set A (with a multi-to-one mapping so that all resources for Set A has a mapped resource for monitoring) - FFS QCL relationship can be used for the mapping, or based on a down-sampling factor - Alt 3: Performance metric is calculated based on the Top K’ predicted beam(s) among the subset of Set A - Note: Top K’ predicted beam(s) among the subset of Set A may or may not be the same as Top K predicted beam(s) reported in the inference report - Alt 4: If the Top K predicted beam(s) is not covered by the resource set(s) for monitoring, the reported result(s) is counted as inaccuracte. - Alt 5: gNB ensures to the Top K predicted beam(s) reported in the inference report is always covered by the resource set(s) for monitoring. - Note1: No spec impact - Note2: This does not preclude to restrict the inference result(s) in a subset of Set A. - Other alternatives are not precluded. - For the case that a differet set of Set A is configured as the resource set(s) for monitoring, support: - Alt 2: For performance metric calculation, with predefined mapping between the full resources for Set A to the resources for monitoring (with a multi-to-one mapping so that all resources for Set A has a mapped resource for monitoring) - FFS QCL relationship can be used for the mapping, or based on a down-sampling factor

For the monitoring Type 1 Option 2 of UE-side model monitoring, the beam prediction accuracy was agreed as a performance metric of the performance monitoring. Calculation of the beam prediction accuracy uses many (e.g., tens or hundreds of) samples, where each sample is based on measurements on the Set A reference signals and the Set B reference signals. Transmission/reception of reference signals corresponding to the full Set A is a huge burden for both the gNB and the UE. It is reasonable to consider using a subset of Set A for the performance monitoring. Then the question becomes how to properly use measurements on the subset of Set A to obtain an accurate performance metric (e.g., with small bias to the performance metric obtained using the full Set A). Five alternatives were listed during RAN1 #119 discussions. Among these alternatives, Alt 1 or Alt 5 seem very restrictive, where the performance monitoring becomes effective only when Top-1 or Top-K is included in the subset of Set A. Alt 4 may lead to undesirable performance metric.

- Support using a subset of Set A beams for performance monitoring. Regarding how to use the subset, at least further consider Alt 3 (e.g., use Top K’ predicted beams among the subset) and Alt 2 (e.g., with predefined mapping between the full resources for Set A to the subset of set A) for calculation of the performance metric.

For FFS: whether to support all the combinations on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report, we can firstly consider aperiodic CSI reporting as the baseline for discussion, especially for the beam prediction accuracy which requires a larger number of reference signal resources. For example, the gNB may occasionally trigger an aperiodic CSI report requesting reporting of the beam prediction accuracy for thorough performance monitoring.

- Support at least aperiodic CSI reporting of beam prediction accuracy for performance monitoring.  FFS on semi-persistent and periodic.

For functionality management, RAN2 agreed that the “network decision, network-initiated” AI/ML management is supported as a baseline, where the network (e.g., gNB) makes decisions on model switching, implying that the UE may have multiple AI/ML models associated with one AI/ML functionality. To support model switching (e.g., when performance of the active model is poor), performance monitoring of inactive model(s) would be also necessary. Otherwise, the gNB may not be able to select the best one to switch.

- In the performance monitoring framework, consider monitoring on multiple models associated with one AI/ML functionality.

## Priority rules and CSI processing

In RAN1#118bis, below has been agreed for counting CPUs:

Agreement For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

In legacy standards, a priority value of a CSI report is determined based on at least the time domain behavior of the CSI report and the report quantity in the CSI report. Even though it has been agreed that the AI/ML-based report for BM-Case 1 or BM-Case 2 reuses the legacy CSI reporting framework, changes such as new report quantities for inference results or performance metrics carried by the AI/ML-based report are expected. Thus, the corresponding priority rules should be updated for the AI/ML-based report.

- Define priority rules between AI/ML-based report and non-AI/ML-based report, and among multiple types of AI/ML-based reports, considering:

- Different use cases (or sub use cases) such as BM Case-1, BM Case-2, or CSI compression.

- Different LCM purposes (e.g., reporting inference results, reporting performance monitoring results, and so on) of an AI/ML model.

- Different report quantity types (e.g., predicted CRI/SSBRI, predicted RSRP, beam prediction accuracy, one-shot performance indicator, and so on).

According to the agreement, the existing CSI processing unit (CPU) mechanism is used as a starting point for the AI/ML-based CSI processing. Nonetheless, the number of CPUs and the corresponding CPU occupancy time need to be defined for both model inference by an AI/ML model and performance monitoring of an AI/ML model. For example, determination of the CPU occupancy time of the AI/ML-based CSI reporting could be different from the legacy CSI reporting. For example, model inference of BM-Case 2 will be performed during a long observation window. Calculation of performance metric of beam prediction accuracy would use measurement samples across a long monitoring window. Thus, it is necessary to at least enhance CPU definition and CPU occupancy time definition for the AI/ML-based CSI reporting.

- Enhance CPU and CPU occupancy considering a long observation window for performance monitoring and inference.

# Conclusion

In this contribution, we make the following proposals and observations.

- Support UE requests to initiate/terminate data collection for a UE-sided model:

- RAN1 to discuss conditions to trigger UE requests to initiate data collection (e.g., for training a new UE-sided model to potentially replace the current UE-sided model deployed for inference).

- RAN1 to discuss events to trigger UE requests stopping data collection (e.g., indicating the model training is completed in case UE performs the training (local refining)).

- For contents in the report of inference results, consider enhancing Opt.1 and Opt. 2 to allow informing prediction quality (e.g., probability information, confidence information).

- Specify RRC configuration of a threshold for prediction quality and discuss report options based on the threshold.

- In terms of reporting, consider options (1) to include predicted beams, satisfying the threshold, from top K predicted beams; (2) to indicate whether each predicted beam satisfies the threshold.

- Support multiple duration values for BM-Case 2 inference report:

- UE select one value among the multiple duration values and inform the value in the report.

- Specify UE-initiated inference results report mechanism, to override previously reported inference results.

- If Option 2 is considered, the CSI report timing should occur before the earliest time instance of the N time instances.

Observation 1: Restricting a beam sweeping, after predicted top K beam reporting, to the predicted top K beams may not be necessary.

- Further discuss beam sweeping to address QCL acquisition for predicted beams. If adopted, consider flexible indication of beams to be swept based on the gNB configuration/indication.

- Consider an approach where gNB indicates spatially correlated Tx beams from Set B for each Tx beam of Set A:

- UE may synthesize QCL properties of an unmeasured beam from the QCL properties of its spatially correlated measured beams.

- FFS how to indicate the spatially correlated measured beams of an unmeasured beam.

- Support to extend unified TCI framework to allow single beam indication for N future time instances.

- Support using a subset of Set A beams for performance monitoring. Regarding how to use the subset, at least further consider Alt 3 (e.g., use Top K’ predicted beams among the subset) and Alt 2 (e.g., with predefined mapping between the full resources for Set A to the subset of set A) for calculation of the performance metric.

- Support at least aperiodic CSI reporting of beam prediction accuracy for performance monitoring. FFS on semi-persistent and periodic.

- In the performance monitoring framework, consider monitoring on multiple models associated with one AI/ML functionality.

- Define priority rules between AI/ML-based report and non-AI/ML-based report, and among multiple types of AI/ML-based reports, considering:

- Different use cases (or sub use cases) such as BM Case-1, BM Case-2, or CSI compression.

- Different LCM purposes (e.g., reporting inference results, reporting performance monitoring results, and so on) of an AI/ML model.

- Different report quantity types (e.g., predicted CRI/SSBRI, predicted RSRP, beam prediction accuracy, one-shot performance indicator, and so on).

- Enhance CPU and CPU occupancy considering a long observation window for performance monitoring and inference.

# References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#102, Dec. 2024

- RAN1 #116 chair notes, 3GPP RAN1, February 2024

- RAN1 #118 chair notes, 3GPP RAN1, August 2024

- RAN1 #118b chair notes, 3GPP RAN1, October 2024

- RAN1 #119 chair notes, 3GPP RAN1, November 2024

- R2-2411152, Reply LS to SA2 on AIML data collection, 3GPP RAN2, Nov 2024

- R1-2410736, FL summary #3 for AI/ML in beam management, Moderator (Samsung), November 2024