3GPP TSG RAN WG1 #120                               		                           	   R1-2500528

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item:	9.10.1

Source:	InterDigital, Inc.

Title:	Discussion on enabling TX/RX for XR during RRM measurements

Document for:	Discussion

# Introduction

The Rel-19 Revised WID on XR Enhancements for NR has the following objective [1]:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

The agreements made in RAN1#119 [2] are provided in the Appendix. In this contribution, we discuss the enhancements for enabling transmissions/receptions during measurement gaps and scheduling restrictions due to intra/inter-frequency measurements.

# Discussion

When configured with MG, the UE is not expected to perform any transmissions or receptions of other signals/channels (e.g. UL-SCH, DL-SCH, SR, HARQ-ACK). The UE is also not expected to monitor PDCCH in the monitoring occasions overlapping with MG occasions. As such, the resulting restrictions for scheduling can have major impact on data transmissions and capacity, especially when transmitting or receiving XR data with tight delay budgets. Delaying the data transmissions until after the MG duration can result in not meeting the QoS.

## Skipping of colliding/overlapping gap occasions

In the last RAN1#119 meeting, the issue of how to handle DCI-based skipping when configured with concurrent/colliding gaps was briefly discussed but no conclusion/agreement was made due to ongoing study on applicable gaps in RAN4.

In legacy [38.133, clause 9.1.8.3], when the UE is configured with concurrent measurement gaps (i.e. gaps with assigned priority), two MG occasions are considered colliding if at least one of the following conditions are met:

- the two occasions are fully or partially overlapping in time domain, or

- the distance between the two occasions is equal to or smaller than 4 ms.

In case of collision between two measurement gap occasions, the UE performs measurements in the occasion of the measurement gap with higher priority, and the occasion of the measurement gap with lower priority is dropped. Also, the requirements of concurrent measurement gaps do not apply when a gap without assigned priority is configured simultaneously with any other gap(s) that affect serving carriers in the same FR and the measurement gaps are colliding with each other [3].

In the last meeting, RAN4 agreed on a way-forward [5] with following types of measurement gaps are under consideration:

Issue 2-2: Types of measurement gaps <Way forward>: • In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG.

As the RAN4 discussion on the types of measurement gaps is ongoing, which also considers concurrent gaps, RAN1 can continue to discuss on how DCI-based skiping indication applies to gap occasions that collide or overlap in time domain. Note that in legacy, the collsion between gap occasions can apply to both R18 concurrent gaps (i.e. gaps with assigned priority) and non-concurrent gaps (i.e. gaps without assigned priority).

Observation 1: In legacy, collsion between gap occasions can apply to both R18 concurrent gaps (i.e. gaps with assigned priority) and non-concurrent gaps (i.e. gaps without assigned priority).

When the UE is configured with gaps that collide/overlap in time domain, there is a possibility for the two gap occasions belonging to different MG configurations to fully/partially overlap in time domain or the distance between two consecutive gap occasions is equal to or less than 4ms.

The RAN4 LS response [6] indicates a minimum time offset of X = 5ms (baseline) and X = 3ms (with optional UE capability) between the DCI-based skiping indication and the gap occasion to be skipped. In this regard, if the 1-bit skipping indication in DCI is intended for a later/second gap occasion than a earlier/first gap occasion there may be an ambiguity at UE on the applicability of the DCI indication. Given the relatively long time offset between the DCI-based skiping indication and the gap occasion to be skipped, the possibility of colliding increases with an ambiguity on whether the skipping applies to the first, second or both gap occasions when the UE is configured with multiple MG configurations. This is also because the gNB may not have an opportunity to indicate skipping in DCI if the skipping is intended for a later/second gap occasion that may be within 4ms from an earlier/first gap occasion.

Observation 2: Given the relatively long time offset (i.e. X = 5ms) between the DCI-based skiping indication and the gap occasion to be skipped, the possibility of colliding increases with an ambiguity on whether the skipping applies to the first, second or both gap occasions when the UE is configured with multiple MG configurations

Regarding the applicability of DCI-based skipping to colliding/overlapping gap occasions, there can be two cases to consider:

- Case 1: Both colliding gaps are of R18 concurrent gap type (i.e. gaps with assigned priority)

- Case 2: At least one of the colliding gaps is of non-concurrent gap type (i.e. gaps without assigned priority or all gaps have equal priority)

For Case 1, following the legacy approach to resolve the collision before applying the DCI-based skipping to the remaining gap occasion is simpler and has lower spec impact. Doing the contrary (e.g. skip before resolving collision) can result in skipping a high priority gap occasion and retaining a low priority gap occasion, which is not preferred. As such, we propose the following:

Proposal 1: For colliding gaps with assigned priority (e.g. R18 concurrent gaps), support UE applying the DCI-based skipping indication to the remaining gap occasion(s) after resolving the collision between the gap occasions

For Case 2, skipping the first/earlier gap occasion is most straightforward since at least one occasion is available for data scheduling and another for RRM measurements. Skipping both gaps may result in compromising measurements. Also, if the NW intends the UE to skip the later gap occasion, it could implicitly indicate the skipping in the scheduling DCI, e.g. by providing a resource that overlaps with the later occasion.

Proposal 2: For colliding gaps without assigned priority, support UE applying the DCI-based skipping indication to the first gap occasion

## DCI-based skipping of gaps/restrictions

In RAN1#118 meeting, the following working assumption was made on enabling data Tx/Rx in gaps/restrictions based on explicit indication in DCI:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561.

In the last meting, the RAN4 LS response [6] confirms that Alt 1-1 is feasible from RAN4 perspective. It was also mentioned that per RAN4 understanding the time offset is the time from the end of the last symbol of the PDCCH carrying the DCI to the start of the corresponding skipped measurement gap indicated by the DCI, where the start of the measurement gap is determined after applying Measurement Gap Timing Advance (mgta) configured by the network according to TS 38.331.

Another related aspect discussed in the last meeting is on whether further clarification is needed on which gap/restriction occasion to skip when receiving the explicit indication in DCI. When receiving the DCI the UE should be aware of the applicable gap occasion and apply the skipping without any ambiguity. Given the ongoing issue on colliding/overlapping gaps on whether the skipping applies to the first, second or both gap occasions (discussed in previous section), there is benefit for including a clarification under Alt 1-1 in the WA above. In our view, indicating “the bit in the DCI is used to indicate whether to skip the first applicable gap/restriction occasion” provides sufficient clarification and resolves any ambiguity regarding the applicability of the skipping indication in DCI. Given this reasoning and the RAN4 LS response, we propose the following:

Proposal 3: Confirm the working assumption to support explicit indication by DCI to skip a particular gap/restriction with the following change:

- The bit in the DCI is used to indicate whether to skip the first applicable gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

# Conclusion

In this contribution, the following observations are made:

Observation 1: In legacy, collsion between gap occasions can apply to both R18 concurrent gaps (i.e. gaps with assigned priority) and non-concurrent gaps (i.e. gaps without assigned priority).

Observation 2: Given the relatively long time offset (i.e. X = 5ms) between the DCI-based skiping indication and the gap occasion to be skipped, the possibility of colliding increases with an ambiguity on whether the skipping applies to the first, second or both gap occasions when the UE is configured with multiple MG configurations

The following proposals are made:

Proposal 1: For colliding gaps with assigned priority (e.g. R18 concurrent gaps), support UE applying the DCI-based skipping indication to the remaining gap occasion(s) after resolving the collision between the gap occasions

Proposal 2: For colliding gaps without assigned priority, support UE applying the DCI-based skipping indication to the first gap occasion

Proposal 3: Confirm the working assumption to support explicit indication by DCI to skip a particular gap/restriction with the following change:

- The bit in the DCI is used to indicate whether to skip the first applicable gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

# Appendix: Agreements on enabling Tx/Rx during RRM measurements

RAN#119 Agreements

Agreement For explicit indication by DCI, bit value equal to “1” indicates the corresponding gap/restriction occasion is to be “skipped”. - FFS: Interpretation of bit value equal to “0”. Agreement A component of an FG can have Option 1 as a first value and Option 2 as a second value: - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. Note: UE indicates only one value of this component. Agreement For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field) Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

RAN#118bis Agreements

Agreement At least for self-scheduling case, for explicit indication by DCI to skip a particular gap/restriction, one bit indication is included as part of DCI formats 0_1/1_1 and 0_2/1_2. - FFS: Cross carrier scheduling. - FFS: DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. Agreement Clarify the note from previous agreement as follows: - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. Agreement In case of cross carrier scheduling, for explicit indication by DCI to skip a particular gap/restriction, one bit indication included as a part of DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell. Agreement From RAN1 perspective, the case where an occasion(s) of gap/restrictions that is caused by RRM measurements is cancelled/skipped partially is not supported in Release 19.

RAN#118 Agreements

Agreement For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select one among the following options: Option 1: Support Alt. 1-1: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s); - Indication is included as part of scheduling DCI: - FFS: Bit-field size is one bit; - FFS: Bit-field size is >1 bit; - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. - FFS: DCI format, DCI content, DCI bit-field size; - FFS: Whether indication is for one or more occasions; - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped. Option 2: Support Alt. 3-1: - Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions; - FFS: Details of pattern: - FFS: Pattern is based on periodicity, offset and duration; - FFS: Pattern is based on a bitmap; - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561.

RAN#117 Agreements

Agreement For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements, select one or combination among only Alt1 and Alt3 from RAN1#116bis. Proposal 2.1.2-v7 For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements consider the following alternatives or combinations for further down-selection: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - FFS: Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s); - Indication is included as part of scheduling DCI: - FFS: Bit-field size is one bit; - FFS: Bit-field size is >1 bit; - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. - FFS: Alt 1-2: Explicit indication by DCI to indicate a time window where to skip a particular gap(s)/restriction(s); - Note: Minimum time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion in time window that is going to be skipped shall be introduced. - FFS: Alt 1-3: Implicit indication by DCI scheduling a transmission/reception overlapping with a gap(s)/restriction(s) to skip the gap(s)/restriction(s); - Note: Minimum time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. - FFS: DCI format, DCI content, DCI bit-field size; - FFS: Whether indication is for one or more occasions; - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped. - Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - FFS: Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions; - FFS: Details of pattern: - FFS: Pattern is based on periodicity, offset and duration; - FFS: Pattern is based on a bitmap; - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions. - FFS: Alt 3-3: Gaps/restrictions that are caused by RRM measurements are skipped if collided with particular semi-statically pre-configured Tx/Rx occasions. - FFS: Alt. 3-4: Gaps/restrictions that are caused by RRM measurements are skipped based on semi-statically configured priority information for particular semi-statically pre-configured Tx/Rx and/or particular gaps/restrictions. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

RAN#116bis Agreements

Proposal 2.1.2-v6: For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements consider the following alternatives or combinations for further down-selection: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - FFS: Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s); - FFS: Alt 1-2: Explicit indication by DCI to indicate a time window where to skip a particular gap(s)/restriction(s); - FFS: Alt 1-3: Implicit indication by DCI scheduling a transmission/reception overlapping with a gap(s)/restriction(s) to skip the gap(s)/restriction(s); - FFS: DCI format, DCI content, DCI bit-field size; - FFS: Whether indication is for one or more occasions; - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped. - Alt. 2: Semi-persistent solution to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. - FFS: Alt 2-1: gNB sends a skipping activation command, UE will skip gaps/restrictions until de-activation command is received. - FFS: Alt 2-1a: gNB sends an activation command to enable pre-configured gap(s)/restriction(s), UE will skip gap(s)/restriction(s) after de-activation command is received. - FFS: Alt 2-2: RRM measurement adaptation is applied to all MG configurations/scheduling restrictions due to all SMTC configurations, or is applied to selected MG configuration(s) and/or scheduling restrictions due to selected SMTC configuration(s) and is conducted in a time-window, and time-windows are derived from a semi-persistent configuration activation for their periodicity, offset and duration. - FFS: Alt 2-3: Activate/de-activate one or more of pre-configured pattern(s) via MAC-CE to indicate occasions where Tx/Rx is prioritized over gap(s)/restriction(s); - FFS: Details of activation/deactivation MAC-CE command - FFS: How to consider time offset between activation/deactivation command and start of gap(s)/restriction(s) occasion that is going to be skipped. - Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - FFS: Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions; - FFS: Details of pattern - FFS: Alt 3-2: Gaps/restrictions skipping is applied to all MG configurations/scheduling restrictions due to all SMTC configurations / RRM measurements, or is applied to selected MG configuration(s) and/or scheduling restrictions due to selected SMTC configuration(s) / RRM measurement(s) and is conducted in a time-window, and time-windows are derived from a semi-static configuration for their periodicity, offset and duration. - FFS: Alt 3-3: Gaps/restrictions that are caused by RRM measurements are skipped if collided with particular semi-statically pre-configured Tx/Rx occasions. - FFS: Alt. 3-4: Gaps/restrictions that are caused by RRM measurements are skipped based on semi-statically configured priority information for particular semi-statically pre-configured Tx/Rx and/or particular gaps/restrictions. Agreement Confirm the working assumption from RAN1 #116 with updates: - RAN1 aims to develop/identify solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements agnostic in RAN1 normative work to types of gaps/restrictions that are caused by RRM measurements. - It is up to RAN4 to discuss which type of gaps/restrictions caused by RRM measurements can be cancelled/skipped - Note: UE features related to the developed solution(s) is a separate discussion Proposal 2.3.2-v3: RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information.

RAN#116 Agreements

Agreement Consider at least solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. - FFS: Other types of solutions. - Whether or not/how to account for any UE assistance information/indication in addition to other information available at the network Agreement From RAN1 perspective, when an occasion(s) of gaps/restrictions that are caused by RRM measurements are cancelled/skipped fully, UE is assumed to receive/transmit in the gaps/restrictions that are caused by RRM measurements as it would without any (measurement etc. related) gaps/restrictions that are caused by RRM measurements. - FFS: Whether or not/How to support of the case where an occasion(s) of gap/restrictions that are caused by RRM measurements are cancelled/skipped partially Proposal 2.6-2: For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements consider at least one of the following alternatives or combinations for further down-selection: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - FFS: details - Alt. 2: Semi-persistent solution to deactivate/ and/or re-activate one or more of gaps/restrictions that are caused by RRM measurements and to enable Tx/Rx during the deactivated in gaps/restrictions that are caused by RRM measurements. - FFS: details - Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - FFS: details - Alt. 4: Dynamic solution to adapt/change gap/SMTC configuration to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - FFS: details - Alt. 5: Rule-based solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements: - FFS: details Companies are encouraged to use the EVM in TR38.835 if they are submitting simulation results. Working Assumption RAN1 aims to develop/identify solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements agnostic in RAN1 normative work to types of gaps/restrictions that are caused by RRM measurements. Note: UE features related to the developed solution(s) is a separate discussion.

# Reference

[1]	RP-241771, Revised WID: XR (eXtended Reality) for NR Phase 3, Sep 2024

[2]	Chair notes, RAN1#119, Nov 2024

[3] 	3GPP TS 38.133, Requirements for support of radio resource management (Release 18), V18.4.0, Dec 2023

[4]	3GPP TR 38.835, Study on XR enhancements for NR (Release 18), V18.0.1, Apr 2023

[5]	R4-2420103, WF on RRM requirements for XR_Ph3, Nov 2024

[6]	R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements, Nov 2024