3GPP TSG RAN WG1 #120		R1-2500550

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.2.2

Source:	Google

Title:	CSI Enhancement for NR MIMO

Document for:	Discussion/Decision

# Introduction

In RAN1 #119 meeting, the following agreements on CSI enhancement were achieved.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, the 1-bit indicator per CSI trigger state applies to all the NTRP configured CSI-RS resources. Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, the timeline (Z/Z’) is determined as Z/Z’ associated with the Rel-18 eType-II CJT, plus Drelax - The value of Drelax is a UE capability, taken from {0, drelax} - FFS: The value of drelax (>0), including whether it depends on SCS - For linking CJTC Dd and Rel-18 eType-II CJT CSI, joint triggering is a separate UE feature group from separate triggering Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, if the UCI associated with each of the M CRIs comprises two parts and is multiplexed on PUCCH, the UE determines the PUCCH resource, the number of PRBs for the PUCCH resource, assuming that the CSI associated with each of the M reported CRIs corresponds to rank 1 Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, regarding linking the CMRs in the two CSI Report Settings, for separate triggering, the UE expects that the number of configured CSI-RS resources associated with the Rel-18 eType-II CJT CSI is always the same as the number of TRS resource sets associated with the Rel-19 CJTC Dd report Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI reported in CSI part-1, if resource-specific RI restriction is configured, zero padding bits are introduced in CSI part 1 for each of the (M-MR) reported CRIs: - For a k-th CRI from the (M-MR) reported CRIs (where k=1, 2, …, (M-MR)), OP,k zero padding bits are inserted where: - OP,k=Nmax-Nreported,k - Nmax=maxi∈S(Ni), where Ni is the size of RI field corresponding to the i-th CRI, and S is the set of CRIs corresponding to the (Ks-MR) resources - Nreported,k is the size of RI field corresponding to the k-th CRI Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, when a single CSI trigger state associated with >1 Rel-18 Type-II CJT CSI reports, the 1-bit indicator per CSI trigger state applies to all the Rel-18 Type-II CJT CSI reports linked with the CJTC Dd report(s). Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s): - The scaling formula is ρ.si where ρ is a multiplicative factor independent of i - Reuse legacy precoder normalization (per discretion of the spec editor) - FFS (RAN1#119): Whether min(ρ.si , 1) operation is needed - FFS (RAN1#119): Whether ρ other than 1 (baseline) is needed (e.g. v or 1v) - Regarding the configuration of the si value (3-bit indicator per SD basis vector group), decide, by RAN1#119, between the following: - Alt1. RI=1 and RI=2 are separately configured (RI-specific) - Alt2. A same configuration is used for RI=1 and RI=2 (RI-common) Conclusion For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding packing order and priority levels of NRep CSI reports with M CRIs, the UCI packing order and the consecutive priority levels (higher to lower) follows the legacy specifications. Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding wideband P/SP-CSI reported using one-part CSI, if resource-specific RI restriction is configured, the zero padding bits for each of the M reported CRI are determined as follows: - For a k-th CRI from the M reported CRIs, OP,k=Nmax-Nreported,k, where: - Nmax= maxj∈Q (Nmax,j) , where Q is the set of CRIs corresponding to Ks resources and Nmax,j is the maximum payload size of associated CSI fields for a j-th CRI, and Nmax,j=NRI(j)+maxrj∈Srank,jB(rj), where Srank,j is the set of rank values rj that are allowed to be reported for the j-th CRI; - Nreported,k= NRI(k)+B(Rk) , where Rk is the reported rank for k-th CRI; - Note: NRI(k) is the size of RI field corresponding to k-th CRI. - Note: The definition of the operator B(∙) is as legacy (as defined in 38.212). Note: Here k, j=1, 2, …, KS Note: How this is captured in the spec (including exact formulation) is up to the editor(s). Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding aperiodic CSI-RS resource configuration, an RRC-configured resource-level slot offset (relative to the resource-set-level slot offset, using the same design as Rel-19 Type-I/II codebook refinement for 48, 64, and 128 ports) is supported for aperiodic CSI-RS resource set - FFS: The number of bits for indicating the resource-level slot offset (relative to the resource-set-level slot offset) for KS resources, including the value(s) of the slot offset - FFS: Whether, in addition, configuring an available slot offset for each CSI-RS resource within the aperiodic CSI-RS resource set - Note: “Available slot offset” is analogous to the Rel-17 SRS triggering offset enhancement Agreement For the Rel-19 Type-I SP codebook refinement for P=48, 64, 128 CSI-RS ports, when the UE reports or multiplexes the CSI that include Part 2 CSI reports on PUCCH, the PUCCH resource, the number of PRBs for the PUCCH resource, and/or the number of Part 2 CSI reports are determined based on the RI value that results in the largest UCI payload. - For Scheme-A, the RI value is: - 8 when the RI value(s) allowed by the configured RI restriction per CSI reporting configuration include 5, 6, 7, and/or 8, - 1 otherwise - For Scheme-B, the RI value is 4 Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint triggering carried on a same PUSCH (hence on a same slot), the UCI associated with the CJTC Dd report is multiplexed in CSI Part 1 - The previously agreed UCI design and mapping order for CJTC Dd report are reused - The legacy UCI design, UCI mapping order, and UCI omission for the Rel-18 eType-II CJT CSI are reused Note: The above proposal reuses the legacy UCI design principles, where the UCI associated with the CJTC Dd is placed in the part of UCI as TS 38212 Table 6.3.1.1.2-13; the CSI part 1 of Rel-18 eType-II CJT CSI is placed in the part of UCI as TS 38.212 Table 6.3.1.1.2-13 and the CSI part 2 of Rel-18 eType-II CJT CSI is placed in the part of UCI as TS 38.212 Table 6.3.1.1.2-14 Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the UE assumes that the NTRP CSI-RS resources are transmitted without DL/UL switching in between the NTRP resources Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s), ρ=1 - Note: In this case, the min(si,1) operation is not needed Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors for the 3-bit scaling factor(s), the configuration of the si value (3-bit indicator per SD basis vector group) is RI-common (a same configuration is used for RI=1 and RI=2) Conclusion For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports with the Rel-15 Type-I SP codebook, joint operation with the Rel-18 NES framework is not supported.

In this contribution, we provide some discussion on the CSI enhancement for NR MIMO.

# Type1 codebook for 128 ports

## Report quantity configuration

Since all the CSI-RS resources in the CSI-RS resource set for channel measurement are used for one CSI measurement, it is unnecessary for the UE to report the CRI. Thus, to distinguish from the legacy Type1 codebook based CSI report based on multiple CSI-RS resources with CRI reported, the report quantity for the CSI feedback for up to 128 ports should be based on “RI-PMI-CQI” or “RI-PMI-CQI-LI”.

Proposal 1: When the NW configures a Type1 codebook with more than 32 ports, support the NW configures the reportQuantity based on “RI-PMI-CQI” and “RI-PMI-CQI-LI” to distinguish from the legacy Type1 codebook based CSI report based on multiple CSI-RS resources with CRI reported.

## PD-NES and Type1 SD-NES

In RAN1 #118 meeting, the Type1 SD-NES for up to 128 ports based CSI has been agreed. In RAN1 #118b meeting, the following is agreed in Rel-18 maintenance for NES. Since for CSI measured based on different number of ports, the EPRE ratio between CSI-RS and PDSCH can be different, it is necessary to support PD-NES in addition to the Type1 SD-NES.

Agreement UEs supporting Type 1 SD is mandated to support feature group 42-6 (joint type 1 SD + PD). Update UE feature description accordingly.

Proposal 2: Support PD-NES for CSI report based on up to 128 ports to be aligned with Rel-18 NES principle that UE should support both Type1 SD and PD.

# Inter-TRP time/frequency offset report for CJT

## CMR configuration

In RAN1 #116b, the following on CMR configuration has been agreed.

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or ‘cjtc-F’ (frequency offset), periodic TRS (‘CSI-RS for tracking’) resource set is used for each of the NTRP NZP CSI-RS resource sets - Extend the maximum allowed number of TRS resource sets to 4 (note: legacy supports max. 3 from Rel-18 TDCP) - FFS: Whether all the resources across the NTRP TRS resource sets are configured with the same bandwidth - FFS: Whether aperiodic TRS resource set can also be used - FFS: Whether CSI-RS for CSI can also be used - FFS: Whether different RE locations (FDM) are supported for the RSs - FFS: additional time separation between RSs - FFS: The exact number of CSI-RS resource(s) within each TRS resource set - FFS: applicable type(s) if joint reporting of both Doffset/d and FO is supported Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), single-port CSI-RS(s) for CSI is used - FFS: Whether multi-port CSI-RS for CSI can also be used - FFS: Whether all the ‘CSI-RS for CSI’ resources within each resource set follow the legacy pre-Rel-19 rules of CSI-RS resources associated with a same resource set, and whether only 1 or NTRP >1 resource sets are used - FFS: The exact number of CSI-RS resource(s) within each resource set - FFS: Whether different RE locations (FDM) are supported for the RSs - FFS: additional restrictions e.g. time separation between RSs, bandwidth

For DO and FO report, the remaining issue is the additional constraints for the TRSs configured as CMR. For TRS, a few constraints have already been defined. The DO and FO measurement do not require cross-TRS measurement. But for single DO or FO report, it is better that the NW can configure whether the TRSs are based on pre-compensation of FO or DO or not. Then it could simplify the UE complexity.

Proposal 3: For the TRSs configured for DO report, support the NW configure the TCI states for the TRSs based on the following schemes:

- Scheme 1 (Without pre-compensation of FO): Separate TCI state for each TRS

- Scheme 2 (With pre-compensation of FO): All the TRSs are configured with a common TCI state for QCL-TypeC indication without average delay, and each TRS is configured with a separate TCI state for QCL-TypeC indication without Doppler shift

Proposal 4: For the TRSs configured for FO report, support the NW configure the TCI states for the TRSs based on the following schemes:

- Scheme 1 (Without pre-compensation of DO): Separate TCI state for each TRS

- Scheme 2 (With pre-compensation of DO): All the TRSs are configured with a common TCI state for QCL-TypeC indication without Doppler shift, and each TRS is configured with a separate TCI state for QCL-TypeC indication without average delay

For PO report, single-port CSI-RS resources can be configured. To maintain the same measurement accuracy as DO/FO report, similar constraints as TRS can be considered, thus the minimum bandwidth for the single-port CSI-RS should be min(52, NBWP) RBs, where NBWP is the number of RBs for the active DL BWP. The frequency domain density can be 3 REs/RB. In addition, the NW can configure the TCI state for the single-port CSI-RSs to tell the UE whether the DO/FO pre-compensation is performed. However, currently, only QCL-TypeB is allowed to be configured for CSI-RS for CSI when QCL-TypeD is not applicable. Then firstly the QCL-TypeC should be allowed for the QCL indication for the CSI-RS. Then the NW can configure whether the DO/FO is pre-compensated by configuring whether the CSI-RSs share a common average delay or Doppler shift.

Proposal 5: For PO report, support the following constraints for the single-port CSI-RS resources configured as CMR:

- The minimum bandwidth for the single-port CSI-RS should be min(52, NBWP) RBs, where NBWP is the number of RBs for the active DL BWP

- The frequency domain density should be 3 REs/RB

Proposal 6: For the single-port CSI-RS resources configured for FO report, support the NW configure the TCI states for the CSI-RS resources based on the following schemes:

- Scheme 1 (Without pre-compensation of DO/FO): Separate TCI state with QCL-TypeC indication for each CSI-RS resource

- Scheme 2 (With pre-compensation of DO): All the CSI-RS resources are configured with a common TCI state for QCL-TypeC indication without Doppler shift, and each CSI-RS resource is configured with a separate TCI state for QCL-TypeC indication without average delay

- Scheme 3 (With pre-compensation of FO): All the CSI-RS resources are configured with a common TCI state for QCL-TypeC indication without average delay, and each CSI-RS resource is configured with a separate TCI state for QCL-TypeC indication without Doppler shift

- Scheme 4 (With pre-compensation of DO and FO): All the CSI-RS resources are configured with a common TCI state for QCL-TypeC indication

## Associated SRS ports for PO measurement

In RAN1 #118b meeting, the following on associated SRS ports for PO measurement has been agreed.

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource, - Confirm the following working assumption as agreement: “the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic” - When periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report - FFS: Whether ‘the earliest SRS transmission occasion after the NTRP CSI-RS occasions’ is also supported as an option - FFS: Whether determination of SRS transmission occasion is needed for aperiodic associated SRS resource, and if so, how

An open issue is whether and how to define the associated SRS transmission occasion for aperiodic SRS resource. Similar to P/SP SRS resource, the aperiodic SRS can be transmitted multiple times, which are triggered by multiple DCIs. Then it is necessary to define the associated SRS transmission occasion for aperiodic SRS. One possible way is to reuse the definition for P/SP SRS. Thus, for aperiodic SRS, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report.

Proposal 7: For aperiodic SRS, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report.

A second issue is that when the NW configures multiple SRS resource sets, how to identify the associated SRS resource and associated SRS port. There can be the following possible cases:

- Case 1: NW configures one periodic SRS resource set and one aperiodic SRS resource set

- Case 2: NW configures two semi-persistent SRS resource sets and activates one of them

- Case 3: NW configures two aperiodic SRS resource sets

For case 1 and case 2 above, a simple way is to select the associated SRS resource set based on the time-domain behavior and activation status. For case 3, a simple way is to select the associated SRS resource set based on the SRS resource set ID.

Proposal 8: Support the UE to select the associated SRS resource set for PO report as follows

- When one periodic SRS resource set and one aperiodic SRS resource set are configured, UE selects the associated SRS resource based on the periodic SRS resource set

- When two semi-persistent SRS resource sets are configured, UE selects the associated SRS resource based on the activated SRS resource set

- When two aperiodic SRS resource sets are configured, UE selects the associated SRS resource based on the SRS resource set with lower set ID

## Delay offset impact on PO measurement

The delay offset (DO) could also cause phase shift in frequency domain, where the phase shift could be different for different subcarriers. Then one possible way is that the NW can perform the DO pre-compensation to transmit the CSI-RS resources for PO measurement. In this case, the UE only needs to perform the PO measurement. Another possible way is that the NW does not perform the DO pre-compensation for the CSI-RS resources for PO measurement. Instead, the UE performs the DO measurement and compensation first. Then it calculates the PO based on the CSI-RS resources for PO measurement after DO compensation. Therefore, with regard to different UE behaviors and UE complexity, the NW can inform the UE whether it transmits the CSI-RS resources for PO measurement with DO pre-compensation.

Proposal 9: Support the NW to inform UE whether it transmits the CSI-RS resources for PO measurement with DO pre-compensation.

## Linked DO and CJT-CSI report

In RAN1 #118b meeting, the following on linked DO and CJT-CSI report have been agreed.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, confirm the following working assumptions as agreement with the following refinement: - When separately triggered, the delay offset value to be compensated is the latest reported delay offset (DO) whose reporting instance’s last symbol is before the first symbol of DCI triggering of the CJT CSI reporting - When jointly triggered, the delay offset value to be compensated is the reported delay offset (DO) in the same reporting instance FFS: Whether an additional UE procedure is needed when the reported DO value is ‘out of range’ FFS: Whether the Dd report codepoints need to be reinterpreted from intervals/ranges to values when the linkage mechanism is configured, or in general, a rule is needed to determine the value of DO for the compensation for each CMR for CJT CSI report (including if multiple Dos are reported)

One open issue is regarding the invalid DO report. If UE reports an invalid DO, i.e., UE cannot identify a valid DO report to be associated with a CJT-CSI report, it should calculate the CJT-CSI based on a default DO value, i.e., DO = 0.

Proposal 10: If UE reports an invalid DO in the associated DO report, i.e., out-of-range, it calculates the CJT-CSI based on a default DO value, i.e., DO = 0.

In RAN1 #119 meeting, the following on the timeline for linked DO and CJT-CSI report is agreed.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, the timeline (Z/Z’) is determined as Z/Z’ associated with the Rel-18 eType-II CJT, plus Drelax - The value of Drelax is a UE capability, taken from {0, drelax} - FFS: The value of drelax (>0), including whether it depends on SCS - For linking CJTC Dd and Rel-18 eType-II CJT CSI, joint triggering is a separate UE feature group from separate triggering

The remaining issue is the additional delay for the linked CJT-CSI and DO report. Since the UE needs to calculate the CJT-CSI and DO separately, the additional delay could be the processing delay for the DO report. It has been agreed that the processing delay for DO report is (Z2, Z2’), therefore, the additional delay Drelax can be defined based on Z2’.

Proposal 11: Support to define the additional delay Drelax for linked CJT-CSI and DO report based on Z2’

## PRB bundling for CSI-RS

For PO measurement, the NW may transmit the CSI-RS based on the precoder that it identifies based on SRS. The NW may apply a wideband precoder or subband precoders for the CSI-RS. Then it is necessary for the NW to tell the UE the PRG size for the precoded CSI-RS, so that the UE can identify the corresponding block size for the channel estimation for the CSI-RS.

Proposal 12: Support the NW to configure the PRG size for the precoded CSI-RS for PO measurement and report.

# Multi-CRI report

In RAN1 #119 meeting, the following has been concluded on multi-CRI report.

Conclusion For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports with the Rel-15 Type-I SP codebook, joint operation with the Rel-18 NES framework is not supported.

The Rel-18 NES includes two features: CSI report for spatial-domain NES and cell DTX. The conclusion above precludes the joint operation with Rel-18 NES framework. The intention should be to preclude the Rel-18 NES CSI report framework, but the joint operation for Rel-19 CRI-based CSI refinement and Rel-18 cell DTX should not be precluded.

Proposal 13: Clarify that the joint operation for Rel-19 CRI-based CSI refinement and Rel-18 cell DTX is supported

# Conclusion

In this contribution, we provided discussion on CSI enhancement for NR MIMO. Based on the discussion, the following proposals are provided.

Proposal 1: When the NW configures a Type1 codebook with more than 32 ports, support the NW configures the reportQuantity based on “RI-PMI-CQI” and “RI-PMI-CQI-LI” to distinguish from the legacy Type1 codebook based CSI report based on multiple CSI-RS resources with CRI reported.

Proposal 2: Support PD-NES for CSI report based on up to 128 ports to be aligned with Rel-18 NES principle that UE should support both Type1 SD and PD.

Proposal 3: For the TRSs configured for DO report, support the NW configure the TCI states for the TRSs based on the following schemes:

- Scheme 1 (Without pre-compensation of FO): Separate TCI state for each TRS

- Scheme 2 (With pre-compensation of FO): All the TRSs are configured with a common TCI state for QCL-TypeC indication without average delay, and each TRS is configured with a separate TCI state for QCL-TypeC indication without Doppler shift

Proposal 4: For the TRSs configured for FO report, support the NW configure the TCI states for the TRSs based on the following schemes:

- Scheme 1 (Without pre-compensation of DO): Separate TCI state for each TRS

- Scheme 2 (With pre-compensation of DO): All the TRSs are configured with a common TCI state for QCL-TypeC indication without Doppler shift, and each TRS is configured with a separate TCI state for QCL-TypeC indication without average delay

Proposal 5: For PO report, support the following constraints for the single-port CSI-RS resources configured as CMR:

- The minimum bandwidth for the single-port CSI-RS should be min(52, NBWP) RBs, where NBWP is the number of RBs for the active DL BWP

- The frequency domain density should be 3 REs/RB

Proposal 6: For the single-port CSI-RS resources configured for FO report, support the NW configure the TCI states for the CSI-RS resources based on the following schemes:

- Scheme 1 (Without pre-compensation of DO/FO): Separate TCI state with QCL-TypeC indication for each CSI-RS resource

- Scheme 2 (With pre-compensation of DO): All the CSI-RS resources are configured with a common TCI state for QCL-TypeC indication without Doppler shift, and each CSI-RS resource is configured with a separate TCI state for QCL-TypeC indication without average delay

- Scheme 3 (With pre-compensation of FO): All the CSI-RS resources are configured with a common TCI state for QCL-TypeC indication without average delay, and each CSI-RS resource is configured with a separate TCI state for QCL-TypeC indication without Doppler shift

- Scheme 4 (With pre-compensation of DO and FO): All the CSI-RS resources are configured with a common TCI state for QCL-TypeC indication

Proposal 7: For aperiodic SRS, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report.

Proposal 8: Support the UE to select the associated SRS resource set for PO report as follows

- When one periodic SRS resource set and one aperiodic SRS resource set are configured, UE selects the associated SRS resource based on the periodic SRS resource set

- When two semi-persistent SRS resource sets are configured, UE selects the associated SRS resource based on the activated SRS resource set

- When two aperiodic SRS resource sets are configured, UE selects the associated SRS resource based on the SRS resource set with lower set ID

Proposal 9: Support the NW to inform UE whether it transmits the CSI-RS resources for PO measurement with DO pre-compensation.

Proposal 10: If UE reports an invalid DO in the associated DO report, i.e., out-of-range, it calculates the CJT-CSI based on a default DO value, i.e., DO = 0.

Proposal 11: Support to define the additional delay Drelax for linked CJT-CSI and DO report based on Z2’

Proposal 12: Support the NW to configure the PRG size for the precoded CSI-RS for PO measurement and report.

Proposal 13: Clarify that the joint operation for Rel-19 CRI-based CSI refinement and Rel-18 cell DTX is supported