3GPP TSG RAN WG1 #120	   	 	  	                	      R1-2500570

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.2.4

Source:	LG Electronics

Title:	Discussions on asymmetric DL sTRP/UL mTRP scenarios

Document for:	Discussion and Decision

Introduction

In the revised WID for phase 5 MIMO [1], objectives for RAN1’s normative work were listed as below.

RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this contribution, we discuss enhancements for asymmetric DL sTRP/UL mTRP scenarios. Specifically, we provide our view on two TAs for asymmetric DL sTRP/UL mTRP scenarios.

Discussions

In previous meetings, followings were agreed for asymmetric DL sTRP/UL mTRP scenarios.

Agreement For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset: - When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW. - Note: it does not intend to increase the number of maintained PLs per cell. - FFS: whether to support associating joint TCI state (if supported) with a PL offset. Further study whether/how to apply a PL offset on PDCCH-order PRACH transmission too. - FFS: how to determine the Tx beam of PRACH towards UL TRP - Note: this does not imply to support 2 TA for single-DCI based system. Agreement For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2. - Joint TCI state mode can be configured at least for FR1 Agreement For FR1, a joint TCI state can be associated with a PL offset. - When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW. Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification - Above is subject to a separate UE capability signaling Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). Conclusion There is no consensus on the following proposal: Proposal 1.6: Support to update a UL PL for a joint/UL TCI state as follows: - When this joint/UL TCI state is activated and it is not in the current active TCI state list, a UL PL is calculated as: UL PL = PL estimated from DL PL RS – the value of PL offset. - When this joint/UL TCI state is activated and it is in the current active TCI state list, the UE updates the UL PL as: new UL PL = current UL PL + the updated delta indicated by the NW. Agreement For the asymmetric DL sTRP/UL mTRP scenarios, study and decide the value range and candidate values of PL offset value Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied. Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3: - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not. Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB - Step size is 4dB Agreement The lower limit of the value range of PL offset is extended to -12 dB Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped. Agreement For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0: - This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset. - When one joint/UL TCI state is indicated in Rel-17 unified TCI, - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power. - FFS: Whether the bit field can be used to indicate other information - FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI Agreement For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission: - Alt2: - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation. - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed. FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission Agreement Support to apply PL offset on PDCCH-order PRACH in FR2 - The design for applying PL offset on PDCCH-order PRACH in FR1 is fully reused here. Note: there is no extra enhancement for Tx beam determination for PDCCH-order PRACH in FR2 in Rel-19 Agreement For a UE supporting the UE capability of “Overlapping UL transmission reduction” of Rel-19 2TA: - The overlapping duration of the latter one of two overlapping UL transmissions is reduced, where the two overlapped UL transmissions are associated with two TAGs.

  - Support of two TAs for asymmetric DL sTRP/UL mTRP scenarios

In previous meeting, it was agreed to support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios, where Rel-18 2TA behavior is reused as much as possible. Furthermore, for indicating PL offset for PDCCH-ordered PRACH, a new 1-bit DCI field in DCI format 1_0 is introduced for both the case of managing only one indicated TCI(Rel-17) and the case of managing two indicated TCI(Rel-18). With the new 1-bit DCI field in DCI format 1_0, gNB can indicate whether or not to use PL offset associated with the indicated TCI for calculating PRACH transmission power when one joint/UL TCI state is indicated. Also, gNB can indicate PL offset associated with the first or second indicated TCI for calculating PRACH transmission power when two joint/UL TCI states are indicated.

We think that remaining issue is which PL-RS can be used for PRACH transmission triggered by the PDCCH-order in Rel-19 asymmetric DL sTRP/UL mTRP deployment scenarios. One may say that PL-RS configured in the (one of two) indicated TCI is natural way to use since gNB can indicate whether to use PL offset associated with the indicated TCI or indicate PL offset associated with the first or second indicated TCI. In this case, because the configured/updated PL offset is based on PL-RS configured in the indicated TCI so that it makes sense for UE to use the both PL-RS and PL offset (as a package) associated with the indicated TCI, for PDCCH-ordered PRACH transmission. However, as described in the below Table 1 and Table 2, if the 1-bit bit field indicates not to use PL offset associated with the indicated TCI in Rel-17 unified TCI or if the 1-bit field indicates first or second indicated TCI not configured with PL offset in Rel-18 unified TCI, legacy PL-RS determination rule (i.e., based on DM-RS of PDCCH-order) can be reused since the PDCCH-ordered PRACH is targeting macro cell with both DL/UL transmission/reception.

Table 1. PL-RS determination rule when one joint/UL TCI state is indicated in Rel-17 unified TCI

| Codepoint of a new 1-bit field in DCI format 1_0 | PL-RS determination rule for PDCCH-ordered PRACH transmission |
| --- | --- |
| “0” (Don’t use PL offset in the indicated TCI state) | legacy PL-RS determination rule (i.e., based on DM-RS of PDCCH-order) |
| “1” (Use PL offset in the indicated TCI state) | PL-RS configured in the indicated TCI state |

Table 2. PL-RS determination rule when two joint/UL TCI states are indicated in Rel-18 unified TCI

| Codepoint of a new 1-bit field in DCI format 1_0 | PL-RS determination rule for PDCCH-ordered PRACH transmission |  |
| --- | --- | --- |
|  | When indicated TCI is configured with PL offset | When indicated TCI is not configured with PL offset |
| “0” (Use PL offset in the first indicated TCI state) | PL-RS configured in the first indicated TCI state | legacy PL-RS determination rule (i.e., based on DM-RS of PDCCH-order) |
| “1” (Use PL offset in the second indicated TCI state) | PL-RS configured in the second indicated TCI state | legacy PL-RS determination rule (i.e., based on DM-RS of PDCCH-order) |

Proposal #1: Support the following PL-RS determination rule for PDCCH-ordered PRACH:

- If the 1-bit bit field indicates not to use PL offset associated with the indicated TCI in Rel-17 unified TCI or if the 1-bit field indicates first or second indicated TCI not configured with PL offset in Rel-18 unified TCI, the legacy PL-RS determination rule can be used for the PDCCH-ordered PRACH

- Otherwise, PL-RS configured in the indicated TCI is used for the PDCCH-ordered PRACH

In PDCCH order, NW can indicate RA preamble index, SSB index, and PRACH mask index to indicate RACH occasion of the PDCCH ordered PRACH for UE. However, UL TRP has no SSB transmission, it should be further discussed how to define/configure RACH occasion for UL TRP. One simple solution is for UL TRP to share RACH occasion of macro cell (even though UL TRP has not SSB transmission), then NW can indicate RACH occasion associated with UL TRP in the PDCCH order, by NW implementation. Meanwhile, PDCCH order can trigger CBRA PRACH (via indicating ra-PreambleIndex as 0b000000), but in this case, UE cannot know which SSB index and/or RACH occasion is associated with macro cell or UL TRP although NW already indicated target TRP of the PDCCH ordered PRACH in the PDCCH order, as discussed above. Furthermore, if UL TRP shares RACH occasion of macro cell, CBRA PRACH towards macro cell can collide with the PDCCH ordered PRACH towards UL TRP in the same RACH occasion with the same RA preamble. In order to handle this, SSB index and/or RACH occasion can be divided into two or more groups to discriminate RACH occasion of macro cell and that of UL TRP(s).

Proposal #2: Further discuss how to define/configure RACH occasion for UL TRP.

Conclusion

In this contribution, we have discussed enhancements for asymmetric DL sTRP/UL mTRP scenarios, and provided the following proposals.

Proposal #1: Support the following PL-RS determination rule for PDCCH-ordered PRACH:

- If the 1-bit bit field indicates not to use PL offset associated with the indicated TCI in Rel-17 unified TCI or if the 1-bit field indicates first or second indicated TCI not configured with PL offset in Rel-18 unified TCI, the legacy PL-RS determination rule can be used for the PDCCH-ordered PRACH

- Otherwise, PL-RS configured in the indicated TCI is used for the PDCCH-ordered PRACH

Proposal #2: Further discuss how to define/configure RACH occasion for UL TRP.

References

[1] RP-242394, “WID revision: NR MIMO Phase 5”, Samsung