3GPP TSG RAN WG1 #120		 	      R1-2500577

Athens, Greece, February 17th – 21st, 2025

Source:           ZTE Corporation, Sanechips

Title:             Discussion on channel modelling for ISAC

Agenda item:      9.7.2

Document for:     Discussion

Introduction

Current 5G-Advance network is basically designed for data transmission, which may not support sensing on many dimensions, e.g., signal, network structure, deployment mode and so on. To support evolution to integrated sensing and communication (ISAC) system, a study item on channel modelling for ISAC for NR is approved in RAN#102 meeting [1]. The objective of the SID is copied below.

Objective of SI The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

Similar as channel modelling for communication, typical channel modelling generation methods can include statistical channel modelling and deterministic channel modelling for sensing, i.e. stochastic geometry and hybrid with ray tracing channel modelling respectively in TR 38.901 [2]. In this contribution, we provide our views for both stochastic geometry and hybrid with ray tracing channel modelling.

Especially on stochastic geometry modeling, our detailed analysis and preference are provided in the following sections on RCS / CPM for UAV, Human, Vehicle, multi-scattering points, concatenation details, absolute time delay, spatial consistency, background modeling, power normalization between target and background, EO type 2, etc.

It is noted that the real experiment results are provided for UAV, Human and Vehicle in Appendix of our previous contribution [17]. In the following sections, Ray-tracing simulation results are shown after the EM calibration based on the real measurement.

RCS calculation

Agreement RAN1 strives to define a single option per target per monostatic/bistatic sensing mode from the following two options to generate RCS values/patterns for a scattering point of a target. - Option 2: The RCS=A*B of a scattering point can be generated by - The component A is commonly applied to any incident/scattered angles at the scattering point - A is [mean] RCS value. FFS value(s) A - Note: Mean RCS value is defined as the mean value of the distribution of RCS - The component B - B is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2 - B is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS - Option 3: The RCS=A*B=A*B1*B2 of a scattering point can be generated by - The component A is commonly applied to any incident/scattered angles at the scattering point - FFS: A = 1 m2 or [mean] RCS value - Note: Mean RCS value is defined as the mean value of the distribution of RCS - The component B is further split into B1, B2, i.e., B=B1*B2 - B1 is deterministic based on incident/scattered angles - FFS: B1 is defined by a function or by a table - B2 is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2 - B2 is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS Agreement RCS Option 3 is selected to model RCS of UAV with single scattering point for monostatic - B2 of UAV is modelled using log-normal distribution for monostatic - Different mean RCS values can be supported for UAV due to different size, shape, frequency, etc. - For UAV of small size (option 2 for UAV size in UAV parameters table) - B1=1 - A is mean RCS value - For UAV of large size (option 1 for UAV size in UAV parameters table) - B1 have dependency on incident/scattered angles - A is mean RCS value Agreement Component B2 of RCS is upper bounded by kσ dB for the log-normal distribution, where σ is the standard deviation of B2 in dB. FFS the value of k.

In RAN#118bis meeting, the above agreements about RCS model has been approved, which split RCS value in linear scale into three parts A, B1 and B2 as shown in the above Option 3. According to the agreements, A is the mean value of large scale RCS, and B (including B1 and B2) is the normalized values in small scale. However, there are two following implicit issues to be clarified.

- The average operation to obtain A should been done for linear-scaled RCS value rather than dB-scale RCS value.

Since the mean value of linear-scaled RCS and the mean value of dB-scale RCS are not the same, which is

Hence, it should be clarified that A should be obtained as  from linear-scaled RCS values.

- The B2 obeys log-normal distribution, which has mean value 1 (1 is a linear-scaled value), and variance V. However, the linear-scaled mean value of 1 of B2 is not equal to 0 mean value in dB-scale.

For example, if B2 obeys a log-normal distribution with parameters  and  , which is noted as . Then  obeys a normal distribution . The mean value of  is expected as  and variance as . Then the linear-scaled mean value of B2 should be derived as ( it should be noticed that is not equal to ).  If we assume that  has a mean value , then  is larger than 1.

To make E[B2] be 1,  should be satisfied.

In short, for log-normal distribution, μ = 1 in linear-scaled value is not equal to μ = 0 in dB-scaled value. To ensure companies provide correct distribution of B2 with linear-scaled mean value 1, we propose to use the following procedure for fitting RCS parameters including A, B1 and B2.

- Step 1: Obtain RCS measurement and simulation results.

Based on measurement or simulation, the RCS of interested sensing target could be sampled, and a set of samples  could be obtained.

- Step 2: Split  into  dependent with angular and independent with angular.

Using smooth operation, or other fitting operation, the total  could be split into two parts. The first part is , which is dependent with angular. The second part is , which is independent with angular.

The  focuses on relationship between shape of sensing target and RCS value, thus there are spatial co-relationship between  and  with different angles of incident or departure . Thus the part of should be modeled as pattern.

The  focuses on the angular glint phenomena of RCS value.

There are following equations:

- Step 3: Normalize the angular dependent .

Estimate the linear-scaled mean value of , and denoted it as .

is the contribution of pattern to , then is obtained by normalizing  with , i.e.

where the  describes the normalized pattern at the corresponding ZOA/AOA/ZOD/AOD representing by sample index .

- Step 4: Normalize the angular independent .

Estimate the linear-scaled mean value of , and denoted it as , which is the contribution of  to .

Then the samples could be normalized as , which could be used to fit the parameters of . The  obeys normal distribution, of which the mean and variance value could be fitted as  and  respectively.

We know the log-normal distribution has mean value equal to , thus we could derive the accurate mean value of  as

to ensure the mean value of  be equal to 1.

- Step 5: Calculate A

According to the discussion before, the A could be derives by

Proposal 1: Clarify RCS components A, B1, B2 as following

- For B2,  to ensure the mean value  be equal to 1

- For B1,  to ensure the mean value  be equal to 1

- For A,  and

For the component B2 of RCS, an upper bound by kσ dB is agreed to truncate the outlier in the last RAN1 meeting. Since the B2 is generated randomly with the same mean and standard deviation for both direct path and indirect paths, there may occur the B2 value is very larger for indirect path than that for direct path. In such case, the power difference between the direct path and indirect path will be diminished, and then the sensing performance will be degraded due to the interference of the indirect paths. Thus, B2 should be limited in a certain range, in order to assure the glint gain resulting from RCS B2 stable. In our viewpoint, the value k equal to 1, that means , is sufficient to draw the glint as well as keep the RCS gain stable as shown in the measurement results in section 4, 5 and 6.

Proposal 2: For the upper boundary kσ dB of the component B2 of RCS, k=1 is suggested.

CPM normalization

Agreement To model the effect of polarization for each direct/indirect path: - Polarization of a direct/indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link - Total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For a LOS ray from Tx to target or from target to Rx, for CPMtx,sp or CPMsp,rx - For a NLOS ray generated by a stochastic cluster from Tx to target or from target to Rx, CPMtx,sp or CPMsp,rx is generated by XPR ratio  and initial random phases referring to TR 38.901 as start point - FFS how to normalize on CPMtx,sp,rx - FFS CPMsp of a scattering point of the target - FFS: how to model the effect of polarization when EO type-2 is present

In RAN#118bis meeting, the above agreements about CPM has been approved. According to the agreement, total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx· CPMsp·CPMtx,sp. However, there is a FFS about how to normalize on CPMtx,sp,rx in order to ensure the amplitude of the final CPM be 1. Based on our analysis, the following normalization method can be utilized.

Draw CPMtx,sp,rx as

CPMtx,sp,rx = CPMsp,rx·CPMsp·CPMtx,sp

= a11a12a21a22∙b11b12b21b22∙c11c12c21c22 = d11d12d21d22.

The normalized CPMtx,sp,rx  can be represented as

CPMtx,sp,rxnorm=d11d12d21d2212(d11+d22)=2(d11+d22)d11d12d21d22

It is noted that CPM will impact path power if the above normalization is not supported. Especially, low power path dropping, e.g. lower than a threshold -25dB will not consider the impact the CPM. Hence, CPMtx,sp,rx without normalization may lead to a path with higher power dropped.

Proposal 3: Denote

CPMtx,sp,rx = CPMsp,rx . CPMsp . CPMtx,sp

= a11a12a21a22∙b11b12b21b22∙c11c12c21c22 = d11d12d21d22.

The normalized CPMtx,sp,rx  can be represented as

CPMtx,sp,rxnorm =d11d12d21d2212(d11+d22)=2(d11+d22)d11d12d21d22

RCS/CPM for UAV

Agreement RCS Option 3 is selected to model RCS of UAV with single scattering point for monostatic - B2 of UAV is modelled using log-normal distribution for monostatic - Different mean RCS values can be supported for UAV due to different size, shape, frequency, etc. - For UAV of small size (option 2 for UAV size in UAV parameters table) - B1=1 - A is mean RCS value - For UAV of large size (option 1 for UAV size in UAV parameters table) - B1 have dependency on incident/scattered angles - A is mean RCS value

For UAV of small size (option 2 for UAV size in UAV parameters table), the UAV size (Length x Width x Height) is 0.3m x 0.4m x 0.2m. The RCS simulation results using RT simulation at 4.9 GHz are as following.

| H-H | V-V |
| --- | --- |
| H-V | V-H |

Figure 4-1 RCS pattern of polarization and mean value for UAV

Based on the agreement of RCS Option 3 of small size UAV, the RCS can be modeled as RCS=A*B=A*B1*B2 , where, B1=1 and A is mean RCS value. From the above simulations results, the linear mean RCS value, i.e. A equals 0.053m2, corresponding -12.74dBsm. B2 corresponding to glint is generated by log-Normal distribution. Draw B2 values as

B2 =10X/10,

Where,  is Gaussian distributed with and .

Furthermore, according to the above RCS values for four polarization, the cross polarization ratios can be achieved. It can be calculated that the mean XPR in dB is about 24.4 and standard deviation is about 3.05. Therefore, the cross polarization power ratios (XPR) for the UAV can be modelled as log-Normal distribution. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 24.4and = 3.05.

The CPM for UAV obtained using the above XPR can be represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

Proposal 4: For UAV of small size (Length x Width x Height: 0.3m x 0.4m x 0.2m) at 4.9GHz for both mono-static and bi-static modes:

  - B1=1

  - A is mean RCS value of 0.053m2 corresponding -12.74 dbsm

  - B2 of UAV is modelled using log-normal distribution. Draw B2 linear-scaled value as

B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for the UAV is modelled as log-Normal distribution. Draw linear-scaled XPR value as

 = 10Y/10,

Where,  is Gaussian distributed with 24.4and = 3.05.

  - The CPM for UAV is represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

RCS/CPM for Pedestrian

Agreement The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc. - Model 1 - B1=0 dB - A is mean RCS value - B2 is modelled using log-normal distribution - Model 2 - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A - FFS: conditions for using which model

To verify pedestrian RCS values in different scattering angles, we further provide some Ray-tracing simulation results based on the calibrated EM parameters [10] [11]. An adult model selected in these simulations are approximately 0.5 meters in length, 0.5 meters in width, and 1.75 meters in height. The distance between the Tx/Rx and the pedestrian model is 15m. The monostatic simulation results at 4.9 GHz are as following.

| H-H | V-V |
| --- | --- |
| H-V | V-H |

Figure 5-1 RCS pattern of Pedestrian

Based on the agreement of RCS Model 1 in last RAN1 meeting, the RCS of pedestrian can be modeled as RCS=A*B=A*B1*B2, where B1=0dB and A is mean RCS value. From the simulations results, mean RCS value equals to 0.4m2, corresponding -4.0 dBsm.

B2 corresponding to glint is generated by log-Normal distribution. Draw B2 values as

B2 =10X/10,

Where,  is Gaussian distributed with and .

Furthermore, according to the above RCS values for four polarization, the cross polarization ratios can be achieved. It can be calculated that the mean XPR in dB is about 25.8 and standard deviation is about 2.99. Therefore, the cross polarization power ratios (XPR) for the pedestrian can be modelled as log-Normal distribution. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 25.8 and = 2.99.

The CPM for pedestrian obtained using the above XPR can be represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

Proposal 5: The model 1 is preferred for pedestrian with single scattering point for both mono-static and bi-static modes:

  - For Adult pedestrian (approximately 0.5 meters in length, 0.5 meters in width, and 1.75 meters in height) at 4.9GHz

    - B1=0dB

    - A is mean RCS value of 0.4m2

    - B2 of pedestrian is modelled using log-normal distribution. Draw B2 values as

B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for pedestrian is modelled using log-Normal distributed. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 25.8 and = 2.99.

  - The CPM for UAV is represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

RCS for Vehicle

The 3D car model with calibrated EM parameters is believed to reflect real EM properties. In this section, the simulation with the calibrated EM parameters is carried on at 4.9GHz as center frequency. The car's dimensions are approximately 5 meters in length, 2.0 meters in width, and 1.6 meters in height.

In the following results, from the car perspective, AOA/ZOA are the input angles, and AOD/ZOD are the output angles. ZOA/ZOD = 0°corresponds z axis, and AOA = 0, 90, 180, 270°correspond to the car left side, back, right side and front respectively.

Figure 6-1 Vehicle model in coordinate

Mono-static

For mono-static sensing mode in the carrier frequency of 4.9GHz, the RCS values for four polarization combinations are simulated and the H-H polarization results are shown as an example in the following figures. For the 3D RCS simulation, Tx (Rx) and ST may be in the different heights.

Figure 6-2 shows the 3D RCS distribution for mono-static. The 3D RCS values are simulated with different ZOA/ZOD and AOA/ZOA values from the vehicle perspective.

Figure 6-2 3D RCS for mono-static for H-H polarization

Based on the 3D RCS distribution, Figure 6-3 provides the top view with region division of 3D RCS values for mono-static. The overall region can be quantified into 5 areas for simplicity.

Figure 6-3 Region division of 3D RCS for mono-static for H-H polarization from top view

Specifically, the area 1 corresponds to the car left side, area 2 corresponds to the back of the car, area 3 corresponds to the car right side, area 4 corresponds to the front of the car, area 5 corresponds to the car roof.

For each region after fitting by gradient descent algorithm, the RCS modeling can refer to the modeling of the 3D antenna beam in TR 38.901. In details, the input parameters are listed in the following table into the formula to obtain the corresponding mono-static RCS pattern rcsdB(θ,φ) in dBsm.

rcsdB(θ,φ)=Gmax−min−AVdBθ+AHdB φ,Amax

Wherein,

AVdBθ=−min12θ−θcenterθ3dB2, Amax,

AHdB φ=−min12φ−φcenterφ3dB2, Amax,

Table 6-1 Parameters for mono-static RCS modeling for vehicle

| Area/n | Side of vehicle | θcenter | φcenter | φ3dB | θ3dB | Amax | Gmax [dBsm] | Applicable range of ZoA/θ | Applicable range of AoA/φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Left | 78.8 | 0° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [0°, 45°]∪[315°, 360°] |
| 2 | Back | 71.6° | 90° | 39.6° | 43.9° | 2.76 | 4.62 | [20°, 90°] | [45°,135°] |
| 3 | Right | 78.8° | 180° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [135°,225°] |
| 4 | Front | 20.0° | 270° | 95.7° | 48.4° | 10.05 | 9.49 | [20°, 90°] | [225°,315°] |
| 5 | Roof | 0° | - | - | 28.42 | 3.63 | 10.99 | [0°, 20°] | [0°,360°] |

The 3D RCS based on the above fitting pattern is shown in below.

Figure 6-4 3D fitting RCS pattern without glint

Figure 6-5 Top view of 3D fitting RCS without glint

Based on the above observation, the slow fast fading of RCS can be modeled as rcsdB(θ,φ). However, the glint, i.e. fast change part is still not reflected. As described in our previous contribution [10], Gaussian distribution without any spatial correlation can be used to model the fast RCS variation due to glint. Hence, the final RCS pattern is shown as below:

RCS_car_dB = rcsdB(θ,φ) + glint, where glint is Normal distribution, N(0,σ)

Also, the glint modeling is shown in the following table after fitting, wherein the standard deviation is around 3.28.

The fitting modeling of the final RCS_car with both slow fast and glint with standard deviation is shown as below. It can be seen the modeling is well matched with the original patterns output by RT simulation.

Figure 6-6 3D fitting RCS with glint without smoothing

Based on the agreement of RCS Option 3 in last RAN1 meeting, the RCS of a car can be modeled as RCS=A*B=A*B1*B2 of a scattering point. The component A as large scale loss is commonly applied to any incident/scattered angles at the scattering point.

From the simulations, for the pattern, the mean value in linear value is 4.58 m2, corresponding 6.61dBsm. For the glint, the mean value in linear value is 2.56 m2, corresponding 4.08dBsm. Therefore, the value of A is 11.72 m2, i.e. 10.69 dBsm, which is obtained by summing 6.61 and 4.08.

B1 in dB is deterministic based on incident/scattered angles as following:

B1dB  = rcsdB(θ,φ)−6.61

B2 corresponding to glint is generated by log-Normal distribution. Draw B2 values as

B2 =10X/10,

Where,  is Gaussian distributed with and .

Furthermore, the mean RCS values for four polarization combinations are achieved in the following table.

Table 6-2 Mean value of statistics for difference areas

| Polarization pair (Tx-Rx) | mean value | Area 1 | Area 2 | Area 3 | Area 4 | Area 5 | All area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H-H |  | 4.9556217 | 1.4828804 | 5.0124016 | 3.2137318 | 9.813468 | 4.8956207 |
| H-V |  | -17.57543 | -17.800802 | -17.683006 | -19.035542 | -14.805525 | -17.380061 |
| V-H |  | -17.534603 | -17.70423 | -17.809626 | -19.084879 | -14.734744 | -17.3736164 |
| V-V |  | 4.907881 | 1.4824759 | 5.029778 | 3.1972172 | 9.783014 | 4.88007322 |

Based on the above mean RCS values for four polarization, the cross polarization ratios can be achieved. It can be observed that the mean XPR is about 22 dB. Therefore, the cross polarization power ratios (XPR) for the vehicle can be modelled as log-Normal distribution. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 22.3 and = 4.71.

Proposal 6: For mono-static, the RCS=A*B=A*B1*B2 of a scattering point for a passenger vehicle Type 1/2 can be generated by

  - A has mean RCS value 11.72 m2 corresponding 10.69 dBsm

  - B1 is deterministic based on incident/scattered angles

B1dB = rcsdB(θ,φ)−6.61.

Where,

rcsdB(θ,φ)=Gmax−min−AVdBθ+AHdB φ,Amax

Where,

AVdBθ=−min12θ−θcenterθ3dB2, Amax,

AHdB φ=−min12φ−φcenterφ3dB2, Amax,

| Area/n | Side of vehicle | θcenter | φcenter | φ3dB | θ3dB | Amax | Gmax | Applicable range of ZoA/θ | Applicable range of AoA/φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Left | 78.8 | 0° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [0°, 45°]∪[315°, 360°] |
| 2 | Back | 71.6° | 90° | 39.6° | 43.9° | 2.76 | 4.62 | [20°, 90°] | [45°,135°] |
| 3 | Right | 78.8° | 180° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [135°,225°] |
| 4 | Front | 20.0° | 270° | 95.7° | 48.4° | 10.05 | 9.49 | [20°, 90°] | [225°,315°] |
| 5 | Roof | 0° | - | - | 28.42 | 3.63 | 10.99 | [0°, 20°] | [0°,360°] |

    - B2 is generated by log-normal distribution

      - Draw B2 values as B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for the Vehicle is modelled using log-Normal distributed. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 22.3 and = 4.71.

Bi-static

For 3D bi-static sensing mode in the carrier frequency of 4.9GHz, the RCS values for four polarization combinations are simulated and the H-H polarization results are shown in the following figures.

In all cases, the incident angle AOA/ZOA is fixed, and from the side of the car, the 3D figure is shown in different AOD/ZOD values.

- Case 1: (AOA=180°, ZOA=90°) is fixed, i.e. incident angle is perpendicular to the right side door of the car. Larger RCS values can be observed around the specular angles, i.e. AOD = 180°. In left side of the car, the received signal power is very weak because of blockage, the RCS in area around AOD = 0°is quite low with only diffraction or front scatter power.

Diffraction area

Diffraction area

Figure 6-7 3D view figure of bi-static with AOA=180°

- Case 2: (AOA=135°, ZOA=90°) is fixed, i.e. the incident angle is 45°from the right side door of the car. The first strongest RCS area can be observed around the specular angles reflected from the right door of the car, i.e. reflection angle AOD = 225°. The second strongest RCS area can be observed around the specular angles reflected from the back of the car, i.e. refection angle AOD = 45°. Because the car back and right side are usually inclined upward, the reflection angles are directed to the air, i.e. the reflection angle ZOD is closed to 0°. The diffraction or front scatter area is round AOD = 135+180= 315°where the RCS values are lowest.

Figure 6-8 Bi-static RCS value distribution area

Diffraction area

Strongest 1

Strongest 2

Figure 6-9 3D view figure of bi-static with AOA=135°

- Case 3: (AOA=100°, ZOA=90°) is fixed. i.e. the incident angle is 10°with the right side door of the car. The second strongest RCS area can be observed around the specular angles reflected from the right door of the car, i.e. reflection angle AOD = 260°. The first strongest RCS area can be observed around the specular angles reflected from the back of the car, i.e. reflection angle AOD = 80°. The diffraction or front scatter area is round AOD = 100+180= 280°.

Diffraction area

Strongest 1

Strongest 2

Figure 6-10 3D view figure of bi-static with AOA=100°

Based on the RCS RT simulation results of vehicles, there is only one strongest RCS area which can be further modelled similar as 3D beam in TR 38.901 for case 1 (incident angle perpendicular to the car’s right door). In contrast, both case 2 and case 3 show two distinct strongest RCS areas, wherein the peak magnitudes and center location of RCS area for case 2 and case3 vary significantly. These simulation results show that modeling the RCS of vehicles for bi-static mode is more complex than that for monostatic mode.

Furthermore, these two strongest RCS areas observed from the above simulation results may be caused by reflections/scattering from vehicle's two sides. For bi-static RCS modelling, we provide additional simulation validations covering AOA values that range from 100 to 180 degrees, with granularity 10 degrees.

Case 3: (AOA=100°, ZOA=90°)

Case 4: (AOA=110°, ZOA=90°)

Case 5: (AOA=120°, ZOA=90°)

Case 6: (AOA=130°, ZOA=90°)

Case 7: (AOA=140°, ZOA=90°)

Case 8: (AOA=150°, ZOA=90°)

Case 9: (AOA=160°, ZOA=90°)

Case 10: (AOA=170°, ZOA=90°)

Case 1: (AOA=180°, ZOA=90°)

Figure 6-11 Bistatic simulations from different AOA

To analyze the right side of the vehicle, the RCS pattern can be modeled as a beam pattern related with a moving center position which related to the AOA. From the above simulation results, the center positions are approximately close to the location of the mirror reflection points of the vehicle's right side as shown in the following figure. The RCS pattern is modeled as a beam pattern wherein the peak point in the beam is around the reflection direction. The actual RCS value for the scattering angle (θs,φs) can be computed based on this beam pattern. It is noted that the refection angle (θreflect,φreflect), i.e. the peak point in the beam pattern relies on the incident angle (θi,φi) and the normal direction (θn,φn).

In the figure, the angle (θn,φn) is the normal direction, i.e. it is in the middle between incident and reflection angles.

Figure 6-12 Reflection beam pattern

In summary, the RCS pattern of the car can be modeled as a 3D beam pattern:

rcsdB(θi,φi,θs,φs)=Greflection−min−AVdBθs+AHdB φs,Amax

Wherein,

AVdBθs=−min12θs−θreflectθ3dB2, Amax,

AHdB φs=−min12φs−φreflectφ3dB2, Amax

To calculate the direction of the reflection angle (θreflect,φreflect), we can use vector reflection formulas as following steps.

Step1. Convert angles to vectors.

The normal vector to the plane is denoted as

N=(sin(θn)cos(φn), sin(θn)sin(φn), cos(θn) ).

The incident vector is:

I=(sin(θi)cos(φi), sin(θi)sin(φi), cos(θi) )

Step2. Calculate the reflection vector.

Based on the parallelogram principle for vector addition, there is R+I = 2(I∙N)N.

Thus, the reflection vector R is given by: R= 2(I∙N)N− I, where the I∙N is the dot product of I and N.

Step3. Convert the reflection vector back to angles.

The reflection vector R=(sin(θreflect)cos(φreflect), sin(θreflect)sin(φreflect), cos(θreflect) )

To extract the angles (θreflect,φreflect) from the reflection vector:

θreflect=arccos(R∙(0, 0, 1))

φreflect=mod(2π+arctan2(R∙(0, 1, 0),  R∙(1, 0, 0)), 2π)

Note: For function arctan2(y, x), the range of values falls within [-π, π).

In summary, the clear formula of and  could be given as follows:

In addition, the 3dB beam width can be consistent with that used in the mono-static scenario for the vehicle. Regarding the beam's peak value Greflection, for AOA values within the range of [90°,180°], the RCS values with AOD range [90°, 270°] are shown in Table 6-3, which can be modeled as

Greflection = Gmax_RCS*cos((φi-φreflect)/2)*cos((θi-θreflect)/2).

Table 6-3 The simulation results of beam’s peak value for different AOAs

| AOA/degree | 100 | 110 | 120 | 130 | 140 | 150 | 160 | 170 | 180 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Greflection from RT simulation | 0.78 | 3.93 | 6.02 | 7.31 | 7.69 | 8.1 | 8.29 | 7.70 | 8.33 |
| Greflection from modelling | 1.52 | 3.0 | 4.39 | 5.64 | 6.73 | 7.61 | 8.25 | 8.64 | 8.78 |

Figure 6-13 Reflection beam’s peak values

Based on the above analysis, we have the following proposal:

Proposal 7: For Bi-static, the RCS=A*B1*B2 of a scattering point for a passenger vehicle Type 1/2 can be generated by

  - A has mean RCS value 11.72 m2 corresponding 10.69 dBsm

  - B1 is deterministic based on incident/scattered angles

B1dB = rcsdB(θi,φi,θs,φs)−6.61.

Where,

rcsdB(θi,φi,θs,φs)=Greflection−min−AVdBθs+AHdB φs,Amax

and,

AVdBθs=−min12θs−θreflect(θi)θ3dB2, Amax,

AHdB φs=−min12φs−φreflect(φi)φ3dB2, Amax,

Greflection= Gmax_RCS*cos((φi-φreflect)/2)*cos((θi-θreflect)/2),

| Side of vehicle | ZoA/θi | AoA/φi | (θn,φn) | φ3dBn | θ3dB | Amax | Gmax_RCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | [20°, 90°] | [0°, 90°]∪ [270°, 360°] | (78.8, 0) | 47.5° | 99.5° | 5.8 | 9.5 |
| Back | [20°, 90°] | (0°,180°) | (71.6, 90) | 39.6° | 43.9° | 2.76 | 4.62 |
| Right | [20°, 90°] | (90°,270°) | (78.8, 180) | 47.5° | 99.5° | 5.8 | 9.5 |
| Front | [20°, 90°] | (180°,360°) | (20, 270) | 95.7° | 48.4° | 10.05 | 9.49 |
| Roof | [0°, 20°] | (0°,360°) | (0, -) | - | 28.42 | 3.63 | 10.99 |

Note： if θn=0,  then  Greflection = Gmax_RCS*cos(θi).

  - B2 is generated by log-normal distribution

      - Draw B2 values as B2 =10X/10,

Where,  is Gaussian distributed with and .

Multi-scattering points

For some sensing targets with the considerable size, such as vehicles in urban grid scenario, they are suitable to be modeled by multiple scattering points. As shown in the following figure, if a vehicle is modeled with multiple scattering points, different numbers of scattering points my be visible for the two TRPs in reality. However, it is hard to determine how many / which scattering points are visible for each TRP by explicit model, especially when the vehicle shape is irregular.

Figure 7-1 RCS issues for Multi-points modelling for a car

To simplify the modeling, we suggest the number of visible scattering points is always fixed, which is generated in the beginning of the simulation and keeps unchanged. Depending on type/size of the target, the numbers of scattering number of different targets can be different. For the same target sensed by different sensing Tx/Rx, its scattering points number and locations are fixed as well. As shown in the above figure, for TRP B, the RCS values for the non-invisible scattering points (yellow points) will be extremely low based on the RCS patterns shown in section 5, it is equivalent that only three red scattering points are modeled for the TRP B.

In addition, the locations of multiple scattering points rely on the shape of the target. Although there is micro motion of the target, the changes of relative locations of multiple scattering points may not be needed to model for simplicity.

For other types of sensing targets including UAV, pedestrian and AGV, even though single scattering point is sufficient for the use case of intrusion, location and tracking as described in Rel-19 SID, we are open to support multiple scattering points for forward compatibility.

For vehicle, based on the single scattering point modeling, the multi-scattering points modeling can be developed with the principle that the RCS value of each scattering point should be smaller than that of the corresponding value of the single scattering point. For each region after fitting by gradient descent algorithm, the RCS modeling can refer to the modeling of the 3D antenna beam in TR 38.901. In details, the input parameters are listed in the following table into the formula to obtain the corresponding RCS pattern of the n-th scattering point rcsdB(n,θ,φ) in dBsm.

mrcsdB(n) = dB[ linear(rcsdB(n))−k=1, k≠n5linear(rcsdB(k))]

Wherein,rcsdB(n) is the RCS value from single scattering point. This operation is to make sure the total RCS values from multiple scattering point modeling is the same as the one for single scattering point modeling.

For example, for each scattering point in mono-scattering modeling,

rcsdB(n,θ,φ)=Gmax −min−AVdBθ+AHdB φ,Amax

and,

AVdBθ=−min12θ−θcenterθ3dB2, Amax,

AHdB φ=−min12φ−φcenterφ3dB2, Amax,

and, if rcsdB(k,θ,φ) = Gmax(k) -Amax(k) :

rcs'dB(k,θ,φ) = Gmax(k) -Amax(k) -20dB. where 20dB is the blockage or diffraction loss as shown in the figures in section 6.2.

Table 7-1 Parameters for multi-scattering points RCS modeling of vehicle

| Point index/n | Side of vehicle | θcenter | φcenter | φ3dB | θ3dB | Amax | Gmax [dBsm] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Left | 78.8 | 0° | 47.5° | 99.5° | 5.8 | 9.5 |
| 2 | Back | 71.6° | 90° | 39.6° | 43.9° | 2.76 | 4.62 |
| 3 | Right | 78.8° | 180° | 47.5° | 99.5° | 5.8 | 9.5 |
| 4 | Front | 20.0° | 270° | 95.7° | 48.4° | 10.05 | 9.49 |
| 5 | Roof | 0° | - | - | 28.42 | 3.63 | 10.99 |

Proposal 8: For a target modeled with M>=1 scattering points, the following characteristics are supported:

- The number of visible scattering points is fixed to M, i.e. not depending on distance between Tx/Rx and the target.

- The number of scattering points of a target is generated at the beginning of the simulation and keeps unchanged

- The number and locations of the scattering points are common to each pair of sensing Tx/Rx

- The relative locations of the M scattering points in LCS of the target are based on the shape of the target

- The changes of the relative locations of multiple scattering points are not modeled

- For vehicle, M = 5 and the RCS pattern of the n-th scattering point rcsdB(n,θ,φ) in component B1 is:

mrcsdB(n) = dB[ linear(rcsdB(n))−k=1, k≠n5linear(rcsdB(k))]

Wherein rcsdB(n) is the RCS value from single scattering point.

In general, a target modeled with multiple scattering points has a considerable shape and size considering the distance between Tx/Rx and the target. In such case, the channel condition and RCS reflection gain are usually different for different scattering points. For example, in the scenario of urban grid with a vehicle as the sensing target, the LOS conditions and the propagation gains for different sides of the vehicle are distinct. Thus, for a target modeled with multiple scattering points, the channel of each scattering point should be generated separately like a target with a single scattering point. In details, for each scattering point, one direct path and several indirect paths are modeled.

Furthermore, for each scattering point, LOS condition can be generated separately following TR 38.901. And the correlation of the LOS condition among multiple scattering points should be built according to spatial consistency. Based on LOS condition and the location of scattering point, the pathloss can be calculated for each scattering point.

Since multiple scattering points are relatively close to each other, the correlation of channels of multiple scattering points should be modeled. Naturally, the spatial consistency can be used to describe this correlation. As the locations of multiple scattering points are not changed during the target moves, each scattering point can be regarded as a target with a single scattering point. And then spatial consistency can be applied for each scattering point separately.

Proposal 9: For a target modeled with multiple scattering points, the target channel for each scattering point are generated separately:

- One direct path and several indirect paths are generated for each scattering point.

- LOS condition is generated separately for each scattering point.

- Pathloss is generated separately for each scattering point based on its LOS condition and location.

- Spatial consistency are applied for channels of multiple scattering points.

Concatenation

Agreement - To generate indirect paths of NLOS ray + NLOS ray in the target channel - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4 - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2, - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1 - Other methods are up to company choice for complexity reduction - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration - The power threshold for path dropping is X=[-25] dB - X is relative to the strongest indirect path in the target channel - FFS: further power normalization of target channel is performed after path dropping, - Note: power normalization when target channel and background channel are combined can be discussed separately - FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

To illustrate the impact of indirect path dropping on target channel, we provide the circulate angle spreads based on Option 0 and Option 3 in case 4. Figure 8-1 shows the circulate angle spread CDF curves in UMi scenario, and Figure 8-2 provides the circulate angle spread CDF curves in Indoor office scenario. It can be seen from Figure 8-1 and Figure 8-2 that circulate angle spread (including AOD spread, AOA spread, ZOD spread and ZOA spread) CDF curves of Option 3 is very close to that of Option 0 in both UMi and Indoor office scenarios. It validates that there is almost no impact of indirect path dropping of Option 3 on target channel compared with Option 0.

(a) circulate AOD spread CDF                           (b) circulate AOA spread CDF

(c) circulate ZOD spread CDF                           (d) circulate ZOA spread CDF

Figure 8-1 Circulate angle spread CDF in UMi scenario

(a) circulate AOD spread CDF                           (b) circulate AOA spread CDF

(c) circulate ZOD spread CDF                           (d) circulate ZOA spread CDF

Figure 8-2 Circulate angle spread CDF in Indoor office scenario

Observation 1: The circulate angle spreads (including AOD spread, AOA spread, ZOD spread and ZOA spread) of Option 3 is almost same as that of Option 0 for sensing channel in case 4 in both Umi and Indoor office scenarios.

In contrast, if power is normolised after paths dropping, the power of each of the remaining NLOS + NLOS path will be increased after power normalization, which will severely degrade sensing performance. Therefore, power normalization should not be supported safter paths dropping.

Proposal 10: Power normalization is not needed after paths dropping

To check the number of clusters for target channel, some RT simulations are provided here. We put a target car in the well-known Manhhatan city model to compare the number of paths of the two cases in UMi scenario with bi-static sensing mode.

- Case 1: the number of rays not passing through the target

- Case 2: the number of rays indirectly passing through the target

Figure 8-3: Bi-static sensing in UMi scenario of Mahhantan city model

In the simulation, the rays of which power 25dB lower than the maximum power of the ray passing through the target are neglected for simplicity. It can be seen that even in such a complex UMi environment, compared with the number of total rays, the number of rays passing through the target is much less.

Figure 8-4: Clusters related to sensing target marked in yellow

Based on the above RT simulation shown in the above figure, the total number of NLOS clusters is about three times of the number of NLOS clusters passing through the target. Therefore, we have the following proposal:

Proposal 11: For single scattering point modeling, the number of NLOS clusters is 13 of that defined in 38.901

  - Each NLOS cluster includes 20 rays as the same as defined TR 38.901

In terms of the pathloss/shadow application on the ISAC target channel, we support it should be added in the last step generating target channel coefficient, e.g.,

Htarget,u,s=σRCS,APLd1∙PLd2∙c24πf2∙SF1∙SF2∙Hu,s(k,p)τ,t

This application method of the pathloss/shadow on the ISAC channel model follows that on the communication channel, which is a natural and simple solution. On the contrary, if the pathloss/shadow is applied to each of Tx-target and target-Rx link before concatenating the links, the computation is increased, whereas the effect is totally same as the application in the last step.

Absolute delay model

Agreement A single direct path is modeled for a scattering point of target - In each of the Tx-target and target RX links, the first NLOS cluster is generated with same delay as the LOS ray (when the absolute delay modelling of Δτ as in section 7.6.9, TR 38.901 is not applied) and with the same direction as the LOS ray. - FFS how to generate NLOS cluster when Δτ is applied

The absolute time delay is used to model the total propagation path length of the cluster, which is applied in all the NLOS clusters. It helps to differentiate the LOS ray and different NLOS paths in time domain, which is significant to simulate the real propagation in simulation. Additionally, the absolute time delay model is beneficial to the simulation performance, especially for the use case of intrusion, location and tracking. Thus, in our view, it is necessity to model the absolute time delay for ISAC channel model. Currently, 3GPP TR 38.901 has specified the absolute delay model for communication model for Indoor factory scenario, which can be reused for the absolute delay model in ISAC channel model. For other scenarios such as UMi, UMa and Indoor office scenario, the absolute time delays are discussed in the study item of the 7-24G channel modeling and the parameters are shown in Table 9-1. The parameters of the absolute time delays in the current TR and concluded in the study item of the 7-24G channel modeling can be reused for ISAC channel model.

Table 9-1 Parameters for the absolute time of arrival model

| Scenarios |  | InF-SL, InF-DL | InF-SH, InF-DH | InH | UMi | UMa |
| --- | --- | --- | --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.5 | -7.9 | -7.5 | -7.4 |
|  | σlgΔτ | 0.4 | 0.4 | 0.3 | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 6 | 11 | 10 | 15 | 50 |

The absolute time delay should be applied for all NLOS clusters including the first cluster, like the application of the absolute time delay in communication channel model. In such case, LOS ray can be detected from the NLOS clusters, which can improve the sensing performance to some extent. If the absolute time delay only uses for stochastic clusters except the first NLOS cluster, the first NLOS cluster and the LOS ray will be mixed in time domain. Since the first NLOS cluster generally has a large power, it may result in interference for target detection, which will degrade the sensing performance in simulation and not aligned with the reality.

Proposal 12: For the absolute time delay modeling, the following are supported:

- The absolute time delay Δτ is necessary for ISAC channel model.

- The absolute time delay Δτ should be used to all the stochastic clusters including the first NLOS cluster.

Spatial consistency

In TR 38.901 clause 7.6.3, the spatial consistency procedure and parameter generation method are described. The motivation is to model the channel spatial consistency between a TRP and different UEs especially which are close to each other. According to the characteristics of different cluster specific parameters and ray specific parameters, the following types on spatial consistency are defined:

- Site-specific: parameters for different BS-UT links are uncorrelated, but the parameters for links from co-sited sectors to a UT are correlated.

- All-correlated: BS-UT links are correlated.

Table 10-1 Correlation type among TRPs

| Parameters | Correlation type |
| --- | --- |
| Delays | Site-specific |
| Cluster powers | Site-specific |
| AOA/ZOA/AOD/ZOD offset | Site-specific |
| AOA/ZOA/AOD/ZOD sign | Site-specific |
| Random coupling | Site-specific |
| XPR | Site-specific |
| Initial random phase | Site-specific |
| LOS/NLOS states | Site-specific |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

For ISAC with bi-static sensing mode, besides the spatial consistency between Tx and Rx for background channel, the correlation of those random parameters should be considered for the link from Tx to ST, as well as the link from ST to Rx.

Figure 10-1 Spatial consistency consideration for ISAC links

As shown in the above figure, the spatial consistency for ISAC channel should be considered for the following links

- (1) The link from Tx1 to Rx1 and the link from Tx1 to Rx2, for spatial consistency of background channel

- (2) The link from Tx1 to Rx1 and the link from Tx1 to ST2, for spatial consistency between target and background channel

- (3) The link from Tx1 to ST1 and the link from Tx1 to ST2, for spatial consistency between Tx and different targets

- (4) The link from ST1 to Rx1 and the link from ST2 to Rx1, for spatial consistency between Rx and different targets

- (5) The link from ST2 to Rx1 and the link from ST2 to Rx2, for spatial consistency between different Rx and a target

Based on the above requirements, the following steps can be considered.

Step 1: a network grid is generated in site-specific or all-correlated manner as legacy for the links from TRP to ST, and from TRP to UE. Then, the above requirements (1) (2) (3) are satisfied.

As shown in the following figure, a network grid is generated in site-specific or all-correlated manner, and the random parameters, i.e. P1, P2 ... P9 are generated in vertices of each square as the same as legacy where the length of each square is the correlation distance.

- For the link 1 or link 2 from TRP to UE1 or to UE2, the random parameters are based on the interpolation according to the positions of UE1 or UE2 and random values P4, P8, P9 and P6.

- For the link 11 or link 12 from TRP to ST1 or to ST2, the random parameters are based on the interpolation according to the positions of ST1 or ST2 and random values P2, P7, P4 and P8.

Based on this procedure, the spatial consistency between sensing targets is realized for the links from TRP to STs. Similarly, the spatial consistency between UEs is realized for the link from ST to UE. Also, the spatial consistency between the links from TRP to STs, and the links from TRP to UEs are realized.

Figure 10-2 ISAC spatial consistency with 1st grid as network grid

Step 2: a Rx-specific grid is generated for the links from ST to Rx. Then, the above requirement (4) is satisfied.

As shown in the following figure, a second UE-specific grid is generated.

- For the link 21 or link 22 from ST to UE1, the random parameters are based on the interpolation according to the positions of UE1.

Figure 10-3 ISAC spatial consistency with 2nd grid per Rx

Based on this procedure, the spatial consistency between sensing targets is realized for the links from different STs to Rx.

Step 3: Model correlation between the Rx-specific grids. Then, the above requirement (5) is satisfied

One possible way is that, all Rx-specific grids are generated based on a shift from a global grid.

As shown in the left figure as follows, the global grid is the same as the grid of UE1 where UE as Rx. For the grid of UE2, the locations of all UEs and STs are moved so that UE2 location is in the origin [0,0] of the global grid as shown in the right figure below. Basically, the two grids are the same, but the relative ST/UE location compared with the origin [0,0] is changed. Based on this, the spatial consistency between the link from ST1 to UE1 and the link from ST1 to UE2 can be obtained.

- Network grid                                        (b) UE-specific grid shifted by the network grid

Figure 10-4 The spatial consistency between link 11 and link 21

If TX is a UE instead of a TRP, more study is needed. In legacy sidelink communication and positioning channel modeling, there was no discussion and achievement on spatial consistency, probably because the spatial correlation between differetn Tx UEs are too small. For ISAC on UE bi-static or mono-static sensing modes, if spatial consistency is supported, the following items shoudl be further studied from our view:

- The correlation distance. Since UE height and distribution is different from TRP, the correlation distance should be much less than that of TRP. This needs much effort on real measurement / RT simulation for each scenario.

- The spatial consistency between Tx UEs. The legacy-liked mechanism, i.e. independent Tx UE specific grid cannot be simply reused because it lacks of correlation among Tx specific grid.

Proposal 13: For spatial consistency of ISAC channel modeling for TRP-UE bi-static sensing mode

- The legacy network grid generated in site-specific or all-correlated manner is reused for the links from TRP to ST, and from TRP to UE

- A Rx-specific grid is generated for the links from ST to Rx

  - The correlation of each Rx-specific grid should be modeled, i.e. all Rx-specific grids are generated based on a shift from a global grid

- FFS whether/how to model spatial consistency for UE bi-static or mono-static sensing modes including correlation distance and correlation between different Tx UEs.

Background for mono-static

Agreement On the background channel for TRP-TRP and UE-UE bistatic sensing mode, the large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as starting point - Update on values of the LSP/SSP parameters can be discussed based on validation data acquired by measurement or ray-tracing model - FFS The power threshold for removing clusters in step 6, i.e., -25 dB is revised to X<-25 dB. FFS X - FFS whether/how to resolve the inconsistency between TRP-TRP channel according to TR 38.858 and the TRP-target (UAV) channel according to TR 36.777 when UAV and TRP are set to same height

Options listed in FL summary in RAN1#119 Solution A: (previous Option 1) On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point - drop N reference points following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and each reference points for a scenario. - FFS the distribution and the mobility for reference points dropping - N is determined by validation results for each scenario, FFS value N Solution B (Previously option 2) - drop N ground clutters following certain distribution, generate the target channel for the N ground clutters, the sum of the N target channels is the background channel - only direct path is modeled for each ground clutter - FFS the number, the distribution, RCS, XPR, etc. for ground clutters

As shown in the above agreement, for background channel modeling in the case of bi-static sensing mode, the large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 have been agreed as the starting point.

In the current TR 38.901 for stochastic channel modeling between gNB and UE, the basic procedures are shown in the following figure, where Pathloss computation, AOA/AOD and d3D for LOS are based on the UE location in the scenario. Pathloss is usually increased according to d3D.

Figure 11-1 The procedures of TR 38.901

Assuming TR 38.901 framework can be reused also for mono-static sensing modes, the key issue is how to compute pathloss, angle for LOS reference, and d3D for the LOS reference. One approach is to drop a virtual Rx location, and further reuse the methodology and procedures specified in 3GPP TR 38.901. As shown in the following the figure, a virtual Rx location is introduced dedicated for background environment channel modeling in the case of BS mono-static sensing mode [9]. When dropping the virtual Rx locations, it is better to ensure the generated channel aligning with the background channel from the real measurement or Ray-tracing simulation.

Figure 11-2 Dropping a virtual Rx to generate background channel based on TR 38.901 framework for mono-static sensing mode.

In other words, the environment channel modeling can be generated between a BS Tx and the virtual Rx. The procedure of 3GPP TR 38.901 [2] can be reused then, for example, the pathloss computation is based on the setting scenario, the height of the virtual Rx, and distance between BS Tx and the virtual Rx. For different scenarios, for example, UMa, or UMi, or Indoor, the location dropping of virtual Rx per BS or per sector could be independent.

Figure 11-3 Virtual Rx to generate Hbackground

As shown in the above figure, the only issue is to study how to drop the virtual Rx for each sector or gNB, where the channel characteristic between the Tx and the virtual Rx shown in the right figure should well reflect the channel of gNB mono-static sensing shown in the left figure.

Hence, the above solution A is preferred. In the remaining sub-sections, we provide our RT/measurement results to validate the solution A, and also show our concern on the solution B.

Proposal 14: On the background channel for monostatic sensing mode, support Solution A, i.e. dropping N reference points following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and each reference points for a scenario.

Validation on Solution A

    - Ray-tracing simulation based validation on solution A

To verify the feasibility, a 500m x 500m urban area (UMa scenario) in Washington D.C. is selected as the Ray-tracing (RT) simulation environment to evaluate the positioning of virtual Receivers (RX) in the scenario. Specifically, mono-static sensing transceiver with 25m height is randomly dropped outdoor for 1000 times, and each drop time is assigned with eight random yaw angles to expand drops. In each time, all rays with strength larger than strongest ray power minus 25dB are selected as valid rays. All AoD, ZoD, delay and pathloss information of valid rays can be seen as Monte-Carlo from RT-simulation world and describe RT-simulated distribution for pathloss, delay, AoD and ZoD specifically, which are used to fit the distribution of location of virtual Rx.

Meanwhile, virtual Rx is randomly dropped for 1000 times, and in each time, pathloss, a list of delay (for N clusters), a list of AoD, and a list of ZoD are generated based on formula defined in TR 38.901 and dropped locations of virtual Rx. Then all Monte-Carlo samples of pathloss, delay, AoD and ZoD could describe sampled distribution for pathloss, delay, AoD and ZoD specifically. We modify the distribution of virtual Rx, trying to make the sampled distribution fitting the RT-simulated distribution.

Figure 11-4 Map of selected UMa scenario in Washington D.C

Figure 11-5 3D model of buildings in selected area

As discussed in RAN1#119, during the channel modeling, the omnidirectional antenna in horizontal domain could be used to reflect the comprehensive channel features distributed in environment. The antenna pattern is defined in TR 38.802 as following table.

Table 11-1: Radiation power pattern of a single omnidirectional antenna element

| Parameter |  | Values |
| --- | --- | --- |
| Single sector | Antenna element vertical radiation pattern (dB) |  |
|  | Antenna element horizontal radiation pattern (dB) |  |
|  | Combining method for 3D antenna element pattern (dB) |  |
|  | Maximum directional gain of an antenna element GE,max | 5dBi |
| Carrier frequency |  | 6GHz |

Using the above omnidirectional antenna pattern, the background clusters could come from any buildings structure located at any horizontal direction of Tx. Thus there is no meaning to limit the azimuth angle of virtual Rx related to Tx, which means the LOS AOD from Tx perspective could be uniformly selected from  to .

Figure 11-6 The topology relationship between virtual Rx and Tx

The distribution of location of virtual Rx is defined by parameterized Gamma distribution with the method of gradient descent. The fitted parameters are shown as follows.

Figure 11-7 Fitted gamma distribution

After getting virtual Rx distribution, the channel generation procedure in TR 38.901 can be reused between the real Tx and virtual Rx. As shown in Figure 11-6, in order to make background clusters could come from any buildings located at any horizontal direction, e.g. even behind of the bore-sight of the Tx antenna, the parameter ASD in TR 38.901 can be modified to fit the multipath clusters from any direction of Tx. An offset to mean value of ASD is derived after parameters fitting:

Table 11-2: Fitted ASD parameters for generating background channel

| AOD spread (ASD) in UMa NLOS scenario lgASD=log10(ASD/1) | lgASD | 1.5 - 0.1144 log10(fc) + 0.7138 |
| --- | --- | --- |
|  | lgASD | 0.28 |

Based on the fitted distribution, a virtual Rx could be dropped following the distribution, and the related sampled distribution for pathloss, delay, AoD and ZoD could be generated based on dropped virtual Rx and step 5 to step 7 defined in TR 38.901 clause 7.5. All parameters are generated in NLOS condition specified in TR 38.901 and the height of virtual Rx is forced to .

CDF of sampled distribution and RT-simulated distribution is visualized as in following figures. In the left figures, the black curve is derived from all valid rays of multiple iterations in RT simulation, and the red curve is derived from generated parameters based on fitted virtual Rx. To derive the statistical PDF of sampled distribution and RT-simulated distribution, a histogram is calculated for each parameter, and an envelope curve is generated as statistical PDF curve based on all centers of histogram bins.

| - CDF of pathloss | - PDF of pathloss |
| --- | --- |
| - CDF of Delay | - PDF of Delay |
| - CDF of AoD | - PDF of AoD |
| - CDF of ZoD | - PDF of ZoD |

Figure 11-8 CDF and PDF of sampled distribution and RT-simulated distribution

During the simulation for 8000 times, we totally collect 158627 paths. In average, for one simulation, the averaged number of existing clusters is 19.828, which is very close to the configuration of 20 (number of clusters) in the UMa NLOS scenario we used in TR38.901. We randomly select RT simulated results and generated results according to virtual RX in one iteration to draw the following scattering points figure. Each scattering point is plotted based on the delay times speed of light and AOD of clusters.

Figure 11-9 Scattering points of RT simulated clusters and generated clusters

It can be seen that the distribution of generated clusters according to virtual Rx could well fit the distribution from the RT simulated clusters. (Note: for stochastic geometry modeling, in one iteration, the location of scattering point may not overlap perfectly, we could only consider the good alignment for angular spread\delay spread\number of clusters in the above figure.)

    - Measurement based validation on solution A

Besides the RT simulation for UMa scenario, we did real measurement in Indoor-Office scenario to research the background channel for UE mono-static mode. The detailed configuration for measurement is shown as following table.

Table 11-3 Configuration for measurement

| Carrier frequency | 4.9GHz |
| --- | --- |
| Transmitting antenna | Horn antenna, vertical polarization, The horizontal 3dB beamwidth is 32.85°and the vertical 3dB beamwidth is 28.93° |
| Receiving antenna | Horn antenna, vertical polarization, The horizontal 3dB beamwidth is 32.85°and the vertical 3dB beamwidth is 28.93° |
| The height of transmitter | 1m |
| The height of receiver | 1m |
| Transmitting / receiving Gain of antenna | 12.5 dBi (The transmitting and receiving gain has been removed from the power of all measured paths) |

Figure 11-10 Antenna used for background measurement

In a typical indoor office scenario, we select 20 positions for measurement. In each measurement position point, the transmitting and receiving horn antenna could be rotated for 12 times in horizontal plane, to detect all multipath of background channel from all directions around the TRP. The height of transmitter and receiver is set as 1m, which is same as the configuration for UE in TR 38.802 / TR38.901. The environment for measurement and the position points selected are shown in following figures:

Figure 11-11 Measurement environment and selected points for measurement

In the measurement campaign, there is only one horn antenna in the transmitter/receiver. Thus the angular information is lost during the measurements.

Based on the measurement, we could obtain the pathloss information in each point, and power and delay information of each path, which could be used to fit the distribution of virtual Rx (just ignore the effect of AoD and ZoD in the fitting temporarily)

We also use the omnidirectional antenna pattern for Indoor Office scenario for fitting. The background clusters could come from any environmental objects located at any horizontal direction of Tx. The LOS AOD of Tx could be uniformly selected from to ，just as described in section 11.1.1.

The distribution of location of virtual Rx is defined by parameterized Gamma distribution with the method of gradient descent. The fitted parameters are shown as follows.

Figure 11-12 Fitted gamma distribution

The fitted results for Pathloss and delay are shown in the following figures.

| - CDF of pathloss | - PDF of pathloss |
| --- | --- |
| - CDF of Delay | - PDF of Delay |

Figure 11-13 CDF and PDF of sampled distribution and measurement distribution

It could be found that the results from virtual Rx based solution can aslo fit well in Indoor Office scenario. It is proved taht the distribution of distance and height of virtual Rx could change with the scenarios, to fit background channel in any scenario.

Proposal 15: For Hbackground modelling in the case of mono-static sensing mode, a virtual Rx location can be randomly generated, and then the stochastic mechanism of TR 38.901 between the real Tx and the virtual Rx is reused.

- Gamma distributions can be used to generate the distance between Tx/Rx and the virtual Rx, and to generate the virtual Rx height

- The virtual Rx is uniformly dropped in the horizontal domain for each TRP.

- ASD parameter in TR 38.901 can be updated to make background clusters in any horizontal directions.

  - ASA generation is not needed where ASA is enforced to be the same as ASD

Proposal 16: In UMa scenario, the Gamma distribution of distance between Tx/Rx and the virtual Rx, and the virtual Rx height could be given by

The extended mean value of log-normal distribution for ASD could

Proposal 17: In Indoor-Office scenario, the Gamma distribution of distance between Tx/Rx and the virtual Rx, and the virtual Rx height could be given by

Concerns on Solution B

    - Compared with RT simulation results

In the UMa scenario described in subsection 11.1.1, with the RT-simulation results, we also assess the stochastic ground clutter model proposed in RAN1#119 meeting, i.e. solution B [18]. In the following assessment, we set a base station at the center point of a ground grid. The height of base station is set as 25m to simulate the UMa scenario. The total areas of the square grid are 100x100 m2 and 250x250 m2 in two simulation cases respectively, where the grid size is equal to 5m.

In each grid point, it is supposed there is one point target, with RCS in dB value obeying the Gaussian distribution of . The base station could transmit signal uniformly toward all directions around the base station. Thus mono-static background is modeled as the sum over the effect of grid points, where the contribution by each grid point is calculated using the radar equation. The carrier frequency used is 6GHz.

When the length of square grid is set as 100m, the fitted results of stochastic ground clutter model compared with the RT-simulated channel is shown as following figures.

| - CDF of pathloss | - PDF of pathloss |
| --- | --- |
| - CDF of Delay | - PDF of Delay |
| - CDF of AoD | - PDF of AoD |
| - CDF of ZoD | - PDF of ZoD |

Figure 11-14 CDF and PDF of stochastic ground clutter model (length of grid equal to 100m) and RT-simulated distribution

When the length of square grid is set as 250m, the fitted results of stochastic ground clutter model compared with the RT-simulated channel is shown as following figures.

| - CDF of pathloss | - PDF of pathloss |
| --- | --- |
| - CDF of Delay | - PDF of Delay |
| - CDF of AoD | - PDF of AoD |
| - CDF of ZoD | - PDF of ZoD |

Figure 11-15 CDF and PDF of stochastic ground clutter model (length of grid equal to 250m) and RT-simulated distribution

From the above simulation results, the following concerns on the stochastic ground clutter model, i.e. Solution B, are observed:

- The pathloss based on Solution B is much smaller than the real values.

- The delay based on the solution B is much smaller and concentrated compared with the real values. That is because all multipaths are derived by grid points. The delay of multipaths is limited by size of grid, which could not be set as too large, causing meaningless memory wasting. For example, if a square grid with length of 500m is generated with grid size of 5m, there would be 10000 target points to be generated, occupying too much computing resources only for background channel. (Much larger than the 400 rays (20 clusters and 20 rays for each cluster) defined in TR 38.901)

- The distribution of AoA (which is same as AoD) is not uniform distribution when the square grid is used. That’s because the numbers of grid points in all directions are not the same. The detailed shape of grid must be researched with more details for fitting the background.

- The distribution of ZoA (which is same as ZoD) highly depends on the size of grid.

In addition, there are many details to be solved for this Solution B, for example, CPM for each background clutter, 3D clutter distribution besides the ground clutter, etc.

    - Compared with measurement

We also research the fitting accuracy of stochastic ground clutter model, i.e. Solution B in measured Indoor office scenario. All the measurement results come from the experiment described in subsection 11.1.2. During generating background channel using the stochastic ground clutter model, we set a Tx/Rx at the center point of a ground grid. The height of Tx/Rx is set as 1m. The length of square grid is set 100m, with the grid size equal to 5m. In each grid point, RCS in dB value obeys the Gaussian distribution of . The carrier frequency used is 4.9GHz, as the configuration of measurements.

The fitted results of stochastic ground clutter model compared with the measured background channel are shown as following figures.

| - CDF of pathloss | - PDF of pathloss |
| --- | --- |
| - CDF of Delay | - PDF of Delay |

Figure 11-16 CDF and PDF of stochastic ground clutter model (length of grid equal to 100m) and measurement

It could be seen that the sampled distribution of pathloss is much different compared to measurement results. The sampled pathloss distribution is much wider than the actual measured pathloss in Indoor Office scenario.

Besides, the power profile of rays is lower than the real measurements, which is shown in the following figure:

Figure 11-17 Measurement PDP compared with stochastic ground clutter model

The above results show there are still a lot of issues to be researched in the stochastic ground clutter model, at least including:

- The shape of grid

- The longest length of grid

- Grid size

- RCS distribution of each grid point (mean value, standard deviation, and other parameters if Gaussian distribution is not used)

- The number of point target which would be dropped on the grid

- The distribution for dropping point target especially in vertical domain

- The location of sensing transmitter(s) and receiver(s)

- CPM

- Complexity issues because of large number of ground grids

- Most importantly, we observe that the stochastic ground clutter model could not fit both the measurement and RT-simultaneous well, when the above parameters and configurations are not provided in details per scenario. Currently, we have no idea on how to derive the above parameters for scenarios based on measurement results and simulation results.

- Observation 2: The background channel based on Solution B for mono-static sensing mode is not matched with the real measurement or RT results for UMa and Indoor office scenarios

- For UMa scenario,

  - The pathloss of stochastic ground clutter model is too small

  - The delay of stochastic ground clutter is too small and concentrated.

  - The distribution of AoA (which is same as AoD) is not uniform distribution when the square grid is used.

  - The distribution of ZoA (which is same as ZoD) highly depends on size of grid.

  - Large grid is too computing resource wasting to be used to generate background channel.

- For indoor scenario,

  - The pathloss distribution is much wider than the real measured pathloss

  - The power profile of rays is much lower than the real measurements

- Observation 3: There are still many open issues for the stochastic ground clutter model, i.e. Solution B, to be solved per scenario:

- The shape of grid

- The longest length of grid

- Grid size

- RCS distribution of each grid point (mean value, standard deviation, and other parameters if Gaussian distribution is not used)

- The number of point target which would be dropped on the grid

- The distribution for dropping point target especially in vertical domain

- The location of sensing transmitter(s) and receiver(s)

- CPM

- Complexity issues because of large number of ground grids

- Observation 4: Until now, there are not any measurement results and simulation results to valid the method to derive the parameters for stochastic ground clutter model. Companies should at least provide a method to obtain the parameters to show the potential feasibility for stochastic ground clutter model to fit with actual background channel.

Power normalization between target and background

- Agreement - The following options are supported to generate the combined ISAC channel - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed. - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between - Alt 1: Power normalization on both target channel and background channel - Alt 2: Power normalization on background channel only - Alt 3: the target channel of a target will replace one cluster in the background channel - FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2 - FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

In our view, there is no need to perform power normalization between target channel and background channel, although both Option 1 and Option 2 have been agreed. It is noted that the pathloss formula was based on the real measurements carried for NR channel modeling in Rel-14 around 2015. In that time, the sensing targets such as UAV, vehicles, AGV, etc., were not considered. On the background channel, if a sensing target newly comes in, the total receiving power should be slightly increased correspondingly, especially when there is direct path in the target channel link Tx-ST-Rx, and the RCS gain is large. This is similar as passive IRS (Intelligent Reflecting Surface) which is used to boost the communication power. Thus, power normalization between target channel and background channel is unnecessary.

Figure 12-1 An illustration of a target in the background

Furthermore, target channel power especially for the direct path should be independent from the background channel. If normalization is done between target channel and background channel, this would lead to different direct path power in different scenarios. For example, for a target channel with direct path of a UAV, the receiving power at Rx in UMi scenarios and RMa scenario will be different after normalization because the background channel is different. This would not be aligned with the reality. So the Option 2 with Alt 1 is not reasonable.

For Alt 3 in Option 2, although the number of the clusters in background channel can be remained unchanged, the power of target channel is very different from that of any clusters of background channel. Just using target channel to replace one cluster of the background channel can’t guarantee the power constant of the background. Besides, the sensing target is modeled with several stochastic clusters, which cluster of the sensing target is chosen to replace the one cluster of background is a problem. Thus, Alt 3 of Option 2 should not be adopted.

Proposal 18: Option 1 is preferred: power normalization between target channel and background channel is not performed.

EO type 2

Agreement When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference - As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00−R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901) - FFS any update to b,R∥specular,R⊥specular - FFS any update taking EO orientation into account - FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size - FFS whether diffraction and scattering can be considered in addition to specular reflection - EO type-2 is an optional modelling component if supported in a sensing scenario - FFS which deployment scenario(s) EO type-2 will apply

We research the reflection model of EO type-2, and the academic works based on Maxwell equation shown that the reflection model could be described clearly based on plane of Incidence, containing the incident ray and the normal vector to the surface. The incidence electric wave should be split to the component perpendicular to the plane of incidence, and component parallel to the plane of incidence. For the ground reflection model, it is fortunately to see the vertically polarized wave as the component parallel to the plane of incidence and horizontal polarized wave as the component perpendicular to the plane of Incidence directly. This is the reason that we can give one matric between the antenna VH gain of Tx and Rx directly. However, for EO type-2 such as wall of buildings typically, which is perpendicular to the ground, we can’t easily describe the relationship between perpendicular / parallel component to the plane of incidence and V/H polarized wave using a simple diagonal matrix. Thus, it is hard to model the effect of EO type-2 with one matrix. We suggest that the effect of EO type-2 could be modeled as following equation.

In which the  and describe ZoD and AoD from Tx to the reflected point on the EO-type2, and  and  describe ZoA and AoA from Rx to the reflected point. The matrix  is used to transform the VH polarized components to the incident parallel / perpendicular components related to the plane of incidence. The matrix  is used to transform the reflected parallel / perpendicular components to VH polarized components of Rx. The matrix  describe the reflected coefficient of parallel / perpendicular components. All parameters in the above equation could be derived as following steps.

Step 1: Calculated the reflected point

The EO type-2 could be described as any plane equation perpendicular to the ground. Considering the EO-type2 is defined as a reflection plane perpendicular to ground, the EO-type2 could be visualized as the Figure 13-1.

Figure 13-1 illustration of EO-type2

Thus we assume the plane equation of EO type-2 is known as

.

in which the actual value of A and B could be given by

The location of Tx and Rx is also known as  and . The distance between Tx and EO is .

The reflected image of Tx is  and

Based on the reflected image of Tx and the plane equation of EO, the location of reflected point could be obtained as  and

If the reflected point is on the EO, which is , the EO is determined as activated to create a reflected path. Otherwise, the EO is not modeled for this Tx-Rx pair.

Step 2: Calculated the ZoD and AoD from Tx to the reflected point on the EO-type2

The ZoD and AoD from Tx to the reflected point could be derived based on the location of Tx and reflected point.

Step 3: Calculated the rotation angle  and the rotation matrix of .

To reuse the definition of reflected coefficients  and , we should rotate the LCS to a new coordinate system whose polar basis vector of the spherical basis vectors is parallel to the plane of incidence, and azimuth basis vector of the spherical basis vectors is perpendicular to the plane of incidence. Thus we should rotate the current polar basis vector of incident ray with  and  to make the new rotated polar basis vector on the plane of incidence.

Firstly, the polar basis vector  and azimuth basis vector before rotating could be described in cartesian coordinate system.

The V polarized wave is transmitted along the direction of  and the H polarized wave is transmitted along the direction of .

The normal vector of plane of incidence could be described a cross product , in which

and

Thus we assume an anticlockwise rotation angle  is used to polar basis vector  and azimuth basis vector with the rotation axis as incidence wave from Tx to reflected point. The anticlockwise rotated polar basis vector and rotated azimuth basis vector could be described as

We want to get the anticlockwise rotation angle  making the new rotated polar basis vector is parallel to the plane of incidence, which means the new rotated polar basis vector is perpendicular to normal vector of plane of incidence, which is

Thus the anticlockwise rotation angle  could be derived as

Thus the rotation matrix could be derived by .

Figure 13-2 Anticlockwise rotate with axis of incident direction⊙

Step 4: Calculated the incidence angle  and related reflection coefficients,

The formula of reflection coefficients  and  could be reused, and only the incidence angle should be modified. Here the incidence angle  is the angle between the incident vector from Tx to reflected point and the normal vector, which is

Thus keep the reflection coefficients defined in TR38.901 unchanged, and use the new incidence angle .

Step 5: Calculated the ZoA and AoA from Rx to the reflected point on the EO-type2]

The ZoA and AoA from Rx to the reflected point could be derived based on the location of Rx and reflected point.

Step 6: Calculated the rotation angle  and the rotation matrix of .

Similar as Step 4, the reflection component parallel and perpendicular to the plane of incidence could be generated by rotating VH wave from Rx to reflection point on EO-type2.

Firstly, the polar basis vector  and azimuth basis vector, which is the united direction vector of V&H wave from Rx to reflection point on EO-type2, could be described in cartesian coordinate system.

Thus we assume a anticlockwise rotation angle  is used to polar basis vector  and azimuth basis vector with the rotation axis as incidence wave from Rx to reflected point. The anticlockwise rotated polar basis vector and rotated azimuth basis vector could be described as

We want to get the anticlockwise rotation angle  making the new rotated polar basis vector is parallel to the plane of incidence, which means the new rotated polar basis vector is perpendicular to normal vector of plane of incidence, which is

Thus the anticlockwise rotation angle  could be derived as

Thus the rotation matrix could be derived by .

Figure 13-3 Anticlockwise rotate with axis direction ⊙ from Rx to reflected point on EO-type2

Step 7: Combine rotation equation and get the final fast fading model of EO-type2

Given the above discussion, we have the following geometry relationships:

- The anticlockwise rotation equation of incident wave from Tx to EO-type2

- The anticlockwise rotation equation of incident wave from Rx to EO-type2

As a reverse operation, the clockwise rotation equation of departure wave from EO-type2 to Rx

- The relationship of departure wave

The reflection coefficients  and  describe the relationship between the strength of electrical wave along with  and , and the strength of electrical wave along with  and . However, the above equations use the base vector of and . Thus we should notice the following relationship.

The whole transformation relationship could be described as Figure 13-4.

Figure 13-4 The transformation relationship from Tx to EO and from EO to Rx

Therefore, the fast fading channel model of EO-type2 could be described as

Here a clockwise rotation is used, which has not been defined in TR 38.901. Thus we define , and the final fast fading channel model of EO-type2 could be described as

Proposal 19: Endorse the above step 1-7 for EO type 2 into TR for ISAC channel modeling, where the channel coefficient for the EO reflected path is given by

LLS

ISAC link-level simulation (LLS) is crucial for comparative assessment of technical solutions such as waveform/signal design and reception processing algorithms, with a focus on the performance of single-link target detection and parameter estimation. This type of simulation particularly focuses on the performance of single-link target detection and parameter estimation. Link-level simulation can model various effects that actual signals undergo during propagation and reception, such as attenuation, multipath, and noise interference, thus providing essential insights for system design and optimization.

In terms of performance assessment, LLS can evaluate the performance of different technical solutions in target detection and parameter estimation, providing decision support for system design. In terms of parameter and system optimization, LLS can optimize system parameters, such as transmission power, reception sensitivity, and signal processing algorithm parameters, to achieve the best system performance. By simulating different signal designs and processing algorithms, LLS can investigate how to improve the signal's interference resistance or optimize the bandwidth utilization. Therefore, it plays a significant role in modeling ISAC link-level systems.

Regarding ISAC LLS, we think leveraging the CDL channel model from TR 38.901 is a strategic decision that should be upheld, ensuring continuity and efficiency in the simulations whenever feasible. This approach streamlines our efforts, allowing us to concentrate on the critical aspects of the system's performance. That is, the extension of CDL channel modeling is suggested for ISAC LLS where the evaluation metrics are the accuracy estimation of angle, delay, and Doppler. Like as traditional CDL model for communication, there is no need to explicitly model gNB, UE, and target locations in the LLS. Thus, it is also unnecessary to explicitly consider different sensing modes.

Basically, there are two methods to model the channel of ISAC LLS:

- Method 1: Select one or more clusters of the existing CDL channel models to serve as the sensing clusters corresponding to the sensing target. Then give Doppler or velocity parameters for these sensing clusters.

- Method 2: Newly generate time delay, power, angle, and velocity parameters for the sensing clusters based on the number of sensing targets. These parameters can be set or randomly generated.

Based on the CDL-B in TR 38.901, there are two examples as following:

Table 14-1. CDL-B based on Method 1

| Cluster # | Normalized delay |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] | Velocity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0000 |  | 0 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 2 | 0.1072 |  | -2.2 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 3 | 0.2155 |  | -4 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 4 | 0.2095 |  | -3.2 |  | -34.1 |  | 125.5 |  | 115.3 |  | 63.3 | 0 |
| 5 | 0.2870 |  | -9.8 |  | -65.4 |  | -88.0 |  | 119.3 |  | 59.9 | 0 |
| 6 | 0.2986 |  | -1.2 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 7 | 0.3752 |  | -3.4 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 8 | 0.5055 |  | -5.2 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 9 | 0.3681 |  | -7.6 |  | -67.2 |  | -89.8 |  | 118.2 |  | 82.6 | 0 |
| 10 | 0.3697 |  | -3 |  | 52.5 |  | 132.1 |  | 102.0 |  | 66.3 | 0 |
| 11 | 0.5700 |  | -8.9 |  | -72 |  | -83.6 |  | 100.4 |  | 61.6 | 0 |
| 12 | 0.5283 |  | -9 |  | 74.3 |  | 95.3 |  | 98.3 |  | 58.0 | 0 |
| 13 | 1.1021 |  | -4.8 |  | -52.2 |  | 103.7 |  | 103.4 |  | 78.2 | 0 |
| 14 | 1.2756 |  | -5.7 |  | -50.5 |  | -87.8 |  | 102.5 |  | 82.0 | 0 |
| 15 | 1.5474 |  | -7.5 |  | 61.4 |  | -92.5 |  | 101.4 |  | 62.4 | 0 |
| 16(sensing cluster) | 1.7842 |  | -1.9 |  | 30.6 |  | -139.1 |  | 103.0 |  | 78.0 | 20 |
| 17 | 2.0169 |  | -7.6 |  | -72.5 |  | -90.6 |  | 100.0 |  | 60.9 | 0 |
| 18 | 2.8294 |  | -12.2 |  | -90.6 |  | 58.6 |  | 115.2 |  | 82.9 | 0 |
| 19 | 3.0219 |  | -9.8 |  | -77.6 |  | -79.0 |  | 100.5 |  | 60.8 | 0 |
| 20 | 3.6187 |  | -11.4 |  | -82.6 |  | 65.8 |  | 119.6 |  | 57.3 | 0 |
| 21 | 4.1067 |  | -14.9 |  | -103.6 |  | 52.7 |  | 118.7 |  | 59.9 | 0 |
| 22 | 4.2790 |  | -9.2 |  | 75.6 |  | 88.7 |  | 117.8 |  | 60.1 | 0 |
| 23 | 4.7834 |  | -11.3 |  | -77.6 |  | -60.4 |  | 115.7 |  | 62.3 | 0 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |  |
| Value |  | 10 |  | 22 |  | 3 |  | 7 |  | 8 |  |  |
| Value(sensing clusters) |  | 2 |  | 2 |  | 2 |  | 2 |  | 8 |  |  |

Table 14-2. CDL-B based on Method 2

| Cluster # | Normalized delay |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] | Velocity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0000 |  | 0 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 2 | 0.1072 |  | -2.2 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 3 | 0.2155 |  | -4 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 4 | 0.2095 |  | -3.2 |  | -34.1 |  | 125.5 |  | 115.3 |  | 63.3 | 0 |
| 5 | 0.2870 |  | -9.8 |  | -65.4 |  | -88.0 |  | 119.3 |  | 59.9 | 0 |
| 6 | 0.2986 |  | -1.2 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 7 | 0.3752 |  | -3.4 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 8 | 0.5055 |  | -5.2 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 9 | 0.3681 |  | -7.6 |  | -67.2 |  | -89.8 |  | 118.2 |  | 82.6 | 0 |
| 10 | 0.3697 |  | -3 |  | 52.5 |  | 132.1 |  | 102.0 |  | 66.3 | 0 |
| 11 | 0.5700 |  | -8.9 |  | -72 |  | -83.6 |  | 100.4 |  | 61.6 | 0 |
| 12 | 0.5283 |  | -9 |  | 74.3 |  | 95.3 |  | 98.3 |  | 58.0 | 0 |
| 13 | 1.1021 |  | -4.8 |  | -52.2 |  | 103.7 |  | 103.4 |  | 78.2 | 0 |
| 14 | 1.2756 |  | -5.7 |  | -50.5 |  | -87.8 |  | 102.5 |  | 82.0 | 0 |
| 15 | 1.5474 |  | -7.5 |  | 61.4 |  | -92.5 |  | 101.4 |  | 62.4 | 0 |
| 16 | 1.7842 |  | -1.9 |  | 30.6 |  | -139.1 |  | 103.0 |  | 78.0 | 0 |
| 17 | 2.0169 |  | -7.6 |  | -72.5 |  | -90.6 |  | 100.0 |  | 60.9 | 0 |
| 18 | 2.8294 |  | -12.2 |  | -90.6 |  | 58.6 |  | 115.2 |  | 82.9 | 0 |
| 19 | 3.0219 |  | -9.8 |  | -77.6 |  | -79.0 |  | 100.5 |  | 60.8 | 0 |
| 20 | 3.6187 |  | -11.4 |  | -82.6 |  | 65.8 |  | 119.6 |  | 57.3 | 0 |
| 21 | 4.1067 |  | -14.9 |  | -103.6 |  | 52.7 |  | 118.7 |  | 59.9 | 0 |
| 22 | 4.2790 |  | -9.2 |  | 75.6 |  | 88.7 |  | 117.8 |  | 60.1 | 0 |
| 23 | 4.7834 |  | -11.3 |  | -77.6 |  | -60.4 |  | 115.7 |  | 62.3 | 0 |
| sensing cluster | 0.3681 |  | -3 |  | 52.5 |  | 102.1 |  | 102.0 |  | 66.3 | 20 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |  |
| Value |  | 10 |  | 22 |  | 3 |  | 7 |  | 8 |  |  |
| Value(sensing clusters) |  | 2 |  | 2 |  | 2 |  | 2 |  | 8 |  |  |

For the above two methods, the generation of channel coefficients and other processes will continue to follow the methodology of SLS.

For Method 1, the CDL channel from TR 38.901 is essentially used in its entirety, with relatively minor modifications. For Method 2, additional random clusters corresponding to the target are generated, and the existing CDL channel can be considered as the background channel, aligning with the system-level simulation channel currently under standardization.

Proposal 20: Using the CDL channel as the starting point for research on ISAC LLS.

Hybrid channel modelling with RT simulation

Hybrid model based on ray tracing simulation is composed of a deterministic component and a stochastic component. In TR 38.901, the hybrid system is designed to evaluate or predict the impacts for communication from environmental structures and materials with the use of digital map. The sensing task makes a requirement for a more accurate and deterministic sensing channel.

For sensing with hybrid channel modelling, the sensing channel related to targets and some environmental objects affecting sensing targets severely should be modelled by ray-tracing method with a higher determinacy and predictability. The environmental channels related background wireless environment, such as ground, woods, buildings, and other things, can be assumed as random clutters and be modelled by stochastic geometry modelling method in TR 38.901 without any changes.

Figure 15-1 Hybrid Channel coefficient generation procedure

Basically, the procedures in TR 38.901 section 8 can be reused with slight modification. This type of modeling is very useful for sensing from our view especially on the impact from the important environmental objects. To align companies’ simulation results, it is better to specify some typical scenarios, characteristics of sensing targets. In other words, digital maps in the simulation should be aligned for scenarios to be evaluated by method of ray-tracing, and some typical maps can be recommended to be researched first.

Based on the further details of analysis in section 3.1, 3.2 and 3.3, we have the following proposal

Proposal 21: The procedure of hybrid channel modelling with RT simulation in TR 38.901 can be reused and enhanced for sensing. Specify the following typical maps and characteristics of sensing targets to align the simulation assumptions:

- Urban grid map defined in 3GPP TR 37.885

- The well-known Manhattan map from open source

- Indoor map defined in IEEE 802.11 WLAN

- EM parameters defined for radar material by ITU

Outdoor map

For outdoor scenarios, we recommend two well-known maps as follows.

- Urban grid in TR 37.885 [4].

Parameters regarding the road configuration for urban grid has been included. Figure 15-2 shows illustrative diagrams of urban grid which is simple for RT simulation. Based on this map, the NLOS links from buildings to sensing targets, and then further to Rx/Tx can be deterministically modeled. Since the location and EM characteristics of buildings are known in the real deployments, such NLOS links may be helpful for sensing detection.

Figure 15-2 Road configuration for urban grid in TR 37.885

- Manhattan map

Manhattan map is a common used urban city model for evaluation of wireless network configuration, and also be recommended to be starting point of hybrid channel modeling method. The digital map of Manhattan can be download from open street map [5] (https://www.openstreetmap.org), which is an open source containing comprehensive maps of most countries, and models of buildings, water objects, forests, roads, railways are all provided by open street map. For simplicity, only building models are proposed to be used for hybrid channel modeling.

For specified coordinate of latitude and longitude, a 500-meter square partial map of Manhattan with latitudes 40.7689 to 40.7736 and longitude -73.961 to -73.955 can be used. The detailed map is shown as follows.

Figure 15-3 Detailed map of part of Manhattan city

Figure 15-4 Digital map of Manhattan city

Indoor map

Indoor map defined in IEEE 802.11 WLAN can be reused in our view. Set up of the living room scenario is shown in Figure 15-5. The dimensions of a living room are 7 m x 7 m x 3 m (L x W x H). The living room has a table in the middle, a chair and a small chair with feet placed around the table. One outer wall has two windows and the opposite outer wall has a door placed in its middle. The target is a cubic object which is made of wood, as shown in Figure 15-6.

Figure 15-5 Set up of the living room scenario.

Figure 15-6 Target illustration and parameters.

The dimensions and materials of all furniture are given in Table 15-1.

Table 15-1 Parameters of the furniture dimensions and materials

| Object | Size | Material |
| --- | --- | --- |
| Chair | 2.8m x 1.05m x 1m (L x W x H) | wood |
| Chair with feet | 1.05m x 1.05m x 1m (L x W x H) | wood |
| Table1 | 1.5m x 0.7m x 0.8m (L x W x H) | wood |
| Table2 | 1m x 0.5m x 0.8m (L x W x H) | wood |
| Target | 0.3m x 1.5m (R x H) | wood |
| Door | 1m x 2.5m (W x H) | wood |
| Windows | 1.5m x 1.5m (W x H) | Glass |
| Wall |  | concrete |
| Floor |  | concrete |
| Ceiling |  | concrete |

EM parameters

For ray-tracing simulation, the radar material of object in digital map affect the simulation result a lot. The defined radar material by ITU is recommended to be reused [7] [8]. For each material, simple expressions for the frequency-dependent values of the real part of the relative permittivity, ', and the conductivity, , were derived. These are:

and:

where f is frequency in GHz and  is in S/m. (' is dimensionless.) The values of a, b, c and d are given in Table 15-2. Where the value of b or d is zero the corresponding value of  or  is a or c respectively, and independent of frequency.

If required, the imaginary part of the relative permittivity " can be obtained from the conductivity and frequency:

Parameters for air, metal and three conditions of ground are included in Table 15-2 for completeness.

Table 15-2 Material properties

| Material class | Real part of relative permittivity |  | Conductivity S/m |  | Frequency range |
| --- | --- | --- | --- | --- | --- |
|  | a | b | c | d | GHz |
| Vacuum (≈ air) | 1 | 0 | 0 | 0 | 0.001-100 |
| Concrete | 5.24 | 0 | 0.0462 | 0.7822 | 1-100 |
| Brick | 3.91 | 0 | 0.0238 | 0.16 | 1-40 |
| Plasterboard | 2.73 | 0 | 0.0085 | 0.9395 | 1-100 |
| Wood | 1.99 | 0 | 0.0047 | 1.0718 | 0.001-100 |
| Glass | 6.31 | 0 | 0.0036 | 1.3394 | 0.1-100 |
| Glass | 5.79 | 0 | 0.0004 | 1.658 | 220-450 |
| Ceiling board | 1.48 | 0 | 0.0011 | 1.0750 | 1-100 |
| Ceiling board | 1.52 | 0 | 0.0029 | 1.029 | 220-450 |
| Chipboard | 2.58 | 0 | 0.0217 | 0.7800 | 1-100 |
| Plywood | 2.71 | 0 | 0.33 | 0 | 1-40 |
| Marble | 7.074 | 0 | 0.0055 | 0.9262 | 1-60 |
| Floorboard | 3.66 | 0 | 0.0044 | 1.3515 | 50-100 |
| Metal | 1 | 0 | 107 | 0 | 1-100 |
| Very dry ground | 3 | 0 | 0.00015 | 2.52 | 1-10 only |
| Medium dry ground | 15 | −0.1 | 0.035 | 1.63 | 1-10 only |
| Wet ground | 30 | −0.4 | 0.15 | 1.30 | 1-10 only |

Conclusion

In this contribution, we provide our measurements, RT simulations, observations, and analysis on ISAC channel modelling, and we have the following observations and proposals:

Proposal 1: Clarify RCS components A, B1, B2 as following

- For B2,  to ensure the mean value  be equal to 1

- For B1,  to ensure the mean value  be equal to 1

- For A,  and

Proposal 2: For the upper boundary kσ dB of the component B2 of RCS, k=1 is suggested.

Proposal 3: Denote

CPMtx,sp,rx = CPMsp,rx . CPMsp . CPMtx,sp

= a11a12a21a22∙b11b12b21b22∙c11c12c21c22 = d11d12d21d22.

The normalized CPMtx,sp,rx  can be represented as

CPMtx,sp,rxnorm =d11d12d21d2212(d11+d22)=2(d11+d22)d11d12d21d22

Proposal 4: For UAV of small size (Length x Width x Height: 0.3m x 0.4m x 0.2m) at 4.9GHz for both mono-static and bi-static modes:

  - B1=1

  - A is mean RCS value of 0.053m2 corresponding -12.74 dbsm

  - B2 of UAV is modelled using log-normal distribution. Draw B2 linear-scaled value as

B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for the UAV is modelled as log-Normal distribution. Draw linear-scaled XPR value as

 = 10Y/10,

Where,  is Gaussian distributed with 24.4and = 3.05.

  - The CPM for UAV is represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

Proposal 5: The model 1 is preferred for pedestrian with single scattering point for both mono-static and bi-static modes:

  - For Adult pedestrian (approximately 0.5 meters in length, 0.5 meters in width, and 1.75 meters in height) at 4.9GHz

    - B1=0dB

    - A is mean RCS value of 0.4m2

    - B2 of pedestrian is modelled using log-normal distribution. Draw B2 values as

B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for pedestrian is modelled using log-Normal distributed. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 25.8 and = 2.99.

  - The CPM for UAV is represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

Proposal 6: For mono-static, the RCS=A*B=A*B1*B2 of a scattering point for a passenger vehicle Type 1/2 can be generated by

  - A has mean RCS value 11.72 m2 corresponding 10.69 dBsm

  - B1 is deterministic based on incident/scattered angles

B1dB = rcsdB(θ,φ)−6.61.

Where,

rcsdB(θ,φ)=Gmax−min−AVdBθ+AHdB φ,Amax

Where,

AVdBθ=−min12θ−θcenterθ3dB2, Amax,

AHdB φ=−min12φ−φcenterφ3dB2, Amax,

| Area/n | Side of vehicle | θcenter | φcenter | φ3dB | θ3dB | Amax | Gmax | Applicable range of ZoA/θ | Applicable range of AoA/φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Left | 78.8 | 0° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [0°, 45°]∪[315°, 360°] |
| 2 | Back | 71.6° | 90° | 39.6° | 43.9° | 2.76 | 4.62 | [20°, 90°] | [45°,135°] |
| 3 | Right | 78.8° | 180° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [135°,225°] |
| 4 | Front | 20.0° | 270° | 95.7° | 48.4° | 10.05 | 9.49 | [20°, 90°] | [225°,315°] |
| 5 | Roof | 0° | - | - | 28.42 | 3.63 | 10.99 | [0°, 20°] | [0°,360°] |

    - B2 is generated by log-normal distribution

      - Draw B2 values as B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for the Vehicle is modelled using log-Normal distributed. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 22.3 and = 4.71.

Proposal 7: For Bi-static, the RCS=A*B1*B2 of a scattering point for a passenger vehicle Type 1/2 can be generated by

  - A has mean RCS value 11.72 m2 corresponding 10.69 dBsm

  - B1 is deterministic based on incident/scattered angles

B1dB = rcsdB(θi,φi,θs,φs)−6.61.

Where,

rcsdB(θi,φi,θs,φs)=Greflection−min−AVdBθs+AHdB φs,Amax

and,

AVdBθs=−min12θs−θreflect(θi)θ3dB2, Amax,

AHdB φs=−min12φs−φreflect(φi)φ3dB2, Amax,

Greflection= Gmax_RCS*cos((φi-φreflect)/2)*cos((θi-θreflect)/2),

| Side of vehicle | ZoA/θi | AoA/φi | (θn,φn) | φ3dBn | θ3dB | Amax | Gmax_RCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | [20°, 90°] | [0°, 90°]∪ [270°, 360°] | (78.8, 0) | 47.5° | 99.5° | 5.8 | 9.5 |
| Back | [20°, 90°] | (0°,180°) | (71.6, 90) | 39.6° | 43.9° | 2.76 | 4.62 |
| Right | [20°, 90°] | (90°,270°) | (78.8, 180) | 47.5° | 99.5° | 5.8 | 9.5 |
| Front | [20°, 90°] | (180°,360°) | (20, 270) | 95.7° | 48.4° | 10.05 | 9.49 |
| Roof | [0°, 20°] | (0°,360°) | (0, -) | - | 28.42 | 3.63 | 10.99 |

Note： if θn=0,  then  Greflection = Gmax_RCS*cos(θi).

  - B2 is generated by log-normal distribution

      - Draw B2 values as B2 =10X/10,

Where,  is Gaussian distributed with and .

Proposal 8: For a target modeled with M>=1 scattering points, the following characteristics are supported:

- The number of visible scattering points is fixed to M, i.e. not depending on distance between Tx/Rx and the target.

- The number of scattering points of a target is generated at the beginning of the simulation and keeps unchanged

- The number and locations of the scattering points are common to each pair of sensing Tx/Rx

- The relative locations of the M scattering points in LCS of the target are based on the shape of the target

- The changes of the relative locations of multiple scattering points are not modeled

- For vehicle, M = 5 and the RCS pattern of the n-th scattering point rcsdB(n,θ,φ) in component B1 is:

mrcsdB(n) = dB[ linear(rcsdB(n))−k=1, k≠n5linear(rcsdB(k))]

Wherein rcsdB(n) is the RCS value from single scattering point.

Proposal 9: For a target modeled with multiple scattering points, the target channel for each scattering point are generated separately:

- One direct path and several indirect paths are generated for each scattering point.

- LOS condition is generated separately for each scattering point.

- Pathloss is generated separately for each scattering point based on its LOS condition and location.

- Spatial consistency are applied for channels of multiple scattering points.

Observation 1: The circulate angle spreads (including AOD spread, AOA spread, ZOD spread and ZOA spread) of Option 3 is almost same as that of Option 0 for sensing channel in case 4 in both Umi and Indoor office scenarios.

Proposal 10: Power normalization is not needed after paths dropping

Proposal 11: For single scattering point modeling, the number of NLOS clusters is 13 of that defined in 38.901

  - Each NLOS cluster includes 20 rays as the same as defined TR 38.901

Proposal 12: For the absolute time delay modeling, the following are supported:

- The absolute time delay Δτ is necessary for ISAC channel model.

- The absolute time delay Δτ should be used to all the stochastic clusters including the first NLOS cluster.

Proposal 13: For spatial consistency of ISAC channel modeling for TRP-UE bi-static sensing mode

- The legacy network grid generated in site-specific or all-correlated manner is reused for the links from TRP to ST, and from TRP to UE

- A Rx-specific grid is generated for the links from ST to Rx

  - The correlation of each Rx-specific grid should be modeled, i.e. all Rx-specific grids are generated based on a shift from a global grid

- FFS whether/how to model spatial consistency for UE bi-static or mono-static sensing modes including correlation distance and correlation between different Tx UEs.

Proposal 14: On the background channel for monostatic sensing mode, support Solution A, i.e. dropping N reference points following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and each reference points for a scenario.

Proposal 15: For Hbackground modelling in the case of mono-static sensing mode, a virtual Rx location can be randomly generated, and then the stochastic mechanism of TR 38.901 between the real Tx and the virtual Rx is reused.

- Gamma distributions can be used to generate the distance between Tx/Rx and the virtual Rx, and to generate the virtual Rx height

- The virtual Rx is uniformly dropped in the horizontal domain for each TRP.

- ASD parameter in TR 38.901 can be updated to make background clusters in any horizontal directions.

  - ASA generation is not needed where ASA is enforced to be the same as ASD

Proposal 16: In UMa scenario, the Gamma distribution of distance between Tx/Rx and the virtual Rx, and the virtual Rx height could be given by

The extended mean value of log-normal distribution for ASD could

Proposal 17: In Indoor-Office scenario, the Gamma distribution of distance between Tx/Rx and the virtual Rx, and the virtual Rx height could be given by

- Observation 2: The background channel based on Solution B for mono-static sensing mode is not matched with the real measurement or RT results for UMa and Indoor office scenarios

- For UMa scenario,

  - The pathloss of stochastic ground clutter model is too small

  - The delay of stochastic ground clutter is too small and concentrated.

  - The distribution of AoA (which is same as AoD) is not uniform distribution when the square grid is used.

  - The distribution of ZoA (which is same as ZoD) highly depends on size of grid.

  - Large grid is too computing resource wasting to be used to generate background channel.

- For indoor scenario,

  - The pathloss distribution is much wider than the real measured pathloss

  - The power profile of rays is much lower than the real measurements

- Observation 3: There are still many open issues for the stochastic ground clutter model, i.e. Solution B, to be solved per scenario:

- The shape of grid

- The longest length of grid

- Grid size

- RCS distribution of each grid point (mean value, standard deviation, and other parameters if Gaussian distribution is not used)

- The number of point target which would be dropped on the grid

- The distribution for dropping point target especially in vertical domain

- The location of sensing transmitter(s) and receiver(s)

- CPM

- Complexity issues because of large number of ground grids

- Observation 4: Until now, there are not any measurement results and simulation results to valid the method to derive the parameters for stochastic ground clutter model. Companies should at least provide a method to obtain the parameters to show the potential feasibility for stochastic ground clutter model to fit with actual background channel.

Proposal 18: Option 1 is preferred: power normalization between target channel and background channel is not performed.

Proposal 19: Endorse the above step 1-7 for EO type 2 into TR for ISAC channel modeling, where the channel coefficient for the EO reflected path is given by

Proposal 20: Using the CDL channel as the starting point for research on ISAC LLS.

Proposal 21: The procedure of hybrid channel modelling with RT simulation in TR 38.901 can be reused and enhanced for sensing. Specify the following typical maps and characteristics of sensing targets to align the simulation assumptions:

- Urban grid map defined in 3GPP TR 37.885

- The well-known Manhattan map from open source

- Indoor map defined in IEEE 802.11 WLAN

- EM parameters defined for radar material by ITU

Reference

- RP-234069 RAN New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, 3GPP TSG RAN Meeting #102.

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V17.1.0, 2024-01.

- 3GPP TR 36.777, Enhanced LTE support for aerial vehicles, V15.0.0, 2018-01.

- 3GPP TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR, V15.3.0, 2019-06.

- Map, Open Street. "Open street map." Acessado em 12 (2017).

- 11-21-0782-02-00bf-channel-models-for-wlan-sensing-systems.doc

- https://www.itu.int/rec/R-REC-P.2040/en

- https://www.itu.int/rec/R-REC-P.527/en

- Jiang, Chuangxin, et al. "A Novel Approach to Model the Scattering Environment in Channel Modeling for Integrated Sensing and Communications." 2024 international wireless communications and mobile computing conference (IWCMC). IEEE, 2024 (accepted).

- R1-2402704 in RAN1#116bis meeting, Discussion on channel modelling for ISAC,  ZTE

- Feldman, Yuri, et al. "The electromagnetic response of human skin in the millimetre and submillimetre wave range." Physics in Medicine & Biology 54.11 (2009): 3341.

- Degli-Esposti, Vittorio, et al. "Measurement and modelling of scattering from buildings." IEEE transactions on antennas and propagation 55.1 (2007): 143-153.

- Schuh M J , Woo A C , Simon M P .The monostatic/bistatic approximation[J].IEEE Antennas & Propagation Magazine, 1994, 36(4):p.76-78.DOI:1045-9243(1994)36:4<76:TMBA>2.0.TX;2-D.

- Jr R L E , Collins P J , Jr A J T ,et al.Bistatic scattering characterization of complex objects[J].Geoscience & Remote Sensing IEEE Transactions on, 2000, 38(5):2078-2092.DOI:10.1109/36.868867.

- R1-2404190 in RAN1#117 meeting, Views on Rel-19 ISAC channel modelling, vivo

- Draft_Minutes_report_RAN1#118_v010

- R1-2408515 in RAN1#118bis meeting, Discussion on channel modelling for ISAC,  ZTE

- R1-2410126 in RAN1#119 meeting, Discussion on ISAC Channel Modelling	 Ericsson