3GPP TSG RAN WG1 #120			 R1-2500717

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.11.2

Source:		Xiaomi

Title:	Discussion on the support of Redcap and eRedcap UEs in NR NTN

Document for:	Discussion

Agreement

For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3,

- Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

- For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all the other use cases above.

    - This is signaled by RRC configuration.

Note: UE shall comply to the following existing procedure in 38.331:

UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Agreement

The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode.

- FFS whether to prioritize UL or prioritize DL.

Introduction

In the 	RAN1#119 meeting, the above agreements were made on support of Redcap and eRedcap UEs in NR-NTN.  In this contribution, we further elaborate our views towards the remaining issues on case 3/4 collision handling and other issues such as enhanced TA reporting.

Discussion

Regarding case 3, the remaining part is the collision handling for the behavior in RRC inactive state. From our understanding, the main issue is aligning the collision handling rules with TN RedCap.

Specifically, in TN, it’s specified that UE monitors paging during paging occasion (type 2 CSS) and cancels CG-PUSCH transmission in RRC inactive state when a paging occasion overlaps with a CG-SDT transmission. We believe this behaviour from TN can be re-used in NTN provided the UE indicates support for CG-SDT in its capability. For other cases involving collisions between Type-0/0A/1/2-PDCCH CSS and semi-static UL transmission, there are two potential approaches:

- Adopt a consistent rule for all cases for RRC Inactive UEs, i.e., always canceling UL transmission

- List specific scenarios, such as Type 1 PDCCH scheduling SIB-19, and clarify that the actual UL or DL transmission direction should be determined by UE implementation or even NW indication

Given that debates on this topic have been lengthy over several meetings, resulting in the current agreement for UEs in RRC connected state, a feasible approach may be to reuse the agreement for RRC connected state for RRC inactive state as well, except for the specific handling case in TN. Thus, we propose the following additional collision handling rule to complete the picture for Case 3:

Proposal 1: For Rel-19 HD-FDD (e)Redcap UE in RRC inactive mode, for collision case 3,

- Handling of collision with type 2 CSS for paging occasion and CG-PUSCH transmission shall follow the TN specification, i.e. UE monitors the paging occasion and cancels CG-PUSCH transmission.

- Handling of collision with other Type-0/0A/1/2-PDCCH CSS in RRC inactive mode is left to UE implementation whether to prioritize UL or prioritize DL.

- For other use cases, default priority rule for collision case 3 in RRC inactive mode is that DL is prioritized.

In TN network, the collision issue for case 4 is resolved relying on the gNB’s implementation. It is specified that “A HD-UE does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols.” which means UE would treat them as error cases in the legacy TN network design and the gNB can handle the collision cases by implementation.

For Case 4, there is no specified handling for NTN concurrent with TN regarding the collision between paging occasions and CG-PUSCH transmission in RRC inactive state. For RRC connected state, the following DL or UL channels in table 1 may be involved in the collision. Given the agreed principle for Case 3 collision handling, we propose reusing this principle for Case 4. A general prioritization principle is to prioritize UL over DL for the following reasons:

- For TN, when it comes to semi-statically configured DL reception and dynamically scheduled UL, reception of DL is cancelled

- For dynamically scheduled UL, the DCI itself is firstly successfully decoded, i.e. the UL transmission is to be performed with the premise that a successful decoding of DL reception of control scheduling such UL transmission is performed at UE side. Thus dropping this UL channel is equivalent to dropping a DL PDCCH for another DL channel, which seems not reasonable.

- For NTN with large RTT, some channels such as PRACH/PUCCH are important in the sense that the follow up DL transmission will rely on the reception of these UL channels and cancellation will result in a delayed or even cancelled DL. If a unified DL vs UL principle is to be established, we had better take into account the DL dependent UL as well.

Table 1 Dynamically scheduled DL channel vs UL channel

| DL channel | UL channel |
| --- | --- |
| Dynamic CSI-RS Dynamic PDSCH | Dynamic PUSCH including CSI report |
|  | Aperiodic PUCCH including CSI |
|  | Dynamically triggered PRACH |
|  | DCI triggered SRS |

Summing up, if we re-use the principle of the agreement for case 3, we can settle UL as the operation kept for case 4 collision handling, subject to potential indication of overriding operation, if needed.

Proposal 2: For the collision case 4, the default priority rule shall be UL is prioritized. FFS potential indication of overriding operation.

Further, it is agreed that to mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied:

- Finer TA report granularity

- Smaller TA offset threshold

- Trigger a TA report when the collision is detected at the UE

- TA reporting with a new triggering mechanism from gNB

TA drifting rate reporting

Section 5.4.8	Timing Advance Reporting

RRC controls Timing Advance reporting by configuring the following parameters:

-	offsetThresholdTA;

-	timingAdvanceSR.

A Timing Advance report (TAR) shall be triggered if any of the following events occur:

-	upon indication from upper layers to trigger a Timing Advance report;

-	upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell;

-	if the variation between the current estimate of the Timing Advance value and the last reported Timing Advance value is equal to or larger than offsetThresholdTA, if configured.

Regarding the proposed potential enhancements on the TA reporting such as defining finer TA report granularity or smaller TA offset threshold, the power consumption is expected to be increased due to the more frequently reporting. In addition, slot-based scheduling is the most typical and common use case in NTN, so finer-grained TA reporting granularity seems to be unnecessary. Another aspect needs to be considered it should be guaranteed that the newly introduced granularity does not make the required number of bits exceed the payload size of the legacy Timing Advance Report MAC CE. The currently supported TA report granularity is 1ms, making the TA uncertainty maximum 2ms due to the potential conversion to integer of the actual TA. Moreover, even when there is such ambiguity in TA reporting, it’s up to gNB to configure the actually applicable TA offset, which may be finetuned in implementation considering the potential case 3/case 4 collision. This finer TA report may work only if the UE is power insensitive and/or the gNB has a need to control the collision from happening in the first place, which seems overoptimization involving cross-group specification work.

Proposal 3: Finer TA report granularity (less than 1ms) is not needed to be further studied to mitigate the collisions of case 3 and case 4.

For the solutions to define new TA reporting mechanism, i.e., periodically triggering by gNB or when the collision is detected at the UE, the benefit is still not clear while additional signaling overhead and UE power consumption is expected. The reason is such additional TA reporting is due to a collision already detected due to a potentially 2 ms TA ambiguity and would be used to mitigate future collision from happening within a same potential 2 ms TA ambiguity. n the aforementioned triggering condition, if the TA mismatch does not really occur as a beyond slot issue b/w NW and UE, then the TA report may be unnecessary to be triggered and worse still cause perhaps additional TA mismatch. For TA mismatch detection, we think using current TA offset threshold, which can be configured as low as 0.5ms, is enough.

Proposal 4: New TA triggering enhancements, i.e., TA reporting based on a new triggering mechanism gNB triggering and triggering a TA report when the collision is detected at the UE, is not needed to be further studied to mitigate the collisions of case3 and case4.

Conclusions

In this contribution, we discuss the potential collision issues to support the HD-FDD Redcap in the NTN scenario. Based on our analysis, we have the following proposals:

Proposal 1: For Rel-19 HD-FDD (e)Redcap UE in RRC inactive mode, for collision case 3,

- Handling of collision with type 2 CSS for paging occasion and CG-PUSCH transmission shall follow the TN specification, i.e. UE monitors the paging occasion and cancels CG-PUSCH transmission.

- Handling of collision with other Type-0/0A/1/2-PDCCH CSS in RRC inactive mode is left to UE implementation whether to prioritize UL or prioritize DL.

- For other use cases, default priority rule for collision case 3 in RRC inactive mode is that DL is prioritized.

Proposal 2: For the collision case 4, the default priority rule shall be UL is prioritized. FFS potential indication of overriding operation.

Proposal 3: Finer TA report granularity (less than 1ms) is not needed to be further studied to mitigate the collisions of case3 and case4.

Proposal 4: New TA triggering enhancements, i.e., TA reporting based on a new triggering mechanism gNB triggering and triggering a TA report when the collision is detected at the UE, is not needed to be further studied to mitigate the collisions of case3 and case4.

References

- 3GPP RAN1 119th meeting chairman note, 3GPP TSG RAN WG1 #119, Orlando, US, November 18th – 22nd, 2024

- 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”, V18.2.0 (2024-06)

- 3GPP TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”, V16.2.0 (2023-03)