3GPP TSG RAN WG1 #120                                            R1-2500843

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.2.4

Source: 	Samsung

Title: 	Views on Rel-19 asymmetric DL sTRP/UL mTRP scenarios

Document for:	Discussion and Decision

Introduction

The Rel-19 WID which has been finally revised in RAN#105 [1] includes the following objectives regarding the Rel-19 asymmetric DL sTRP/UL mTRP scenarios.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

This contribution provides Samsung’s view regarding the Rel-19 asymmetric DL sTRP/UL mTRP scenarios highlighted above.

Potential issues on asymmetric DL sTRP/UL mTRP scenarios

The main objective on this agenda item is to specify necessary enhancements for supporting asymmetric DL sTRP/UL mTRP deployment scenarios, which is depicted as in Figure 1. TRP1 (i.e., both DL/UL capable TRP) in this scenario can operate both DL transmission and UL reception, while TRP2 (i.e., UL TRP) can only operate UL reception.

Figure 1. Asymmetric DL sTRP/UL mTRP scenario

Basic assumptions of UL TRP are as follows:

    - From UL TRP, there is neither DL control information nor DL RS including pathloss RS (PL RS), hence scheduling/control information for UL TRP could be transmitted from TRP1 (i.e., both DL/UL capable TRP), and pathloss between UL only and a UE can be calculated by PL RS from TRP1, and pathloss offset (PL offset) configuration is additionally considered which is defined as difference between two pathloss values where the 1st pathloss is between TRP1 and UE, and the 2nd pathloss is between TRP2 and UE, respectively.

    - Considering heterogeneous network deployment, i.e., the coverage of TRP1 (i.e., both DL/UL capable TRP) and TRP2 (i.e., UL TRP) could be different (e.g., TRP2 can provide a smaller coverage than TRP1).

Within the assumptions as above, the potential issues on asymmetric DL sTRP/UL mTRP scenarios considering UL TRP are listed in the following each of sub-sections.

  - Association between Pathloss RS and Pathloss offset

Since there is no DL transmission from UL TRP, a UE cannot measure pathloss value from UL TRP. Hence, the alternative way considering in Rel-19 is introducing a concept of PL offset which can be defined as pathloss difference between UE and each of TRPs. Per Rel-19 WID, a UE is at least configured with PL offset value for determining pathloss value for UL transmission toward an UL TRP.

To acquire PL offset value from gNB side, a certain assistance from UE side is needed, e.g., transmitting a periodic UL signal toward each of TRPs with same UL transmission power based on same power control parameters. It is important to apply same power control parameters for generating same UL transmission power toward each of TRPs to acquire PL offset so that corresponding impact on difference between UL reception powers at each of TRPs from power control parameters other than pathloss value can be minimized and ignored. To determine the same UL transmission power based on same power control parameters, a pathloss value, which is one of factors to give an impact to calculate PL offset, shall be measured by the same PL RS received from a DL/UL capable TRP.

Given same UL transmission power based on same power control parameters including same PL RS, gNB can calculate the difference between UL reception power at each of TRPs, then PL offset can be obtained in the end. In other words, PL offset can be measured from gNB side, with a corresponding PL RS. Therefore, we would like to introduce an association between PL RS and PL offset so that the PL offset is obtained by using the PL RS.

Proposal 1. Introduce association between PL RS and PL offset.

  - Multi-TRP set-up

In current specification, there are two possible multi-TRP setups as follows:

    - Single-DCI based multi-TRP: a certain TRP can fully control and provide scheduling information for both TRPs via a single DCI, assuming ideal backhaul meaning that it is possible to have tight coordination within a low latency between two TRPs.

    - Multi-DCI based multi-TRP: each TRP can only control itself and provide its own scheduling information, assuming non-ideal backhaul meaning that there is a relatively rough coordination/connection between two TRPs rather than that of single-DCI based multi-TRP set-up.

Considering asymmetric DL sTRP/UL mTRP scenarios, there is no DL transmission from UL TRP which means that the UL TRP cannot provide its own scheduling information to UE. Hence, tighter coordination between DL/UL capable TRP and UL TRP is needed to exchange scheduling information for UL TRP. Also, it is further needed to calculate and manage PL offset value between two TRPs. Therefore, we would like to prioritize a single-DCI based multi-TRP set-up, rather than multi-DCI based multi-TRP, which is not suitable.

Proposal 2. Support single-DCI based multi-TRP framework but not support multi-DCI based multi-TRP framework for asymmetric DL sTRP/UL mTRP scenarios.

  - Unified TCI state type

In RAN1#116 [2], association between UL TCI state and PL (pathloss) offset, and the supported type (joint or separate TCI state type) of unified TCI state for asymmetric MTRP scenario have been agreed as follows:

Agreement For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset: - When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW. - Note: it does not intend to increase the number of maintained PLs per cell. - FFS: whether to support associating joint TCI state (if supported) with a PL offset. Further study whether/how to apply a PL offset on PDCCH-order PRACH transmission too. - FFS: how to determine the Tx beam of PRACH towards UL TRP - Note: this does not imply to support 2 TA for single-DCI based system. Agreement For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2. - Joint TCI state mode can be configured at least for FR1

The reason why joint TCI state type is only agreed in FR1 is that the corresponding QCL source RS of QCL-TypeD, which is applied to UL channel/signal, could be only DL RS (e.g., SSB or CSI-RS). In asymmetric DL sTRP/UL mTRP scenario, there is no DL transmission from UL TRP, so it is not possible to support joint TCI state for UL TRP in FR2 without enhancement on unified TCI framework. Hence, we support joint TCI state mode only in FR1, and don’t support in FR2.

For asymmetric DL sTRP/UL mTRP scenario, although it is possible to reuse separate TCI state type for both FR1 and FR2, and joint TCI state type in FR1, there are some remaining problems as follows:

    - The problem of separate TCI state type considering multi-TRP is that DL TCI state and UL TCI state shall be indicated separately, hence, given the limited number of codepoints in TCI state field, the flexibility to indicate appropriate combinations of DL TCI state and UL TCI states are limited, rather than that of joint TCI state.

    - Since a same type shall be applied to both links between two TRPs and UE, although there is a beam correspondence in the 1st link between TRP1 and UE, if not in the 2nd link between TRP2 and UE, separate TCI state type shall be utilized considering the 2nd link, and joint TCI state type.

On top of reusing separate TCI state type in both FR1 and FR2, and joint TCI state type in FR1 for asymmetric DL sTRP/UL mTRP scenarios, whether/how to utilize benefits of two types together for supporting asymmetric DL sTRP/UL mTRP scenarios could be further investigated.

Proposal 3. Further investigate whether/how to utilize benefits of joint and separate TCI state type together, considering asymmetric DL sTRP/UL mTRP scenarios.

  - Pathloss offset configuration and update via MAC-CE

In RAN1#117 [3], it was agreed that MAC-CE based update for PL offset included in TCI state as follows:

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Based on the above agreement, it implies that multiple joint or UL TCI states can be updated by a single MAC-CE. Similar with other TCI state update/activation MAC-CE, we would like to introduce simultaneous update/activation on PL offset among multiple CCs, based on CC lists. If a MAC-CE indicates a serving cell configured as part of a CC list, this MAC-CE can apply to all the other serving cells in the CC list.

Proposal 4. Support to introduce a functionality of simultaneous update on PL offset among multiple CCs in the CC list via MAC-CE.

Also, if MAC-CE updates multiple PL offsets for multiple TCI states, the bitwidth of MAC-CE could be larger. We think that a same PL offset update can be applied to multiple TCI states which share same PL RS, or same power control parameters corresponding to a certain UL TRP. Hence, in order to reduce MAC-CE overhead, it is better to introduce TCI state groups where each of TCI state groups includes multiple TCI states which are corresponding to a certain UL TRP. Then, when a UE receives a MAC-CE updating PL offset of a certain TCI state which is included in a TCI state group, then same PL offset update is applied to all other TCI states in the TCI state group.

Proposal 5. Support to introduce “a TCI state group” which includes a number of TCI states, in order to update PL offset in those TCI states via a single MAC-CE simultaneously.

In addition, we would like to clarify which joint/UL TCI state(s) can be applied by PL offset update MAC-CE. There can be two distinguished groups of Joint/UL TCI state(s): 1) joint/UL TCI state(s) configured with PL offset (where the configured PL offset value can be zero or non-zero), 2) joint/UL TCI state(s) not configured with PL offset. Our understanding on the above cyan highlighted part in the agreement is that the PL offset update MAC-CE is only applicable to group 1), since it said the PL offset value configured in a joint/UL TCI state can be updated by MAC-CE. Regarding group 2), the PL offset update MAC-CE cannot be applied, and UE can assume the corresponding PL offset value as zero.

Proposal 6. Clarify that PL offset update MAC-CE can be only applied to joint/UL TCI state(s) which PL offset value is configured. In other words, PL offset update MAC-CE cannot be applied to joint/UL TCI state(s) which PL offset is not configured.

  - PL offset for PDCCH order triggered PRACH

During several meetings, whether/how to apply PL offset on PDCCH order triggered PRACH has been discussed, and the following is the corresponding agreement:

Agreement For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission: - Alt2: - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation. - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed. FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission

Remaining issue is that whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed or not, as yellow highlighted above. Our view is that this restriction is harmful on gNB’s scheduling/deployment flexibility, e.g., the number of UL TRPs in a cell, and UL scheduling toward UL TRP(s). Also, since a UE assumes the PL offset value of a joint/UL TCI state not configured with PL offset as zero, if a certain gNB wants to restrict that only one of the indicated joint/UL TCI states is associated with PL offset, then one of the indicated joint/UL TCI state can be configured with PL offset as zero value, which is effectively same as joint/UL TCI state not configured with PL offset.

Proposal 7. Not support a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset.

  - PL offset for other PRACH transmissions

Until now, RAN1 only considered applying PL offset to PRACH transmission when the PRACH is triggered by PDCCH order. BTW, there are some PRACH transmissions which are not triggered by PDCCH order (e.g., PRACH for PCell beam failure recovery procedure). Hence, we would like to clarify whether applying PL offset is also considered PRACH transmission other than triggered by PDCCH order.

Proposal 8. Clarify whether PL offset can be applied to PRACH transmission other than PDCCH order triggered PRACH or not (e.g., PRACH for PCell beam failure recovery procedure).

  - Remaining issue on SRS with PL offset and closed loop

In RAN1#118 [4] and RAN1#119 [7], it was agreed to study whether/how to apply PL offset and SRS CLPC adjustment state as follows:

Agreement in RAN1#118 - Study whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state - Study whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state - E.g., defining i0 as the default CLPC for SRS resource set in this case. E.g,, configure one of the separate SRS CLPC adjustment states to the SRS resource set. Agreement in RAN1#119 For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state, down-select from the following Alts: - Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no extra enhancement. - Alt2: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17), the SRS with lowest ID is always configured with one joint/UL TCI state, the “referenceSignal” in a UL TCI state is optional. - Alt3: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set, and they are applied on the SRS resources. - Alt4: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one PL offset and one CLPC adjustment state are applied on the SRS resources. - FFS how to define the PL offset and CLPC adjustment state

In current specification, how to determine power control parameters on SRS resource set not configured with followUnifiedTCI-StateSRS is specified as follows:

- in clause 7.3.1, if p0AlphaSetforSRS is provided, - if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of 𝑃O_UE_SRS,𝑏,𝑓,𝑐(𝑞𝑠), 𝛼SRS,𝑏,𝑓,𝑐(𝑞𝑠), and SRS power control adjustment state 𝑙 are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State - else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of 𝑃O_UE_SRS,𝑏,𝑓,𝑐(𝑞𝑠), 𝛼SRS,𝑏,𝑓,𝑐(𝑞𝑠), and SRS power control adjustment state 𝑙 are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index 𝑞𝑑 for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set 𝑃O_SRS,𝑏,𝑓,𝑐(𝑞𝑠) is the sum of the component 𝑃O_UE_SRS,𝑏,𝑓,𝑐(𝑞𝑠) and a component p0 provided by SRS-ResourceSet corresponding to the SRS resource set.

Regarding whether/how to apply PL offset and SRS CLPC adjustment state, our view is summarized as follows:

    - There is a clear rule (highlighted as cyan above) from Rel-17 (1st release of unified TCI framework) when an indicated TCI is not applied to SRS resources in a SRS resource set (highlighted as yellow above).

    - Hence, it is common understanding from Rel-17 that SRS resource with the lowest ID in SRS resource set not following an indicated TCI shall be configured with a TCI state including power control parameters for SRS. Based on this rule, PL offset is also applicable, i.e., PL offset is used, which is associated with a TCI state configured in the SRS resource with the lowest ID in SRS resource set not following an indicated TCI state.

    - It was even discussed in RAN1#116bis [6] but not pursued, hence there is no consensus.

    - There is no reason why such understanding shall be changed in Rel-19 asymmetric MTRP scenario.

Proposal 9. Not support to further discuss on some cases for study on SRS, e.g., whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state, and whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state. (i.e., Alt1 in Agreement from RAN1#119).

Conclusion

In this contribution, the following observation and proposals are given:

Proposal 1. Introduce association between PL RS and PL offset.

Proposal 2. Support single-DCI based multi-TRP framework but not support multi-DCI based multi-TRP framework for asymmetric DL sTRP/UL mTRP scenarios.

Proposal 3. Further investigate whether/how to utilize benefits of joint and separate TCI state type together, considering asymmetric DL sTRP/UL mTRP scenarios.

Proposal 4. Support to introduce a functionality of simultaneous update on PL offset among multiple CCs in the CC list via MAC-CE.

Proposal 5. Support to introduce “a TCI state group” which includes a number of TCI states, in order to update PL offset in those TCI states via a single MAC-CE simultaneously.

Proposal 6. Clarify that PL offset update MAC-CE can be only applied to joint/UL TCI state(s) which PL offset value is configured. In other words, PL offset update MAC-CE cannot be applied to joint/UL TCI state(s) which PL offset is not configured.

Proposal 7. Not support a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset.

Proposal 8. Clarify whether PL offset can be applied to PRACH transmission other than PDCCH order triggered PRACH or not (e.g., PRACH for PCell beam failure recovery procedure).

Proposal 9. Not support to further discuss on some cases for study on SRS, e.g., whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state, and whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state.

References

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105.

- Chair’s note, RAN1#116.

- Chair’s note, RAN1#117.

- Chair’s note, RAN1#118.

- Chair’s note, RAN1#118bis.

- R1-2403753, Summary of correction on power control for SRS in unified TCI framework, CATT, RAN1#116bis.

- Chair’s note, RAN1#119.