3GPP TSG RAN WG1 #120		R1-2500852

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.4.3

Source:	Samsung

Title:	Views on timing acquisition and synchronization

Document for:	Discussion and decision

# Introduction

In this contribution, we share our views on timing acquisition and synchronization signals according to the WID [1]. The discussion involves R2D/D2R preamble design, functionalities of midamble for D2R and the corresponding design, and functionalities of postamble for R2D/D2R and the corresponding design.

# Preamble

Figure 1 below illustrates a basic PRDCH/PDRCH structure, including preamble, header and payload.

Figure 1 General PRDCH/PDRCH structure with preamble

For R2D, a preamble immediately precedes the transmission of PRDCH, and includes a start-indicator part (SIP) providing the start of the R2D transmission and a clock-acquisition part (CAP) providing a functionality for clock synchronization including determining the OOK chip rate of the subsequent PRDCH transmission.

For D2R, a preamble immediately precedes the transmission of PDRCH, and includes CAP providing a functionality for synchronization and time tracking, including determination of OOK chip rate or BPSK symbol rate of the subsequent PDRCH transmission, and channel estimation for coherent detection of the PDRCH.

## R2D

Start-indicator part (SIP)

During the SI, the following principles for SIP design were made [2] .

- Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including

  - Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations

  - Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration

- Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation

For both options, a fixed duration SIP can be considered, regardless of the value of M used for PRDCH transmissions. Furthermore, for both options, it is beneficial that the SIP is distinguishable at least from other parts of the R2D transmissions.

The Option 2, i.e., a sequence-based SIP, requires a device to implement a more complex detector such as sequence correlator. Given that the only functionality required for SIP is providing the start of the R2D transmission, it is not needed for the SIP to be based on a sequence-based signal. Therefore, we prefer Option 1 for R2D SIP design, which can be detected using a simple pattern matting.

Observation 1: A sequence-based SIP (Option 2) requires a device to implement a more complex detector, which is not required given its functionality.

Proposal 1: The SIP of R2D preamble is based on ON-OFF transmission, which can be detected using energy/edge detection.

In the following, we share our views on SIP design principles based on ON-OFF transmission (Option 1):

- SIP starts from an OFDM symbol boundary and occupies an OFDM symbol duration.

- SIP is distinguishable from other parts of the R2D transmissions, i.e., a unique pattern that does not occur with the used line encoding scheme, i.e., Manchester.

- SIP has ON duration longer than OFF duration for setting up the envelop detector threshold.

- SIP has a fixed pattern that is easily detectable based on the ON/OFF ratio or pattern matching without having the clock calibrated.

Based on the above SIP design principles, the following figure illustrates example SIP patterns for Option 1-Alt 1

Figure 2 SIP design examples

In order for the SIP to be distinguishable from other parts of the R2D transmissions, the ratio of the ON duration to the OFF duration, denoted by X:1, can be chosen such that it cannot occur during the line coding. With Manchester encoding, X can be 1 or 2 at most. Therefore, X can be chosen to be greater than or equal to 3. For example, when X=3, the ON duration is comprised of three consecutive high voltage chips and the OFF duration is comprised of a single low voltage chip in one OFDM symbol duration. Based on such a prior knowledge, the device can acquire a reference chip length prior to the reception of CAP.

The SIP provides one and the only functionality of start indication. For such a simple purpose, a single ON/OFF pattern shall be sufficient and a complex pattern or a sequence-based design is not preferred.

Proposal 2: The SIP of R2D preamble is based on a single ON-OFF transmission having ON duration longer than OFF duration.

Clock-acquisition part (CAP)

During the SI, it was agreed that CAP is based on OOK without line coding including at least two rising or at least two falling edges for the device to determine the OOK chip duration. The following options for design of the CAP were studied [2]:

- Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M.

- Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant.

In the following, we share our views on CAP design principles:

- CAP starts from an OFDM symbol boundary and occupies a number of OFDM symbol durations.

- CAP provides a sufficient number of chips for synchronization, while managing the overhead, esp., for small M values.

- CAP does not include any irregular chip of high voltage or a low voltage pulse due to CP insertion, which may affect the chip synchronization performance.

- CAP has a fixed pattern for a given M value.

The receiver obtains the value of M that is used in the following PRDCH by receiving the CAP. The CAP can be specific to different values of M such as the total length and the number of chips in the CAP. Generally speaking, for a small value of M, the receiver does not require a very accurate synchronization and, therefore, the receiver can obtain chip synchronization from a small number of chips in the CAP. Keeping constant duration of CAP for small M, i.e., Option 2, can be wasteful for small M, as the CAP will include an excessive number of chips.

Observation 2: Keeping constant duration of CAP for small M, i.e., Option 2, can be wasteful for small M, as the CAP will include an excessive number of chips.

In order to reduce the overhead, the CAP may include a smaller number of chips for synchronization for a smaller M. As an example, for 1 ≤ M ≤ M1, the CAP part is comprised of L1 number of OFDM symbols and, for M1 ≤ M ≤ M2, the CAP part is comprised of L2 number of OFDM symbols, wherein L2 is smaller than L1.

Proposal 3: The duration of the CAP is variable for different M values, i.e., the duration becomes shorter with increasing value of M.

In our view, CAP based on a number of repetitions of ON-OFF pattern is the most effective for achieving OOK chip synchronization.

Proposal 4: The CAP is based on a number of repetitions of ON-OFF patterns.

Figure 3 CAP design examples for M=1, 2, and 4 (the chosen number of symbols are only for illustration)

The figure above illustrates CAP design examples for M = 1, 2, and 4. It is noted that the above figure is to describe principles of CAP design, and the actual numbers of symbols, i.e., CAP duration, can be different.

When M is equal to 1, the CAP is comprised of at least four OFDM symbol duration of ON-OFF-ON-OFF pattern. As an example, the CAP can be a repetition of ON-OFF pattern over a number of OFDM symbols. Since the CAP is unique for a given M value, the receiver determines the M value from the first one or a few chips and determine the number of chips in the CAP and, accordingly, the end of the CAP. As M increases, CAP needs to provide a greater number of chips for more precise chip synchronization, while the CAP duration may become shorter to reduce overhead.

Proposal 5: For M1 > M2, the CAP duration for M1 is shorter than or at most equal to that for M2. A number of chips included in the CAP for M1 is larger than or at least equal to that for M2.

Since M is an even number, except M = 1, the last chip of CAP always ends with OFF chip, if CAP is based on a repetition of ON-OFF transmission. Therefore, for M value having a chip duration longer than CP, the addition of CP only has the effect of extending the last OFF chip of the previous symbol in the CAP. For M=2 or 4 in the above example, the last OFF duration in each symbol duration is extended by the CP in the following symbol. Therefore, the length of the first M-1 ON/OFF duration and the last Mth OFF duration become different. In certain implementation, a receiver performs chips synchronization only based on the ON duration. In another implementation, the receiver performed chip synchronization based on both ON and OFF durations while considering the extension of the last OFF duration by the amount of CP.

For an OFDM system with 15 kHz subcarrier spacing, the CP length for the long CP and the normal CP has 5.21 us and 4.69 us duration, respectively, and the payload has 66.67 us duration. Therefore, for M value of up to 12, a chip length is larger than a CP length. Accordingly, for M value up to 12, the example design for M=4 can be similarly applicable.

Proposal 6: For M ≤ 12, the CAP is designed such that the last chip in each symbol duration has OFF state such that it does not incur any invalid chip in the CAP.

Figure 4 CAP design issue due to CP insertion for M ≥ 16

When M ≥ 16, a chip duration becomes shorter than the CP. Therefore, as illustrated in the figure, the CP insertion may introduce a high voltage or a low voltage chip having an irregular length compared to other normal chips, which can affect the performance of a chip synchronization. Such invalid chips may be left for device implementation to handle, which is not preferable for A-IoT devices with low complexity as it may be confused with normal chips and affects the chip synchronization performance.

Observation 3: For M ≥ 16, a chip duration becomes shorter than the CP, and the CP insertion can introduce a chip having an irregular length, which can affect the performance of a chip synchronization.

Figure 5 CAP design example M ≥ 16 (the actual duration can be longer than 1 symbol)

For M = 16, 24, the CP length is shorter than two chips. For M = 32, a chip length is 2.08 us and, therefore, the CP length is slightly longer than two chips. For M = 16, 24, the CAP can be designed such that the last two chips has OFF states. For M = 32, the CAP can be designed such that the last three chips has OFF states. In this way, the CP has a flat OFF duration, which is easy to be identified and ignored.

Proposal 7: For M ≥ 16, the CAP is designed such that the last chips copied as CP in each symbol duration have OFF state such that it does not incur any invalid chip in the CAP.

## D2R

For D2R, a preamble immediately precedes each PDRCH, and includes CAP providing a functionality for synchronization and time tracking, and channel estimation for coherent detection. The D2R preamble may not include a separate start-indicator as D2R transmission is always triggered and expected by a reader. Therefore, D2R preamble itself can also serve as a start-indicator. As the D2R preamble is used for channel/interference estimation by the reader for coherent detection, it is necessary for the preamble to provide a good auto/cross-correlation property.

Observation 4: As the D2R preamble is used for channel/interference estimation by the reader for coherent detection, it is necessary for the preamble to provide a good auto/cross-correlation property.

Considering good correlation properties and availability of large number of distinct sequences and complementary pairs, our view is that Golay sequence can be a good candidate for D2R preamble.

Proposal 8: D2R preamble is based on Golay sequence. FFS: details including sequence design.

When PDRCH is OOK modulated, the D2R preamble based on Golay sequence can be encoded with a used line coding scheme, i.e., Manchester encoding, or directly modulated to OOK chips without line coding. In other words, the binary (or bipolar) bit is directly mapped to either ON or OFF state. When PDRCH is BPSK modulated, the D2R preamble based on Golay sequence is also modulated with BPSK.

Denote by GaN and GbN a pair of Golay complementary sequences of length N, as a basic building block. As an example, N can be 32, 64, or 128. A longer sequence can be generated and used for D2R preamble via a number of concatenations of any of GaN, -GaN, GbN, -GbN. In one example, D2R preamble is comprised of a number of GaN, followed by -GaN at the end. Similarly, D2R preamble is comprised of a number of GbN, followed by -GbN at the end.

A reader correlates the received D2R preamble and detects the presence of D2R preamble based on correlation spikes. The periodicity of the spikes can be used to determine the OOK chip rate, if OOK modulation is used, or BPSK symbol rate, if BPSK modulation is used. The spike amplitude can be used for setting up the AGC, and the detection of a negative spike, if inserted at the end of preamble, can be used for the detection of the end of the preamble.

The construction of the D2R preamble can also take advantage of the complementary property by grouping a combination of GaN, GbN, - GaN, and - GbN into GuM and GvM, where M is the length of the sequence.

Since D2R transmissions are triggered by a reader, the reader may indicate a format of the clock acquisition part to a device, e.g., long and short formats, used for the subsequent D2R transmissions. The use of different formats, e.g., long and short formats, may be predefined in the specifications based on a certain condition, e.g., packet length, D2R transmission duration or type, etc. For instance, if a certain condition is met, a device always uses a predefined format regardless of the presence or the value of the indication. Different length Golay sequences can be easily generated from the basic building block sequences, i.e., GaN and GbN.

Proposal 9: For D2R preamble, short and long formats are supported via Golay sequence construction, and the format is indicated in the preceding PRDCH.

# Midamble

We envision functionalities of midamble to be the same or similar to those of preamble, such as synchronization and time tracking, and channel estimation for coherent detection.

Proposal 10: The functionalities of D2R midamble are 1) synchronization and time tracking, and 2) channel estimation for coherent detection.

Figure 6 and Figure 7 illustrate the performance gain of midamble for D2R with payload size of 20 bits and 96 bits, respectively. SFO value of both [0.1~1]*105 ppm and [0.1~1]*104 ppm are evaluated. In each case, one midamble is inserted in the middle of PDRCH payload. More details of evaluation assumptions are provided in the Appendix. It can be observed that for lower payload size of 20 bits, the BLER-SNR performance of D2R with and without midamble are similar. For SFO value of [0.1~1]*105 ppm with the larger payload size of 96 bits, the insertion of midamble provides ~0.5 dB SNR gain at 10% BLER with TDL-A channel and ~0.9 dB SNR gain with TDL-D channel; and for SFO value of [0.1~1]*104 ppm, the insertion of midamble provides ~1 dB SNR gain at 10% BLER with TDL-A channel and ~1.4 dB SNR gain with TDL-D channel. Therefore, for a typical payload size of 96 bits, the insertion of midamble can improve D2R detection performance for both SFO values assumed and in both TDL-A and TDL-D channel models.

(a) TDL-A                                     (b) TDL-D

Figure 6 D2R reception performance with/without midamble, payload size 20 bits

(a) TDL-A                            (b) TDL-D

Figure 7 D2R reception performance with/without midamble, payload size 96 bits

In our view, the D2R midamble provides the same functionality as D2R preamble, i.e., synchronization and time tracking, and channel estimation for coherent detection. Therefore, the D2R preamble design can be reused for D2R midamble. The SIP can be reused to distinguish PDRCH payload and midamble, and the CAP can be reused for synchronization and time tracking as well as channel estimation.

Proposal 11: D2R preamble signal is reused for D2R midamble.

The insertion of midamble can be dependent on the message type or size. For instance, if the PDRCH message type falls into one of a predefined set of message types, or the indicated payload size is greater than a certain predefined threshold, the device may assume payload segmentation based on a predefined payload segment size, and a number of midambles to be inserted correspondingly. On the other hand, the length of midamble can be dependent on the payload segment size, i.e., a shorter midamble can be used for a smaller payload segment size while a longer midamble can be used for a larger payload segment size. Insertion of a midamble may be also explicitly indicated by a reader in the preceding PRDCH. For example, it can be a simple on/off indication, and the device performs payload segmentation assuming a predefined payload size per segment, if indicated.

Proposal 12: For D2R midamble, the presence, location and the number of midambles can be determined based on 1) explicit indication in the preceding PRDCH, or 2) predefined rule such as the message type or payload size.

Figure 8 illustrates an example PDRCH signal structure with midamble.

Figure 8 PDRCH signal structure with midamble

Each of the payload segment can be attached with a separate CRC, which helps to decode payloads sequentially without waiting for the end of the signal reception and requires less memory buffer size at the receiver.

Proposal 13: Each payload segment of PDRCH divided by a midamble is attached with a separate CRC.

# Postamble

Potential functionalities of postamble can be 1) end-of-signal (EOS) detection, 2) synchronization and time tracking, and 3) channel estimation for coherent detection. Given that the postamble is located at the end of a signal, in order to utilize postamble for 2) synchronization and time tracking, and 3) channel estimation for coherent detection, a receiver has to buffer the received signal and can start decoding only after reception of the postamble and compensation based on the estimation from the postamble.

Observation 5: In order to utilize postamble for synchronization and time tracking, and channel estimation for coherent detection, the receiver has to buffer the received signal and can start decoding only after reception of the postamble.

For R2D postamble, the functionality of channel estimation for coherent detection is not needed as non-coherent detection will be used by the device. Furthermore, the functionality of synchronization and time tracking from postamble requires a more complex receiver implementation at the device such as buffering the received signal. Furthermore, synchronization and time tracking can be achieved from PRDCH reception itself, which is encoded with Manchester line code. Therefore, the functionality of synchronization and time tracking is also not needed for R2D.

Observation 6: For R2D postamble, the functionality of channel estimation for coherent detection is not needed.  The functionality of synchronization and time tracking is also not needed as it can be achieved from PRDCH encoded with Manchester line code.

The determination of the end of PRDCH can be based on either L1 R2D control information or postamble. Given a possible clock drift at the device, postamble can be still useful in determining the end of PRDCH even when the L1 R2D control information is provided.

Proposal 14: The functionality of R2D postamble is only for determining the end of PRDCH.

For D2R, postamble can be considered for synchronization and time tracking, and channel estimation for coherent detection. However, as described earlier, utilizing postamble requires a more complex receiver implementation, which can be a burden even for a reader to perform buffering and interpolation of channel measurements. Furthermore, such functionalities of synchronization and time tracking, and channel estimation can be achieved from midamble, if supported. Therefore, postamble can be mainly considered for determining the end of PDRCH, even when the D2R control information indicates the message size due to a possible clock drift at the device.

Observation 7: For D2R postamble, the functionalities of channel estimation for coherent detection, and synchronization and time tracking are not needed, since D2R midamble provides such functionalities.

Proposal 15: The functionality of D2R postamble is only for determining the end of PDRCH.

When R2D or D2R postamble provides the only functionality of the end of PRDCH/PDRCH determination, the postamble signal design can be largely based on the R2D SIP design principles. For instance, a postamble can be a single or multiple ON-OFF (or OFF-ON) transmissions, whose presence can be determined via simple energy/edge detection. The postamble may start from an OFDM symbol boundary and occupy an OFDM symbol duration, and distinguishable from other parts of the R2D/D2R transmissions, i.e., a unique pattern that does not occur with the used line encoding scheme, i.e., Manchester. As an example, postamble can be an inverse of the R2D SIP.

# Conclusion

In this contribution, we shared our views on timing acquisition and synchronization signals. More specifically, the discussion involves R2D/D2R preamble design, functionalities of midamble for D2R and the corresponding design, and functionalities of postamble for R2D/D2R and the corresponding design. To this end, the following observations and proposals were made:

Observation 1: A sequence-based SIP (Option 2) requires a device to implement a more complex detector, which is not required given its functionality.

Proposal 1: The SIP of R2D preamble is based on ON-OFF transmission, which can be detected using energy/edge detection.

Proposal 2: The SIP of R2D preamble is based on a single ON-OFF transmission having ON duration longer than OFF duration.

Observation 2: Keeping constant duration of CAP for small M, i.e., Option 2, can be wasteful for small M, as the CAP will include an excessive number of chips.

Proposal 3: The duration of the CAP is variable for different M values, i.e., the duration becomes shorter with increasing value of M.

Proposal 4: The CAP is based on a number of repetitions of ON-OFF patterns.

Proposal 5: For M1 > M2, the CAP duration for M1 is shorter than or at most equal to that for M2. A number of chips included in the CAP for M1 is larger than or at least equal to that for M2.

Proposal 6: For M ≤ 12, the CAP is designed such that the last chip in each symbol duration has OFF state such that it does not incur any invalid chip in the CAP.

Observation 3: For M ≥ 16, a chip duration becomes shorter than the CP, and the CP insertion can introduce a chip having an irregular length, which can affect the performance of a chip synchronization.

Proposal 7: For M ≥ 16, the CAP is designed such that the last chips copied as CP in each symbol duration have OFF state such that it does not incur any invalid chip in the CAP.

Observation 4: As the D2R preamble is used for channel/interference estimation by the reader for coherent detection, it is necessary for the preamble to provide a good auto/cross-correlation property.

Proposal 8: D2R preamble is based on Golay sequence. FFS: details including sequence design.

Proposal 9: For D2R preamble, short and long formats are supported via Golay sequence construction, and the format is indicated in the preceding PRDCH.

Proposal 10: The functionalities of D2R midamble are 1) synchronization and time tracking, and 2) channel estimation for coherent detection.

Proposal 11: D2R preamble signal is reused for D2R midamble.

Proposal 12: For D2R midamble, the presence, location and the number of midambles can be determined based on 1) explicit indication in the preceding PRDCH, or 2) predefined rule such as the message type or payload size.

Proposal 13: Each payload segment of PDRCH divided by a midamble is attached with a separate CRC.

Observation 5: In order to utilize postamble for synchronization and time tracking, and channel estimation for coherent detection, the receiver has to buffer the received signal and can start decoding only after reception of the postamble.

Observation 6: For R2D postamble, the functionality of channel estimation for coherent detection is not needed.  The functionality of synchronization and time tracking is also not needed as it can be achieved from PRDCH encoded with Manchester line code.

Proposal 14: The functionality of R2D postamble is only for determining the end of PRDCH.

Observation 7: For D2R postamble, the functionalities of channel estimation for coherent detection, and synchronization and time tracking are not needed, since D2R midamble provides such functionalities.

Proposal 15: The functionality of D2R postamble is only for determining the end of PDRCH.

# Appendix

The evaluation parameters for D2R performance with/without midamble are provided as below.

Table 1 Evaluation assumption for D2R performance with/without midamble

| System Parameter | Value |
| --- | --- |
| Sampling rate | 1.92 [MHz] |
| Carrier frequency | 900 [MHz] |
| Data rate | 5 [kbps] |
| Payload structure and size | 20/90 bits + CRC-6 |
| Number of Rx antenna | 2 |
| Channel type | TDL-A 30ns, TDL-D 30ns |
| Channel velocity | 3km/h |
| Line encoding scheme | Manchester |
| Initial SFO | [0.1~1]*10^5 ppm [0.1~1]*10^4 ppm |
| CW type | Single-tone |
| Detector | Coherent detection, soft decision for Manchester |

# References:

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

- TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, version 19.0.0, January 15, 2025