3GPP TSG RAN WG1 #120	R1-2501596

# Athens, Greece, February 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0696 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RACH-less handover for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator(ASUSTeK), Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-2-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When introducing RACH-less handover for NTN, CG-SDT should be used as baseline as agreed in RAN2 #124: RAN2 Agreement: … 7. If CG is configured in RACH-less HO, UE uses the earliest available CG occasion associated to the selected SSB for the initial UL transmission. Spec impact is FFS and can use CG-SDT as baseline (if applicable) Thus, text of RACH-less handover for NTN is specified similar to CG-SDT. However, UL BWP for type-1 CG resource specified for NTN is different than UL BWP for type-1 CG resource for SDT due to text specified in TS 38.331 that a mandatory parameter “firstActiveUplinkBWP-Id” should be provided upon HO and the first UL BWP indicated by the parameter is considered to be active UL BWP. It’s not clear whether first actvie UL BWP is the same as the initial UL BWP, and if first actvie UL BWP is not the initial UL BWP and type-1 CG resource for HO is always on the initial UL BWP, UE needs to change active UL BWP from first active UL BWP to the initial UL BWP resulting in additional latency. TS 38.331: ServingCellConfig information element -- ASN1START -- TAG-SERVINGCELLCONFIG-START ServingCellConfig ::= SEQUENCE { … uplinkConfig UplinkConfig OPTIONAL, -- Need M … UplinkConfig ::= SEQUENCE { initialUplinkBWP BWP-UplinkDedicated OPTIONAL, -- Need M uplinkBWP-ToReleaseList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id OPTIONAL, -- Need N uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink OPTIONAL, -- Need N firstActiveUplinkBWP-Id BWP-Id OPTIONAL, -- Cond SyncAndCellAdd SyncAndCellAdd; This field is mandatory present for a SpCell upon reconfiguration with reconfigurationWithSync and upon RRCSetup/RRCResume. The field is optionally present for an SpCell, Need N, upon reconfiguration without reconfigurationWithSync. The field is mandatory present for an SCell upon addition, and absent for SCell in other cases, Need M.. 5.3.5.5.7 SpCell Configuration The UE shall: … 2> if the SpCellConfig contains spCellConfigDedicated: 3> configure the SpCell in accordance with the spCellConfigDedicated; 3> consider the bandwidth part indicated in firstActiveUplinkBWP-Id, if included in the spCellConfigDedicated, to be the active uplink bandwidth part; |
| Summary of change: |  | Corrected CG type1 PUSCH transmissions being on the active UL BWP according to TS 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It’s not clear whether first actvie UL BWP is the same as the initial UL BWP. If first actvie UL BWP is not the initial UL BWP and type-1 CG resource for HO is always on the initial UL BWP, UE needs to change active UL BWP from first active UL BWP to the initial UL BWP resulting in additional latency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 22	PUSCH transmission in NTN RACH-less handover

## 22.1	Configured-grant PUSCH transmission

A UE indicated to perform PUSCH transmission in RACH-less handover can be provided one or more configurations by respective one or more ConfiguredGrantConfig, for configured grant Type 1 PUSCH transmissions on the active UL BWP [12, TS 38.331]. For the remaining of this clause, PUSCH transmissions refer to configured grant Type-1 PUSCH transmissions for a configuration provided by ConfiguredGrantConfig.

A UE can be provided by rrc-SSB-Subset a number of SS/PBCH block indexes NPUSCHSS/PBCH to map to a number of valid PUSCH occasions for PUSCH transmissions over an association period. If the UE is not provided rrc-SSB-Subset, the UE determines NPUSCHSS/PBCH from the value of ssb-PositionsInBurst in ServingCellConfigCommon. A PUSCH occasion for a PUSCH transmission is defined by a time resource and a frequency resource and is associated with a DM-RS provided by cg-DMRS-Configuration for the configuration of PUSCH transmissions. A UE can be provided a number of repetitions for a PUSCH transmission by repK or numberOfRepetitions. If the number of repetitions is provided and larger than 1, all the PUSCH occasions of the repetitions for the PUSCH transmission are mapped to the same SS/PBCH block index(es). For the initial transmission or autonomous retransmission of an initial transport block provided for PUSCH transmission in RACH-less handover, the UE encodes the transport block using redundancy version number 0 if the UE is not provided repK-RV.