3GPP TSG RAN WG1 #120		                                   						R1-2501396

Athens, Greece, February 17th – 21st, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.7

Agenda Item:		9.15.7

Document for: 	Endorsement

- 9.15.7 UE features for XR for NR Phase 3

For RAN1#120, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

Agreement:

Introduce following basic FG for XR Phase 3, and FFS on some potential components (that may be separate FGs).

| X-1 | Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1] | 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour. [4. The minimum time offset from the DCI carrying the gap/restriction cancellation to the cancelled gap/restriction] FFS: other component (which may be separate FG) | FFS | YES | n/a | UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1] | [Per UE or Per band] |  |  |  | Candidate values of component 3: {Option 1, Option 2} [Candidate values of component 4: {5ms, 3ms}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Confirm the following component 4 in X-1 for XR

- [4. The minimum time offset from the end of PDCCHDCI carrying the gap/restriction cancellation to the start of cancelled gap/restriction]

Agreement:

Confirm the following candidate values for component 4 in X-1 for XR

- [Candidate values of component 4: {5ms, 3ms}]

Proposal 2:

For separation of X-1 per DCI format(s), consider the following alternatives:

- Alt-1: A FG (i.e., X-1) is defined for the support of XR basic feature for all the DCI formats (0_1/1_1/0_2/1_2/0_3/1_3)

  - Alt-1a: The support of XR for DCI format 0_2/1_2/0_3/1_3 are subject to the corresponding UE capability until Rel-18

    - A UE supporting X-1 supports “Enabling TX/RX during measurement gap scheduling restriction by DCI format 0_2/1_2”, if the UE also support DCI 0_2/1_2 (i.e., with prerequisite FG 11-1)

    - A UE supporting X-1 supports “Enabling TX/RX during measurement gap scheduling restriction by DCI format 0_3/1_3”, if the UE also support DCI 0_3/1_3 (i.e., with prerequisite FG 49-1, 49-1b, 49-2, 49-2b)

  - Alt-1b: Whether to support XR for DCI format 0_2/1_2/0_3/1_3 are subject to a component in X-1

- Alt-2: A FG (e.g., X-1) is defined for the support of XR basic feature for DCI format 0_1/1_1/0_2/1_2. Another FG (e.g., X-2) is defined for the support of XR basic feature for DCI format 0_3/1_3.

  - Alt-2a: The support of XR for DCI format 0_2/1_2 are subject to the corresponding UE capability in Rel-16

    - A UE supporting X-1 supports “Enabling TX/RX during measurement gap scheduling restriction by DCI format 0_2/1_2”, if the UE also support DCI 0_2/1_2 (i.e., with prerequisite FG 11-1)

  - Alt-2b: Whether to support XR for DCI format 0_2/1_2/ are subject to a component in X-1

  - FFS: Prerequisite relationship between FG X-1 and X-2

- Alt-3: A FG (e.g., X-1) is defined for the support of XR basic feature for DCI format 0_1/1_1. Another FG (e.g., X-2) is defined for the support of XR basic feature for DCI format 0_2/1_2. Another FG (e.g., X-3) is defined for the support of XR basic feature for DCI format 0_3/1_3.

  - FFS: Prerequisite relationship between FG X-1, X-2 and X-3

R1-2500103	UE feature for R19 XR	Huawei, HiSilicon

R1-2500114	Views on Rel-19 XR UE features	Nokia

R1-2500137	Discussion on XR features	ZTE Corporation, Sanechips

R1-2500189	Discussion on UE features for XR for NR Phase 3	Spreadtrum, UNISOC

R1-2500216	XR UE feature	CATT

R1-2500309	Discussion on UE features for XR for NR Phase 3	CMCC

R1-2500376	Discussion on Rel-19 UE features for XR for NR Phase 3	vivo

R1-2500460	Discussion on UE features for Rel-19 XR	OPPO

R1-2500812	Views on UE features for XR	Apple

R1-2500875	UE features for XR for NR Phase 3	Samsung

R1-2501007	Initial UE feature list for Rel-19 XR for NR Phase3	Nokia, Qualcomm (WI rapporteurs)

R1-2501185	UE features for Rel-19 XR for NR Phase3	Qualcomm Incorporated

R1-2501224	Discussion on UE features for Rel-19 XR enhancements	NTT DOCOMO, INC.

R1-2501344	UE features for XR enhancements Phase 3	Ericsson