3GPP TSG RAN WG1 #120		                                   						R1-2501396

Athens, Greece, February 17th – 21st, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.7

Agenda Item:		9.15.7

Document for: 	Endorsement

- 9.15.7 UE features for XR for NR Phase 3

For RAN1#120, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

Agreement:

Introduce following basic FG for XR Phase 3, and FFS on some potential components (that may be separate FGs).

| X-1 | Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1] | 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour. [4. The minimum time offset from the DCI carrying the gap/restriction cancellation to the cancelled gap/restriction] FFS: other component (which may be separate FG) | FFS | YES | n/a | UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1] | [Per UE or Per band] |  |  |  | Candidate values of component 3: {Option 1, Option 2} [Candidate values of component 4: {5ms, 3ms}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Confirm the following component 4 in X-1 for XR

- [4. The minimum time offset from the end of PDCCHDCI carrying the gap/restriction cancellation to the start of cancelled gap/restriction]

Agreement:

Confirm the following candidate values for component 4 in X-1 for XR

- [Candidate values of component 4: {5ms, 3ms}]

R1-2500103	UE feature for R19 XR	Huawei, HiSilicon

R1-2500114	Views on Rel-19 XR UE features	Nokia

R1-2500137	Discussion on XR features	ZTE Corporation, Sanechips

R1-2500189	Discussion on UE features for XR for NR Phase 3	Spreadtrum, UNISOC

R1-2500216	XR UE feature	CATT

R1-2500309	Discussion on UE features for XR for NR Phase 3	CMCC

R1-2500376	Discussion on Rel-19 UE features for XR for NR Phase 3	vivo

R1-2500460	Discussion on UE features for Rel-19 XR	OPPO

R1-2500812	Views on UE features for XR	Apple

R1-2500875	UE features for XR for NR Phase 3	Samsung

R1-2501007	Initial UE feature list for Rel-19 XR for NR Phase3	Nokia, Qualcomm (WI rapporteurs)

R1-2501185	UE features for Rel-19 XR for NR Phase3	Qualcomm Incorporated

R1-2501224	Discussion on UE features for Rel-19 XR enhancements	NTT DOCOMO, INC.

R1-2501344	UE features for XR enhancements Phase 3	Ericsson