3GPP TSG RAN WG1 #120		                                  R1-250XXXX

Athens, Greece, February 17th – 21st, 2025

Source:	Moderator (vivo)

Title:                     Summary of discussion on CR on QCL assumption

Agenda Item:	7

Document for:	Discussion and Decision

- Discussion

In [1], a draft CR is provided for clarifying QCL assumptions, two issues are discussed.

Issue 1:

In TS38.331, the condition of applying the indicated TCI state for aperiodic CSI-RS resource(s) is clearly stated per the field description of qcl-info quoted as follows:

When this field is absent for aperiodic CSI RS, the UE shall use QCL information included in the  "indicated" DL only/Joint TCI state as specified in TS 38.214

However, TS38.214 has wrongly captured the above condition:

If dl-OrJointTCI-StateList-r17 is provided, the UE may assume that a CSI-RS resource in an aperiodic CSI-RS resource set configured without trs-Info is quasi co-located with the RS(s) in the indicated TCI state.

Issue 2:

In Rel-17, group-based beam reporting has been enhanced to be associated with two CSI resource sets as stated in TS38.214,

For aperiodic CSI, and for aperiodic CSI resource settings, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where the CSI-ReportConfig configured with groupBasedBeamReporting-r17 is associated with resourcesForChannel and resourcesForChannel2, which correspond to first and second resource sets, respectively, for L1-RSRP measurement.

Accordingly, qcl-info2 can be configured in resourcesForChannel2 as specified in TS38.331. However, related description is missing in TS38.214.

Based on the above reasons following two changes are proposed for 38.214 section 5.2.1.5.1.

-	If dl-OrJointTCI-StateList-r17 is provided, the UE may assume that a CSI-RS resource in an aperiodic CSI-RS resource set configured without trs-Info qcl-info is quasi co-located with the RS(s) in the indicated TCI state.

Change#1

Please provide your view on whether you agree with the proposed change.

| Company | Comments |
| --- | --- |

- For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in clause 5.1.5, through higher layer signaling of qcl-info or qcl-info2 which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP.

Change#2

Please provide your view on whether you agree with the proposed change.

| Company | Comments |
| --- | --- |

- Conclusion

xxx

- References

R1-2500329	Draft CR on QCL assumption of AP CSI-RS	vivo

R1-2500330	Draft mirror CR on QCL assumption of AP CSI-RS (Rel-18 mirror)	vivo