3GPP TSG RAN WG1 Meeting #120bis	R1-2501716

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.2.1

Source:	Futurewei

Title:	Enhancements for UE-initiated/Event-driven Beam Management

Document for:	Discussion and decision

# Introduction

During the RAN1#120 meeting, many agreements were reached [1], including additional indication of whether the CRI/SSBRI satisfies the condition of Event-2, measurement window for initiating the UE-initiated/event-driven (UEI/ED) beam reporting procedure, candidate values of RRC parameters for Event 7, the offset for determining available transmission occasion of the second UL channel for Mode-B, the configuration of new beams for Event 1, the dropping rule of handling the case that the 1-bit first PUCCH is collided with other signals,  the triggering procedure based on CSI trigger state in Step-2 of Mode-A, the association between one PUCCH resource of first PUCCH and one or multiple CSI report configurations, and cross-CC beam report measurement. However, some important issues and details of UEI/ED beam management remain unclear or are still under discussion (FFS/TBD). In this paper, we will discuss the unsolved issues related to UEI/ED beam management, focusing on reducing signaling overhead and beam application latency, as well as other leftover topics. We also provide potential solutions and outline our preferences, particularly for procedure(s) required to facilitate fast beam switching and activation. Other specific considerations include determination of the measurement window for initiating UEI/ED beam reporting, how to deal with the case of multiple UE initiated beam report procedures occurring, whether multiple UEI/ED beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH, and how NW confirms beam(s) reported by the UEI/ED beam report.

# Discussion

## UEI/ED beam management for reducing overhead and latency

### Overhead and beam application latency reduction after UEI/ED beam report

In terms of beam application latency reduction after UEI/ED beam report, the following agreement was reached during RAN1#119 meeting [1]:

Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception. - Other alternatives are not precluded.

To reduce beam application latency after UEI/ED beam report, three alternatives (i.e., Alt-1, Alt-2, and Alt-3) were proposed and discussed as above in previous RAN1 meetings.

In Alt-1, after receiving confirmation/acknowledgement from NW, UE can directly apply the reported new beam without requiring additional RRC reconfiguration message or MAC-CE activation signaling, as illustrated in Figure 1.

For instance, if Event-2 or Event-7 is fulfilled, but the reported new beam(s) are not included in the preconfigured TCI state pool, UE can update the preconfigured TCI state pool and the activated TCI state list based on the reported new beam(s) after confirmation/acknowledgement from NW. This eliminates the need for the RRC reconfiguration message and MAC CE activation command typically required in legacy beam management procedure, replacing them with just a confirmation or acknowledgment from the NW. Some may argue that the reported new beam(s) can be always preconfigured in the original TCI state pool, therefore the RRC reconfiguration of TCI state pool is not needed.  However, in high-frequency scenarios, it is common for UE to experience challenges, especially when moving at high speeds. Considering the potentially long interval between the configuration of the original TCI state pool and the occurrence of the triggering event, it becomes difficult for the NW to predict all the potential new beams the UE may experience and preconfigure them in the original TCI state pool. As a result, it is highly likely that the new beam(s) will not be included in the original TCI state pool. With the legacy approach, updating the original TCI state pool would require an additional RRC reconfiguration message. Furthermore, if Event-2 or Event-7 is fulfilled and the reported new beam(s) are included in the preconfigured TCI state pool, UE can update the activated TCI state list based on the reported new beam(s) after confirmation/acknowledgement from the NW. In this case, the MAC CE activation/deactivation command of legacy beam management procedure is also not needed. This approach (i.e., Alt-1) reduces both signaling overhead and beam application latency in UEI/ED beam management, as we discussed in previous meetings.

Since UE determines the reported new beam(s)/reference signal(s) (RS(s)) through its own beam measurements, it inherently obtains the QCL properties of these reported beam(s)/RS(s) before sending the UE-initiated beam report, particularly for beam(s)/RS(s) satisfying event conditions. After sending the beam report, UE can store the QCL properties (e.g., those of the SSB) associated with the RS(s) reported in the beam report, without the need of additional SSB reception for activating the corresponding beam(s).

From our perspective, the reported new beam(s)/RS(s) in the UEI/ED beam report that satisfy event conditions should be used as soon as possible to update the preconfigured TCI state pool/activated TCI state list and/or switch the serving beam (depending on the event type). Delays in utilization may render the QCL properties stored by the UE obsolete, as channel conditions can change rapidly over time, particularly in FR2 and for high-velocity UEs. This could render the information in the UEI/ED beam report ineffective and should be avoided.

To address this, Alt-1 replaces the time consuming signaling in conventional beam management procedures (such as RRC reconfiguration messages and/or MAC CE activation commands) with just a confirmation indication (e.g., L1 signaling or MAC CE) from the NW. This significantly reduces signaling overhead and beam application latency for updating the preconfigured TCI state pool/activated TCI state list and/or for switching the serving beam. In our view, this method is preferred.

Figure 1: Beam application latency reduction based on confirmation from NW after UEI/ED beam report

Both Alt-2 and Alt-3 aim to reduce beam application latency specifically related to activating new beams based on a legacy TCI state activation command (such as the TCI state activation/deactivation MAC CE). In our understanding, the key difference between Alt-2 and Alt-3 lies in whether/how to convey synchronization information for a new beam reported in the UEI/ED beam report.

In Alt-2, after receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. A reported new beam is considered synchronized by the UE if the UE stores the QCL properties associated with the reported new beam(s) after sending the UEI/ED beam report. However, an FFS point still exists in Alt-2: how to inform a reported new beam in a UEI/ED beam report—whether by introducing a one-bit indicator for each reported new beam or by assuming that all reported new beams are synchronized. In our view, introducing a one-bit indicator in the UEI/ED beam report (i.e., for each reported new beam to indicate whether it is synchronized or not) is unnecessary for two reasons. First, it would lead to high signaling overhead if a one-bit indicator is introduced for each reported new beam. Second, all reported new beams should be assumed to be synchronized, as the UE determines the reported new beam(s)/RS(s) based on its beam measurement and can also obtain the QCL properties of the reported new beam(s)/RS(s) before sending the UE-initiated beam report, especially for those reported new beam(s)/RS(s) that meet event criteria.

In Alt-3, after sending a UE-initiated beam report, the UE may store the QCL properties of the SSB associated with the reference signal reported in the beam report. In such a case, upon receiving a subsequent Unified TCI States Activation/Deactivation MAC CE, the UE can activate new beam(s) without additional SSB reception.

If all reported new beams are assumed to be synchronized (i.e., the UE can obtain the QCL properties of the reported new beam(s)/RS(s) before sending the UE-initiated beam report), Alt-2 is equivalent to Alt-3. In essence, both Alt-2 and Alt-3 aim to reduce beam application latency related to activating new beams based on the legacy TCI state activation command (i.e., the TCI state activation/deactivation MAC CE) without additional SSB reception. However, the TCI state activation/deactivation MAC CE is encapsulated within a transport block and transmitted over the PDSCH channel. While the associated HARQ process ensures reliability for the transport block, it may also contribute to increased latency in activating the new beam. Moreover, if the new beam reported in the UEI/ED beam report falls outside the preconfigured TCI state pool for Alt-2 and Alt-3, an additional RRC reconfiguration message will be required, potentially further increasing the latency of new beam application.

To provide a clearer and more accurate comparison, the three alternatives are summarized based on two cases (i.e., new beam(s) being within or outside the preconfigured TCI state pool) in Table 1 below:

Table 1: Comparisons of beam application latency and signaling overhead for the three alternatives

| case {1}/ case {2} | Latency of case {1} that new beam is outside the preconfigured TCI state pool | Signaling overhead of case {1} that new beam is outside preconfigured TCI state pool | Latency of case {2} that new beam is within preconfigured TCI state pool | Signaling overhead of case {2} that new beam is within preconfigured TCI state pool |
| --- | --- | --- | --- | --- |
| Alt-1 | Low | Low | Low [L1 signaling] | Low |
| Alt-2 | High [Legacy signaling] | High [Legacy signaling] | Medium [Legacy signaling] | Low [Legacy signaling] |
| Alt-3 | High [Legacy signaling] | High [Legacy signaling] | Medium [Legacy signaling] | Low [Legacy signaling] |

Based on the above discussions and analysis, we have the following observations and proposals:

Observation 1: The reported new beam(s) in the UEI/ED beam report that satisfy event conditions should be used as soon as possible to update the activated TCI state list/preconfigured TCI state pool as delays in utilization may render the QCL properties stored by the UE obsolete.

Observation 2: Alt-1 can significantly reduce signaling overhead and beam application latency for both cases of reported new beam(s) being within and outside preconfigured TCI state pool.

Observation 3: For Alt-2 and Alt-3, the latency and overhead are higher than those of Alt-1, especially for the case of reported new beam(s) being outside preconfigured TCI state pool.

Proposal 1: All reported new beams should be assumed to be synchronized, as UE determines the reported new beams/RS(s) based on its beam measurements.

Proposal 2: To reduce beam application latency after sending UE-initiated beam report, a unified solution, for both cases of reported new beam(s) being outside and within the preconfigured TCI state pool, is preferred.

Proposal 3: To reduce beam application latency after sending UE-initiated beam report, support Alt-1.

- Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

  - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

  - update TCI state(s) with the reported new beam(s)

  - activate new beam(s) without additional SSB reception

### Confirmation procedures for UE-initiated/event-driven beam report

As mentioned earlier, the UEI/ED beam report including the reported new beam(s), together with the direct confirmation from NW (i.e., Alt-1 in Section 2.1.1), can significantly reduce signaling overhead and latency for fast beam switching or update at the UE. For instance, as discussed in Section 2.1.1, if the UEI/ED beam report is used to update new TCI state(s)/beam(s) in the preconfigured TCI state pool (a scenario where TCI state configuration with RRC reconfiguration message is required in legacy beam management), direct confirmation from the NW can reduce beam application latency by about 30 ms due to the removal of RRC reconfiguration message and additional SSB reception [2]. Even if the UEI/ED beam report is only utilized to update the active TCI state list (where no RRC reconfiguration is necessary), direct confirmation from the NW can still reduce latency by over 20 ms due to the removal of additional SSB reception [2].

Regarding the confirmation of the UEI/ED beam report, two options can be considered.

In the first option, the confirmation procedure for the beam failure recovery of the SpCell in the current standard can serve as a reference or baseline. For instance, in current standard, if the contention-free Random-Access Preamble for beam failure recovery request was transmitted by UE for beam failure recovery procedure of SpCell, UE may monitor for a PDCCH transmission, on a dedicated search space indicated by “recoverySearchSpaceId” of the SpCell, identified by the C-RNTI. If notification of a reception of a PDCCH transmission on the dedicated search space indicated by “recoverySearchSpaceId” is received from lower layers on the Serving Cell where the preamble was transmitted and PDCCH transmission is addressed to the C-RNTI, UE may consider the Random-Access procedure/beam failure recovery procedure is successfully completed for the SpCell.

Therefore, the confirmation procedure for UEI/ED beam report can be leveraged based on the confirmation procedure for beam failure recovery procedure for SpCell, in particular for aspects such as, a dedicated search space (i.e., L1 signalling) can be defined and used for the confirmation and indicating which reported new beam(s) can be used for beam switching/update by the UE, as illustrated in Figure 2 below.

Figure 2: Confirmation procedure for the UEI/ED beam report indicating reported beam(s) by UE for beam switching/update

In the second option, a new MAC CE or an enhancement of the legacy TCI state activation/deactivation command MAC CE may be used for confirming the UEI/ED beam report. The MAC CE may indicate one or multiple beams of the reported new beams as a confirmation from the NW. The UE can then use the confirmed reported beam(s) to update the preconfigured TCI state pool, if needed, without requiring an additional RRC reconfiguration message, and also update the activated TCI state list.

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 4: The confirmation for the UEI/ED beam report can either leverage the confirmation procedure of the beam failure recovery procedure of the SpCell or utilize a new MAC CE (or an enhancement of the legacy TCI state activation/deactivation command MAC CE).

### Update preconfigured TCI state pool and activated TCI state list

To update the preconfigured TCI state pool and the activated TCI state list, both two scenarios (i.e., Case 1 and Case 2 as aforementioned) need to be considered. A potential solution is that each of the new beam(s) can be preconfigured with an associated TCI state, which may be outside the preconfigured TCI state pool for Case 1 or within the preconfigured TCI state pool for Case 2. In addition, the association between a new beam and a preconfigured TCI state can be updated, such as by a MAC CE.

Case 1: The reported new beam is outside the preconfigured TCI state pool

In this case, both the preconfigured TCI state pool and the activated TCI state list need to be updated by the UE based on the reported new beam(s) after receiving the confirmation from NW, as illustrated in Figure 3 below. The confirmation from NW may explicitly indicate one or multiple beams of the reported new beam(s). The confirmation may also not explicitly indicate any of the reported new beam(s), which implies that all the reported new beam(s) are confirmed. The UE can then update both the preconfigured TCI state pool, and the activated TCI state list based on TCI state(s) associated with the confirmed reported new beam(s).

Figure 3: The reported new beam(s) being outside the preconfigured TCI state pool

Case 2: The reported new beam is within the preconfigured TCI state pool

In this case, only the activated TCI state list needs to be updated by the UE based on the reported new beam(s) after receiving the confirmation from NW, as illustrated in Figure 4 below. As in Case 1, the confirmation from NW may explicitly indicate one or multiple beams of the reported new beam(s). The confirmation may also not explicitly indicate any of the reported new beam(s), which implies that all the reported new beam(s) are confirmed. The UE can then update the activated TCI state list based on TCI state(s) associated with the confirmed reported new beam(s).

Figure 4: The reported new beam(s) being within the preconfigured TCI state pool

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 5: Support that each of the new beam(s) can be preconfigured with an associated TCI state and the confirmation from NW can indicate one or multiple beams of the reported new beam(s). The TCI state(s) associated with the confirmed reported new beam(s) are used to update the activated TCI state list and the TCI state pool, if applicable.

## UL signaling content(s) for UEI/ED beam report

Regarding UL signaling content(s) for UEI/ED beam report, the below agreements were reached during RAN1#119 and RAN1#120 meetings [1][3]:

Agreement On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured - Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured. - Option-3: No further enhancement. Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2 FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A. Agreement On UE-initiated/event-driven beam reporting, Option-1 is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - Note: The presence of this field is only for the case that N > 1 and the time window and M are configured Above is NOT applicable for event 1.

In RAN1#120 meeting, Option-1 was agreed to extend the L1-RSRP report format for UEI/ED beam report, specifically addressing Event-2.  However, an open issue remains regarding how to handle the case where the UE has not sent any initial PUCCH associated with UEI/ED beam reporting in Mode-A. We understand that if Event 2 is met, the UE should have no reason not to send the first PUCCH for UEI/ED beam reporting. Therefore, a more reasonable scenario is that Event 2 is not met, and the UE is not required to send the first PUCCH for UEI/ED beam reporting. However, the UE may still receive an uplink grant from the network, triggering the UEI/ED beam reporting.

Additionally, two potential solutions were discussed in RAN1#120 meeting [4]. The first solution involves revising the RAN1#117 agreement to state that “at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event 2 only for Mode B” in a UEI/ED beam report. The second solution suggests relying on UE/NW implementation, such as network blind detection.

Regarding the first solution, we believe it should be ensured that “at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event 2” applies to both Mode A and Mode B. Otherwise, UEI/ED beam reporting may lose its relevance and value for the network. Therefore, revising such a critical agreement for this case is not reasonable and should not be supported.

As for the second solution, which depends on UE/NW implementation, network blind detection may not be necessary or useful here since it was already agreed in RAN1#120 meeting that a CSI trigger state related to UEI/ED beam reporting cannot be associated with a legacy AP-CSI report configuration [3]. In that regard, the NW can clearly differentiate whether UEI/ED beam reporting is received or not based on the uplink grant scheduling. However, a new rule can be defined for the UE to address this issue.  For example, the UE does not expect to receive an uplink grant triggering the UEI/ED beam reporting if it has not sent any first PUCCH associated for UEI/ED beam reporting.

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 6: If UE has not sent any first PUCCH associated for UEI/ED beam reporting, UE does not expect to receive an uplink grant triggering the UEI/ED beam reporting.

## UL signaling medium/container considering the UE-initiated/event-driven UL transmission for beam report

During RAN1#120 meeting, the below agreements on beam report transmission procedure for UEI/ED beam reporting were reached [3]:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, regarding Event-2. - Only a single UEI beam report is carried in the second PUSCH. - Additional indication of one ‘CSI report configuration’ is provided in the report format. - The CSI report configurations associated with the same PUCCH resource are ordered in ascending order of corresponding CSI-ReportConfigId, the number of bits of the additional indication field is ceil(log2(N_CSIconfig)), where the N_CSIconfig denotes the number of CSI report configurations associated with the same PUCCH resource. - The payload size of the single UEI beam report is determined according to the max payload size among the associated CSI report configurations. - Zero-padding can be appended if the payload size of the UEI beam report is less than the max report payload. - The reported UEI beam report should satisfy the triggering condition. - If multiple UE initiated beam report procedures occur, down-select one of the following options: - Option-1: It is up to UE implementation to select one of configuration. - Option-2: The UEI beam report with highest priority is reported - Option-3: The report triggered in the latest measurement is reported in PUSCH - The multiple CSI report configurations associated with the one first PUCCH resource should be configured in the same CC. - Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - For Mode-B, the multiple CSI report configurations associated with the same PUCCH resource should be associated with the same second configured PUSCH FFS: Alt-2 Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH. - If RAN1 cannot converge on the support of Alt-2 in RAN1#120bis, this alternative will be dropped from Rel-19 Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rule for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules, i.e., the first PUCCH is dropped. - FFS: whether/how to handle the case of different PHY priorities.

Regarding handling multiple UE-initiated beam report procedures associated with the same PUCCH resource, three options (i.e., Option-1, Option-2, and Option-3) were proposed and discussed in RAN1#120 meeting for further down-selection.

For Option-1, the selection of a configuration is determined by the UE implementation. The UEI/ED beam report is based on UE’s beam measurement and event evaluation, allowing the UE greater flexibility in the beam reporting. Similarly, if multiple UE-initiated beam report procedures occur for the same PUCCH resource, UE should also have the flexibility to select one of the configurations.

For Option-2, the UEI beam report with the highest priority is transmitted. From a system perspective, this approach ensures better performance by enabling timely UEI/ED beam reporting with the highest priority. However, the priority definitions for different configurations need further clarification before finalizing the selection, as relying solely on the CSI configuration index to decide the priority may not be the optimal choice.

For Option-3, the report triggered in the latest measurement is reported in PUSCH. This option provides the least flexibility for the UE in performing UEI/ED beam reporting. Additionally, the most recent measurement may not always yield the most desirable UEI/ED beam report from a performance standpoint, potentially compromising overall system performance for this option.

Based on the above discussions and analysis, we have the following proposal:

Proposal 7: Both Option-1 and Option-2 are viable considerations, whereas Option-3 is not preferred.

    - Option-1: It is up to UE implementation to select one of configuration.

    - Option-2: The UEI beam report with highest priority is reported

    - Option-3: The report triggered in the latest measurement is reported in PUSCH

Regarding the Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState, we support this working assumption from both complexity and efficiency perspectives. Otherwise, configuring multiple dedicated CSI-AperiodicTriggerStates for different CSI report configurations would significantly reduce resource efficiency and increase system complexity. Therefore, we have the following proposal:

Proposal 8: Confirm the Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState.

Regarding multiple UEI beam reports associated with the same PUCCH resource, an open FFS issue remains:

Alt-2 – Multiple UEI beam reports associated with the same PUCCH resource for the first PUCCH can be transmitted in the second PUSCH.

In our view, adopting two solutions to address the same problem is unnecessary. If a single PUCCH resource of the first PUCCH is associated with multiple CSI report configurations, it has already been agreed that only one UEI beam report will be carried in the second PUSCH. Additionally, some details of this solution, such as the method for selecting a single UEI beam report from multiple CSI report configurations, have been proposed and discussed.

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 9: DO NOT support Alt-2.

    - Alt-2: Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH

Regarding the multiplexing/dropping rules for a 1-bit first PUCCH that collides or overlaps with a PUSCH, three options (Option-1, Option-3, and Option-4) were proposed and discussed in RAN1#120 meeting for further down selection.

- Option-1: Prioritize the first PUCCH over the PUSCH, meaning the PUSCH is dropped. This approach may significantly impact PUSCH transmission performance, particularly when the PUSCH utilizes a large size of uplink resources and in URLLC-type services.

- Option-4: Reuse the SR dropping rules, where the first PUCCH is dropped. This option does not guarantee timely and reliable transmission of the UEI/ED beam report, as the first PUCCH may be discarded. As a result, it could negatively affect beam management performance, especially in fast beam management scenarios at high frequencies.

- Option-3: A compromise between Option-1 and Option-4, this approach piggybacks a 1-bit indication of the first PUCCH into the PUSCH. This ensures the transmission performance of both the UEI/ED beam report and the PUSCH.

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 10: Support Option-3.

  - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

    - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

## Trigger-event detection for UEI/ED beam report

On the definition of event to drive UEI/ED beam report, the below agreement was reached during RAN1 #120 meeting [3]:

Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, further study the following options for possible down-selection - Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured. - Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured. - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s). - Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW. - Option-4: If an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: T_window is the agreed time window parameter for measurement. - Note: Other options are not precluded.

Regarding the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, four options (i.e., Option-1, Option-2, Option-3, and Option-4) were proposed and discussed in RAN1#120 meeting for further down selection.

For Option-1, the measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, as shown in the below Figure 5-1, and T_proc is RRC configured. In our view, Option-1 may be too restrictive in requiring the time offset between the end of the measurement window and the transmission occasion of the first PUCCH to be exactly T_proc. In practice, it is sufficient if the end of the measurement window is before T_PUCCH - T_proc, ensuring the UE has adequate time to prepare for the first PUCCH transmission. Nevertheless, Option-1 remains a viable approach for determining the measurement window.

Figure 5-1: Measurement window for Option-1

For Option-2, the measurement window is from T_Instance – T_window to T_Instance, as shown in the below Figure 5-2, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured. In addition, the UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s). However, it is unclear how T_proc is applied in Option-2. Nonetheless, Option-2 can offer greater flexibility than Option-1 regarding restrictions imposed by T_PUCCH. Furthermore, it appears that the measurement window in Option-2 is determined solely by the last event evaluation instance, T_Instance, within the window. This approach can simplify measurement window determination, reduce sliding window maintenance complexity, and may even replace measurement window maintenance with a simple event satisfaction check. Given these advantages, in our view, Option-2 should also be considered.

Figure 5-2: Measurement window for Option-2

For Option-3, the length, slot offset, and periodicity of a measurement window are pre-configured per CSI report configuration by NW. In our view, determining the measurement window solely based on high layer signaling configuration is neither accurate nor efficient. For instance, in this option, measurement reference signals (RSs) falling outside the predefined window cannot be used for event evaluation, leading to wasted measurement resources as well as reduced event evaluation accuracy and reliability. Moreover, a potential risk for this approach is that the pre-configured measurement window may not directly align with physical layer signals relevant to the beam measurement report, such as the first PUCCH transmission or event evaluation instances. Therefore, this option is not preferred.

For Option-4, if an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window), as shown in the below Figure 5-3. This option also has greater flexibility, as the measurement window is not constrained by T_PUCCH or other preconfigured parameters. Additionally, unlike Option-2, where the measurement window is based on the last event evaluation instance, this approach determines the starting point of the measurement window (or timer) based on the first event evaluation instance within the measurement window (or timer). However, for each event evaluation instance, a respective measurement window (or a timer) needs to be (re)started and maintained until the measurement window is expired. This approach significantly increases complexity of management and maintenance for the measurement windows for UE, which is not preferred.

Figure 5-3: Measurement window for Option-4

In summary, the four options can be presented as in Table 2 below:

Table 2: Comparisons of four options for measurement window determination

|  | Flexibility | Accuracy | Reliability | Complexity |
| --- | --- | --- | --- | --- |
| Option-1 | Low | High | High | Low |
| Option-2 | High | High | High | Low |
| Option-3 | Low | Low | Low | Medium |
| Option-4 | High | High | High | High |

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 11: Option-1 and Option-2 can be considered for further discussions.

  - Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured.

  - Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured.

    - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s).

# Conclusions

Based on above discussions, we have the following observations and proposals:

Observation 1: The reported new beam(s) in the UEI/ED beam report that satisfy event conditions should be used as soon as possible to update the activated TCI state list/preconfigured TCI state pool as delays in utilization may render the QCL properties stored by the UE obsolete.

Observation 2: Alt-1 can significantly reduce signaling overhead and beam application latency for both cases of reported new beam(s) being within and outside preconfigured TCI state pool.

Observation 3: For Alt-2 and Alt-3, the latency and overhead are higher than those of Alt-1, especially for the case of reported new beam(s) being outside preconfigured TCI state pool.

Proposal 1: All reported new beams should be assumed to be synchronized, as UE determines the reported new beams/RS(s) based on its beam measurements.

Proposal 2: To reduce beam application latency after sending UE-initiated beam report, a unified solution, for both cases of reported new beam(s) being outside and within the preconfigured TCI state pool, is preferred.

Proposal 3: To reduce beam application latency after sending UE-initiated beam report, support Alt-1.

- Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

  - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

  - update TCI state(s) with the reported new beam(s)

  - activate new beam(s) without additional SSB reception

Proposal 4: The confirmation for the UEI/ED beam report can either leverage the confirmation procedure of the beam failure recovery procedure of the SpCell or utilize a new MAC CE (or an enhancement of the legacy TCI state activation/deactivation command MAC CE).

Proposal 5: Support that each of the new beam(s) can be preconfigured with an associated TCI state and the confirmation from NW can indicate one or multiple beams of the reported new beam(s). The TCI state(s) associated with the confirmed reported new beam(s) are used to update the activated TCI state list and the TCI state pool, if applicable.

Proposal 6: If UE has not sent any first PUCCH associated for UEI/ED beam reporting, UE does not expect to receive an uplink grant triggering the UEI/ED beam reporting.

Proposal 7: Both Option-1 and Option-2 are viable considerations, whereas Option-3 is not preferred.

    - Option-1: It is up to UE implementation to select one of configuration.

    - Option-2: The UEI beam report with highest priority is reported

    - Option-3: The report triggered in the latest measurement is reported in PUSCH

Proposal 8: Confirm the Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState.

Proposal 9: DO NOT support Alt-2.

    - Alt-2: Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH

Proposal 10: Support Option-3.

  - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

    - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

Proposal 11: Option-1 and Option-2 can be considered for further discussions.

  - Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured.

  - Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured.

    - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s).

# References

Draft_Minutes_report_RAN1#119_v030.

TS 38.133, NR: Requirements for support of radio resource management (Release 16).

Draft_Minutes_report_RAN1#120_v020

R1-2500943 Moderator Summary #1 on UEIBM - FINAL - ZTE, RAN1#120.