3GPP TSG RAN WG1 #120bis                                                     R1-2501877

Wuhan, China, April 07th – 11th, 2025

Agenda Item:	9.6.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on LP-WUS operation in CONNECTED modes

Document for:	Discussion and decision

Introduction

It was agreed in WID [1] to specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS.

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

LP-WUS monitoring occasions

In RAN1#120, the following agreements on LP-WUS monitoring occasions configuration had been achieved [2].

Agreement For LP-WUS MOs in connected mode for Option 1-1, support Approach 1: - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start - UE monitors the first Z LP-WUS MO(s) at or after Time offset2. - FFS: How to determine the value of Z - Time offset2 is RRC configured - UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined. Agreement For LP-WUS MOs in connected mode for Option 1-2, support Approach 1: - LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - FFS one or multiple MOs per periodicity - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4. - The time offset4 configured by the network indicating a time, after which the UE starts PDCCH monitoring via starting the new timer. - FFS: Definition of time offset4

In current specification, for DCI 2-6, the PDCCH monitoring occasions are the ones in the first Ts slots indicated by duration, or Ts=1 slot if duration is not provided, starting from the first slot of the first Ts slots and ending prior to the start of drx-onDurationTimer. The UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots, where X corresponds to the MinTimeGap or MinTimeGapFR2-2 value of the SCS of the active DL BWP.

TS 38.213

- an offset by ps-Offset indicating a time, where the UE starts monitoring PDCCH for detection of DCI format 2_6 according to the number of search space sets, prior to a slot where the drx-onDurationTimer would start on the PCell or on the SpCell [11, TS 38.321] - for each search space set, the PDCCH monitoring occasions are the ones in the first Ts slots indicated by duration, or Ts=1 slot if duration is not provided, starting from the first slot of the first Ts slots and ending prior to the start of drx-onDurationTimer.

If a UE reports for an active DL BWP a MinTimeGap or MinTimeGapFR2-2 value that is X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots, where X corresponds to the MinTimeGap or MinTimeGapFR2-2 value of the SCS of the active DL BWP in Table 10.3-1. Table 10.3-1 Minimum time gap value X SCS (kHz); Minimum Time Gap X (slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. 480; 8; 96. 960; 16; 192.

In our view, for the determination of the value of Z and X, the same mechanism as DCP can be reused.

Proposal 1: For the determination of the value of Z and X, the same mechanism as DCP can be reused.

Activation/deactivation of LP-WUS monitoring

In RAN1#116, the following agreement on activation/deactivation of LP-WUS monitoring had been achieved [3].

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Regarding signaling of activation/deactivation of LP-WUS monitoring, the following schemes can be considered.

- Activation/deactivation of LP-WUS monitoring by gNB RRC signaling

- Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling, e.g., DCI or MAC CE.

- Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer

In our view, the activation and deactivation of LP-WUS listeners depends on the arrival of traffic. gNB can activate the LP-WUS monitoring when there is no traffic and deactivate the LP-WUS monitoring when traffic arrives. It may be not flexible enough to trigger activation/deactivation of LP-WUS monitoring by RRC signaling. Therefore, we prefer activation/deactivation of LP-WUS monitoring by gNB L1 signaling, e.g., DCI.

Proposal 2: Activation/deactivation of LP-WUS monitoring by gNB L1 signaling (e.g., DCI) can be considered.

LP-WUS Procedures to trigger PDCCH monitoring in CA/DC case

In RAN1#120, the following agreements on LP-WUS Procedures to trigger PDCCH monitoring in the case where a UE is configured with CA/DC in RRC CONNECTED mode had been achieved.

Agreement - For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell - LP-WUS indication is applicable to all serving cells - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell or can be RRC configured cell other than PCell Agreement - For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell per cell-group - This does not imply that LP-WUS has to be configured on both cell groups - LP-WUS indication is applicable to all serving cells in the cell-group - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS whether to support this case as separate UE capability from CA case - FFS UE capability for monitoring LP-WUS on one cell group or both cell groups - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell

For the case when UE is configured with CA without dual DRX groups, the serving cell on which the LP-WUS is configured can be RRC configured cell other than PCell.

For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA, the serving cell on which the LP-WUS is configured can be RRC configured cell other than Pcell/PSCell.

Proposal 3: For the case when UE is configured with CA without dual DRX groups, the serving cell on which the LP-WUS is configured can be RRC configured cell other than PCell.

Proposal 4: For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA, the serving cell on which the LP-WUS is configured can be RRC configured cell other than Pcell/PSCell.

Coexistence with existing UE power saving features

In RAN1#120, the following agreements on coexistence with existing UE power saving features had been achieved.

Agreement - LP-WUS can be configured with legacy PDCCH skipping. - Note: There is no intention to modify legacy PDCCH skipping Agreement - For RRC CONNECTED mode, LP-WUS can be configured with Rel-17 SSSG switching Agreement - For RRC CONNECTED mode, LP-WUS can be configured with Cell DTX - FFS During Cell DTX inactive time, whether the UE is expected to monitor LP-WUS or not

Considering the power consumption of LP-WUS monitoring is small, flexibility in network scheduling and reducing the service latency, during Cell DTX inactive time, the UE is expected to monitor LP-WUS.

Proposal 5: During Cell DTX inactive time, the UE is expected to monitor LP-WUS.

QCL/TCI-state for LP-WUS

In previous RAN1 meeting, the following agreements on QCL/TCI-state for LP-WUS had been achieved.

Agreement For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET Agreement - For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS - FFS applicable QCL type(s) Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable. Agreement For LP-WUS monitoring in RRC CONNECTED mode, select one of the options on how to define/configure/indicate QCL source of LP-WUS - Opt1: Separate indication/configuration for QCL source of LP-WUS - Opt2: Association of QCL source between LP-WUS and PDCCH/CORESET

Regarding on how to define/configure/indicate QCL source of LP-WUS, we prefer separate indication/configuration for QCL source of LP-WUS.

Proposal 6: Separate indication/configuration for QCL source of LP-WUS should be supported.

Conclusion

In summary, we have following proposals:

Proposal 1: For the determination of the value of Z and X, the same mechanism as DCP can be reused.

Proposal 2: Activation/deactivation of LP-WUS monitoring by gNB L1 signaling (e.g., DCI) can be considered.

Proposal 3: For the case when UE is configured with CA without dual DRX groups, the serving cell on which the LP-WUS is configured can be RRC configured cell other than PCell.

Proposal 4: For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA, the serving cell on which the LP-WUS is configured can be RRC configured cell other than Pcell/PSCell.

Proposal 5: During Cell DTX inactive time, the UE is expected to monitor LP-WUS.

Proposal 6: Separate indication/configuration for QCL source of LP-WUS should be supported.

Reference

- RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, CMCC, RAN#102, Dec. 11th - 15th, 2023.

- 3GPP RAN1#120 Chairman’s Notes, February 17th – 21st, 2025.

- 3GPP RAN1#116 Chairman’s Notes, Orlando, US, November 18th – 22nd, 2024.