3GPP TSG RAN WG1 #120bis	R1-2501920

Wuhan, China, April 7th – 11th, 2025

Title: 	Discussion on other aspects of AI/ML model and data

Source: 	ZTE Corporation, Sanechips

Agenda item:	9.15.1

Document for:	Discussion/Decision

Introduction

In RAN1#120bis meeting, the UE feature discussion for AI/ML use cases for air interfaces will start. In this document, we provide our analysis and proposals for the first set of UE features for AI/ML use cases for air interfaces.

Discussion

General principles

The Rel-19 UE features for AI/ML use cases for air interfaces will be the first set of AI/ML UE features in 3GPP for physical layer. The related UE features should be carefully designed in order to facilitate the potential commercialization of AI/ML use cases for physical layer and establish a solid foundation for 6G discussion. Below we provide two general principles for designing the UE features for AI/ML use cases in physical layer.

Principle#1: Balance between UE capability report and applicability report

RAN2 has designed detailed procedure for applicability report for AI beam prediction and AI POS enhancement [1].

RAN2 further has made following agreements and signalling procedure (see the attached figure) on applicable functionality reporting for beam management UE-sided model: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

Basically, the UE feature report is more like static reporting, while the applicability report is more like semi-static reporting. On easier example to better understand the difference between UE feature report and applicability report is hardware capability and software capability, although it is not 100% accurate. UE feature report is more related to hardware capability, while applicability report is more related to software capability that may be subject to infrequent update.

Designing UE features for AI/ML use cases for physical layer should consider the balance between UE feature report and applicability report. If the design leans towards the UE capability reporting, it will result with UE capability with extreme finer granularity. If the design leans towards the applicability reporting, it will result with frequent applicability reporting.

Principle#2: Balance between UE implementation flexibility and UE implementation fragmentation

The UE feature design should always prioritize the balance between UE implementation flexibility and UE implementation fragmentation. Finer granularity of UE capability reporting leads to higher UE implementation flexibility at the cost of higher UE implementation fragmentation, which is not friendly for the network implementation and commercialization. Rel-19 will be the first release for AI/ML use cases for physical layer, it is essential to prioritize the balance between UE implementation flexibility and UE implementation fragmentation to set solid foundation for future commercialization.

Proposal 1: Consider the following two principles for Rel-19 UE feature design for AI/ML use cases for physical layer

- Principle#1: Balance between UE capability report and applicability report

- Principle#2: Balance between UE implementation flexibility and UE implementation fragmentation

AI beam prediction

Overall, there are 4 different sub-use-cases for AI beam prediction:

- Beam prediction case 1 with NW side model

- Beam prediction case 2 with NW side model

- Beam prediction case 1 with UE side model

- Beam prediction case 2 with UE side model

For beam prediction case 1 with NW side model and case 2 with NW side model, the main enhancement at UE side is to report more than 4 beam related information in L1 signalling. It can be transparent to the UE whether the reported more than 4 beam related information is for NW side beam prediction or for other potential use cases. From this perspective, one UE feature for more than 4 beam related information in L1 signalling can be sufficient for beam prediction case 1 and 2 with NW side model. While for beam prediction case 1 and case 2 with UE side model, separate basic UE feature groups for them may be needed.

Proposal 2: For Rel-19 AI beam prediction, three basic UE feature groups are sufficient

- One basic UE feature group for more than 4 beam related information in L1 signalling

- One basic UE feature group for beam prediction case 1 with UE side model

- One basic UE feature group for beam prediction case 2 with UE side model

Overall, we think the following UE feature design can be considered.

Proposal 3: Consider the following UE features for AI beam prediction in Rel-19.

| Index | Feature group | Components | Granularity |
| --- | --- | --- | --- |
| B-1 | More than 4 beam related information in L1 signalling | In a beam report initiated by network, based on one measurement resource set, support the report of M beam related information in L1 signaling - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRPof a measurement resource set Maximum value of M is one of {8, 16, 32, 64，[128, 256]} and up to UE report. Note: M = 4 has already been supported by UE feature 2-29 | per UE |
| B-2-1 | Basic UE feature group for case 1 with UE side model | Inference measurement - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference - Support the CSI-RS resource types: Periodic (P) CSI-RS, Semi-persistent (SP) CSI-RS, Aperiodic (AP) CSI-RS Inference report - The beam information in the inference report refers to the resource set for Set A - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A. - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - Report type: Periodic (P) CSI report, Aperiodic (AP) CSI report Performance monitoring - Option1 FFS: CPU related information | per band |
| B-2-2 | Option 2 performance monitoring for beam case 1 with UE side model | Option 2 performance monitoring for beam case 1 with UE side model | per band |
| B-3-1 | Basic UE feature group for case 2 with UE side model | Inference measurement - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference - Support the CSI-RS resource types: Periodic (P) CSI-RS, Semi-persistent (SP) CSI-RS, [Aperiodic (AP) CSI-RS] Inference report - for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - UE report the maximum value of N = [1, 2, 4, 8] - By default, UE supports all the time gap between two consecutive future time instances, time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report. - for inference report, support periodic CSI report, aperiodic CSI report. Performance monitoring - Option1 FFS: CPU report |  |
| B-3-2 | Option 2 performance monitoring for beam case 2 with UE side model | Option 2 performance monitoring for beam case 1 with UE side model | per band |

AI POS enhancement

At the initial stage of Rel-19, 5 sub use cases are defined for AI POS enhancement, i.e.,

- Direct AI/ML positioning:

  - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

  - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

  - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- AI/ML assisted positioning

  - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

  - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

During RAN#107 meeting, it is agreed that the 2nd priority use cases won’t be pursued in Rel-19. In other words, only the following three AI POS enhancement use cases will be specified in Rel-19.

- Direct AI/ML positioning:

  - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

  - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- AI/ML assisted positioning

  - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

Among these three 1st priority use cases, case 3a is with gNB side model and case 3b is with LMF side model. The UE side operation is the same as what we have, i.e., UE only needs to transmit SRS to the base station. From this perspective, there is no need to define any new UE feature for AI POS enhancement case 3a and case 3b.

Proposal 4: New UE feature for AI POS enhancement case 1 is needed, while no need to define any new UE feature for AI POS enhancement case 3a and case 3b.

Overall, we think the following UE feature design can be considered.

Proposal 5: Consider the following UE features for AI POS enhancement in Rel-19.

| Index | Feature group | Components | Granularity |
| --- | --- | --- | --- |
| P-1 | Inference result report for Rel-19 enhanced positioning case 1 | Support of inference result report for Rel-19 enhanced positioning case 1 Support of performance monitoring outcome report for Rel-19 enhanced positioning case 1 | per UE |

There are also some other UE features related to the legacy POS methods, more discussion may be needed to determine whether UE can follow the legacy UE feature or new UE features are required.

- Common DL PRS Processing Capability, e.g., FG 13-1

- DL PRS Resources, e.g., FG 13-3, 13-3a, 13-3b

- Support of PRS measurement in RRC_INACTIVE state, e.g., FG 27-18a

Proposal 6: RAN1 further discusses whether new UE features are required for the following aspects specifically for the AI POS enhancement case 1.

- Common DL PRS Processing Capability, e.g., FG 13-1

- DL PRS Resources, e.g., FG 13-3, 13-3a, 13-3b

- Support of PRS measurement in RRC_INACTIVE state, e.g., FG 27-18a

AI CSI prediction

Since the AI CSI prediction discussion in Rel-19 has just started for one meeting. It is proposed to prioritize UE feature discussion for other AI use cases first and come back to the UE feature for AI CSI prediction in May or August meeting.

Proposal 7: RAN1 prioritizes UE feature discussion for other AI use cases first and come back to the UE feature discussion for AI CSI prediction in May or August meeting.

Conclusion

In this contribution, we provide our analysis and proposals for the UE feature design for Rel-19 AI/ML use cases for air interface.

General principle

Proposal 1: Consider the following two principles for Rel-19 UE feature design for AI/ML use cases for physical layer

- Principle#1: Balance between UE capability report and applicability report

- Principle#2: Balance between UE implementation flexibility and UE implementation fragmentation

AI beam prediction

Proposal 2: For Rel-19 AI beam prediction, three basic UE feature groups are sufficient

- One basic UE feature group for more than 4 beam related information in L1 signalling

- One basic UE feature group for beam prediction case 1 with UE side model

- One basic UE feature group for beam prediction case 2 with UE side model

Proposal 3: Consider the following UE features for AI beam prediction in Rel-19.

| Index | Feature group | Components | Granularity |
| --- | --- | --- | --- |
| B-1 | More than 4 beam related information in L1 signalling | In a beam report initiated by network, based on one measurement resource set, support the report of M beam related information in L1 signaling - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRPof a measurement resource set Maximum value of M is one of {8, 16, 32, 64，[128, 256]} and up to UE report. Note: M = 4 has already been supported by UE feature 2-29 | per UE |
| B-2-1 | Basic UE feature group for case 1 with UE side model | Inference measurement - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference - Support the CSI-RS resource types: Periodic (P) CSI-RS, Semi-persistent (SP) CSI-RS, Aperiodic (AP) CSI-RS Inference report - The beam information in the inference report refers to the resource set for Set A - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A. - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - Report type: Periodic (P) CSI report, Aperiodic (AP) CSI report Performance monitoring - Option1 FFS: CPU related information | per band |
| B-2-2 | Option 2 performance monitoring for beam case 1 with UE side model | Option 2 performance monitoring for beam case 1 with UE side model | per band |
| B-3-1 | Basic UE feature group for case 2 with UE side model | Inference measurement - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference - Support the CSI-RS resource types: Periodic (P) CSI-RS, Semi-persistent (SP) CSI-RS, [Aperiodic (AP) CSI-RS] Inference report - for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - UE report the maximum value of N = [1, 2, 4, 8] - By default, UE supports all the time gap between two consecutive future time instances, time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report. - for inference report, support periodic CSI report, aperiodic CSI report. Performance monitoring - Option1 FFS: CPU report |  |
| B-3-2 | Option 2 performance monitoring for beam case 2 with UE side model | Option 2 performance monitoring for beam case 1 with UE side model | per band |

AI POS enhancement

Proposal 4: New UE feature for AI POS enhancement case 1 is needed, while no need to define any new UE feature for AI POS enhancement case 3a and case 3b.

Proposal 5: Consider the following UE features for AI POS enhancement in Rel-19.

| Index | Feature group | Components | Granularity |
| --- | --- | --- | --- |
| P-1 | Inference result report for Rel-19 enhanced positioning case 1 | Support of inference result report for Rel-19 enhanced positioning case 1 Support of performance monitoring outcome report for Rel-19 enhanced positioning case 1 | per UE |

Proposal 6: RAN1 further discusses whether new UE features are required for the following aspects specifically for the AI POS enhancement case 1.

- Common DL PRS Processing Capability, e.g., FG 13-1

- DL PRS Resources, e.g., FG 13-3, 13-3a, 13-3b

- Support of PRS measurement in RRC_INACTIVE state, e.g., FG 27-18a

AI CSI prediction

Proposal 7: RAN1 prioritizes UE feature discussion for other AI use cases first and come back to the UE feature discussion for AI CSI prediction in May or August meeting.