3GPP TSG RAN WG1 #120-bis         						R1- 2502012

Wuhan, China, April 07th – April 11th, 2025

Agenda item:	      9.2.4

Source:	      Tejas Networks Ltd

Title:		      Enhancement for asymmetric DL sTRP/UL mTRP scenarios

Document for:     Discussion and Decision

Introduction

The Rel-19 WID on NR MIMO Phase 5 [1][2] specifies the following objectives on enhancement for asymmetric DL sTRP/UL mTRP scenarios. Based on the WID revision in RAN#105 [3], for asymmetric DL sTRP/UL mTRP deployment scenarios, two TAs for UL transmissions was additionally approved.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g., UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Discussion on enhancements for asymmetric DL sTRP/UL mTR

In this contribution, we provide our views on Rel 19 enhancements for asymmetric DL sTRP/UL mTRP scenarios for the open issues identified in RAN1#120 [4] and previous meetings.

## Two Separate CLPC adjustment states for SRS

Agreement For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state, down-select from the following Alts: - Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no extra enhancement. - Alt2: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17), the SRS with lowest ID is always configured with one joint/UL TCI state, the “referenceSignal” in a UL TCI state is optional. - Alt3: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set, and they are applied on the SRS resources. - Alt4: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one PL offset and one CLPC adjustment state are applied on the SRS resources. - FFS how to define the PL offset and CLPC adjustment state

Aligned with the discussions during RAN1#119 and RAN1#120 meetings, we agree to continue our support for Alt 1 that is aligned with the current specification. In our view, the scenario, where an SRS resource set is not configured with followUnifiedTCI-StateSRS and the SRS resource with lowest ID not configured with a TCI state, can be considered as an error case and can be avoided through gNB implementation.

Regarding PL offset and CLPC adjustment states for SRS, wherein an SRS resource set is not configured with followUnifiedTCI-StateSRS, support Alt1 as a way forward.

## Two TA Enhancement

Based on RAN #105 discussion and the revised WID RP-242394 [3], it has been approved to support two TA for asymmetric DL sTRP/UL mTRP deployments. It is decided to reuse Rel-18 specification of two TAs for multi-DCI-based multi-TRP and remove the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources.

Aligned with this scope extension, the following agreements were made in RAN1 #118bis and RAN1#119 meetings.

Agreement (RAN1 #118bis) Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped. Agreement (RAN1 #119) For a UE supporting the UE capability of “Overlapping UL transmission reduction” of Rel-19 2TA: - The overlapping duration of the latter one of two overlapping UL transmissions is reduced, where the two overlapped UL transmissions are associated with two TAGs.

Traditionally, for asymmetric DL sTRP/UL mTRP scenarios based on S-DCI, only one DL reference timing can be considered, which could be based on the first detected path of the DL frame from the anchor TRP in the TAG. It was agreed in RAN1 #118bis to support one downlink reference timing and apply to both TAGs. This is particularly relevant as there is no downlink reference signal associated with the configured TAG of UL TRP.

With the support for one downlink reference timing applied to both TAGs, the UE will monitor a single DL reference timing and apply both TA commands to the same DL reference timing to determine the UL transmission timings toward the two TAGs.

Regarding two TA enhancement with one downlink reference signal for both TAGs, consider to study the conditions under which UE monitors the reference signal and applies both TA commands for determining two TAs toward two TAGs.

During the offline discussions and follow up agreements of RAN1 #118bis, the necessity of introducing an optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2 TA based asymmetric mTRP was emphasised. In RAN1 #119, aspect of the overlapping duration was discussed and agreed. For the remaining FFS, we provide our views as follows to be considered in RAN1 #120-bis meeting.

If a UE reports “Overlapping UL transmission reduction” capability, it implies that UE expects two overlapping transmissions and can apply the capability to handle all overlapping UL transmissions with repetition including Type A and Type B.

Regarding two TA enhancement, do not support the need to separately address overlapping intra-slot PUSCH type-B repetition transmissions associated with different TAGs.

Conclusions

In this section we summarize our observations and proposals.

With the support for one downlink reference timing applied to both TAGs, the UE will monitor a single DL reference timing and apply both TA commands to the same DL reference timing to determine the UL transmission timings toward the two TAGs.

If a UE reports “Overlapping UL transmission reduction” capability, it implies that UE expects two overlapping transmissions and can apply the capability to handle all overlapping UL transmissions with repetition including Type A and Type B.

Regarding PL offset and CLPC adjustment states for SRS, wherein an SRS resource set is not configured with followUnifiedTCI-StateSRS support Alt1 as a way forward.

Regarding two TA enhancement with one downlink reference signal for both TAGs, consider to study the conditions under which UE monitors the reference signal and applies both TA commands for determining two TAs toward two TAGs.

Regarding two TA enhancement, do not support the need to separately address overlapping intra-slot PUSCH type-B repetition transmissions associated with different TAGs.

References

- RP-234007, “New WID: NR MIMO Phase 5”, Samsung (Moderator), 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023.

- RP-244087, “WID revision: NR MIMO Phase 5”, Samsung (Moderator), 3GPP TSG RAN Meeting #104, Maastricht, Netherlands, March 18-21, 2024.

- RP-242394, “WID revision: NR MIMO Phase 5”, Samsung (Moderator), 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

- Chairman’s Notes, RAN1#120 eom1, Athens, Greece, February 17th – 21st, 2025