3GPP TSG RAN WG1 #120-bis          						R1-2502016

China, Wuhan, April 7th – 11th, 2025

Source:	Tejas Networks Ltd.

Title:	Discussion on modulation aspects for A-IoT physical channel

Agenda item:	9.4.1

Document for:     Discussion and Decision

# Introduction

The A-IoT physical channel has one uplink channel called PDRCH and one downlink channel called PRDCH. The physical channel consists of L1 control information and data. The data is the higher layer information received by L1. The transport blocks for the modulation and signal generation of L1 data frame, L1 control information, and pre/post/mid-amble can be different. This work includes the modulation aspects of PRDCH/PDRCH generation and pre/post/mid-amble generation.

# Modulation Aspects of A-IoT Physical Channel

The section discusses the estimation of maximum M value for R2D transmission and an optimum M-value to be chosen within the maximum M value for each R2D transmission.

# Estimate maximum and optimal value of M for R2D transmission

The R2D transmission bandwidth agreed on RAN1#118 is [1]:

Agreement The following table is a starting point for the study of M values and the associated minimum Btx,R2D value - Reader can use any R2D transmission bandwidth >= minimum Btx,R2D - FFS: Impacts, if any, of CP handling solutions, and other impacts - Note: depending on further study, the maximum value of M may be less than 32 M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 2. 32; 3.

The agreements made in RAN1#120 [2] on the maximum M-value is as follows:

Agreement

For R2D, for the OOK-4 modulation for M-chip per OFDM symbol transmission:

- The maximum M value is no less than 16, and to be down-selected from 32, 24 or 16 at RAN1#120bis.

(a) M = 4                                                                                 (b) M = 8

(c) M = 12                                                                                 (d) M = 24

Fig 1: SNR vs BLER and SNR vs Data rate for M = 4,8,12,24

Table I: SNR and data rate for 10% BLER

| M | SNR (dB) | Data rate (kbps) |
| --- | --- | --- |
| 4 | 6.7 | 25 |
| 8 | 6.8 | 45 |
| 12 | 7.9 | 72 |
| 16 | 8.1 | 94 |
| 24 | 8.8 | 125 |
| 32 | 9 | 160 |

Observation 1: For M = 24, the data rate is above 100kbps and for M = 16, the data rate is below 100kbps.

Proposal 1: We propose M = 24 as the maximum M value, as it provides data rate > 100kbps.

The following are discussed in RP#106 on R2D modulation [3]:

The following objectives are set, within the General Scope:

- RAN1 scope:

  - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively.

  - R2D and D2R signal(s)

  - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA.

  - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations.

According to [2], only OOK-4 modulation is considered for R2D transmission. Thus, it is important to define the M-value for OOK-4 modulation for all DL message signals.

The following is discussed in TR 38.769, section 6.1.2 on L1 R2D control information [4]:

Reference signals including DMRS, PTRS, SRS, are not considered for D2R. CSI feedback and autonomous SR are not considered for L1 D2R control information.

Fig. 2a: Device estimates the M-value and send to reader through msg1.

Fig. 2b: Reader estimates the M-value from the known preambles received through msg1.

According to [1], reference signals are not considered for D2R to provide CSI feedback to the reader. Therefore, we propose the following three options to decide the optimum M value.

Option 1: Msg0 contains known preamble from which the device can estimate the channel condition and choose an optimum M value from a lookup table stored in the device and convey the same through msg1 (Fig. 2a).

Option 2: Msg1 contains known preamble from which the reader can estimate the channel condition and choose an optimum M value from a lookup table stored in the reader and the M value will be used for subsequent DL transmissions (Fig. 2b).

Option 3: An optimum value can be decided based on the following parameters:

- Received signal strength of the UL message signals received form the device

- Block error rate

- Data rate and latency

Observation 2: As R2D signals are transmitted using OOK-4 generated by OFDM modulator, it is important to define the M-value for OOK-4 modulation for all DL message signals.

Proposal 2: we propose the following three options to decide the optimum M value.

    - Option 1: Msg0 contains known preamble from which the device can estimate the channel condition and choose an optimum M value from a lookup table stored in the device and convey the same through msg.

    - Option 2: Msg1 contains known preamble from which the reader can estimate the channel condition and choose an optimum M value from a lookup table stored in the reader and the M value will be used for subsequent DL transmissions.

    - Option 3: An optimum value can be decided based on the following parameters:

          - Received signal strength of the UL message signals received form the device

          - Block error rate

          - Data rate and latency

# CP Handling for R2D

The CP is added to R2D signals which should be known to the device to decode the signal. The CP length also depends on the M-value. Initially msg0 can be transmitted at a default M-value which has a default CP length. The preamble sequence consists of two parts: 1) start indicator and 2) clock-acquisition. By decoding the length of clock-acquisition signal, the device estimates the M-value of the received PRDCH signal. Since CP length is dependent on the M-value, the device can estimate the CP length of the received PRDCH signal form the M-value.

Observation 3: In order to decode the PRDCH frame including the control information, the device should have the information of CP length of the OOK-4 modulated signal generated from OFDM modulator even before receiving the control information.

Proposal 3: CP length can be derived as a function of M-value, which is detected from the length of clock-acquisition signal. The device estimates the M-value of the received PRDCH signal from the clock-acquisition signal and derive the CP length form the M-value.

# Conclusion

This work includes the modulation aspects of A-IoT Physical Channel, estimation of optimum M-value for R2D signal generation, and CP handling for R2D. We have made the following observations and proposals related to the above-mentioned aspects of A-IoT:

Observation 1: For M = 24, the data rate is above 100kbps and for M = 16, the data rate is below 100kbps.

Proposal 1: We propose M = 24 as the maximum M value, as it provides data rate > 100kbps.

Observation 2: As R2D signals are transmitted using OOK-4 generated by OFDM modulator, it is important to define the M-value for OOK-4 modulation for all DL message signals.

Proposal 2: we propose the following three options to decide the optimum M value.

    - Option 1: Msg0 contains known preamble from which the device can estimate the channel condition and choose an optimum M value from a lookup table stored in the device and convey the same through msg.

    - Option 2: Msg1 contains known preamble from which the reader can estimate the channel condition and choose an optimum M value from a lookup table stored in the reader and the M value will be used for subsequent DL transmissions.

    - Option 3: An optimum value can be decided based on the following parameters:

          - Received signal strength of the UL message signals received form the device

          - Block error rate

          - Data rate and latency

Observation 3: In order to decode the PRDCH frame including the control information, the device should have the information of CP length of the OOK-4 modulated signal generated from OFDM modulator even before receiving the control information.

Proposal 3: CP length can be derived as a function of M-value, which is detected from the length of clock-acquisition signal. The device estimates the M-value of the received PRDCH signal from the clock-acquisition signal and derive the CP length form the M-value.

# References

[1] 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #118, Maastricht, Netherlands, August 19th – 23rd, 2024.

[2] 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #120, Athens, Greece, Feb 17th – 21st, 2025.

[3] RP-243326, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

[4] TR 38.769, Study on solutions for ambient IoT (Internet of Things) in RAN, V2.0.0.