3GPP TSG RAN WG1 #120bis	R1- 2502139

Wuhan, China, 7 – 11 April 2025

Agenda item:		9.15.8

Source:	Nokia

Title:	Views on Rel-19 NTN phase 3 UE features

Release:	Rel-19

Document for:		Discussion and Decision

# Introduction

This contribution presents our initial views on Rel-19 NR over NTN phase 3 UE features.

# Discussion

Based on the agreements reached so far for NR over NTN phase 3, the following UE features are suggested as a starting point for discussions.

| Index | Feature group | Components | Prerequisite feature groups | Candidate values |
| --- | --- | --- | --- | --- |
| 65-1 | NTN DL coverage enhancements | Support for extended SSB periodicity with a maximum value of 160 ms. [Support for inter-slot PDCCH repetitions for Type-0 CSS] [Support for PDSCH repetitions messages containing SIB1] Support for PDSCH repetitions for messages containing Msg4 as part of the random access procedure. | 26-1, |  |
| 65-2 | NTN enhancements for support of HD-FDD RedCap and eRedCap UEs. | Support for [additional] collision rules for collision case 3 (collision between semistatic DL and semistatic UL, where Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS], and Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS). Support for [additional] collision rules for collision case 4 (collision between dynamic scheduled downlink and dynamic scheduled uplink, [further definition of use cases needed]). Support for network configuration of UL priority for collision case 3. Support for network configuration of [UL,DL] priority for collision case 4. | 26-1, [26-4] (TA reporting), 28-1 (RedCap), 28-3 (HD-FDD RedCap), 48-1 (eRedCap) |  |
| 65-3 | NTN UL capacity enhancements | Support for transmitting PUSCH with repetitions, where the PUSCH is subject to orthogonal cover codes (OCC) Support for RV cycling across OCC groups (terminology may change). Support for OCC with sequence length as indicated. | 26-1, One of {5-14, 5-16, 5-17, 5-17a, 30-1, 30-2} (PUSCH repetitions), 44-2 (DMRS bundling) | Supported OCC lengths: {Length 2 (default), Length 2 and Length 4} |

# Conclusion

In this contribution we have proposed our views on Rel-19 NR over NTN phase 3 UE features.