3GPP TSG RAN WG1 #120bis																	  R1-2502149

Wuhan, China, April 7th–11th, 2025

Source: 	CMCC

Title:	Discussion on specification support for beam management

Agenda item:	9.1.1

Document for:	Discussion & Decision

Introduction

In RAN1#120 meeting [1], the following agreements were made regarding the spec impacts of AI based beam management.

Agreement For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size. Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value. Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion. Agreement For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report. Agreement For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8] - Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report. Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model. Agreement For UE-sided model, in CSI-ReportConfig for inference - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

In this contribution, we focus on the discussion on the the procedure and the potential specification impact of AI based beam management during different LCM operation.

Discussion on specification impact of data collection

In this section, we discuss the specification impact of data collection for training, inference and monitoring.

Data collection of NW-side model for training

  - 2.1.1 Reporting Content

In RAN1#119 meeting, FL has provided following proposal about content for NW-side data collection for training for higher layer report.

For #120 Proposal C Send LS to RAN 2, to inform RAN 2 that from RAN 1 perspective, for content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, the following three types of contents for each instance can be considered: - Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE - Type 2: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information of K beams based on some resources (for Set A) configured to UE - Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE - Wherein the K beams are the beams with the largest K L1-RSRPs based on the resources (for Set A) in one instance - K is configured by NW.

Data collection content for different purposes can be different. For example, data collection content for inference is only related to L1-RSRPs and/or beam information based on one resource set, data collection content for monitoring depends on monitoring KPI. We prefer to discuss data collection content for different purposes independently, and focus on data collection content for training in this section.

With Type 1, gNB has flexibility to train classification model or regression model, but Type 1 may require UE with high storage capability. Type 2 is deprioritized since it is restricted to classification model. Type 3 is a compromise of Type 1 and Type 2, and is compatible with classification model and regression model with K configured by NW. For classification model, K can be small. For regression model, K can be equal to total number of RS in set A and all L1-RSRPs of RS for set A are collected. Type 3 is also suitable for UE with different capability, gNB can configure K value depends on UE capability. Above all, considering different output of AI model and UE capability, Type 1 and 3 are supported.

Proposal 1: From RAN 1 perspective, for content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, type 1 and type 3 for each instance can be supported:

- Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE

- Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE

For L1 signaling, since existing CSI framework can be reused and latency of report is small, L1 signaling is also supported for training data collection. To reduce reporting overhead of each sample/instance, reported number of beams can be reduced by setting RSRP threshold (low RSRP that contributes less to beam prediction is omitted) or new quantization method (e.g. temporal difference among multiple past time instances for BM-Case2). Bitmap can be used as beam information to represent RSRP of which beam is reserved.

Proposal 2: L1 signaling can be supported for NW-sided training data collection.

  - 2.1.2 Configuration of Set A

Either best Rx beam or quasi-optimal Rx beam assumption is considered, beam number of set A may be larger than 64. Existing UE capability for RS measurement and CSI reporting framework need enhancement.

As described in TS 38.213 and TS 38.331, existing UE capability on maximum number of RS for RSRP measurement is up to 64 for each resource set and up to 64 for all resource set across all CCs. With quasi-optimal Rx beam assumption, UE capability for RS number per resource set needs extension and only one periodic/semi-persistent resource set is needed, or the number of resource set in one CSI-ResourceConfig needs extension, to support measurement of set A with more than 64 RS.

TS 38.213 - 5.2.1.4.1 Resource Setting configuration A UE is not expected to be configured with more than 64 NZP CSI-RS resources and/or SS/PBCH block resources in resource setting for channel measurement for a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', 'cri-RI-CQI', 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR' or 'ssb-Index-SINR'. If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP' or 'ssb-Index-RSRP', - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting. - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters. - 5.2.1.4.3 L1-RSRP Reporting For L1-RSRP computation - the UE may be configured with CSI-RS resources, SS/PBCH Block resources or both CSI-RS and SS/PBCH block resources, when resource-wise quasi co-located with 'QCL-Type C' and 'QCL-TypeD' when applicable. - the UE may be configured with CSI-RS resource setting up to 16 CSI-RS resource sets having up to 64 resources within each set. The total number of different CSI-RS resources over all resource sets is no more than 128. TS 38.331 BeamManagementSSB-CSI-RS ::= SEQUENCE { maxNumberSSB-CSI-RS-ResourceOneTx ENUMERATED {n0, n8, n16, n32, n64}, maxNumberCSI-RS-Resource ENUMERATED {n0, n4, n8, n16, n32, n64}, maxNumberCSI-RS-ResourceTwoTx ENUMERATED {n0, n4, n8, n16, n32, n64}, supportedCSI-RS-Density ENUMERATED {one, three, oneAndThree} OPTIONAL, maxNumberAperiodicCSI-RS-Resource ENUMERATED {n0, n1, n4, n8, n16, n32, n64} }

With best Rx beam assumption, existing periodic/semi-persistent CSI reporting framework can not support unified Tx beam and Rx beam sweeping with more than 64 RS within a short time. Based on existing spec, one periodic/semi-persistent CSI-ReportConfig is associated with one resource set. If repetition is on, the resource set is configured for Rx beam sweeping. If repetition is off, the resource set is configured for Tx beam sweeping, the Rx beam determination is up to UE implementation. If multiple CSI-ReportConfig associated with resource set with repetition on is used for unified beam sweeping, the required number of CSI-ReportConfig is as large as the number of Tx beams. If multiple CSI-ReportConfig associated with resource set with repetition off is used for unified beam sweeping, the required number of CSI-ReportConfig is small, e.g. the same as the number of Rx beams. But UE does not assume to switch Rx beam among different resource set, resulting in incomplete unified beam sweeping. Thus, CSI reporting enhancement is needed to support unified beam measurement for data collection.

With quasi-optimal Rx beam assumption, only Tx beam sweeping is needed, Rx beam can refer to QCL source. To perform Tx beam sweeping with more than 64 RS within each period, one possible way is to extend maximum number of RS for RSRP measurement per resource set. Time offset of each RS can be different so that the number of RS in each slot still satisfies measurement of maximum 64 RS in each slot. Another possible way is to configure multiple resource sets in one CSI-ResourceConfig to constitute set A, where time offset of each resource set is different. This enhancement on CSI reporting framework is also suitable for configuration of set A for inference and monitoring.

Proposal 3: Regarding data collection for NW-side model, following option is supported for the configuration of Set A:

- Option 2: The configuration of set A can contain multiple resource sets in one CSI-ResourceConfig

Data collection of UE-side model for training

  - 2.2.1 UE request for data collection

If data collection is performed at the UE side, there is no need to disclose the detailed Rx beam information to the network, but some basic information like the required number of samples, the required number of Tx beams in Set B and Set A should be reported to the network.

Different AI models/algorithms may have different preferred set B pattern, and UE reports model-specific preferred set B pattern when requests for data collection for training. If set B pattern transmitted by gNB is determined by AI model/algorithm, the RS overhead during training and inference is large. To reduce RS overhead at gNB, gNB can pre-configure several set B patterns, and then based on UE’s reporting, gNB chooses to transmit one or multiple set B pattern to UE for generating the input data of AI model.

Proposal 4: Regarding data collection of UE-side model for training, UE can report or request gNB for the preferred set B (pattern).

  - 2.2.2 Configuration of set A

Similar as data collection for NW-side model, measurement of Tx beams in a short time for each sample is preferred, The number of resource set needs enhancement, the configuration method of set A for NW-side model can be reused.

Proposal 5: Regarding data collection of UE-side model, the configuration of set A can contain multiple resource sets in one CSI-ResourceConfig.

Discussion on specification impact of inference

Model inference may perform at NW side or UE side. To increase the prediction accuracy, Top-K best beams can be predicted by AI model and the best beam can be selected by measuring the L1-RSRP of Top-K best beams. In this section, we discuss the necessity of Top-K beam sweeping procedure during inference and specification impact of inference.

Model inference at NW side

The model inference procedure of Tx beam prediction at NW side is shown in Fig. 1.

- Firstly, gNB transmits sparse Tx beams in Set B, where Set B beam pattern is cell-specific.

- In step 2, UE reports the L1-RSRP of measured beams.

- In step 3, gNB inputs L1-RSRP of measured beams into AI model and outputs the index and L1-RSRP of Top K best beams of Set A..

- In step 4, gNB jointly sweeps Top K beams and all Rx beams. UE measures L1-RSRP of Top K best beams.

- In step 5, UE reports the index and L1-RSRP of practical Top 1 best beam or part of Top K best beams. Legacy beam reporting mechanism is reused.

- In step 6, gNB indicates the beam for DL data transmission. Legacy beam indication mechanism is reused.

Fig. 1 inference at NW side

  - 3.1.1 Necessity of Top-K beam sweeping procedure

One controversial issue is whether top K beam sweeping is necessary. The comparison of with and without top K beam sweeping procedure from the perspective of beam prediction accuracy, RS overhead, delay, TCI state indication is summarized in Table 1.

Table 1 comparison of with and without top K beam sweeping procedure

|  | with top K beam sweeping | without top K beam sweeping |
| --- | --- | --- |
| beam prediction accuracy | - >90% | - ~80% |
| RS overhead | - comparatively large additional RS measurement overhead | - small RS measurement overhead |
| delay | - Top K beam sweeping delay - known TCI state switching delay is short | - unknown TCI switching delay includes RSRP measurement and SSB measurement delay as defined in 38.133 - The delay difference is small between with and without top K beam sweeping |
| TCI state indication | - if target TCI state is in the active TCI state list for PDSCH/PDCCH: DCI based beam indication - if target TCI state is not in the active TCI state list for PDSCH/PDCCH: MAC-CE based beam activation | - known target TCI indication: DCI or MAC-CE - unknown target TCI indication: MAC-CE based beam activation |

With top K beam sweeping, gNB can indicate top 1 genie-aided beam based on measured RSRP. Top K/1 beam prediction accuracy can be larger than 90% if K is approximately chosen. Top K beam sweeping introduces comparatively large additional RS measurement overhead for UE. Considering the worst case, gNB needs to transmit aperiodic RS in set A after each round of inference.

Between inference and new target TCI state applies, Top K beam sweeping delay and known TCI state switching delay are included. As defined in 38.133, if the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length, where

-	Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TSSB-proc = 2 ms;

-	TOk = 1 if target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

If target TCI state is in the active TCI state list for PDSCH/PDCCH, TCI state switch delay is short and DCI based beam indication is performed. If target TCI state is not in the active TCI state list for PDSCH/PDCCH, TCI state switch delay includes SSB measurement delay, and MAC-CE based beam activation is performed.

Without top K beam sweeping, gNB performs beam indication based on predicted RSRP, top 1 beam prediction accuracy can be 10%~20% lower than top K/1 beam prediction accuracy based on evaluation results in SI phase. Beam indication depends on gNB, then target TCI may be known TCI, target TCI can also be unknown TCI if it is not measured within 1280 ms.

For unknown target TCI indication, only MAC-CE based beam activation is workable, unknown target TCI needs to be measured after MAC-CE activation command to acquire QCL information. For a specific UE, only one unknown target TCI needs to be measured after inference, additional RS measurement overhead for UE is small. RS overhead for gNB is the same as with top K beam sweeping, since in the worst case gNB needs to transmit aperiodic RS in set A after each inference.

Between inference and new target TCI state applies, unknown TCI state switching delay may be included. As defined in 38.133, if the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc)) / NR slot length, where

-	T L1-RSRP = 0 in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise,

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

-	TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement when TCI state switching involves QCL-TypeD

-	TOuk = 1 when TCI state switching involves other QCL types only

-	Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD;

-	Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE for other QCL types;

Without top K beam sweeping, L1-RSRP measurement and SSB measurement are also needed for CSI-RS beam, the total delay is similar to that of with top K beam sweeping procedure.

Based on above analysis, top K beam sweeping procedure introduces some RS measurement overhead but also provides higher beam prediction accuracy, the delay between inference and new target TCI state application is similar to that of without top K beam sweeping procedure. Thus top K beam sweeping procedure can be introduced. Whether top K beam sweeping procedure is enabled is configurable by gNB.

Proposal 6: Top K beam sweeping procedure can be supported to obtain Top 1 beam.

One problem to configure Top K beam sweeping is Top K beams often change after each inference. If periodic/semi-persistent CSI report is used for Top K beam sweeping, set A is associated with the CSI report. A DL signaling is needed to indicate Top K beam subset for measurement before each report, signaling overhead is large. If aperiodic CSI report is used for Top K beam sweeping, one trigger state can be associated with multiple CSI report and multiple resource sets. Since the maximum number of trigger states for aperiodic CSI reporting and maximum number of resource sets associated with one trigger state is limited, it is difficult to configure all combinations of Top K beams and also occupies too many trigger states and resource sets oriented for other purposes. The method to indicated all combinations of Top K beams for aperiodic CSI reporting with low signaling overhead needs further discussion.

Proposal 7: The indication of Top K beam (resource) set with low signaling overhead can be further discussed.

In RAN1#120 meeting, FL summarized following solutions of Top-K beam sweeping:

- Option 1: NW indicates that the corresponding beam IDs/TCI states for the P2 ResourceSet follows the UE predicted Top-K beams, - Option 2: NW configures a number of aPeriodicTriggerStates, and the UE provides Top-K predictions restricted to the valid TCI states matching one of the aPeriodicTrigger states, - Option 3: For both UE/NW sided model, NW configures dynamically as part of the MAC CE the beam IDs or TCI states that are part of the Top-K beam sweep. FFS on details. - Option 4: The NW uses legacy signaling, and in a best-effort manner configure a set of aPeriodicTriggerStates - Option5: NW configures all Set A measurement resources, and UE is expected to measure only predicted top-K beams after each inference result. - Note 1: Option 2 and 4 would imply reduced performance since P2 cannot support all Top-K combinations - Note 2: NW indicating beam IDs could be used during monitoring

As we discussed above, Option 2 and 4 intend to configure all combinations of Top K beams, which occupies too many trigger states and resource sets oriented for other purposes. For Option 2, to make sure Top K beam combination is included in trigger states, the reported Top K beam id is impacted by configured trigger state. This will degrade model performance. Top K beam sweeping can match Top K beams in inference report in best-effort manner, but reported Top K beam id match Top K beam sweeping does not make sense.

Option 1 and 3 are preferred but the difference between option 1 and 3 is not clear. Option 3 provides more detail, where MAC CE indicates beam IDs or TCI states of Top-K beams or subset of Top-K beams. We think MACCE is an appropriate signaling to indicate variable Top-K beam combination with low latency.

Option 5 needs more details. It is acceptable if gNB indicates UE to measure only predicted top-K beams within set A. But UE should not determine to measure Top-K beams or subset of Top-K beams within set A by implementation.

Proposal 8: For Top-K beam sweeping, NW indicates the beam IDs or TCI states that are part of the Top-K beams by MAC CE.

  - 3.1.2 Reporting content

In RAN1#117 meeting, following agreement regarding reporting content for inference at NW-sided model has been achieved.

Agreement (RAN1#117) For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

If L1-RSRPs of Top M beams are reported and M<the size of the measurement resource set, CRI/SSBRI is the baseline of beam information. Bitmap can also be supported when bitmap has lower payload than CRI/SSBRI.

Proposal 9: For NW-sided model inference reporting, at least for BM-Case 1, when the value of M (Top-M beams) is smaller than the size of measurement resource set, Option 1 is supported:

- Option 1: CRI/SSBRI

Since M is a fixed value configured by gNB, the drawback is if M is small, some beams with high L1-RSRP beside Top M beams are omitted. If M is large, some beams with low L1-RSRP inside Top M beams are reported, resulting in waste of UL resource.

To remain beams with high L1-RSRP in report content and save UL resource, one flexible method is to report L1-RSRPs and corresponding beam information of beams within X dB gap to the largest measured L1-RSRP. In this case, the payload size of reporting content may change. Similar to two part report of CSI typeI/typeII codebook, reporting content can be divided into part I and part II, part I includes beam information (e.g. bitmap) with fixed size, part II includes differential L1-RSRPs of variable number of beams. The UL resource allocation for variable payload size needs further discussion.

Proposal 10: For NW-sided model inference reporting, at least for BM-Case 1, L1-RSRPs and corresponding beam information of beams within X dB gap to the largest measured value of L1-RSRP can be supported.

- The X dB gap can be configured by gNB

- UL resource allocation for variable number of reported beams can be further discussed.

  - 3.1.3 TCI indication for beam in set A but not in set B

After inference, gNB may choose a beam in set A but not in set B. In the following, we discuss the TCI indication in this case.

One possible way is set A is configured and transmitted for monitoring, or top K beam sweeping is performed. Then TCI indication is associated with measured RS resource and can reuse legacy unify TCI framework.

Another possible way is gNB directly activate an unknown TCI state based on predicted RSRP, TCI indication is associated with virtual RS without transmission. Here, virtual RS in set A is more like aperiodic resource with resource configuration but not transmitted. Based on existing unknown TCI activation timeline, gNB should trigger transmission of this virtual RS once for aperiodic measurement at UE to obtain QCL parameters.

Above all, the RS associated with TCI indication should be measured at least once before TCI application. The UE cannot consider a TCI indication associated with virtual RS without transmission as known TCI state since QCL parameters are unknown. This applies for both NW-sided model and UE-sided model.

Proposal 11: The RS associated with TCI state indication should be measured at least once before TCI state indication or application.

Model inference at UE side

  - 3.2.1 Necessity of Top-K beam sweeping procedure

The model inference procedure of Tx beam prediction at UE side is shown in Fig. 2.

- Firstly, gNB transmits sparse Tx beams in Set B, where Set B beam pattern may be fixed or variable for different UEs. The detailed configuration of beams in Set B is up to gNB or reported by UE.

- In step 2, UE inputs L1-RSRP of measured beams into AI model and outputs index and L1-RSRP of Top K best beams among all beams.

- In step 3, UE reports the index of predicted Top K beams, the corresponding L1-RSRP can also be reported optionally.

- In step 4, gNB jointly sweeps Top K beams and all Rx beams. UE measures L1-RSRP of Top K best beams.

- In step 5, UE reports the index and L1-RSRP of practical Top 1 best beam or part of Top K best beams. Legacy beam reporting mechanism is reused.

- In step 6, gNB indicates the beam for DL data transmission. Legacy beam indication mechanism is reused.

Fig. 2 inference at UE side

Similar as inference at NW side, Top K beam sweeping procedure is configurable by gNB.

  - 3.2.2 Combination of BM-Case 1 and BM-Case 2

RAN1 has achieved following agreement for UE-sided model inference for BM Case 1 and BM Case 2. For BM Case 2, the following agreement is on the premise that BM Case 1 and BM Case 2 can be combined, otherwise one resource set for set A in the CSI report configuration is enough. However, WID and existing RAN1 agreement have not mentioned the combination of BM Case 1 and BM Case 2. We think one explicit agreement is needed to allow combination of BM Case 1 and BM Case 2.

Agreement For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A Agreement For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration, - Two CSI-ResourceConfigId s are configured for Set A and Set B separately

Proposal 12: The combination of BM Case 1 and BM Case 2 can be supported.

  - 3.2.3 Configuration of set B for BM-Case 2

For BM-Case 2 of UE-side model, for inference, periodic/aperiodic/semi-persistent CSI report are supported, but only periodic and semi-persistent CSI-RS resource types for CMR are supported since aperiodic CSI report of aperiodic CSI-RS introduces PUSCH out-of -order issue.

Proposal 13: For BM-Case 2 of UE-side model, for inference, aperiodic resource type for CMR is not supported.

  - 3.2.4 Inference results reporting

In prior RAN1 agreement, beam information and/or RSRP of Top K beams have been supported as quantity of inference results reporting. The definition of predicted Top K beam(s) is still not clear. Since AI model output can be RSRP or probability, predicted Top K beams can be defined as the K beams with largest predicted RSRP values or the K beams with the largest probability of each beam in Set A to be the Top-1 beam.

Proposal 14: For inference results reporting of UE side model, predicted Top K beams are the K beams with largest predicted RSRP values or the K beams with the largest probability to be the Top-1 beam in Set A.

The concept of probability of each beam to be the Top-1 beam is used to define Top K beams, but the probability is not required to report in inference result reporting. gNB can configure report quantity of Top 1 beam id to know Top 1 beam with highest probability. If gNB configures report quantity of Top K beam id, it implies that the prediction accuracy of Top K beams is high. And the beam chosen for data transmission also depends on traffic load of different beams, reporting probability seems do not provide more useful information for beam selection.

Proposal 15: For inference results reporting of UE side model, probability information of predicted Top K beams does not need to be reported.

In the case of Top K beam id reporting, the ranking within one beam reporting may not be needed. If gNB configures CSI report with report quantity of Top K beam id, gNB trusts the predicted Top K beams of AI model has high accuracy, otherwise gNB can configure report quantity of Top 1 beam id or configure Top K beam sweeping to find Top 1 beam.

Proposal 16: In the case of Top K beam id reporting, the ranking within one beam reporting may not be needed.

In RAN1#120 meeting, following agreement about configuration of inference results reporting for BM-Case 2 has achieved.

RAN1#120 Agreement For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8] - Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

In RAN1#120 meeting, it was agreed that both N and time gap of future time instances are configured by gNB, the reference time of the earliest future time instance was defined. To determine the slot of earliest future time instance, the time offset between reference time and earliest future time instance is needed. Different AI model can predict future time instance with different time offset, thus the time offset can be configured by gNB. When RS in set B is SSB and RS in set A is CSI-RS, even if time gap of future time instances and periodicity of set B are the same, the time offset is needed to avoid overlapping of SSB and CSI-RS.

Proposal 17: For UE-side AI/ML model inference of BM-Case2, the time offset between reference time and earliest future time instance can be configured by gNB.

- The predicted time instance can be determined by the slot of reference time+offset+P*(N-1)

  - 3.2.5 Associated ID and applicable functionality reporting

To deal with the consistency of NW-side additional condition across training and inference for UE-sided model, RAN1 118 meeting has agreed to support associated ID per cell.

RAN1#118 Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Regarding the similar properties information behind the associated ID, since it may relate to proprietary information of gNB and also the commercial deployment strategy, it is not necessary to enforce gNB to disclose those information. Namely there is no need to define the content of the similar properties behind the associated id. If there is a need, an offline alignment for the content of the NW side additional condition can be carried out between gNB and UE vendors. Within a cell, the candidate properties can include the transmission filter, the order of resources (corresponding to beams) for Set A/B, the Tx power of gNB, the antenna height and down tilt, and deployment scenarios. If gNB considers some properties within a certain range will not impact inference performance, these properties can be mapped to a same associated ID.

Proposal 18: There is no need to define the content of the similar properties behind the associated id.

Based on the discussion above, since the content of the similar properties behind the associated id will not be defined, it is not necessary to mandate the gNB to configure the associated id for the inference configurations. Then the associated id can be optionally configured.

The associated id is defined to keep the consistency between the training and inference. With the associated id aligned between the gNB and UE, the inference results can satisfy the performance requirements since the training and inference are carried out in the same scenarios. But without the associated id, though the performance may degrade due to the mismatch of the additional conditions between training and inference, it does not mean that the model for inference cannot work in the current scenario. Our understanding is that, with the UE capability reporting, gNB can provide a proper inference configuration for UE to work. Though without the associated id, the inference performance of the model can be monitored based on the model monitoring procedure.

Observation 1: With the UE capability reporting, gNB can provide a proper inference configuration for UE to work even without the configuration of associated id. The inference performance of the model can be monitored based on the model monitoring procedure.

Even without associated ID for data collection/inference, the UE can determine whether the functionality is applicable or not with the inference configuration or inference related parameters provided in Step 3 of applicable functionality reporting, and with further information of UE-side additional conditions and model availability in devices. The applicable functionality reporting procedure is still workable. In this case, whether UE can assume similar properties of a DL Tx beam set associated with the same cell is up to UE implementation, UE may assume some similar properties of set A for example when configuration of set A never changes during data collection and inference.

If the associated ID is configured for data collection/inference, but associated ID in CSI-ReportConfig for inference cannot match associated ID of supported functionalities, UE still may report these CSI-ReportConfig as applicable functionalities based on the related configuration or the configuration information. This is because based on RAN2 agreed terminologies, applicable functionalities refer to functionalities that the UE is ready to apply for inference. The definition of applicable functionality does not include the matching of associated ID and does not require the AI functionality’s performance. Whether the AI functionality can be applied does not depend on the matching of associated ID and the performance of inference. If UE thinks one functionality has good generalization capability and wants to try the performance of the functionality, UE can report it as applicable and may assume some similar properties of DL Tx beam set up to implementation. Above all, the matching of associated ID will not be used for the determination of the applicable functionality.

Proposal 19: The associated ID can be optionally configured for inference.

Proposal 20: When the associated ID is configured for data collection/inference, the determination of applicable functionality can be based on the configuration or the related information but without associated id. Namely, the associated id will not be used for the determination of the applicable functionality.

During RAN1#119 meeting, different cases for the configuration of associated id were listed for discussion [3].

//For the following cases, I think the discussion point can be Case 2-1. Further study whether to support the following cases for configuration of the associated ID: - Case 1: The associated ID is configured for data collection. - Case 1-0: The associated ID is configured for inference/step 3 - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - Case 1-1: The associated ID is not configured for inference/step 3. - Alt 1: UE may report it is not applicable - Alt 2: UE may report it is applicable, and UE assume the similar properties of a DL Tx beam or beam set/list for Set A/Set B associated with in the same cell. - Alt 3: UE may report it is applicable, but UE cannot assume anything. - Case 2: The associated ID is not configured for data collection - Alt A: UE can report the feature is not supported. - Alt B: If UE can support Case 2: - Case 2-0: The associated ID is configured for inference - This case is not supported - Case 2-1: The associated ID is not configured for inference/step 3. - Alt 1: UE may report it is not applicable - Alt 2: UE may report it is applicable, and UE assume the similar properties of a DL Tx beam or beam set/list for Set A/Set B associated with in the same cell. - Alt 3: UE may report it is applicable, but UE cannot assume anything. - Note: The supported cases/alternatives can be up to UE capability.

In Case1, the associated id is configured for data collection. The associated id can either be configured for the inference or not. When the associated id is configured in the inference configuration and the associated is matched with the one for the data collection, then the UE may assume the similar properties of a DL Tx beam or beam set/list. But when the associated id is not configured for inference, it is also possible for UE to assume that the model can work under current gNB’s additional conditions. UE determines whether the model can work depending on the configuration information and the UE side additional conditions (e.g. battery status, memory status). Also concluded by RAN2 reply LS that, applicable functionalities refer to functionalities that the UE is ready to apply for inference. Whether the functionalities are applicable or not is not related to the associated id.

RAN2 agreed the following understandings on terminologies: Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) Applicable functionalities refer to functionalities that the UE is ready to apply for inference Activated functionalities refer to functionalities already enabled for performing inference.

Observation 2: According to the definition from RAN2, whether the functionalities are applicable or not is not related to the associated id.

Then when the associated id is not configured for inference, UE can still determine whether the model is applicable or not. UE can either report the mode is applicable or not applicable. When UE report it is applicable, it can maintain the assumption of gNB’s additional conditions as that for data collection.

Proposal 21: When the associated ID is configured for data collection but not configured for inference (Case 1-1), the UE can report it is either applicable or not applicable.

Proposal 22: When the associated ID is configured for data collection but not configured for inference (Case 1-1), the UE can report it is applicable and have the same assumption of gNB’s additional conditions as that for data collections.

When the associated ID is not configured for data collection, UE can either report the feature is supported or not in Case 2. If the UE can support the Case 2 and the associated id is not configured for inference, the UE can still report it is applicable or not applicable. When the UE report the model is applicable, UE can assume the same properties as for training. Namly, the UE can assume the similar properties of a DL Tx beam or beam set/list for Set A/Set B associated with in the same cell. Since there is no associated id for data collection and inference configuration, there could be a mismatch between the training and inference. The model monitoring can be used to solve the performance issue.

Proposal 23: When the associated ID is not configured for data collection and not configured for inference (Case 2-1), the UE can report it is either applicable or not applicable.

Proposal 24: When the associated ID is not configured for data collection and not configured for inference (Case 2-1), the UE can report it is applicable and have the same assumption of gNB’s additional conditions as that for data collections.

RAN1#119 meeting has agreed following option A and/or option B for applicable functionality reporting.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Regarding the configuration location of associated ID, based on reply LS of applicable functionality reporting procedure for UE-sided model, associated ID can be part of inference configuration when both associated ID and inference configuration are provided in step 3 as in the procedure of Option A. On the other hand, if Option B is adopted, associated id can be also part of inference related parameters and out of CSI-ReportConfig.

Proposal 25: As in the procedure of Option B, the associated ID can be configured as part of inference related parameters and out of CSI-ReportConfig.

With option B, the content of inference related parameters for UE to determine applicable functionalities is still under discussion. As we discussed above, associated ID can be included but is optional. Set A related information includes RS type and set A size. Set B related information includes RS type, set B size, and association of set A and set B. Reporting quantities related information can include CRI and/or L1-RSRP . For BM-Case 2, inference related parameters include number and interval of time instances for measurement, number and interval of time instances for prediction.

Proposal 26: The content of inference related parameters at least includes:

- Associated ID

  - Associated ID is optional

- Set A related information (e.g. RS type, set A size)

- Set B related information (e.g. RS type, set B size, association of set A and set B)

- Reporting quantities (e.g. CRI, RSRP)

- For BM-Case 2, additional information can be included,

  - Number and interval of time instances for measurement

  - Number and interval of time instances for prediction

Discussion on specification impact of monitoring

Model monitoring can timely identify model performance degradation and performs model updating/switching/fallback to guarantee good system performance, which is important for model life cycle management. In this section, we discuss the monitoring mechanism, KPI and benchmark.

Type 1 monitoring Option 1

For Type 1 performance monitoring Option 1, UE can send a report to NW for the calculation of performance metric. The UE’s reporting can be a single sample based measurement result reporting. The content of the reporting can be measurement results of the resource set for monitoring, e.g., L1-RSRP and/or Top 1 RS ID. In this case, the gNB can have enough flexibility for the KPI calculation, such as Top-1 or Top-3/1 or Top-K/1 prediction accuracy, set B or Top K RSRP difference. But the issue is that the reporting overhead may be very large. Another way is that, gNB can configure the UE to report whether Top-K/1 beam prediction is correct per sample or per measurement instance. The overhead of reporting is very low, since the reporting content only occupies one bit to indicate it is correct or not.

Proposal 27: Regarding Type 1 performance monitoring Option 1 of UE-side AI/ML model, UE’s reporting can be a single sample based measurement result reporting. The content of the reporting can be the measurement results of the resource set for model monitoring, e.g. L1-RSRP and/or Top 1 RS ID, or Top-K/1 prediction accuracy per sample or per measurement instance.

Type 1 monitoring Option 2

  - 4.2.1 Resource type of monitoring set

One motivation to introduce AI/ML based BM is to reduce the efforts of UE measurements. But the still the performance monitoring is also important for the AI/ML based BM. During the performance monitoring, UEs have to measure the real transmitted signals for monitoring, which contradictively introduce more measurement overhead. So the overhead reduction of the measurement for model monitoring should be considered. Multiple solutions can be considered to reduce the overhead of model monitoring.

- Option 1, full set of Set A can be configured within the monitor resource set but with a longer periodicity.

- Option 2, a subset of Set A can be configured for the monitor resource set.

- Option 3, other reference signals which have an association with Set A but with lower overhead can be configured for the monitoring set.

From our understanding, both NZP CSI-RS and the SSB can be considered as the candidate resources for model monitoring. Normally, the NZP CSI-RS can be considered as the components of the Set A. Then the monitor resource set can be configured with a full set of Set A of NZP CSI-RS or a subset of Set A of NZP CSI-RS. And SSB which is used for DL synchronization and will be transmitted consistently can be considered as the candidate for the model monitoring. If the SSB is adopted for model monitoring, it will not introduced additional efforts or overhead for model monitoring. Since the SSB will be always transmitted, and UE has to measure it from time to time. Using SSB for model monitoring will not introduce additional overhead from the perspective of NW. And the QCL relationship between SSB and NZP CSI-RS can be used to determine the model performance.

Proposal 28: Both NZP CSI-RS and SSB should be considered as the candidate reference signal for model monitoring.

  - 4.2.2 Calculation and quantity for monitoring KPI

A new reporting quantity for beam prediction accuracy can be introduced in the CSI report for monitoring, The new reporting quantity has following two options.

- Option 1: Np, and the accuracy is Np/N.

- Option 2: Np/N

  - Where N (N≥1) prediction instance results of inference linked with monitoring instance are used for the calculation of accuracy

  - Np is the number of the prediction instance results out of N,

Between two options of reporting quantity, we prefer to report a ratio than the number of correct inference results out of N with variable reporting overhead log2N. To further reduce reporting overhead, the quantification method of the new reporting quantity needs to be discussed.

Proposal 29: Regarding Type 1 performance monitoring Option 2 of UE-side AI/ML model, Option 2 with new quantity of beam prediction accuracy is preferred.

- Option 2: Np/N

  - Where N (N≥1) prediction instance results of inference linked with performance monitoring instance are used for the calculation of accuracy

  - Np is the number of the prediction instance results out of N

Proposal 30: The quantification method of the new reporting quantity of beam prediction accuracy needs further discussion.

Np is calculated based on the detailed definition of beam prediction accuracy, the definition of beam prediction accuracy has following options:

    - Option 1 (Top-M/K): at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s)

      - Where K is the number of reported predicted beam(s) in the corresponding inference report

      - Where Top M beam(s) is the beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring

      - M is configured by NW in CSI report configuration for monitoring

        - M=1, and M is up to 4

    - Option 2 (Top-1/K with margin): The beam with largest measured value of L1-RSRP of Top-K predicted beams is within a margin X dB of largested measured value of L1-RSRP of the resource set(s) for monitoring

      - X=0, 1, FFS on whether to support other values

      - K=1 or K is equal to the number of reported predicted beam configured by inference report configuration, FFS other values

Option 1 is suitable for the case monitoring set is full set or subset of set A. The appropriate M value will change with the density of monitoring set in set A. When monitoring set is full set A or non-uniform down-sampling of set A, e.g. monitoring set configuration is based on UE location, M>1 is better than M=1 since beam prediction is considered as accurate when Top 2~Top M monitoring RS with high RSRP is included in Top K predicted beams. The case that all measured Top-M beams of monitoring set are not included in Top-K predicted beams will not occur.

When monitoring set is uniform down-sampling of set A, M>1 may over estimate performance of AI model since beam prediction is considered as accurate even if one of Top 2~Top M monitoring RS with low RSRP is included in Top K predicted beams. In this case, M with small value is more suitable. The measured Top-M beams of monitoring set may be out of the Top-K predicted beams, especially when K is small. This issue can be solved by gNB configuration, K value needs to be larger and monitoring set needs to more dense among set A.

Option 2 obtains some support in last two meetings since beam prediction accuracy with RSRP margin is higher than without RSRP margin, but the determination of X value seems difficult and time-consuming.

Another option similar to option 2 is the Top-1 beam with largest measured value of monitoring set and any of the Top-K predicted beam have a certain QCL relationship or the same QCL source, which means these two beams are under the same SSB or CSI-RS beam. The advantage of the new option over option 2 is that it does not need to configure the L1-RSRP margin, the discussions about L1-RSRP margin for option 2 may complicate the issue further.

The QCL-based method for calculating beam prediction accuracy ensures fundamental directional consistency of the predicted beams and Top 1 beam of monitor set, while eliminating complex parameter configuration. This simplified approach effectively prevents further complication of the issue.

Specific conditions are defined as:

    - Monitor set is configured as an SSB set:

      - A prediction is deemed correct if any of the predicted Top-K beams in Set A is QCLed with the Top-1 measured beam in monitor set.

    - Set M is configured as a subset of Set A:

      - A prediction is deemed correct if the Top-1 measured beam in monitor set shares the same source reference signal as any of the predicted Top-K beams in Set A.

Fig. 3 the case when monitoring resource set is different from Set A or a subset of Set A

Proposal 31: Regarding the calculation of Np, the following option can be considered.

  - Option 3 (Top-1/K with QCL relationship): the Top-1 beam with largest measured value of the resource set(s) for monitoring and any of the predicted Top-K beams have a certain QCL relationship or the same QCL source

For Type 1 Option 2 of UE-side model monitoring, to calculate beam prediction accuracy, a linkage between inference result and monitoring instance is required to compare inference result and prediction result. The linkage method has two options:

- Option 1:

  - for BM-Case 1, for an inference result with [a CSI reference resource or a transmission occasion of the resource] for Set B, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource which is “closest” to [the CSI reference resource or the transmission occasion of the resource]

  - for BM-Case 2, for an inference result for a time instance for prediction, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource which is “closest” to the time instance for prediction

- Option 2:

  - for BM-Case 1, for a monitoring occasion of the CSI-RS/SSB resource, the linked inference result is an inference result with [a CSI reference resource or a transmission occasion] for Set B which is “closest” to the monitoring occasion of the CSI-RS/SSB resource

  - for BM-Case 2, for a monitoring occasion of the CSI-RS/SSB resource, the linked inference result is an inference result for a prediction time instance which is “closest” to the monitoring occasion of the CSI-RS/SSB resource

The periodicity of monitoring set is possibly larger than periodicity of set B transmission occasion or time gap of prediction instance. With option 1, each inference result will link with a monitoring occasion, even of the time gap between inference result and monitoring occasion is large. For example, for BM-Case 1, when periodicity of set B transmission occasion is 40ms and periodicity of monitoring set is 120ms, the time gap between inference result and monitoring occasion can be up to 60ms. A time gap threshold is needed for option 1, the linked inference result and monitoring occasion should be within time gap threshold. The inference result and monitoring occasion with time gap larger than threshold will not be linked, since beam prediction accuracy calculated based on them is not accurate.

With option 2, each monitoring occasion will link with an inference result, but not all inference results have linkage with monitoring occasion, not all inference results are used for the calculation of accuracy, If one-to-one linkage is considered, the time gap between inference result and monitoring occasion is small. For example, for BM-Case 1, when periodicity of set B transmission occasion is 40ms and periodicity of monitoring set is 120ms, the time gap between inference result and monitoring occasion is up to 20ms. Therefore, option 2 is preferred to option 1.

For BM-Case 1, an argument is the time instance for inference result should be based on CSI reference resource or transmission occasion for Set B. Considering inference result is calculated after transmission occasion and before CSI reference resource, it is fine to use CSI reference resource as time instance for inference result.

For both options, if one of linked pair of inference result and monitoring occasion is after CSI reference resource of CSI-ReportConfig for monitoring, this linked pair can not used for calculation of accuracy, this prediction instance is not calculated in N prediction instances.

Proposal 32: For Type 1 Option 2 of UE-side model monitoring, to calculate beam prediction accuracy,

- for BM-Case 1, for a monitoring occasion of the CSI-RS/SSB resource, the linked inference result is the inference result with a CSI reference resource for Set B which is “closest” to the monitoring occasion of the CSI-RS/SSB resource

- for BM-Case 2, for a monitoring occasion of the CSI-RS/SSB resource, the linked inference result is the  inference result for a prediction time instance which is “closest” to the monitoring occasion of the CSI-RS/SSB resource

Proposal 33: With Np/N as new quantity of beam prediction accuracy, if one of linked inference result and monitoring occasion is after CSI reference resource of CSI-ReportConfig for monitoring, this prediction instance is not calculated in N prediction instances.

For the calculation of beam prediction accuracy for BM-Case2, one remaining issue is it is calculated per time instance or per report. In our understanding, if it is calculated per time instance, each prediction instance correspond to one accuracy, AI model with N prediction instances requires N accuracy reported in one CSI report. gNB can obtain more precise accuracy.

If it is calculated per report, AI model with N prediction instances requires one accuracy reported in one CSI report, this accuracy is more like the mean of accuracy of N prediction instances. We think per report calculation is enough to reflect the performance of AI model. Although per instance calculation is more precise, if some prediction instances have high accuracy and some prediction instances have low accuracy, gNB still needs to fallback or switch function. Thus, a coarse granularity of accuracy is enough.

Proposal 34: The beam prediction accuracy for BM-Case2 can be calculated per report.

RAN1#118 meeting agreed to study following alternatives of monitoring KPI for Option 2 (UE-assisted performance monitoring).

Agreement（118） For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

For regression model, L1-RSRP difference is also supported as monitoring KPI. Alt 2 calculates the L1-RSRP difference based on measurement results and measurement results, the motivation is confused. Alt 3 calculating the L1-RSRP difference based on prediction results and measurement results is more reasonable. In detail, the RSRP difference can be defined as between the predicted RSRP and measured RSRP of Top K predicted beam within monitoring resource set in case Top K predicted beam of set A is out of monitoring set. gNB has flexibility to configure set B as monitoring set to reduce measurement overhead.

Proposal 35: Regarding Type 1 performance monitoring Option 2 of UE-side AI/ML model, Alt3 can be supported with following modification.

- Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of Top K predicted beams within monitoring resource set

  - Note: resources for Set B for monitoring are not precluded.

  - 4.2.3 Event triggered reporting

NW can configure an event for UE to further reduce the reporting frequency. For example, when an event occurs, a reporting is triggered to inform the gNB that the performance of UE side AI is degrading. The event triggering the reporting of monitoring KPI can refer to the event of BFD. When consecutive N1 instances of KPI or system performance (e.g. ACK/NACK, UPT, BLER) are less than a threshold happen or if accumulated N2 instances of KPI or system performance (e.g. ACK/NACK, UPT, BLER) are less than a threshold happens within a time period, an event can be triggered.

Proposal 36: Regarding Type 1 performance monitoring Option 2 of UE-side AI/ML model, NW can configure an event to trigger the reporting of monitoring KPI. There are two options can be considered：

- Option 1: Consecutive N1 instances of KPI or system performance less than a threshold

- Option 2: Accumulated N2 instances of KPI or system performance less than a threshold within a time period

Configuration of the monitoring report and monitoring set

For monitoring Type 1 Option 2 of UE-side model monitoring, RAN1#119 has agreed to configure dedicate CSI report configuration for monitoring.

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

One remaining issue is the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report. The reporting of monitoring may happen after activation of inference for a period of time instead of becoming active together with inference. Reporting type of inference and monitoring can be different, for example, inference reporting is periodic and monitoring reporting is periodic, semi-persistent or aperiodic. When inference reporting is semi-persistent, monitoring reporting is semi-persistent or aperiodic. When inference reporting is aperiodic, monitoring reporting is aperiodic.

Another issue is the combination on time domain resource type (e.g. P, SP or AP) of set A in inference report and monitoring set in monitoring report. We think time domain resource type of two resource set should be the same. On one hand, set A may be a virtual set. Even if RS in set A has resource mapping, there seems no motivation to transmit the same beam twice in set A and monitoring set separately. On the other hand, if time domain resource type of set A and monitoring set is different, the same beam may correspond to different RS ID in different set. For example, P RS has TCI state but SP RS does not has TCI state in resource configuration. Even if monitoring set is full set A, additional mapping is needed between RS in set A and RS in monitoring set, which complicates Type 1 Option 2 monitoring further. Therefore, time domain resource type of two resource set should be the same.

Proposal 37: The combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report can be as follow.

| reportConfigType for infernece report | reportConfigType for monitoring report |
| --- | --- |
| periodic | periodic, semi-persistent, aperiodic |
| semi-persistent | semi-persistent, aperiodic |
| aperiodic | aperiodic |

Proposal 38: Time domain behavior of reference signals in Set A in inference configuration and the ones in the monitoring configuration should be kept the same.

The third issue is the configuration of the monitoring resource set to reduce measurement overhead of set A. The monitoring set can be a subset of set A and gNB has flexibility to configure monitoring set that is down sampled uniformly or non-uniformly from set A. The configuration method of monitoring set should support both uniformly and non-uniformly down-sampling of set A. gNB can explicitly configure monitoring resource set, and the same RS ID in both set A and monitoring set corresponds to the same beam.

Proposal 39: The monitoring set can be supported as a subset of set A. The configuration of monitoring set can be uniformly or non-uniformly down-sampling of set A.

Proposal 40: When monitoring set is a subset of set A, the association between the resources for monitoring and resources of Set A can be based on RS ID.

Type 2 monitoring of UE-side model

For UE-side model, the following model monitoring mechanisms have been captured in TR 38.843, and Type 1 performance monitoring has been agreed in RAN1#117 meeting.

TR 38.843 - Type 1 performance monitoring: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE may have different operations - Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - Indication from NW for UE to do LCM operations - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type 2 performance monitoring: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - If it is for UE side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation Agreement (#117) For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

Agreements: (RAN2#125bis) - For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). - “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

RAN2#125bis has approved that for function-based LCM of UE-sided model, the “network decision, network-initiated” AI/ML management is supported as a baseline. “UE autonomous, decision reported to the network” can be considered further. “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19.

We think UE autonomous AI/ML management corresponding to Type 2 performance monitoring can be supported. But it is necessary that UE reports its decision, such as the change of AI functionality, to the NW since UE decision may impact the NW’s configuration. For example, if the model switching results in an update of the configuration of set B, the UE has to report that a model switching is needed and the corresponding configuration of Set B needs a update. Any decision can be applied only after UE receiving gNB’s acknowledgement.

For Type 2 monitoring, NW can configure a threshold criterion to facilitate UE to make decision. For example, if beam prediction accuracy of the activated model is larger than a threshold, the model switching or fallback to legacy mode is not necessary. If beam prediction accuracy of the activated model is smaller than the threshold, inactive models whose beam prediction accuracy is larger than the threshold can be considered as candidate model for model switching.

Proposal 41: Type 2 monitoring of UE-side AI/ML model, indication/requesting from UE but with gNB’s decision can be supported.

- Proposal 42: Regarding Type 2 monitoring of UE-side AI/ML model, NW can configure a threshold criterion to facilitate UE to make decisions of the model selection / activation/ deactivation / switching/ fallback.

AI/ML-based CSI processing

In RAN1#118b meeting, for UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing.

Agreement For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

In RAN1#120 meeting, FL provided following proposal about CPU pool of AI/ML based and legacy CSI report.

For UE-side model, at least for AI/ML based beam management, for processing of a CSI report at least for inference, study the following options: - Option 1: The CPU occupied by the CSI report at least for inference and the CPU occupied by the non-AI/ML based CSI report for share the same pool. - Option 2: Introduce a dedicated CPU pool as AI/ML-based CSI processing unit (ACPU), FFS - Alt 1: Processing of a CSI report at least for inference only occupies a number of ACPU(s) for a number of symbols - Note: the legacy CPU pool will not impact by the activation of AI/ML functionality - Alt 2: Processing of a CSI report at least for inference occupies X number of ACPU(s) for M symbols and Y number of CPU(s) for N symbols - FFS whether AI/ML based beam management and other AI/ML based features, e.g., AI/ML based CSI prediction, share the same (A)CPU pool or not.

From our view, the CSI processing unit of AI/ML should be separated from the legacy CSI reporting. Since the hardware of AI/ML based CSI process and the legacy one may be separated hardware or chips. Then it is straightforward to use separate CPU counting or pool CPU pool resources. The alternative 2 under the option 2 is confusing. If the computing resource of AI/ML CSI processing and the legacy process is separated, it is difficult to understand the reason that both legacy process unit and the AI based process unit are occupied at same time.

Among multiple AI/ML use cases, e.g. BM, CSI prediction and CSI compression, the AI/ML related CPU occupation can be discussed separately.

Proposal 43: At least for AI/ML based BM of UE sided model, a dedicated CPU pool for AI/ML-based CSI processing unit can be considered.

  - Alt 1: Processing of a CSI report at least for inference only occupies a number of ACPU(s) for a number of symbols

    - Note: the legacy CPU pool will not impact by the activation of AI/ML functionality

Proposal 44: The CSI processing mixed with legacy CPU and AI/ML related CPU (Alt 2) should be deprioritized.

Conclusion

In this contribution, we focus on the discussion on the the procedure and the potential specification impact of AI based beam management during different LCM operation. The observations and proposals are listed below.

Observation 1: With the UE capability reporting, gNB can provide a proper inference configuration for UE to work even without the configuration of associated id. The inference performance of the model can be monitored based on the model monitoring procedure.

Observation 2: According to the definition from RAN2, whether the functionalities are applicable or not is not related to the associated id.

Proposal 1: From RAN 1 perspective, for content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, type 1 and type 3 for each instance can be supported:

- Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE

- Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE

Proposal 2: L1 signaling can be supported for NW-sided training data collection.

Proposal 3: Regarding data collection for NW-side model, following option is supported for the configuration of Set A:

- Option 2: The configuration of set A can contain multiple resource sets in one CSI-ResourceConfig

Proposal 4: Regarding data collection of UE-side model for training, UE can report or request gNB for the preferred set B (pattern).

Proposal 5: Regarding data collection of UE-side model, the configuration of set A can contain multiple resource sets in one CSI-ResourceConfig.

Proposal 6: Top K beam sweeping procedure can be supported to obtain Top 1 beam.

Proposal 7: The indication of Top K beam (resource) set with low signaling overhead can be further discussed.

Proposal 8: For Top-K beam sweeping, NW indicates the beam IDs or TCI states that are part of the Top-K beams by MAC CE.

Proposal 9: For NW-sided model inference reporting, at least for BM-Case 1, when the value of M (Top-M beams) is smaller than the size of measurement resource set, Option 1 is supported:

- Option 1: CRI/SSBRI

Proposal 10: For NW-sided model inference reporting, at least for BM-Case 1, L1-RSRPs and corresponding beam information of beams within X dB gap to the largest measured value of L1-RSRP can be supported.

- The X dB gap can be configured by gNB

- UL resource allocation for variable number of reported beams can be further discussed.

Proposal 11: The RS associated with TCI state indication should be measured at least once before TCI state indication or application.

Proposal 12: The combination of BM Case 1 and BM Case 2 can be supported.

Proposal 13: For BM-Case 2 of UE-side model, for inference, aperiodic resource type for CMR is not supported.

Proposal 14: For inference results reporting of UE side model, predicted Top K beams are the K beams with largest predicted RSRP values or the K beams with the largest probability to be the Top-1 beam in Set A.

Proposal 15: For inference results reporting of UE side model, probability information of predicted Top K beams does not need to be reported.

Proposal 16: In the case of Top K beam id reporting, the ranking within one beam reporting may not be needed.

Proposal 17: For UE-side AI/ML model inference of BM-Case2, the time offset between reference time and earliest future time instance can be configured by gNB.

- The predicted time instance can be determined by the slot of reference time+offset+P*(N-1)

Proposal 18: There is no need to define the content of the similar properties behind the associated id.

Proposal 19: The associated ID can be optionally configured for inference.

Proposal 20: When the associated ID is configured for data collection/inference, the determination of applicable functionality can be based on the configuration or the related information but without associated id. Namely, the associated id will not be used for the determination of the applicable functionality.

Proposal 21: When the associated ID is configured for data collection but not configured for inference (Case 1-1), the UE can report it is either applicable or not applicable.

Proposal 22: When the associated ID is configured for data collection but not configured for inference (Case 1-1), the UE can report it is applicable and have the same assumption of gNB’s additional conditions as that for data collections.

Proposal 23: When the associated ID is not configured for data collection and not configured for inference (Case 2-1), the UE can report it is either applicable or not applicable.

Proposal 24: When the associated ID is not configured for data collection and not configured for inference (Case 2-1), the UE can report it is applicable and have the same assumption of gNB’s additional conditions as that for data collections.

Proposal 25: As in the procedure of Option B, the associated ID can be configured as part of inference related parameters and out of CSI-ReportConfig.

Proposal 26: The content of inference related parameters at least includes:

- Associated ID

  - Associated ID is optional

- Set A related information (e.g. RS type, set A size)

- Set B related information (e.g. RS type, set B size, association of set A and set B)

- Reporting quantities (e.g. CRI, RSRP)

- For BM-Case 2, additional information can be included,

  - Number and interval of time instances for measurement

  - Number and interval of time instances for prediction

Proposal 27: Regarding Type 1 performance monitoring Option 1 of UE-side AI/ML model, UE’s reporting can be a single sample based measurement result reporting. The content of the reporting can be the measurement results of the resource set for model monitoring, e.g. L1-RSRP and/or Top 1 RS ID, or Top-K/1 prediction accuracy per sample or per measurement instance.

Proposal 28: Both NZP CSI-RS and SSB should be considered as the candidate reference signal for model monitoring.

Proposal 29: Regarding Type 1 performance monitoring Option 2 of UE-side AI/ML model, Option 2 with new quantity of beam prediction accuracy is preferred.

- Option 2: Np/N

  - Where N (N≥1) prediction instance results of inference linked with performance monitoring instance are used for the calculation of accuracy

  - Np is the number of the prediction instance results out of N

Proposal 30: The quantification method of the new reporting quantity of beam prediction accuracy needs further discussion.

Proposal 31: Regarding the calculation of Np, the following option can be considered.

  - Option 3 (Top-1/K with QCL relationship): the Top-1 beam with largest measured value of the resource set(s) for monitoring and any of the predicted Top-K beams have a certain QCL relationship or the same QCL source

Proposal 32: For Type 1 Option 2 of UE-side model monitoring, to calculate beam prediction accuracy,

- for BM-Case 1, for a monitoring occasion of the CSI-RS/SSB resource, the linked inference result is the inference result with a CSI reference resource for Set B which is “closest” to the monitoring occasion of the CSI-RS/SSB resource

- for BM-Case 2, for a monitoring occasion of the CSI-RS/SSB resource, the linked inference result is the  inference result for a prediction time instance which is “closest” to the monitoring occasion of the CSI-RS/SSB resource

Proposal 33: With Np/N as new quantity of beam prediction accuracy, if one of linked inference result and monitoring occasion is after CSI reference resource of CSI-ReportConfig for monitoring, this prediction instance is not calculated in N prediction instances.

Proposal 34: The beam prediction accuracy for BM-Case2 can be calculated per report.

Proposal 35: Regarding Type 1 performance monitoring Option 2 of UE-side AI/ML model, Alt3 can be supported with following modification.

- Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of Top K predicted beams within monitoring resource set

  - Note: resources for Set B for monitoring are not precluded.

Proposal 36: Regarding Type 1 performance monitoring Option 2 of UE-side AI/ML model, NW can configure an event to trigger the reporting of monitoring KPI. There are two options can be considered：

- Option 1: Consecutive N1 instances of KPI or system performance less than a threshold

- Option 2: Accumulated N2 instances of KPI or system performance less than a threshold within a time period

Proposal 37: The combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report can be as follow.

| reportConfigType for infernece report | reportConfigType for monitoring report |
| --- | --- |
| periodic | periodic, semi-persistent, aperiodic |
| semi-persistent | semi-persistent, aperiodic |
| aperiodic | aperiodic |

Proposal 38: Time domain behavior of reference signals in Set A in inference configuration and the ones in the monitoring configuration should be kept the same.

Proposal 39: The monitoring set can be supported as a subset of set A. The configuration of monitoring set can be uniformly or non-uniformly down-sampling of set A.

Proposal 40: When monitoring set is a subset of set A, the association between the resources for monitoring and resources of Set A can be based on RS ID.

Proposal 41: Type 2 monitoring of UE-side AI/ML model, indication/requesting from UE but with gNB’s decision can be supported.

- Proposal 42: Regarding Type 2 monitoring of UE-side AI/ML model, NW can configure a threshold criterion to facilitate UE to make decisions of the model selection / activation/ deactivation / switching/ fallback.

Proposal 43: At least for AI/ML based BM of UE sided model, a dedicated CPU pool for AI/ML-based CSI processing unit can be considered.

  - Alt 1: Processing of a CSI report at least for inference only occupies a number of ACPU(s) for a number of symbols

    - Note: the legacy CPU pool will not impact by the activation of AI/ML functionality

Proposal 44: The CSI processing mixed with legacy CPU and AI/ML related CPU (Alt 2) should be deprioritized.

References

- Chair notes RAN1#120 eom1, Athens, Greece, February 17th–21st, 2025.

- R1-2407604, LS on applicable functionality reporting for beam management UE-sided model, 3GPP TSG RAN WG1 #118, Hefei, China, October 14th – 18th, 2024.

- R1-2410892, FL summary #5 for AI/ML in beam management, 3GPP TSG RAN WG1 #119, Orlando, US, November 18th – 22nd, 2024.