3GPP TSG-RAN WG1 Meeting #120bis	R1-2502805

Wuhan, China, April 7th – April 11th, 2025

Agenda Item:	9.6.1

Source:	Ericsson

Title:	LP-WUS and LP-SS design

Document for:	Discussion

# Introduction

In this contribution, we present our view on the design of LP-WUS and LP-SS. In RAN1-120, the following agreements were reached:

Agreement For LP-WUS information carried by OOK, select alt 1 for codepoint to meet FAR and MDR performance. - Alt 1: Coding with rate matching if any - For more than 2 information bits: RM coding as in section 5.3.3.3 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 2 information bits (if supported): coding as in section 5.3.3.2 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 1 information bit (if supported): repetition as in section 5.3.3.1 of 38.212 (FFS: whether scrambling in the same section is applied or not) - FFS: Code block length - FFS: Further repetition before or after Manchester coding Conclusion For RRC idle/inactive, LP-WUS is not supported for the case where associated CD-SSB and initial DL BWP have different SCSs Agreement At least for M>1, for the overlaid OFDM sequence length, support - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included) Agreement Proposal 4.3-1: Update the agreements in RAN1 #118bis as below Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Applicable to both OOK-1 and OOK-4 - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 only and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences Agreement For RRC connected, for LP-WUS SCS: - Alt 1: LP-WUS SCS is same as the active DL BWP Working Assumption Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs: - Codepoint based - The maximum number of codepoints checked per MO by a UE is up to 8 - Depending on UE capability, a UE may support less than 8 Agreement For M=2, 4, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any. Agreement For RRC connected, LP-WUS frequency resource can be outside of active DL BWP but has to be within the same carrier as the active DL BWP - Basic capability is LP-WUS, if present, frequency resource within active DL BWP. LP-WUS frequency resource outside of active DL BWP is subject to separate UE capability - No RAN1 optimization specific to the case where LP-WUS frequency resource is outside of active DL BWP Agreement For M=1, L=4 (if supported), the set of LP-SS binary sequences is: [0 1 0 1] [0 1 1 0] [1 0 0 1] [1 0 1 0] Agreement For M=1, L=6 (if supported), the set of LP-SS sequence is: [1 0 1 0 1 0] [0 1 0 1 0 1] [1 0 0 1 0 1] [1 0 1 0 0 1] Agreement For M=1, L=8 (if supported), the set of LP-SS sequence is: [1 0 1 0 0 1 0 1] [1 0 1 0 1 0 0 1] [1 0 0 1 0 1 0 1] [0 1 0 1 0 1 0 1] Agreement For M=2, L=8 (if supported), the set of LP-SS sequence is: [0 1 0 1 1 0 0 1] [0 1 1 0 0 1 0 1] [0 1 1 0 1 0 0 1] [1 0 0 1 0 1 1 0] Agreement For M=2, L=12 (if supported), the set of LP-SS sequence is: [1 0 0 1 1 0 0 1 1 0 0 1] [0 1 1 0 1 0 0 1 1 0 0 1] [0 1 1 0 0 1 1 0 1 0 0 1] [0 1 1 0 0 1 0 1 1 0 0 1] Agreement For M=2, L=16 (if supported), the set of LP-SS sequence is: [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1] [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1] [1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1] [1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] Agreement For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between: - Set 1 [0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0] [0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0] [1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1] [1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] - Set 2 [1 0 0 0 1 0 0 0 0 0 0 1 0 0 1 0] [1 0 0 0 0 1 0 0 1 0 0 0 0 0 1 0] [1 0 0 0 0 1 0 0 0 1 0 0 1 0 0 0] [1 0 0 0 0 1 0 0 0 0 0 1 0 0 0 1] Agreement For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between: - Set 1: [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] [0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] [0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1] - Set 2 [0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0] [0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0] [0 0 1 0 0 0 1 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 0 1 0 0 1 0 0 0 1 0] [0 0 1 0 0 1 0 0 0 0 0 1 1 0 0 0 0 0 1 0 1 0 0 0 0 1 0 0 0 0 1 0] Agreement For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. Agreement For WUS information carried by the overlaid OFDM sequence(s), consider at least the following alternatives: - Alt 1: Raw information bits are mapped to sequence(s) - Alt 2: Raw information bits are mapped to sequence(s) after channel coding - Same channel coding scheme(s) as OOK is applied. - FFS same or different rate matching and/or repetition factor as OOK - Alt 3: Codepoint/Subgroup is mapped to sequence(s)

# LP-WUS design

## General design principles

From a network vendor and ecosystem perspective, it is important that the gNB should be able to transmit the LP-WUS using existing gNB hardware and not require any new emissions or compliance requirements. Otherwise, it is difficult to enable widespread WUS support in existing deployments.

In order to not negatively impact network capacity, it is important that the LP-WUS can be multiplexed with other NR transmissions in time and frequency, and that any unused LP-WUS resources can be reused for dynamic scheduling. Also, to avoid impacts on the gNB complexity and other NR transmissions, same SCS should be considered for WUS and other NR transmissions in a given carrier.

Following principles should be considered for LP-WUS and LP-SS design

  - It should be possible to generate LP-WUS/LP-SS transmissions using existing gNB hardware and not trigger any new emissions or compliance requirements.

  - It should be possible to multiplex the LP-WUS/LP-SS with other NR transmissions in time or frequency domain without causing interference.

  - It should be possible to reuse any unused LP-WUS time and frequency resources for other transmissions.

Also, as agreed during the SI, in the design and monitoring of LP-WUS, it needs to be ensured that the paging missed detection is not impacted and the targets for missed detection probability and false alarm probability are satisfied. Specifically, to ensure UE reachability, the missed detection target of 10-2 similar to the paging PDCCH requirement should be met.

Paging misdetection performance of the UE should not be impacted when LP-WUS is used by the UE for power savings.

## M value for OOK WUS and LP-SS

In RAN1#119 the following agreement was reached:

Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

The generation and reception of OOK with M=1 (OOK-1) has lower complexity for both UE and gNB compared to M>1. In terms of coverage OOK-1 [M=1] slightly outperforms OOK [M=2,4] for the same overhead and total transmit power (as shown in Figure 1). Therefore, considering -3 dB SNR, OOK [M>1] does not provide any benefit in terms of spectral efficiency. Moreover, compared to OOK-1, OOK [M>1] is more sensitive to the timing error. In particular, it is challenging to satisfy the timing accuracy requirement for M=4. As will be discussed in Section 4, LP-SS duration 4-6 OFDM symbols is sufficient for M=1 and M=2 while at least 8 symbols are needed for M=4. Therefore, the network energy consumption is higher for M=4 compared to M=1 or 2.

In general, using the same M value for LP-WUS and LP-SS is beneficial in terms of UE and gNB complexity. The motivation for using M>1 for LP-SS is to improve the timing synchronization accuracy which is needed for LP-WUS with M>1 as it is more sensitive to the timing error than M=1. For LP-WUS M=1, sufficient timing accuracy can be achieved by LP-SS with M=1. Therefore, the same M value should be supported for LP-WUS and LP-SS.

Meanwhile, if the UE supports all M values, the case that M values for LP-WUS and LP-SS can be configured to be same or different provides flexibility for supporting different combinations. Therefore, Alt3 can be supported provided that UEs support all M values {1,2,4}.

Compared to OOK-4 [M>1], OOK-1 [M=1] has lower complexity for both the UE and gNB and it can provide slightly better coverage. It is more robust against timing errors and requires a shorter LP-SS, leading to lower network energy consumption.

In the Idle/Inactive mode, OOK-4 [M>1] does not provide benefit over OOK1 while increasing both gNB and UE complexity and M=4 especially does not provide any benefit over M=2 while being more sensitive to timing error.

Regarding M value for LP-WUS and LP-SS, Alt 1 “the M values for LP-WUS and LP-SS are always same” is supported.

## LP-WUS structure for OOK

Regarding number of subgroups, we have the following agreements in the previous meetings:

Agreement The maximum number of subgroups per PO supported in Rel-19 is 31. Agreement At least the following codepoints are supported for LP-WUS: - One codepoint corresponding to each of the subgroups that can be indicated by LP-WUS - One codepoint corresponding to all the subgroups that can be indicated by LP-WUS Conclusion There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping.

### OOK WUS duration and MOs

Here, we provide our results on the OOK-based WUS duration needed to reach the -3 dB SNR. First, in Figure 1, we show that even under ideal condition (without time/freq error) and single sequence detection, the OOK WUS duration of one slot is not sufficient to meet the -3dB SNR requirement at 1% or 0.1% FAR.

Figure 1: OOK detection performance without timing and freq error (single sequence detection) for one slot WUS duration.

When comparing the detection performance of OOK-1 [M=1] and OOK-4 [M=2,4] for the same payload, overhead, and total transmit power, results indicate that OOK-1 slightly outperforms OOK-4. This is due to OOK-4 waveform generation which results in power leakage in the OFF segments of OOK-4 which can be avoided for OOK-1, as illustrated in Figure 2.

Figure 2: Time samples of OOK-1 and OOK-4 (M=2) over two OFDM symbols.

Even under ideal condition (without time/freq error) and single sequence detection, the OOK WUS duration of one slot is not sufficient to meet the -3dB SNR requirement.

Table 1 and Table 2 provide the required WUS duration for OOK to achieve 1%MDR at -3dB SNR. Timing error of 2 us, 1 us, and 0.5 are assumed for OOK-1, OOK-4 [M=2], and OOK-4 [M=4] respectively.

Table 1: Required WUS duration (#OFDM symbols) for OOK 
(1%MDR at -3dB SNR, 1% FAR, non-ideal timing).

| Number of sequences checked by UE | OOK1[M=1] | OOK4 [M=2] | OOK4 [M=4] |
| --- | --- | --- | --- |
| Case1: One sequence | 18 | 20 | 22 |
| Case2: Two sequences | 22 | 24 | 24 |

Table 2: Required WUS duration (#OFDM symbols) for OOK 
(1%MDR at -3dB SNR, 0.1% FAR, non-ideal timing).

| Number of sequences checked by UE | OOK1[M=1] | OOK4 [M=2] | OOK4 [M=4] |
| --- | --- | --- | --- |
| Case1: One sequence | 24 | 28 | 30 |
| Case2: Two sequences | 28 | 32 | 34 |

It should be noted that regardless of number of subgroups configured by gNB (which can be any number between 1-32), UE needs to only check for sequences that map to codepoints which provide wakeup for the UEs subgroup. For example, if 15 subgroups (sg0-sg14) are configured, Case 2 (two sequences checked by the UE) corresponds to Idle mode operation where UE checks for one OOK binary sequence corresponding to codepoint of its sub-group ID (i.e., one of sg0-sg14) and one more OOK binary sequence corresponding the codepoint mapped to wake-up of all subgroups.

It can be seen from the Tables that depending on FAR assumption (0.1% is more appropriate for Idle mode as UE is expected to be in ultra-deep sleep state and penalty for false wake up is high) and number of sequences checked by the UE, around 18-28 OFDM symbols are required for OOK-1 based for LP-WUS and more for OOK-4 based LP-WUS.

For OOK based LP-WUS,

- For OOK-1 (M=1), 18-28 OFDM symbols are required to achieve 1%MDR for -3dB SNR depending on FAR assumption (0.1% vs 1%), and number of sequences checked by the UE.

- For OOK-4 (M=2,4) slightly more OFDM symbols than OOK-1 are needed to achieve same performance when same total power is assumed for the duration of LP-WUS transmission.

In terms of mapping LP-WUS information to slots, some of the symbols in the slots must be reserved for other NR channels/signals and typically only a subset of OFDM symbols is available for LP-WUS mapping. Figure 3 shows some example structures for a WUS Monitoring occasion (WUS MO) where 11-13 symbols are available for WUS per slot and rest used for PDCCH. To accommodate the WUS duration required for acceptable MDR/FAR, WUS MO should span at least 2 slots. Considering different deployment scenarios, gNB flexibility to reserve some symbols in each slot for other transmissions, UE implementation margins etc., we propose that WUS MO duration for OOK for Idle should be configurable between 1 to X slots.

Figure 3: Example structures for an OOK-based LP-WUS monitoring occasion (WUS MO).

For mapping LP-WUS information to slots, some of the OFDM symbols in the slots must be reserved for other NR channels/signals and typically only a subset of OFDM symbols is available for LP-WUS mapping.

### Encoding scheme

The following was agreed in RAN1#120 regarding encoding schemes:

Agreement For LP-WUS information carried by OOK, select alt 1 for codepoint to meet FAR and MDR performance. - Alt 1: Coding with rate matching if any - For more than 2 information bits: RM coding as in section 5.3.3.3 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 2 information bits (if supported): coding as in section 5.3.3.2 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 1 information bit (if supported): repetition as in section 5.3.3.1 of 38.212 (FFS: whether scrambling in the same section is applied or not) - FFS: Code block length - FFS: Further repetition before or after Manchester coding

Figure 4 below illustrates example processing chain for mapping LP-WUS payload to OOK symbols.

Figure 4: Mapping of LP-WUS codepoints to OOK symbols.

Following aspects should be considered for mapping of LP-WUS codepoints to OOK symbols

- It should be possible to flexibly map different codeword lengths L (e.g., at least L= 4 for 16 codepoints, L=5 for 32 codepoints) to flexible number of available OFDM symbols in a LP-WUS MO.

- Manchester encoding is used before mapping coded bits to OOK symbols

Scrambling can be additionally considered (e.g., before Manchester encoding in Figure 4) if the LP-WUS needs to be differentiated between different cells.

If additional repetitions are required, they can be performed by repeating the OOK symbols obtained after Manchester coding N_rep times. In previous meetings, it was discussed whether repetitions are considered part of same LP-WUS MO or whether different repetitions are provided in different MOs. This depends on how MOs are defined. If MO is defined as an integer number of slots, then it is more efficient to have the repetitions to be part of an MO, as shown in Figure 5, as there is more flexibility in number of symbols used for each repetition.

Figure 5: OOK-based LP-WUS MO with two repetitions.

RM encoding allows flexible mapping of codepoints to different WUS durations. Figure 6 shows the OOK detection performance for different WUS durations based on RM encoding (UE checks 2 sequences over different WUS durations). For example, for the case of 32 codepoints, the codepoint (e.g., one of 00000-11111) is encoded and rate-matched to result in 12 coded bits that are the r=1/2 Manchester encoded and transmitted on 24 OOK symbols (i.e., 24 OFDM symbols since OOK-1 is evaluated for UE checking 2 sequences). For the evaluation, the Reed-Muller block code in 38.212 is used and the resulting parity bits are adjusted to rate-match to available OFDM symbols (if all symbols are available in a slot 28 symbols would be mapped to two slots).

Figure 6: OOK-1 performance with Reed-Muller encoding for 32 codepoints and sequence-based detection by LP-WUR (1% FAR).

In RAN1#120 it was agreed to reuse the existing coding and rate-matching approaches to map the LP-WUS information carried by OOK. Some details including, need for scrambling, how to determine code-block length and whether additional repetition is needed have to be finalized.

Regarding code-block length after channel encoding (i.e., N in 38.212 in section 5.3.3.3), it would be given by min (Y, (F*M)/2), if the Manchester encoded LP-WUS is mapped to F OFDM symbols where M is the number of OOK symbols per OFDM symbol. Y=32 for case of more than 2 information bits, and Y=1 or 3 for the cases of one or two information bits per OFDM symbol. If (F*M)/2 > Y, then the code-block is cyclically extended to fit the available OOK symbols.

For example, consider case where 16 codepoints (15 subgroups+1 common codepoint), K=4 information bits are needed. If Manchester encoded LP-WUS is mapped to F=28 symbols for this case, then the code-block length after channel coding is given by N= min (32, (28*M)/2). For different M, the code-block lengths are: N=14 for M=1, N=28 for M=2, and N=32 for M=4. Note that for M=4, (F*M)/2=56 which is greater than Y=32, thus the code-block is cyclically extended to length 56 to fit the available OOK symbols. For M=1 and M=2, however, cyclic extension is not needed in this example.

The OFDM symbols used for LP-WUS can be indicated to the UE by configuring number of slots (A) used for LP-WUS (A=1,2 or 3 slots) and an additional parameter indicating the available OFDM symbols in each slot (e.g., by configuring a 14bit bitmap or by using a SLIV value as done for PDSCH/PUSCH). Alternatively, LP-WUS duration can be directly configured in terms of OFDM symbols but in that case mapping rules to avoid other physical channels like PDCCH should be additionally defined.

Regarding repetitions, configuring a repetition factor Nrep is beneficial to support WUS with sufficient coverage considering different deployment scenarios, UE implementation margins etc. If Nrep>1 is configured by gNB, then the Manchester encoded LP-WUS that spans F OFDM symbols discussed above should be repeated Nrep times. With this framework, the MO duration of LP-WUS will be X=A*N_rep slots.

LP-WUS should support any number of codepoints from 1 to 32 (targeting up to 31 subgroups and one common codepoint).

Regarding LP-WUS information bits, all cases of 1 information bit, 2 information bits and larger than 2 information bits should be supported.

The OFDM symbols used for LP-WUS should be indicated to the UE by configuring number of slots (A) used for LP-WUS (A=1,2 or [3] slots) an additional parameter indicating the available OFDM symbols in each slot (e.g., by configuring a 14-bit bitmap or by using a SLIV value).

If repetition is supported for OOK-based LP-WUS, the OOK symbols after Manchester coding are repeated N_rep times. The N_rep repetitions occur in same MO.

Maximum N_rep=[5] repetitions can be supported in the specifications considering different deployment scenarios, gNB flexibility to reserve some symbols in each slot for other transmissions, UE implementation margins etc., resulting in maximum MO duration for LP-WUS (i.e., after maximum repetitions) of [15] slots.

The code-block length after channel encoding (i.e., N in 38.212 in section 5.3.3.3) is min (Y, (F*M)/2), where F is the number of WUS OFDM symbols with Manchester encoding, M is the number of OOK symbols per OFDM symbol, and Y=1, 3, and 32 for 1-bit, 2-bit, and ≥2 bits information, respectively. When (F*M)/2 > Y, the code-block is cyclically extended to fit the available OOK symbols.

It should be possible to support or indicate 32 UE subgroup with one WUS, no segmentation.

## Frequency position of LP-WUS/LP-SS in Idle mode

Regarding frequency location of LP-WUS/LP-SS in Idle/Inactive mode, the following alternative are being considered:

- Alt 1: LP-WUS/LP-SS frequency resource is confined within initial DL BWP.

- Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP.

With Alt 1, having LP-WUS/LP-SS frequency resource within initial DL BWP allows reusing existing BWP framework and procedures. The benefits of Alt2 over Alt 1 include more flexibility for placing WUS/LP-SS in the carrier, less congestion in the initial DL BWP, and potentially smaller overhead when supporting both RedCap and regular UEs.

In the current specifications, the UE should monitor PDCCH within the bandwidth of CORESET#0 to acquire SIB1 which provides configuration of the initial BWP. Within IE BWP-DownlinkCommon, the location and bandwidth of the initial DL BWP can be determined based on IE locationAndBandwidth. The initial DL BWP can be configured to have a larger than the CORESET#0 bandwidth (potentially as wide as the carrier bandwidth). While the potential congestion issue due to placing LP-WUS/LP-SS within the initial BWP can be mitigated by configuring a larger initial DL BWP, it increases UE power consumption.

For RedCap UEs, the maximum RF bandwidth is 20 MHz in FR1 and 100 MHz in FR2. Also, a separate initial DL BWP can be configured for RedCap UEs which can overlap with initial BWP of regular UEs. Note that the initial BWP can be shared between RedCap and regular UEs if its bandwidth does not exceed the maximum supported bandwidth of RedCap. If a separate initial DL BWP is configured for RedCap and is used for random access and not paging, it may not include the CD-SSB and entire CORESET#0. However, for paging, the RedCap UE should monitor paging in the initial BWP that includes CORESET#0 (specification texts are copied below). Also, in a typical deployment scenario, the RedCap BWPs are placed around the carrier edge to minimize the resource fragmentation.

Specification text from TS 38.331 regarding paging search space for RedCap:

pagingSearchSpace ID of the search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing, shall monitor paging in the initial DL BWP that includes CORESET#0.

Specification text from TS. 38.213 regarding presence of SSB and CORESET#0 in RedCap initial DL BWP:

For an initial DL BWP provided by initialDownlinkBWP-RedCap, if a UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, the UE does not expect the initial DL BWP to include SS/PBCH blocks and the CORESET with index 0.

To support WUS for both RedCap and regular UEs, the following cases can be considered:

- Case 1: the regular initial DL BWP does not cover the RedCap initial BWP: For this case, with Alt 1, the gNB needs to transmit separate WUSs for RedCap and regular UEs which increases the overhead and network energy consumption. An example of such BWPs configuration is shown in Figure 7. With Alt2, this issue can be avoided.

- Case 2: the regular initial DL BWP is as wide as the carrier bandwidth and it cover the RedCap initial BWP: the gNB can transmit WUS within the RedCap initial BWP (which is also within the regular initial BWP). Hence, the duplication of WUS transmissions for RedCap and regular UEs in Alt1 can be avoided. An example of such BWPs configuration is shown in Figure 8.

- Case 3: for regular UEs, the LP-WUS/LP-SS is located within the initial DL BWP. For RedCap, the LP-WUS/LP-SS can be located outside the separate RedCap initial DL BWP but the RedCap UE can monitor LP-WUS/LP-SS in an initial DL BWP that includes CORESET#0 and used for paging (as illustrated in Figure 9).

Figure 7: Example of configuring initial BWPs for regular UEs and RedCap UEs for Case 1 (non-overlapping BWPs, separate WUS with Alt1).

Figure 8: Example of configuring initial BWPs for regular UEs and RedCap UEs for Case 2 (overlapping BWPs, shared WUS).

Figure 9: Example of configuring initial BWPs for regular UEs and RedCap UEs for Case 3 (RedCap can monitor WUS outside its separate initial DL BWP, shared WUS).

The above discussions show that the possibility of placing WUS and LP-SS outside the initial DL BWP can be beneficial for supporting both RedCap and regular UEs considering the overhead and network energy consumption. Based on the current specifications, RedCap UEs should monitor paging in an initial DL BWP that contains CORESET#0 (i.e., UE may need to retune to regular initial DL BWP). Therefore Case 3 can be naturally supported where for regular UEs LP-WUS/LP-SS is located within the initial BWP (Alt1) while for RedCap UEs it can be located outside the RedCap-specific initial DL BWP (Alt2).

Regarding frequency position of LP-WUS/LP-SS:

- Alt 1 (LP-WUS/LP-SS frequency resource is confined within initial DL BWP) allows reusing existing BWP framework and procedures and reduces the need for UE RF returning and facilitates offloading of RRM measurements to WUR.

- The benefits of Alt2 (LP-WUS/LP-SS frequency resource can be out of initial DL BWP) over Alt 1 include more flexibility for placing WUS/LP-SS in the carrier, less congestion in the initial DL BWP, and potentially smaller overhead when supporting both RedCap and regular UEs.

Based on the current specifications, RedCap UEs should monitor paging in an initial DL BWP that contains CORESET#0 (i.e., UE may need to retune to regular initial DL BWP).

For regular UEs, the LP-WUS/LP-SS should be located within the initial DL BWP (as in Alt1). For RedCap, the LP-WUS/LP-SS can be located outside the separate RedCap initial DL BWP (as in Alt2) but within an initial DL BWP that includes CORESET#0 used for paging.

# Overlaid OFDM sequence for LP-WUS

## Overlaid OFDM on OOK-4 [M=1]

The following working assumption was considered in RAN1#118bis:

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, down-select between following two options for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainàDFT/LSàIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1. Note: Concerns were raised by Ericsson and CATT that the above is not beneficial in terms of implementation complexity.

Regarding the DFT size, limiting it to power of 2 is restrictive as it only allows bandwidths of 2N subcarriers (no additional operation in the frequency domain). It is important to have the flexibility or possibility to configure different bandwidths for WUS. Moreover, according to the current specifications, there is no limitation on DFT size being a power of 2. For example, the bandwidth of the PUSCH in terms of resource blocks shall fulfil (TS 38.211) the following:

where  is a set of non-negative integers. In terms of number of subcarriers (12*RBs) some examples of non-power of 2 values based on the above equations are: 144, 120, 108.

Furthermore, for M>1, it is agreed to have sequence length LZC=12XM. To have a unified design for all M, the sequence length for M=1 should be 12X which requires a non-power of 2 DFT to be transformed to the frequency domain. Hence, the constraint on the DFT size should be removed in the Working Assumption.

According to the current specifications (TS 38.211), there is no limitation on DFT size being a power of 2.

In the working assumption from RAN1#118bis regarding OOK-1 and OOK-4 [M=1], the following note should be removed: “DFT size is 2^n”.

## Early termination for OFDM WUR

Evaluations in the SI have shown that detecting information sent using OFDM sequences using an OFDM-based LP-WUR requires much smaller duration (in the order of few symbols) compared to overhead of sending information via OOK and detecting it using an OOK-based LP-WUR (in the order of slots). Figure 10 shows the detection performance of ZC sequence (overlaid on OOK-1) considering 1% FAR, 4 OFDM symbols duration, 0.5 ppm frequency error, and different number of sequences that are checked by the UE. As we can see, the duration of up to around 4 OFDM symbols is sufficient to reach the -3 dB SNR at 1% FAR.

Figure 10: Detection performance of OFDM sequence (ZC sequence overlaid on OOK1, 4 OFDM symbols) 1% and 0.1% FAR.

As shown in Figure 11, LP-WUS design should allow OFDM-based LP-WUR to detect the information payload with smaller monitoring duration compared to that of OOK-based LP-WUR. The per slot power consumption of OFDM based LP-WUR would be higher than that of OOK-based LP-WUR and having such ‘early termination’ of WUS monitoring is critical for obtaining power savings with OFDM-based LP-WUR.

Figure 11: LP-WUS with different monitoring duration for OFDM-based WUR vs OOK-based WUR.

Additionally, LP-WUS design should consider complexity reduction techniques to avoid excessive blind decodes or monitoring duration for payload detection using OFDM-based LP-WUR. This can be done for example by ensuring that sufficient ‘ON’ symbols are present at least in the initial part of LP-WUS. Alternatively, Manchester encoding can facilitate early termination as the UE can expect one ON symbol among two consecutive symbols.

For OFDM detection, WUS duration of around 4 OFDM symbols is sufficient to reach -3 dB SNR.

LP-WUS design should allow OFDM-based LP-WUR to detect the information sent using OFDM sequences using a smaller monitoring duration compared to that of OOK-based LP-WUR (which detects information sent via OOK). i.e., low detection complexity and early termination should be ensured for OFDM WUR.

## OFDM sequences and how to carry information

In RAN1#119 and RAN1#120, the following agreements were reached regarding number of overlaid OFDM sequences:

Agreement support maximum 4 candidates overlaid sequences for M=4 Agreement For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip.

Timing and frequency error for OFDM WUR:

In RAN1#118, the following was agreed:

Agreement As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift. For OOK-based LP-WUR, the LP-WUS design assumes the following: As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift.

Based on the above agreement on 0.1-5ppm frequency error for OFDM WUR, the frequency error is in the range of 0.26 kHz-13 kHz for 2.6 GHz frequency. Note that for OOK WUR the frequency error is 5-10ppm. Given that OFDM WUR has superior capability over the OOK WUR, its frequency error is expected to be considerably lower than the OOK WUR. For example, in our view around 0.5 ppm frequency error is reasonable for the OFDM WUR which results in 1.3 kHz frequency error at 2.6 GHz frequency.

The OFDM WUR is expected to use SSB for synchronization. The timing error for different SSS periodicities and residual frequency errors are provided in the table below. As we can see, for the OFDM WUR, the timing error due to the clock error is small (< 0.2 us). For sequence selection, the frequency error can be assumed to be less than few Hz and the timing error be considerably less than 1 us.

Table 3: Timing error due to clock error for OFDM WUR.

| SSB periodicity | OFDM WUR Fr=0.5 ppm, (F’=0.05 ppm/s) | OFDM WUR Fr=1 ppm | OFDM WUR Fr=2 ppm |
| --- | --- | --- | --- |
| 20 ms | 0.01 us | 0.02 us | 0.04 us |
| 40 ms | 0.02 us | 0.04 us | 0.08 us |
| 80 ms | 0.04 us | 0.08 us | 0.16 us |

For OFDM WUR, the frequency error can be up to a few kHz (e.g., 1-3 kHz) and the timing error is typically less than 0.2 us by relying on SSB.

Overlaid ZC sequences:

To enable early termination of OFDM WUR and maximizing the power saving gain, it should be possible to convey the WUS information in a short duration. Hence, from power saving perspective, a large number of candidate overlaid OFDM sequences is desired. However, another consideration is that the sequence should have good auto/cross correlation properties. For ZC sequence, different sequences can be created by combinations of different roots and cyclic shifts. For ZC sequence of length P (prime number), there are P-1 distinct sequences with different roots. In addition, for each root, different cyclic shifts can be applied to create more sequences.

Note that as long as the sequences have good cross correlation property, false alarm due to other sequence transmissions is not an issue. If the ZC sequences are generated with different roots and/or cyclic shifts (with sufficient separation), the cross correlations between them are low and do not increase the FAR. Figure 12 shows FAR due to transmissions from other sequences for two ZC sequences with different roots (left figure) and cyclic shifts (right figure). This result shows that FAR does not increase when ZC sequences have different roots. However, for the case with different cyclic shifts, the FAR can be significant when cyclic shifts are not separated enough. Hence, the ZC sequences should be generated with different roots or different cyclic shifts which are sufficiently separated (e.g., CS separation >5).

Figure 12: FAR due to other sequence transmissions (two sequences with different roots or different cyclic shifts).

To limit the UE complexity in terms of memory size, the number of sequences with different roots can be limited. However, sequences with different roots are more robust against timing error/uncertainty compared to different cyclic shifts. Hence, multiple roots need to be considered for OFDM sequences. For example, the number of different roots can be 4 and additional sequences are created by different cyclic shifts which are sufficiently separated. Assuming 4 roots for each M value:

- M=4: four sequence candidates are created using four different roots.

- M=2: eight sequence candidates are created using four different roots and two cyclic shifts per root.

- M=1: sixteen sequence candidates are created using four different roots and four cyclic shifts per root.

In general, sequences with different roots have low cross-correlation when they are relatively prime to the sequence length. To find the sequence roots, one approach is to find best roots (i.e., minimum cross-correlation) using exhaustive search. Table 4 provides some examples of ZC sequence roots (four values) for different M values where large root separation is ensured in each set.

Regarding the cyclic shifts, it is desired to be configurable by the network to have more flexibility for sequence generation. That is, for sequence length P, the network can configure any cyclic shift from 0 to P-1. In general, by increasing the separation of the cyclic shifts, sequences become more robust against timing uncertainty. Examples of different cyclic shifts are provided in Table 5.

Regarding whether the same or different sets of candidate overlaid sequences are used for different cells, our view is that it can be left to gNB implementation.

Table 4: Examples of different ZC roots (four roots are considered).

| M value and sequence length | Root set (based on exhaustive search, 4 roots) |
| --- | --- |
| M=1 (ZC sequence length: 131) | 2, 65, 68, 88 |
| M=2 (ZC sequence length: 61) | 11, 35, 42, 44 |
| M=4 (ZC sequence length: 31) | 5, 12, 23, 25 |

Table 5: Examples of different ZC cyclic shifts (four values are considered).

| M value and sequence length | Cyclic shifts with maximum separation (4 cyclic shifts) |
| --- | --- |
| M=1 (ZC sequence length: 131) | 0,32,65,98 |
| M=2 (ZC sequence length: 61) | 0,15,30,45 |
| M=4 (ZC sequence length: 31) | 0,7,15,23 |

For ZC sequence, different sequences can be created by combinations of different roots and cyclic shifts. For ZC sequence of length P (prime number), there are P-1 distinct sequences with different roots and multiple cyclic shifts for each root.

If the ZC sequences are generated with different roots and/or cyclic shifts (with sufficient separation), the cross correlations between them are low and do not increase the FAR.

To avoid any degradation on the sequence detection, the UE should only detect its intended sequences.

In the Idle mode the UE needs to detect up to 2 sequences: one its own sequence (or codepoint) and one common sequence (or codepoint).

The cyclic shifts of the OFDM sequences should be configured by the network.

Sequences with different roots are more robust against timing error/uncertainty compared to different cyclic shifts. Hence, multiple roots need to be considered for OFDM sequences.

The number of roots for OFDM sequences (in the specification) for each M value should be at least 4.

Optimal set of sequence roots can be determined based on a minimum cross-correlation metric. For four sequences roots, consider sets {2, 65, 68, 88}, {11, 35, 42, 44}, {5, 12, 23, 25} for M=1, 2, and 4, respectively.

How to carry information:

Based on RAN1#120 agreements the following alternative are considered for carrying OFDM information:

Agreement For WUS information carried by the overlaid OFDM sequence(s), consider at least the following alternatives: - Alt 1: Raw information bits are mapped to sequence(s) - Alt 2: Raw information bits are mapped to sequence(s) after channel coding - Same channel coding scheme(s) as OOK is applied. - FFS same or different rate matching and/or repetition factor as OOK - Alt 3: Codepoint/Subgroup is mapped to sequence(s)

To indicate a codepoint from N candidate codepoints, multiple symbols are required if only K<N candidate sequences are allowed per symbol as shown in Figure 13. The minimum number of required OOK ON symbols is log2N/log2K. Clearly, as the number of candidate sequences increases, the WUS duration can be shorter. To reach the coverage target, repetitions might also be needed.

Note that since the UE knows its subgroup ID, it should know its own sequence and there is no need to detect other sequences. That is, the receiver can exploit the knowledge of the sequence and improves its detection performance. In the Idle mode the UE needs to detect up to 2 sequences: one its own sequence (or codepoint) and one common sequence (or codepoint).

Alt 1 is a straightforward solution. The benefit of Alt 2 is not clear and it can increase the overhead.

In Alt 3 (from RAN1#120), each codepoint index can be mapped to a sequence and the UE detects its sequence to find the codepoint/subgroup. The sequence can be composed of multiple sequences as shown in Figure 13. In this case, a lookup table can provide mapping of  each of 32 codepoints to a sequence based on the number of overlaid sequence candidates. An example is provided in Table 1 for a case that there are 4 OFDM sequence candidates per OOK ON chip/symbol and the entire sequence spans over 3 OOK ON symbols. For example, to indicate codepoint 2, sequence z2: [s2,s1,s1] is transmitted over 3 OOK ON symbols.

The benefit of Alt 3 is that the mapping for subgroup ID can be determined by an entire sequence over multiple OOK symbols rather than bit-by-bit mapping done per OOK symbol and further optimization is possible by arranging the sequence candidates. Thus, Alt 3 can potentially provide better detection performance compared other alternatives. For example, Figure 14 shows the average cross-correlation for different sequence/codepoint indices (cross-correlation of each sequence with others) for Alt1 and Alt3 considering 32 codepoints, 4 overlaid sequences, and 3 OOK ON symbols. In Alt3, the order of OFDM sequences in different OOK symbols are further optimized to minimize the cross correlation. Example of codepoints/subgroup index mapping to OFDM sequences (3 OOK ON symbols, 4 OFDM sequence candidates) for Alt1 and Alt3 are provided in Table 6.

Figure 13: Decoding codepoint value via sequence detection.

Table 6: Example of codepoints/subgroup index mapping to OFDM sequences (3 OOK ON symbols, 4 OFDM sequence candidates) for Alt1 and Alt3.

| Codepoint (subgroup index) | Alt1: OFDM sequence pattern in (OOK ON symbol 1, OOK ON symbol 2, OOK ON symbol 3), {s1: 00, s2: 01, s3:10, s4:11} | Alt3: OFDM sequence pattern in (OOK ON symbol 1, OOK ON symbol 2, OOK ON symbol 3) with optimized ordering |
| --- | --- | --- |
| Z1 | s1 s1 s1 | s1 s1 s1 |
| Z2 | s1 s1 s3 | s2 s2 s2 |
| Z3 | s1 s2 s1 | s3 s3 s3 |
| Z4 | s1 s2 s3 | s4 s4 s4 |
| Z5 | s1 s3 s1 | s3 s2 s1 |
| Z6 | s1 s3 s3 | s4 s1 s2 |
| Z7 | s1 s4 s1 | s1 s4 s3 |
| Z8 | s1 s4 s3 | s2 s3 s4 |
| Z9 | s2 s1 s1 | s4 s3 s1 |
| Z10 | s2 s1 s3 | s3 s4 s2 |
| Z11 | s2 s2 s1 | s2 s1 s3 |
| Z12 | s2 s2 s3 | s1 s2 s4 |
| Z13 | s2 s3 s1 | s2 s4 s1 |
| Z14 | s2 s3 s3 | s1 s3 s2 |
| Z15 | s2 s4 s1 | s4 s2 s3 |
| Z16 | s2 s4 s3 | s3 s1 s4 |
| Z17 | s3 s1 s1 | s2 s1 s1 |
| Z18 | s3 s1 s3 | s1 s2 s2 |
| Z19 | s3 s2 s1 | s4 s3 s3 |
| Z20 | s3 s2 s3 | s3 s4 s4 |
| Z21 | s3 s3 s1 | s3 s1 s1 |
| Z22 | s3 s3 s3 | s4 s2 s2 |
| Z23 | s3 s4 s1 | s1 s3 s3 |
| Z24 | s3 s4 s3 | s2 s4 s4 |
| Z25 | s4 s1 s1 | s4 s1 s1 |
| Z26 | s4 s1 s3 | s3 s2 s2 |
| Z27 | s4 s2 s1 | s2 s3 s3 |
| Z28 | s4 s2 s3 | s1 s4 s4 |
| Z29 | s4 s3 s1 | s1 s2 s1 |
| Z30 | s4 s3 s3 | s2 s1 s2 |
| Z31 | s4 s4 s1 | s3 s4 s3 |
| Z32 | s4 s4 s3 | s4 s3 s4 |

Figure 14: Average cross-correlation for different sequence/codepoint indices (cross-correlation of each sequence with others).

The benefit of Alt 3 for carrying OFDM information is that the mapping of subgroup ID can be determined by an entire sequence over multiple OOK symbols and further optimization is possible by arranging the sequence candidates to reduce the cross-correlations/false alarms among the sequences.

To carry OFDM information, further consider “Alt3: Codepoint/Subgroup is mapped to sequence” or “Alt1: Raw information bits are mapped to sequence(s)”.  “Alt 2: Raw information bits are mapped to sequence(s) after channel coding” should not be considered.

## LP-WUS reliability and overhead in Connected mode

Here, we provide link-level results for LP-WUS detection and for PDCCH with different aggregation levels (ALs) and number of Rx antennas,

Figure 15 shows link-level results for PDCCH for different aggregation levels (ALs) and number of Rx antennas. Table 7 and Table 8 show the required SNR and WUS duration to match PDCCH performance (i.e., 1% MDR for WUS to match 1%BLER for PDCCH) for different PDCCH ALs. WUR noise figure (NF) of MR+2dB, MR+5dB and MR+8dB are assumed which are based on RAN1#117 evaluation agreement.

Figure 15: PDCCH BLER performance for different ALs and number of Rx antennas.

Table 7: Required SNR to match PDCCH (1%BLER) for LP-WUS with different WUR NF values

| PDCCH case | SNR at 1% BLER for PDCCH | WUS case: LR_NF = MR_NF+ 8dB | WUS case: LR_NF = MR_NF+ 5dB | WUS case: LR_NF = MR_NF+ 2dB |
| --- | --- | --- | --- | --- |
| AL1, 1Rx device | 17 dB | 10.7 dB | 13.3 dB | 15.7 dB |
| AL2, 1Rx device | 10 dB | 3.7 dB | 6.3 dB | 8.7 dB |
| AL4, 1Rx device | 5 dB | -1.3 dB | 1.3 dB | 3.7 dB |
| AL8, 1Rx device | 0 dB | -6.3 dB | -3.7 dB | -1.3 dB |
| AL16, 1Rx device | -3 dB | -9.3 dB | -6.7 dB | -4.3 dB |
| AL1, 2Rx device | 10 dB | 3.7 dB | 6.3 dB | 8.7 dB |
| AL2, 2Rx device | 4 dB | -2.3 dB | 0.3 dB | 2.7 dB |
| AL4, 2Rx device | -0.5 dB | -6.8 dB | -4.2 dB | -1.8 dB |
| AL8, 2Rx device | -3.5 dB | -9.8 dB | -7.2 dB | -4.8 dB |
| AL16, 2Rx device | -6 dB | -12.3 dB | -9.7 dB | -7.3 dB |
| AL1, 4Rx device | 4 dB | -2.3 dB | 0.3 dB | 2.7 dB |
| AL2, 4Rx device | -1.5 dB | -7.8 dB | -5.2 dB | -2.8 dB |
| AL4, 4Rx device | -5 dB | -11.3 dB | -8.7 dB | -6.3 dB |
| AL8, 4Rx device | -7.5 dB | -13.8 dB | -11.2 dB | -8.8 dB |
| AL16, 4Rx device | -9.5 dB | -15.8 dB | -13.2 dB | -10.8 dB |

Table 8: Required WUS duration (single sequence detection) to match PDCCH performance

| PDCCH case | SNR for WUS case: LR_NF = MR_NF+ 8dB | Required symbols for OOK WUS (LR_NF = MR_NF+ 8dB) | SNR for WUS case: LR_NF = MR_NF+ 5dB | Required symbols for OOK WUS (LR_NF = MR_NF+ 5dB) | SNR for WUS case: LR_NF = MR_NF+ 2dB | Required symbols for OFDM WUS (LR_NF = MR_NF+ 2dB |
| --- | --- | --- | --- | --- | --- | --- |
| AL1, 1Rx device | 10.7 dB | ≤2 symbols | 13.3 dB | ≤2 symbols | 15.7 dB | 1 symbol |
| AL2, 1Rx device | 3.7 dB | 2 symbols | 6.3 dB | ≤2 symbols | 8.7 dB | 1 symbol |
| AL4, 1Rx device | -1.3 dB | 10 symbols | 1.3 dB | 4 symbols | 3.7 dB | 1 symbol |
| AL8, 1Rx device | -6.3 dB | 57 symbols (>4 slots) | -3.7 dB | 23 symbols | -1.3 dB | 2 symbols |
| AL16, 1Rx device | -9.3 dB | 160 symbols (>10 slots) | -6.7 dB | 65 symbols (5 slots) | -4.3 dB | 4 symbols |
| AL1, 2Rx device | 3.7 dB | 2 symbols | 6.3 dB | ≤2 symbols | 8.7 dB | 1 symbol |
| AL2, 2Rx device | -2.3 dB | 14 symbols | 0.3 dB | 6 symbols | 2.7 dB | 1 symbol |
| AL4, 2Rx device | -6.8 dB | 65 symbols (5 slots) | -4.2 dB | 28 symbols | -1.8 dB | 2 symbols |
| AL8, 2Rx device | -9.8 dB | 190 symbols (>13 slots) | -7.2 dB | 78 symbols (6 slots) | -4.8 dB | 4 symbols |
| AL16, 2Rx device | -12.3 dB | 450 (>30 slots) | -9.7 dB | 184 (>10 slots) | -7.3 dB | 8 symbols |
| AL1, 4Rx device | -2.3 dB | 15 | 0.3 dB | 6 | 2.7 dB | 1 symbol |
| AL2, 4Rx device | -7.8 dB | 95 (>6slots) | -5.2 dB | 38 (3 slots) | -2.8 dB | 4 symbols |
| AL4, 4Rx device | -11.3 dB | 320 (>20 slots) | -8.7 dB | 130 (>9 slots) | -6.3 dB | 8 symbols |
| AL8, 4Rx device | -13.8 dB | 760 (>50 slots) | -11.2 dB | 308 (>20 slots) | -8.8 dB | 16 symbols |
| AL16, 4Rx device | -15.8 dB | 1500 (>100 slots) | -13.2 dB | 618 (>40 slots) | -10.8 dB | 24 symbols |

## Codepoint for WUS transmission in Connected mode

In RAN1#120, the following working assumption was made:

Working Assumption Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs: - Codepoint based - The maximum number of codepoints checked per MO by a UE is up to 8 - Depending on UE capability, a UE may support less than 8

In the connected mode, link adaptation is possible thus codepoint has the flexibility to address each UE based on its coverage condition. However, bitmap addresses multiple UEs thus it needs to cover UEs with the worst coverage. Therefore, unlike the bitmap case, the WUS duration for codepoint can be flexible. In terms of overhead, as shown in our RAN1#120 paper (R1-2501254), the overhead of codepoint is lower than bitmap. In particular, for low-medium load scenarios the overhead for bitmap approach is twice that of the codepoint approach. Moreover, given that codepoint is considered for Idle mode, the same scheme should be considered for connected mode to have a unified design. Therefore, codepoint-based scheme should be supported in connected mode.

To have the possibility to simultaneously address multiple UEs (similar to bitmap scheme), the UE can check multiple codepoints per MO. In RAN1#120 working assumption, the maximum number of codepoints checked per MO by a UE is up to 8. This allows addressing up to 8 different combinations of each UE with other UEs per MO which provides sufficient scheduling flexibility.

Considering this, we propose to confirm the WA from RAN1#120.

Confirm the Working Assumption from RAN1#120 regarding codepoint scheme in connected mode.

# LP-SS design

During the Rel-18 SI, different LP-WUR architectures were considered. One of the differentiating factors in terms of LP-WUR is the type of the signal it can detect or receive. WUR with only envelope detection capability can only detect amplitude modulated signal such as OOK signal, while a more capable LP-WUR can detect/receive signal with I/Q components, making it able to receive existing OFDM-based NR sequences. For the latter type, it is clarified in the WID objective that existing synchronization signal (e.g., SSS) can be used for the purpose of synchronization and RRM measurement.

For the WUR only capable of envelope detection (OOK-based WUR), if synchronization and/or RRM measurement is to be performed by WUR, a new synchronization signal, LP-SS, that is compatible with envelope detection is needed.

Figure 16. Flexible LP-SS placement options to minimize NW overhead and energy efficiency impact.

To ensure NW scheduling flexibility, it should be possible to have flexible placement of LP-SS resources in frequency and time. For example, flexibility to configure LP-SS resource near existing NR signals (FDM/TDM) is desirable to minimize gNB impact (Some examples shown in Figure 16). Since both SSB and LP-SS are broadcasted periodically, it is beneficial if the location of LP-SS can be set such that it allows the gNB to go into micro sleep mode to help with NW energy saving.

It should be possible for NW to flexibly configure the placement of LP-SS resources in frequency and time to minimize overhead and NW energy efficiency impact.

## LP-SS periodicity

LP-SS periodicity will have a direct impact on additional resource overhead as well as NW energy consumption. While smaller periodicity of LP-SS can improve the timing synchronization and RRM performance of WUR, such always-on transmission has considerable impact on NW energy consumption and resource overhead.   To limit the negative impacts on NW overhead, larger periodicity values are preferred. Thus, for flexible operation, LP-SS periodicity should be configurable by the NW. For candidate values for periodicity, the following range is preferred: 320ms, 640ms, 1280ms, 2560ms.

In the following, we evaluate the impact of LP-SS periodicity and duration on the NW energy consumption. Table 9 lists the evaluation assumptions for both baseline case (SSB and SIB1) and LP-SS. Table 10 shows LP-SS NW energy consumption compared to the baseline.

Table 9: Assumptions for LP-SS network energy consumption evaluation.

|  | Assumptions |
| --- | --- |
| Baseline | SSB: 20PRBs, 20ms periodicity, occupying 4 symbols, 8 beams SIB1: 20MHz, 160ms periodicity, occupying 1 slot, 8 beams |
| LP-SS | Duration: {4,6,8} OFDM symbols, Periodicity: {640, 320, 160, 80} ms, 11 PRBs, 8 beams |

Table 10: LP-SS NW energy consumption (relative to baseline)

| LP-SS duration | LP-SS periodicity [ms] | Additional network energy consumption |
| --- | --- | --- |
| 4 OFDM symbols | 80 | 5% |
|  | 160 | 2.3% |
|  | 320 | 1.2% |
|  | 640 | 0.6% |
| 6 OFDM symbols | 80 | 7.1% |
|  | 160 | 3.4% |
|  | 320 | 1.7% |
|  | 640 | 0.9% |
| 8 OFDM symbols | 80 | 9.4% |
|  | 160 | 4.5% |
|  | 320 | 2.3% |
|  | 640 | 1.2% |

The above results show that the additional network energy consumption can be significant for LP-SS periodicity of smaller than 320 ms. While it may be argued that smaller LP-SS periodicity can be beneficial for improving the timing accuracy, our results show that using preamble is more efficient than frequent LP-SS transmissions. Therefore, LP-SS periodicities smaller than 320 ms should not be supported.

The additional overhead and network energy consumption is significant for LP-SS periodicities 80ms or 160 ms.

LP-SS periodicities smaller than 320 ms should not be supported. Consider following values for configuring LP-SS periodicity: 320ms, 640ms, 1280ms, 2560ms.

## LP-SS duration and sequences

The following was agreed in RAN1#119 regarding LP-SS sequence length:

Agreement For the length L of LP-SS binary sequence, limit the selection to the following: - M=1, L= {4, 6, 8} - M=2, L= {8, 12, 16} - M=4, L= {16, 32}

### Timing error tolerance for OOK

Figure 17 show the impact of timing error on detection performance of OOK-1 and OOK-4 (overlaid with ZC sequence). As we can see:

- To keep SNR degradation <1dB due to timing error (at 1% MDR)

  - For OOK-1, timing error should be ≤ 6 us

  - For OOK-4, M=2, timing error should be ≤ 2 us

  - For OOK-4, M=4, timing error should be ≤ 1 us

Figure 17: Impact of timing error on OOK-1 and OOK-4 [M=2, 4] detection performance

To keep SNR degradation <1dB due to timing error (at 1%MDR)

- For OOK-1, timing error should be ≤ 6us

- For OOK-4, M=2, timing error should be ≤ 2us

- For OOK-4, M=4, timing error should be ≤ 1us

The timing synchronization accuracy achieved by the LP-SS depends on the LP-SS duration, SNR condition, and LP-SS waveform. Here we provide results for residual timing error after LP-SS detection.  Figure 18 shows LP-SS timing accuracy for OOK1 and OOK4 (M=2) at -3dB SNR. For receiver, we assumed low-detection complexity scheme where the receiver performs simple correlation with the received signal through a sliding window of +/-25us as per agreements. No additional operation (e.g., regarding CP, etc.) is considered on the UE side.

Figure 18: LP-SS timing accuracy for OOK1 and OOK4 (M=2) at -3dB SNR.

The timing accuracy improves by increasing the LP-SS duration due to increased signal energy. Nevertheless, doubling the LP-SS duration slightly improves the timing accuracy at the cost of increased system overhead and network energy consumption. Given that the number of different LP-SS sequences should be 4, LP-SS duration of 4 or 6 OFDM symbols can provide sufficient timing accuracy at least for OOK1 and OOK4 (M=2), and allows having four distinct sequences. The 4 or 6 symbols LP-SS duration corresponds to different lengths for different M values:

- M=1, L= {4,6}

- M=2, L= {8,12}

Note that with LP-SS duration 4 or 6 OFDM symbols, the LP-SS beam sweeping can be done more efficiently compared to the 8-symbol LP-SS. Typically, in a slot 2 OFDM symbols are allocated for PDCCH thus 12 OFDM symbols can be available for LP-SS. With LP-SS duration of 4 or 6 OFDM symbols, two or three beams can perfectly fit within 12 available OFDM symbols. However, this is not possible with 8-symbol LP-SS and around 4 OFDM symbols of a slot may not be utilized.

For M=4, two LP-SS lengths {16,32} are under consideration. LP-SS duration of 8 OFDM symbols is needed to reach the agreed timing accuracy for M=4 (as 4-symbol LP-SS is not enough). Hence, L=32 should be supported for M=4.

To ensure network flexibility for LP-SS transmissions and balance the tradeoff between overhead/network energy efficiency and timing accuracy in different SNR conditions, multiple LP-SS sequence lengths should be supported for each M value.

LP-SS duration of 4 or 6 OFDM symbols is suitable to provide sufficient timing accuracy for OOK1 and OOK4 (M=2). For M=4, the L-SS duration of 8 OFDM symbols is needed.

With LP-SS duration 4 or 6 OFDM symbols, the LP-SS beam sweeping can be done more efficiently compared to the 8-symbol LP-SS as two or three beams can perfectly fit within a slot with 12 available OFDM symbols.

To ensure network flexibility for LP-SS transmissions and balance the tradeoff between overhead/network energy efficiency and timing accuracy in different SNR conditions, multiple LP-SS sequence lengths should be supported for each M value.

LP-SS duration should be 4 or 6 OFDM symbols, corresponding to the following lengths for different M values: L= {4,6} for M=1, L= {8,12} for M=2, and L=32 for M=4.

### Sequences for M=4

For M=4, two sets of sequences were identified in RAN1#1120. Set 1 ensures balanced 1/0 within each OFDM symbol while Set 2 corresponds to unbalanced 1/0s.

Agreement For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between: - Set 1 [0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0] [0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0] [1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1] [1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] - Set 2 [1 0 0 0 1 0 0 0 0 0 0 1 0 0 1 0] [1 0 0 0 0 1 0 0 1 0 0 0 0 0 1 0] [1 0 0 0 0 1 0 0 0 1 0 0 1 0 0 0] [1 0 0 0 0 1 0 0 0 0 0 1 0 0 0 1] Agreement For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between: - Set 1: [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] [0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] [0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1] - Set 2 [0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0] [0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0] [0 0 1 0 0 0 1 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 0 1 0 0 1 0 0 0 1 0] [0 0 1 0 0 1 0 0 0 0 0 1 1 0 0 0 0 0 1 0 1 0 0 0 0 1 0 0 0 0 1 0]

In terms of timing accuracy assuming the same total transmit power, these two sets have similar performance. For example, based on our evaluations for L=32, the average timing accuracy using set 1 is around 1 us and it is around 0.95 us using set 2. However, as illustrated in Figure 19, set 2 has 3 dB higher PAPR compared to set 1 which has negative impact on the gNB implementation. Figure 20 show the CDF of PAPR for NR transmissions with and without WUS. As we can see, the PAPR increases for the unbalanced OOK [M=4] sequence. Therefore, considering the similar performance of these two sets while lower PAPR of set 1, set 1 should be supported.

Figure 19: Example of time domain signal for set 1 and set 2 of LP-SS sequences in one OFDM symbol assuming (same total transmit power).

Figure 20: CDF of PAPR with and without WUS.

For LP-SS sequences of M=4, set 1 should be supported as it has lower PAPR while it provides similar performance as set 2.

## Evaluation of RRM using LP-SS

Table 12 below shows initial evaluation of RSRP and RSRQ accuracy for different LP-SS waveform candidates and for SSS. Detailed evaluation assumptions are given in Annex. Results in the table indicate that at least for the evaluated scenarios (AWGN, TDL-C300_3 with no timing or frequency error) 2-6 OFDM symbols of LP-SS transmission duration is needed to achieve comparable RSRP/RSRQ accuracy as 1 symbol SSS.

Table 12: Evaluation for RSRP and RSRQ accuracy.

| Waveform used for RSRP measurement | #OFDM symbols used for RSRP measurement | SNR, channel assumed for RSRP measurement | RSRP accuracy (±XdB @ 90%, no Te) | RSRP accuracy (±XdB @ 90%,±2us Te ) |
| --- | --- | --- | --- | --- |
| SSS | 1 | -3dB, AWGN | 1.13 |  |
| LP-SS (OOK-1) | 2 | -3dB, AWGN | 1.15 | 1.8 |
| LP-SS (OOK-4, M=2) | 2 | -3dB, AWGN | 1.2 | 1.74 |
| LP-SS (OOK-4, M=4) | 2 | -3dB, AWGN | 1.84 | 2.62 |
| SSS | 1 | -6dB, AWGN | 0.58 |  |
| LP-SS (OOK-1) | 2 | -6dB, AWGN | 2.33 | 3.12 |
| LP-SS (OOK-4, M=2) | 2 | -6dB, AWGN | 2.01 | 2.8 |
| LP-SS (OOK-4, M=4) | 2 | -6dB, AWGN | 2.13 | 2.94 |
| SSS | 1 | -3dB, TDLC300_3 |  |  |
| LP-SS (OOK-1) | 4 | -3dB, TDLC300_3 | 1.26 | 2.13 |
| LP-SS (OOK-4, M=2) | 4 | -3dB, TDLC300_3 | 1.28 | 2.2 |
| LP-SS (OOK-4, M=4) | 4 | -3dB, TDLC300_3 | 1.32 | 2.37 |
| SSS | 1 | -6dB, TDLC300_3 | 0.88 |  |
| LP-SS (OOK-1) | 4 | -6dB, TDLC300_3 | 2.68 | 3.4 |
| LP-SS (OOK-4, M=2) | 4 | -6dB, TDLC300_3 | 2.12 | 3.14 |
| LP-SS (OOK-4, M=4) | 4 | -6dB, TDLC300_3 | 2.15 | 3.2 |

For AWGN and TDL-C300_3 channel model with -3dB or -6dB SNR: 2-6 OFDM symbols of LP-SS transmission duration is needed to achieve comparable RSRP/RSRQ accuracy (less than ~3dB error) as 1 symbol SSS.

# Preamble

The following agreement was reached in RAN1#118 regarding timing error evaluation:

Agreement For OOK-based LP-WUR, the LP-WUS design assumes the following: - As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift. Agreement As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift. Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4

In RAN1#119, the following was agreed regarding the need for additional synchronization signal (i.e., preamble):

Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present

## Need for preamble

In the SI, using a preamble for LP-WUS was discussed. In general, the use of preamble in the WUS structure is beneficial for fine time and frequency synchronization and improves LP-WUS detection performance. Having a preamble before data part of LP-WUS transmission also reduces the need for frequent LP-SS transmissions.

Table 11 presents timing error for 320 ms LP-SS periodicity (with 4 or 6 OFDM symbol duration), different residual frequency error (Fr) values, F’=0.1 ppm/s (frequency drift), and different residual timing errors (Tr) after LP-SS detection. The frequency error correction can be achieved by assistance from MR and WUR architecture with parallel filters for detecting LP-SS. The total timing error is determined based on residual frequency error and residual timing error. Results in Table 1 indicate that while LP-SS provides certain timing synchronization, it may not suitable to use such a periodic signal to provide sufficient for OOK-4 with M=2,4.

The use of preamble is more important for larger timing errors and in case of OOK-4 which is relatively more susceptible to the timing error. Another benefit of the preamble is that it avoids frequent transmission of the LP-SS as it assists LP-SS by providing fine synchronization. Therefore, preamble can be included in the WUS structure if LP-WUR implementations that do not provide accurate frequency error correction are to be supported. Note that the preamble helps correcting the timing error due to the clock drift and there is residual timing error after detecting the preamble. With preamble, the residual timing error is reduced at the cost of additional overhead.

The results in Table 11 show that the residual timing error for OOK-4 exceed the tolerable limit. For OOK-1, however, the residual timing error is acceptable assuming 320 ms LP-SS periodicity. It should be noted that OOK-1 can properly work without the preamble given that it is relatively robust against the timing error. To show this, in Figure 21 we compare the following cases (OOK-1, 28 total WUS duration, 0.1% FAR):

- WUS duration of 24 OFDM symbols with 4 symbols preamble (total WUS duration: 28 symbols), residual timing error is 3 us.

- WUS duration of 28 OFDM symbols without preamble, residual timing error is 6 us.

Figure 21 shows that the performance of the above two cases is similar. Hence, preamble is not necessary for OOK-1 as similar performance can be achieved by prolonging the WUS.

Table 11: Total timing error (ΔT) due to residual frequency error (Fr) and residual timing error (Tr) for different cases (with timing accuracy achieved by 4 symbols LP-SS).

| LP-SS periodicity and duration | Fr=5 ppm for OOK-WUR |  |  | Fr=10 ppm for OOK-WUR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 320 ms, 4 OFDM symbols | Tr=2.8 us [OOK-1] | Tr=1.5 us [OOK4, M=2] | Tr=1.4 us [OOK4, M=4] | Tr=2.8 us [OOK-1] | Tr=1.5 us [OOK4, M=2] | Tr=1.4 us [OOK4, M=4] |
|  | 4.4 us | 3.1 us | 3 us | 6 us | 4.7 us | 4.6 us |
| 320 ms, 6 OFDM symbols | Tr=2.2 us [OOK-1] | Tr=1.1 us [OOK4, M=2] | Tr=1 us [OOK4, M=4] | Tr=2.2 us [OOK-1] | Tr=1.1 us [OOK4, M=2] | Tr=1 us [OOK4, M=4] |
|  | 3.8 us | 2.7 us | 2.6 us | 5.4 us | 4.3 us | 4.2 us |

Figure 21: Detection performance of OOK-1 with and without preamble for the same overhead.

With 320 ms LP-SS periodicity, OOK-1 can properly work without the preamble given that it is relatively robust against the timing error. Hence, preamble is not necessary for OOK-1.

For OOK-4 (M=2, 4) and 320 ms LP-SS periodicity, the residual timing error can be reduced by preamble.

Including a preamble part before the data part of LP-WUS transmissions should be considered for OOK-based LP-WUS. The presence of preamble should be configurable.

## Preamble vs. more frequent LP-SS

Here, we evaluate the performance of preamble and LP-SS in terms of overhead and achieved timing accuracy.

Overhead:

- Preamble duration: 4 OFDM symbols

- LP-SS duration: 6 OFDM symbols

- 20 MHz system bandwidth, 5 MHz WUS/LP-SS total bandwidth, 8 beams for beam sweeping, and TDD UL/DL: 1:4

- Paging assumptions: 32 subgroups, 32 PO, 1024 number of UEs

The preamble overhead depends on the paging rate (i.e., number of WUS transmissions). In idle mode where relatively low paging rate is expected, the overhead of preamble can be smaller than LP-SS. For synchronization, two cases can be considered: 1) preamble+LP-SS and 2) more frequent LP-SS without preamble. In Table 12, the overhead of LP-SS overhead (320 ms periodicity) is considered as the baseline.

Based on the results in Table 12, we see that the overhead of preamble+LP-SS (320 ms periodicity) is smaller than LP-SS-only (80 ms periodicity) for per-UE paging rate 1%. Specifically, decreasing the LP-SS periodicity from 320 ms to 80 ms increases the overhead by 4x while by adding the preamble (with 320 ms LP-SS periodicity) the overhead increases by 1.02x-2.7x depending on the paging rate. Annex C also shows additional results with different preamble and LP-SS length.

Table 12: Overhead of preamble and LP-SS (normalized to overhead of 320ms LP-SS).

|  | 0.01% per UE paging rate | 0.1% per UE paging rate | 0.4% per UE paging rate | 1% per UE paging rate |
| --- | --- | --- | --- | --- |
| Preamble overhead | 0.02 | 0.2 | 0.7 | 1.7 |
| LP-SS overhead (320 ms periodicity) | 1 | 1 | 1 | 1 |
| LP-SS overhead (160 ms periodicity) | 2 | 2 | 2 | 2 |
| LP-SS overhead (80 ms periodicity) | 4 | 4 | 4 | 4 |
| Preamble+LP-SS (320 ms periodicity) | 1.02 | 1.2 | 1.7 | 2.7 |
| Note: according to the SI (TR 38.869), the following per-UE paging rates can be considered: RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s Note: for 0.4% per UE paging rate, the PO group paging rate is over 10%. Note: 4-symbol preamble and 6-symbol LP-SS are assumed. |  |  |  |  |

Timing accuracy:

Compared to LP-SS, the advantage of preamble is that the clock error/drift does not contribute to the timing error as WUS follows the preamble. As we can see, at the time of WUS detection, 4-symbol preamble can provide better timing accuracy than 6-symbol LP-SS with 80 ms or 160 ms periodicity. As shown above, the overhead of preamble is smaller than LP-SS with 80ms periodicity.

Moreover, decreasing the periodicity of LP-SS significantly increases the network energy consumption.

Table 13: Residual timing error after preamble or LP-SS detection.

|  | Residual timing error for WUS detection (OOK-1), Fr=10 ppm | Residual timing error for WUS detection (OOK-1), Fr=5 ppm |
| --- | --- | --- |
| Preamble (4 symbols) | 2.8 us | 2.8 us |
| LP-SS (6 sym, 320 ms periodicity) | 5.4 us | 3.8 us |
| LP-SS (6 sym, 160 ms periodicity) | 3.8 us | 3 us |
| LP-SS (6 sym, 80 ms periodicity) | 3 us | 2.6 us |
| Note: timing accuracy after detecting 6-symbol LP-SS is around 2.2 us in this case. |  |  |

The overhead of preamble+LP-SS (320 ms periodicity) can be smaller than LP-SS-only (with < 320m periodicity).

Decreasing the LP-SS periodicity from 320 ms to 80 ms increases the overhead by 4x while by adding the preamble (with 320 ms LP-SS periodicity) the overhead increases by 1.02x-2.7x depending on the paging rate.  Moreover, decreasing LP-SS periodicity significantly increases the network energy consumption.

Considering both overhead and timing accuracy, using the preamble is preferred over more frequent LP-SS transmissions.

LP-SS periodicity smaller than 320 ms should not be supported.

In short, among different options identified in RAN1#119, Option 1B should be supported for OOK-1:

- Option 1B: No additional sync signal, LP-SS periodicity =320ms

  - No additional support of other periodicities

For M>1, Option 2 can be considered:

- Option 2: With additional sync signal, LP-SS periodicity =320ms

  - FFS additionally support other LP-SS periodicities

  - FFS details on additional sync signal

  - FFS additional sync signal is configurable and/or conditionally present

Regarding additional synchronization signal (i.e., preamble), among options identified in RAN1#119, Option 1B should be supported at least for OOK-1. For OOK-4 (M>1), Option 2 can be considered further.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Compared to OOK-4 [M>1], OOK-1 [M=1] has lower complexity for both the UE and gNB and it can provide slightly better coverage. It is more robust against timing errors and requires a shorter LP-SS, leading to lower network energy consumption.

Observation 2	In the Idle/Inactive mode, OOK-4 [M>1] does not provide benefit over OOK1 while increasing both gNB and UE complexity and M=4 especially does not provide any benefit over M=2 while being more sensitive to timing error.

Observation 3	Even under ideal condition (without time/freq error) and single sequence detection, the OOK WUS duration of one slot is not sufficient to meet the -3dB SNR requirement.

Observation 4	For OOK based LP-WUS,

	For OOK-1 (M=1), 18-28 OFDM symbols are required to achieve 1%MDR for -3dB SNR depending on FAR assumption (0.1% vs 1%), and number of sequences checked by the UE.

	For OOK-4 (M=2,4) slightly more OFDM symbols than OOK-1 are needed to achieve same performance when same total power is assumed for the duration of LP-WUS transmission.

Observation 5	For mapping LP-WUS information to slots, some of the OFDM symbols in the slots must be reserved for other NR channels/signals and typically only a subset of OFDM symbols is available for LP-WUS mapping.

Observation 6	Regarding frequency position of LP-WUS/LP-SS:

	Alt 1 (LP-WUS/LP-SS frequency resource is confined within initial DL BWP) allows reusing existing BWP framework and procedures and reduces the need for UE RF returning and facilitates offloading of RRM measurements to WUR.

	The benefits of Alt2 (LP-WUS/LP-SS frequency resource can be out of initial DL BWP) over Alt 1 include more flexibility for placing WUS/LP-SS in the carrier, less congestion in the initial DL BWP, and potentially smaller overhead when supporting both RedCap and regular UEs.

Observation 7	Based on the current specifications, RedCap UEs should monitor paging in an initial DL BWP that contains CORESET#0 (i.e., UE may need to retune to regular initial DL BWP).

Observation 8	According to the current specifications (TS 38.211), there is no limitation on DFT size being a power of 2.

Observation 9	For OFDM detection, WUS duration of around 4 OFDM symbols is sufficient to reach -3 dB SNR.

Observation 10	For OFDM WUR, the frequency error can be up to a few kHz (e.g., 1-3 kHz) and the timing error is typically less than 0.2 us by relying on SSB.

Observation 11	For ZC sequence, different sequences can be created by combinations of different roots and cyclic shifts. For ZC sequence of length P (prime number), there are P-1 distinct sequences with different roots and multiple cyclic shifts for each root.

Observation 12	If the ZC sequences are generated with different roots and/or cyclic shifts (with sufficient separation), the cross correlations between them are low and do not increase the FAR.

Observation 13	To avoid any degradation on the sequence detection, the UE should only detect its intended sequences.

Observation 14	In the Idle mode the UE needs to detect up to 2 sequences: one its own sequence (or codepoint) and one common sequence (or codepoint).

Observation 15	Sequences with different roots are more robust against timing error/uncertainty compared to different cyclic shifts. Hence, multiple roots need to be considered for OFDM sequences.

Observation 16	The benefit of Alt 3 for carrying OFDM information is that the mapping of subgroup ID can be determined by an entire sequence over multiple OOK symbols and further optimization is possible by arranging the sequence candidates to reduce the cross-correlations/false alarms among the sequences.

Observation 17	The additional overhead and network energy consumption is significant for LP-SS periodicities 80ms or 160 ms.

Observation 18	To keep SNR degradation <1dB due to timing error (at 1%MDR)

	For OOK-1, timing error should be ≤ 6us

	For OOK-4, M=2, timing error should be ≤ 2us

	For OOK-4, M=4, timing error should be ≤ 1us

Observation 19	LP-SS duration of 4 or 6 OFDM symbols is suitable to provide sufficient timing accuracy for OOK1 and OOK4 (M=2). For M=4, the L-SS duration of 8 OFDM symbols is needed.

Observation 20	With LP-SS duration 4 or 6 OFDM symbols, the LP-SS beam sweeping can be done more efficiently compared to the 8-symbol LP-SS as two or three beams can perfectly fit within a slot with 12 available OFDM symbols.

Observation 21	For AWGN and TDL-C300_3 channel model with -3dB or -6dB SNR: 2-6 OFDM symbols of LP-SS transmission duration is needed to achieve comparable RSRP/RSRQ accuracy (less than ~3dB error) as 1 symbol SSS.

Observation 22	With 320 ms LP-SS periodicity, OOK-1 can properly work without the preamble given that it is relatively robust against the timing error. Hence, preamble is not necessary for OOK-1.

Observation 23	For OOK-4 (M=2, 4) and 320 ms LP-SS periodicity, the residual timing error can be reduced by preamble.

Observation 24	The overhead of preamble+LP-SS (320 ms periodicity) can be smaller than LP-SS-only (with < 320m periodicity).

Observation 25	Decreasing the LP-SS periodicity from 320 ms to 80 ms increases the overhead by 4x while by adding the preamble (with 320 ms LP-SS periodicity) the overhead increases by 1.02x-2.7x depending on the paging rate.  Moreover, decreasing LP-SS periodicity significantly increases the network energy consumption.

Observation 26	Considering both overhead and timing accuracy, using the preamble is preferred over more frequent LP-SS transmissions.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Following principles should be considered for LP-WUS and LP-SS design

a.	It should be possible to generate LP-WUS/LP-SS transmissions using existing gNB hardware and not trigger any new emissions or compliance requirements.

b.	It should be possible to multiplex the LP-WUS/LP-SS with other NR transmissions in time or frequency domain without causing interference.

c.	It should be possible to reuse any unused LP-WUS time and frequency resources for other transmissions.

Proposal 2	Paging misdetection performance of the UE should not be impacted when LP-WUS is used by the UE for power savings.

Proposal 3	Regarding M value for LP-WUS and LP-SS, Alt 1 “the M values for LP-WUS and LP-SS are always same” is supported.

Proposal 4	LP-WUS should support any number of codepoints from 1 to 32 (targeting up to 31 subgroups and one common codepoint).

Proposal 5	Regarding LP-WUS information bits, all cases of 1 information bit, 2 information bits and larger than 2 information bits should be supported.

Proposal 6	The OFDM symbols used for LP-WUS should be indicated to the UE by configuring number of slots (A) used for LP-WUS (A=1,2 or [3] slots) an additional parameter indicating the available OFDM symbols in each slot (e.g., by configuring a 14-bit bitmap or by using a SLIV value).

Proposal 7	If repetition is supported for OOK-based LP-WUS, the OOK symbols after Manchester coding are repeated N_rep times. The N_rep repetitions occur in same MO.

Proposal 8	Maximum N_rep=[5] repetitions can be supported in the specifications considering different deployment scenarios, gNB flexibility to reserve some symbols in each slot for other transmissions, UE implementation margins etc., resulting in maximum MO duration for LP-WUS (i.e., after maximum repetitions) of [15] slots.

Proposal 9	The code-block length after channel encoding (i.e., N in 38.212 in section 5.3.3.3) is min (Y, (F*M)/2), where F is the number of WUS OFDM symbols with Manchester encoding, M is the number of OOK symbols per OFDM symbol, and Y=1, 3, and 32 for 1-bit, 2-bit, and ≥2 bits information, respectively. When (F*M)/2 > Y, the code-block is cyclically extended to fit the available OOK symbols.

Proposal 10	It should be possible to support or indicate 32 UE subgroup with one WUS, no segmentation.

Proposal 11	For regular UEs, the LP-WUS/LP-SS should be located within the initial DL BWP (as in Alt1). For RedCap, the LP-WUS/LP-SS can be located outside the separate RedCap initial DL BWP (as in Alt2) but within an initial DL BWP that includes CORESET#0 used for paging.

Proposal 12	In the working assumption from RAN1#118bis regarding OOK-1 and OOK-4 [M=1], the following note should be removed: “DFT size is 2^n”.

Proposal 13	LP-WUS design should allow OFDM-based LP-WUR to detect the information sent using OFDM sequences using a smaller monitoring duration compared to that of OOK-based LP-WUR (which detects information sent via OOK). i.e., low detection complexity and early termination should be ensured for OFDM WUR.

Proposal 14	The cyclic shifts of the OFDM sequences should be configured by the network.

Proposal 15	The number of roots for OFDM sequences (in the specification) for each M value should be at least 4.

Proposal 16	Optimal set of sequence roots can be determined based on a minimum cross-correlation metric. For four sequences roots, consider sets {2, 65, 68, 88}, {11, 35, 42, 44}, {5, 12, 23, 25} for M=1, 2, and 4, respectively.

Proposal 17	To carry OFDM information, further consider “Alt3: Codepoint/Subgroup is mapped to sequence” or “Alt1: Raw information bits are mapped to sequence(s)”.  “Alt 2: Raw information bits are mapped to sequence(s) after channel coding” should not be considered.

Proposal 18	Confirm the Working Assumption from RAN1#120 regarding codepoint scheme in connected mode.

Proposal 19	It should be possible for NW to flexibly configure the placement of LP-SS resources in frequency and time to minimize overhead and NW energy efficiency impact.

Proposal 20	LP-SS periodicities smaller than 320 ms should not be supported. Consider following values for configuring LP-SS periodicity: 320ms, 640ms, 1280ms, 2560ms.

Proposal 21	To ensure network flexibility for LP-SS transmissions and balance the tradeoff between overhead/network energy efficiency and timing accuracy in different SNR conditions, multiple LP-SS sequence lengths should be supported for each M value.

Proposal 22	LP-SS duration should be 4 or 6 OFDM symbols, corresponding to the following lengths for different M values: L= {4,6} for M=1, L= {8,12} for M=2, and L=32 for M=4.

Proposal 23	For LP-SS sequences of M=4, set 1 should be supported as it has lower PAPR while it provides similar performance as set 2.

Proposal 24	Including a preamble part before the data part of LP-WUS transmissions should be considered for OOK-based LP-WUS. The presence of preamble should be configurable.

Proposal 25	LP-SS periodicity smaller than 320 ms should not be supported.

Proposal 26	Regarding additional synchronization signal (i.e., preamble), among options identified in RAN1#119, Option 1B should be supported at least for OOK-1. For OOK-4 (M>1), Option 2 can be considered further.

# References

3GPP RP-240801, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR),” 3GPP TSG RAN meeting #103, March 2024.

3GPP TR 38.869 v2.00, “Study on low-power Wake-up Signal and Receiver for NR),” Dec. 2023.

R1-2405108, “LP-WUS and LP-SS design”, RAN1#117, Fukuoka, Japan, May 2024.

# Annex A: evaluation assumptions

Table 14: Simulation assumptions for RSRP evaluations.

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2.6 GHz |
| Channel | AWGN, TDL-C |
| SCS | 30 kHz |
| IFFT size in transmitter | 2048 |
| Delay spread | 300 ns (TDL_C) |
| UE speed | 3 km/h, 30km/h (TDLC) |
| UE Rx antennas | 1 for WUR |
| WUR sampling rate | 7.68 M samples/s |
| SNR | -6 dB. -9 dB |
| Bandwidth | For OOK: - 5 MHz (for 30 kHz SCS: 11 PRBs+ ~ 1 PRB total guard band) For SSS: - 3.81 MHz (for 30 kHz SCS: 127 subcarriers for SSS) |
| Number of ADC bits | 8 |
| Frequency Error | none |
| Timing error | none |
| Receiver filter | 3th order Butterworth, filter BW is the same as WUS BW (~5MHz) |
| Non-WUS transmissions | Adjacent channel interference from other NR transmissions is considered. |
| OOK pattern | ON and OFF symbol is transmitted alternatively |
| RSRP calculation | Delta measurement = (estimated measurement – ideal measurement) [dB] |

# Annex B: additional LP-SS results

LP-SS timing accuracy for -6dB SNR OOK1

Figure 22: LP-SS timing accuracy for OOK-1 at -6dB SNR.

Preamble vs. LP-SS overhead for 8-sym LP-SS:

|  | 0.01% per UE paging rate | 0.1% per UE paging rate | 0.4% per UE paging rate | 1% per UE paging rate |
| --- | --- | --- | --- | --- |
| Preamble overhead | 0.02 | 0.15 | 0.6 | 1.3 |
| LP-SS overhead (320 ms periodicity) | 1 | 1 | 1 | 1 |
| LP-SS overhead (160 ms periodicity) | 2 | 2 | 2 | 2 |
| LP-SS overhead (80 ms periodicity) | 4 | 4 | 4 | 4 |
| Preamble+LP-SS (320 ms periodicity) | 1.02 | 1.15 | 1.6 | 2.3 |
| Note: according to the SI (TR 38.869), the following per-UE paging rates can be considered: RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s Note: for 0.4% per UE paging rate, the PO group paging rate is over 10%. Note: 4-symbol preamble and 8-symbol LP-SS are assumed. |  |  |  |  |

Table 15: Residual timing error after preamble or LP-SS detection.

|  | Residual timing error for WUS detection (OOK-1), Fr=10 ppm | Residual timing error for WUS detection (OOK-1), Fr=5 ppm |
| --- | --- | --- |
| Preamble (4 symbols) | 2.5 us | 2 us |
| LP-SS (6 sym, 640 ms periodicity) | 7.9 us | 4.7 us |
| LP-SS (6 sym, 320 ms periodicity) | 4.7 us | 3.1 us |
| LP-SS (6 sym, 160 ms periodicity) | 3.1 us | 2.3 us |
| LP-SS (6 sym, 80 ms periodicity) | 2.3 us | 1.9 us |
| Note: timing accuracy after detecting 8-symbol LP-SS is around 1.5 us in this case. |  |  |