3GPP TSG-RAN WG1 Meeting #120bis	R1-2502811

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.2.2

Source:	Ericsson

Title:	Remaining issues on CSI enhancements for large antenna arrays and CJT

Document for:	Discussion, Decision

# 1	Introduction

In the work item description (WID) for NR MIMO Phase 5 we have the following objectives related to CSI enhancement and CJT.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

In this paper, we provide Ericsson’s views on the maintainance issues related to the above objectives.

# 2	CSI Support for up to 128 ports

## 2.1 Extension of Rel-19 Type I codebooks to number of ports ≤32

During pre-RAN1#119 offline email discussion [3], it was discussed whether to extend Rel-19 Type I codebooks introduced for 48, 64, and 128 ports to smaller of ports as well.  We provide some simulation results to compare the performances between Rel-15 and Rel-19 Type I codebooks for 32 ports.  The results are provided in Table 2 for ranks 1-4, and the evaluation assumptions are provided in the appendix in Table 4.

As can be seen in the results, Rel-19 Type I Scheme A (no array splitting) shows around 3% (4%) gain at 50% RU and around 7% (8%) gain at 70% RU when compared to the Rel-15 baseline for mean user throughput (cell edge throughput).  Rel-19 Type I Scheme B shows around 7% (12%) gain at 50% RU and around 16% (24%) gain at 70% RU when compared to the Rel-15 baseline for mean user throughput (cell edge throughput). ​

Our results suggest that Rel-19 Type I Scheme B provides larger gains than Rel-19 Type I Scheme A.  Based on the results, we support extension of Rel-19 Type I codebooks to number of ports smaller or equal to 32.  To avoid introduction of too many UE capabilities, our preference is to introduce this extension to only 16, 24, and 32 ports.

- Proposal 1:  For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to 16, 24, and 32 CSI-RS ports, for all applicable RI values with K=1 only.

Table 2: Comparison between Rel-15 and Rel-19 Type I codebooks for 32 ports for ranks 1-4

## 2.2 CSI-RS port indexing

- One issue that still needs to be resolved is which specification CSI-RS port indexing for enhancements related to 48, 64, and 128 ports in Rel-19.  Currently, the port indexing aspect is captured in draft CRs of both 38.211 [1] and 38.214 [2].  In clause 7.4.1.5.3 of 38.211 [1], the specification enhancement is written such that the channel measurements are performed

- on CSI-RS ports p=3000, 3001,…, 3047 for the 48 port case,

- on CSI-RS ports p=3000, 3001,…, 3063 for the 64 port case, and

- on CSI-RS ports p=3000, 3001,…, 3127 for the 128 port case.

- In addition, changes to how the sequence r(m) is mapped to different resource elements are also captured in the draft CR of 38.211 [1].  Note that RAN1 has previously specified aggregation-based CSI-RS in Rel-13/14 for LTE advanced-Pro where CSI-RS port indexing for larger number of ports was specified in TS 38.211.

- In contrast, the text in draft CR of 38.214 introduces a new notion of codebook port index which deviates from earlier specification approaches to CSI-RS port indexing.  In our view, it is sufficient to capture CSI-RS port indexing for large number of ports in 38.211 and we don’t see the need to introduce a new concept such as codebook port indexing.  Hence, we propose to remove the relevant texts related to codebook port indexing from the draft CR of 38.214.

- Proposal 2:  Retain the CSI-RS port indexing for 48, 64, and 128 ports in draft CR for 38.211 and remove texts related to codebook port indexing from draft CR for 38.214.  Agree TP1 proposed below:

- ---------------------------------------------------  Start of TP #1 for 38.214  ------------------------------------------------------

##### 5.2.1.4.2	Report quantity configurations

<Omitted Text>

-	the higher layer bitmap parameter portSubsetIndicator-r19, of length 48, 64 or 128 bits, which contains the bit sequence b0,b1,…,bP-1, where b0 is the MSB and bP-1 is the LSB, bit bp' corresponds to the codebook CSI-RS antenna port p'=0,1,…,P-1, after CSI-RS resource aggregation, in order of increasing codebook CSI-RS antenna port index as specified in Clause 7.4.1.5 of [4, TS 38.211] , and codebook port p' corresponds to antenna port 3000+p, with index p∈0,1,…,Pm-1, of CSI-RS resource k∈{0,1,…,K-1}, according to the mapping configured by higher layer parameter portMappingMethod, for the values of N1 and N2 configured by the higher layer parameter n1-n2-typeI-r19, as described in Clause 5.2.2.2.1a, where K is the number of CSI-RS resources configured in the resource set for channel measurement and Pm=P/K is the number of ports per resource, configured by higher layer parameter nrofPorts. A bit value 0 in portSubsetIndicator-r19 indicates that the corresponding antenna port is disabled for the sub-configuration, whereas bit value 1 indicates that the antenna port is enabled and belongs to the antenna port subset for the sub-configuration. For the derivation of PMI, codebook ports corresponding to all bits with value of 1 in portSubsetIndicator-r19 are mapped to consecutive codebook ports in increasing order of the bit position in portSubsetIndicator-r19.

<Omitted Text>

##### 5.2.2.2.1a	Refined Type I Single-Panel Codebook

<Omitted Text>

-	The port mapping method between the antenna port 3000+p, with index p=0,1,…,PCSI-RS/K-1, of CSI-RS resource k=0,1,…,K-1, and the codebook port index, p'=0,1,…,PCSI-RS-1, obtained by aggregating K CSI-RS resources, as described in Clause 5.2.1.4.1, is configured with the higher layer parameter portMappingMethod. Let the port index p within a CSI-RS resource k correspond to the intermediate indices (ι1,ι2,q), such that p=n1n2q+n2ι1+ι2, where ι1=0,1,…,n1-1, ι2=0,1,…n2-1, and q=0,1 is the polarization index, and where (n1,n2) are defined such that N1,N2=(Kn1,n2) if portMappingMethod is set to 'method1', or N1,N2=(n1,Kn2) if portMappingMethod is set to 'method2'. The codebook port index, p', is obtained from the indices (ι1,ι2,q,k) as follows.

-	If portMappingMethod is set to 'method1',

p'=N1N2q+N1N2Kk+N2ι1+ι2

-	If portMappingMethod is set to 'method2',

p'=N1N2q+N2ι1+N2Kk+ι2

<Omitted Text>

##### 5.2.2.2.2a	Refined Type I Multi-Panel Codebook

<Omitted Text>

-	The port mapping method between the antenna port 3000+p, with index p=0,1,…,PCSI-RS/K-1, of CSI-RS resource k=0,1,…,K-1, and the codebook port index, p'=0,1,…,PCSI-RS-1, obtained by aggregating K CSI-RS resources, as described in Clause 5.2.1.4.1, is obtained as follows. Let the port index p within a CSI-RS resource k correspond to the intermediate indices (ι1,ι2,q), such that p=N1N2q+n2ι1+ι2, where ι1=0,1,…,N1-1, ι2=0,1,…N2-1, and q=0,1 is the polarization index. The codebook port index, p', is given by

p'=KN1N2q+N1N2k+N2ι1+ι2

<Omitted Text>

##### 5.2.2.2.5a	Refined eType II Codebook

<Omitted Text>

-	The port mapping method between the antenna port 3000+p, of index p=0,1,…,PCSI-RS/K-1, of CSI-RS resource k=0,1,…,K-1, and the codebook port index, p'=0,1,…,PCSI-RS-1, obtained by aggregating K CSI-RS resources, as described in Clause 5.2.1.4.1, is configured with the higher layer parameter portMappingMethod and defined as in Clause 5.2.2.2.1a.

<Omitted Text>

##### 5.2.2.2.9a	Refined FeType II Port Selection Codebook

<Omitted Text>

-	The port mapping method between the antenna port 3000+p, of index p=0,1,…,PCSI-RS/K-1, of CSI-RS resource k=0,1,…,K-1, and the codebook port index, p'=0,1,…,PCSI-RS-1, obtained by aggregating K CSI-RS resources, as described in Clause 5.2.1.4.1, is configured with the higher layer parameter portMappingMethod and defined as in Clause 5.2.2.2.1a.

<Omitted Text>

##### 5.2.2.2.11a	Refined eType II Codebook for predicted PMI

<Omitted Text>

-	The port mapping method between the antenna port 3000+p, of index p=0,1,…,PCSI-RS/K-1, of CSI-RS resource k=0,1,…,K-1, and the codebook port index, p'=0,1,…,PCSI-RS-1, obtained by aggregating K CSI-RS resources, as described in Clause 5.2.1.4.1, is configured with the higher layer parameter portMappingMethod and defined as in Clause 5.2.2.2.1a.

<Omitted Text>

-	The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Clause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+P-1], as given by

y3000(i)⋯y3000+P-1(i)=W(i)x0(i)⋯xν-1(i)

where  is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211], P∈1,2,4,8,12,16,24,32, 48, 64, 128 is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Clause 5.2.1.4.2. The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1

-	For a UE configured with 48, 64 and 128 antenna ports, obtained by aggregating K CSI-RS resources for channel measurement, for CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…,1000+υ-1], for υ layers, would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000+p0,3000+p1,…,3000+p(P-1)], with index p(p')∈0,1,…,P/K-1, of the respective CSI-RS resource [k0,k1,…,k(P-1)] with kp'∈{0,1,…,K-1}, corresponding to the codebook ports p'=0,1,…,P-1, in order of increasing codebook port index, according to the mapping described in Clause 5.2.2.2.1a for codebookType set to 'typeI-SinglePanel-r19', 'eTypeII-r19', 'typeII-FePortSelection-r19' or 'typeII-Doppler-r19' and in Clause 5.2.2.2.2a for codebookType set 'typeI-MultiPanel-r19', such that

yk0(3000+p(0))(i)yk1(3000+p(1))(i)⋮ykP-1(3000+p(P-1))(i)=W(i)x0(i)⋯xυ-1(i)

---------------------------------------------------  End of TP #1 for 38.214  -----------------------------------------------------

## 2.3 Spatial beam indication for Refined eType II Codebook

In the draft CR for 38.214 [2], the refined eType II codebook is captured in Clause 5.2.2.2.5a.  In Clause 5.2.2.2.5a, it is stated “The codebook is defined as in Clause 5.2.2.2.5”.  In Clause 5.2.2.2.5 of 38.214, the following text describes identification of the L spatial vectors.

“L vectors, vm1i,m2(i),i=0,1,…,L-1, are identified by the indices q1,q2, n1, n2, indicated by i1,1, i1,2, obtained as in 5.2.2.2.3, where the values of Cx,y are given in Table 5.2.2.2.5-4.”

It should be noted that Table 5.2.2.2.5-4 of 38.214 only contains 19 rows (with x index up to 18) which is not sufficient to indicate spatial beam vectors for up to 128 ports.  However, in draft CR for 38.214 [2] a new table is captured for combinatorial coefficients C(x,y) in Table 5.2.2.2.1a-5.  For identification of the L spatial vectors, for the Refined eTypeII codebook of Rel-19, the Tables 5.2.2.2.5-4 and 5.2.2.2.1a-5 can be used together.  Hence, we make the following proposal:

- Proposal 3:  For spatial beam indication of refined eTypeII codebook with 48, 64, and 128 ports in draft CR for 38.214 agree TP2 proposed below:

- ---------------------------------------------------  Start of TP #2 for 38.214  ------------------------------------------------------

##### 5.2.2.2.5a	Refined eType II Codebook

For 48, 64 and 128 antenna ports, obtained by aggregating K CSI-RS resources for channel measurement, and the UE configured with higher layer parameter codebookType set to 'eTypeII-r19'

<Omitted Text>

The codebook is defined as in Clause 5.2.2.2.5. L vectors, vm1i,m2(i),i=0,1,…,L-1, are identified by the indices q1,q2, n1, n2, indicated by i1,1, i1,2, obtained as in 5.2.2.2.3, where the values of Cx,y are given in Table 5.2.2.2.5-4 and and Table 5.2.2.2.1a-5.

The bitmap parameter typeII-CBSR-r19 forms the bit sequence aAc-1,…,a1,a0, where a0 is the LSB and aAc-1 is the MSB and the number of bits is given by Ac=N1N2/X1(c)X2(c), where the values X1(c),X2(c) are configured by the higher layer parameters valueOfX1-typeII-CBSR-r19 and valueOfX2-typeII-CBSR-r19, respectively. The supported combinations of X1(c),X2(c) for a given combination of (N1,N2) are given in Table. A bit value of zero for bit aN2X2(c)x1+x2 indicates that PMI reporting is not allowed to correspond to any precoder associated with vector group Ga(x1,x2), x1=0,…,N1X1(c)-1, x2=0,…,N2X2(c)-1, given by

---------------------------------------------------  End of TP #2 for 38.214  -----------------------------------------------------

# 4. Conclusion

- Proposal 1:  For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to 16, 24, and 32 CSI-RS ports, for all applicable RI values with K=1 only.

- Proposal 2:  Retain the CSI-RS port indexing for 48, 64, and 128 ports in draft CR for 38.211 and remove texts related to codebook port indexing from draft CR for 38.214.  Agree TP1 proposed below:

- --------------------------------------------------  Start of TP #1 for 38.214  ------------------------------------------------------

##### 5.2.1.4.2	Report quantity configurations

<Omitted Text>

-	the higher layer bitmap parameter portSubsetIndicator-r19, of length 48, 64 or 128 bits, which contains the bit sequence b0,b1,…,bP-1, where b0 is the MSB and bP-1 is the LSB, bit bp' corresponds to the codebook CSI-RS antenna port p'=0,1,…,P-1, after CSI-RS resource aggregation, in order of increasing codebook CSI-RS antenna port index as specified in Clause 7.4.1.5 of [4, TS 38.211] , and codebook port p' corresponds to antenna port 3000+p, with index p∈0,1,…,Pm-1, of CSI-RS resource k∈{0,1,…,K-1}, according to the mapping configured by higher layer parameter portMappingMethod, for the values of N1 and N2 configured by the higher layer parameter n1-n2-typeI-r19, as described in Clause 5.2.2.2.1a, where K is the number of CSI-RS resources configured in the resource set for channel measurement and Pm=P/K is the number of ports per resource, configured by higher layer parameter nrofPorts. A bit value 0 in portSubsetIndicator-r19 indicates that the corresponding antenna port is disabled for the sub-configuration, whereas bit value 1 indicates that the antenna port is enabled and belongs to the antenna port subset for the sub-configuration. For the derivation of PMI, codebook ports corresponding to all bits with value of 1 in portSubsetIndicator-r19 are mapped to consecutive codebook ports in increasing order of the bit position in portSubsetIndicator-r19.

<Omitted Text>

##### 5.2.2.2.1a	Refined Type I Single-Panel Codebook

<Omitted Text>

-	The port mapping method between the antenna port 3000+p, with index p=0,1,…,PCSI-RS/K-1, of CSI-RS resource k=0,1,…,K-1, and the codebook port index, p'=0,1,…,PCSI-RS-1, obtained by aggregating K CSI-RS resources, as described in Clause 5.2.1.4.1, is configured with the higher layer parameter portMappingMethod. Let the port index p within a CSI-RS resource k correspond to the intermediate indices (ι1,ι2,q), such that p=n1n2q+n2ι1+ι2, where ι1=0,1,…,n1-1, ι2=0,1,…n2-1, and q=0,1 is the polarization index, and where (n1,n2) are defined such that N1,N2=(Kn1,n2) if portMappingMethod is set to 'method1', or N1,N2=(n1,Kn2) if portMappingMethod is set to 'method2'. The codebook port index, p', is obtained from the indices (ι1,ι2,q,k) as follows.

-	If portMappingMethod is set to 'method1',

p'=N1N2q+N1N2Kk+N2ι1+ι2

-	If portMappingMethod is set to 'method2',

p'=N1N2q+N2ι1+N2Kk+ι2

<Omitted Text>

##### 5.2.2.2.2a	Refined Type I Multi-Panel Codebook

<Omitted Text>

-	The port mapping method between the antenna port 3000+p, with index p=0,1,…,PCSI-RS/K-1, of CSI-RS resource k=0,1,…,K-1, and the codebook port index, p'=0,1,…,PCSI-RS-1, obtained by aggregating K CSI-RS resources, as described in Clause 5.2.1.4.1, is obtained as follows. Let the port index p within a CSI-RS resource k correspond to the intermediate indices (ι1,ι2,q), such that p=N1N2q+n2ι1+ι2, where ι1=0,1,…,N1-1, ι2=0,1,…N2-1, and q=0,1 is the polarization index. The codebook port index, p', is given by

p'=KN1N2q+N1N2k+N2ι1+ι2

<Omitted Text>

##### 5.2.2.2.5a	Refined eType II Codebook

<Omitted Text>

-	The port mapping method between the antenna port 3000+p, of index p=0,1,…,PCSI-RS/K-1, of CSI-RS resource k=0,1,…,K-1, and the codebook port index, p'=0,1,…,PCSI-RS-1, obtained by aggregating K CSI-RS resources, as described in Clause 5.2.1.4.1, is configured with the higher layer parameter portMappingMethod and defined as in Clause 5.2.2.2.1a.

<Omitted Text>

##### 5.2.2.2.9a	Refined FeType II Port Selection Codebook

<Omitted Text>

-	The port mapping method between the antenna port 3000+p, of index p=0,1,…,PCSI-RS/K-1, of CSI-RS resource k=0,1,…,K-1, and the codebook port index, p'=0,1,…,PCSI-RS-1, obtained by aggregating K CSI-RS resources, as described in Clause 5.2.1.4.1, is configured with the higher layer parameter portMappingMethod and defined as in Clause 5.2.2.2.1a.

<Omitted Text>

##### 5.2.2.2.11a	Refined eType II Codebook for predicted PMI

<Omitted Text>

-	The port mapping method between the antenna port 3000+p, of index p=0,1,…,PCSI-RS/K-1, of CSI-RS resource k=0,1,…,K-1, and the codebook port index, p'=0,1,…,PCSI-RS-1, obtained by aggregating K CSI-RS resources, as described in Clause 5.2.1.4.1, is configured with the higher layer parameter portMappingMethod and defined as in Clause 5.2.2.2.1a.

<Omitted Text>

-	The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Clause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+P-1], as given by

y3000(i)⋯y3000+P-1(i)=W(i)x0(i)⋯xν-1(i)

where  is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211], P∈1,2,4,8,12,16,24,32, 48, 64, 128 is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Clause 5.2.1.4.2. The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1

-	For a UE configured with 48, 64 and 128 antenna ports, obtained by aggregating K CSI-RS resources for channel measurement, for CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…,1000+υ-1], for υ layers, would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000+p0,3000+p1,…,3000+p(P-1)], with index p(p')∈0,1,…,P/K-1, of the respective CSI-RS resource [k0,k1,…,k(P-1)] with kp'∈{0,1,…,K-1}, corresponding to the codebook ports p'=0,1,…,P-1, in order of increasing codebook port index, according to the mapping described in Clause 5.2.2.2.1a for codebookType set to 'typeI-SinglePanel-r19', 'eTypeII-r19', 'typeII-FePortSelection-r19' or 'typeII-Doppler-r19' and in Clause 5.2.2.2.2a for codebookType set 'typeI-MultiPanel-r19', such that

yk0(3000+p(0))(i)yk1(3000+p(1))(i)⋮ykP-1(3000+p(P-1))(i)=W(i)x0(i)⋯xυ-1(i)

---------------------------------------------------  End of TP #1 for 38.214  -----------------------------------------------------

- Proposal 3:  For spatial beam indication of refined eTypeII codebook with 48, 64, and 128 ports in draft CR for 38.214 agree TP2 proposed below:

- --------------------------------------------------  Start of TP #2 for 38.214  ------------------------------------------------------

##### 5.2.2.2.5a	Refined eType II Codebook

For 48, 64 and 128 antenna ports, obtained by aggregating K CSI-RS resources for channel measurement, and the UE configured with higher layer parameter codebookType set to 'eTypeII-r19'

<Omitted Text>

The codebook is defined as in Clause 5.2.2.2.5. L vectors, vm1i,m2(i),i=0,1,…,L-1, are identified by the indices q1,q2, n1, n2, indicated by i1,1, i1,2, obtained as in 5.2.2.2.3, where the values of Cx,y are given in Table 5.2.2.2.5-4 and and Table 5.2.2.2.1a-5.

The bitmap parameter typeII-CBSR-r19 forms the bit sequence aAc-1,…,a1,a0, where a0 is the LSB and aAc-1 is the MSB and the number of bits is given by Ac=N1N2/X1(c)X2(c), where the values X1(c),X2(c) are configured by the higher layer parameters valueOfX1-typeII-CBSR-r19 and valueOfX2-typeII-CBSR-r19, respectively. The supported combinations of X1(c),X2(c) for a given combination of (N1,N2) are given in Table. A bit value of zero for bit aN2X2(c)x1+x2 indicates that PMI reporting is not allowed to correspond to any precoder associated with vector group Ga(x1,x2), x1=0,…,N1X1(c)-1, x2=0,…,N2X2(c)-1, given by

---------------------------------------------------  End of TP #2 for 38.214  -----------------------------------------------------

# References

R1-2501656, draft CR for 38.211 on Introduction of NR MIMO Phase 5, RAN1#120, Athens, Greece, February 17-21, 2025.

R1-2501667, draft CR for 38.214 on Introduction of CSI enhancements for NR MIMOO Phase 5, RAN1#120, Athens, Greece, February 17-21, 2025.