3GPP TSG RAN WG1 #120-bis			R1-2502995

Wuhan, China, April 7th – 11th, 2025

Agenda item:	9.14

Source: 	EBU, SWR, ORS, BNE, Rohde & Schwarz, Qualcomm Incorporated

Title: 	Muting of always-on signals for 5G Broadcast

Document for:	Discussion and Decision

# Motivation

Existing broadcast technologies such as DVB-T2 [1] presume the usage of dedicated receivers to display TV programmes. Almost all DVB-T deployments are based on stationary reception even though the standard allows for serving devices under mobile receiving conditions. However, existing broadcast transmissions cannot be received on 3GPP devices such as smartphones and tablets. 5G Broadcast has been specified in 3GPP starting with Release 14 and following in order to reach mobile users with attractive linear broadcast content. To this end, existing broadcast transmitter networks can be utilized using broadcast spectrum in the band 470 – 694/698 MHz. However, in most countries this spectrum is heavily used by existing DVB-T2 deployments. In order to facilitate the deployment of 5G Broadcast services Time-Division-Multiplexing (TDM) of a given UHF channel would provide a means to pave the way for a low-barrier market introduction of 5G Broadcast. This would meet the needs of broadcasters for future technology migration paths. DVB-T2 already has in-built provisions for TDM with other technologies via the concept of Future Extension Frames (FEF). As of now, sharing of a UHF channel between DVB-T2 and 5G Broadcast is not feasible as the latter technology does not foresee corresponding mechanisms. However, it appears to be rather simple and straightforward to do so by muting parts of the 5G Broadcast signal. This only calls for a simple signalling addition to the 5G Broadcast configuration by defining the corresponding parameters needed for periodic muting of CAS, MCCH and MSI. This document provides a detailed description how to implement such a feature.

# Muting always-on signals in 5G Broadcast

The “always on signals” in an MBMS-dedicated cell are:

- the Cell Acquisition Subframe (CAS), which is a non-MBSFN subframe of 1 ms duration, occurring once every 40 ms. The CAS includes the synchronization signals—PSS/SSS/PBCH (containing MIB-MBMS for MBMS-dedicated cells), as well as the SIBs, including SIB1-MBMS for MBMS-dedicated cells;

- the MCCH, which conveys common control information for all the PMCHs in an MBSFN Area. The MCCH has a preconfigured periodicity from within a set of once every {1,2,4,8,16,32,64,128,256} radio frames (rfs);

- the MCH scheduling information (MSI) MAC-CE, which is associated with each PMCH in a list of PMCHs in a MBSFN Area. The MSI has a preconfigured periodicity from within a set of once every {1,2,4,8,16,32,64,128,256,512,1024} radio frames (rfs).

Observation 1: The signals that need to be muted, to create blanks in 5G Broadcast transmissions in an MBMS-dedicated cell (which may be used to time-division multiplex other broadcast technologies with 5G Broadcast) are:

- The Cell Acquisition Subframe (CAS)

- The MCCH

- The MSI

The signals transmitted in the CAS may be combined (e.g., to increase the received SNR) over up to 4 consecutive CAS transmissions—i.e., over a time duration of 4×40=160 ms—during which the contents of the CAS (e.g., the MIB-MBMS and SIB1-MBMS) do not change. Each such 160 ms duration described above starts in a radio frame with system frame number (SFN) given by SFN mod 16=0.

The structure of the CAS described above dictates that a repeating period, comprising a CAS ON duration followed by a CAS muted duration, must be a multiple of 160 ms, so that (at least the first set of) successive CASs within that period can be combined, and that every period begins with combinable CASs, starting at radio frames satisfying SFN mod 16=0.

Observation 2: A repeating period, comprising a CAS ON duration followed by a CAS muted duration, must be a multiple of 160 ms in duration

- This facilitates combining across (at least the first) four consecutive CASs, spaced 40 ms apart, within each such period, as supported in the current specifications.

A valid CAS ON/OFF pattern within each such repeating period would have the first few CASs ON (i.e., non-muted CAS) within the n×160 ms. Since each set of m×160 ms (m<n) units will comprise four consecutive CASs that can be combined, we can use this as a starting point for the CAS ON duration, for different values of m. However, additional CASs may also be needed, e.g., to facilitate efficient alignment with the coexistence parameters provided by other broadcast standards. This can be achieved by adding a further k×40 ms (i.e., k additional CASs), on top of the baseline m×160 ms, to the CAS ON duration.

An example where this is relevant is co-existence with DVB-T2, which provides Future Extension Frames (FEFs) of duration up to 250 ms, within which 5G Broadcast can be TDM-ed. Configuring m=1, k=2 ensures a CAS ON duration of 240 ms, which is closely aligned (minimizing resource wastage) with the DVB-T2 FEFs.

The values of values of n,m,k will, in general, be configured from a set of candidate values. This results in a “duty cycle” of m+k4n for 5G Broadcast, a fraction n-m-k4n for time-division multiplexing with other broadcasting technologies, such as DVB-T2. This is illustrated in Fig. 1 below.

Figure 1: Illustration of CAS muting in 5G Broadcast, and potential coexistence with DVB-T2.

Observation 3: A CAS “ON duration” comprising the first m×160+k×40 ms out of every (n×160) ms facilitates a 5G Broadcast duty cycle of m+k4/n and allows for TDM-ing with other broadcasting technologies with a duty cycle of n-m-k4/n.

Observation 4: Setting m=1,k=2 results in a CAS ON duration of 240 ms, which minimizes resource wastage when 5G Broadcast is TDM-ed within the Future Extension Frames (FEFs) of DVB-T2, which are up to 250 ms duration each.

We further observe that, for the other “always on” signals in 5G broadcast—the MCCH and the MSI—described above, there are several candidate values of “n” such that a periodicity of n×160 ms (i.e., 16n rfs) aligns with an existing periodicity of the MCCH and/or the MSI.

Specifically, the values of n in the set {2,4,8,16} can be supported with compatible MCCH and MSI periodicities that are specified up to Release 16, such that the MCCHs and MSIs are not transmitted in the CAS-muted regions. We also note that, there are currently 7 spare values in the configuration for MCCH periodicity, which can be utilized to further augment the values of n that may be supportable.

Observation 5: For a periodicity of n×160ms, defining the periodic CAS muting pattern, values of n in the set {2,4,8,16} are compatible with existing MCCH and MSI periodicities (up to Release 16), such that the MCCHs and MSIs are only transmitted in the CAS “ON durations”.

- Larger values of n can be supported by defining larger MCCH periodicities; there are currently seven spare values to achieve this.

With the above analysis, we have established that to support muting of the always-on signals for 5G Broadcast (to facilitate flexible TDM-ing with other broadcasting technologies such as DVB-T2), at the higher layers, we only need to define a semi-static muting configuration in SIB1-MBMS, where the configuration will state when the CAS is on, and when it is off. This is the only signalling update needed in TS 36.331.

With this in mind, we make the following proposal.

Proposal 1: The non-MBSFN subframes containing the cell acquisition signals (PSS/SSS/PBCH/SIB1) in an MBMS-dedicated cell shall only be transmitted within the first m×160+k×40 ms out of every n×160 ms.

- Introduce a new parameter n, to be configured in SIB1-MBMS, from the set {2,4,8,16}.

- Introduce a new parameter m, to be configured in SIB1-MBMS, from the set {1,2,3, … ,15}.

- Introduce a new parameter k, to be configured in SIB1-MBMS, from the set {0,1,2,3}

- In the absence of the above parameters m,n,k, all non-MBSFN subframes containing cell acquisition signals in an MBMS-dedicated cell shall be transmitted

# Conclusion

In this contribution we presented our views on muting the always on signals for 5G Broadcast, to facilitate TDM-ing with other broadcasting technologies, such as DVB-T2. Our observations and proposal are summarized below.

Observation 1: The signals that need to be muted, to create blanks in 5G Broadcast transmissions in an MBMS-dedicated cell (which may be used to time-division multiplex other broadcast technologies with 5G Broadcast) are:

- The Cell Acquisition Subframe (CAS)

- The MCCH

- The MSI

Observation 2: A repeating period, comprising a CAS ON duration followed by a CAS muted duration, must be a multiple of 160 ms in duration

- This facilitates combining across (at least the first) four consecutive CASs, spaced 40 ms apart, within each such period, as supported in the current specifications.

Observation 3: A CAS “ON duration” comprising the first m×160+k×40 ms out of every (n×160) ms facilitates a 5G Broadcast duty cycle of m+k4/n and allows for TDM-ing with other broadcasting technologies with a duty cycle of n-m-k4/n.

Observation 4: Setting m=1,k=2 results in a CAS ON duration of 240 ms, which minimizes resource wastage when 5G Broadcast is TDM-ed within the Future Extension Frames (FEFs) of DVB-T2, which are up to 250 ms duration each.

Observation 5: For a periodicity of n×160ms, defining the periodic CAS muting pattern, values of n in the set {2,4,8,16} are compatible with existing MCCH and MSI periodicities (up to Release 16), such that the MCCHs and MSIs are only transmitted in the CAS “ON durations”.

- Larger values of n can be supported by defining larger MCCH periodicities; there are currently seven spare values to achieve this.

Proposal 1: The non-MBSFN subframes containing the cell acquisition signals (PSS/SSS/PBCH/SIB1) in an MBMS-dedicated cell shall only be transmitted within the first m×160+k×40 ms out of every n×160 ms.

- Introduce a new parameter n, to be configured in SIB1-MBMS, from the set {2,4,8,16}.

- Introduce a new parameter m, to be configured in SIB1-MBMS, from the set {1,2,3, … ,15}.

- Introduce a new parameter k, to be configured in SIB1-MBMS, from the set {0,1,2,3}

- In the absence of the above parameters m,n,k, all non-MBSFN subframes containing cell acquisition signals in an MBMS-dedicated cell shall be transmitted

# 4. References

[1] ETSI TS 102 755, V1.1.1 Digital Video Broadcasting (DVB); Frame structure channel coding and modulation for a second generation digital terrestrial television broadcasting system (DVB-T2), 2023.