3GPP TSG RAN WG1 #120bis	                                   					       												                           R1-2503133

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.15.4

Source:	Moderator (AT&T)

Title:	Final summary of UE features for enhancements of network energy savings for NR

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion [120bis-R19-UE_features] during RAN1 #120bis. According to the Chair’s Notes:

[120bis-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO), Hiroki (DOCOMO) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

The following was discussed during RAN1 #120bis within the scope of [120bis-R19-UE_features]. All proposals are based on the latest RAN1 UE features list for Rel. 19 in [1].

# Summary of Contributions Submitted to RAN1 #120bis

The following is the moderator’s summary of contributions submitted to RAN1 #120bis in this agenda item.

## On-demand SSB SCell operation

| 61. Netw_Energy_NR_enh | 61-1 | On-demand SSB SCell operation indicated by RRC based signaling in Case #1 | 1. Support RRC based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell) | FFS | Yes |  | UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #1 | [Per band] | n/a | n/a | n/a | FFS: supported deactivation mechanisms | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| ZTE Corporation/Sanechips [2] | According to the agreement in RAN1#119 meeting [1], there were two mechanisms for on-demand SSB deactivation. Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer. If separate FGs for on-demand SSB indication and deactivation, it will be complicated to include the case that MAC CE activation plus N times for on demand SSB transmission configured in RRC signaling. In our understanding, it belongs to the case of simultaneous on-demand SSB indication and deactivation, so that a single FG for indication and deactivation is much clearer to cover the case MAC CE activation plus N times for on-demand SSB transmission configured in the RRC signaling. Support a single FG for on-demand SSB SCell indication/deactivation. In previous meeting [2], RRC signaling and MAC CE signaling for on-demand SSB SCell indication were agreed. Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable. But in our views, RRC signaling and MAC CE signaling should be supported in separate FGs, since it is unnecessary for UE to support both indication signalings for on-demand SSB SCell indication. Split MAC CE indication/deactivation and RRC indication/deactivation in separate FGs. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB SCell indication/deactivation on the cell in Case #1 (No always-on SSB on the cell); FFS; Yes; UE does not support on-demand SSB SCell indication/deactivation indicated by RRC based signaling in Case #1; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Vivo [3] | Regarding FFS: supported deactivation mechanisms, we support to have different deactivation mechanism for different cases. However, the details may need more RAN1 progress. Proposal 2: Keep FFS: supported deactivation mechanisms for all FGs and wait for more RAN1 progress to complete the details. Regarding [Per band], we support to keep per band for each FG. Proposal 4: Keep Per band for FG 64-1, 64-2, 64-3 and 64-4. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB indication/deactivation on the cell in Case #1 (No always-on SSB on the cell); Yes; UE does not support on-demand SSB indication/deactivation indicated by RRC based signaling in Case #1; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| FUTUREWEI [4] | In RAN1 #119, it was decided that a UE supporting on-demand SSB will support at least two options for deactivation of on-demand SSB. Thus, gNB may indicate via MCA-CE and RRC the on-demand SSB and via MAC-CE and RRC the deactivation of on-demand SSB. Thus, the text “[indication/deactivation]” may be approved. Proposal 3: For on-demand SSB FG, approve the text “[indication/deactivation]”. In the last meeting RAN1 #120 companies agreed on supporting the same signalling solutions for both scenarios Case #1 and Case #2. For instance, for supporting time domain location of on-demand SSB RRC, and for supporting a new higher layer parameter for configuration of multiple parameters. We also note that, from the implementation point of view of UE supporting Case #1 and respectively Case#2, there is very little difference. If there are distinct FGs for supporting Case #1and Case #2, not only does it lead to market fragmentation, but it may be inefficient from a deployment point of view, as it would limit access of some UE to certain SCell deployments. Indeed, UEs supporting FG supporting Case #2 should be able to be deployed in the scenarios Case #2, but not the other way around. For these reasons, we strongly prefer to have a single feature related to on-demand SSB. Proposal 4: Merge all FG 61-x related to on-demand SSB FG in a single FG 61-1. Adopt the proposed FG 61-1. With the previous comments and proposals, the FG61 table will become a single row. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation; - Support RRC based signalling to indicate on-demand SSB SCell (indication/deactivation]) on the cell in Case #1 (No always-on SSB on the cell) - 1. Support RRC based signalling to indicate on-demand SSB SCell (indication /deactivation) on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) - Support MAC CE based signalling to indicate on-demand SSB SCell (indication /deactivation) on the cell in Case #1 (No always-on SSB on the cell) - 1. Support MAC CE based signalling to indicate on-demand SSB SCell (indication/deactivation])on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell); FFS; Yes; UE does not support on-demand SSB SCell; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| CATT [5] | RAN1 has agreed to use MAC-CE based signaling to indicate or deactivate the transmission of on-demand SSB. In addition, RRC based signaling has also been supported to indicate on-demand SSB transmission. Deactivation by RRC is up to RAN2 [1]. The following agreement was achieved by RAN2 in RAN2#127bis meeting [2], which means RRC based signaling could deactivate on-demand SSB transmission. 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.. Intuitively, activation and deactivation by the same type of signaling should be supported by a UE. Further, RRC based signaling and MAC-CE based signaling should also both be supported by the UE as complementary indication mechanisms. - Support RRC and MAC-CE based signalling to indicate on-demand SSB SCell indication/deactivation on the cell in Case #1 (No always-on SSB on the cell) - For these feature groups, the type of granularity should be Per Band. - Merge FG 61-3 with FG 61-1. Proposal 1: For FG 61-1 and 61-3: - Merge FG 61-3 with FG 61-1. - The type of granularity should be Per Band. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC and MAC-CE based signaling in Case #1; 1. Support RRC and MAC-CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell); FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC and MAC-CE based signaling in Case #1; [Per band]. |
| China Telecom [6] | 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell); FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Ofinno [7] | For feature groups 61-1 – 61-4, we suggest to clarify whether the UE supports these features for intra-band or inter-band (e.g., either in the type definition and/or using a component). Supporting OD-SSB for intra-band may, for example, be simpler for many UEs while supporting inter-band may be more complex. Proposal 1: Clarify for FGs 61-1 to 61-4 whether these features are for intra-band, inter-band, or both. For feature groups 61-1 and 61-2, we suggest to a component for a maximum number of OD-SSB configurations/periodicity values, which can be indicated/configured by the RRC. Proposal 3: Add a component to FG 61-1 and FG 61-2 to indicate the maximum number of OD-SSB configurations/periodicity values by the RRC. 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell); 2. Support RRC based signalling to indicate the maximum number of OD-SSB configurations/periodicity values;; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Nokia [8] | 61-1; On-demand SSB SCell operation when no always-on SSB on the SCell; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell 2. Support MAC CE based signalling to indicate on-demand SSB SCell indication/deactivation on the cell; FFS: supported deactivation mechanisms. |
| CMCC [9] | Regarding the FFS part on Components and Consequence if the feature is not supported by the UE column, we support to keep the wording in indication/deactivation while remove the wording for frequency. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB SCell indication/deactivation on the cell in Case #1 (No always-on SSB on the cell); Yes; UE does not support on-demand SSB SCell indication/deactivation indicated by RRC based signaling in Case #1; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Huawei/HiSilicon [10] | Based on the agreement of #118 meeting, RRC based signalling to indicate on-demand SSB transmission on the cell is supported at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. However, only UE supporting directMCG-SCellActivation-r16 or directSCG-SCellActivation-r16 can support direct NR SCell activation upon SCell addition via RRC signalling. It is proposed to add directMCG-SCellActivation-r16 or directSCG-SCellActivation-r16 as the prerequisite FGs for the UE capability of RRC based signalling to indicate on-demand SSB transmission. Proposal 2: For on-demand SSB SCell operation, add directMCG-SCellActivation-r16 or directSCG-SCellActivation-r16 as the Prerequisite FGs for the UE capability of RRC based signalling to indicate on-demand SSB transmission. As for the reporting type, we think per band reporting is sufficient. Proposal 3: Set the reporting type for on-demand SSB SCell operation as per band. As for the FFS issue that whether to merge MAC CE based signalling with RRC based signalling, UE needs to follow the timeline to determine the SSB time domain positions of OD SSB on the cell for MAC CE based signalling, which is more stringent than RRC based signalling. Therefore, separate UE capabilities are preferred. Proposal 4: Do not merge the UE capability of MAC CE based signalling with RRC based signalling. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell); FFS 18-5 from RAN2 (Direct NR MCG SCell activation) or 18-6 from RAN2 (Direct NR SCG SCell activation); Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| OPPO [11] | For FG 61-1, 61-2, according to RAN2’s discussions, there are two different ways for RRC-based OD-SSB activation/deactivation. In the first way, the network configures OD-SSB parameters by RRC signaling and the configured OD-SSB is automatically activated and the OD-SSB can be deactivated by RRC reconfiguration. Here by deactivation means that the RRC reconfiguration does not configure OD-SSB parameters. In the second way, the network can configure OD-SSB parameters and sets an initial activation status (either activation or deactivation), then using another RRC signaling to control the activation/deactivation of OD-SSB, which serves a similar function as MAC-CE based activation/deactivation and the difference is to use RRC signaling to replace MAC-CE signaling. Therefore, regarding UE capability, it should be allowed the UE to only support either of these two ways to support RRC-based OD-SSB. Thus, we suggest that to split these two ways by FG61-1a to reflect the first way, and FG61-1b to reflect the second way. Similar to FG61-2. Observation: There are two different ways for RRC-based OD-SSB activation/deactivation. The UE feature for FG 61-1 and 61-2 need to capture separately different UE capabilities. Proposal: Adopt the following modification to separate different UE capabilities. 61. Netw_Energy_NR_enh; 61-1a; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell). RRC based signaling only indicates the initial status of on-demand SSB.; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #1.RRC based signaling only indicates the initial status of on-demand SSB.; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-1b; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell). RRC based signaling controls on-demand SSB activation/deactivation during non-initial phase; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #1.RRC based signaling controls on-demand SSB activation/deactivation during non-initial phase; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Samsung [12] | 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell) 2. Support L1 measurement based on on-demand SSB 3. Support L3 measurement based on on-demand SSB; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #1; [Per band] Per BC; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Xiaomi [13] | On-demand SSB is only applicable to SCell. There are two cases for on-demand SSB, i.e., case#1 and case#2. - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell Furthermore, the following agreement was achieved in RAN1#116bis meeting: Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.. Observation 1: For FG related to on-demand SSB, at least the following scenario and case should be captured as component: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 For on-demand SSB, the following agreement was achieved: Agreement New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.. Furthermore, there is no agreement touching physical structure for on-demand SSB. From our perspective, we don’t see motivation on changing content of on demand SSB burst if always on SSB is assumed. For on demand SSB, the following content as shown in proposal 2 should be capture in the FG. Observation 2: The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms}. - If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB. For case #2, always on SSB is assumed and only the additional SSB can be turned on or turned off. Regarding the always on SSB, we don’t see the necessity to put any restriction on it, i.e., it can be either CD-SSB or NCD-SSB. Regarding to the relationship between always-on SSB and additional SSB, the frequency location can be same or different. However, if AO-SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency from the frequency location of always-on SSB. [1] Observation 3: If always on SSB is CD-SSB on synchronization raster, the frequency location of always on SSB and additional SSB is different. - Otherwise, the frequency location of always on SSB and additional SSB can be different. Per RAN1 agreements, only gNB-triggered on-demand SSB is supported. Specifically, gNB can activate/indicate on-demand SSB transmission and deactivate on-demand SSB transmission. Either MAC CE or RRC signaling can be used as indication. RRC signaling is a universal solution across different scenarios. On the other hand, MAC CE mechanism can be applicable at least to scenario 2A and scenario 2. Observation 4: For OD-SSB capable UE, it supports both RRC-based indication and MAC CE based indication for OD-SSB activation/deactivation. - For MAC CE based indication, it is applicable to scenario 2A and scenario 2. FFS other scenarios. One major functionality of SSB is it can be used for measurement, i.e., L1 measurement and L3 measurement. RAN1 achieved the following agreements on measurement based on on-demand SSB. Agreement Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4 Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not. Despite of always-on SSB and on-demand SSB, neither physical structure nor functionality is changed. From this perspective, SSB type doesn’t impact measurement at UE side. As shown in the above agreements, it has been agreed that on-demand SSB can be used for L3 measurement. Considering that L1 measurement is a key step for L3 measurement, i.e., UE measures several SSB samples which can be regarded as L1 measurement and then filtering all L1 measurement results to get L3 measurement result, there is no barrier to use on-demand SSB for L1 measurement as well. In particular, the following agreement on spatial assumption between on-demand SSB and always on SSB was made in RAN1#119 meeting, which we believe further fortify aforementioned analyses. Agreement Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1: - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.. Observation 5: For OD-SSB capable UE, on-demand SSB is used for both L1 measurement and L3 measurement. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell) 2. RRC based signaling to indicate on-demand SSB SCell [indication/deactivation] on the cell is applicable to [scenario#2 and scenario#2A] 3. The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms}. If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB. 4. On-demand SSB is used for both L1 measurement and L3 measurement; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| LG Electronics [14] | Agreement (RAN1#120) Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB. As highlighted above, the case where the frequency location of on-demand SSB is different from the frequency location of always-on SSB is supported, subject to separate UE capability. Thus, in order to cover this case, it would be straight-forward to define new FG for Case #2 (Always-on SSB is periodically transmitted on the cell). Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement (RAN1#119) For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer. Regarding on-demand SSB activation/deactivation, the followings need to be considered. - The phrase “signaling to indicate on-demand SSB transmission” as highlighted in cyan above can be reused to describe FGs 61-1 to 61-4. - As highlighted yellow above, deactivation of on-demand SSB via RRC signaling is up to RAN2. Thus, deactivation in FG 61-1 or 61-2 can be deleted. - Although two options are agreed for on-demand SSB deactivation, separate FGs corresponding to each option don’t seem to be necessary in our view. - Considering RRC or MAC CE signaling for on-demand SSB transmission indication is UE-specific, introducing separate FGs depending on RRC or MAC CE based signalling mechanism for on-demand SSB transmission indication (i.e., keeping FG 61-1 and FG 61-3 separate) would be reasonable. 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB SCell transmission on the cell in Case #1 (No always-on SSB on the cell); UE does not support on-demand SSB SCell transmission indicated by RRC based signaling in Case #1. |
| Apple [15] |  |
| NTT DOCOMO, INC. [16] | On the FFS of supported deactivation mechanisms in each FG, we will have at least two deactivation schemes, based on explicit MAC CE signaling and implicit timer or counter. We are open to discuss whether to decouple those two schemes into two separate FGs; however, we believe we should decide a default scheme from the two. From our perspective, it should be straightforward to support MAC CE deactivation as a default, given that the activation is addressed by MAC CE only. Thus MAC CE based deactivation can be included in each FG, or a component in each FG can report either “support of MAC CE based deactivation” or “support of both MAC CE based deactivation and timer-based deactivation”. Regarding the type, per-band is our best preference, which would contribute to easier operation from NW operators’ perspective. Meanwhile, if finer granularity than per-band is really necessary, it should be per BC, not per-FS or per FSPC. It is because, among the band combos there should be a freedom for NW to choose PCell. If OD-SSB features are defined to be per-FS/FSPC granularity, it would result in that NW must choose particular band/CC to be PCell in order to achieve NES operation because of UE implementation. Also, per-FS/FSPC allows different UEs to support different condition to support OD-SSB operation, which eventually suppress the benefit of NES in real field. To make sure OD-SSB is beneficial in real field, we prefer to support per BC. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC and MAC CE based signaling in Case #1; - Support RRC based signalling to indicate on-demand SSB SCell activation [indication/deactivation] on the Scell in Case #1 (No always-on SSB on the cell) - Support MAC CE based signalling to indicate on-demand SSB SCell activation and deactivation [indication /deactivation] on the Scell in Case #1 (No always-on SSB on the cell) - Support implicit deactivation of on-demand SSB; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Ericsson [17] | - Components - Component 1: RRC signaling should support both start and stop of the on-demand SSBs on the SCell. Suggest replacing “[indication/deactivation]” with “indication/deactivation”. - No need for pre-requisite FGs for this FG. Basic CA operation, etc is assumed but it is not necessary to explicitly list the CA capabilities as prerequisite. - Should be per band. - Note column - RRC based deactivation mechanism is supported. If UE additionally support MAC-CE (one of FG 61-3, 61-4), UE also supports MAC CE based deactivation mechanism to deactivate the on-demand SSB. - Suggest updating as “FFS: supported deactivation mechanisms”. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB SCell indication/deactivation on the cell in Case #1 (No always-on SSB on the cell); Yes; UE does not support on-demand SSB SCell indication/deactivation indicated by RRC based signaling in Case #1; per band; n/a; n/a; n/a; Note 1: RRC based deactivation mechanism is supported. If UE additionally support MAC-CE (one of FG 61-3, 61-4), UE also supports MAC CE based deactivation mechanism to deactivate the on-demand SSB.; Optional with capability signaling. |
| Qualcomm Incorporated [18] | 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate on-demand SSB transmission SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell); FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |

| 61. Netw_Energy_NR_enh | 61-2 | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency] | 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency] | FFS | Yes |  | UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 [for same center frequency] | [Per band] | n/a | n/a | n/a | FFS: supported deactivation mechanisms | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| ZTE Corporation/Sanechips [2] | According to the agreement in RAN1#119 meeting [1], there were two mechanisms for on-demand SSB deactivation. Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer. If separate FGs for on-demand SSB indication and deactivation, it will be complicated to include the case that MAC CE activation plus N times for on demand SSB transmission configured in RRC signaling. In our understanding, it belongs to the case of simultaneous on-demand SSB indication and deactivation, so that a single FG for indication and deactivation is much clearer to cover the case MAC CE activation plus N times for on-demand SSB transmission configured in the RRC signaling. Support a single FG for on-demand SSB SCell indication/deactivation. In previous meeting [2], RRC signaling and MAC CE signaling for on-demand SSB SCell indication were agreed. Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable. But in our views, RRC signaling and MAC CE signaling should be supported in separate FGs, since it is unnecessary for UE to support both indication signalings for on-demand SSB SCell indication. Split MAC CE indication/deactivation and RRC indication/deactivation in separate FGs. 61. Netw_Energy_NR_enh; 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency]; 1. Support RRC based signalling to indicate on-demand SSB SCell indication /deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]; FFS; Yes; UE does not support on-demand SSB SCell indication/deactivation indicated by RRC based signaling in Case #2 [for same center frequency]; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Vivo [3] | Regarding [for same center frequency] in FG 61-2 and 61-4, it should be kept since Case#2 with different frequency is subject to a separate UE capability according to the following RAN1 agreement: Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB Proposal 1: Keep [for same center frequency] for FG 61-2 and 61-4 and introduce separate FG for Case #2 with different center frequency. Regarding FFS: supported deactivation mechanisms, we support to have different deactivation mechanism for different cases. However, the details may need more RAN1 progress. Proposal 2: Keep FFS: supported deactivation mechanisms for all FGs and wait for more RAN1 progress to complete the details. Regarding [Per band], we support to keep per band for each FG. Proposal 4: Keep Per band for FG 64-1, 64-2, 64-3 and 64-4. 61. Netw_Energy_NR_enh; 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for same center frequency; 1. Support RRC based signalling to indicate on-demand SSB indication /deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same [center] frequency; Yes; UE does not support on-demand SSB indication/deactivation indicated by RRC based signaling in Case #2 for same center frequency; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| FUTUREWEI [4] | In the last RAN1 (#120), it was agreed the support of Alt 1. • Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. While in RAN1, #119 it was established that: • The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. In the view of these agreements, the temporary text “[for same center frequency]” under FG column should be removed. Similarly, the text “[for same [center] frequency]” should be removed from the Components column. Proposal 1: In the FG 61-2 and FG 61-4 rows remove temporary text marked by yellow highlight related to “[for same center frequency]” In RAN1 #119, it was decided that a UE supporting on-demand SSB will support at least two options for deactivation of on-demand SSB. Thus, gNB may indicate via MCA-CE and RRC the on-demand SSB and via MAC-CE and RRC the deactivation of on-demand SSB. Thus, the text “[indication/deactivation]” may be approved. Proposal 3: For on-demand SSB FG, approve the text “[indication/deactivation]”. In the last meeting RAN1 #120 companies agreed on supporting the same signalling solutions for both scenarios Case #1 and Case #2. For instance, for supporting time domain location of on-demand SSB RRC, and for supporting a new higher layer parameter for configuration of multiple parameters. We also note that, from the implementation point of view of UE supporting Case #1 and respectively Case#2, there is very little difference. If there are distinct FGs for supporting Case #1and Case #2, not only does it lead to market fragmentation, but it may be inefficient from a deployment point of view, as it would limit access of some UE to certain SCell deployments. Indeed, UEs supporting FG supporting Case #2 should be able to be deployed in the scenarios Case #2, but not the other way around. For these reasons, we strongly prefer to have a single feature related to on-demand SSB. Proposal 4: Merge all FG 61-x related to on-demand SSB FG in a single FG 61-1. Adopt the proposed FG 61-1. With the previous comments and proposals, the FG61 table will become a single row. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation; - Support RRC based signalling to indicate on-demand SSB SCell (indication/deactivation]) on the cell in Case #1 (No always-on SSB on the cell) - 1. Support RRC based signalling to indicate on-demand SSB SCell (indication /deactivation) on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) - Support MAC CE based signalling to indicate on-demand SSB SCell (indication /deactivation) on the cell in Case #1 (No always-on SSB on the cell) - 1. Support MAC CE based signalling to indicate on-demand SSB SCell (indication/deactivation])on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell); FFS; Yes; UE does not support on-demand SSB SCell; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| CATT [5] | RAN1 has agreed to use MAC-CE based signaling to indicate or deactivate the transmission of on-demand SSB. In addition, RRC based signaling has also been supported to indicate on-demand SSB transmission. Deactivation by RRC is up to RAN2 [1]. The following agreement was achieved by RAN2 in RAN2#127bis meeting [2], which means RRC based signaling could deactivate on-demand SSB transmission. 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.. Intuitively, activation and deactivation by the same type of signaling should be supported by a UE. Further, RRC based signaling and MAC-CE based signaling should also both be supported by the UE as complementary indication mechanisms. The following agreement was made in RAN1#120 related to start-indicator part of R-TAS [2]: Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB. According to above agreement, for Case 2, regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, subject to separate UE capability, if always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. Hence, whether or not the UE can measure the OD-SSB at a center frequency different from that of the always-on SSB should be introduced as separate UE capability. Observation 1: Whether or not the UE can measure the OD-SSB at a center frequency different from that of the always-on SSB should be introduced as separate UE capability. New separate UE feature group whose index is 61-2-2 are introduced for the UE capability of whether or not the UE can measure the OD-SSB at a center frequency different from that of the always-on SSB. The original index of UE feature group 61-2 is changed to 61-2-1 for the UE capability of whether or not the UE can measure the OD-SSB at a center frequency same as that of the always-on SSB. FG 61-4 is merged. For the sake of clear description, the text "[for same center frequency]" in the name of original feature group 61-2 has been changed to "for the on-demand SSB at a center frequency same as that of the always-on SSB". For these feature groups, the type of granularity should be Per Band. Proposal 2: For FG 61-2 and 61-4: - Merge FG 61-4 with FG 61-2. - The original index of UE feature group 61-2 is changed to 61-2-1 for the UE capability of whether or not the UE can measure the OD-SSB at a center frequency same as that of the always-on SSB. - For the sake of clear description, the text "[for same center frequency]" in the name of original feature group 61-2 has been changed to "for the on-demand SSB at a center frequency same as that of the always-on SSB". - For these feature groups, the type of granularity should be Per Band. Proposal 3: A new separate UE feature group whose index is 61-2-2 should be introduced for the UE capability of whether or not the UE can measure the OD-SSB at a center frequency different from that of the always-on SSB. 61. Netw_Energy_NR_enh; 61-2-1; On-demand SSB SCell operation indicated by RRC and MAC-CE based signaling in Case #2 for the on-demand SSB at a center frequency same as that of the always-on SSB. [for same center frequency]; 1. Support RRC and MAC-CE based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for the on-demand SSB at a center frequency same as that of the always-on SSB.[for same [center] frequency]; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC and MAC-CE based signaling in Case #2 for the on-demand SSB at a center frequency same as that of the always-on SSB.[for same center frequency]; [Per band]. 61. Netw_Energy_NR_enh; 61-2-2; On-demand SSB SCell operation indicated by RRC and MAC-CE based signaling in Case #2 for the on-demand SSB at a center frequency different from that of the always-on SSB.; 1. Support RRC and MAC-CE based signalling to indicate on-demand SSB SCell indication /deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for the on-demand SSB at a center frequency different from that of the always-on SSB.; FFS; Yes; UE does not support on-demand SSB SCell indication/deactivation indicated by RRC and MAC-CE based signaling in Case #2 for the on-demand SSB at a center frequency different from that of the always-on SSB.; Per band. |
| China Telecom [6] | 61. Netw_Energy_NR_enh; 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency]; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]; FFS; Yes; UE does not support on-demand SSB SCell [indication/ deactivation]] indicated by RRC based signaling in Case #2 [for same center frequency]; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Ofinno [7] | For feature groups 61-1 – 61-4, we suggest to clarify whether the UE supports these features for intra-band or inter-band (e.g., either in the type definition and/or using a component). Supporting OD-SSB for intra-band may, for example, be simpler for many UEs while supporting inter-band may be more complex. Proposal 1: Clarify for FGs 61-1 to 61-4 whether these features are for intra-band, inter-band, or both. For feature groups 61-1 and 61-2, we suggest to a component for a maximum number of OD-SSB configurations/periodicity values, which can be indicated/configured by the RRC. Proposal 3: Add a component to FG 61-1 and FG 61-2 to indicate the maximum number of OD-SSB configurations/periodicity values by the RRC. 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency]; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]; 2. Support RRC based signalling to indicate the maximum number of OD-SSB configurations/periodicity values;; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 [for same center frequency]; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Nokia [8] | 61-2; On-demand SSB SCell operation when always-on SSB is periodically transmitted on the SCell [for same center frequency]; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell[for same [center] frequency] 2. Support MAC CE based signalling to indicate on-demand SSB SCell indication/deactivation on the cell [for same [center] frequency]; FFS: supported deactivation mechanisms. |
| CMCC [9] | - Regarding the FFS part on Feature group column, since it is agreed in last RAN1 meeting that the frequency for on-demand SSB and always-on SSB can be same or different, the FFS part should be removed. - Regarding the FFS part on Components and Consequence if the feature is not supported by the UE column, we support to keep the wording in indication/deactivation while remove the wording for frequency. - Regarding the FFS part on Type column, we support the current wording. 61. Netw_Energy_NR_enh; 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2; 1. Support RRC based signalling to indicate on-demand SSB SCell indication /deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell); Yes; UE does not support on-demand SSB SCell indication/deactivation indicated by RRC based signaling in Case #2; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Huawei/HiSilicon [10] | Based on the agreement of #120 meeting, if always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB, and this is subject to a UE capability which is different from the UE capability on the support of the same frequency location for on-demand SSB and always-on SSB. Therefore, for case #2, two separate UE capabilities can be introduced for RRC based signalling and MAC CE based signalling respectively. Proposal 1: For case #2, two separate UE capabilities can be introduced for RRC based signalling and MAC CE based signalling individually to support the case where the frequency location of on-demand SSB is different from the frequency location of always-on SSB if always-on SSB is CD-SSB on a synchronization raster. Based on the agreement of #118 meeting, RRC based signalling to indicate on-demand SSB transmission on the cell is supported at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. However, only UE supporting directMCG-SCellActivation-r16 or directSCG-SCellActivation-r16 can support direct NR SCell activation upon SCell addition via RRC signalling. It is proposed to add directMCG-SCellActivation-r16 or directSCG-SCellActivation-r16 as the prerequisite FGs for the UE capability of RRC based signalling to indicate on-demand SSB transmission. Proposal 2: For on-demand SSB SCell operation, add directMCG-SCellActivation-r16 or directSCG-SCellActivation-r16 as the Prerequisite FGs for the UE capability of RRC based signalling to indicate on-demand SSB transmission. As for the reporting type, we think per band reporting is sufficient. Proposal 3: Set the reporting type for on-demand SSB SCell operation as per band. As for the FFS issue that whether to merge MAC CE based signalling with RRC based signalling, UE needs to follow the timeline to determine the SSB time domain positions of OD SSB on the cell for MAC CE based signalling, which is more stringent than RRC based signalling. Therefore, separate UE capabilities are preferred. Proposal 4: Do not merge the UE capability of MAC CE based signalling with RRC based signalling. 61. Netw_Energy_NR_enh; 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency]; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]; FFS 18-5 from RAN2 (Direct NR MCG SCell activation) or 18-6 from RAN2 (Direct NR SCG SCell activation); Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 [for same center frequency]; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-2a; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequency; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency; FFS 18-5 from RAN2 (Direct NR MCG SCell activation) or 18-6 from RAN2 (Direct NR SCG SCell activation); Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 for different center frequency; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| OPPO [11] | For FG 61-1, 61-2, according to RAN2’s discussions, there are two different ways for RRC-based OD-SSB activation/deactivation. In the first way, the network configures OD-SSB parameters by RRC signaling and the configured OD-SSB is automatically activated and the OD-SSB can be deactivated by RRC reconfiguration. Here by deactivation means that the RRC reconfiguration does not configure OD-SSB parameters. In the second way, the network can configure OD-SSB parameters and sets an initial activation status (either activation or deactivation), then using another RRC signaling to control the activation/deactivation of OD-SSB, which serves a similar function as MAC-CE based activation/deactivation and the difference is to use RRC signaling to replace MAC-CE signaling. Therefore, regarding UE capability, it should be allowed the UE to only support either of these two ways to support RRC-based OD-SSB. Thus, we suggest that to split these two ways by FG61-1a to reflect the first way, and FG61-1b to reflect the second way. Similar to FG61-2. Observation: There are two different ways for RRC-based OD-SSB activation/deactivation. The UE feature for FG 61-1 and 61-2 need to capture separately different UE capabilities. Proposal: Adopt the following modification to separate different UE capabilities. 61. Netw_Energy_NR_enh; 61-2a; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency]; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]. RRC based signaling only indicates the initial status of on-demand SSB.; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 [for same center frequency].RRC based signaling only indicates the initial status of on-demand SSB.; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-2b; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency]; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]. RRC based signaling controls on-demand SSB activation/deactivation during non-initial phase; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 [for same center frequency].RRC based signaling controls on-demand SSB activation/deactivation during non-initial phase; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Samsung [12] | 61. Netw_Energy_NR_enh; 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency]; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency] 2. Support L1 measurement based on on-demand SSB 3. Support L3 measurement based on on-demand SSB; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 [for same center frequency]; [Per band] Per BC; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-2a; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequency; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency 2. Support L1 measurement based on on-demand SSB 3. Support L3 measurement based on on-demand SSB; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 for different center frequency; Per BC; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Xiaomi [13] | On-demand SSB is only applicable to SCell. There are two cases for on-demand SSB, i.e., case#1 and case#2. - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell Furthermore, the following agreement was achieved in RAN1#116bis meeting: Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.. Observation 1: For FG related to on-demand SSB, at least the following scenario and case should be captured as component: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 For on-demand SSB, the following agreement was achieved: Agreement New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.. Furthermore, there is no agreement touching physical structure for on-demand SSB. From our perspective, we don’t see motivation on changing content of on demand SSB burst if always on SSB is assumed. For on demand SSB, the following content as shown in proposal 2 should be capture in the FG. Observation 2: The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms}. - If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB. For case #2, always on SSB is assumed and only the additional SSB can be turned on or turned off. Regarding the always on SSB, we don’t see the necessity to put any restriction on it, i.e., it can be either CD-SSB or NCD-SSB. Regarding to the relationship between always-on SSB and additional SSB, the frequency location can be same or different. However, if AO-SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency from the frequency location of always-on SSB. [1] Observation 3: If always on SSB is CD-SSB on synchronization raster, the frequency location of always on SSB and additional SSB is different. - Otherwise, the frequency location of always on SSB and additional SSB can be different. Per RAN1 agreements, only gNB-triggered on-demand SSB is supported. Specifically, gNB can activate/indicate on-demand SSB transmission and deactivate on-demand SSB transmission. Either MAC CE or RRC signaling can be used as indication. RRC signaling is a universal solution across different scenarios. On the other hand, MAC CE mechanism can be applicable at least to scenario 2A and scenario 2. Observation 4: For OD-SSB capable UE, it supports both RRC-based indication and MAC CE based indication for OD-SSB activation/deactivation. - For MAC CE based indication, it is applicable to scenario 2A and scenario 2. FFS other scenarios. One major functionality of SSB is it can be used for measurement, i.e., L1 measurement and L3 measurement. RAN1 achieved the following agreements on measurement based on on-demand SSB. Agreement Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4 Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not. Despite of always-on SSB and on-demand SSB, neither physical structure nor functionality is changed. From this perspective, SSB type doesn’t impact measurement at UE side. As shown in the above agreements, it has been agreed that on-demand SSB can be used for L3 measurement. Considering that L1 measurement is a key step for L3 measurement, i.e., UE measures several SSB samples which can be regarded as L1 measurement and then filtering all L1 measurement results to get L3 measurement result, there is no barrier to use on-demand SSB for L1 measurement as well. In particular, the following agreement on spatial assumption between on-demand SSB and always on SSB was made in RAN1#119 meeting, which we believe further fortify aforementioned analyses. Agreement Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1: - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.. Observation 5: For OD-SSB capable UE, on-demand SSB is used for both L1 measurement and L3 measurement. 61. Netw_Energy_NR_enh; 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency]; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency] 2. RRC based signaling to indicate on-demand SSB SCell [indication/deactivation] on the cell is applicable to [scenario#2 and scenario#2A] 3. The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms}. If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB. 4. If always on SSB is CD-SSB on synchronization raster, the frequency location of always on SSB and additional SSB is different. Otherwise, the frequency location of always on SSB and additional SSB can be different. 5. On-demand SSB is used for both L1 measurement and L3 measurement; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 [for same center frequency]; [Per band]; n/a; n/a; n/a; - FFS: supported deactivation mechanisms; Optional with capability signaling. |
| LG Electronics [14] | Agreement (RAN1#120) Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB. As highlighted above, the case where the frequency location of on-demand SSB is different from the frequency location of always-on SSB is supported, subject to separate UE capability. Thus, in order to cover this case, it would be straight-forward to define new FG for Case #2 (Always-on SSB is periodically transmitted on the cell). Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement (RAN1#119) For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer. Regarding on-demand SSB activation/deactivation, the followings need to be considered. - As highlighted yellow above, deactivation of on-demand SSB via RRC signaling is up to RAN2. Thus, deactivation in FG 61-1 or 61-2 can be deleted. - Although two options are agreed for on-demand SSB deactivation, separate FGs corresponding to each option don’t seem to be necessary in our view. 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for same center frequency; 1. Support RRC based signalling to indicate on-demand SSB SCell transmission on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency between on-demand SSB and always-on SSB; UE does not support on-demand SSB SCell transmission indicated by RRC based signaling in Case #2 for same center frequency. 61-2a; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequency; 1. Support RRC based signalling to indicate on-demand SSB SCell transmission on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency between on-demand SSB and always-on SSB; UE does not support on-demand SSB SCell transmission indicated by RRC based signaling in Case #2 for different center frequency. |
| Apple [15] | 61. Netw_Energy_NR_enh; 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for same center frequency between AO-SSB and OD-SSB; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency between AO-SSB and OD-SSB; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 for same center frequency between AO-SSB and OD-SSB; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-2a; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequencies between AO-SSB and OD-SSB within the same BWP; 1. Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequencies between AO-SSB and OD-SSB within the same BWP; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 for different center frequencies between AO-SSB and OD-SSB within the same BWP; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| NTT DOCOMO, INC. [16] | On the FFS of supported deactivation mechanisms in each FG, we will have at least two deactivation schemes, based on explicit MAC CE signaling and implicit timer or counter. We are open to discuss whether to decouple those two schemes into two separate FGs; however, we believe we should decide a default scheme from the two. From our perspective, it should be straightforward to support MAC CE deactivation as a default, given that the activation is addressed by MAC CE only. Thus MAC CE based deactivation can be included in each FG, or a component in each FG can report either “support of MAC CE based deactivation” or “support of both MAC CE based deactivation and timer-based deactivation”. On whether same or different frequency b/w AO and OD-SSB in case#2, as RAN1 agreed below, separate capabilities should be taken. It seems a reasonable way to indicate what frequency relation is supported by the UE, e.g., {same only, different only, both}. The final decision on how/what element should be defined in the list would depend on detailed UE behaviors (e.g., which behavior is different for UE to implement). Agreement@RAN1#120 Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability Note: UE is not required to measure both AO-SSB and OD-SS. Regarding the type, per-band is our best preference, which would contribute to easier operation from NW operators’ perspective. Meanwhile, if finer granularity than per-band is really necessary, it should be per BC, not per-FS or per FSPC. It is because, among the band combos there should be a freedom for NW to choose PCell. If OD-SSB features are defined to be per-FS/FSPC granularity, it would result in that NW must choose particular band/CC to be PCell in order to achieve NES operation because of UE implementation. Also, per-FS/FSPC allows different UEs to support different condition to support OD-SSB operation, which eventually suppress the benefit of NES in real field. To make sure OD-SSB is beneficial in real field, we prefer to support per BC. 61. Netw_Energy_NR_enh; 61-2; On-demand SSB SCell operation indicated by RRC and MAC CE based signaling in Case #2 [for same center frequency]; - Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency] - Support MAC CE based signalling to indicate on-demand SSB SCell activation and deactivation [indication /deactivation] on the Scell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency] - Support implicit deactivation of on-demand SSB; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated by RRC based signaling in Case #2 [for same center frequency]; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms Component1 and component2 indicates: {same only, different only, both}; Optional with capability signaling. |
| Ericsson [17] | - FG name: - Same or different center frequency can become candidate values that UE reports. - Suggest updating FG name as “On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency]“. - Components - Component 1: RRC signaling should support both start and stop of the on-demand SSBs on the SCell. Suggesting updating as “Support RRC based signalling to indicate on-demand SSB SCell [indication /deactivation] indication/deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]” - (New) Component 2: Add “Supported relation between center frequency locations of on-demand SSB and always-on SSB”. Candidate value set {‘same-only’, ‘same or different’}. - No need for pre-requisite FGs for this. Basic CA operation, etc is assumed but it is not necessary to explicitly list the CA capabilities as prerequisite. - Should be per band. - Note column - Same comment as for 61-1. 61. Netw_Energy_NR_enh; 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2; 1. Support RRC based signalling to indicate on-demand SSB SCell indication/deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) 2. Supported relation between center frequency locations of on-demand SSB and always-on SSB; Yes; UE does not support on-demand SSB SCell indication/deactivation indicated by RRC based signaling in Case #2; per band; n/a; n/a; n/a; Note 1: RRC based deactivation mechanism is supported. If UE additionally support MAC-CE (one of FG 61-3, 61-4), UE also supports MAC CE based deactivation mechanism to deactivate the on-demand SSB. Component 2, Candidate value set {‘same-only’, ‘same or different’}.; Optional with capability signaling. |
| Qualcomm Incorporated [18] | RAN1#120 made the following agreement for time-domain relationship between on-demand SSB and always-on SSB. Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted Send an LS to RAN4 to inform them of the above agreement. Final LS in R1-2501633.. From our perspective, FGs 61-2 and 61-4 should be applicable to the case when the center frequency locations of always-on SSB and on-demand SSB are same and Alt Time-C1 in the agreement (The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification)). In addition, we propose to define new FG 61-2a and new FG 61-4a for indicating the support of Case 2 with time relations other than Alt Time-C1 (e.g., Alt Time-C2 or the case when the center frequency locations of always-on SSB and on-demand SSB are different). 61. Netw_Energy_NR_enh; 61-2; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency] and periodic time domain SSB pattern during OD-SSB transmission; 1. Support RRC based signalling to indicate on-demand SSB transmission SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) with [for same center frequency] for on-demand SSB and always-on SSB; FFS; Yes; UE does not support on-demand SSB transmission SCell [indication/deactivation] indicated by RRC based signaling in Case #2 with [for same center frequency] for on-demand SSB and always-on SSB; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms Note: during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern (the interval between SSB bursts is even and supported in legacy specification).; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-2a; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for same center frequency and non-periodic time domain SSB pattern during OD-SSB transmission; 1. Support RRC based signalling to indicate on-demand SSB transmission on the cell in Case #2 in which during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern.; FFS; Yes; UE does not support on-demand SSB transmission indicated by RRC based signaling in Case #2 in which during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern.; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-2b; On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequencies; 1. Support RRC based signalling to indicate on-demand SSB transmission on the cell in Case #2 in which AO-SSB and OD-SSB have different center frequency.; FFS; Yes; UE does not support on-demand SSB transmission indicated by RRC based signaling in Case #2 in which AO-SSB and OD-SSB have different center frequencies; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |

| 61. Netw_Energy_NR_enh | 61-3 | On-demand SSB SCell operation indicated via MAC CE in Case #1 | 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #1 (No always-on SSB on the cell) | FFS | Yes |  | UE does not support on-demand SSB SCell [indication/deactivation ]indicated via MAC CE in Case #1 | [Per band] | n/a | n/a | n/a | FFS: supported deactivation mechanisms FFS: whether to merge with 61-1 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| ZTE Corporation/Sanechips [2] | According to the agreement in RAN1#119 meeting [1], there were two mechanisms for on-demand SSB deactivation. Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer. If separate FGs for on-demand SSB indication and deactivation, it will be complicated to include the case that MAC CE activation plus N times for on demand SSB transmission configured in RRC signaling. In our understanding, it belongs to the case of simultaneous on-demand SSB indication and deactivation, so that a single FG for indication and deactivation is much clearer to cover the case MAC CE activation plus N times for on-demand SSB transmission configured in the RRC signaling. Support a single FG for on-demand SSB SCell indication/deactivation. In previous meeting [2], RRC signaling and MAC CE signaling for on-demand SSB SCell indication were agreed. Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable. But in our views, RRC signaling and MAC CE signaling should be supported in separate FGs, since it is unnecessary for UE to support both indication signalings for on-demand SSB SCell indication. Split MAC CE indication/deactivation and RRC indication/deactivation in separate FGs. 61. Netw_Energy_NR_enh; 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB SCell indication /deactivation on the cell in Case #1 (No always-on SSB on the cell); FFS; Yes; UE does not support on-demand SSB SCell indication/deactivation indicated via MAC CE in Case #1; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-1; Optional with capability signaling. |
| Vivo [3] | Regarding FFS: supported deactivation mechanisms, we support to have different deactivation mechanism for different cases. However, the details may need more RAN1 progress. Proposal 2: Keep FFS: supported deactivation mechanisms for all FGs and wait for more RAN1 progress to complete the details. Regarding FFS: whether to merge with 61-1 and FFS: whether to merge with 61-2 in FG 61-3 and FG 61-4 respectively, we don’t support to merge them since there is no need for a UE to support two different mechanisms for the same purpose. Proposal 3: Remove FFS: whether to merge with 61-1 and FFS: whether to merge with 61-2 in FG 61-3 and FG 61-4 respectively. Regarding [Per band], we support to keep per band for each FG. Proposal 4: Keep Per band for FG 64-1, 64-2, 64-3 and 64-4. 61. Netw_Energy_NR_enh; 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB indication /deactivation on the cell in Case #1 (No always-on SSB on the cell); Yes; UE does not support on-demand SSB indication/deactivation indicated via MAC CE in Case #1; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| FUTUREWEI [4] | As mentioned above, our preference is to limit as much the distinctive FG related to the same feature, to avoid UE market fragmentation. To this purpose at least the MAC-CE and RRC should be supported by the same FG. Proposal 2: Merge FG 61-3 to FG 61-1, and FG 61-4 to FG 61-2. In RAN1 #119, it was decided that a UE supporting on-demand SSB will support at least two options for deactivation of on-demand SSB. Thus, gNB may indicate via MCA-CE and RRC the on-demand SSB and via MAC-CE and RRC the deactivation of on-demand SSB. Thus, the text “[indication/deactivation]” may be approved. Proposal 3: For on-demand SSB FG, approve the text “[indication/deactivation]”. In the last meeting RAN1 #120 companies agreed on supporting the same signalling solutions for both scenarios Case #1 and Case #2. For instance, for supporting time domain location of on-demand SSB RRC, and for supporting a new higher layer parameter for configuration of multiple parameters. We also note that, from the implementation point of view of UE supporting Case #1 and respectively Case#2, there is very little difference. If there are distinct FGs for supporting Case #1and Case #2, not only does it lead to market fragmentation, but it may be inefficient from a deployment point of view, as it would limit access of some UE to certain SCell deployments. Indeed, UEs supporting FG supporting Case #2 should be able to be deployed in the scenarios Case #2, but not the other way around. For these reasons, we strongly prefer to have a single feature related to on-demand SSB. Proposal 4: Merge all FG 61-x related to on-demand SSB FG in a single FG 61-1. Adopt the proposed FG 61-1. With the previous comments and proposals, the FG61 table will become a single row. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation; - Support RRC based signalling to indicate on-demand SSB SCell (indication/deactivation]) on the cell in Case #1 (No always-on SSB on the cell) - 1. Support RRC based signalling to indicate on-demand SSB SCell (indication /deactivation) on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) - Support MAC CE based signalling to indicate on-demand SSB SCell (indication /deactivation) on the cell in Case #1 (No always-on SSB on the cell) - 1. Support MAC CE based signalling to indicate on-demand SSB SCell (indication/deactivation])on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell); FFS; Yes; UE does not support on-demand SSB SCell; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| CATT [5] | RAN1 has agreed to use MAC-CE based signaling to indicate or deactivate the transmission of on-demand SSB. In addition, RRC based signaling has also been supported to indicate on-demand SSB transmission. Deactivation by RRC is up to RAN2 [1]. The following agreement was achieved by RAN2 in RAN2#127bis meeting [2], which means RRC based signaling could deactivate on-demand SSB transmission. 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.. Intuitively, activation and deactivation by the same type of signaling should be supported by a UE. Further, RRC based signaling and MAC-CE based signaling should also both be supported by the UE as complementary indication mechanisms. - Support RRC and MAC-CE based signalling to indicate on-demand SSB SCell indication/deactivation on the cell in Case #1 (No always-on SSB on the cell) - For these feature groups, the type of granularity should be Per Band. - Merge FG 61-3 with FG 61-1. Proposal 1: For FG 61-1 and 61-3: - Merge FG 61-3 with FG 61-1. - The type of granularity should be Per Band. |
| China Telecom [6] | 61. Netw_Energy_NR_enh; 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #1 (No always-on SSB on the cell); FFS; Yes; UE does not support on-demand SSB SCell [indication/ deactivation]]indicated via MAC CE in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-1; Optional with capability signaling. |
| Ofinno [7] | For feature groups 61-1 – 61-4, we suggest to clarify whether the UE supports these features for intra-band or inter-band (e.g., either in the type definition and/or using a component). Supporting OD-SSB for intra-band may, for example, be simpler for many UEs while supporting inter-band may be more complex. Proposal 1: Clarify for FGs 61-1 to 61-4 whether these features are for intra-band, inter-band, or both. For feature groups 61-3 and FG 61-4 (MAC CE activation), we suggest to add a component for the maximum number of SCells that the UE can support the FG for. Proposal 2: Add a component to FG 61-3 and FG 61-4 for maximum configured number of cells with OD-SSB. 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #1 (No always-on SSB on the cell); FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation ]indicated via MAC CE in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-1; Optional with capability signaling. |
| Nokia [8] |  |
| CMCC [9] | - Regarding the FFS part on Components and Consequence if the feature is not supported by the UE column, we support to keep the wording in indication/deactivation while remove the wording for frequency. - Regarding the FFS part on Prerequisite feature groups, our view is that RRC-based indication shall be a basic capability while MAC CE-based indication is an additional capability. Therefore, we prefer that FG 61-3 and 61-4 should merged with FG 61-1 and 61-2, respectively. - Regarding the FFS part on Type column, we support the current wording. 61. Netw_Energy_NR_enh; 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB SCell indication /deactivation on the cell in Case #1 (No always-on SSB on the cell); 61-1; Yes; UE does not support on-demand SSB SCell indication/deactivation indicated via MAC CE in Case #1; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Huawei/HiSilicon [10] | As for the reporting type, we think per band reporting is sufficient. Proposal 3: Set the reporting type for on-demand SSB SCell operation as per band. As for the FFS issue that whether to merge MAC CE based signalling with RRC based signalling, UE needs to follow the timeline to determine the SSB time domain positions of OD SSB on the cell for MAC CE based signalling, which is more stringent than RRC based signalling. Therefore, separate UE capabilities are preferred. Proposal 4: Do not merge the UE capability of MAC CE based signalling with RRC based signalling. 61. Netw_Energy_NR_enh; 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #1 (No always-on SSB on the cell); FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation ]indicated via MAC CE in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-1; Optional with capability signaling. |
| OPPO [11] |  |
| Samsung [12] | 61. Netw_Energy_NR_enh; 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #1 (No always-on SSB on the cell) 2. Support L1 measurement based on on-demand SSB 3. Support L3 measurement based on on-demand SSB; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation ]indicated via MAC CE in Case #1; [Per band] Per BC; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-1; Optional with capability signaling. |
| Xiaomi [13] | On-demand SSB is only applicable to SCell. There are two cases for on-demand SSB, i.e., case#1 and case#2. - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell Furthermore, the following agreement was achieved in RAN1#116bis meeting: Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.. Observation 1: For FG related to on-demand SSB, at least the following scenario and case should be captured as component: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 For on-demand SSB, the following agreement was achieved: Agreement New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.. Furthermore, there is no agreement touching physical structure for on-demand SSB. From our perspective, we don’t see motivation on changing content of on demand SSB burst if always on SSB is assumed. For on demand SSB, the following content as shown in proposal 2 should be capture in the FG. Observation 2: The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms}. - If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB. For case #2, always on SSB is assumed and only the additional SSB can be turned on or turned off. Regarding the always on SSB, we don’t see the necessity to put any restriction on it, i.e., it can be either CD-SSB or NCD-SSB. Regarding to the relationship between always-on SSB and additional SSB, the frequency location can be same or different. However, if AO-SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency from the frequency location of always-on SSB. [1] Observation 3: If always on SSB is CD-SSB on synchronization raster, the frequency location of always on SSB and additional SSB is different. - Otherwise, the frequency location of always on SSB and additional SSB can be different. Per RAN1 agreements, only gNB-triggered on-demand SSB is supported. Specifically, gNB can activate/indicate on-demand SSB transmission and deactivate on-demand SSB transmission. Either MAC CE or RRC signaling can be used as indication. RRC signaling is a universal solution across different scenarios. On the other hand, MAC CE mechanism can be applicable at least to scenario 2A and scenario 2. Observation 4: For OD-SSB capable UE, it supports both RRC-based indication and MAC CE based indication for OD-SSB activation/deactivation. - For MAC CE based indication, it is applicable to scenario 2A and scenario 2. FFS other scenarios. One major functionality of SSB is it can be used for measurement, i.e., L1 measurement and L3 measurement. RAN1 achieved the following agreements on measurement based on on-demand SSB. Agreement Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4 Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not. Despite of always-on SSB and on-demand SSB, neither physical structure nor functionality is changed. From this perspective, SSB type doesn’t impact measurement at UE side. As shown in the above agreements, it has been agreed that on-demand SSB can be used for L3 measurement. Considering that L1 measurement is a key step for L3 measurement, i.e., UE measures several SSB samples which can be regarded as L1 measurement and then filtering all L1 measurement results to get L3 measurement result, there is no barrier to use on-demand SSB for L1 measurement as well. In particular, the following agreement on spatial assumption between on-demand SSB and always on SSB was made in RAN1#119 meeting, which we believe further fortify aforementioned analyses. Agreement Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1: - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.. Observation 5: For OD-SSB capable UE, on-demand SSB is used for both L1 measurement and L3 measurement. 61. Netw_Energy_NR_enh; 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #1 (No always-on SSB on the cell) 2. MAC CE based signaling to indicate on-demand SSB SCell [indication/deactivation] on the cell is applicable to [scenario#2A] 3. The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms}. If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB. 4. On-demand SSB is used for both L1 measurement and L3 measurement; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation ]indicated via MAC CE in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-1; Optional with capability signaling. |
| LG Electronics [14] | Agreement (RAN1#120) Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB. As highlighted above, the case where the frequency location of on-demand SSB is different from the frequency location of always-on SSB is supported, subject to separate UE capability. Thus, in order to cover this case, it would be straight-forward to define new FG for Case #2 (Always-on SSB is periodically transmitted on the cell). Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement (RAN1#119) For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer. - Although two options are agreed for on-demand SSB deactivation, separate FGs corresponding to each option don’t seem to be necessary in our view. - Considering RRC or MAC CE signaling for on-demand SSB transmission indication is UE-specific, introducing separate FGs depending on RRC or MAC CE based signalling mechanism for on-demand SSB transmission indication (i.e., keeping FG 61-1 and FG 61-3 separate) would be reasonable. 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB SCell transmission/deactivation on the cell in Case #1 (No always-on SSB on the cell); UE does not support on-demand SSB SCell transmission/deactivation indicated via MAC CE in Case #1. |
| Apple [15] |  |
| NTT DOCOMO, INC. [16] | On the FFS of supported deactivation mechanisms in each FG, we will have at least two deactivation schemes, based on explicit MAC CE signaling and implicit timer or counter. We are open to discuss whether to decouple those two schemes into two separate FGs; however, we believe we should decide a default scheme from the two. From our perspective, it should be straightforward to support MAC CE deactivation as a default, given that the activation is addressed by MAC CE only. Thus MAC CE based deactivation can be included in each FG, or a component in each FG can report either “support of MAC CE based deactivation” or “support of both MAC CE based deactivation and timer-based deactivation”. On the FFS of whether to merge FG 61-3 with FG 61-1 and FG 61-4 with FG 61-2, we support merging them respectively. Since RRC based OD-SSB indication is supported only for direct SCell activation (meaning OD-SSB indication is always provided along with SCell activation), the standalone support of the RRC indication is difficult to operate. This is because the legacy direct SCell activation is optional feature for CA operation which was introduced in Rel-16, and even for UEs capable of direct SCell activation, once SCell is deactivated, MAC CE based SCell activation and/or OD-SSB activation should be appropriate operation as the UE already has SCell configuration, no RRC signaling is needed again. Regarding the type, per-band is our best preference, which would contribute to easier operation from NW operators’ perspective. Meanwhile, if finer granularity than per-band is really necessary, it should be per BC, not per-FS or per FSPC. It is because, among the band combos there should be a freedom for NW to choose PCell. If OD-SSB features are defined to be per-FS/FSPC granularity, it would result in that NW must choose particular band/CC to be PCell in order to achieve NES operation because of UE implementation. Also, per-FS/FSPC allows different UEs to support different condition to support OD-SSB operation, which eventually suppress the benefit of NES in real field. To make sure OD-SSB is beneficial in real field, we prefer to support per BC. 61. Netw_Energy_NR_enh; 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication /deactivation] on the cell in Case #1 (No always-on SSB on the cell); FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation ]indicated via MAC CE in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-1; Optional with capability signaling. |
| Ericsson [17] | - Components - Component 1: MAC CE signaling should support both start and stop of the on-demand SSBs on the SCell. Suggest replacing “[indication/deactivation]” with “indication/deactivation”. - (New) component 2. Add the following. “Supported deactivation mechanisms: - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 2: Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx).” - No need for pre-requisite FGs for this. Basic CA operation, etc is assumed but it is not necessary to explicitly list the CA capabilities as prerequisite. - Should be per band. - Note column - The supported deactivation mechanism should be both options 1 and 2 for this FG (as described in Component 1 discussion above). - Suggest updating as “FFS: supported deactivation mechanisms”. - This FG should be merged with 61-1. 61. Netw_Energy_NR_enh; 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB SCell indication/deactivation on the cell in Case #1 (No always-on SSB on the cell) 2. Supported deactivation mechanisms : • Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication • Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx).; Yes; UE does not support on-demand SSB SCell indication/deactivation]indicated via MAC CE in Case #1; per band; n/a; n/a; n/a; merge with 61-1; Optional with capability signaling. |
| Qualcomm Incorporated [18] | 61. Netw_Energy_NR_enh; 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate on-demand SSB transmission SCell [indication /deactivation] on the cell in Case #1 (No always-on SSB on the cell); FFS; Yes; UE does not support on-demand SSB transmission SCell [indication /deactivation] indicated via MAC CE in Case #1; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-1; Optional with capability signaling. |

| 61. Netw_Energy_NR_enh | 61-4 | On-demand SSB SCell operation indicated via MAC CE in Case #2 [for same center frequency] | 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency] | FFS | Yes |  | UE does not support on-demand SSB SCell [indication/deactivation] indicated via MAC CE in Case #2 [for same center frequency] | [Per band] | n/a | n/a | n/a | FFS: supported deactivation mechanisms FFS: whether to merge with 61-2 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| ZTE Corporation/Sanechips [2] | According to the agreement in RAN1#119 meeting [1], there were two mechanisms for on-demand SSB deactivation. Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer. If separate FGs for on-demand SSB indication and deactivation, it will be complicated to include the case that MAC CE activation plus N times for on demand SSB transmission configured in RRC signaling. In our understanding, it belongs to the case of simultaneous on-demand SSB indication and deactivation, so that a single FG for indication and deactivation is much clearer to cover the case MAC CE activation plus N times for on-demand SSB transmission configured in the RRC signaling. Support a single FG for on-demand SSB SCell indication/deactivation. In previous meeting [2], RRC signaling and MAC CE signaling for on-demand SSB SCell indication were agreed. Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable. But in our views, RRC signaling and MAC CE signaling should be supported in separate FGs, since it is unnecessary for UE to support both indication signalings for on-demand SSB SCell indication. Split MAC CE indication/deactivation and RRC indication/deactivation in separate FGs. 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 [for same center frequency]; 1. Support MAC CE based signalling to indicate on-demand SSB SCell indication/deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]; FFS; Yes; UE does not support on-demand SSB SCell indication/deactivation indicated via MAC CE in Case #2 [for same center frequency]; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-2; Optional with capability signaling. |
| Vivo [3] | Regarding [for same center frequency] in FG 61-2 and 61-4, it should be kept since Case#2 with different frequency is subject to a separate UE capability according to the following RAN1 agreement: Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB Proposal 1: Keep [for same center frequency] for FG 61-2 and 61-4 and introduce separate FG for Case #2 with different center frequency. Regarding FFS: supported deactivation mechanisms, we support to have different deactivation mechanism for different cases. However, the details may need more RAN1 progress. Proposal 2: Keep FFS: supported deactivation mechanisms for all FGs and wait for more RAN1 progress to complete the details. Regarding FFS: whether to merge with 61-1 and FFS: whether to merge with 61-2 in FG 61-3 and FG 61-4 respectively, we don’t support to merge them since there is no need for a UE to support two different mechanisms for the same purpose. Proposal 3: Remove FFS: whether to merge with 61-1 and FFS: whether to merge with 61-2 in FG 61-3 and FG 61-4 respectively. Regarding [Per band], we support to keep per band for each FG. Proposal 4: Keep Per band for FG 64-1, 64-2, 64-3 and 64-4. 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 for same center frequency; 1. Support MAC CE based signalling to indicate on-demand SSB indication/deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency; Yes; UE does not support on-demand SSB indication/deactivation indicated via MAC CE in Case #2 for same center frequency; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| FUTUREWEI [4] | In the last RAN1 (#120), it was agreed the support of Alt 1. • Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. While in RAN1, #119 it was established that: • The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. In the view of these agreements, the temporary text “[for same center frequency]” under FG column should be removed. Similarly, the text “[for same [center] frequency]” should be removed from the Components column. Proposal 1: In the FG 61-2 and FG 61-4 rows remove temporary text marked by yellow highlight related to “[for same center frequency]” As mentioned above, our preference is to limit as much the distinctive FG related to the same feature, to avoid UE market fragmentation. To this purpose at least the MAC-CE and RRC should be supported by the same FG. Proposal 2: Merge FG 61-3 to FG 61-1, and FG 61-4 to FG 61-2. In RAN1 #119, it was decided that a UE supporting on-demand SSB will support at least two options for deactivation of on-demand SSB. Thus, gNB may indicate via MCA-CE and RRC the on-demand SSB and via MAC-CE and RRC the deactivation of on-demand SSB. Thus, the text “[indication/deactivation]” may be approved. Proposal 3: For on-demand SSB FG, approve the text “[indication/deactivation]”. In the last meeting RAN1 #120 companies agreed on supporting the same signalling solutions for both scenarios Case #1 and Case #2. For instance, for supporting time domain location of on-demand SSB RRC, and for supporting a new higher layer parameter for configuration of multiple parameters. We also note that, from the implementation point of view of UE supporting Case #1 and respectively Case#2, there is very little difference. If there are distinct FGs for supporting Case #1and Case #2, not only does it lead to market fragmentation, but it may be inefficient from a deployment point of view, as it would limit access of some UE to certain SCell deployments. Indeed, UEs supporting FG supporting Case #2 should be able to be deployed in the scenarios Case #2, but not the other way around. For these reasons, we strongly prefer to have a single feature related to on-demand SSB. Proposal 4: Merge all FG 61-x related to on-demand SSB FG in a single FG 61-1. Adopt the proposed FG 61-1. With the previous comments and proposals, the FG61 table will become a single row. 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation; - Support RRC based signalling to indicate on-demand SSB SCell (indication/deactivation]) on the cell in Case #1 (No always-on SSB on the cell) - 1. Support RRC based signalling to indicate on-demand SSB SCell (indication /deactivation) on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) - Support MAC CE based signalling to indicate on-demand SSB SCell (indication /deactivation) on the cell in Case #1 (No always-on SSB on the cell) - 1. Support MAC CE based signalling to indicate on-demand SSB SCell (indication/deactivation])on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell); FFS; Yes; UE does not support on-demand SSB SCell; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| CATT [5] | RAN1 has agreed to use MAC-CE based signaling to indicate or deactivate the transmission of on-demand SSB. In addition, RRC based signaling has also been supported to indicate on-demand SSB transmission. Deactivation by RRC is up to RAN2 [1]. The following agreement was achieved by RAN2 in RAN2#127bis meeting [2], which means RRC based signaling could deactivate on-demand SSB transmission. 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.. Intuitively, activation and deactivation by the same type of signaling should be supported by a UE. Further, RRC based signaling and MAC-CE based signaling should also both be supported by the UE as complementary indication mechanisms. The following agreement was made in RAN1#120 related to start-indicator part of R-TAS [2]: Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB. According to above agreement, for Case 2, regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, subject to separate UE capability, if always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. Hence, whether or not the UE can measure the OD-SSB at a center frequency different from that of the always-on SSB should be introduced as separate UE capability. Observation 1: Whether or not the UE can measure the OD-SSB at a center frequency different from that of the always-on SSB should be introduced as separate UE capability. New separate UE feature group whose index is 61-2-2 are introduced for the UE capability of whether or not the UE can measure the OD-SSB at a center frequency different from that of the always-on SSB. The original index of UE feature group 61-2 is changed to 61-2-1 for the UE capability of whether or not the UE can measure the OD-SSB at a center frequency same as that of the always-on SSB. FG 61-4 is merged. For the sake of clear description, the text "[for same center frequency]" in the name of original feature group 61-2 has been changed to "for the on-demand SSB at a center frequency same as that of the always-on SSB". For these feature groups, the type of granularity should be Per Band. Proposal 2: For FG 61-2 and 61-4: - Merge FG 61-4 with FG 61-2. - The original index of UE feature group 61-2 is changed to 61-2-1 for the UE capability of whether or not the UE can measure the OD-SSB at a center frequency same as that of the always-on SSB. - For the sake of clear description, the text "[for same center frequency]" in the name of original feature group 61-2 has been changed to "for the on-demand SSB at a center frequency same as that of the always-on SSB". - For these feature groups, the type of granularity should be Per Band. |
| China Telecom [6] | 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 [for same center frequency]; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]; FFS; Yes; UE does not support on-demand SSB SCell [indication/ deactivation]] indicated via MAC CE in Case #2 [for same center frequency]; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-2; Optional with capability signaling. |
| Ofinno [7] | For feature groups 61-1 – 61-4, we suggest to clarify whether the UE supports these features for intra-band or inter-band (e.g., either in the type definition and/or using a component). Supporting OD-SSB for intra-band may, for example, be simpler for many UEs while supporting inter-band may be more complex. Proposal 1: Clarify for FGs 61-1 to 61-4 whether these features are for intra-band, inter-band, or both. For feature groups 61-3 and FG 61-4 (MAC CE activation), we suggest to add a component for the maximum number of SCells that the UE can support the FG for. Proposal 2: Add a component to FG 61-3 and FG 61-4 for maximum configured number of cells with OD-SSB. 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 [for same center frequency]; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated via MAC CE in Case #2 [for same center frequency]; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-2; Optional with capability signaling. |
| Nokia [8] |  |
| CMCC [9] | - Regarding the FFS part on Feature group column, since it is agreed in last RAN1 meeting that the frequency for on-demand SSB and always-on SSB can be same or different, the FFS part should be removed. - Regarding the FFS part on Components and Consequence if the feature is not supported by the UE column, we support to keep the wording in indication/deactivation while remove the wording for frequency. - Regarding the FFS part on Prerequisite feature groups, our view is that RRC-based indication shall be a basic capability while MAC CE-based indication is an additional capability. Therefore, we prefer that FG 61-3 and 61-4 should merged with FG 61-1 and 61-2, respectively. - Regarding the FFS part on Type column, we support the current wording. 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2; 1. Support MAC CE based signalling to indicate on-demand SSB SCell indication/deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell); 61-2; Yes; UE does not support on-demand SSB SCell indication/deactivation indicated via MAC CE in Case #2; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| Huawei/HiSilicon [10] | Based on the agreement of #120 meeting, if always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB, and this is subject to a UE capability which is different from the UE capability on the support of the same frequency location for on-demand SSB and always-on SSB. Therefore, for case #2, two separate UE capabilities can be introduced for RRC based signalling and MAC CE based signalling respectively. Proposal 1: For case #2, two separate UE capabilities can be introduced for RRC based signalling and MAC CE based signalling individually to support the case where the frequency location of on-demand SSB is different from the frequency location of always-on SSB if always-on SSB is CD-SSB on a synchronization raster. As for the reporting type, we think per band reporting is sufficient. Proposal 3: Set the reporting type for on-demand SSB SCell operation as per band. As for the FFS issue that whether to merge MAC CE based signalling with RRC based signalling, UE needs to follow the timeline to determine the SSB time domain positions of OD SSB on the cell for MAC CE based signalling, which is more stringent than RRC based signalling. Therefore, separate UE capabilities are preferred. Proposal 4: Do not merge the UE capability of MAC CE based signalling with RRC based signalling. 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 [for same center frequency]; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated via MAC CE in Case #2 [for same center frequency]; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-2; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-4a; On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequency; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated via MAC CE in Case #2 for different center frequency; Per band; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |
| OPPO [11] |  |
| Samsung [12] | 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 [for same center frequency]; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency] 2. Support L1 measurement based on on-demand SSB 3. Support L3 measurement based on on-demand SSB; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated via MAC CE in Case #2 [for same center frequency]; [Per band] Per BC; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-2; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-4a; On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequency; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency 2. Support L1 measurement based on on-demand SSB 3. Support L3 measurement based on on-demand SSB; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated via MAC CE in Case #2 for different center frequency; Per BC; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-2a; Optional with capability signaling. |
| Xiaomi [13] | On-demand SSB is only applicable to SCell. There are two cases for on-demand SSB, i.e., case#1 and case#2. - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell Furthermore, the following agreement was achieved in RAN1#116bis meeting: Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.. Observation 1: For FG related to on-demand SSB, at least the following scenario and case should be captured as component: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 For on-demand SSB, the following agreement was achieved: Agreement New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.. Furthermore, there is no agreement touching physical structure for on-demand SSB. From our perspective, we don’t see motivation on changing content of on demand SSB burst if always on SSB is assumed. For on demand SSB, the following content as shown in proposal 2 should be capture in the FG. Observation 2: The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms}. - If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB. For case #2, always on SSB is assumed and only the additional SSB can be turned on or turned off. Regarding the always on SSB, we don’t see the necessity to put any restriction on it, i.e., it can be either CD-SSB or NCD-SSB. Regarding to the relationship between always-on SSB and additional SSB, the frequency location can be same or different. However, if AO-SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency from the frequency location of always-on SSB. [1] Observation 3: If always on SSB is CD-SSB on synchronization raster, the frequency location of always on SSB and additional SSB is different. - Otherwise, the frequency location of always on SSB and additional SSB can be different. Per RAN1 agreements, only gNB-triggered on-demand SSB is supported. Specifically, gNB can activate/indicate on-demand SSB transmission and deactivate on-demand SSB transmission. Either MAC CE or RRC signaling can be used as indication. RRC signaling is a universal solution across different scenarios. On the other hand, MAC CE mechanism can be applicable at least to scenario 2A and scenario 2. Observation 4: For OD-SSB capable UE, it supports both RRC-based indication and MAC CE based indication for OD-SSB activation/deactivation. - For MAC CE based indication, it is applicable to scenario 2A and scenario 2. FFS other scenarios. One major functionality of SSB is it can be used for measurement, i.e., L1 measurement and L3 measurement. RAN1 achieved the following agreements on measurement based on on-demand SSB. Agreement Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4 Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not. Despite of always-on SSB and on-demand SSB, neither physical structure nor functionality is changed. From this perspective, SSB type doesn’t impact measurement at UE side. As shown in the above agreements, it has been agreed that on-demand SSB can be used for L3 measurement. Considering that L1 measurement is a key step for L3 measurement, i.e., UE measures several SSB samples which can be regarded as L1 measurement and then filtering all L1 measurement results to get L3 measurement result, there is no barrier to use on-demand SSB for L1 measurement as well. In particular, the following agreement on spatial assumption between on-demand SSB and always on SSB was made in RAN1#119 meeting, which we believe further fortify aforementioned analyses. Agreement Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1: - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.. Observation 5: For OD-SSB capable UE, on-demand SSB is used for both L1 measurement and L3 measurement. 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 [for same center frequency]; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency] 2. MAC CE based signaling to indicate on-demand SSB SCell [indication/deactivation] on the cell is applicable to [scenario#2 and scenario#2A] 3. The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms}. If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB. 4. If always on SSB is CD-SSB on synchronization raster, the frequency location of always on SSB and additional SSB is different. Otherwise, the frequency location of always on SSB and additional SSB can be different. 5. On-demand SSB is used for both L1 measurement and L3 measurement; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated via MAC CE in Case #2 [for same center frequency]; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-2; Optional with capability signaling. |
| LG Electronics [14] | Agreement (RAN1#120) Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB. As highlighted above, the case where the frequency location of on-demand SSB is different from the frequency location of always-on SSB is supported, subject to separate UE capability. Thus, in order to cover this case, it would be straight-forward to define new FG for Case #2 (Always-on SSB is periodically transmitted on the cell). Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement (RAN1#119) For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer. Regarding on-demand SSB activation/deactivation, the followings need to be considered. - The phrase “signaling to indicate on-demand SSB transmission” as highlighted in cyan above can be reused to describe FGs 61-1 to 61-4. - Although two options are agreed for on-demand SSB deactivation, separate FGs corresponding to each option don’t seem to be necessary in our view. 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 for same center frequency; 1. Support MAC CE based signalling to indicate on-demand SSB SCell transmission/deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency between on-demand SSB and always-on SSB; UE does not support on-demand SSB SCell transmission/deactivation indicated via MAC CE in Case #2 for same center frequency. 61-4a; On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequency; 1. Support MAC CE based signalling to indicate on-demand SSB SCell transmission/deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same different frequency between on-demand SSB and always-on SSB; UE does not support on-demand SSB SCell transmission/deactivation indicated via MAC CE in Case #2 for different center frequency. |
| Apple [15] | 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 for same center frequency between AO-SSB and OD-SSB; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency between AO-SSB and OD-SSB; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated via MAC CE in Case #2 for same center frequency between AO-SSB and OD-SSB; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-2; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-4a; On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequencies between AO-SSB and OD-SSB within the same BWP; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequencies between AO-SSB and OD-SSB within the same BWP; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated via MAC CE in Case #2 for different center frequencies between AO-SSB and OD-SSB within the same BWP; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-2; Optional with capability signaling. |
| NTT DOCOMO, INC. [16] | On the FFS of supported deactivation mechanisms in each FG, we will have at least two deactivation schemes, based on explicit MAC CE signaling and implicit timer or counter. We are open to discuss whether to decouple those two schemes into two separate FGs; however, we believe we should decide a default scheme from the two. From our perspective, it should be straightforward to support MAC CE deactivation as a default, given that the activation is addressed by MAC CE only. Thus MAC CE based deactivation can be included in each FG, or a component in each FG can report either “support of MAC CE based deactivation” or “support of both MAC CE based deactivation and timer-based deactivation”. On the FFS of whether to merge FG 61-3 with FG 61-1 and FG 61-4 with FG 61-2, we support merging them respectively. Since RRC based OD-SSB indication is supported only for direct SCell activation (meaning OD-SSB indication is always provided along with SCell activation), the standalone support of the RRC indication is difficult to operate. This is because the legacy direct SCell activation is optional feature for CA operation which was introduced in Rel-16, and even for UEs capable of direct SCell activation, once SCell is deactivated, MAC CE based SCell activation and/or OD-SSB activation should be appropriate operation as the UE already has SCell configuration, no RRC signaling is needed again. On whether same or different frequency b/w AO and OD-SSB in case#2, as RAN1 agreed below, separate capabilities should be taken. It seems a reasonable way to indicate what frequency relation is supported by the UE, e.g., {same only, different only, both}. The final decision on how/what element should be defined in the list would depend on detailed UE behaviors (e.g., which behavior is different for UE to implement). Agreement@RAN1#120 Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability Note: UE is not required to measure both AO-SSB and OD-SS. Regarding the type, per-band is our best preference, which would contribute to easier operation from NW operators’ perspective. Meanwhile, if finer granularity than per-band is really necessary, it should be per BC, not per-FS or per FSPC. It is because, among the band combos there should be a freedom for NW to choose PCell. If OD-SSB features are defined to be per-FS/FSPC granularity, it would result in that NW must choose particular band/CC to be PCell in order to achieve NES operation because of UE implementation. Also, per-FS/FSPC allows different UEs to support different condition to support OD-SSB operation, which eventually suppress the benefit of NES in real field. To make sure OD-SSB is beneficial in real field, we prefer to support per BC. 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 [for same center frequency]; 1. Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]; FFS; Yes; UE does not support on-demand SSB SCell [indication/deactivation] indicated via MAC CE in Case #2 [for same center frequency]; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-2; Optional with capability signaling. |
| Ericsson [17] | - FG name: - Same or different center frequency can become candidate values that UE reports. - Suggest updating FG name as “On-demand SSB SCell operation indicated via MAC CE in Case #2 [for same center frequency]”. - Components - Component 1: - Suggest updating as “Support MAC CE based signalling to indicate on-demand SSB SCell [indication/deactivation] indication/deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency]” - Deactivation : same comment as for FG 61-3. - (New) Component 2: same as for FG 61-3 - No need for pre-requisite FGs for this. Basic CA operation, etc is assumed but it is not necessary to explicitly list the CA capabilities as prerequisite. - Should be per band. - Note column - Suggest updating as “FFS: supported deactivation mechanisms”. - This FG should be merged with 61-2. 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2; 1. Support MAC CE based signalling to indicate on-demand SSB SCell indication/deactivation on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) 2. Supported deactivation mechanisms : • Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication • Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx).; Yes; UE does not support on-demand SSB SCell indication/deactivation indicated via MAC CE in Case #2; per band; n/a; n/a; n/a; merge with 61-2; Optional with capability signaling. |
| Qualcomm Incorporated [18] | RAN1#120 made the following agreement for time-domain relationship between on-demand SSB and always-on SSB. Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted Send an LS to RAN4 to inform them of the above agreement. Final LS in R1-2501633.. From our perspective, FGs 61-2 and 61-4 should be applicable to the case when the center frequency locations of always-on SSB and on-demand SSB are same and Alt Time-C1 in the agreement (The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification)). In addition, we propose to define new FG 61-2a and new FG 61-4a for indicating the support of Case 2 with time relations other than Alt Time-C1 (e.g., Alt Time-C2 or the case when the center frequency locations of always-on SSB and on-demand SSB are different). 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 [for same center frequency] and periodic time domain SSB pattern during OD-SSB transmission; 1. Support MAC CE based signalling to indicate on-demand SSB transmission SCell [indication /deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) with [for same center frequency] for on-demand SSB and always-on SSB; FFS; Yes; UE does not support on-demand SSB transmission SCell [indication /deactivation] indicated via MAC CE in Case #2 [for same center frequency]; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms FFS: whether to merge with 61-2 Note: during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern (the interval between SSB bursts is even and supported in legacy specification).; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-4a; On-demand SSB SCell operation indicated via MAC-CE in Case #2 for same center frequency and non-periodic time SSB domain pattern during OD-SSB transmission; 1. Support MAC-CE based signalling to indicate on-demand SSB transmission on the cell in Case #2 in which during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern.; FFS; Yes; UE does not support on-demand SSB transmission indicated by MAC-CE in Case #2 in which during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern.; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-4b; On-demand SSB SCell operation indicated by MAC-CE based signaling in Case #2 for different center frequencies; 1. Support MAC-CE based signalling to indicate on-demand SSB transmission on the cell in Case #2 in which AO-SSB and OD-SSB have different center frequency.; FFS; Yes; UE does not support on-demand SSB transmission indicated by MAC-CE based signaling in Case #2 in which AO-SSB and OD-SSB have different center frequencies; [Per band]; n/a; n/a; n/a; FFS: supported deactivation mechanisms; Optional with capability signaling. |

## On-demand SIB1 for idle/inactive mode UEs

| 61. Netw_Energy_NR_enh | 61-5 | SIB1 request [for idle/inactive UEs] | 1. Reception of [RACH] configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request |  |  |  |  | FFS | FFS | FFS | FFS | [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1] | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| ZTE Corporation/Sanechips [2] | Per agreements in previous meeting in RAN2, the uplink WUS configuration is transmitted in SIB in Cell A [4]. Agreement NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration). In legacy, for the function transmitted in SIB, such as the FG for basic initial access channels and procedures, the FG for support of polarization signaling in NR NTN, and the FG for TRS resources for idle/inactive UEs, it is mandatory or optional without capability signaling. To this end, similar with the UE features mentioned above, UE features for OD-SIB1 should be ‘optional without capability signaling’, i.e., OD-SIB1 is a mandatory UE feature for Rel-19 UE. UE feature for on-demand SIB1 is optional without capability signaling. In RAN1#120 meeting, there was an agreement for on-demand SIB1 transmission [3]. Agreement The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding to at least the SSB associated with the PRACH for UL-WUS if this is indicated via UL WUS configuration - FFS: Whether UE expect PDCCH on PDCCH monitoring occasion corresponding to SSB other than the one mentioned above - FFS: Whether gNB can indicate (in UL-WUS configuration) the SSB the UE can expect PDCCH transmission on. To this end, it should be described clearly that the OD-SIB1 request is within the OD-SIB1 window. Support the description of Component 3 in UE feature for the on-demand SIB1 (61-5), i.e., ‘3. Reception of SIB1 in a window at least upon SIB1 request’ 61. Netw_Energy_NR_enh; 61-5; SIB1 request for idle/inactive UEs; 1. Reception of RACH configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 in a window at least upon SIB1 request; N/A; NO; NO; FFS; [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1]; Optional [with/without] capability signaling. |
| Vivo [3] | Regarding [for idle/inactive UEs], it is preferred to add connected UEs since RAN2 already agreed that a connected UE can send WUS to request SIB1 when RLF happens. Considering this is not RAN1 scope, we can support to add a note, i.e. “whether to merge SIB1 request for connected UEs is up to RAN2”. Proposal 5: Add a note in FG 61-5, i.e. whether to merge SIB1 request for connected UEs is up to RAN2. Regarding Optional [with/without] capability signalling, we are fine with both but slightly prefer with capability signalling so that network may know how much ratio of UE support this feature and determine whether to apply OD-SIB1 in target cells. Proposal 6: Support optional with capability signaling for FG 61-5. 61. Netw_Energy_NR_enh; 61-5; SIB1 request for idle/inactive UEs; 1. Reception of RACH configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 in a window at least upon SIB1 request; Per UE; n/a; n/a; n/a; A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1 Note: whether to merge SIB1 request for connected UEs is up to RAN2; Optional with capability signaling. |
| FUTUREWEI [4] | We note that the NES WID for SIB1 on-demand define the WI as “Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]”, thus the provisional text [for idle/inactive UEs] should be adopted. Proposal 5: For on-demand SIB1 FG adopt provisional text [for idle/inactive UEs]. We note that the component “Transmission of PRACH on the uplink to request SIB1 of the cell” is not quite accurate. PRACH is the Physical Random Access Channel and is associated with uplink initial access. Therefore, the correct formulation is “Transmission on the uplink PRACH to request SIB1 of the cell.” The Consequence if the feature is not supported by the UE is that UE does not support on-demand SIB1 operation. With the proposed changes (including those for on-demand SSB) the FG 61-5 becomes FG 61-2: 61. Netw_Energy_NR_enh; 61-5; SIB1 request for idle/inactive UEs; 1. Reception of [RACH] configuration associated with SIB1 request for a cell 2. Transmission on the uplink PRACH to request SIB1 of the cell. 3. Reception of SIB1 [in a window] [at least] upon SIB1 request; FFS; FFS; FFS; FFS; [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1]; Optional [with/without] capability signaling. Proposal 6: For on-demand SIB1 FG adopt the proposed FG 61-5. |
| CATT [5] | In our view, if a UE has SIB1 request configuration of a cell and before transmitting UL WUS, it can detects whether on-demand SIB1 is transmitting and monitor Type 0 PDCCH for SIB1. The monitoring time window of Type 0 PDCCH is up to UE implementation. According to the above analysis, SIB1 reception does not necessarily have to be preceded by the sending of the SIB1 request. So the word ‘at least’ can be accepted. Proposal 4: Adopt the following for FG 61-5: - For component 1, support reception of RACH configuration associated with SIB1 request for a cell. - For component 3, support reception of SIB1 in a window at least upon SIB1 request. - The type of granularity should be Per UE. 61. Netw_Energy_NR_enh; 61-5; SIB1 request [for idle/inactive UEs]; 1. Reception of [RACH] configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request; UE does not support on-demand SIB1 for idle and inactive UEs; FFS Per UE. |
| China Telecom [6] | It is reasonable for UE to support OD-SIB1 on one band but not on another band. Thus, the FG1-5 should be per band. And the components of this FG can be feature discussed after the discussion on the SIB1 transmission state indication in AI 9.5.2. Proposal 3: FG 61-5 should be per band. Based on above proposals, we propose the initial table for UE feature of on-demand SIB1 operation as follows (the proposed parts are marked in red). 61. Netw_Energy_NR_enh; 61-5; SIB1 request [for idle/inactive UEs]; 1. Reception of [RACH] configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request; na; Per band; FFS; FFS; FFS; [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1]; Optional [with/without] capability signaling. |
| Ofinno [7] | Per the WID it is agreed that OD-SIB1 request (i.e., UL WUS) is sent on the NES cell. We think the FG should clarify that the transmission is via the NES cell. Proposal 4: Clarify in FG 61-5 that the transmission of the PRACH is on the NES cell to request SIB1. 61-5; SIB1 request [for idle/inactive UEs]; 1. Reception of [RACH] configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request; FFS; FFS; FFS; FFS; [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1]; Optional [with/without] capability signaling. |
| Nokia [8] |  |
| CMCC [9] | Regarding the FFS part (i.e. wording in yellow highlight), our considerations are as follows: - Regarding the FFS part on Feature group column, since it is agreed in last RAN2 meeting that the on-demand SIB1 can also be used for RRC_CONNECTED UE, the FFS part should be removed. - Regarding the FFS part on Components column, we support to keep the current wording. - Regarding the FFS part on Prerequisite feature groups, we support it as “UE does not support SIB1 request”. - Regarding the FFS part on Type column, we support per band. - Regarding the FFS part on Need of FDD/TDD differentiation, Need of FR1/FR2 differentiation and Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 column, we support it as N/A. - Regarding the FFS part on Mandatory/Optional column, our view is that UE can not report the signaling if it does not support such capability, and the content here can be ‘Optional without capability signaling’, and the Need for the gNB to know if the feature is supported column can be set as No. Proposal 2: Take the following UE feature list for the feature of on-demand SIB1 operation: 61. Netw_Energy_NR_enh; 61-5; SIB1 request; 1. Reception of RACH configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 in a window at least upon SIB1 request; No; UE does not support SIB1 request; Per band; n/a; n/a; n/a; Optional without capability signaling. |
| Huawei/HiSilicon [10] | For SIB1 request for idle/inactive UEs, the issue about whether to define the FG needs to be solved first. R19 NES UE can transmit a wake up signal to request SIB1, and then the network can know UE capability via UL WUS detection. But before transmitting the wake up signal, the UE needs to get UL WUS configurations first. For the network, whether to activate NES cell and broadcast the UL WUS configurations via Cell A or NES Cell depends on whether there are enough R19 NES UEs existing in the network. In our view, based on the UE capability reporting in connected mode, network can get a good knowledge about the distributing of R19 NES UEs and legacy UEs, which can help the network to decide whether to activate NES cell and broadcast the UL WUS configurations. So, we propose to report the UE capability of SIB1 request for idle/inactive UEs. Based on the agreements of RAN2 #129 meeting, SIB1 request can also be supported by the UEs in RRC_CONNECTED when T311 is running, and the OD-SIB1 acquisition behaviour is same as that of RRC_IDLE/INACTIVE UEs. In our view, there is no need to set a separate UE capability for RRC_CONNECTED mode, because the UE behaviour is the same for the OD-SIB1 acquisition in different modes. As for the reporting type, we think per UE reporting is sufficient as reception of RRC configuration and transmission of PRACH is rather basic UE capability regardless supported bands or band combinations. As for the ‘Need of FDD/TDD differentiation’ and the ‘Need of FR1/FR2 differentiation’, we don’t see much difference from UE complexity point of view. Proposal 6: FG 61-5 is extended to the UEs in RRC_CONNECTED mode, and the reporting type is per UE. 61. Netw_Energy_NR_enh; 61-5; SIB1 request [for idle/inactive UEs]; 1. Reception of [RACH] configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request; UE does not support SIB1 request; FFSPer UE; FFSNo; FFSNo; FFSN/A; [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1]; Optional [with/without] capability signaling. |
| OPPO [11] |  |
| Samsung [12] | For on-demand SIB1, RAN2 made an agreement to apply the same procedure for requesting on-demand SIB1 in RRC_CONNECTED mode, and another agreement that SIB1 reception only occurs after receiving the RAR of the PRACH, so we propose the following changes to reflect those agreements. 61. Netw_Energy_NR_enh; 61-5; SIB1 request [for idle/inactive/connected UEs]; 1. Reception of [RACH] configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon reception of RAR for the SIB1 request; FFS; FFS; FFS; FFS; [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1]; Optional [with/without] capability signaling. |
| Xiaomi [13] | In RAN1#118 meeting, the following agreement was achieved: Agreement RAN1 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.. Based on the above agreement, it is not possible for OD-SIB1 capable UE to indicate its capability to network. On the other hand, gNB doesn’t need to receive UE’s capability on supporting OD-SIB1. UL WUS configuration is provided by SIBx from cell A which is transmitted in cell-specific manner. In the other words, network provides UL WUS configuration for OD-SIB1 capable cell without considering dedicated UE capability. For an OD-SIB1 capable UE, it requests SIB1 on NES cell based on aforementioned cell-specific configuration. For a UE not capable for OD-SIB1, it ignores UL WUS configuration. Observation 6: For an OD-SIB1 capable UE, it doesn’t need to report its capability to network. Recall on-demand SI procedure, there is no UE capability reporting for IDLE/CONNECTED mode. In R16 TEI, the following UE feature was defined for on-demand request procedure of SIB(s) or posSIB(s) while in RRC_CONNECTED. 24-4; Support of on demand request procedure in RRC CONNECTED; Indicates whether the UE supports the on-demand request procedure of SIB(s) or posSIB(s) while in RRC_CONNECTED, as specified in TS 38.331 [2].. It was discussed on the necessity of defining UE feature related to OD-SIB1 in RRC_IDLE/INACTIVE mode needs to be clarified. The common understanding is that the UE may not need to report relevant capability on whether support OD-SIB1 in IDLE/INACTIVE mode, it is still necessary to define the UE feature in order to show what UE supports. Accordingly, the following feature was agreed as starting point for further discussion. Agreement: Introduce the following Rel. 19 UE FG (yellow highlighting, if any, shows text that’s not yet agreed) 61. Netw_Energy_NR_enh; 61-5; SIB1 request [for idle/inactive UEs]; 1. Reception of [RACH] configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request; FFS; FFS; FFS; FFS; [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1]; Optional [with/without] capability signaling. 61. Netw_Energy_NR_enh; 61-5; SIB1 request [for idle/inactive UEs]; 1. Reception of [RACH] UL WUS configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink cell to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request; FFS; FFS; FFS; FFS; [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1]; Optional [with/without] capability signaling. |
| LG Electronics [14] |  |
| Apple [15] | 61. Netw_Energy_NR_enh; 61-5; SIB1 request [for idle/inactive UEs]; 1. Reception of [RACH UL WUS] configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon reception of response to SIB1 request; FFS; FFS; FFS; FFS; [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1]; Optional [with/without] capability signaling. |
| NTT DOCOMO, INC. [16] | For on-demand SIB1, the whole procedure of UE triggering UL WUS and monitoring subsequent SIB1 was discussed. On whether to define the corresponding capability signalling or to define FG without capability signalling, it is preferred to define both FG and capability signalling. The capability signaling is useful for NW to decide whether to operate a cell as OD-SIB1 operation, for example, after non-NES cell (that can be normal cell or cell A providing UL WUS config.) obtains a certain number of UE’s support from the signaling, NW would decide to operate some neighboring cells as OD-SIB1 cell determined by its NW implementation. This feature is not related to CA, so type should be per band. We show our preference for other minor wording as below: On the name feature group, at least “idle/inactive” should be removed. RAN2 agreed that a UE can request OD-SIB1 during the procedure of radio link failure. Throughout the RLF procedure, the UE is kept in RRC connected mode as RAN2 discussed, so it is not appropriate to just say that support of SIB1 request for idle/inactive mode. Rather, more general wording should be better. The configurations and parameters needed to request OD-SIB1 are not only RACH related but also frame configurations, and so on. The wording of “RACH” in the component 1 should be removed. It is expected that some RRC parameters related to the configurations provided by cell A would be named “UL WUS configuration”. Naming so in UE capabilities as well would be easy to understand and read the specification. Regarding FFS wording in the 3rd component, i.e., [in a window]” and “[at least]”, our understanding is that RAN1 is now discussing the monitoring behavior on PDCCH SIB1 according to UL WUS config. before request of OD-SIB1 in main session. To avoid potential conflict with the decision to be made in main session, we think it is safer for now to have more general wording or keep them as FFS. 61. Netw_Energy_NR_enh; 61-5; Support of on-demand SIB1 operation SIB1 request [for idle/inactive UEs]; 1. Reception of [RACH] UL WUS configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request according to the UL WUS configuration associated with the SIB1.; FFS; FFS; FFS; FFS; [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1]; Optional [with/without] capability signaling. |
| Ericsson [17] | - FG name: OK to update the FG name as “” SIB1 request [for idle/inactive UEs] idle/inactive UEs”. It is up to RAN2 to address any connected mode aspects of SIB1 reception, and RAN2 can update the text as needed (a new note can be added). - Components - Component 1: In last meeting, there were suggestions to not use “UL-WUS” in this component. Suggest replacing “[RACH] configuration” with “SIB1 request configuration”. - Component 3: The UE should be able to receive SIB1 upon request or by simply monitoring the SIB1 occasions. Suggest updating to “Reception of SIB1 [in a window] [at least] at least upon SIB1 request”. - Should be per UE. - Consequence if the feature is not supported by the UE: “UE does not camp on the cell” - No Need of a) FDD/TDD differentiation, b) FR1/FR2 differentiation, c) Capability interpretation for mixture of FDD/TDD and/or FR1/FR2. - Note column - OK to confirm text in yellow, i.e. “A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1”. - Add (new) “Note 2: More details can be added by RAN2.”. - Should be optional without capability signaling. 61. Netw_Energy_NR_enh; 61-5; SIB1 request idle/inactive UEs; 1. Reception of SIB1 Request configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 at least upon SIB1 request; per UE; n/a; n/a; n/a; A UE indicates support of this FG if it transmits a wake-up signal on the uplink to request SIB1 Note 2: More details can be added by RAN2.; Optional without capability signaling. |
| Qualcomm Incorporated [18] | Proposal 2: For on-demand SIB1, adopt the following updates to FG 61-5 61. Netw_Energy_NR_enh; 61-5; SIB1 request [for idle/inactive UEs]; 1. Reception of [RACH] uplink wake-up signal configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request; FFS; FFS; FFS; FFS; [A UE indicates support of this FG if it transmits PRACH a wake-up signal on the uplink to request SIB1]; Optional [with/without] capability signaling. |

## Adaptation of SSB transmissions

| 61. Netw_Energy_NR_enh | 61-6 | SSB burst periodicity adaptation for SCell operation | Adaptation of SSB burst periodicity for SCell |  | Yes |  | UE does not support adaptation of SSB burst periodicity for SCell | FFS | FFS | FFS | FFS | Note: the SSB for this FG is not cell defining | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| ZTE Corporation/Sanechips [2] | DCI-based SSB adaptation was supported in RAN1#120 meeting [3]. Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation. As a result, the component of UE feature for SSB adaptation in time domain should be updated accordingly. Moreover, more details, such as DCI format, need to be further captured in the component after they are agreed in 9.5.3. Add ‘DCI-based indication for SSB burst periodicity’ in the component of UE features for SSB adaptation in time domain. 61. Netw_Energy_NR_enh; 61-6; SSB burst periodicity adaptation for SCell operation; 1.Adaptation of SSB burst periodicity for SCell 2.DCI-based indication of additional PRACH resources; Yes; UE does not support adaptation of SSB burst periodicity for SCell; Per band; n/a; n/a; n/a; Note: the SSB for this FG is not cell defining; Optional with capability signaling. |
| Vivo [3] | 61. Netw_Energy_NR_enh; 61-6; SSB burst periodicity adaptation for SCell operation; Adaptation of SSB burst periodicity for SCell; Yes; UE does not support adaptation of SSB burst periodicity for SCell; Per band; n/a; n/a; n/a; Note: the SSB for this FG is not cell defining; Optional with capability signaling. |
| FUTUREWEI [4] | Additional agreements in RAN1 #120 on adaptation of SSB in time domain provide additional components to this FG. - Group common DCI signalling using DCI format 2_9 Proposal 7: Add Group common DCI signalling using DCI format 2_9 to the components of FG SSB burst periodicity adaptation for SCell operation. 61. Netw_Energy_NR_enh; 61-6; SSB burst periodicity adaptation for SCell operation; Adaptation of SSB burst periodicity for SCell Adaptation of SSB based on group common DCI signalling using DCI format 2_9; Yes; UE does not support adaptation of SSB burst periodicity for SCell; FFS; FFS; FFS; FFS; Note: the SSB for this FG is not cell defining; Optional with capability signaling. |
| CATT [5] | In RAN1#120, the following agreements on the SSB adaptation were achieved. Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation. It was agreed to support DCI format 2_9 with [cellDTRX-RNTI] for switching the SSB burst periodicity. It can be one of the components for the FG 61-6. Also, there is another agreement that the UE can be configured with [X] additional SSB burst periodicity for the Scell for switching the SSB burst periodicity, it can be another components for the FG 61-6.Regarding the type, since this feature is designed for the SCell, it can be defined as per Band. Proposal 5: Adopt the following for FG 61-6: - Introduce component 1: DCI format 2_9 for switching the SSB burst periodicity for SCell. - Introduce component 2: Support to be configure with [X] additional SSB burst periodicity for the SCell. - For this feature group, the type of granularity should be Per Band. 61. Netw_Energy_NR_enh; 61-6; SSB burst periodicity adaptation for SCell operation; 1. Support DCI format 2_9 for switching the SSB burst periodicity for SCell. 2. Support to be configure with [X] additional SSB burst periodicity for the Scell; Yes; UE does not support adaptation of SSB burst periodicity for SCell; [Per band]. |
| China Telecom [6] | In RAN1 #120 meeting, it has been agreed that the SSB adaptation should be indicated with DCI format 2_9 which can be updated to the description and components of FG 61-6. Proposal 4: Support to add using DCI format 2_9 to indicate the SSB adaptation in FG 61-6 And the feature is supported for NCD SSB and can be supported for different band, the feature should be per band. Proposal 5: Support FG 61-6 to be per band. Based on above proposals, we propose the initial table for UE feature of adaptation of SSB in time-domain as follows (the proposed parts are marked in red). 61. Netw_Energy_NR_enh; 61-6; SSB burst periodicity adaptation for SCell operation indicated by DCI format 2_9 with [cellDTRX-RNTI]; - Reception of DCI format 2_9 with [cellDTRX-RNTI] - Adaptation of SSB burst periodicity for SCell; Yes; UE does not support adaptation of SSB burst periodicity for SCell; FFS per band; FFS; FFS; FFS; Note: the SSB for this FG is not cell defining; Optional with capability signaling. |
| Ofinno [7] |  |
| Nokia [8] | The FG 61-6 should also indicate how the adaptation is done, this can be achieved by adding a component indicating support for DCI format 2_9. Proposal 5: Add a component to indicate support for DCI format 2_9 based indication of SSB adaptation to FG 61-6. 61-6; SSB burst periodicity adaptation for SCell operation; Adaptation of SSB burst periodicity for SCell 1. Support for DCI format 2_9 based signalling to indicate SSB burst periodicity adaptation on the cell; Note: the SSB for this FG is not cell defining; Optional with capability signaling. |
| CMCC [9] | Regarding the FFS part (i.e. wording in yellow highlight) in SSB adaptation, our considerations are as follows: - Regarding the FFS part on Component column, since it is agreed in last RAN1 meeting that DCI 2_9 can be used for SSB adaptation indication, we suggest to add the corresponding description. - Regarding the FFS part on Type column, we support per band which is similar as on-demand SSB SCell procedure. - Regarding the FFS part on Need of FDD/TDD differentiation, Need of FR1/FR2 differentiation and Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 column, we support it as N/A. Proposal 3: Take the following UE feature list for the feature of adaptation of common signal/channel transmissions: 61. Netw_Energy_NR_enh; 61-6; SSB burst periodicity adaptation for SCell operation; Adaptation of SSB burst periodicity for SCell 1. Configuration of SSB periodicities via higher layer signaling 2. DCI 2_9 based indication of SSB periodicity adaptation; Yes; UE does not support adaptation of SSB burst periodicity for SCell; Per band; n/a; n/a; n/a; Note: the SSB for this FG is not cell defining; Optional with capability signaling. |
| Huawei/HiSilicon [10] | For the reporting type of FG 61-6, we think per band reporting is sufficient. This in general aligns with many CA related features. Proposal 8: For adaptation of SSB in time-domain, set the reporting type as per band. Proposal 9: Update UE features as in Table 3 for adaptation of SSB in time-domain in Rel-19 NES. 61. Netw_Energy_NR_enh; 61-6; SSB burst periodicity adaptation for SCell operation; Adaptation of SSB burst periodicity for SCell; Yes; UE does not support adaptation of SSB burst periodicity for SCell; FFS[Per band]; FFSN/A; FFSN/A; FFSN/A; Note: the SSB for this FG is not cell defining; Optional with capability signaling. |
| OPPO [11] |  |
| Samsung [12] | Similar to on-demand SSB, since the feature of SSB adaptation in time domain is supported for SCell only, it would be more beneficial for UE implantation to make the FG type as “per band combination”. 61. Netw_Energy_NR_enh; 61-6; SSB burst periodicity adaptation for SCell operation for SCell indicated by DCI format 2_9 with [cellDTRX-RNTI]; Support of aAdaptation of SSB burst periodicity for SCell 1. Reception of configuration of X additional SSB burst periodicities for the SCell via higher layer signaling 2. Reception of indication of SSB burst periodicity by DCI format 2_9 with [cellDTRX-RNTI]; Yes; UE does not support adaptation of SSB burst periodicity for SCell indicated by DCI format 2_9 with [cellDTRX-RNTI]; FFS Per BC; FFS; FFS; FFS; Note: the SSB for this FG is not cell defining; Optional with capability signaling. |
| Xiaomi [13] | The following agreements were achieved in previous RAN1 meeting: Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’). For feature group related to SSB adaptation: - Periodicity of SSB is adapted according to gNB indication. - For PCell, SSB adaptation is not supported. - For SCell, adaptation for SSB that is not CD-SSB on or not on sync raster. - SSB adaptation is not supported for RRC IDLE/INACTIVE mode. - The frequency location is same before and after SSB adaptation. - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - there is no change to actually transmitted SSBs within a burst before and after SSB adaptation. - [SSB adaptation indication is carried by DCI format 1_0.] Observation 8: For feature group related to SSB adaptation, the following component can be considered as starting point: - Periodicity of SSB is adapted according to gNB indication. - For PCell, SSB adaptation is not supported. - For SCell, adaptation for SSB that is not CD-SSB on or not on sync raster. - SSB adaptation is not supported for RRC IDLE/INACTIVE mode. - The frequency location is same before and after SSB adaptation. - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - there is no change to actually transmitted SSBs within a burst before and after SSB adaptation. - SSB adaptation indication is carried by DCI format 2_9. 61. Netw_Energy_NR_enh; 61-6; SSB burst periodicity adaptation for SCell operation; 1. Support of adaptation of SSB burst periodicity for SCell via DCI format 2_9 2. Additional SSB for adaptation is not CD-SSB on or not on sync raster 3. The frequency location is same before and after SSB adaptation 4. There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. 5. There is no change to actually transmitted SSBs within a burst before and after SSB adaptation.; Yes; UE does not support adaptation of SSB burst periodicity for SCell; FFS; FFS; FFS; FFS; Note: the SSB for this FG is not cell defining; Optional with capability signaling. |
| LG Electronics [14] |  |
| Apple [15] |  |
| NTT DOCOMO, INC. [16] |  |
| Ericsson [17] | - Components - Update to add “…based on DCI format 2_9”. - Should be per band. - No Need of a) FDD/TDD differentiation, b) FR1/FR2 differentiation, c) Capability interpretation for mixture of FDD/TDD and/or FR1/FR2. 61. Netw_Energy_NR_enh; 61-6; SSB burst periodicity adaptation for SCell operation; Adaptation of SSB burst periodicity for SCell based on DCI format 2_9; Yes; UE does not support adaptation of SSB burst periodicity for SCell; per band; n/a; n/a; n/a; Note: the SSB for this FG is not cell defining; Optional with capability signaling. |
| Qualcomm Incorporated [18] | Proposal 3: For SSB burst periodicity adaptation, adopt the following updates to FG 61-6 61. Netw_Energy_NR_enh; 61-6; SSB burst periodicity adaptation for SCell operation; Adaptation of SSB burst periodicity for SCell by DCI format 2_9; Yes; UE does not support adaptation of SSB burst periodicity for SCell; FFS; FFS; FFS; FFS; Note: the SSB for this FG is not cell defining; Optional with capability signaling. |

## Adaptation of RACH transmissions

| 61. Netw_Energy_NR_enh | 61-7 | Adaptation of RACH in time domain based on additional RACH resources | Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources |  | FFS |  | UE does not support adaptation of RACH in time domain based on additional RACH resources | [Per band/Per UE] | FFS | FFS | FFS | [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| ZTE Corporation/Sanechips [2] | Separate UE feature for UE in idle/inactive mode and for UE in connected mode or not In our opinion, it is more reasonable to consider one single UE feature for both UE in idle/inactive mode and UE in connected mode. On the one hand, for UEs in idle/inactive/connected mode, all of them need monitor P-RNTI DCI with PRACH adaptation indication. On the other hand, some additional PRACH resources are shared by both UEs in idle/inactive mode and UEs in connected mode. If separate FGs are considered, the adaptation information may have different interpretations for UEs in idle/inactive mode and UEs in connected mode, respectively. Support a single UE feature group for PRACH adaptation in time domain for both UEs in idle/inactive mode and UEs in connected mode. As mentioned in 3.2, we believe that PRACH adaptation in time domain should be mandatory feature for Rel-19 UE, since the additional PRACH resources may be configured in SIB1. UE feature for adaptation of RACH in time domain is optional without capability signaling. 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources by P-RNTI DCI; Support of adaptation of RACH in time domain based on additional RACH resources in idle/inactive/connected mode 1. Configuration of additional PRACH resources via higher layer signaling 2. P-RNTI DCI-based indication of additional PRACH resources; FFS; UE does not support adaptation of RACH in time domain based on additional RACH resources; [Per band/] N/A; NO; NO; FFS; [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode; Optional [with/without] capability signaling. |
| Vivo [3] | Regarding [idle/inactive/connected] and FFS: separate row for connected mode, we support to have a common row since there seems to exist some problem if differentiation of them. For example, RO configured in RACHConfigcommon may also be used in both idle and connected mode. It seems strange for a UE supporting RACH adaptation in idle state only doesn’t support RACH adaptation when it becomes connected mode. Proposal 7: Keep Idle/inactive/conncected and remove FFS: separate row for connected mode in FG 61-7. 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources; Support of adaptation of RACH in time domain based on additional RACH resources in idle/inactive/connected mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources; UE does not support adaptation of RACH in time domain based on additional RACH resources; Per band; n/a; n/a; n/a; A UE that supports transmission of PRACH in additional RO supports this FG; Optional with capability signaling. |
| FUTUREWEI [4] | In the last meeting RAN1 adopted two signaling ways for RACH time adaptation. - Connected UEs: 1-bit field in DCI 1_0 with C-RNTI - Idle/Inactive UEs: DCI with P-RNTI Proposal 8: Add DCI 1_0 with C-RNTI and DCI 1_0 with P-RNTI as components of FG Adaptation of RACH in time domain based on additional RACH resources. In addition, it was agreed to optionally support a mask to select a subset of PRACH added resources. - Optional mask support semi-static signaling of additional resources. Because this is an optional support, RAN1 should consider whether to split the FG Adaptation of RACH in time domain based on additional RACH resources. Proposal 9: Consider splitting FG Adaptation of RACH in time domain based on additional RACH resources in two FGs, where the first FG does not support the optional mask and is pre-requisite for the second FG. 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources; Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI 1_0 with C-RNTI based indication of additional PRACH resources 3. DCI 1_0 with P-RNTI based indication of additional PRACH resources; FFS; UE does not support aadaptation of RACH in time domain based on additional RACH resources; [Per band/Per UE]; FFS; FFS; FFS; [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode; Optional [with/without] capability signaling. 61. Netw_Energy_NR_enh; 61-8; Adaptation of RACH in time domain based on additional RACH resources using mask support for semi-static signalling of additional resources; Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode Support mask support semi-static signaling of additional resources; FFS; UE does not support aadaptation of RACH in time domain based on additional RACH resources with mask support semi-static signalling of additional resources; [Per band/Per UE]; FFS; FFS; FFS; [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode; Optional [with/without] capability signaling. |
| CATT [5] | According to the agreement in RAN1#120, DCI 1_0 with P-RNTI will be introduced to indicate the additional PRACH resource is available or not. Therefore, there is no need to distinguish idle/inactive/connected mode and no need for UE capacity signaling. Proposal 6: Adopt the following for FG 61-7: - Support of adaptation of RACH in time domain based on additional RACH resources in idle/inactive/connected mode. - No need to introduce separate row for connected mode. - The type of granularity should be Per UE. 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources; Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources; FFS No; UE does not support adaptation of RACH in time domain based on additional RACH resources; [Per band/Per UE]. |
| China Telecom [6] | It has agreed that at least DCI format 1_0 with P-RNTI can be used to indicated the PRACH adaptation. Therefore, it is proposed to explicit introduce above component in FG 61-7 first, the details can be added later after the further discussion in AI 9.5.3. Proposal 6: Support to explicitly introduce the PRACH adaptation in time domain indicated by DCI format 0_1 with P-RNTI. Also, it is agreed to introduce PRACH mask mechanism for the PRACH adaptation in time domain for NES, which is a new feature besides the adaptation with additional PRACH resources. Regardless of the details of the PRACH mask, we think the FG for PRACH mask should be introduced first. Proposal 7: Support to introduce a new FG for PRACH mask. Based on above proposals, we propose the initial table for UE feature of adaptation of SSB in time-domain as follows (the proposed parts are marked in red). 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources; Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources via DCI format 1_0 with P-RNTI; FFS; UE does not support adaptation of RACH in time domain based on additional RACH resources; [Per band/Per UE]; FFS; FFS; FFS; [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode Note: Other DCI formats are not excluded currently.; Optional [with/without] capability signaling. 61. Netw_Energy_NR_enh; 61-7; Adaptation of PRACH with PRACH Mask; Support of adaptation of PRACH via PRACH mask; FFS; UE does not support adaptation of PRACH via PRACH mask; Per band; FFS; FFS; FFS; Optional with capability signaling. |
| Ofinno [7] |  |
| Nokia [8] | Concerning the adaptation of RACH in time domain based on additional RACH resources, we support the introduction of a separate row for connected mode, i.e., 61-7 should apply only to idle/inactive mode and connected mode should have a dedicated row. Proposal 6: Add a separate row, i.e., 61-8, related to the support of adaptation of RACH in time domain based on additional RACH resources for connected UEs. Proposal 7: Let 61-7 be related to the support of adaptation of RACH in time domain based on additional RACH resources for idle/inactive UEs only. We also see merit in discussing the possibility of having explicit capability signalling for at least the connected mode UEs. This would allow gNB to know whether a UE that did not perform access using an additional RO can support a Msg1 transmission using an additional RO, e.g., for CFRA. Explicit capability signaling for 61-7, assuming it pertains only to idle/inactive UEs, as per our proposal above, would also be desirable. This would allow gNB to have a fair estimate of how many UEs in a cell, on average, support Msg1 transmission over additional ROs at all times. Additional ROs configuration/activation could then be otpimized accordingly. Proposal 8: 61-7 and 61-8 are “optional with capability signalling” 61-7; Adaptation of RACH in time domain based on additional RACH resources for idle/inactive UEs.; Support of adaptation of RACH in time domain based on additional RACH resources in idle/inactive mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources; [A UE that supports transmission of PRACH in additional RO supports this FG]; Optional with capability signaling. 61-8; Adaptation of RACH in time domain based on additional RACH resources for connected UEs.; Support of adaptation of RACH in time domain based on additional RACH resources in connected mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources; [A UE that supports transmission of PRACH in additional RO supports this FG]; Optional with capability signaling. |
| CMCC [9] | Regarding the FFS part (i.e. wording in yellow highlight) in PRACH adaptation, our considerations are as follows: - Regarding the FFS part on Component column, since it is agreed in last RAN1 meeting that DCI 1_0 with C-RNTI to trigger PRACH can be used for PRACH adaptation indication for RRC_CONNECTED UE and it is agreed in last RAN2 meeting that MAC CE is introduced for PRACH adaptation for RRC_CONNECTED UE, we suggest to have separately description for each indication method, and to have separate row for DCI-based indication and MAC CE-based indication. - Regarding the FFS part on Type column, we support per band. - Regarding the FFS part on Need of FDD/TDD differentiation, Need of FR1/FR2 differentiation and Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 column, we support it as N/A. - Regarding the FFS part on Mandatory/Optional column, our view is that UE can not report the signaling if it does not support such capability, and the content here can be ‘Optional without capability signaling’, and the Need for the gNB to know if the feature is supported column can be set as No. Proposal 3: Take the following UE feature list for the feature of adaptation of common signal/channel transmissions: 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources with DCI; Support of adaptation of RACH in time domain based on additional RACH resources 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI 1_0 with P-RNTI based indication of additional PRACH resources 3. DCI 1_0 with C-RNTI based indication of additional PRACH resources in connected mode; No; UE does not support adaptation of RACH in time domain based on additional RACH resources with DCI; Per band; n/a; n/a; n/a; Optional without capability signaling. 61. Netw_Energy_NR_enh; 61-8; Adaptation of RACH in time domain based on additional RACH resources with MAC CE; Support of adaptation of RACH in time domain based on additional RACH resources in connected mode 1. Configuration of additional PRACH resources via higher layer signaling 2. MAC CE-based indication of additional PRACH resources in connected mode; No; UE does not support adaptation of RACH in time domain based on additional RACH resources with MAC CE; Per band; n/a; n/a; n/a; Optional without capability signaling. |
| Huawei/HiSilicon [10] | Based on the agreement of RAN1 #120 meeting, DCI-based adaptation for additional PRACH resources can also be used for connected mode UE in CFRA case, for which the details of the UE behaviour is still under discussion. Whether to set a separate UE capability for connected mode UE can be paused until the detailed UE behaviour is agreed. Even if the UE capability for connected mode UE is set as a separate one, for adaptation of RACH in idle/inactive mode, there is still need to report it. For the network, whether to configure and activate additional PRACH resources and detect these additional ROs depends on whether there are enough R19 NES UEs existing in the network. In our view, based on the UE capability reporting, network can get a good knowledge about the distributing of R19 NES UEs and legacy UEs, which can help the network to make the decision. So, we propose to report the UE capability with signalling. As for the reporting type, we think per UE reporting is sufficient as the concerned components are rather basic UE capabilities, including the relevant DCI indication (via either P-RNTI scrambled DCI or PDCCH order). As for the ‘Need of FDD/TDD differentiation’ and the ‘Need of FR1/FR2 differentiation’, we don’t see much difference from UE complexity point of view. Proposal 10: For adaptation of PRACH in time-domain, the reporting type is per UE and there is a need to differentiate between FR1 and FR2. 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources; Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources; FFS; UE does not support adaptation of RACH in time domain based on additional RACH resources; [Per band/Per UE]; FFSNo; FFSNo; FFSN/A; [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode; Optional [with/without] capability signaling. |
| OPPO [11] |  |
| Samsung [12] | Similar to on-demand SSB, since the feature of SSB adaptation in time domain is supported for SCell only, it would be more beneficial for UE implantation to make the FG type as “per band combination”. 61. Netw_Energy_NR_enh; 61-7; Adaptation of PRACH in time domain based on additional PRACH resources indicated by DCI format 1_0 with P-RNTI; Support of adaptation of PRACH in time domain based on additional PRACH resources in [idle/inactive/connected] mode 1. Reception of cConfiguration of additional PRACH resources via higher layer signaling 2. Reception of DCI-based indication of additional PRACH resources by DCI format 1_0 with P-RNTI; FFS; UE does not support adaptation of PRACH in time domain based on additional PRACH resources indicated by DCI format 1_0 with P-RNTI; [Per band/Per UE]; FFS; FFS; FFS; [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode; Optional [with/without] capability signaling. 61. Netw_Energy_NR_enh; 61-7a; Adaptation of PRACH in time domain based on additional RACH resources indicated by DCI format 1_0 with C-RNTI; Support of adaptation of PRACH in time domain based on additional RACH resources in connected mode 1. Reception of configuration of additional PRACH resources via higher layer signaling 2. Reception of indication of additional PRACH resources by DCI format 1_0 with C-RNTI; FFS; UE does not support adaptation of PRACH in time domain based on additional PRACH resources indicated by DCI format 1_0 with C-RNTI; [Per band/Per UE]; FFS; FFS; FFS; Optional with capability signaling. - Note: If in addition to P-RNTI other DCI formats and/or RNTIs are agreed for DCI-based indication, it is FFS whether they will be included in FG 61-7 or be captured in one or more additional FGs |
| Xiaomi [13] | The following agreements were achieved in previous RAN1 meeting: Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode. For paging transmission and reception, it is applicable to RRC CONNECTED mode and RRC IDLE/INACTIVE mode. Same as OD-SIB1 for IDLE/INACTIVE UE, the motivation of defining UE capability related to PRACH adaptation and paging adaptation for IDLE/INACTIVE mode needs to be further clarified. For feature group related to PRACH adaptation: - Time domain resources of additional PRACH is adapted according to adaptation indication carried by DCI format 1_0 with P-RNTI. - The same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources. - The PRACH configuration index for the additional PRACH resources is same as or different from the PRACH configuration index for the legacy resources. - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. This mapping is not impacted by time domain PRACH adaptation, which is applied to the following cases: - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains Observation 9: For feature group related to PRACH adaptation, the following component can be considered as starting point: - Time domain resources of additional PRACH is adapted according to adaptation indication carried by DCI format 1_0 with P-RNTI. - The PRACH configuration index for the additional PRACH resources is same as or different from the PRACH configuration index for the legacy resources. - The same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources. - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. This mapping is not impacted by time domain PRACH adaptation, which is applied to the following cases: - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources; Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources with DCI format 1_0 scrambled by P-RNTI 3. The PRACH configuration index for the additional PRACH resources is same as or different from the PRACH configuration index for the legacy resources. 4. The same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources. 5. SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs. 6. Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.; FFS; UE does not support adaptation of RACH in time domain based on additional RACH resources; [Per band/Per UE]; FFS; FFS; FFS; [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode; Optional [with/without] capability signaling. |
| LG Electronics [14] | Agreement (RAN1#120) For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) - FFS: Details on how to reuse existing bit for the 1-bit indication. In RAN1#120, it was agreed to support dynamic RO adaptation indication by PDCCH order DCI. In [2], there is a note that “If in addition to P-RNTI other DCI formats and/or RNTIs are agreed for DCI-based indication, it is FFS whether they will be included in FG 61-7 or be captured in one or more additional FGs”. For common design of PRACH adaptation in idle/inactive/connected mode UEs, it is preferred to not differentiate RO adaptation feature depending on idle/inactive/connected mode and depending on DCI format/RNTI. Proposal #2: For PRACH adaptation, FG 61-7 includes both P-RNTI DCI and PDCCH order DCI which can be used for adaptation of PRACH in time domain. |
| Apple [15] | 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources for IDLE/INACTIVE UEs; Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources; FFS; UE does not support adaptation of RACH in time domain based on additional RACH resources; [Per band/Per UE]; FFS; FFS; FFS; [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode; Optional [with/without] capability signaling. 61. Netw_Energy_NR_enh; 61-7a; Adaptation of RACH in time domain based on additional RACH resources for CONNECTED UEs; Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources; FFS; UE does not support adaptation of RACH in time domain based on additional RACH resources; [Per band]; FFS; FFS; FFS; Optional [with/without] capability signaling. |
| NTT DOCOMO, INC. [16] | On whether to report with signaling, we believe UE capabilities with signaling should be supported. One reason is that we believe the same logic as OD-SIB1, i.e., the capability signaling is useful for NW to decide whether to operate a cell as PRACH adaptive cell, will naturally applicable. Moreover, this feature may potentially be used for SCell operation, in which the capability signaling will definitely be used for NW to decide how the SCell operation should be. Regarding separation among RRC states and RNTIs, it would be clear and beneficial for both of UE and NW to define different FGs for different RNTIs and all RRC states to be supported in each RNTI. The background is RAN1 agreed to support C-RNTI for PRACH adaptation in RRC connected mode only. Different RNTIs would provide different use-cases, so separating FGs in the aspects is reasonable, while it is challenging for NW to manage adaptive PRACH resources indicated the DCI with P-RNTI for UEs in different RRC status if different FGs are defined. At the same time, we still think that adaptation via DCI with P-RNTI is more important and essential feature than C-RNTI, so the former one should be prerequisite for the latter one. This is because DCI with C-RNTI can be sent in a UE-specific manner and, in this case, dedicated preamble can be indicated for each UE even in the legacy specification, which means no strong motivation to provide additional ones. Proposal 3: Update the following FGs for adaptation of common signal and channel. 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources via DCI format 1_0 with CRC scrambled by P-RNTI; Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources via DCI format 1_0 with CRC scrambled by P-RNTI; FFS; UE does not support adaptation of RACH in time domain based on additional RACH resources; [Per band/Per UE]; FFS; FFS; FFS; [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode; Optional [with/without] capability signaling. 61. Netw_Energy_NR_enh; 61-8; Adaptation of RACH in time domain based on additional RACH resources via DCI format 1_0 with CRC scrambled by C-RNTI; Support of adaptation of RACH in time domain based on additional RACH resources in connected mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources via DCI format 1_0 with CRC scrambled by C-RNTI; 61-7; FFS; UE does not support adaptation of RACH in time domain based on additional RACH resources; [Per band/Per UE]; FFS; FFS; FFS; [A UE that supports transmission of PRACH in additional RO supports this FG]; Optional with capability signaling. |
| Ericsson [17] | - Components: - FG should be applicable to idle/inactive/connected mode. - Component 1: Update to add “..including support of PRACH mask to identify the subset of the additional PRACH resources”. - Component 2: Update to add “ with P-RNTI and C-RNTI” - (New) Component 3: Add “Support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH using additional PRACH resources”. - Should be per band. - No Need of a) FDD/TDD differentiation, b) FR1/FR2 differentiation, c) Capability interpretation for mixture of FDD/TDD and/or FR1/FR2. - Note column: OK to confirm the first part “A UE that supports transmission of PRACH in additional RO supports this FG” although it may not be necessary. - Remove 2nd FFS (not necessary to have separate row for connected mode). - Should be optional with UE capability signaling. 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources; Support of adaptation of RACH in time domain based on additional RACH resources in idle/inactive/connected mode 1. Configuration of additional PRACH resources via higher layer signaling, including support of PRACH mask to identify the subset of the additional PRACH resources 2. DCI-based indication of additional PRACH resources with P-RNTI and with C-RNTI 3. Support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH using additional PRACH resources; Yes; UE does not support adaptation of RACH in time domain based on additional RACH resources; per band; n/a; n/a; n/a; A UE that supports transmission of PRACH in additional RO supports this FG; Optional with capability signaling. |
| Qualcomm Incorporated [18] | RAN1#120 made the following agreement related to support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. Agreement For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) - FFS: Details on how to reuse existing bit for the 1-bit indication. From our perspectives, FG 61-7 is to indicate support of adaptation of RACH in time domain based on additional RACH resources by DCI 1_0 with CRC scrambled by P-RNTI. Furthermore, we propose to introduce a new FG 61-8 on support of adaptation of RACH in time domain based on additional RACH resources for PDCCH ordered RACH in connected mode. Proposal 4: For PRACH adaptation in time domain, - Adopt the following updates to FG 61-7 - Introduce a new FG 61-8 for support of PRACH adaptation with additional PRACH resource triggered by DCI 1_0 with CRC scrambled by C-RNTI 61. Netw_Energy_NR_enh; 61-7; Adaptation of RACH in time domain based on additional RACH resources by DCI 1_0 with CRC scrambled by P-RNTI; Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources by DCI 1_0 with CRC scrambled by P-RNTI; FFS; UE does not support adaptation of RACH in time domain based on additional RACH resources by DCI 1_0 with CRC scrambled by P-RNTI; [Per band/Per UE]; FFS; FFS; FFS; [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode; Optional [with/without] capability signaling. 61. Netw_Energy_NR_enh; 61-8; Adaptation of RACH in time domain based on additional RACH resources by DCI 1_0 with C-RNTI triggering PRACH in connected mode; Support of adaptation of RACH in time domain based on additional RACH resources for PDCCH ordered RACH in connected mode 1. Configuration of additional PRACH resources via higher layer signaling 2. DCI-based indication of additional PRACH resources by DCI 1_0 with C-RNTI triggering PRACH; FFS; UE does not support adaptation of RACH in time domain based on additional RACH resources by DCI 1_0 with C-RNTI triggering PRACH; [Per band/Per UE]; FFS; FFS; FFS; Optional with capability signaling. |

## Others

| Company | Summary |
| --- | --- |
| ZTE Corporation/Sanechips [2] | In RAN1#120 meeting, there was an agreement on C-RNTI DCI based adaptation of RACH in time domain [3]. Agreement For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) - FFS: Details on how to reuse existing bit for the 1-bit indication. Since C-RNTI DCI is monitored only by UEs in connected mode, separate FG for C-RNTI DCI based adaptation of RACH in time domain should be considered. Moreover, the details for this feature will be updated based on the conclusion/agreement in 9.5.3. Add a new FG (i.e., 61-8) for C-RNTI DCI based adaptation of RACH in time domain 61. Netw_Energy_NR_enh; 61-8; Adaptation of RACH in time domain based on additional RACH resources by C-RNTI DCI; 1.Support of adaptation of RACH in time domain based on additional RACH resources in connected mode 2.Configuration of additional PRACH resources via higher layer signaling 3. C-RNTI DCI-based indication of additional PRACH resources; UE does not support adaptation of RACH in time domain based on additional RACH resources in connected mode; N/A; NO; NO; Optional with capability signaling. |
| Vivo [3] |  |
| FUTUREWEI [4] |  |
| CATT [5] |  |
| China Telecom [6] | Besides, the OD-SSB should only be transmitted in a period of time, if the deactivation and activation are not supported at the same time, the FG is not complete. However, there is no need to restrict that the OD-SSB is activated and deactivated with same kind of signalling. For instance, the RRC indicated OD-SSB can be deactivated with MAC CE signalling, and the MAC CE indicated OD-SSB can be deactivated after N times transmission, which can be configured in the RRC parameters. Thus, we think the deactivation of OD-SSB should be separate FGs. And there is also no needs to distinguish it is for Case #1 or Case #2 Proposal 2：Support to introduce additional FGs for the deactivation of OD-SSB SCell Operation. 61. Netw_Energy_NR_enh; 61-a; On-demand SSB SCell operation deactivated by configured/indicated N of OD-SSB bursts to be transmitted after on demand SSB is indicated; - UE support on-demand SSB SCell deactivated via configuration of number N of OD-SSB to be transmitted after OD-SSB is indicated - Value of N; At least one of 61-1/2/3/4; Yes; UE does not support on-demand SSB SCell deactivated via configuration of number N of OD-SSB to be transmitted after OD-SSB is indicated; Per band; Optional with capability signaling. 61. Netw_Energy_NR_enh; 61-b; On-demand SSB SCell operation deactivated by via MAC-CE for on-demand SSB transmission indication; - UE support explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication; At least one of 61-1/2/3/4; Yes; UE does not support explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication; Per band; Optional with capability signaling. |
| Ofinno [7] | Proposal 5: Add a UE feature for the adaptation of SSB periodicity with the following components: - Support of adaptation of SSB periodicity in RRC_CONNECTED state for PCell. - Support for the maximum number of SSB periodicities for SSB periodicity adaptation . - Support DCI based signalling for adaption of SSB periodicity. 61-X; Adaptation of SSB periodicity; 1) Support of adaptation of SSB periodicity in RRC_CONNECTED state for PCell; 2) Support for the maximum number of SSB periodicities for SSB periodicity adaptation; 3) Support DCI based signalling for adaption of SSB periodicity.; FFS; UE does not support adaptation of SSB periodicity; [Per band/Per UE]; FFS; FFS; FFS; Optional [with/without] capability signaling. |
| Nokia [8] |  |
| CMCC [9] |  |
| Huawei/HiSilicon [10] |  |
| OPPO [11] |  |
| Samsung [12] |  |
| Xiaomi [13] |  |
| LG Electronics [14] |  |
| Apple [15] |  |
| NTT DOCOMO, INC. [16] |  |
| Ericsson [17] |  |
| Qualcomm Incorporated [18] |  |

# Discussion Items during RAN1 #120bis

After review of contributions submitted to RAN1 #120bis in this agenda item, the following topics were identified by the moderator for discussion during RAN1 #120bis.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

## On-demand SSB SCell operation

After review of contributions submitted to RAN1 #120bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-1 | On-demand SSB SCell operation indicated by RRC based signaling in Case #1 | 1. Support RRC based signalling to indicate on-demand SSB transmission SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell) 2. Support L1 measurement based on on-demand SSB 3. Support L3 measurement based on on-demand SSB | FFS 18-5 from RAN2 (Direct NR MCG SCell activation) or 18-6 from RAN2 (Direct NR SCG SCell activation) | Yes |  | UE does not support on-demand SSB transmission SCell [indication/deactivation] indicated by RRC based signaling in Case #1 | [Per band] | n/a | n/a | n/a | FFS: supported deactivation mechanisms Note: RRC based deactivation mechanism is supported. If UE additionally support MAC-CE (one of FG 61-3, 61-4), UE also supports MAC CE based deactivation mechanism to deactivate the on-demand SSB Note: The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms} Note: If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | RAN1 agreed that deactivation by RRC is decided by RAN2, as shown below. Agreement (RAN1#119) For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer. So, we propose the following modification for the note in the second last column. Note: RRC based deactivation mechanism is supported. If UE additionally supports MAC-CE (one of FG 61-3, 61-4), UE also supports MAC CE based deactivation mechanism to deactivate the on-demand SSB |
| Qualcomm | For the components, we suggest to remove components 2/3 in all FGs since the support should be specified in RAN1 specs. On the note column, we would like to keep FFS on deactivation and remove the first note since more RAN1 progress is needed. The 2nd note should belong to RRC parameter sheet. |
| DCM | Comments on the components for OD-SSB operation We suggest to conbine RRC based indication and MAC CE based indication/deactivation mechanism for OD-SSB operation as follows - Support RRC based signalling to indicate on-demand SSB activation on the Scell in Case #1 (No always-on SSB on the cell) - Support MAC CE based signalling to indicate on-demand SSB activation and deactivation on the Scell in Case #1 (No always-on SSB on the cell) - Support implicit deactivation of on-demand SSB And we believe that RRC based indication and MAC CE indication should be within one FG. Supporting only RRC indication cannot work well according to the current agreement (OD-SSB RRC indication can only be used together with SCell configuration/activation (direct SCell activation)). After a UE transitions from an activated SCell to a deactivated SCell (e.g. after receiving a SCell deactivation command while the UE has a SCell configuration), the UE should be able to indicate OD-SSB activation or SCell activation independently via MAC CE during scenario2. Comments on the Notes - For the first Notes in the proposal, RRC based deactivation mechanism has not been supported yet.(it is up to RAN2 decision as agreed in the previous RAN1 meeting) Thus, in our understanding, the first Note is inaccurate and should be removed. The same discussions can also be made in 61-2, 61-2a. - In addition, regarding second Note, we do not need to capture as it will be captured in the spec if needed. |
| Ericsson1 | Generally OK with FL proposal– LG suggestion is also fine, but also suggest to a note “Note : RAN2 can update FG further e.g. Deacitvation by RRC is up to RAN2”. The last two notes are not necessary – they would be part of normative specification. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-2 | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency] | 1. Support RRC based signalling to indicate on-demand SSB SSB transmission SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency] 2. Support L1 measurement based on on-demand SSB 3. Support L3 measurement based on on-demand SSB | FFS 18-5 from RAN2 (Direct NR MCG SCell activation) or 18-6 from RAN2 (Direct NR SCG SCell activation) | Yes |  | UE does not support on-demand SSB SSB transmission SCell [indication/deactivation] indicated by RRC based signaling in Case #2 [for same center frequency] | [Per band] | n/a | n/a | n/a | FFS: supported deactivation mechanisms Note: RRC based deactivation mechanism is supported. If UE additionally support MAC-CE (one of FG 61-3, 61-4), UE also supports MAC CE based deactivation mechanism to deactivate the on-demand SSB Note: RRC based signaling to indicate on-demand SSB SCell [indication/deactivation] on the cell is applicable to [scenario#2 and scenario#2A] Note: The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms}. If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB. Note: If always on SSB is CD-SSB on synchronization raster, the frequency location of always on SSB and additional SSB is different. Otherwise, the frequency location of always on SSB and additional SSB can be different. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61. Netw_Energy_NR_enh | 61-2a | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequency | 1. Support RRC based signalling to indicate on-demand SSB SSB transmission on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same different frequency 2. Support L1 measurement based on on-demand SSB 3. Support L3 measurement based on on-demand SSB | 18-5 from RAN2 (Direct NR MCG SCell activation) or 18-6 from RAN2 (Direct NR SCG SCell activation) | Yes |  | UE does not support on-demand SSB SSB transmission indicated by RRC based signaling in Case #2 for same different frequency | Per band | n/a | n/a | n/a | Note: RRC based deactivation mechanism is supported. If UE additionally support MAC-CE (one of FG 61-3, 61-4), UE also supports MAC CE based deactivation mechanism to deactivate the on-demand SSB Note: RRC based signaling to indicate on-demand SSB SCell [indication/deactivation] on the cell is applicable to [scenario#2 and scenario#2A] Note: The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms}. If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB. Note: If always on SSB is CD-SSB on synchronization raster, the frequency location of always on SSB and additional SSB is different. Otherwise, the frequency location of always on SSB and additional SSB can be different. | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | Commonly for FG 61-2 and FG 61-2a, the following modifications are suggested. Note: RRC based deactivation mechanism is supported. If UE additionally supports MAC-CE (one of FG 61-3, 61-4), UE also supports MAC CE based deactivation mechanism to deactivate the on-demand SSB Note: If always on SSB is CD-SSB on synchronization raster, the frequency location of always on SSB and on-demand additional SSB is different. Otherwise, the frequency location of always on SSB and additional SSB is same can be different. For FG 61-2a, the following modifications are suggested. On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequency; 1. Support RRC based signalling to indicate on-demand SSB SSB transmission on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same different center frequency between always-on SSB and on-demand SSB 2. Support L1 measurement based on on-demand SSB 3. Support L3 measurement based on on-demand SSB; 18-5 from RAN2 (Direct NR MCG SCell activation) or 18-6 from RAN2 (Direct NR SCG SCell activation); Yes; UE does not support on-demand SSB SSB transmission indicated by RRC based signaling in Case #2 for same different center frequency. |
| Qualcomm | FG 61-2 - For the same center frequency, we have two following cases agreed in RAN1 - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. Our proposal was to have two separate FGs for these two cases. However, if companies would like to have a single FG for this same center frequency case, we propose to add a new component - New component: Supported time domain relation between on-demand SSB and always-on SSB - Candidate value of this component = {Time-C1, Time-C2, Time-C1nC2} - Add the following notes for this component: - Time-C1: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Time-C2: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - Time-C1nC2 includes both Time-C1 and Time-C2 - We should keep FFS on deactivation for further RAN1 progress - The first and last notes should be removed. The 3rd note belongs to RRC parameter spreadsheet. The 2nd note can be captured in RAN1 spec. FG 62-2a - Same comments on the notes as being provided in FG 61-2. In addition, we can have FFS on deactivation methods |
| DCM | All notes are not needed, at least, at this stage. RAN1 can further add clarification description later on according to RAN1 agreement and PHY spec description. - For the first Notes in the proposal, RRC based deactivation mechanism has not been supported yet. Also, MAC-CE based deactivation mechanism has agreed to be supported in RAN1#119 meeting. Thus, the first Note is inaccurate and should be removed. The same discussions can also be made in 61-2, 61-2a. - For the second bullet, which scenario the OD-SSB transmission indication will be used with RRC based indication has not been decided. It is too early to put on the notes about which scenario to be used. - In addition, regarding third Note, we do not need to capture as it will be captured in the spec. For the fourth bullet in the note, the motivation to put the statement is unclear. We should eliminate this note. |
| Ericsson1 | We prefer to keep to add a new component in the existing FG instead. The UE can report whether it supports same and different center frequency. “Supported relation between center frequency locations of on-demand SSB and always-on SSB”. Candidate value set {‘same-only’, ‘same or different’}. The entire text in second note can be made FFS (yellow). The last two notes are not necessary – they would be part of normative specification. |
| Qualcomm | As we discussed during online discussion, the current proposals for FG 61-2 and FG 61-2A are not agreeable for us. We made a suggested proposal for further discussion after this feedback table. The change compared to the version discussed online is in green. The same change principles should be applied to FG 61-4 and FG 61-4A. |
| Ericsson2 | We would be OK with the direction suggested by Qualcomm, but suggest below updates. Pre-requisite should be left as FFS for now. FG 61-2 (of Proposal (by Qualcomm):)-E/// update Component name : Type A On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency] with periodic time domain SSB pattern during OD-SSB transmission Component 1 : 1. Support RRC based signalling to indicate on-demand SSB SSB transmission SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for the following case [for same [center] frequency] with periodic time domain SSB pattern during OD-SSB transmission - Same center frequency between always-on SSB and on-demand SSB and the union of always-on SSB transmission and on-demand SSB transmission has a periodic time domain pattern during on-demand SSB transmission (the interval between SSB bursts is even and supported in legacy specification). Pre-requisite: FFS 18-5 from RAN2 (Direct NR MCG SCell activation) or 18-6 from RAN2 (Direct NR SCG SCell activation) Consequences : UE does not support Type A on-demand SSB SSB transmission SCell [indication/deactivation] indicated by RRC based signaling in Case #2 [for same center frequency] in which the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern during OD-SSB transmission (the interval between SSB bursts is even and supported in legacy specification) FG 61-2a (of Proposal (by Qualcomm):) -E/// update Component name : Type B On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency] with periodic time domain SSB pattern during OD-SSB transmission Component 1 : 1. Support RRC based signalling to indicate on-demand SSB SSB transmission on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for the following cases different center frequency between always-on SSB and on-demand SSB or in Case #2 for the same center frequency with non-periodic time domain SSB pattern during OD-SSB transmission - different center frequency between always-on SSB and on-demand SSB - for the same center frequency between always-on SSB and on-demand SSB and with non-periodic time domain SSB pattern during OD-SSB transmission Consequences : UE does not support Type B on-demand SSB SSB transmission indicated by RRC based signaling in Case #2 for different center frequency or in Case #2 for same center frequency in which the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern during OD-SSB transmission |
| Huawei, HiSilicon | - Ok with spelling out the Time-C1 and Time-C2. However, we would assume if a UE support Time-C2 then Time-C1 should be supported by default. Therefore, no need to have Time-C1nC2. Regarding E//’s update, not clear on the reason that the following two falling into the same FG/component - different center frequency between always-on SSB and on-demand SSB - for the same center frequency between always-on SSB and on-demand SSB and with non-periodic time domain SSB pattern during OD-SSB transmission - None of the Notes are necessary. |
| Ericsson3 | Responding to Huawei/HiSi, our original preference was one FG for same and different frequency case (UE reports support via candidate values). However, 61-2 case is straight-forward and could be a separate and basic FG. For remaining two cases, we prefer a single FG, and are open to discuss UE reporting support via candidate values. In any case, the case description for the components needs refinement first. |

Proposal (by Qualcomm): Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-2 | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 [for same center frequency] with periodic time domain SSB pattern during OD-SSB transmission | 1. Support RRC based signalling to indicate on-demand SSB SSB transmission SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency] with periodic time domain SSB pattern during OD-SSB transmission | FFS 18-5 from RAN2 (Direct NR MCG SCell activation) or 18-6 from RAN2 (Direct NR SCG SCell activation) | Yes |  | UE does not support on-demand SSB SSB transmission SCell [indication/deactivation] indicated by RRC based signaling in Case #2 [for same center frequency] in which the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern during OD-SSB transmission (the interval between SSB bursts is even and supported in legacy specification) | [Per band] | n/a | n/a | n/a | FFS: supported deactivation mechanisms Note: If UE additionally supports MAC-CE (one of FG 61-3, 61-4), UE supports MAC CE based deactivation mechanism to deactivate the on-demand SSB | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61. Netw_Energy_NR_enh | 61-2a | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequency or in Case #2 for the same center frequency with non-periodic time domain SSB pattern during OD-SSB transmission | 1. Support RRC based signalling to indicate on-demand SSB SSB transmission on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency between always-on SSB and on-demand SSB or in Case #2 for the same center frequency with non-periodic time domain SSB pattern during OD-SSB transmission | 61-2 18-5 from RAN2 (Direct NR MCG SCell activation) or 18-6 from RAN2 (Direct NR SCG SCell activation) | Yes |  | UE does not support on-demand SSB SSB transmission indicated by RRC based signaling in Case #2 for different center frequency or in Case #2 for same center frequency in which the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern during OD-SSB transmission | Per band | n/a | n/a | n/a | FFS: supported deactivation mechanisms Note: If UE additionally supports MAC-CE (one of FG 61-3, 61-4), UE supports MAC CE based deactivation mechanism to deactivate the on-demand SSB | Optional with capability signaling |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-3 | On-demand SSB SCell operation indicated via MAC CE in Case #1 | 1. Support MAC CE based signalling to indicate on-demand SSB transmission SCell [indication/deactivation] on the cell in Case #1 (No always-on SSB on the cell) 2. Supported deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx). 3. Support L1 measurement based on on-demand SSB 4. Support L3 measurement based on on-demand SSB | FFS | Yes |  | UE does not support on-demand SSB transmission SCell [indication/deactivation] indicated via MAC CE in Case #1 | [Per band] | n/a | n/a | n/a | FFS: supported deactivation mechanisms FFS: whether to merge with 61-1 Note: The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms}. Note: If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | Same comments on components 3 & 4 as providided in FG 61-1/2/2a. For component 2, we can discuss it later since more RAN 1 progress is needed since it is not clear on how UE knows whether explicit or implicit deactivation is supported. |
| DCM | FG 61-1 (RRC) and 61-3 (MAC CE) should be merged as discussed in the comment for FG 61-1. Third and fourth notes should be removed, at least, at this stage. RAN1 can further add clarification description later on according to RAN1 agreement and PHY spec description. For the deactivation via number N of OD-SSB bursts, the parameter name has not been decided yet, so (i.e., od-ssb-nrofTx) is not necessary and more general wording is preferable. |
| Ericsson1 | OK with FL proposal except the notes - these are not necessary – these would be part of normative specification. |
| Huawei, HiSilicon | None of the Notes are necessary. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-4 | On-demand SSB SCell operation indicated via MAC CE in Case #2 [for same center frequency] | 1. Support MAC CE based signalling to indicate on-demand SSB transmission SCell [indication/deactivation] on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) [for same [center] frequency] 2. Supported deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) 3. Support L1 measurement based on on-demand SSB 4. Support L3 measurement based on on-demand SSB | FFS | Yes |  | UE does not support on-demand SSB transmission SCell [indication/deactivation] indicated via MAC CE in Case #2 [for same center frequency] | [Per band] | n/a | n/a | n/a | FFS: supported deactivation mechanisms FFS: whether to merge with 61-2 Note: MAC CE based signaling to indicate on-demand SSB SCell [indication/deactivation] on the cell is applicable to [scenario#2 and scenario#2A] Note: The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms} Note: If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61. Netw_Energy_NR_enh | 61-4a | On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequency | 1. Support MAC CE based signalling to indicate on-demand SSB transmission on the cell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency 2. Supported deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) 3. Support L1 measurement based on on-demand SSB 4. Support L3 measurement based on on-demand SSB | FFS | Yes |  | UE does not support on-demand SSB transmission indicated via MAC CE in Case #2 for different center frequency | Per band | n/a | n/a | n/a | Note: MAC CE based signaling to indicate on-demand SSB SCell [indication/deactivation] on the cell is applicable to [scenario#2 and scenario#2A] Note: The periodicity of on demand SSB is selected from values of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms} Note: If always on SSB is assumed, the periodicity of on-demand SSB is equal or smaller than that of always on SSB | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | The following note can be added, similar to FG 61-2/61-2a. Note: If always on SSB is CD-SSB on synchronization raster, the frequency location of always on SSB and on-demand additional SSB is different. Otherwise, the frequency location of always on SSB and additional SSB is same can be different. |
| Qualcomm | FG 61-4 - For the same center frequency, we have two following cases agreed in RAN1 - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. Our proposal was to have two separate FGs for these two cases. However, if companies would like to have a single FG for this same center frequency case, we propose to add a new component - New component: Supported time domain relation between on-demand SSB and always-on SSB - Candidate value of this component = {Time-C1, Time-C2, Time-C1nC2} - Add the following notes for this component: - Time-C1: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Time-C2: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - Time-C1nC2 includes both Time-C1 and Time-C2 - We should keep FFS on deactivation for further RAN1 progress - The first should be removed. The 2nd note belongs to RRC parameter spreadsheet. Remove “If always on SSB is assumed,” in the last note FG 62-4a Same comments as being provided in FG 61-3. In addition, we can have FFS on deactivation methods |
| DCM | FG 61-2/2a (RRC) and 61-4/61-4a (MAC CE) should be merged as discussed in the comment for FG 61-1. All notes (except for “whether to merge with 61-2) should be removed, at least, at this stage. RAN1 can further add clarification description later on according to RAN1 agreement and PHY spec description. On separation of 61-4 and 61-4a, we are OK for UE to report either of {same frequency b/w ao and od-ssb, different}, at the same time, regarindg FG structure, it is better to define the separation as candidate value (e.g., {same, different, both}) in one FG for easier maintenance and avoding unsessesary duplicated descriptions. For the deactivation via number N of OD-SSB bursts, the parameter name has not been decided yet, so (i.e., od-ssb-nrofTx) is not necessary and more general wording is preferable. |
| Ericsson1 | We prefer to keep to add a new component in the existing FG instead. The UE can report whether it supports same and different center frequency. “Supported relation between center frequency locations of on-demand SSB and always-on SSB”. Candidate value set {‘same-only’, ‘same or different’}. The entire text in first note can be made FFS (yellow). The last two notes are not necessary – these would be part of normative specification. |
| Huawei, HiSilicon | None of the Notes are necessary. |

## On-demand SIB1 for idle/inactive mode UEs

After review of contributions submitted to RAN1 #120bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-5 | SIB1 request [for idle/inactive UEs] | 1. Reception of [RACH] SIB1 request configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request |  | No |  |  | FFS N/A | FFS N/A | FFS N/A | FFS N/A | [A UE indicates support of this FG if it transmits a wake-up signal on the uplink to SIB1 request SIB1] Note: whether to merge SIB1 request for connected UEs is up to RAN2 | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson1 | OK with FL proposal. |

## Adaptation of SSB transmissions

After review of contributions submitted to RAN1 #120bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-6 | SSB burst periodicity adaptation for SCell operation | Support of adaptation of SSB burst periodicity for SCell by DCI format 2_9 |  | Yes |  | UE does not support adaptation of SSB burst periodicity for SCell | FFS Per band | FFS N/A | FFS N/A | FFS N/A | Note: the SSB for this FG is not cell defining Note: Additional SSB for adaptation is not CD-SSB on or not on sync raster Note: The frequency location is same before and after SSB adaptation Note: There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. Note: There is no change to actually transmitted SSBs within a burst before and after SSB adaptation. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’). According to the above agreement, the following modification is needed. Note: Additional SSB for adaptation is not CD-SSB on or not on sync raster |
| Ericsson1 | First note in red – not clear what is meant by “additional SSB”. The note is not needed since there is existing note “Note: the SSB for this FG is not cell defining”. The last three notes in red are not necessary – these would be part of normative specification. |

## Adaptation of RACH transmissions

After review of contributions submitted to RAN1 #120bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-7 | Adaptation of RACH in time domain based on additional RACH resources by DCI format 1_0 with P-RNTI | 1. Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive[/connected] mode 2. Configuration of additional PRACH resources via higher layer signaling 3. DCI-based indication of additional PRACH resources by DCI format 1_0 with P-RNTI |  | FFS No |  | UE does not support adaptation of RACH in time domain based on additional RACH resources by DCI format 1_0 with P-RNTI | [Per band/Per UE] N/A | FFS N/A | FFS N/A | FFS N/A | [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61. Netw_Energy_NR_enh | 61-7 | Adaptation of RACH in time domain based on additional RACH resources by DCI format 1_0 with C-RNTI | 1. Support of adaptation of RACH in time domain based on additional RACH resources in connected mode 2. Configuration of additional PRACH resources via higher layer signaling 3. DCI-based indication of additional PRACH resources by DCI format 1_0 with C-RNTI |  | Yes |  | UE does not support adaptation of RACH in time domain based on additional RACH resources by DCI format 1_0 with C-RNTI | Per band | N/A | N/A | N/A |  | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | Our preference is to merge the above two FGs in a single FG. |
| DCM | If we go with separating different FGs depending on which RNTI to be used, then our preference is that 61-7 (DCI format 1_0 with P-RNTI) should be prerequisite. For the component of DCI format 1_0 with P-RNTI, the PRACH adaptation should be applicable for idle/inactive/connected mode UE, so the brackets regarding the UE mode should be removed. |
| Nokia | We support the presence of the two FGs. On the other hand, we highlight the relevance of having “Optional with capability signaling” for both FGs. While we acknowledge that the use case is different, and that the case of DCI format 1_0 with C-RNTI is evident, we also note that without capability signalling NW would not be able to learn how many UEs in the cell on average supports PRACH adaptation for NES, irrespective of which DCI 1_0 is used to indicate availability. This would prevent NW to optimize the configuration of additional PRACH resources accordingly, which seems a very relevant aspect to consider to maximize the NES gain and/or reduce impact to NW UL scheduler. Indeed, if this capability is not signalled, then NW would only be aware of how many UEs in the cells supports PRACH adaptation through DCI 1_0 with C-RNTI. However, nothing will be known for sure with respect to: (i) how many such UEs also support PRACH adaptation with DCI format 1_0 with P-RNTI, (ii) whether there are some UEs in the cell that do not support PRACH adaptation via DCI 1_0 with C-RNTI but support PRACH adaptation via DCI 1_0 with P-RNTI. This would be very sub-optimal, thus we propose to have “Optional with capability signaling” for both FGs (this does not impact UE complexity and helps NW…). |
| Ericsson1 | Do not support. FG 61-7 shoud be “Optional with capability signaling”. No need to introduce additional FG for C-RNTI. |

## Others

After review of contributions submitted to RAN1 #120bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 61. Netw_Energy_NR_enh | 61-8 | Adaptation of RACH in time domain based on additional RACH resources using mask support for semi-static signalling of additional resources | Support of adaptation of RACH in time domain based on additional RACH resources in [idle/inactive/connected] mode Support mask support semi-static signaling of additional resources |  | FFS |  | UE does not support aadaptation of RACH in time domain based on additional RACH resources with mask support semi-static signalling of additional resources | [Per band/Per UE] | FFS | FFS | FFS | [A UE that supports transmission of PRACH in additional RO supports this FG] FFS: separate row for connected mode | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| DCM | Our original thought was not to need explicit description of the mask for additional PRACH. If such description is really needed, we think it is enough to define it as a component in FG61-7 and FG61-8. Otherwise, NW has to allocate additional PRACH resources without mask considering the presence of UEs not supporting mask, which might increase Network energy. |
| Nokia | The mask would always be provided to the UE semi-statically together with the PRACH configuration index/indices in SIB1. One of the reasons why we agreed that such semi-static signalling is used is because RAN1 wanted to simply UE’s operations and prevent any sort of dynamic modification indicated through DCI. For this reason, we do not see the necessity of having this FG. |
| Ericsson1 | Do not support. Support of PRACH mask to identify subset of additional PRACH resources should be component of existing 61-7. New FG creates unnecessary fragmentation and reduces NES potential. The mask options under discussion in RAN1 are simply selection at PRACH association period-level (either a PRACH mask index/bitmap) - we do not see the need to introduce new capability for it. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 61. Netw_Energy_NR_enh | 61-8 | Adaptation of SSB periodicity | 1) Support of adaptation of SSB periodicity in RRC_CONNECTED state for PCell 2) Support for the maximum number of SSB periodicities for SSB periodicity adaptation 3) Support DCI based signalling for adaption of SSB periodicity |  | FFS |  | UE does not support adaptation of SSB periodicity | [Per band/Per UE] | FFS | FFS | FFS |  | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | This doesn’t seem to be necessary since we already have FG 61-6 for SSB periodicity adaptation. |
| DCM | Based on the agreement made so far, the operation of SSB adaptation in PCell is not assumed. This proposal does not alilgn to the previous agreement, so it should be out of scope. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 61. Netw_Energy_NR_enh | 61-8 | Adaptation of RACH in time domain based on additional RACH resources with MAC CE | Support of adaptation of RACH in time domain based on additional RACH resources in connected mode 1. Configuration of additional PRACH resources via higher layer signaling 2. MAC CE-based indication of additional PRACH resources in connected mode |  | No |  | UE does not support adaptation of RACH in time domain based on additional RACH resources with MAC CE | Per band | n/a | n/a | n/a |  | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | We can wait until further RAN1 agreement for MAC CE based RACH adaptation is made. |
| DCM | MAC CE based adaptation of PRACH has not been agreed, so it should avoid discussion unless anything related will be agreed. |
| Nokia | Same view as LG Electronics and DCM. |

# Conclusion

Agreements reached during RAN1 #120bis as part of this agenda item are summarized in [19].

# References

- R1-2501397, Initial RAN1 UE features list for Rel-19 NR after RAN1 #120, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2501770, Discussion on NES features, ZTE Corporation/Sanechips

- R1-2501832, Discussions on UE features for enhancements of network energy savings for NR, vivo

- R1-2501932, Discussion of Rel-19 NES UE Features, FUTUREWEI

- R1-2501982, Discussion on UE features for Rel-19 NES, CATT

- R1-2502035, Discussion on UE features for Rel-19 Network Energy Saving, China Telecom

- R1-2502050, UE features for enhancements of network energy savings for NR, Ofinno

- R1-2502136, Views on Network Energy Saving Enhancement UE features, Nokia

- R1-2502181, Discussion on UE features for enhancements of network energy savings for NR, CMCC

- R1-2502232, UE features for Rel-19 NES, Huawei/HiSilicon

- R1-2502264, Discussion on UE features for enhancements of network energy savings for NR, OPPO

- R1-2502394, UE features for enhancements of network energy savings for NR, Samsung

- R1-2502462, UE features for NES enhancements, Xiaomi

- R1-2502486, Discussion on UE features for enhancements of NES, LG Electronics

- R1-2502642, Views on UE features for Rel-19 NES, Apple

- R1-2502788, Discussion on UE features for eNES, NTT DOCOMO, INC.

- R1-2502808, UE features for R19 NES, Ericsson

- R1-2502867, UE features for Rel-19 NES, Qualcomm Incorporated

- R1-2502961, Session Notes of AI 9.15.4, Ad-Hoc Chair (NTT DOCOMO, INC.)