3GPP TSG RAN WG1 #120bis	                                   R1-2502963

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.13

Agenda Item:		9.15.13

Document for: 	Endorsement

- 9.15.13	UE features for LTE based 5G broadcast Phase 2

For RAN1#120bis, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

R1-2502742	Summary of UE features for LTE based 5G broadcast Phase 2	Moderator (AT&T)

Proposal: Introduce the following Rel. 19 UE feature (yellow highlighting, if any, shows text that’s not yet agreed)

| 4. LTE_TERR_BCAST | 4-1 | Time-interleaving | 1. Support of PMCH transmission pattern, excluding MCCH and MSI, with time interleaving 2. Maximum supported value of M 3. Maximum supported value of N 4. Support of TBS determination for the scaled TB 5. Support of determining the starting point for reading from the circular buffer (k0) for each subframe 4. Maximum TBS supported in a TTI for MCH with time interleaving | Support of fembmsDedicatedCell | Yes | N/A | UE is not able to support time-interleaving for LTE-based 5G broadcast | Per band | No | N/A | Componernt 21 candidate values: N: {1, 1and4,1and4and8} Componernt 3 candidate values M: {8, 8and16} Componernt 4 candidate values: FFS Note: One TB is mapped to N non-consecutive subframes. Two transmissions of the same TB are separated by (M-1) subframes. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal:

- Discuss whether RAN1 or RAN2 will specify Rel. 19 UE feature(s) for MBMSInterestIndication aspects

- Inform RAN2 about the conclusion via the LS accompanying the agreements from RAN1 #120bis for the updated Rel. 19 UE features list

Proposal:

- Alt. 1: Introduce the following Rel. 19 UE feature (yellow highlighting, if any, shows text that’s not yet agreed)

| 4. LTE_TERR_BCAST | 4-2 | Frequency-interleaving | 1. Support of frequency-interleaving for MCCH/MTCH/MSI | Support of fembmsDedicatedCell | Yes | N/A | UE does not support frequency-interleaving for MCCH/MTCH/MSI transmission | Per band | No | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Alt. 2: postpone discussions on frequency hopping for MCCH/MTCH/MSI until RAN1 #121

R1-2501915	Discussion on UE features for Rel-19 LTE based 5G broadcast	ZTE Corporation, Sanechips

R1-2502252	UE features for LTE based 5G broadcast Phase 2	Huawei, HiSilicon

R1-2502401	UE features for LTE-based 5G broadcast	Samsung

R1-2502547	UE feature list for Rel-19 LTE-based 5G Broadcast after RAN1#120	EBU

R1-2502874	UE features for LTE-based 5G Broadcast Phase 2	Qualcomm Incorporated