3GPP TSG RAN WG1 #120bis	                                   R1-2502964

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.14

Agenda Item:		9.15.14

Document for: 	Endorsement

- 9.15.14	Others

For discussions on UE features for Rel-19 TEI and other relevant issues.

R1-2502743	Summary of UE features for Rel-19 TEI and other relevant issues	Moderator (AT&T)

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 67. TEI19 | 67-1 | 32 DL HARQ processes for TN in FR1 and FR2-1 [TN32HARQ] | Support 32 HARQ processes in DL for TN in FR1 and FR2-1 |  | Yes | N/A | 32 DL HARQ processes for TN in FR1 and FR2-1 is not supported | Per FSPC | N/A | N/A | N/A | For FR1, the maximum number of layers configured for PDSCH is up to 4 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 67. TEI19 | 67-1a | 32 UL HARQ processes for TN in FR1 and FR2-1 [TN32HARQ] | Support 32 HARQ processes in UL for TN in FR1 and FR2-1 |  | Yes | N/A | 32 UL HARQ processes for TN in FR1 and FR2-1 is not supported | Per FSPC | N/A | N/A | N/A | For FR1, the maximum number of layers configured for PUSCH is up to 4 | Optional with capability signalling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 67. TEI19 | 67-2 | Support of positioning SRS with Tx frequency hopping in RRC_CONNECTED for non-RedCap UEs [Pos_SRSHop] | 1. Maximum SRS bandwidth across all hops 2. Maximum number of hops 3. RF Tx retuning time between consecutive hops 4. Switching time between active BWP and frequency hop 5. Overlapping PRB(s) between adjacent hops 6. Support of {0,1,2,4} overlapping PRB(s) between adjacent hops 7. Maximum number of positioning SRS resources with Tx frequency hopping | 13-8 | Yes | n/a | Positioning SRS with Tx hopping in RRC_CONNECTED is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: FR1: {40, 50, 80, 100} FR2: {100, 200, 400} Component 2 candidate values: {2,3,4,5,6} Component 3 candidate values: FR1: {0us, 70us, 140us, 210us} FR2: {0us, 35us, 70us, 140us} Component 4 candidate values: {0us, 100us, 140us, 200us, 300us, 500us} Component 7 candidate values: Periodic: {1,2,4,8,16,32,64} Aperiodic: {0,1,2,4,8,16,32,64} Semi-persistent: {0,1,2,4,8,16,32,64} Note: No additional UE requirements shall be specified for the case of Tx hopping with non-overlapping hops compared to the case of Tx hopping with overlapping hops, e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops Need for location server to know if the feature is supported | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 67. TEI19 | 67-2a | Support of positioning SRS with Tx frequency hopping in RRC_INACTIVE for non-RedCap UEs [Pos_SRSHop] | 1. Maximum SRS bandwidth across all hops 2. Maximum number of hops 3. RF Tx retuning time between consecutive hops 4. Switching time between active BWP and frequency hop 5. Overlapping PRB(s) between adjacent hops 6. Support of {0,1,2,4} overlapping PRB(s) between adjacent hops 7. Maximum number of positioning SRS resources with Tx frequency hopping | 27-15b | Yes | n/a | Positioning SRS with Tx hopping in RRC_INACTIVE is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: FR1: {40, 50, 80, 100} FR2: {100, 200, 400} Component 2 candidate values: {2,3,4,5,6} Component 3 candidate values: FR1: {0us, 70us, 140us, 210us} FR2: {0us, 35us, 70us, 140us} Component 4 candidate values: {0us, 100us, 140us, 200us, 300us, 500us} Component 7 candidate values: Periodic: {1,2,4,8,16,32,64} Semi-persistent: {0,1,2,4,8,16,32,64} Note: No additional UE requirements shall be specified for the case of Tx hopping with non-overlapping hops compared to the case of Tx hopping with overlapping hops, e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops Need for location server to know if the feature is supported | Optional with capability signalling |
| 67. TEI19 | 67-2b | UL Time Window and transmission of SRS for positioning with Tx Frequency hopping within the window for non-RedCap UEs [Pos_SRSHop] | Support of UL Time Window and transmission of SRS for positioning with Tx Frequency hopping within the window | 67-2 | No | N.A. | UE does not support the UL time window for SRS for positioning with Tx frequency hopping | Per band | N.A. | N.A. | N.A. |  | Optional with capability signaling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 67. TEI19 | 67-3 | Fallback to a SSSG with designated index after a PUCCH carrying an SR is transmitted [SRTrig_SSSGSwitch] | Support of start PDCCH monitoring according to search space sets with a designated group index and stops PDCCH monitoring according to search space sets with a group index other than the designated SSSG index from the first slot that is at least symbols after the last symbol of a PUCCH carrying an SR, if the UE is instructed to monitor PDCCH according to search space sets with the SSSG index other than the designated SSSG index before the transmission of PUCCH carrying the SR. | 29-3b | Yes | n/a | SR-triggered SSSG switching is not supported | Per band | n/a | n/a | n/a | Note: symbols is specified in Table 10.4-1 of TS 38.213 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 67. TEI19 | 67-4 | Support of simultaneous SRS carrier switching [Simul_SRSCS] | 1.- Support simultaneous SRS carrier switches. Two SRS carrier switches are considered to be simultaneous if the SRS transmission (including RF retuning time) in both CCs overlap in time | 2-56 | Yes | N/A | Simultaneous SRS CS across multiple CC is not supported | Per band combination | N/A | N/A | N/A | For each target band, the UE can indicate with which other target bands in the band combination can SRS carrier switching be simultaneously triggered | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 67. TEI19 | 67-3 | Reporting inter-cell switching time when source is sharing Tx chain [SRSCS_ULTxSwitch] | Reporting inter-cell switching time when SRS carrier switching is configured with UL Tx Switching |  | Yes | n/a | Simultaneous operation of SRS CS and Ul Tx Sw is not supported [if not reported by UE, max(uplinkTxSwitchingPeriod-r16, SRS-SwitchingTimeNR) is assumed for the configured triple is assumed.] | Per BC | n/a | n/a | n/a | Indicates, for the band combination, the switching time from a carrier that is sharing uplink chains with the source carrier. The UE reports one of {“max”,”sum”}, where “max” indicates that the switching time is the maximum between the uplink Tx switching time and SRS carrier switching times, and “sum” indicates that the switching time is the sum of the uplink Tx switching tie and the SRS carrier switching time. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2501916	Discussion on UE features for Rel-19 TEIs	ZTE Corporation, Sanechips

R1-2502253	UE features for TEI-19 features	Huawei, HiSilicon

R1-2502649	UE features for Rel-19 TEI SRSCS_ULTxSwitch	Apple

R1-2502875	UE features for Rel-19 TEI	Qualcomm Incorporated