3GPP TSG RAN WG1 #120bis	                                   						R1-2502965

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.3

Agenda Item:		9.15.3

Document for: 	Endorsement

- 9.15.3 UE features for evolution of NR duplex operation: SBFD

Proposal 4-3-1:

Introduce following FG for L1 SRS-RSRP measurement/reporting.

| 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [Periodic, semi-persistent and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | L1 SRS-RSRP measurement and reporting is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4-3-3:

Introduce a new FG for simultaneous reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows

- Capture in LS on Rel-19 RAN1 UE feature list for NR after RAN1#120bis that “RAN1 would like to inform RAN4, as a response to R1-2501698/R4-2502632, that RAN1 has decided to define an optional UE capability for FDM-ed DL reception and SRS measurement as in FG 60-9”

| 60-9a | Simultaneous reception of DL signals/channels and L1 SRS-RSRP measurement resource | 1. Support of simultaneous reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 | YES | n/a | Simultaneous reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4-2-1:

Introduce following FG for L1 CLI-RSSI measurement/reporting.

| 60-8 | L1 CLI-RSSI measurement and reporting | 1. Support of L1 CLI-RSSI measurement and reporting 2. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Supported CLI-RSSI measurement resource type #1/#2/#3 3. [Periodic, semi-persistent and] aperiodic CLI-RSSI measurement resource 4. Maximum number of [configured] L1 CLI-RSSI measurement resources [(sum of aperiodic/semi-persistent/periodic)] across all CCs FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | L1 CLI-RSSI measurement and reporting is not supported | [Per Band] | TDD only | [n/a] | [n/a] | Component 2 candidate values: [FFS] Component 4 candidate values: [FFS] Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands Measurement resource type#3: One CLI-RSSI measurement resource is configured within a UL subband | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4-1-1:

Introduce the following FG for UL resource muting

| 60-7 | UL resource muting [for CP-OFDM waveform] | 1. Support semi-static configuration of time location and frequency location of UL resource muting [for CP-OFDM waveform] - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS Whether UL muting symbol configuration is common across DG/CG Type 2 PUSCH and CG Type 1 PUSCH, or separate configuration between DG/CG Type 2 PUSCH and CG Type 1 PUSCH] [- FFS Whether UL muting symbol configuration is common across all CG Type 1 PUSCH configurations, or separate configuration for each of the multiple CG Type 1 PUSCH configurations] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | [FFS] | YES | n/a | UL resource muting [for CP-OFDM waveform] is not supported | [Per Band] | TDD only | n/a | n/a | FFS: Whether to merge/decouple this FG between CP-OFDM and DFTS-OFDM | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [62-7a] | UL resource muting for DFTS-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS Whether UL muting symbol configuration is common across DG/CG Type 2 PUSCH and CG Type 1 PUSCH, or separate configuration between DG/CG Type 2 PUSCH and CG Type 1 PUSCH] [- FFS Whether UL muting symbol configuration is common across all CG Type 1 PUSCH configurations, or separate configuration for each of the multiple CG Type 1 PUSCH configurations] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | FFS | YES | n/a | UL resource muting for DFTS-OFDM waveform is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |

Proposal 2-4-1:

Introduce following FG(s) for [transmissions and] receptions across SBFD and non-SBFD symbols (Configuration 2), and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not

- Alt.1: Configuration 2 for transmission and reception should be defined in a single FG

- Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs

| 60-2 | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of DL reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: whether component 2 is defined as separate FG |  | YES | n/a | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3-1-2:

Adopt “Per band” type for FG 60-3.

Proposal 3-2-2:

Adopt “Per band” type for FG 60-4

R1-2501831	Discussion on UE features for evolution of NR duplex operation: SBFD	vivo

R1-2501887	Discussion on UE features for evolution of NR duplex operation: SBFD	Spreadtrum, UNISOC

R1-2501913	Discussion on UE features for Rel-19 SBFD	ZTE Corporation, Sanechips

R1-2501981	UE features for SBFD	CATT

R1-2502049	Discussion on Rel-19 NR UE features for evolution of NR duplex operation	Ofinno

R1-2502135	Views on SBFD UE features	Nokia

R1-2502143	Discussion on UE features for evolution of NR duplex operation	MediaTek Inc

Withdrawn

R1-2502180	Discussion on UE features for evolution of NR duplex operation: SBFD	CMCC

R1-2502244	UE features for evolution of NR duplex operation	Huawei, HiSilicon

R1-2502280	Discussion on UE features for Rel-19 NR Duplex	OPPO

R1-2502393	UE features for Rel-19 Duplex Evolution for NR	Samsung

R1-2502425	UE-features for SBFD	Ericsson

R1-2502461	UE features for SBFD	Xiaomi

R1-2502641	Views on UE features for Rel-19 SBFD	Apple

R1-2502787	Discussion on UE features for evolution of NR duplex operation	NTT DOCOMO, INC.