3GPP TSG RAN WG1 #120bis	                                   						R1-2502965

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.3

Agenda Item:		9.15.3

Document for: 	Endorsement

- 9.15.3 UE features for evolution of NR duplex operation: SBFD

Proposal 4-3-1:

Introduce following FG for L1 SRS-RSRP measurement/reporting.

| 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [Periodic, semi-persistent and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | L1 SRS-RSRP measurement and reporting is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4-3-3:

Introduce a new FG for simultaneous reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows

| 60-9a | FDM-ed Simultaneous reception of DL signals/channels and L1 SRS-RSRP measurement resource | 1. Support of FDMed simultaneous reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 | YES | n/a | FDMed simultaneous reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4-2-1:

Introduce following FG for L1 CLI-RSSI measurement/reporting.

| 60-8 | L1 CLI-RSSI measurement and reporting | 1. Support of L1 CLI-RSSI measurement and reporting 2. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Supported CLI-RSSI measurement resource type #1/#2/#3 3. [Periodic, semi-persistent and] aperiodic CLI-RSSI measurement resource 4. Maximum number of [configured] L1 CLI-RSSI measurement resources [(sum of aperiodic/semi-persistent/periodic)] across all CCs FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | L1 CLI-RSSI measurement and reporting is not supported | [Per Band] | TDD only | [n/a] | [n/a] | Component 2 candidate values: [FFS] Component 4 candidate values: [FFS] Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands Measurement resource type#3: One CLI-RSSI measurement resource is configured within a UL subband | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4-1-1:

Introduce the following FG for UL resource muting

| 60-7 | UL resource muting [for CP-OFDM waveform] | 1. Support semi-static configuration of time location and frequency location of UL resource muting [for CP-OFDM waveform] - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS Whether UL muting symbol configuration is common across DG/CG Type 2 PUSCH and CG Type 1 PUSCH, or separate configuration between DG/CG Type 2 PUSCH and CG Type 1 PUSCH] [- FFS Whether UL muting symbol configuration is common across all CG Type 1 PUSCH configurations, or separate configuration for each of the multiple CG Type 1 PUSCH configurations] [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 1/2 CG PUSCH, and common pattern for multiple CG Type 1 PUSCH configurations] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | [FFS] | YES | n/a | UL resource muting [for CP-OFDM waveform] is not supported | [Per Band] | TDD only | n/a | n/a | FFS: Whether to merge/decouple this FG between CP-OFDM and DFTS-OFDM | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [62-7a] | UL resource muting for DFTS-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS Whether UL muting symbol configuration is common across DG/CG Type 2 PUSCH and CG Type 1 PUSCH, or separate configuration between DG/CG Type 2 PUSCH and CG Type 1 PUSCH] [- FFS Whether UL muting symbol configuration is common across all CG Type 1 PUSCH configurations, or separate configuration for each of the multiple CG Type 1 PUSCH configurations] [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 1/2 CG PUSCH, and common pattern for multiple CG Type 1 PUSCH configurations] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | FFS | YES | n/a | UL resource muting for DFTS-OFDM waveform is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |

Proposal 2-4-1:

Introduce following FG(s) for [transmissions and] receptions across SBFD and non-SBFD symbols (Configuration 2), and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not

- Alt.1: Configuration 2 for transmission and reception should be defined in a single FG

- Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs

| 60-2 | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of DL reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: whether component 2 is defined as separate FG |  | YES | n/a | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3-1-2:

Adopt “Per band” type for FG 60-3.

Proposal 3-2-2:

Adopt “Per band” type for FG 60-4

Proposal 2-5-1:

Add the following components in FG 60-1 (basic FG for SBFD operation)

- Support of separate PUCCH resource configuration

- Support of separate power control parameter

- Support of separate frequency hopping offset

- [FFS: Support of separate SRS resource set configuration]

Proposal 3-3-1:

Introduce following FG for PRACH repetition in additional ROs, and FFS on whether separate FGs for RACH configuration Option 1 and Option 2 are necessary.

- Alt.1: separate FGs for RACH configuration Option 1 and Option 2 are NOT defined

- Alt.2: separate FGs for RACH configuration Option 1 and Option 2 are define

| 60-5 | Preamble repetition for SBFD RA operation [with RACH configuration Option 1] within additional-ROs | 1. Determination of the set of valid additional-ROs and the time period of the set(s) of valid additional-ROs 2. [For RACH configuration Option 1,] support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. [3. For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.] FFS: separate FGs for RACH configuration Option 1 and Option 2 are necessary | 54-1, 60-3 [or 60-4,] | YES | n/a | Preamble repetition for SBFD RA operation [with RACH configuration Option 1] within additional-ROs is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-5a | Preamble repetition for SBFD RA operation [with RACH configuration Option 2] | 1. Determination of the set of valid additional-ROs and the time period of the set(s) of valid additional-ROs 2. For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. | 54-1, 60-4 | YES | n/a | Preamble repetition for SBFD RA operation with RACH configuration Option 2 is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |

Proposal 3-4-1:

Regarding Msg.3 repetition in SBFD symbols,

- Alt-1: Clarify in FG 60-4 Note column that “Note: When a UE supports this FG and FG 30-6, the UE also support Msg.3 repetition in SBFD symbol”

| 60-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | 1. Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG | [60-1, 60-3] | YES | n/a | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | [Per band] | TDD only | n/a | n/a | Note: When a UE supports this FG and FG 30-6, the UE also support Msg.3 repetition in SBFD symbol | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Alt-2: Define a new separate FG

| 60-6 | Msg.3 repetition in SBFD symbols | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols | 30-6, [60-3 or 60-4] | YES | n/a | Msg.3 repetition in SBFD symbols is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2-1-1/2:

Update FG 60-1 as follows:

- Confirm “cell-common” in component 1

- Confirm “UL transmission in one UL subband and DL reception in one DL subband in SBFD symbols”

- Confirm Component 3: “3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling”

- Add “Support of separate CSI reports for SBFD and non-SBFD symbols

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | [Per band] | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Remaining issues:

Question 2-2-2 (and then Question 2-2-3 potentially)

Question 2-3-1:

R1-2501831	Discussion on UE features for evolution of NR duplex operation: SBFD	vivo

R1-2501887	Discussion on UE features for evolution of NR duplex operation: SBFD	Spreadtrum, UNISOC

R1-2501913	Discussion on UE features for Rel-19 SBFD	ZTE Corporation, Sanechips

R1-2501981	UE features for SBFD	CATT

R1-2502049	Discussion on Rel-19 NR UE features for evolution of NR duplex operation	Ofinno

R1-2502135	Views on SBFD UE features	Nokia

R1-2502143	Discussion on UE features for evolution of NR duplex operation	MediaTek Inc

Withdrawn

R1-2502180	Discussion on UE features for evolution of NR duplex operation: SBFD	CMCC

R1-2502244	UE features for evolution of NR duplex operation	Huawei, HiSilicon

R1-2502280	Discussion on UE features for Rel-19 NR Duplex	OPPO

R1-2502393	UE features for Rel-19 Duplex Evolution for NR	Samsung

R1-2502425	UE-features for SBFD	Ericsson

R1-2502461	UE features for SBFD	Xiaomi

R1-2502641	Views on UE features for Rel-19 SBFD	Apple

R1-2502787	Discussion on UE features for evolution of NR duplex operation	NTT DOCOMO, INC.