3GPP TSG RAN WG1 #120bis	                                   						R1-2502966

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.5

Agenda Item:		9.15.5

Document for: 	Endorsement

- 9.15.5 UE features for LP-WUS/WUR for NR

Agreement:

- Define new FG 62-3 for the support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode

- Add a component in FG 62-2/2a for the support of LP-WUS frequency resource within active DL BWP

| 62-3 | LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode | FG 62-2 or 62-2a | YES | n/a | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Adopt “Per band” for FG 62-1 and 62-1a:

Agreement:

Update FG 62-1 and 62-1a as follows:

| 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |

R1-2501712	Views on UE Features for LP-WUS	FUTUREWEI

R1-2501771	Discussion on WUR features	ZTE Corporation, Sanechips

R1-2501833	UE features for Rel-19 LP-WUS/WUR	vivo

R1-2501983	LP-WUS/WUR UE feature	CATT

R1-2502137	Views on LP-WUS/WUR UE features	Nokia

R1-2502182	Discussion on UE features for low-power wake-up signal and receiver for NR	CMCC

R1-2502240	UE features for Rel-19 LP-WUS	Huawei, HiSilicon

R1-2502260	Discussion for UE features of LP-WUS/WUR	OPPO

R1-2502395	Discussion on UE features for LP-WUS/WUR	Samsung

R1-2502463	UE features for NR LP-WUS/WUR	Xiaomi

R1-2502487	UE features for LP-WUS/WUR for NR	LG Electronics

R1-2502643	On Rel-19 LP-WUS/WUR UE features	Apple

R1-2502724	UE features for LP-WUS/WUR for NR	MediaTek Inc.

R1-2502789	Discussion on UE features for LP-WUS/WUR for NR	NTT DOCOMO, INC.

R1-2502809	UE features for Rel-19 LP-WUS/WUR WI	Ericsson

R1-2502868	UE features for LP-WUS/WUR for NR	Qualcomm Incorporated

R1-2502910	UE Features for LP-WUS	Vodafone, Deutsche Telekom