3GPP TSG RAN WG1 #120bis	                                   						R1-2502968

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.9

Agenda Item:		9.15.9

Document for: 	Endorsement

- 9.15.9 UE features for NTN for Internet of Things (IoT) Phase 3

For RAN1#120bis, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

Agreement:

Define the following FGs for Rel-19 IoT-NTN Phase 3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. IoT_NTN_Ph3 | 1-1 | OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED | 1. Support of symbol-level length-2 OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED [2. Support of TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa] [3. Dynamic activation/deactivation of OCC feature] [FFS: Whether to have other component(s)] |  | Yes | N/A | UE does not support OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED | [Per band or Per UE] | [N/A] | [N/A] | [FFS: Whether to separate this FG for GSO and NGSO] [Note: This FG is applicable only to [IoT-NTN band]] | Optional with capability signalling |
| 1. IoT_NTN_Ph3 | 1-2 | OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED | 1. Support of slot-level length-2 OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED [2. Support of CDM DMRS with the pattern supported for NPUSCH format 1 with 15 kHz SCS] [3. Dynamic activation/deactivation of OCC feature] [FFS: Whether to have other component(s)] |  | Yes | N/A | UE does not support OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED | [Per band or Per UE] | [N/A] | [N/A] | [FFS: Whether to separate this FG for GSO and NGSO] [Note: This FG is applicable only to [IoT-NTN band]] | Optional with capability signalling |

R1-2501836	Discussion on UE features for NTN for IoT Phase 3	vivo

R1-2501888	Discussion on UE features for NTN for NR	Spreadtrum, UNISOC

R1-2501943	Discussion on the UE feature for IoT-NTN Phase-3	ZTE Corporation, Sanechips

R1-2502140	Views on Rel-19 NTN for Internet of Things (IoT) Phase 3 UE features	Nokia

R1-2502185	Discussion on UE features for NTN for IoT Phase 3	CMCC

R1-2502249	UE features for IoT-NTN Phase 3	Huawei, HiSilicon

R1-2502271	Discussion on UE features for NTN for Internet of Things Phase 3	OPPO

R1-2502308	On UE features for IoT-NTN Phase 3	Ericsson

R1-2502398	UE features for NTN for Internet of Things (IoT) Phase 3	Samsung

R1-2502646	On Rel-19 IoT-NTN UE Features	Apple

R1-2502871	UE features for NTN IOT	Qualcomm Incorporated