3GPP TSG RAN WG1 #121		R1-2503371

St. Julian’s, Malta, 19th – 23rd May, 2025

Source:	vivo

Title:	Remaining issues on LP-WUS operation in CONNECTED modes

Agenda Item:	9.6.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the remaining issues for the LP-WUS operation in CONNECTED modes.

- Procedures of LP-WUS triggering PDCCH monitoring

For LP-WUS CONNECTED mode operation Option 1-1 and Option 1-2, the procedures of LP-WUS triggering PDCCH monitoring are almost clear. One remaining issue is whether LR and MR can work simultaneously, and if not what the UE behavior is. Related to this issue, the following LS [1] is also received from RAN2 and our draft reply can be found in [2]. In the following we provide our views on the questions asked by RAN2.

RAN2 R2-2503187/R1-2503616 LS on LP-WUS in RRC_CONNECTED ## 1.1 Short DRX RAN2 discussed the Short DRX cycle applicability with LP-WUS in RRC_CONNECTED mode and made the following agreement and a working assumption: For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used. ## 1.2 LP-WUS monitoring RAN2 discussed whether there are any scenarios on when the UE would not be able to monitor LP-WUS and made the following working assumption: Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. Based on this working assumption, RAN2 would like to ask RAN1: 1. Whether there are any cases/scenarios on when the UE is not able to monitor LP-WUS? - RAN2 would like to note that for DCP, such cases/scenarios included collision with Active Time, measurement gap, interruption caused by BWP switching, and RAR window monitoring for BFR. 2. Is there any case when the UE is not able to monitor LR and MR simultaneously? ## 1.3 UE time offset preference RAN2 also discussed and agreed the following: If configured, the UE can signal a preferred time offset via UAI signalling. RAN2 would like to know whether there were any further conclusions in RAN1 on what information the preferred time offset via UAI should provide (e.g. in relation to the minimum time offset provided as a UE capability)? To RAN WG1 ACTION: RAN2 respectfully asks RAN1 to take the above agreements and working assumptions into account and provide feedback, if any; and provide responses to the questions asked.

For LP-WUS monitoring

For LP-WUS CONNECTED mode operation Option 1-1 and Option 1-2, it was agreed in RAN1#120 meeting that UE does not need to monitor LP-WUS during C-DRX active time. In our view, it also kind of implies that LP-WUR and MR cannot work simultaneously.

In general, it would be better for LP-WUR to share certain module of MR to reduce UE size and cost, as illustrated in figure 1, antennas are shared by LP-WUR and MR. If LP-WUR and MR are supported to work simultaneously, a splitter could be considered to split the input signal but causing insertion loss to both LP-WUR and MR, as given in figure 1(a). To avoid such loss, a switcher could be used to switch between MR and LP-WUR, allowing either MR or LP-WUR works at given time, as given in figure 1 (b).

Therefore, it should be considered as a basic implementation that LP-WUR and MR cannot operate simultaneously. While it does not preclude the capable UE can support LP-WUR and MR work simultaneously, which can be optionally supported based on UE capability report. Details on the UE capability can be discussed in UE feature session.

- LP-WUR and MR sharing antennas supported by splitter

- LP-WUR and MR sharing antennas supported switcher

Figure 1 Example of LP-WUR and MR sharing antennas

Related to this issue, another point needs to be discussed is how fast the switching can be done between the MR and LR. It may have some impact on the scheduling restriction. For example, if the switching between MR and LR is fast e.g., ~a few microseconds, the switching time does not impact on either NR signal or WUS reception. However, if the switching between MR and LR is slow e.g., in OFDM symbol level, the scheduling for NR signal or WUS should take into account the switching time to avoid the negative impact on the reception.

Proposal 1:

- LP-WUR and MR cannot work simultaneously should be UE basic capability.

- LP-WUR and MR can work simultaneously can be optionally supported based on UE capability.

- FFS how fast the switching can be done between the MR and LR.

For RAN2’s question on whether there are any cases/scenarios on when the UE is not able to monitor LP-WUS. Before discussing the cases/scenarios, we would like to clarify the understanding on “UE is NOT able to monitor LP-WUS”. It should be clear that UE is not able to monitor WUS in RAN2’s LS is due to some hardware issue. It is different from the cases/scenarios that UE is not expected to monitor LP-WUS, when LP-WUS MO(s) within the Cell DTX inactive time and when LP-WUS MO collides with NR signals/channels/slot structure, where network does not or cannot transmit the LP-WUS. With this understanding that UE is not able to monitor WUS in RAN2’s LS is because LP-WUR and MR cannot work simultaneously, we agree with the cases/scenarios listed by RAN2. In addition to those cases/scenarios, one more case/scenario results in the UE being not able to monitor the LP-WUS occasion(s) should be added that is the collision with PDCCH CSS sets monitoring except for a Type3-PDCCH CSS set.

In addition, RAN1#120bis meeting agreed that UE is not expected to monitor LP-WUS, when LP-WUS MO(s) within the Cell DTX inactive time. Obviously, such case does not belong to the case of “UE is is not able to monitor LP-WUS”. Correspondingly, UE does not start the Timer for PDCCH monitoring (e.g., start drx-OnDurationTimer for Option 1-1 or LPWUS_PDCCHMonitoringTimer for Option 1-2).

Proposal 2: The understanding on “UE is not able to monitor LP-WUS” in RAN2’s LS is because MR is active and UE does not support LP-WUR and MR work simultaneously. Based on this understanding, RAN1 agree the cases/scenarios listed in RAN2’s LS with adding one more case as following:

- Such cases/scenarios resulting UE is not able to monitor LP-WUS include collision with Active Time, measurement gap, interruption caused by BWP switching, and RAR window monitoring for BFR, PDCCH monitoring associated with PDCCH CSS sets except for a Type3-PDCCH CSS set.

Proposal 3: A UE does not start the drx-OnDurationTimer for LP-WUS connected mode operation Option 1-1 or LPWUS_PDCCHMonitoringTimer for LP-WUS connected mode operation Option 1-2 when the LP-WUS monitoring occasion(s) collide with DTX inactive period for the primary cell.

For above cases/scenarios on when the UE is not able to monitor LP-WUS due to MR and LP-WUR cannot work simultaneously, corresponding UE behavior should be discussed. For Option 1-1, the working assumption made by RAN2 is reasonable. Since the typical configuration of WUS monitoring occasions/periodicity for option 1-1 are sparse and aligned with the long DRX cycle. If UE does not start drx-OnDurationTimer, the delay for waking up the UE will be as long as one long DRX cycle (e.g., 160ms), which is unacceptable. Therefore, RAN2’s working assumption make sense for LP-WUS operation option 1-1. However, for LP-WUS operation option 1-2, for the collision cases/scenarios on when the UE is not able to monitor the LP-WUS occasion(s), the UE should NOT start the LP-WUS_PDCCHMonitoringTimer is more reasonable. Because different from option 1-1, option 1-2 typically has more frequent WUS monitoring occasion configuration. Network has sufficient flexibility and opportunities to send WUS to timely wake up MR. If we follow the same solution as for option 1-1, considering the dense LP-WUS monitoring occasions and short periodicity which results in more frequent collisions, the MR will be always waked up unnecessarily and the power consumption will be increased significantly. Therefore, a more reasonable solution should be that for Option 1-2, the UE only wakes up upon detecting a LP-WUS.

Proposal 4: For LP-WUS connected mode Option 1-2, for the cases/scenarios on when the UE is not able to monitor the LP-WUS occasion(s), the UE does not start the LP-WUS_PDCCHMonitoringTimer .

For UE time offset preference

About the relation between the preferred time offset via UAI and the minimum time offset provided as a UE capability, in our view, the minimum time offset is defined from UE processing capability perspective. Then it is more reasonable to assume that the preferred time offset via UAI should be equal to or longer than the minimum time gap.

Proposal 5: The preferred time offset indicated via UAI value(s) should be equal to or longer than the minimum time gap reported by UE capability.

Clarification for option 1-2 for periodic CSI/L1-RSRP reporting operation

Agreement@RAN1#119 meeting For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required.

For above agreement, when the parameter is configured, if the UE is not indicated to wake up by “LP-WUS”, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config. One unclear point is on which MO(s) or during which time period, the LP-WUS detection result (i.e. UE is not indicated to wake up) determines whether the periodic CSI/L1-RSRP is report during the drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time. Take following figure 1 as example,

- for the 2nd DRX cycle, it is reasonable that the detection of the LP-WUS on the two LP-WUS MOs (MO#1 and MO#2) before the time given by the drx-onDurationTimer determines whether the periodic CSI/L1-RSRP is reported during the on duration time

- for the 3rd DRX cycle, it is reasonable that the detection of the LP-WUS on the four LP-WUS MOs (MO#3, #4, #5 and #6) before the time given by the drx-onDurationTimer determines whether the periodic CSI/L1-RSRP is reported during the on duration time.

Figure 1 example for option 1-2 for periodic CSI/L1-RSRP reporting operation

Therefore, we propose following CR update in TS 38.214 [3].

Proposal 6: Adopt the following update for TS 38.214:

5.1.6.1 CSI-RS reception procedure <omitted text> If the UE is configured with DRX and to monitor either DCI format 2_6 or [LP-WUS], - if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI or [LP-WUS TransmitOtherPeriodicCSI] to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'cri-RSRP-Index', 'ssb-Index-RSRP' and 'ssb-Index-RSRP-Index' - when drx-onDurationTimer in DRX-Config is not started in case DCI format 2_6 or [LP-WUS option 1-1] is configured, or - when WUS_PDCCHMonitoringTimer [in XYZxxx] has not been started during the time of the previous DRX cycle in case [LP-WUS option 1-2] is configured, - the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported; - if the UE is configured by higher layer parameter ps-TransmitPeriodicL1-RSRP or [LP-WUS TransmitPeriodicL1-RSRP] to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP' or 'cri-RSRP-Index' - when drx-onDurationTimer in DRX-Config is not started in case DCI format 2_6 or [LP-WUS option 1-1] is configured, or - when WUS_PDCCHMonitoringTimer [in XYZxxx] has not been started during the time of the previous DRX cycle in case [LP-WUS option 1-2] is configured, - the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported. <omitted text> 5.2.2.5 CSI reference resource definition <omitted text> When the UE is configured to monitor DCI format 2_6 or [WUS], - if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI or [LPWUS-TransmitOtherPeriodicCSI] to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', and 'ssb-Index-RSRP- Index ' - when drx-onDurationTimer in DRX-Config is not started in case DCI format 2_6 or [LP-WUS option 1-1] is configured, or - when WUS_PDCCHMonitoringTimer [in XYZxxx] has not been started during the time of the previous DRX cycle in case [LP-WUS option 1-2] is configured, - the UE shall report CSI with the reportQuantity not set to ‘ssb-Index-SINR’ or ‘ssb-Index-SINR-Index’ during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. - if the UE is configured with a CSI report configuration containing a list of sub-configurations provided by csi-ReportSubConfigToAddModList, and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI or [LPWUS-TransmitOtherPeriodicCSI] to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', and 'ssb-Index-RSRP- Index' - when drx-onDurationTimer in DRX-Config is not started in case DCI format 2_6 or [LP-WUS option 1-1] is configured, or - when WUS_PDCCHMonitoringTimer [in XYZxxx] has not been started during the time of the previous DRX cycle in case [LP-WUS option 1-2] is configured, - UE shall report a CSI report including one or more sub-reports only during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time, per sub-configuration, no later than CSI reference resource and drops the report otherwise, where the sub-configuration is the configured one for P-CSI reporting. - if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP or [LPWUS-TransmitOtherPeriodicL1-RSRP] to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', or 'ssb-Index-RSRP- Index' - when drx-onDurationTimer in DRX-Config is not started in case DCI format 2_6 or [LP-WUS option 1-1] is configured, or - when WUS_PDCCHMonitoringTimer [in XYZxxx] has not been started during the time of the previous DRX cycle in case [LP-WUS option 1-2] is configured, - the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' or 'cri-RSRP- Index' if receiving at least one CSI-RS transmission occasion for channel measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. <omitted text>

- Timeline between LP-WUS reception and PDCCH monitoring

Following agreements were achieved in RAN1#120 and #120bis meetings:

Agreement@ RAN1#120 - For the X = [2, 3] candidate values for the UE capability report on the minimum time gap for each SCS, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20, 30, 40}ms - Note: Other values are not precluded - Only for reported value of [20ms,] 30ms and 40ms (if supported), the reported values assume SSB periodicity of 20ms and ensures SSB(s) before PDCCH reception and during time gap - FFS: translation of minimum time gap for different SSB periodicities including different candidates for different SSB periodicities - FFS whether the same set of candidates are used for different SCSs - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR - FFS whether/how to consider the case of CA - FFS: candidate values that can be indicated by UAI Agreement@ RAN1#120bis For the UE capability report on the minimum time gap is between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, support X=3 candidate values corresponding to {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3. - FFS: Whether to support different set of values for different receiver types

About the candidate value for V1, V2 and V3, during the SI, the transition time for deep sleep state, light sleep and micro sleep is 20ms, 6ms and 0ms including the ramp-up and ramp-down time. Considering the cases for power saving benefit, MR with light sleep state and deep sleep state with synchronization and without synchronization should be supported. Consequently, the value for V1 corresponds to MR with light sleep can be determined as (6ms/2+TWUS-processing-time) = (3ms+TWUS-processing-time); Similarly, the value for V2 corresponds to MR with deep sleep with synchronization is (10ms+TWUS-processing-time) and the value for V3 corresponds to MR with deep sleep without synchronization is (10ms+TSSB+TWUS-processing-time)=(30ms+TWUS-processing-time) for SSB with 20ms periodicity. About the LP-WUS processing time TWUS-processing-time, we think 3ms is too slow, much smaller value like up to 1ms can be considered.

About whether the same or different values are selected for OOK-based WUR and OFDM-based WUR, considering the major component is MR ramp up time, we prefer the same set values for different receiver types. About whether/how to consider the case of CA, it can be discussed in UE feature session.

Proposal 7:

- Support V1=[3+TWUS-processing-time]ms, V2=[10+TWUS-processing-time]ms and V3=[30+TWUS-processing-time]ms, where TWUS-processing-time is the LP-WUS processsing time, as candidate values for the minimum time gap reported by the UE capability.

  - LP-WUS processsing time, i.e., TWUS-processing-time is less than 3ms, up to 1ms can be the considered.

- The same set of values is applied for different receiver types.

- Activation and deactivation procedure for LP-WUS monitoring

Following agreements and conclusion were achieved in previous RAN1 meetings:

Agreement@RAN1#116 For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission. Conclusion@RAN1#120 From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour - Above applies at least for Option 1-1 Conclusion@RAN1#120bis From RAN1 perspective, when LP-WUS monitoring Option 1-2 is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers PDCCH monitoring according to the legacy UE behaviour

Regarding Option 2 and Option 3 to enable/disable the procedure/feature of PDCCH monitoring triggered by LP-WUS, our views are following:

- Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance.

For Option 2, the L1 signaling for activation/deactivation WUS monitoring can be a new signaling or can enhance the legacy i.e., Rel-17 PDCCH skipping signaling for dynamically switching between the PDCCH monitoring and LP-WUS monitoring. More details can be found in section 5 on Enhanced L1 PDCCH skipping indication for LP-WUS activation.

- Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s).

The following conditions are discussed in previous meetings.

- Condition 1: the insufficient/sufficient coverage of LP-WUS

- Condition 2: Timer based activation/deactivation of WUS monitoring.

For Condition 1 of insufficient coverage of LP-WUS, as discussed, for RRC-connected UE, RRM/RLM/BFD/CSI measurements are performed by MR and network can use RRC signaling or L1/L2 signaling to disable or deactivate the WUS operation based on the measurement results. Therefore, condition 1 of insufficient/sufficient coverage of WUS is not needed for deactivation/activation of LP-WUS.

For Condition 2, since a new Timer was agreed that is used for resuming LP-WUS monitoring at the expiration of this new timer (i.e. no PDCCH is detected during the new timer running), the Timer here for condition 2 is viewed as LP-WUS deactivation Timer, which is used to avoid the case that UE does not receive the WUS for a long time due to bad channel condition. The motivation for such WUS deactivation Timer is the same as Condition 1 that in our view is unnecessary.

It should be noted that not having a specified condition to deactivate LP-WUS monitoring due to poor channel conditions does not prevent the UE falling back from LP-WUS monitoring to PDCCH monitoring. In other words, autonomous fallback to PDCCH monitoring should be allowed for a UE monitoring LP-WUS.

Autonomous fallback to PDCCH monitoring

During the last meeting, some companies noted that UE autonomous fallback to PDCCH monitoring contradicts the previous RAN1 agreement, where LP-WUS monitoring by the UE is known to the gNB. Our understanding is that the primary motivation for LP-WUS monitoring being known to the gNB is to prevent instances where the UE misses the PDCCH sent by the gNB. This does not imply that the UE should be prohibited from switching to PDCCH monitoring autonomously. In fact, autonomous fallback to PDCCH monitoring by the UE helps avoid situations where the UE misses the PDCCH sent by the gNB.

Regarding the definition of fallback conditions and the necessity of additional fallback indication from the UE, we believe there is no need to specify any fallback conditions or introduce indications from the UE to inform the gNB of its fallback status. It is important to note that in connected mode, UE RRM/RLM/BFD/CSI measurements are still performed by MR in RRC connected mode. Therefore, based on the measurement report, the gNB can disable the LP-WUS operation through RRC signaling before losing control of the UE’s LP-WUS monitoring. Such autonomous fallback cases should be considered rare, and maintaining network connectivity should remain a top priority, with the UE continuing PDCCH monitoring if necessary. Defining fallback conditions would unnecessarily increase RAN4 workload and test cases, while introducing additional indications from the UE would require the network to preconfigure or reserve UL resources, thereby increasing RAN1 workload and unnecessary overhead. Regarding the UE fallback behavior, our views for option 1-1 and option 1-2 are following:

- For LP-WUS operation option 1-1, the UE is allowed to fallback for PDCCH monitoring according to C-DRX configuration.

- For LP-WUS operation option 1-2, although fallback to Legacy C-DRX configuration could work, it requires gNB to perform blind checks. For example, after sending LP-WUS to wake up a UE, if no feedback is received, the gNB assumes the UE fallback to legacy C-DRX and re-sends the PDCCH based on the UE's C-DRX configuration. This increases the delay and resource overhead. To avoid such gNB guessing behavior, fallback to always-on PDCCH monitoring for the LP-WUS operation option 1-2 reduces the delay and more resource-efficient.

Proposal 8: When LP-WUS monitoring is enabled for RRC CONNECTED mode, UE by implementation can fallback to PDCCH monitoring when UE monitors LP-WUS without any specification work.

- No need to specify any condition for the fallback

- No need to have indication from UE when it falls back to PDCCH monitoring.

- For LP-WUS operation option 1-1, the fallback behavior for PDCCH monitoring can be according to the C-DRX configuration

- For LP-WUS operation option 1-2, the fallback behavior for PDCCH monitoring can be always-on PDCCH monitoring.

- Configuration for WUS monitoring occasions

Following agreements were achieved during RAN1#120bis meeting:

Agreement Update previous agreement in red: Agreement For LP-WUS MOs in connected mode for Option 1-1, support Approach 1: - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start - UE monitors the first Z LP-WUS MO(s) at or after Time offset2. - FFS: How to determine the value of Z is RRC configured - Time offset2 is RRC configured UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined given based on the minimum time gap reported by UE capability. Agreement - Update previous agreement in red: Agreement - For LP-WUS MOs in connected mode for Option 1-2, support Approach 1: - LP-WUS MOs, including a periodicity and a time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - FFS one or multiple MOs per periodicity - UE is configured with a number of MOs for the periodicity by RRC - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS - FFS: Definition of time offset4 - Alt1: - TheA time offset4 configured by the network indicating a time from the first LP-WUS MO per periodicity, after which the UE starts PDCCH monitoring via starting the new timer. - The time gap from the last LP-WUS MO per periodicity and the start of new timer is not smaller than the minimum time gap given based on UE capability

  - Overview for WUS configuration per BWP or per cell group

Table 1 Views on the Type (per BWP or per cell group) for WUS Parameters in connected mode

| Parameter Name | Description/Function | Type: Per BWP or Per Cell group |
| --- | --- | --- |
| LP-WUS_Mvalue_CONNECTED | Indicates the number of OOK symbols in an OFDM symbol in RRC_CONNECTED | Per BWP (since SCS for different BWP can be different, the suitable M value can also be different) |
| LP-WUS_overlaidSeq_roots_CONNECTED | Configuration of one or two roots for the overlaid OFDM sequence used per OOK symbol for LP-WUS in CONNECTED | Per BWP (since SCS for different BWP can be different, the suitable M value can also be different, different roots can be configured for each M as agreed) |
| LP-WUS_num_overlaidseq_CONNECTED | Configures the number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, - maximum 16 candidates overlaid sequences for M=1 - maximum 8 candidates overlaid sequences for M=2 - maximum 4 candidates overlaid sequences for M=4 | Per BWP (since SCS for different BWP can be different, the suitable M value can also be different, thus different number of sequences) |
| LP-WUS_startRB_CONNECTED | Indicates the starting RB of LP-WUS in RRC CONNECTED | Per BWP (Since SCS for different BWP can be different, the configured starting RB should be relative to carrier boundary corresponding to the SCS, to allow configuring WUS resource within or outside the DL BWP) |
| WUS_codepoint_CONNECTED | To configure the codepoint(s) that UE monitors per MO. The maximum number of codepoints checked per MO by a UE is up to 8. | Slightly prefer per BWP If the UE capability of "number of codepoints per MO" is SCS dependent, then this parameter should be configured per BWP, to accommodate different SCS for different BWP. |
| Max_codepoints_CONNECTED | To configure the maximum number of codepoints that can be indicated by a LP-WUS | Per BWP (to align above WUS_codepoint_CONNECTED) |
| Time_offset_CONNECTED_Option1-1 | For WUS operation option 1-1, the time offset (time offset2 as in RAN1#120 agreement) determines the start of LP-WUS monitoring relative to the start of drx-onDurationTimer | Per cell group (The value range for the Time_offset_CONNECTED_Option1-1 is in unit of ms. Similar as Rel-16 DCP that the PS_offset is defined per cell group) |
| Time_offset_CONNECTED_Option1-2 | For WUS operation option 1-2, the time offset (time offset4 as in RAN1#120agreement) determines the start of UE PDCCH monitoring via starting the LPWUS_PDCCHMonitoringTimer after LP-WUS is detected. | Per cell group (The value range for the Time_offset_CONNECTED_Option1-2 is in unit of ms. Similar as Rel-16 DCP that the PS_offset is defined per cell group) |
| LPWUS_MO_CONNECTED_Option1-1 | Configuration for LP-WUS MOs for option 1-1, including at least the following: - Periodicity and time offset (time offset1 as agreed in RAN1#120). | Per BWP (Since SCS for different BWP can be different, the periodicity is configured as number of slots, it should be per BWP) |
| LPWUS_MO_CONNECTED_Option1-2 | Configuration for LP-WUS MOs for option 1-2, including at least the following: - Periodicity and time offset (time offset3 as agreed in RAN1#120). | Per BWP (Since SCS for different BWP can be different, the periodicity is configured as number of slots, it should be per BWP) |
| NumOfMO_Monitoring_Option 1-1 | For CONNECTED mode optoin 1-1, UE monitors the first Z LP-WUS MO(s) at or after time offset prior to a slot where the drx-onDurationTimer would start. Z is configured by RRC. | Per BWP |
| NumOfMO_Monitoring_Option 1-2 | For Option 1-2, UE can be configured with a number of MOs per periodicity. | Per BWP |
| LP-WUS_TCI_state(s)_CONNECTED | For LP-WUS in CONNECTED mode and Rel-17 unified TCI framework is NOT configured, RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them | Per BWP (since a list of TCI states defined in pdsch-Config is per BWP) |
| LP-WUS_unifiedTCI_CONNECTED | To configure LP-WUS to follow the Rel-17 unified TCI framework | Per cell group (note that Rel-17 unified TCI state is configured in ServingCellConfig) |
| LP-WUS-TransmitOtherPeriodicCSI | To enable/disable periodic CSI reporting other than L1-RSRP during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up | Per cell group and Pcell/PScell only |
| LP-WUS-TransmitPeriodicL1-RSRP | To enable/disable periodic L1-RSRP reporting during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up | Per cell group and Pcell/PScell only |
| LPWUS_PDCCHMonitoringTimer | For option 1-2, when LP-WUS is detected, the UE starts a new Timer. | Per cell group and Pcell/PScell only |

Proposal 9: Use Table 1 as starting point to discuss the Type of per BWP or per cell group configuration for WUS parameters used for connected mode operation.

In the following, we provide other details for the configuration of LP-WUS MOs for Option 1-1 and Option 1-2 respectively.

  - Details for Option 1-1 on LP-WUS MO configuration

- About LP-WUS periodicityMO-Option 1-1 and offsetMO-Option 1-1 for LPWUS-MOCONNECTED-Option1-1, since it is independently configured from the C-DRX periodicity/offset, then following values can be considered to offer network sufficient flexibility. Note that for WUS MO configuration, it is better to define the periodicity and offset from the slot perspective, similar as Rel-16 DCP.

  - sl1, NULL,

  - sl2, INTEGER {0..1},

  - sl4, INTEGER {0..3},

  - sl5, INTEGER {0..4},

  - sl8 INTEGER {0..7}

  - sl10, INTEGER {0..9}

  - sl16, INTEGER {0..15}

  - sl20, INTEGER {0..19}

  - sl40, INTEGER {0..39}

  - sl80, INTEGER {0..79}

  - sl160, INTEGER {0..159}

  - sl320, INTEGER {0..319}

  - sl640, INTEGER {0..639}

  - sl1280, INTEGER {0..1279}

  - sl2560, INTEGER {0..2559}

- LP-WUS time offset2 i.e., timeOffsetCONNECTEDOption1-1 is configured to indicate a time offset of LP-WUS, prior to a slot where the drx-onDurationTimer would start.

  - Different from Rel-16 DCP, the candidate values for time offset2 should account for the ramp up time for MR and the time needed for multiple code-points waking up different UEs to provide network sufficient flexibility.

    - On the time needed for multiple LP-WUS MOs for multiple code-points transmission, based on our evaluation in [4], the shortest duration for one LP-WUS MO can be 1 OFDM symbol for OFDM based LP-WUR, while the worst case for one LP-WUS MO for OOK based LP-WUR can be up to 5 slots. Therefore, for Time_offset_CONNECTED_Option1-1,

      - the minimum value can be calculated as (1 OFDM symbol WUS MO duration)* (4 MOs) + the shortest MR transition time (e.g., 3ms if supported) = (4 OFDM symbols)+3ms, for simplicity, it is around 1 slot + 3ms, considering 120KHz SCS, and using 0.125ms as granularity it can be expressed as 25.

      - Similarly, the maximum value can be calculated as (5 slot WUS MO duration)* (4 MOs) + the longest MR transition time (e.g., 33ms if supported) = (20 slots)+33ms, considering 15KHz SCS and using 0.125ms as granularity, it can be expressed as 53ms/0.125ms=424.

      - In summary, the values for timeOffsetCONNECTEDOption1-1 is in multiples of 0.125ms (milliseconds), INTEGER (25..424), where 25 corresponds to (25*0.125ms) =3.125ms, 26 corresponds to (26*0.125ms)=3.25ms and so on.

- About “the first Z i.e., numMO-Option 1-1 LP-WUS MO(s) at or after Time offset2” in the agreement, the candidate values on the numMO-Option 1-1 can be: INTEGER {1..4}

Proposal 10: For LP-WUS MOs in connected mode for Option 1-1,

- The LP-WUS periodicityMO-Option 1-1 and offsetMO-Option 1-1 provide the periodicity and offset of the LP-WUS MO, following candidate values can be considered:

  - sl1, NULL,

  - sl2, INTEGER {0..1},

  - sl4, INTEGER {0..3},

  - sl5, INTEGER {0..4},

  - sl8 INTEGER {0..7}

  - sl10, INTEGER {0..9}

  - sl16, INTEGER {0..15}

  - sl20, INTEGER {0..19}

  - sl40, INTEGER {0..39}

  - sl80, INTEGER {0..79}

  - sl160, INTEGER {0..159}

  - sl320, INTEGER {0..319}

  - sl640, INTEGER {0..639}

  - sl1280, INTEGER {0..1279}

  - sl2560, INTEGER {0..2559}

- timeOffsetCONNECTEDOption1-1 indicates a time offset where the UE starts monitoring LP-WUS at LP-WUS MO(s), prior to a slot where the drx-onDurationTimer would start

  - The values for timeOffsetCONNECTEDOption1-1 is in multiples of 0.125ms (milliseconds), INTEGER (25..424).

- UE monitors the first LP-WUS MO(s) provided by numMO-Option 1-1 at or after Time offset2 (determined by timeOffsetCONNECTEDOption1-1).

  - The candidate values on the numMO-Option 1-1 is: INTEGER {1..4}

  - Details for Option 1-2 on LP-WUS MO configuration

- The main use case for LPWUS-MOCONNECTED-Option1-2 is to provide more frequent WUS monitoring occasions to timely wake up MR and achieve the power saving benefit at the same time. Therefore, the periodicity for option 1-2 should be much smaller compared to option 1-1 to have a clear differentiation. Following candidate values can be considered for periodicityMO-Option 1-2 and offsetMO-Option 1-2:

  - sl1, NULL,

  - sl2, INTEGER {0..1},

  - sl4, INTEGER {0..3},

  - sl5, INTEGER {0..4},

  - sl8 INTEGER {0..7}

  - sl10, INTEGER {0..9}

  - sl16, INTEGER {0..15}

  - sl20, INTEGER {0..19}

  - sl40, INTEGER {0..39}

  - sl80, INTEGER {0..79}

- numMO-Option 1-2: we think INTEGER (1..4) is sufficient per periodicity.

  - The starting time for the subsequent MO(s) in case multiple MOs per periodicity is configured can adopt the same way as multiple MOs configured within one LO for RRC idle/inactive mode. That is: the start time location of a subsequent LP-WUS MO is determined implicitly based on the previous LP-WUS MO i.e., the LP-WUS MOs are located back-to-back.

- The UE monitors LP-WUS in the LP-WUS MOs and if triggered to wake up, starts a lpwus-PDCCHMonitoringTimer for PDCCH monitoring, after a timeOffsetCONNECTEDOption1-2 with respect to the start of the first WUS monitoring occasion from the number of WUS monitoring occasions per periodicity.

  - The timeOffsetCONNECTEDOption1-2 should consider the time for MR ramp up and multiple WUS reception(s) since the reference time for timeOffsetCONNECTEDOption1-2 is the start of the first WUS monitoring occasion. Following candidate values for timeOffsetCONNECTEDOption1-2 can be considered, ranging from 25 to 424 in multiple of 0.125ms, i.e., INTEGER {25..424}

  - The candidate values for lpwus-PDCCHMonitoringTimer can be following

    - ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60

Proposal 11: For LP-WUS MOs in connected mode for Option 1-2,

- The LP-WUS periodicityMO-Option 1-2 and offsetMO-Option 1-2 provide the periodicity and offset of the LP-WUS MO, following candidate values can be considered:

    - sl1, NULL,

    - sl2, INTEGER {0..1},

    - sl4, INTEGER {0..3},

    - sl5, INTEGER {0..4},

    - sl8 INTEGER {0..7}

    - sl10, INTEGER {0..9}

    - sl16, INTEGER {0..15}

    - sl20, INTEGER {0..19}

    - sl40, INTEGER {0..39}

    - sl80, INTEGER {0..79}

- The candidate values for numMO-Option 1-2 is INTEGER {1..4}

- The timeOffsetCONNECTEDOption1-2 indicates a time,  after which the UE starts lpwus-PDCCHMonitoringTimer with respect to the start of the first WUS monitoring occasion from the number of numMO-Option 1-2 WUS monitoring occasions per periodicity.

  - The candidate values for timeOffsetCONNECTEDOption1-2 is in multiples of 0.125ms (milliseconds), INTEGER (25..424).

  - The candidate values for lpwus-PDCCHMonitoringTimer can be ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60

- TCI-state for LP-WUS

Following agreements were achieved during RAN1#120bis meeting [3]:

Agreement Update previous agreement in red: Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable Agreement The case where a UE does NOT supports Rel-17 unified TCI framework but supports LP-WUS is supported in Rel-19. For the TCI state of LP-WUS in RRC CONNECTED mode, select one of the following in RAN1#121 for this case: - Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS - Alt2: RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them The case where a UE supports Rel-17 unified TCI framework and supports LP-WUS is supported in Rel-19. For the TCI state of LP-WUS in RRC CONNECTED mode, - When Rel-17 unified TCI framework is configured, LP-WUS follows the activated/indicated TCI-state by MAC-CE/DCI, same as other DL channels/signals (no change to DCI format) - When Rel-17 unified TCI framework is NOT configured, select one of the following in RAN1#121 for this case - Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS - Alt2: RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them Same alternative to be chosen for both cases.

For the TCI state of LP-WUS in RRC CONNECTED mode, there are two alternatives to be down-selected for the case where a UE does NOT support Rel-17 unified TCI framework:

- Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS

- Alt2: RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them

In our view, the LP-WUS transmission may target for a group of UEs, the TCI used for LP-WUS can be different from the TCI used for MR for a UE specific transmission. In addition, for reliability, the beam used for WUS is likely to be coarse, compared to the fine beam used by MR with updated CSI report. Therefore, Alt2 is preferred.

Proposal 12: For the case where a UE does NOT support Rel-17 unified TCI framework and the case where a UE supports Rel-17 unified TCI framework but is not configured with Rel-17 unified TCI, for the TCI state of LP-WUS in RRC CONNECTED mode, select Alt2 that RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them.

- For Alt.2, following new RRC parameters for configuring K TCI states for LP-WUS from the TCI states defined in pdsch-Config with tci-StatesToAddModList is needed.

- tci-StatesWUS-ToAddList SEQUENCE (SIZE(1..maxNrofTCI-StatesWUS)) OF TCI-StateId

- tci-StatesWUS-ToReleaseList SEQUENCE (SIZE(1..maxNrofTCI-StatesWUS)) OF TCI-StateId

  - maxNrofTCI-StateWUS = K = 64

- Suggestion on the terminologies used in CR 38.213

During the CR discussion phase, we think the following coresponding realstionship is assumed and adopted by editor, according to the text in CR 38.213 that “A WUS monitoring occasion includes N⋅M WUS receptions”:

- a WUS monitoring occasion  LO;

- WUS reception a WUS monitoring occasion;

We can follow this mapping based on the understand of editor, but also suggest to change some of “WUS reception” to “WUS reception occasion” to make it clearer. Since we find the usage of “WUS reception” in the current CR 28.213 is still not aligned. For example, in some texts of CR 38.213 (e.g., “a number of OOK symbols per OFDM symbol, the first RB, and [one or more] overlaid OFDM sequences per OOK symbol for WUS reception”), a WUS reception is understanded as a reception of a WUS, while in other texts (e.g., “A WUS monitoring occasion includes N⋅M WUS receptions”) a WUS reception is more like a WUS reception attempt. Moreover, we also suggest to align the description based on this mapping between RRC idle/inactive paragraph and RRC connected paragraph, as the TP suggested below with modification in red.

Proposal 13: Adopt the TP suggested below with modification in red.

## 10.4C PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE A UE configured with DRX mode operation and operating in the RRC_IDLE or RRC_INACTIVE state can be provided for LPSS/WUS reception - a number of OOK symbols per OFDM symbol, a first RB, and an overlaid OFDM sequence per OOK symbol for LPSS reception [4, TS 38.211], - a periodicity and time offset for LPSS receptions by LP-SS-periodicityoffset, - a number of OOK symbols per OFDM symbol, the first RB, and [one or more] overlaid OFDM sequences per OOK symbol for WUS reception [4, TS 38.211], and - a number of subgroups per paging occasion, by subgroupNumber-PO-LPWUS A UE determines to receive LPSS/WUS based on procedures defined in [17, TS 38.304]. A UE assumes that an SCS configuration for LPSS/WUS receptions is same as an SCS of the initial DL BWP and an SCS configuration of an SS/PBCH block the UE used to obtain SIB1.A UE monitors WUS monitoring occasions with a periodicity equal to the DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines paging occasions associated with a WUS monitoring occasion based on PO-to-LO association. Each WUS monitoring occasion starts a number of frames prior to the first of a number of paging frames associated with the one or more WUS monitoring occasions. Each number of frames is provided by LO-FrameOffsets. If multiple values for the number of frames provided by LO-FrameOffsets are larger than or equal to the value of XYZ, the UE monitors WUS starting at an WUS monitoring occasion corresponding to the smallest of the multiple values. If all values for the number of frames provided by LO-FrameOffsets are smaller than the value of XYZ, the UE monitors PDCCH according to Type2-PDCCH CSS sets associated with the paging occasion and does not monitor WUS. A WUS monitoring occasion includes N⋅M WUS reception occasions, where - N is the number of [transmitted] SS/PBCH blocks as provided by [ssb-PositionsInBurst in SIB1], M is a number of WUS reception occasions associated with each of the N [transmitted] SS/PBCH blocks provided by MONumperLO - a WUS reception occasion with index n⋅M+m+1, m=0,1,…,M−1 and n=0,1,…,N−1, corresponds to [transmitted] SS/PBCH block [with index n+1], and - a WUS reception occasion with index n⋅M+m+1 is quasi co-located with a n-th [transmitted] SS/PBCH block provided by [ssb-PositionsInBurst in SIB1], with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable A UE assumes a periodicity for LPSS receptions is TBD and time occasions for LPSS receptions within an LPSS period are TBD. An LPSS reception for a (n+1)-th time occasion is quasi co-located with a (n+1)-th transmitted SS/PBCH block provided by [ssb-PositionsInBurst in SIB1], with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable. When, from a WUS reception, a UE determines a subgroup associated with the UE [17, TS 38.304], the UE performs PDCCH monitoring according to Type2-PDCCH CSS sets for the paging occasion associated with the WUS monitoring occasion reception when a time from the end of the WUS reception to the start of the PDCCH monitoring occasion is not smaller than the value of XYZ; otherwise, the UE is not required to perform the PDCCH monitoring. The UE may also perform PDCCH monitoring for Type3-PDCCH CSS sets for DCI format 2_7 based on its implementation. ## 10.4D PDCCH monitoring activation by WUS in RRC_CONNECTED A UE configured with DRX mode operation and operating in the RRC_CONNECTED state can be provided for WUS reception on the primary cell of a cell group - a number of OOK symbols per OFDM symbol, a first RB, and overlaid OFDM sequences per OOK symbol for WUS reception [4, TS 38.211], and - a number of [codepoints] provided for the UE by the WUS [6, TS 38.212], by WUS-codepointCONNECTED A UE assumes that a WUS is quasi co-located with an SS/PBCH block or a CSI-RS with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable. A UE assumes that an SCS configuration for WUS reception is same as an SCS configuration for the active DL BWP. A UE does not monitor a WUS during Active Time [11, TS 38.321]. A UE does not monitor WUS during DTX inactive period for the primary cell. If a UE detects a [codepoint/WUS information bit sequence] in a WUS reception occasion, from the number of [codepoints/WUS information bit sequences], on the primary cell of the cell group, the UE starts monitoring PDCCH on all applicable serving cells of the cell group. A UE can be provided by LPWUS-MOCONNECTED-Option1-1 a periodicity, by periodicityMO-Option 1-1, and a time offset, by offsetMO-Option 1-1, to determine WUS reception occasions. The UE starts to monitor WUS in a first WUS reception occasion, provided by numMO-Option 1-1, that is not earlier than a first slot that is prior to a second slot where the drx-onDurationTimer would start by a time provided by timeOffsetCONNECTEDOption1-1, and monitors WUS for a number of monitoring occasions provided by numMO-Option 1-1. The UE reports a number of slots [18, TS 38.306] where the UE is not required to monitor WUS prior to the slot where the drx-onDurationTimer would start, and is not required to monitor WUS within the reported number of slots prior to the slot where the drx-onDurationTimer would start. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts the drx-onDurationTimer [11, TS 38.321]. A UE can be provided by LPWUS-MOCONNECTED-Option1-2 a periodicity, by periodicityMO-Option 1-2, and a time offset, by offsetMO-Option 1-2, to determine first WUS reception occasions from a number of WUS reception occasions per periodicity, provided by numMO-Option 1-2. The UE reports a number of slots [18, TS 38.306] and expects that a time gap, from a last WUS reception occasion from the number of WUS monitoring occasions per periodicity to the slot where the lpwus-PDCCHMonitoringTimer would start, is no smaller than the reported number of slots. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts lpwus-PDCCHMonitoringTimer [11, TS 38.321] after a time, provided by timeOffsetCONNECTEDOption1-2, with respect to the start of the first WUS reception occasion from the number of WUS reception occasions per periodicity. ## *** Unchanged parts are omitted ***

- Conclusions

In this contribution, we provide our views on LP-WUS operation for UEs in RRC CONNECTED modes. The proposals are summarized as follows.

Proposal 1:

- LP-WUR and MR cannot work simultaneously should be UE basic capability.

- LP-WUR and MR can work simultaneously can be optionally supported based on UE capability.

- FFS how fast the switching can be done between the MR and LR.

Proposal 2: The understanding on “UE is not able to monitor LP-WUS” in RAN2’s LS is because MR is active and UE does not support LP-WUR and MR work simultaneously. Based on this understanding, RAN1 agree the cases/scenarios listed in RAN2’s LS with adding one more case as following:

- Such cases/scenarios resulting UE is not able to monitor LP-WUS include collision with Active Time, measurement gap, interruption caused by BWP switching, and RAR window monitoring for BFR, PDCCH monitoring associated with PDCCH CSS sets except for a Type3-PDCCH CSS set.

Proposal 3: A UE does not start the drx-OnDurationTimer for LP-WUS connected mode operation Option 1-1 or LPWUS_PDCCHMonitoringTimer for LP-WUS connected mode operation Option 1-2 when the LP-WUS monitoring occasion(s) collide with DTX inactive period for the primary cell.

Proposal 4: For LP-WUS connected mode Option 1-2, for the cases/scenarios on when the UE is not able to monitor the LP-WUS occasion(s), the UE does not start the LP-WUS_PDCCHMonitoringTimer .

Proposal 5: The preferred time offset indicated via UAI value(s) should be equal to or longer than the minimum time gap reported by UE capability.

Proposal 6: Adopt the following update for TS 38.214:

5.1.6.1 CSI-RS reception procedure <omitted text> If the UE is configured with DRX and to monitor either DCI format 2_6 or [LP-WUS], - if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI or [LP-WUS TransmitOtherPeriodicCSI] to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'cri-RSRP-Index', 'ssb-Index-RSRP' and 'ssb-Index-RSRP-Index' - when drx-onDurationTimer in DRX-Config is not started in case DCI format 2_6 or [LP-WUS option 1-1] is configured, or - when WUS_PDCCHMonitoringTimer [in XYZxxx] has not been started during the time of the previous DRX cycle in case [LP-WUS option 1-2] is configured, - the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported; - if the UE is configured by higher layer parameter ps-TransmitPeriodicL1-RSRP or [LP-WUS TransmitPeriodicL1-RSRP] to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP' or 'cri-RSRP-Index' - when drx-onDurationTimer in DRX-Config is not started in case DCI format 2_6 or [LP-WUS option 1-1] is configured, or - when WUS_PDCCHMonitoringTimer [in XYZxxx] has not been started during the time of the previous DRX cycle in case [LP-WUS option 1-2] is configured, - the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported. <omitted text> 5.2.2.5 CSI reference resource definition <omitted text> When the UE is configured to monitor DCI format 2_6 or [WUS], - if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI or [LPWUS-TransmitOtherPeriodicCSI] to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', and 'ssb-Index-RSRP- Index ' - when drx-onDurationTimer in DRX-Config is not started in case DCI format 2_6 or [LP-WUS option 1-1] is configured, or - when WUS_PDCCHMonitoringTimer [in XYZxxx] has not been started during the time of the previous DRX cycle in case [LP-WUS option 1-2] is configured, - the UE shall report CSI with the reportQuantity not set to ‘ssb-Index-SINR’ or ‘ssb-Index-SINR-Index’ during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. - if the UE is configured with a CSI report configuration containing a list of sub-configurations provided by csi-ReportSubConfigToAddModList, and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI or [LPWUS-TransmitOtherPeriodicCSI] to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', and 'ssb-Index-RSRP- Index' - when drx-onDurationTimer in DRX-Config is not started in case DCI format 2_6 or [LP-WUS option 1-1] is configured, or - when WUS_PDCCHMonitoringTimer [in XYZxxx] has not been started during the time of the previous DRX cycle in case [LP-WUS option 1-2] is configured, - UE shall report a CSI report including one or more sub-reports only during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time, per sub-configuration, no later than CSI reference resource and drops the report otherwise, where the sub-configuration is the configured one for P-CSI reporting. - if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP or [LPWUS-TransmitOtherPeriodicL1-RSRP] to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', or 'ssb-Index-RSRP- Index' - when drx-onDurationTimer in DRX-Config is not started in case DCI format 2_6 or [LP-WUS option 1-1] is configured, or - when WUS_PDCCHMonitoringTimer [in XYZxxx] has not been started during the time of the previous DRX cycle in case [LP-WUS option 1-2] is configured, - the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' or 'cri-RSRP- Index' if receiving at least one CSI-RS transmission occasion for channel measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. <omitted text>

Proposal 7:

- Support V1=[3+TWUS-processing-time]ms, V2=[10+TWUS-processing-time]ms and V3=[30+TWUS-processing-time]ms, where TWUS-processing-time is the LP-WUS processsing time, as candidate values for the minimum time gap reported by the UE capability.

  - LP-WUS processsing time, i.e., TWUS-processing-time is less than 3ms, up to 1ms can be the considered.

- The same set of values is applied for different receiver types.

Proposal 8: When LP-WUS monitoring is enabled for RRC CONNECTED mode, UE by implementation can fallback to PDCCH monitoring when UE monitors LP-WUS without any specification work.

- No need to specify any condition for the fallback

- No need to have indication from UE when it falls back to PDCCH monitoring.

- For LP-WUS operation option 1-1, the fallback behavior for PDCCH monitoring can be according to the C-DRX configuration

- For LP-WUS operation option 1-2, the fallback behavior for PDCCH monitoring can be always-on PDCCH monitoring.

Proposal 9: Use Table 1 as starting point to discuss the Type of per BWP or per cell group configuration for WUS parameters used for connected mode operation.

Table 1 Views on the Type (per BWP or per cell group) for WUS Parameters in connected mode

| Parameter Name | Description/Function | Type: Per BWP or Per Cell group |
| --- | --- | --- |
| LP-WUS_Mvalue_CONNECTED | Indicates the number of OOK symbols in an OFDM symbol in RRC_CONNECTED | Per BWP (since SCS for different BWP can be different, the suitable M value can also be different) |
| LP-WUS_overlaidSeq_roots_CONNECTED | Configuration of one or two roots for the overlaid OFDM sequence used per OOK symbol for LP-WUS in CONNECTED | Per BWP (since SCS for different BWP can be different, the suitable M value can also be different, different roots can be configured for each M as agreed) |
| LP-WUS_num_overlaidseq_CONNECTED | Configures the number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, - maximum 16 candidates overlaid sequences for M=1 - maximum 8 candidates overlaid sequences for M=2 - maximum 4 candidates overlaid sequences for M=4 | Per BWP (since SCS for different BWP can be different, the suitable M value can also be different, thus different number of sequences) |
| LP-WUS_startRB_CONNECTED | Indicates the starting RB of LP-WUS in RRC CONNECTED | Per BWP (Since SCS for different BWP can be different, the configured starting RB should be relative to carrier boundary corresponding to the SCS, to allow configuring WUS resource within or outside the DL BWP) |
| WUS_codepoint_CONNECTED | To configure the codepoint(s) that UE monitors per MO. The maximum number of codepoints checked per MO by a UE is up to 8. | Slightly prefer per BWP If the UE capability of "number of codepoints per MO" is SCS dependent, then this parameter should be configured per BWP, to accommodate different SCS for different BWP. |
| Max_codepoints_CONNECTED | To configure the maximum number of codepoints that can be indicated by a LP-WUS | Per BWP (to align above WUS_codepoint_CONNECTED) |
| Time_offset_CONNECTED_Option1-1 | For WUS operation option 1-1, the time offset (time offset2 as in RAN1#120 agreement) determines the start of LP-WUS monitoring relative to the start of drx-onDurationTimer | Per cell group (The value range for the Time_offset_CONNECTED_Option1-1 is in unit of ms. Similar as Rel-16 DCP that the PS_offset is defined per cell group) |
| Time_offset_CONNECTED_Option1-2 | For WUS operation option 1-2, the time offset (time offset4 as in RAN1#120agreement) determines the start of UE PDCCH monitoring via starting the LPWUS_PDCCHMonitoringTimer after LP-WUS is detected. | Per cell group (The value range for the Time_offset_CONNECTED_Option1-2 is in unit of ms. Similar as Rel-16 DCP that the PS_offset is defined per cell group) |
| LPWUS_MO_CONNECTED_Option1-1 | Configuration for LP-WUS MOs for option 1-1, including at least the following: - Periodicity and time offset (time offset1 as agreed in RAN1#120). | Per BWP (Since SCS for different BWP can be different, the periodicity is configured as number of slots, it should be per BWP) |
| LPWUS_MO_CONNECTED_Option1-2 | Configuration for LP-WUS MOs for option 1-2, including at least the following: - Periodicity and time offset (time offset3 as agreed in RAN1#120). | Per BWP (Since SCS for different BWP can be different, the periodicity is configured as number of slots, it should be per BWP) |
| NumOfMO_Monitoring_Option 1-1 | For CONNECTED mode optoin 1-1, UE monitors the first Z LP-WUS MO(s) at or after time offset prior to a slot where the drx-onDurationTimer would start. Z is configured by RRC. | Per BWP |
| NumOfMO_Monitoring_Option 1-2 | For Option 1-2, UE can be configured with a number of MOs per periodicity. | Per BWP |
| LP-WUS_TCI_state(s)_CONNECTED | For LP-WUS in CONNECTED mode and Rel-17 unified TCI framework is NOT configured, RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them | Per BWP (since a list of TCI states defined in pdsch-Config is per BWP) |
| LP-WUS_unifiedTCI_CONNECTED | To configure LP-WUS to follow the Rel-17 unified TCI framework | Per cell group (note that Rel-17 unified TCI state is configured in ServingCellConfig) |
| LP-WUS-TransmitOtherPeriodicCSI | To enable/disable periodic CSI reporting other than L1-RSRP during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up | Per cell group and Pcell/PScell only |
| LP-WUS-TransmitPeriodicL1-RSRP | To enable/disable periodic L1-RSRP reporting during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up | Per cell group and Pcell/PScell only |
| LPWUS_PDCCHMonitoringTimer | For option 1-2, when LP-WUS is detected, the UE starts a new Timer. | Per cell group and Pcell/PScell only |

Proposal 10: For LP-WUS MOs in connected mode for Option 1-1,

- The LP-WUS periodicityMO-Option 1-1 and offsetMO-Option 1-1 provide the periodicity and offset of the LP-WUS MO, following candidate values can be considered:

  - sl1, NULL,

  - sl2, INTEGER {0..1},

  - sl4, INTEGER {0..3},

  - sl5, INTEGER {0..4},

  - sl8 INTEGER {0..7}

  - sl10, INTEGER {0..9}

  - sl16, INTEGER {0..15}

  - sl20, INTEGER {0..19}

  - sl40, INTEGER {0..39}

  - sl80, INTEGER {0..79}

  - sl160, INTEGER {0..159}

  - sl320, INTEGER {0..319}

  - sl640, INTEGER {0..639}

  - sl1280, INTEGER {0..1279}

  - sl2560, INTEGER {0..2559}

- timeOffsetCONNECTEDOption1-1 indicates a time offset where the UE starts monitoring LP-WUS at LP-WUS MO(s), prior to a slot where the drx-onDurationTimer would start

  - The values for timeOffsetCONNECTEDOption1-1 is in multiples of 0.125ms (milliseconds), INTEGER (25..424).

- UE monitors the first LP-WUS MO(s) provided by numMO-Option 1-1 at or after Time offset2 (determined by timeOffsetCONNECTEDOption1-1).

  - The candidate values on the numMO-Option 1-1 is: INTEGER {1..4}

Proposal 11: For LP-WUS MOs in connected mode for Option 1-2,

- The LP-WUS periodicityMO-Option 1-2 and offsetMO-Option 1-2 provide the periodicity and offset of the LP-WUS MO, following candidate values can be considered:

    - sl1, NULL,

    - sl2, INTEGER {0..1},

    - sl4, INTEGER {0..3},

    - sl5, INTEGER {0..4},

    - sl8 INTEGER {0..7}

    - sl10, INTEGER {0..9}

    - sl16, INTEGER {0..15}

    - sl20, INTEGER {0..19}

    - sl40, INTEGER {0..39}

    - sl80, INTEGER {0..79}

- The candidate values for numMO-Option 1-2 is INTEGER {1..4}

- The timeOffsetCONNECTEDOption1-2 indicates a time,  after which the UE starts lpwus-PDCCHMonitoringTimer with respect to the start of the first WUS monitoring occasion from the number of numMO-Option 1-2 WUS monitoring occasions per periodicity.

  - The candidate values for timeOffsetCONNECTEDOption1-2 is in multiples of 0.125ms (milliseconds), INTEGER (25..424).

  - The candidate values for lpwus-PDCCHMonitoringTimer can be ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60

Proposal 12: For the case where a UE does NOT support Rel-17 unified TCI framework and the case where a UE supports Rel-17 unified TCI framework but is not configured with Rel-17 unified TCI, for the TCI state of LP-WUS in RRC CONNECTED mode, select Alt2 that RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them.

- For Alt.2, following new RRC parameters for configuring K TCI states for LP-WUS from the TCI states defined in pdsch-Config with tci-StatesToAddModList is needed.

- tci-StatesWUS-ToAddList SEQUENCE (SIZE(1..maxNrofTCI-StatesWUS)) OF TCI-StateId

- tci-StatesWUS-ToReleaseList SEQUENCE (SIZE(1..maxNrofTCI-StatesWUS)) OF TCI-StateId

  - maxNrofTCI-StateWUS = K = 64

Proposal 13: Adopt the TP suggested below with modification in red.

## 10.4C PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE A UE configured with DRX mode operation and operating in the RRC_IDLE or RRC_INACTIVE state can be provided for LPSS/WUS reception - a number of OOK symbols per OFDM symbol, a first RB, and an overlaid OFDM sequence per OOK symbol for LPSS reception [4, TS 38.211], - a periodicity and time offset for LPSS receptions by LP-SS-periodicityoffset, - a number of OOK symbols per OFDM symbol, the first RB, and [one or more] overlaid OFDM sequences per OOK symbol for WUS reception [4, TS 38.211], and - a number of subgroups per paging occasion, by subgroupNumber-PO-LPWUS A UE determines to receive LPSS/WUS based on procedures defined in [17, TS 38.304]. A UE assumes that an SCS configuration for LPSS/WUS receptions is same as an SCS of the initial DL BWP and an SCS configuration of an SS/PBCH block the UE used to obtain SIB1.A UE monitors WUS monitoring occasions with a periodicity equal to the DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines paging occasions associated with a WUS monitoring occasion based on PO-to-LO association. Each WUS monitoring occasion starts a number of frames prior to the first of a number of paging frames associated with the one or more WUS monitoring occasions. Each number of frames is provided by LO-FrameOffsets. If multiple values for the number of frames provided by LO-FrameOffsets are larger than or equal to the value of XYZ, the UE monitors WUS starting at an WUS monitoring occasion corresponding to the smallest of the multiple values. If all values for the number of frames provided by LO-FrameOffsets are smaller than the value of XYZ, the UE monitors PDCCH according to Type2-PDCCH CSS sets associated with the paging occasion and does not monitor WUS. A WUS monitoring occasion includes N⋅M WUS reception occasions, where - N is the number of [transmitted] SS/PBCH blocks as provided by [ssb-PositionsInBurst in SIB1], M is a number of WUS reception occasions associated with each of the N [transmitted] SS/PBCH blocks provided by MONumperLO - a WUS reception occasion with index n⋅M+m+1, m=0,1,…,M−1 and n=0,1,…,N−1, corresponds to [transmitted] SS/PBCH block [with index n+1], and - a WUS reception occasion with index n⋅M+m+1 is quasi co-located with a n-th [transmitted] SS/PBCH block provided by [ssb-PositionsInBurst in SIB1], with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable A UE assumes a periodicity for LPSS receptions is TBD and time occasions for LPSS receptions within an LPSS period are TBD. An LPSS reception for a (n+1)-th time occasion is quasi co-located with a (n+1)-th transmitted SS/PBCH block provided by [ssb-PositionsInBurst in SIB1], with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable. When, from a WUS reception, a UE determines a subgroup associated with the UE [17, TS 38.304], the UE performs PDCCH monitoring according to Type2-PDCCH CSS sets for the paging occasion associated with the WUS monitoring occasion reception when a time from the end of the WUS reception to the start of the PDCCH monitoring occasion is not smaller than the value of XYZ; otherwise, the UE is not required to perform the PDCCH monitoring. The UE may also perform PDCCH monitoring for Type3-PDCCH CSS sets for DCI format 2_7 based on its implementation. ## 10.4D PDCCH monitoring activation by WUS in RRC_CONNECTED A UE configured with DRX mode operation and operating in the RRC_CONNECTED state can be provided for WUS reception on the primary cell of a cell group - a number of OOK symbols per OFDM symbol, a first RB, and overlaid OFDM sequences per OOK symbol for WUS reception [4, TS 38.211], and - a number of [codepoints] provided for the UE by the WUS [6, TS 38.212], by WUS-codepointCONNECTED A UE assumes that a WUS is quasi co-located with an SS/PBCH block or a CSI-RS with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable. A UE assumes that an SCS configuration for WUS reception is same as an SCS configuration for the active DL BWP. A UE does not monitor a WUS during Active Time [11, TS 38.321]. A UE does not monitor WUS during DTX inactive period for the primary cell. If a UE detects a [codepoint/WUS information bit sequence] in a WUS reception occasion, from the number of [codepoints/WUS information bit sequences], on the primary cell of the cell group, the UE starts monitoring PDCCH on all applicable serving cells of the cell group. A UE can be provided by LPWUS-MOCONNECTED-Option1-1 a periodicity, by periodicityMO-Option 1-1, and a time offset, by offsetMO-Option 1-1, to determine WUS reception occasions. The UE starts to monitor WUS in a first WUS reception occasion, provided by numMO-Option 1-1, that is not earlier than a first slot that is prior to a second slot where the drx-onDurationTimer would start by a time provided by timeOffsetCONNECTEDOption1-1, and monitors WUS for a number of monitoring occasions provided by numMO-Option 1-1. The UE reports a number of slots [18, TS 38.306] where the UE is not required to monitor WUS prior to the slot where the drx-onDurationTimer would start, and is not required to monitor WUS within the reported number of slots prior to the slot where the drx-onDurationTimer would start. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts the drx-onDurationTimer [11, TS 38.321]. A UE can be provided by LPWUS-MOCONNECTED-Option1-2 a periodicity, by periodicityMO-Option 1-2, and a time offset, by offsetMO-Option 1-2, to determine first WUS reception occasions from a number of WUS reception occasions per periodicity, provided by numMO-Option 1-2. The UE reports a number of slots [18, TS 38.306] and expects that a time gap, from a last WUS reception occasion from the number of WUS monitoring occasions per periodicity to the slot where the lpwus-PDCCHMonitoringTimer would start, is no smaller than the reported number of slots. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts lpwus-PDCCHMonitoringTimer [11, TS 38.321] after a time, provided by timeOffsetCONNECTEDOption1-2, with respect to the start of the first WUS reception occasion from the number of WUS reception occasions per periodicity. ## *** Unchanged parts are omitted ***

# References

- R1-2503616, LS on LP-WUS in RRC_CONNECTED, RAN2, InterDigital

- R1-2503343, Draft reply LS on LP-WUS in RRC_CONNECTED, vivo

- R1-2503482, Introduction of specification support for WUS functionality	, Nokia

- R1-2501814, Discussion on LP-WUS and LP-SS design, vivo