3GPP TSG RAN WG1 #121		                                               R1-2503396

St Julian’s, Malta, May 19th – 23rd, 2025

Source: 	vivo

Title:	Discussion on UE features for Low band carrier aggregation via switching

Agenda Item:	9.15.12

Document for:	Discussion and Decision

- Introduction

In the RAN#106 meeting, a new WID on low band carrier aggregation via switching was approved[1]. The objectives of the LB-CA work item are shown as follows. In this contribution, the UE capability discussion on the low band carrier aggregation via switching are provided.

- Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

- Discussion

Since the low band CA via switching has been agreed to be supported, a new FG supporting low band CA via switching should be introduced. Furthermore, the FG can include a component that a restriction of maximum X switch(es) within Y slot(s).

Proposal 1: Introduce a UE capability supporting Low band CA via switching which includes a component that a restriction of maximum X switch(es) within Y slot(s).

- Conclusion

In this contribution, the proposal is summarized as follows:

Proposal 1: Introduce a UE capability supporting Low band CA via switching which includes a component that a restriction of maximum X switch(es) within Y slot(s).

Proposal 2: The UE feature is list in Table 1.

Table 1 UE feature for LB CA via switching

| Feature | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X. NR_LBCA_Sw | X-1 | Supporting Low band CA via switching | - Support of Low band CA via switching with a switching pattern configured by RRC. - A restriction of maximum X switch(es) within Y slot(s). FFS: the value of X and Y. - The configured switching pattern shall guarantee that UE does not stay continuously on SDL carrier for longer than 15 slots. | N/A | Yes | N/A | UE does not support Low band CA via switching | Per BC | N/A | N/A | N/A | This UE feature applies to the band combination CA_n5A-n29A | Optional with capability signaling |

# Reference

- RP-243317 	New WID on low band carrier aggregation via switching