3GPP TSG RAN WG1 #121			R1-2503513

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.3.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on SBFD random access operation

Document for:	Discussion and decision

Introduction

In RAN#104, the revised WI on evolution of NR duplex operation was approved [1].

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

In this contribution, we discuss some technical issues related to random access in subband non-overlapping full duplex (SBFD), including both of RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode.

Discussion

Issues in 38.211

Reason for change:

The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs only for the second PRACH occasions. While the first PRACH occasions still use the legacy method. nRAstart is still with respect to physical resource block 0 of the active UL BWP, only msg1-FrequencyStart is reinterpreted.

| Original spec | - nRAstart is the frequency offset of the lowest PRACH transmission occasion in frequency domain with respect to physical resource block 0 of the active uplink bandwidth part. The quantity nRAstart is given by the higher-layer parameter msgA-RO-FrequencyStart if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213], otherwise by msg1-FrequencyStart as described in clause 8.1 of [5 TS 38.213]; |
| --- | --- |
|  | Agreement RAN#119 For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, Alt 2-1 is supported. - Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - ROs that are outside of UL usable PRBs are invalid |
| Draft CR | - nRAstart is the frequency offset of the lowest PRACH transmission occasion in frequency domain and is given by the higher-layer parameter msgA-RO-FrequencyStart if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213], otherwise by msg1-FrequencyStart as described in clause 8.1 of [5 TS 38.213] - if the higher-layer parameter sbfd-RACHSingleConfig is configured, the quantity nRAstart is defined relative the lowest-numbered usable uplink physical resource block; - otherwise, the quantity nRAstart is defined relative to physical resource block 0 of the active uplink bandwidth part. |
| Proposed TP#1 on the top of original spec | - nRAstart is the frequency offset of the lowest PRACH transmission occasion in frequency domain with respect to physical resource block 0 of the active uplink bandwidth part. The quantity nRAstart is given by the higher-layer parameter msgA-RO-FrequencyStart if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213], otherwise by msg1-FrequencyStart as described in clause 8.1 of [5 TS 38.213]; - if the higher-layer parameter sbfd-RACHSingleConfig is configured, the quantity msg1-FrequencyStart is relative to the lowest-numbered the PRBs that are both in the active UL BWP and in the UL sub-band with reference to the start of UL active BWP for the second PRACH occasions; |

- TP#1 can be adopted to nRAstart in 38.211

Issues in 38.213

First/second PRACH occasions

Reason for change:

The first PRACH occasions are legacy ROs, so it does not matter flexible symbol by tdd-UL-DL-ConfigurationCommon is SBFD symbol or not for the yellow highlight sentence, for both of RACH configuration 1 and 2.

The second PRACH occasions of RACH configuration 2 are configured by the additional RACH configuration, not the legacy in rach-ConfigCommon. Thus, the second PRACH occasion can be all SBFD symbols configured as flexible symbols, which is different from RACH configuration 1. It would be very hard to define the first/second PRACH occasions by the symbol type for Configuration 2, such as the bule highlight part can also be first PRACH occasions if it is configured by rach-ConfigCommon. Therefore, we suggest distinguishing them by rach-ConfigCommon and sbfd-RACH-DualConfig.

|  | First PRACH occasions | Second PRACH occasions |
| --- | --- | --- |
| RACH configuration 1 | symbols that are indicated as uplink or flexible configured as SBFD or non-SBFD symbol by tdd-UL-DL-ConfigurationCommon | SBFD symbols with at least one DL SBFD symbols |
| RACH configuration 2 | symbols that are indicated as uplink or flexible configured as SBFD or non-SBFD symbol by tdd-UL-DL-ConfigurationCommon, configured by rach-ConfigCommon | only with SBFD symbols (DL and/or flexible) configured by sbfd-RACH-DualConfig, or, with non-SBFD and SBFD symbols (DL and/or flexible) configured by sbfd-RACH-DualConfig- ValidROAcrossSymbolTypes |

We suggest to the following red part changes for first and second PRACH occasions.

| Draft CR in section 8 | In the remaining of this clause, when a symbol is not stated as an SBFD symbol, the symbol is a non-SBFD symbol. Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a random access procedure using: - first PRACH occasions each including only symbols that are indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon and considered as uplink for the random access procedure, or - second PRACH occasions associated either only with SBFD symbols that include at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon, when the UE is provided sbfd-RACHSingleConfig, or associated only with SBFD symbols when the UE is provided sbfd-RACHDualConfig and is not provided sbfd-RACHDualConfig-ValidROAcrossSymbolTypes, or associated both with non-SBFD and SBFD symbols when the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes |
| --- | --- |
| Proposed TP#2 changes on the top of draft CR: | In the remaining of this clause, when a symbol is not stated as an SBFD symbol, the symbol is a non-SBFD symbol. Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a random access procedure using: - first PRACH occasions each including only symbols that are indicated as uplink or flexible configured as SBFD or non-SBFD symbol by tdd-UL-DL-ConfigurationCommon, and associated with rach-ConfigCommon considered as uplink for the random access procedure, or - second PRACH occasions associated either only with SBFD symbols that include at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon, when the UE is provided sbfd-RACHSingleConfig, or associated with sbfd-RACHDualConfig and only with SBFD symbols when the UE is provided sbfd-RACHDualConfig and is not provided sbfd-RACHDualConfig-ValidROAcrossSymbolTypes, or associated with sbfd-RACHDualConfig and both with non-SBFD and SBFD symbols when the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes |

- TP#2 can be adopted to first and second PRACH occasions.

Validation rule of RO

Reason for change:

Since the procedures in clause 8.1 apply separately for the first PRACH occasions and the second PRACH occasions, the first PRACH occasions with flexible symbol are assumed valid due to it is within SBFD symbol and without check the SSB positions in the third bullet. It makes a wrong validation decisions. Thus, it is good to separate the validation procedure for the first and second PRACH occasions.

| Draft CR in section 8.1 | - if a UE is provided tdd-UL-DL-ConfigurationCommon for a cell, a PRACH occasion for the cell in a PRACH slot is valid if - it is only within UL symbols, or - it is only within SBFD symbols, that include at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon, and in RBs that are both in the active UL BWP and in the UL sub-band if the UE is provided either sbfd-RACHSingleConfig or sbfd-RACHDualConfig, or it starts from an SBFD symbol and ends in a non-SBFD symbols and is in RBs that are both in the active UL BWP and in the UL sub-band if the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes, or - it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if channelAccessMode = "semiStatic" is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213] - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or in SSB-MTC-AdditionalPCI corresponding to the cell, as described in clause 4.1 |
| --- | --- |
| Proposed TP#3 changes on the top of original specification: | - if a UE is provided tdd-UL-DL-ConfigurationCommon, a first PRACH occasion for a cell in a PRACH slot is valid if - it is within UL symbols, or - it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if channelAccessMode = "semiStatic" is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213] - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or in SSB-MTC-AdditionalPCI corresponding to the cell, as described in clause 4.1 - if a UE is provided tdd-UL-DL-ConfigurationCommon, a second PRACH occasion for a cell in a PRACH slot is valid if - it is only within SBFD symbols, that include at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon, and in RBs that are both in the active UL BWP and in the UL sub-band if the UE is provided either sbfd-RACHSingleConfig or sbfd-RACHDualConfig, or it starts from an SBFD symbol and ends in a non-SBFD symbols and is in RBs that are both in the active UL BWP and in the UL sub-band if the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes, and - it does not overlap with SS/PBCH block and starts at least Ngap symbols after a last non-SBFD symbol indicated as downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2. - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon |

- TP#3 can be adopted to the validation rule of first and second PRACH occasions.

MSG3 frequency resources and frequency hopping

Reason for change:

For frequency offset for 2nd hop is based on the size of UL usable PRBs, but the current draft CR may implies it also determine the NUL,hop Hopping Bits, which is against the agreement. So it can make clear it is only for frequency offset in Table 8.3-1. And it is only applied if PRACH transmission is over additional ROs.

|  | Agreement RAN#119 For SBFD aware UEs, for Msg3 PUSCH transmission, - the number of NUL,hop hopping bits is based on the number of PRBs in initial UL BWP as in legacy specification. - the frequency offset for the 2nd hop is reserved for the case when the value of NUL,hop= 11 as in legacy specification. Agreement RAN1#120b For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the random access procedure. - FFS: Whether specification change is necessary |
| --- | --- |
| Draft CR in section 8.3 | If the PUSCH transmission is in SBFD symbols, as described in clause 11.1, NBWPsize in Table 8.3-1 is the number of RBs that are in the active UL BWP and in the UL sub-band. |
| Proposed TP#4 changes on the top of draft CR: | If the PUSCH transmission is associated with a PRACH transmission in second PRACH occasions and over SBFD symbols, as described in clause 11.1, NBWPsize determining the frequency offset for 2nd hop in 3rd column of Table 8.3-1 is the number of RBs that are in the active UL BWP and in the UL sub-band. |

- TP#4 can be adopted to MSG3 transmission.

PUCCH resource

Reason for change:

Before dedicated PUCH resource configuration, the lowest PRB indices of PUCCH resource in SBFD symbol was agreed as below. However, the draft CR may means if PRACH using the additional RO, PUCCH always only within UL usable PRBs, even it is in non-SBFD symbols due to the otherwise condition. If additional RO applied for PRACH transmission, PUCCH can be in SBFD symbols or non-SBFD symbols.

|  | Agreement RAN#119 For SBFD aware UEs, update the following equations to determine the lowest PRB indices of the PUCCH transmission in the first hop and second hop in SBFD symbols before dedicated PUCCH resource configuration: - if rPUCCH/8=0 - RBUL SB start+RBBWPoffset∙NRB+rPUCCH/NCS∙NRB for the first hop - RBUL SB start+NUL SBsize-RBBWPoffset∙NRB-(1+rPUCCH/NCS) ∙NRB for the second hop - if rPUCCH/8=1 - RBUL SB start+NUL SBsize-RBBWPoffset∙NRB-(1+(rPUCCH-8)/NCS) ∙NRB for the first hop - RBUL SB start+RBBWPoffset∙NRB+(rPUCCH-8)/NCS∙NRB for the second hop Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBBWPoffset is the RB offset with reference to the lowest PRB of UL usable PRBs. Note: Definitions of NRB, rPUCCH and NCS are unchanged from existing specifications. |
| --- | --- |
| Draft CR in section 9.2.1 | If the PUCCH transmission is associated with a PRACH transmission in first PRACH occasions, as described in clause 8, NBWPsize is the number of RBs in the active UL BWP and RBUL_SBstart=0; otherwise, NBWPsize is the number of RBs in the active UL BWP and in the UL sub-band and RBUL_SBstart is the index of the first RB from the RBs that are in the active UL BWP and in the UL sub-band as described in clause 11.1. |
| Proposed TP#5 changes on the top of draft CR: | If the PUCCH transmission is associated with a PRACH transmission in first PRACH occasions as described in clause 8, or over non-SBFD symbols as described in clause 11.1 and associated with second PRACH occasions as described in clause 8, NBWPsize is the number of RBs in the active UL BWP and RBUL_SBstart=0; otherwise, NBWPsize is the number of RBs in the active UL BWP and in the UL sub-band and RBUL_SBstart is the index of the first RB from the RBs that are in the active UL BWP and in the UL sub-band as described in clause 11.1. |

- TP#5 can be adopted to MSG4 HARQ-ACK PUCCH transmission.

Conclusion

In this contribution, we made the following proposals.

- TP#1 can be adopted to nRAstart in 38.211

- TP#2 can be adopted to first and second PRACH occasions.

- TP#3 can be adopted to the validation rule of first and second PRACH occasions.

- TP#4 can be adopted to MSG3 transmission.

- TP#5 can be adopted to MSG4 HARQ-ACK PUCCH transmission.

References

- RP-2401614	Revised WI on evolution of NR duplex operation

- R1-2503455	Introduction of sub-band full duplex (SBFD)	Ericsson

- R1-2503456	Editor’s summary on draft CR 38.211 for NR_duplex_evo	Moderator (Ericsson)

- R1-2503169	Introduction of evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R1-2503170	Summary of email discussions [120bis-R19-38.213-NR_duplex_evo]