3GPP TSG RAN WG1 #121	        		             R1-2503580

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	9.9.1

Source: 	Samsung

Title: 	Views on Rel-19 measurement related enhancements for LTM

Document for:	Discussion and Decision

# Introduction

In RAN#105, the WID for NR mobility enhancements Phase 4 was revised [1]. In particular, for measurement related enhancements, the corresponding descriptions in the WID were updated as follows:

Measurement related enhancements in the WID: - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

According to the updated WID, in addition to specifying support of CSI-RS measurements/CSI-RS based beam management for LTM procedures and necessary enhancements for the event triggered L1 measurement and reporting, RAN1 would also need to specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch. In this contribution, we provide our views on various design aspects/components related to the above subject matters descripted in the WID.

# Discussions on support for CSI-RS measurements for LTM procedures and enable CSI-RS based BM

Regarding configuration/association of CSI-RS resource(s) for L1 measurement(s) on/with candidate cell(s), the following agreements were respectively made in RAN1#118 and RAN1#120bis:

Agreement - Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported Agreement In the resource set configured for candidate cell L1-RSRP measurement, a sequence of candidate cell indices is included together with a sequence of CSI-RS resource indices, where each CSI-RS resource index is associated with one candidate cell index.

As agreed in RAN1, the CSI-RS resources can be explicitly provided in a resource set for candidate cell L1 measurement(s), and the resource set can also comprise a set of candidate cell IDs each associated to a configured CSI-RS resource in the set. In addition to the explicit association, implicit association is also possible, and both of them can leverage what has been done in Rel-18 LTM for associating between the configured SSB(s) and the candidate cell(s). For instance, the configured CSI-RS resource(s) can be implicitly associated to different candidate cells by referring to their QCL’ed source SSBs (and therefore, their associations to the different candidate cells).

Proposal 1: For associating between the configured CSI-RS resources and candidate cells for L1 measurement(s), further support

- (implicit association): association between the configured CSI-RS resource(s) and the candidate cell(s) is based on association between the SSBs (QCL’ed with the configured CSI-RS resource(s)) and the candidate cells

Another discussion point for the support of CSI-RS based measurement for LTM is whether the introduced CSI-RS resource for LTM measurement can be applicable for L3 measurement for mobility. From NW’s perspective, LTM procedure and L3 procedure can be enabled simultaneously to obtain UE mobility information in different time-domain granularities. For more efficient use of CSI-RS resource for measurement, one solution is to use shared CSI-RS resource for LTM and L3 mobility. For example, the CSI-RS introduced for LTM measurement can also be configured for L3 mobility.

Proposal 2: For gNB scheduled reporting and event triggered reporting, support shared CSI-RS resource for both LTM and L3 mobility, e.g., the CSI-RS introduced for LTM measurement can be configured for L3 mobility as well.

# CSI acquisition on candidate cell(s)

In RAN1#120 [2] and 120bis [3], the following agreements regarding procedure(s) related to CSI acquisition for LTM were made:

Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 and RAN3 to inform this agreement Agreement For the container of LTM CSI report to a target cell, UCI is used with a solution to avoid blind detection at gNB if valid CSI is not available - FFS: the definition of valid CSI Agreement For the solution to avoid blind detection for LTM CSI report if valid CSI is not available, - Option 1: CSI is reported when valid CSI is not available - Option 1-1: one CQI codepoint is used to indicate that valid CSI is not available - Option 1-1-1: Codepoint with the lowest CQI index is reported Agreement Regarding CSI acquisition, for a candidate cell, - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability Agreement Following restrictions are introduced - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition - The supported max rank is up to separate UE capability - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition - Up to 128 ports is supported - The supported max number of CSI-RS ports is up to separate UE capability

Below, we provide our views on some of the design details related to report timing, triggering mechanism and etc. for the CSI acquisition procedures based on reception of the LTM CSC MAC CE.

In LTM configurations, whether a candidate cell is for CSI acquisition can be enabled or disabled by the network. If the network determines that only a subset of the configured candidate cells is up for CSI acquisition, the UE can be provided the subset during the LTM preparation phase. In a later phase, the UE can perform CSI acquisition on a target cell – out of the subset of candidate cell(s) – during LTM switch. This can save signaling overhead if CSI acquisition on a target cell is triggered or activated in a dynamic manner. Furthermore, for CSI-RS measurement before reception of LTM CSC MAC CE, it is also preferable to limit the total CSI acquisition for candidate cell(s), e.g., based on the RS(s) in the activated TCI state(s) for the corresponding candidate cell(s) and corresponding UE’s capability(s).

Proposal 3: For CSI acquisition,

- a UE can be provided by the network whether a candidate cell is enabled or not for CSI acquisition, and

- for CSI measurement before CSC MAC CE, study mechanism for limiting total CSI acquisition for candidate cell(s) by the same UE before cell switching command. e.g. based on the RS(s) in the TCI state(s) activated for the corresponding candidate cell(s) and UE capabilities.

In addition, during CSI measurement before CSC, a UE does not know whether a candidate cell will become the actual target cell. Without limitation, the UE may need to store all the acquired CSI for multiple candidate cells before cell switch command, which would result in additional burden to the UE. The UE only needs to store CSI with up to a number of candidate cell(s) indicated by a corresponding UE’s capability.

Proposal 4: For CSI acquisition for candidate cell during LTM, the UE only needs to keep/store CSI for a limited number of candidate cell(s) before cell switching command, subject to corresponding UE’s capabilities.

After receiving the CSC, the UE can measure the configured CSI-RS(s) corresponding to the target cell indicated by the CSC. There is no need for the UE to further select a subset of the configured CSI-RS(s) or for the NW to indicate a subset of the configured CSI-RS(s) for the UE to measure and report the corresponding CRI(s).

Proposal 5: For CSI measurement after CSC MAC CE for CSI acquisition, do not support further selecting a subset of the configured CSI-RS(s) for the target cell via either sub-selection by the UE or indication by the gNB.

Regarding CSI reporting, the UE can send the CSI report at least a time gap after reception of the LTM CSC MAC CE. For both RACH-less and RACH-based LTM procedures, we do not see a need to have separate mechanisms to determine the first UL resource(s) to carry the CSI report – in this sense, the first UL resource(s) should correspond the first CG/DG PUSCH that carries the RRC completion message. If the UE cannot send the CSI report on the first UL resource(s) defined above due to timeline issue, the UE can always be triggered to perform aperiodic CSI reporting towards the target cell (i.e., the “new” serving cell after the LTM procedure is complete).

Proposal 6: For CSI acquisition,

- support the UE to send the CSI report at least a time gap after reception of the LTM CSC,

- for both RACH-less and RACH-based LTM procedures, support the first CG/DG PUSCH that carries the RRC completion message to carry the CSI report

# Conclusions

In this contribution, we provide our views on various design aspects related to Rel-19 LTM operations, focusing on necessary components to support CSI-RS for LTM measurement/reporting, event triggered L1 measurement reporting and CSI acquisition on candidate cell(s). In particular, we present the following proposals and observations focusing on various design aspects related to the Rel-19 LTM enhancements:

Proposal 1: For associating between the configured CSI-RS resources and candidate cells for L1 measurement(s), further support

- (implicit association): association between the configured CSI-RS resource(s) and the candidate cell(s) is based on association between the SSBs (QCL’ed with the configured CSI-RS resource(s)) and the candidate cells

Proposal 2: For gNB scheduled reporting and event triggered reporting, support shared CSI-RS resource for both LTM and L3 mobility, e.g., the CSI-RS introduced for LTM measurement can be configured for L3 mobility as well.

Proposal 3: For CSI acquisition,

- a UE can be provided by the network whether a candidate cell is enabled or not for CSI acquisition, and

- for CSI measurement before CSC MAC CE, study mechanism for limiting total CSI acquisition for candidate cell(s) by the same UE before cell switching command. e.g. based on the RS(s) in the TCI state(s) activated for the corresponding candidate cell(s) and UE capabilities.

Proposal 4: For CSI acquisition for candidate cell during LTM, the UE only needs to keep/store CSI for a limited number of candidate cell(s) before cell switching command, subject to corresponding UE’s capabilities.

Proposal 5: For CSI measurement after CSC MAC CE for CSI acquisition, do not support further selecting a subset of the configured CSI-RS(s) for the target cell via either sub-selection by the UE or indication by the gNB.

Proposal 6: For CSI acquisition,

- support the UE to send the CSI report at least a time gap after reception of the LTM CSC,

- for both RACH-less and RACH-based LTM procedures, support the first CG/DG PUSCH that carries the RRC completion message to carry the CSI report

# References

[1] 3GPP RAN#105, RP-242356, September, 2024.

[2] Chairman Notes, 3GPP RAN1#120, February 17th – 21st, 2025.

[3] Chairman Notes, 3GPP RAN1#120bis, April 7th – 11th, 2025.