3GPP TSG RAN WG1 Meeting #121	R1-2503645

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	Intel Corporation

Title:	Discussion on correction of RE mapping for UCI only PUSCH

Agenda item:	7

Document for:	Discussion

# Introduction

In this document, we discuss the potential ambiguity on the resource mapping of PUSCH on DMRS symbols in case of UCI only without UL-SCH. As presented in our companion contribution [1], we propose a draft CR to address this issue.

# Discussion

As agreed at RAN#1 2018 Jan Ad hoc meeting, regardless of whether UL-SCH is present on PUSCH, UCI is not transmitted on the DMRS symbols.

Agreements: - It is clarified that based on previous agreements, when UCI is piggybacked on PUSCH, UCI is not FDMed with DMRS - This applies to the case regardless of whether UL-SCH is present on PUSCH or not

Further, as defined in Clause 6.2.7 in TS38.212 [2], it is clearly stated that for any OFDM symbol that carries DMRS associated with PUSCH transmission, the set of resource elements are not used for UCI transmission on PUSCH, as highlighted below with yellow color.

### 6.2.7 Data and control multiplexing -- unchanged text omitted – Denote l as the OFDM symbol index of the scheduled PUSCH, starting from 0 to Nsymb,allPUSCH-1, where Nsymb,allPUSCH is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS. Denote k as the subcarrier index of the scheduled PUSCH, starting from 0 to MscPUSCH-1, where MscPUSCH is expressed as a number of subcarriers. Denote ΦlUL-SCH as the set of resource elements, in ascending order of indices k, available for transmission of data in OFDM symbol l, for l=0, 1, 2, ...,Nsymb,allPUSCH-1. Denote MscUL-SCHl=ΦlUL-SCH as the number of elements in set ΦlUL-SCH. Denote ΦlUL-SCHj as the j -th element in ΦlUL-SCH. Denote ΦlUCI as the set of resource elements, in ascending order of indices k, available for transmission of UCI in OFDM symbol l, for l=0, 1, 2, ...,Nsymb,allPUSCH-1. Denote MscUCIl=ΦlUCI as the number of elements in set ΦlUCI. Denote ΦlUCIj as the j -th element in ΦlUCI. For any OFDM symbol that carriers DMRS of the PUSCH, ΦlUCI=∅. For any OFDM symbol that does not carry DMRS of the PUSCH, ΦlUCI=ΦlUL-SCH. -- unchanged text omitted –

Note that in TS38.211 [3], the procedure for PUSCH symbol generation including scrambling on the encoded bits, modulation, layer mapping and resource mapping on the allocated resources was described. In case of UCI only on PUSCH without UL-SCH, encoded bits for UCI are generated and concatenated in case of multiple UCI types as defined in TS38.212 [2] and subsequently fed for scrambling operation as defined in TS38.211 [3].

For the resource mapping of UCI only on PUSCH, the existing specification in Clause 6.3.1.6 in TS38.211 [3] does not mention whether UE would need to transmit PUSCH on the OFDM symbols that carry the associated DMRS. More specifically, TS38.211 takes in data bit stream from TS38.212 and processes them based on description in TS38.211. In this case, potential ambiguity may exist on the implementation of PUSCH resource mapping on the DMRS symbols in case of UCI only without UL-SCH.

Without clarification it is unclear how the resource elements in OFDM symbols that carry DM-RS are processed for transmission at the UE. TS38.211 does not state that the resources in DMRS symbols are excluded from RE mapping process, although may be hinted in TS38.212 during the bit multiplexing of control and data.

From our understanding and based on agreements that were used to reflect specification, the resources elements in OFDM symbols that carry DM-RS should not be used for UCI regardless of whether data is present in PUSCH. This would be how all vendors would have implemented the NR system. In addition, this would clarify those resources elements not used by UCI when data-less PUSCH should be zero powered. An illustration of two potential possibilities from the current TS38.211 specification is provided in Figure 2-1. Our understanding is possibility 2 is what the TS38.211 description should follow. This may not be strongly evident from the description available in TS38.211, so we believe there is value to clarify this at the same time.

Figure 2-1. Illustration of potential possibilities of RE power of OFDM symbol that carry DM-RS for UCI only PUSCH transmission

In order to address this issue and avoid potential ambiguity in the implementation, it is more appropriate to clarify the resource mapping of PUSCH on DMRS symbols in case of UCI only without UL-SCH. As presented in our companion contribution [1], the following draft CR is proposed in Clause 6.3.1.6 in TS38.211 [3]:

-- unchanged text omitted – #### 6.3.1.6 Mapping to virtual resource blocks For each of the antenna ports used for transmission of the PUSCH, the block of complex-valued symbols shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, TS 38.213] and mapped in sequence starting with to resource elements (k',l)p,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission, and - the corresponding resource elements in the corresponding physical resource blocks are not used for transmission of the associated DM-RS, PT-RS, or DM-RS intended for other co-scheduled UEs as described in clause 6.4.1.1.3, and - for transmission on the PUSCH without UL-SCH, the corresponding resource elements in the corresponding physical resource blocks are not used for transmission of OFDM symbols that carry DM-RS. The mapping to resource elements (k',l)p,μ allocated for PUSCH according to [6, TS 38.214] shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index , with the starting position given by [6, TS 38.214]. -- unchanged text omitted –

Proposal 1:

- RAN1 to clarify the resource mapping of PUSCH on DMRS symbols in case of UCI only without UL-SCH.

- RAN1 to adopt the draft CR as in R1-2503644.

# Conclusions

In this contribution, we discussed potential ambiguity on the resource mapping of PUSCH on DMRS symbols in case of UCI only without UL-SCH. The following is a summary of proposals made.

Proposal 1:

- RAN1 to clarify the resource mapping of PUSCH on DMRS symbols in case of UCI only without UL-SCH.

- RAN1 to adopt the draft CR as in R1-2503644.

# References

- R1-2503644, “Rel-15 correction of RE mapping for UCI only PUSCH”, Intel Corporation.

- 3GPP TS 38.212 v15.10.0, “Physical channels and modulation”

- 3GPP TS 38.211 v15.13.0, “Multiplexing and channel coding”