3GPP TSG RAN WG1 #121 		                                    		   R1-2503753

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.7.2

Source:	InterDigital, Inc.

Title:	Discussion on ISAC channel modeling

Document for:	Discussion and Decision

# Introduction

SID for channel modelling for ISAC was approved in RAN#102. In the study, scenarios related to detecting and/or tracking an object are to be considered and corresponding channel models will be studied. Based on the agreement made in RAN1#120b [1], we discuss RCS modelling and details of the channel models that need to be considered in this contribution.

# ISAC scenarios

## RCS modelling

### Details related to RCS modelling for a human target

In RAN1#120b, the following agreement was made.

Agreement On the monostatic RCS for human with RCS model 2 - The monostatic RCS for a scattering point of the target is generated by - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax Where, σVdBθ=-min12θ-θcenterθ3dB2, σmax σHdB φ=-min12φ-φcenterφ3dB2, σmax - FFS how many rows of the values/pattern A*B1 are defined for the target - Note: each row has a defined applicable range of θ and φ - Note: whether the RCS is elevation angle dependent or dependent on both elevation and horizontal angles can be separately discussed

Figure 1 Definition of the four sides of a human target

The primary contribution to the RCS comes from the human trunk. Accordingly, the right and left sides represent the smallest cross sections of the human trunk, potentially resulting in a lower bound on the RCS. Conversely, the largest trunk cross sections are from the front and back, providing potentially a higher bound on the RCS contribution. An example of four sides for a human target is illustrated in Figure 1.

Proposal 1: The parameterization of component A*B1 of human RCS model 2 can be represented by 4 rows, with parameters defined as follows:

| Human side | φ_center | φ_3dB,k | θ_center | θ_3dB,k | G_max | σ_max | Applicable Range of φ | Applicable Range of θ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0° | - | 90° | - | - | - | [315°, 45°] | [0°, 180°] |
| Left | 90° | - | 90° | - | - | - | [45°, 135°] | [0°, 180°] |
| Back | 180° | - | 90° | - | - | - | [135°, 225°] | [0°, 180°] |
| Right | 270° | - | 90° | - | - | - | [225°, 315°] | [0°, 180°] |

Observation 1: Average RCS for human RCS model 2 (component A) depends on various parameters and conditions of the body such as:

- Body size (kid, adult)

- Body type (thin, fat)

- Body position (standing, sitting)

- Gender (male, female)

- Attire (light, thick)

- Angle, frequency, etc.

Proposal 2: For human RCS model 2, support multiple values for A to reflect different human body characteristics.

The applicable angle range for the defined parameters needs to be characterized for the relevant scenarios (e.g., ground-based sensing vs. airborne sensing), as such, different angle ranges may be applicable which may impact final RCS values.

Proposal 3: RCS measurements provided by the companies should include details related to sensing type and scenario (ground based, airborne based, etc.).

## Details in channel modelling

### Spatial consistency and mobility procedure for ISAC channel models

In RAN1#118bis, the following agreement defined the general framework for modeling cross-polarization power ratios and initial random phase of the target channel [2]:

Agreement To model the effect of polarization for each direct/indirect path: - Polarization of a direct/indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link - Total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For a LOS ray from Tx to target or from target to Rx, CPM=100-1 for CPMtx,sp or CPMsp,rx - For a NLOS ray generated by a stochastic cluster from Tx to target or from target to Rx, CPMtx,sp or CPMsp,rx is generated by XPR ratio k and initial random phases referring to TR 38.901 as start point - FFS how to normalize on CPMtx,sp,rx - FFS CPMsp of a scattering point of the target - FFS: how to model the effect of polarization when EO type-2 is present

This understanding was further clarified in RAN1#120 with the following agreement:

Agreement If EO type-2 is modelled in an indirect path, only specular reflection is modeled for EO type-2 - polarization of the indirect path is product of polarization matrix of the target-Rx link, the target, and the Tx-target link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For the specular reflected ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., Tx-EO type-2-target, or target-EO type-2-Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2 - To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point - FFS applicability if the surface of EO type-2 is tilted

In this three-component model, CPtx,sp defines the depolarization effect of scattering that occurs as the sensing signal propagates form the sensing Tx to the scattering point.  Similarly, CPsp,rx defines the depolarization effect of the scattered signal as it propagates from the scattering point to the receiver.  The component, CPMsp models a more intrinsic feature of the scattering point itself representing the specific polar-dependent scattering of the target on which the signal is incident.

Two additional agreements were made in RAN1#120 [3] that impact the spatially consistent modelling of both cross-polarization power ratios and initial random phase of the target channel:

Agreement Correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links - Definition of link Correlated: parameters for any two links between STs/UTs are correlated, subjected to correlation distance. Table 4: Correlation type for links between STs/UTs Parameters; Correlation type. Delays; link Correlated. Cluster powers; link Correlated. AOA/ZOA/AOD/ZOD offset; link Correlated. AOA/ZOA/AOD/ZOD sign; link Correlated. Random coupling; link Correlated. XPR; link Correlated. Initial random phase; link Correlated. LOS/NLOS states; link Correlated. Blockage (Model A); All-correlated. O2I penetration loss; All-correlated. Indoor distance; All-correlated. Indoor states; All-correlated.

Agreement To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target - in Rel-19 study item (e.g., UAV, human, vehicle, AGV), αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e., CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ Where, - κsp,i is XPR ratio is randomly generated by log-normal distribution per target type - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π - FFS: spatial consistency of random phase when a scattering point moves - FFS: whether the base station rotation procedure in 38.901 can be reused to support rotation of the target

Although it was not specifically clarified, the link-correlated applicability of initial random phase and XPR may be assumed, in the sensing case to apply to CPStx,sp, CPMsp, and CPMsp,rx.

TR 38.901 [4] specifies two different procedures for constructing spatially consistent channel models:

- Spatial consistency for 2 UTs that are closely located communicating with the same TRP:  In this case the assumption is that both UTs observe signals propagating off of similar scatterers and so the cluster level statistics should be highly correlated.  If UT 1 is dropped at location 1 the cluster-specific parameters including random delay, shadowing, and AOA/ZOA/AOD/ZOD and ray-specific parameters including ray coupling, XPR, and random phase may be generated independently.  If a UT 2 is dropped in location 2 near location1, the same parameters for channel realization 2 are expected to be correlated with channel 1 subject to the proximity of location 1 and 2.

- Spatial consistency for 1 mobile UE: In this case a modification of the first approach is applied for only 1 UT where the assumption is that the location of the single UT at time snapshot t1 is close to the UT location at t2 when the UE velocity is relatively small and t2-t1 is short.  In this case for similar reasons to case 1 the channel at t2 can be assumed to be correlated to the channel realization at t1, subject to the displacement of the UE at those two times.

A modification of this spatial consistency approach has been agreed with the introduction of link-correlated applicability for spatial consistency. An illustrative example of a link-correlated channel is provided in Figure 2 for both spatial consistency and spatially consistent mobility modeling.  In this example, the corrHtx,sp1,Htx,sp2→1 and corrHsp1,rx,Hsp2,rx→1 as the displacement r=r1-r2→0, for sp location r1, and r2. Likewise, for spatially consistent mobility, the corrHtx,spt1,Htx,spt2 →1 and corrHsp,rxt1,Hsp,rxt2→1 as the displacement r=r t1-r t2→0, for sp location at time t1 and t2.

Figure 2: Link-correlated spatial consistency and spatially consistent mobility modeling

In general, this modification should be sufficient for spatial consistent mobility modeling as, CPMtx,sp, CPMsp, and CPMrx,sp can be assumed to be highly correlated for a single scattering point, subject to the correlation distance of that scattering point at two different times. These assumptions, however, do not hold when considering the spatial consistency of two targets that are closely located.  While CPMtx,sp and CPMsp,rx may be assumed to be spatially consistent based on the assumption of similar propagation phenomenon, CPMsp is largely dependent on intrinsic properties of the target. For this reason, CPMsp of two different targets may never be correlated regardless of how close those two targets are to each other.

Proposal 4: Initial random phase and XPR of CPMtx,sp and CPMsp,rx are link-correlated for spatial consistency modeling and spatially consistent mobility modeling of the same target.

Proposal 5: Initial random phase and XPR of CPMsp is link-correlated for spatially consistent mobility modeling of the same target only.

Similar to XPR and initial phase, the RCS of the target should reflect angle-dependent correlation, at least when modeling spatial consistency; however, this is currently not possible with the existing model for RCS due to the parameter B2 being independent.  To model RCS in practical tracking scenarios, it will be necessary to track a consistent feature of the signal response produced by the target.  This may include phase, power, delay, or angle.  For this reason, B2 should also be modelled in a spatially consistent manner.

Proposal 6: The RCS parameter, B2, is link-correlated for spatially consistent mobility modeling of the same target only.

### Angular correlation of phase, amplitude, and XPR for target RCS

In RAN1#120bis, there was a parallel discussion on modeling XPR and initial random phase with angle-dependent correlation, which in our view is closely related to the discussion of spatial consistency

The underlying assumption of the existing spatial consistency modeling framework is that spatially consistent parameters are correlated for small changes in UT position including changes in angle. Spatially correlated parameters in the spatial consistency model include AOA and AOD, initial phase and XPR as indicated in Table7.6.3.4-1 of [4]. This implication of this model approach is that small changes in scattered paths angles should produce correlated initial random phase and XPR of those paths.  It is reasonable to assume that the modeling assumptions for spatial consistency of the scattered paths in the environment should hold for scattered paths interacting with the target as well.

Companies may prefer that this angular correlation only holds in the case where spatial consistency is modelled, which case if XPR and initial random phase are included in spatially consistent mobility modeling of a scattering point, it will not be necessary to model angular correlation of these parameters in general.

Observation 2: If XPR and initial random phase of CPMsp are modeled as link correlated for spatially consistency mobility modeling of a scattering point, it is not necessary to model angular correlation of these parameters as part of the RCS model for the target.

### LoS modelling for UAV target

In the previous meeting, the following agreement was made regarding Reference TR for generating the ISAC channel for aerial UE.

Agreement In order to generate Tx-target link, target-Rx link and the background channel, the above table on reference TRs (excluding the already agreed part) is adopted for the mapping between reference TRs and a pair of nodes (STX, SRX, target) - Note: continue discussion for updating the table with RSU type UE - FFS: the generation of background channel based on reference TRs is subject to the addition of low-energy clusters Case; Node 1; Node 2; Existing TRs as starting point. 7; normal UE; aerial UE; UMi-AV, UMa-AV, and RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1 - LOS probability is not reused, FFS new LOS probability - FFS pathloss model, shadowing fading - Working assumption: Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used. 9; aerial UE; aerial UE; UMi-AV, UMa-AV, RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE for FR1 - LOS probability is not reused, FFS new LOS probability - FFS pathloss model, shadowing fading, angular spread - Working assumption: Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used.

As mentioned in the above agreement, it was agreed to not reuse the LoS probability for the ISAC channel for both Case 7 and Case 9.

In TR 36.777 [5], the LoS probability modelling is defined between a base station (BS) at a fixed height and an aerial UE at varying heights (between 1.5m to 300m). In Case 9, considering that both the nodes are aerial UEs with variable heights, the defined LoS probability accurately model the channel. Similarly, for Case 7 involving one terrestrial UE and one aerial UE, the LoS probability may be redefined due to the variation in relative height difference compared to the reference TR.

Ideally, a measurement campaign backed modelling is required for accurate modelling of the LoS probability for cases 7 and 9. However, due to the time constraints for ISAC channel modelling work in Rel. 19, full measurement campaigns may not be feasible.

Alternatively, we propose to use validated and calibrated channel models in 3GPP with slight modifications to model the LoS probability for Case 7 and Case 9.

Observation 3: The accurate LoS probability modelling for Case 7 and Case 9 requires measurement campaigns, which is not feasible due to time limitations in Rel. 19.

Proposal 7: Use validated 3GPP channel models with modifications to model the LoS probability for Case 7 and 9.

In TR 36.777, the LoS probability between a BS and an aerial UE is defined by segmenting the aerial UE heights into three regions:

- Low UAV-region:

  - Defined around the height of BS or terrestrial UE (e.g., from 1.5 meters until 10m (for RMa-AV) or until 22.5 meters (for UMi-AV, UMa-AV)).

  - LoS probability as defined in TR 38.901 [7].

- High-UAV region:

  - Defined around the height with no obstructions in the channel between the BS and aerial UE from 40m for RMa-AV and 100m for UMa-AV scenario.

  - LoS probability defined as 1 (100% LoS).

- Mid-UAV region:

  - Defined in between the low-UAV and the high-UAV height ranges.

  - LoS probability equation is defined in TR 36.777.

The height segmentation for the UAV height regions from TR 36.777 are summarized in Table 1 below:

Table 1: Height regions for aerial UE for different scenarios as described in TR 36.777

| UAV height regions | UMi-AV | UMa-AV | RMa-AV |
| --- | --- | --- | --- |
| Low-UAV region | 1.5m to 22.5m | 1.5m to 22.5m | 1.5m to 10m |
| Mid-UAV region | 22.5m to 300m | 22.5m to 100m | 10m to 40m |
| High-UAV region | N/A | 100m to 300m | 40m to 300m |

For Case 7 and Case 9, we propose to use the same approach of height segments-based modelling. Given the combinations of variations in heights between the two nodes especially for Case 9, height segment-based modelling is a preferable option with balance between trade-off for accuracy of modelling with complexity.

Observation 4: The LoS probability in TR 36.777 is modelled based on the height segments (low-UAV region, mid-UAV region and high-UAV region) of the aerial UE.

Proposal 8: The LoS probability for Case 7 and Case 9 is defined based on the aerial UE height segments, similarly to TR 36.777.

LoS probability for Case 9:

For modelling the channel between two nodes, both in the low-UAV region, the LoS probability defined in TR 38.901 can be reused. The probability is defined for the BS to a terrestrial UE channel in the low UAV height region. Additionally, such generalization has been considered for UE-to-UE channel model in TR 38.858 [15] for NR Duplex study. Hence, such generalization can be made for Case 9 as well.

Likewise, the LoS probability defined for a BS to a mid UAV region in TR 36.777 can be generalized and reused between a node in the mid-UAV region to another node in low-UAV or a mid-UAV region as the channel between the nodes in this region can be expected to experience similar LoS probability as a function of both distance and height of the nodes.

For the case between a node in a low-UAV region and a node in a high-UAV region, the channel may depend on the angle between the nodes relative to the ground. Hence, we propose to use the LoS probability defined in NTN channel model (Table 6.6.1-1 in TR 38.811 [6]). Although the NTN model defines the channel between a terrestrial UE and a satellite at a height much higher than that of the aerial UE, since the probability is defined dependent only on the angle between the two nodes, the LoS probability can be generalized to the case between two aerial UEs.

For the remaining cases of high-UAV range to mid-UAV or high-UAV ranges, a LoS probability of 1 should be considered. In TR 36.777, a probability of 1 is considered between a node in high UAV region and a BS. Hence, such generalization can be made to the channel between a node in the high-UAV regions and another node at any height above the BS height which is the mid-UAV and the high-UAV region.

The LoS probability for such case can be summarized in Table 2 below:

Table 2: Table defining the LoS probability between two nodes (node 1 and node 2) as a function of height regions for each node

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi, UMa, RMa LoS probability as defined from Table 7.4.2-1 in TR 38.901 | UMi-AV, UMa-AV, RMa-AV Los probability as defined for BS to mid UAV region from Table B-1 in TR 36.777 | Dense urban, urban and suburban and rural LoS probability from Table 6.6.1-1 in TR 38.811 (NTN model) |
| Mid-UAV | UMi-AV, UMa-AV, RMa-AV Los probability as defined for BS to mid UAV region from Table B-1 in TR 36.777 | UMi-AV, UMa-AV, RMa-AV Los probability as defined for BS to mid UAV region from Table B-1 in TR 36.777 | 1 |
| High-UAV | Dense urban, urban and suburban and rural LoS probability from Table 6.6.1-1 in TR 38.811 (NTN model) | 1 | 1 |
| Note: The corresponding height values for each node for each scenario (UMi-AV, UMa-AV and RMa-AV) is defined in Table 1. |  |  |  |

The LoS probability for each scenario (e.g., UMi-AV, UMa-AV, RMa-AV) is defined as the corresponding urban and rural scenarios in the reference TRs as defined in Table 2.

Proposal 9: Use Table 2 for defining the LoS probability for Case 9 between two aerial UE nodes for each scenario (UMi-AV, UMa-AV, RMa-AV).

LoS probability for Case 7:

Case 7 is considered as a special case of Case 9, where one node as a normal UE in the low-UAV region and another node as an aerial UE in low, mid and high-UAV regions. Hence, the LoS probability for Case 7 can be defined based on the first row of Table 2.

Observation 5: Case 7 can be considered as a special case of Case 9 with height of one node in the low-UAV region.

Proposal 10: Define the LoS probability for Case 7 between a normal UE and an aerial UE for each scenario (UMi-AV, UMa-AV, RMa-AV) using the following table

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| normal UE | UMi, UMa, RMa LoS probability as defined from Table 7.4.2-1 in TR 38.901 | UMi-AV, UMa-AV, RMa-AV Los probability as defined for BS to mid UAV region from Table B-1 in TR 36.777 | Dense urban, urban and suburban and rural LoS probability from Table 6.6.1-1 in TR 38.811 (NTN model) |
| Note: The corresponding height values for each node for each scenario (UMi-AV, UMa-AV and RMa-AV) is defined in Table 1. |  |  |  |

### Pathloss modelling for UAV target

For case 7 and case 9, height segment-based modelling is proposed for LOS probability in section 2.2.3. Pathloss modelling can follow the same height segment-based modelling approach for case 7 and case 9. Pathloss and shadow fading models for such case can be summarized in Table 3 below:

Table 3: Pathloss and shadow fading between two nodes as a function of height regions for each node

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi, UMa, RMa Pathloss models and shadow fading from Table 7.4.1-1 in TR 38.901 | UMi-AV, UMa-AV, RMa-AV Pathloss models from Table B-2 and shadow fading from Table B-3 in TR 36.777 | The basic path loss model (including clutter loss and shadow fading) models from equation 6.6-4 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 |
| Mid-UAV | UMi-AV, UMa-AV, RMa-AV Pathloss models from Table B-2 and shadow fading from Table B-3 in TR 36.777 | UMi-AV, UMa-AV, RMa-AV Pathloss models from Table B-2 and shadow fading from Table B-3 in TR 36.777 | The free space path loss (FSPL) model from equation 6.6-2 and shadow fading for LOS in Tables 6.6.2-1, 6.6.2-2, 6.6.2.-3 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 |
| High-UAV | The basic path loss model (including clutter loss and shadow fading) models from equation 6.6-4 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 | FSPL model from equation 6.6-2 and shadow fading for LOS in Tables 6.6.2-1, 6.6.2-2, 6.6.2.-3 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 | FSPL model from equation 6.6-2 and shadow fading for LOS in Tables 6.6.2-1, 6.6.2-2, 6.6.2.-3 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 |
| Note: The corresponding height values for each node for each scenario (UMi-AV, UMa-AV and RMa-AV) is defined in Table 1. |  |  |  |

Pathloss and shadow fading for each scenario (e.g., UMi-AV, UMa-AV, RMa-AV) is defined as the corresponding urban and rural scenarios in the reference TRs as defined in Table 3.

Proposal 11: Use Table 3 for defining pathloss and shadow fading for Case 9 between two aerial UE nodes for each scenario (UMi-AV, UMa-AV, RMa-AV).

Proposal 12: Use the following table to define pathloss and shadow fading for Case 7 between a normal UE and an aerial UE for each scenario (UMi-AV, UMa-AV, RMa-AV).

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| normal UE | UMi, UMa, RMa Pathloss models and shadow fading from Table 7.4.1-1 in TR 38.901 | UMi-AV, UMa-AV, RMa-AV Pathloss models from Table B-2 and shadow fading from Table B-3 in TR 36.777 | The basic path loss model (including clutter loss and shadow fading) models from equation 6.6-4 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 |
| Note: The corresponding height values for each node for each scenario (UMi-AV, UMa-AV and RMa-AV) is defined in Table 1. |  |  |  |

### Stochastic vs. ray-tracing/map based channel modelling

Two methods of channel modelling have been proposed so far, namely stochastic channel modelling and ray-tracing based approach. While stochastic channel modelling can be validated with simulations across companies, ray-tracing approach requires field trials for modelling and validation. To develop a model that can be validated by companies in 3GPP, we propose to adopt stochastic channel modelling as the baseline and study the need for ray-tracing based channel modelling for ISAC channel modelling.

It should be noted that map-based hybrid model is included TR38.901 [4] to capture actual building, walls, etc. For ISAC, there are too many scenarios with diverse environments which makes calibration between companies more challenging for map-based modelling.

Proposal 6: Support stochastic ISAC channel modelling as baseline and focus on characterizing the Tx-Target-Rx link.

Proposal 7: Work on stochastic ISAC channel models is prioritized over Ray-tracing or map-based ISAC channel modelling where the latter should be studied only if time permits

### Background channel

The following agreement was made in RAN1#116 [7].

Agreement The common framework for ISAC channel model is composed of a component of target channel and a component of background channel, HISAC=Htarget+Hbackground - Target channel Htarget includes all [multipath] components impacted by the sensing target(s). - FFS details of the target channel - Background channel Hbackground includes other [multipath] components not belonging to target channel - FFS details of the background channel - FFS whether/how to model environment object(s), i.e., object(s) with known location, other than sensing target(s) - FFS whether/how to model propagation path(s) between the target(s) and the environment object(s) - FFS whether/how to model propagation path(s) between the target(s) and the stochastic clutter(s) - Note: the notation HISAC can be revised later if needed

The channel model used as the common framework is shown below.

HISAC=Htarget+Hbackground

Hbackground can be modelled using channel generation described in TR 38.901. Htarget needs to consider two links, namely one path between Tx and target (Tx2T link) and another path between target and Rx (T2Rx link). Whether Tx2T and T2Rx link are to be modelled separately should be discussed. An illustration of target and background channel is shown in Figure 3.

Figure 3 Relationship between Background channel and Tx-target-Rx channel

Communication channel coefficients are generated using legacy TR 38.901 [4].  However, in the baseline communication channel, the coefficient time delays are normalized to begin at time zero. Clearly, sensing channel coefficient time delays should not be normalized so that estimating the object location can be properly computed. Therefore, when combining these two channels, the communication coefficient delays should not be normalized.  An algorithm to use absolute arrival times under “Additional modelling components” (Section 7.6.9) is defined in TR 38.901, which we can utilize for the communication channel.

In the current version of TR 38.901, channel coefficients are generated as per equation 7.5-30 when Tx-Rx link is in LOS condition and as per equation 7.5-27 when Tx-Rx link is in NLOS condition. Target channel and background channel can be combined after applying scaling factor as illustrated in equation (1).  The scaling parameter can be used to control interference of the background channel.

Hs,uISACτ,t=Hs,uTargetτ,t+αbackgroundHs,uBackgroundτ,t			 (1)

where the coefficients in Equation (1) are defined as follows:

- Hs,uBackground: channel coefficients for background channel.

- Hs,uTarget: channel coefficients for target channel.

- αbackground: scaling factor for background channel

In addition, the following agreement was made in RAN1#119. In the following, several options are proposed to clarify the relationship of power in the target channel and background channel. From the above analysis, our proposal is to normalize power of the target and background channel. In addition, a scaling factor should be introduced such that interference from the background channel can be controlled during evaluation phase of sensing methods. By properly normalizing power of both channels, a proper scaling factor can be determined to control interference of the background channel.

Agreement - The following options are supported to generate the combined ISAC channel - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed. - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between - Alt 1: Power normalization on both target channel and background channel - Alt 2: Power normalization on background channel only - Alt 3: the target channel of a target will replace one cluster in the background channel - FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2 - FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

Thus, we make the following proposals.

Proposal 13: Regarding combination of target and background channel, support Option 2 Alt1 “Power normalization on both target channel and background channel”

Proposal 14: Adopt a scaling parameter which defines a relative power difference between the background channel and target channel

### Determination of LOS/NLOS condition in the presence of EO type-2

In RAN1#119, the following agreement was made [8].

Agreement When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition - FFS changes to the LOS probability defined in existing TRs - FFS details on blockage by EO type-2 - Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

In the above agreement, how the presence of EO type 2 determines the LOS/NLOS status was discussed. EO type 2 may be more suitable to serve as a blockage. Power attenuation factor based on the EO type 2 can be determined using Blockage model A or model B in TR 38.901. Blockage model A is a stochastic approach to determine the attenuation factor. Blockage model B is a size-specific approach to determine the attenuation factor.

Proposal 15: When the EO type-2 is modeled in the target channel, adopt Option B, “Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model”

Effect of the presence of target on the LOS status in the target channel

The following agreement was made in RAN1#119.

Agreement EO type-1 (when modelled) is modelled in the same way as a sensing target in the ISAC channel model.

As an unintended target serves as an additional element in the target channel model, the LOS status should be determined based on the conventional modelling method (e.g., distance). The unintended target may be more suitable to serve as a blockage. Power attenuation factor based on the unintended can be determined using Blockage model A or model B in TR 38.901. Blockage model A is a stochastic approach to determine the attenuation factor. Blockage model B is a size-specific approach to determine the attenuation factor.

Proposal 16: An unintended target is a blockage factor in the target channel and further study whether Blockage model A or Blockage model B should be implemented

### Concatenation of Tx-target and target-Rx links

The following agreement was made in RAN1#119 regarding how to link Tx-object and object-Rx links.

Agreement - To generate indirect paths of NLOS ray + NLOS ray in the target channel - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4 - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2, - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1 - Other methods are up to company choice for complexity reduction - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration - The power threshold for path dropping is X=[-25] dB - X is relative to the strongest indirect path in the target channel - FFS: further power normalization of target channel is performed after path dropping, - Note: power normalization when target channel and background channel are combined can be discussed separately - FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

In addition, the following agreement was made in RAN1#120.

Agreement - For target channel - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link - The power threshold for path dropping after concatenation is relaxed to [X=-40] dB - For background channel - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., [-25 dB] is reused to generated the background channel - FFS: whether to add additional very low power clusters - FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power, max. target-Rx link cluster power)

RAN1 still needs to discuss the path dropping threshold after concatenation.

In this section, we provide an analysis of the need for power normalization for option 0. Assuming N clusters and M rays per cluster in both Tx-target link and target-Rx link, ray level full convolutional coupling (Option 0) between Tx-target and target-Rx link, there are (MN)^2 indirect NLOS-NLOS paths. Since each ray in Tx-target link is associated with all rays in target-Rx link, we observe M (number of rays per cluster) times higher ray power in NLOS-NLOS indirect paths compared to direct path as rays associated with a cluster inherit the delay parameter of the cluster. To compensate for this additional higher power observed in option 0, we normalize ray power of each ray in Tx-target link by number of rays per cluster(M).

Observation 6: For option 0 to generate indirect paths of NLOS ray+ NLOS ray in the target channel, we need to normalize ray power by number of rays per cluster(M).

Proposal 17: Normalize rays in Option 0 by a scaling factor (e.g.,  1/M where M is the number of rays in target-Rx link) to avoid power inflation through concatenation

Figure 4 CDF of paths with or without path dropping according to a threshold (Indoor Open Office scenario)

Figure 5 CDF of paths with or without path dropping according to a threshold (Indoor Factory scenario)

We evaluated the impact of thresholds used for path dropping on the distribution of rays. We evaluated Option 0 with Case 1 (LOS for Tx-target and target-Rx links) with different power thresholds for NLOS-NLOS path dropping. Detailed simulation parameters are described in Appendix B. Multiple samples of sensing target channel are aggregated for path dropping threshold X= {-20 dB, -25 dB, -30 dB, -35dB, -40dB, -45dB, -50dB} and compared against results without path dropping in Figure 4(Indoor Open Office scenario) and Figure 6(Indoor Factory scenario). In Table 4, a comparison between proportion of paths dropped and different values of path dropping thresholds is shown. It is clear from the table that nearly 40% of the paths are dropped if the path dropping threshold of -25 dB is applied. It is also noticeable from the table that approximately 3% of the paths are dropped with the path dropping threshold of -50dB. From Figure 4 and Figure 5, it is also clear that changing thresholds have a significant impact on power distribution of rays in the channel.

Observation 7: For Indoor Open Office scenario, nearly 40% and 3% of paths are dropped with the path dropping threshold of -25dB and -50dB, respectively.

Observation 8: For Indoor Factory scenario, nearly 45% and 4% of paths are dropped with the path dropping threshold of -25dB and -50dB, respectively.

Table 4 Comparison between the proportion of paths dropped and path dropping thresholds

| Threshold (dB) | -20 | -25 | -30 | -35 | -40 | -45 | -50 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Proportion of dropped paths (%), InH | 59.04 | 41.34 | 26.99 | 17.21 | 10.65 | 6.05 | 3.19 |
| Proportion of dropped paths (%), InF-SH | 64.57 | 47.33 | 32.14 | 20.58 | 12.44 | 6.95 | 3.83 |

From the evaluation results, it is shown that path dropping in Option 0 can reduce the number of paths included in the channel model. However, the impact such channels with reduced complexity can have on the performance of sensing methods is not clear yet. In addition, Option 3 should be sufficient if the ISAC channel needs to be generated with low complexity. Thus, we make the following proposal:

Proposal 18: Do not adopt path dropping after concatenation

## Channel modelling to incorporate micro Doppler characteristics

The current agreement in RAN1 is to include both macro-Doppler and/or micro-Doppler of a sensing target in a unified formula: vspt=vmacrot+vmicro,pt. In view of the placeholder function, RAN1 can include details for the micro-Doppler functions for certain sensing targets as part of the R19 study.

The micro-Doppler motion displacement φmicro,pt induces phase variations on the reflected signals from the sensing target expressed as 2πφmicro,ptλ., where λ is the carrier wavelength. The micro-Doppler motion displacement, φmicro,pt, can be expressed through the micro-Doppler velocity function, vmicro,pτ, using the following expression: φmicro,p(t)=0tvmicro,p(τ)dτ. Micro-Doppler functions for UAV and human targets with different motions [9] are shown in Table 5.

Proposal 19: Adopt micro-Doppler functions for humans and UAVs provided in Table 5.

Table 5: Micro-Doppler functions for micro-motions of humans and UAVs.

| Type of target | Formula of micro-Doppler function φmicro,p(t) |
| --- | --- |
| Human respiration/heartbeat | φmicro,p(t)=R0+rpDsin2πfrtmax is the initial location offset. is the location vector of human. is the maximum amplitude of respiration/heartbeat. is the frequency of respiration/heartbeat. |
| Human walking/running | φmicro,p(t)=R0+rpp=1NpDpsin2πfpt is the initial location offset. is the location vector of human. is the number of human segments. is the maximum amplitude of motion of pth segment. is the frequency of motion of pth segment. |
| UAV propeller rotation | φmicro,k,p(t)=Rj+ruL2cosΩjt+ϕj+k2πNB j=1,⋯,NR;k=0,⋯,NB-1 For kth rotation blade on the jth rotor of UAV is the initial location offset of jth rotor and is the initial phase of jth rotor. is the location vector of UAV is the the distance between the center of blade and the center of the rotation and is the frequency of jth blade rotation. |

## Sensing area

In sensing, a sensing area can be defined as an area within which the UE or network can localize an object within a specific accuracy requirement. An example is depicted in Figure 6 where the sensing coverage area may be defined by a major and minor axis and τBS indicates the propagation distance from the Tx to the UE via an object at the edge of the area. If an object that is located far with respect to Tx or Rx, it may not be detectable or locatable since the received power at the receiver for the reflected signal may be too weak. The received power should be strong enough to determine the presence of the object.

A similar concept can be found in positioning evaluation scenario. In positioning evaluations, a “convex hull” was defined for the indoor factory scenarios [10] to define where a UE can be dropped so the positioning system can obtain reasonable accuracy. The convex hull is an area that is surrounded by TRPs and the UE located within the convex hull can be located accurately, compared to the UEs located outside of the convex hull (e.g., corner of the factory hall).

To visualize the area in which an object can be detected or localized with reasonable accuracy, an example of received power, incorporating the RCS characteristics of the plate, is shown in Figure 7, where the TRP and UE are located at the coordinate (x,y)=(10, 30) meter and (x,y)=(110, 30) meter, respectively. The presented analysis is applicable to bi-static sensing and we assume TRP and UE as Tx and Rx, respectively.  Evaluation assumptions are shown in Appendix C.

As near-field channel modelling is still under the discussion in 3GPP, received power when the plate is near the Tx or Rx is not included in the figure. In this experiment, we assume antenna aperture size of 0.24 m for Tx and 0.085 m for Rx. As presented in Table 1, Rayleigh distance values corresponding to these aperture sizes for 3.5 GHz carrier frequency are 1.37 m and 0.17 m respectively.  To avoid dropping objects in near field, we consider minimum distance between gNB and Object equals to 2 m and minimum distance between UE and Object equals to 1 m. In addition, the received power when the plate is placed along the LOS path is not indicated in the figure since blockage effect is not considered in the evaluation.

In the evaluation, a metal plate is dropped in a 1 m2 grid and simulation is repeated for each grid placed systematically across the indoor factory hall, and received power of the reflected signal from the plate is recorded for each drop of the plate.

As the figure shows, relatively higher received power is obtained from an object placed near the LOS path and diminishing received power is observed when the object is placed further away from the LOS path. Pathloss used in the simulation is calculated based on following formula [11].

PLsensing,dB=PLdtxObj+PLdobjRx+10log10λ24π-10log10σRCS		(7)

- dtxObj: Distance between Tx to the object.

- dobjRx: Distance between object to the Rx.

- σRCS: RCS of sensing object.

- λ: wavelength corresponding to carrier frequency.

Symmetric and concave shape of the received power is due to the change in the incident angle from different relative locations, with respect to the UE or TRP, of the dropped plate. Change in the incident angle results in different values for RCS, which is one of the factors in changing received power.

Based on the above observations, the following proposal is made.

Proposal 20: Define a sensing coverage area for the ISAC channel model where the area includes only possible points for which the UE or network can localize an object within a specific accuracy requirement.

Whether or not material absorption is factored into the definition of sensing area can be left for further study.

Figure 6 An illustration of a sensing area

Figure 7 Received power measured by the UE when an object (plate) is placed at 1m2 grid in the InF hall

# Conclusion

RCS details

Proposal 1: The parameterization of component A*B1 of human RCS model 2 can be represented by 4 rows, with parameters defined as follows:

| Human side | φ_center | φ_3dB,k | θ_center | θ_3dB,k | G_max | σ_max | Applicable Range of φ | Applicable Range of θ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0° | - | 90° | - | - | - | [315°, 45°] | [0°, 180°] |
| Left | 90° | - | 90° | - | - | - | [45°, 135°] | [0°, 180°] |
| Back | 180° | - | 90° | - | - | - | [135°, 225°] | [0°, 180°] |
| Right | 270° | - | 90° | - | - | - | [225°, 315°] | [0°, 180°] |

Observation 1: Average RCS for human RCS model 2 (component A) depends on various parameters and conditions of the body such as:

- Body size (kid, adult)

- Body type (thin, fat)

- Body position (standing, sitting)

- Gender (male, female)

- Attire (light, thick)

- Angle, frequency, etc.

Proposal 2: For human RCS model 2, support multiple values for A to reflect different human body characteristics.

Proposal 3: RCS measurements provided by the companies should include details related to sensing type and scenario (ground based, airborne based, etc.).

Spatial consistency

Proposal 4: Initial random phase and XPR of CPMtx,sp and CPMsp,rx are link-correlated for spatial consistency modeling and spatially consistent mobility modeling of the same target.

Proposal 5: Initial random phase and XPR of CPMsp is link-correlated for spatially consistent mobility modeling of the same target only.

Proposal 6: The RCS parameter, B2, is link-correlated for spatially consistent mobility modeling of the same target only.

Observation 2: If XPR and initial random phase of CPMsp are modeled as link correlated for spatially consistency mobility modeling of a scattering point, it is not necessary to model angular correlation of these parameters as part of the RCS model for the target.

LOS probability for UAV targets; Case 7 & Case 9

Observation 3: The accurate LoS probability modelling for Case 7 and Case 9 requires measurement campaigns, which is not feasible due to time limitations in Rel. 19.

Proposal 7: Use validated 3GPP channel models with modifications to model the LoS probability for Case 7 and 9.

Observation 4: The LoS probability in TR 36.777 is modelled based on the height segments (low-UAV region, mid-UAV region and high-UAV region) of the aerial UE.

Proposal 8: The LoS probability for Case 7 and Case 9 is defined based on the aerial UE height segments, similarly to TR 36.777.

Proposal 9: Use Table 2 for defining the LoS probability for Case 9 between two aerial UE nodes for each scenario (UMi-AV, UMa-AV, RMa-AV).

Table 2: Table defining the LoS probability between two nodes (node 1 and node 2) as a function of height regions for each node

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi, UMa, RMa LoS probability as defined from Table 7.4.2-1 in TR 38.901 | UMi-AV, UMa-AV, RMa-AV Los probability as defined for BS to mid UAV region from Table B-1 in TR 36.777 | Dense urban, urban and suburban and rural LoS probability from Table 6.6.1-1 in TR 38.811 (NTN model) |
| Mid-UAV | UMi-AV, UMa-AV, RMa-AV Los probability as defined for BS to mid UAV region from Table B-1 in TR 36.777 | UMi-AV, UMa-AV, RMa-AV Los probability as defined for BS to mid UAV region from Table B-1 in TR 36.777 | 1 |
| High-UAV | Dense urban, urban and suburban and rural LoS probability from Table 6.6.1-1 in TR 38.811 (NTN model) | 1 | 1 |
| Note: The corresponding height values for each node for each scenario (UMi-AV, UMa-AV and RMa-AV) is defined in Table 1. |  |  |  |

Observation 5: Case 7 can be considered as a special case of Case 9 with height of one node in the low-UAV region.

Proposal 10: Define the LoS probability for Case 7 between a normal UE and an aerial UE for each scenario (UMi-AV, UMa-AV, RMa-AV) using the following table

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| normal UE | UMi, UMa, RMa LoS probability as defined from Table 7.4.2-1 in TR 38.901 | UMi-AV, UMa-AV, RMa-AV Los probability as defined for BS to mid UAV region from Table B-1 in TR 36.777 | Dense urban, urban and suburban and rural LoS probability from Table 6.6.1-1 in TR 38.811 (NTN model) |
| Note: The corresponding height values for each node for each scenario (UMi-AV, UMa-AV and RMa-AV) is defined in Table 1. |  |  |  |

Shadow fading

Proposal 11: Use Table 3 for defining pathloss and shadow fading for Case 9 between two aerial UE nodes for each scenario (UMi-AV, UMa-AV, RMa-AV).

Table 3: Pathloss and shadow fading between two nodes as a function of height regions for each node

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi, UMa, RMa Pathloss models and shadow fading from Table 7.4.1-1 in TR 38.901 | UMi-AV, UMa-AV, RMa-AV Pathloss models from Table B-2 and shadow fading from Table B-3 in TR 36.777 | The basic path loss model (including clutter loss and shadow fading) models from equation 6.6-4 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 |
| Mid-UAV | UMi-AV, UMa-AV, RMa-AV Pathloss models from Table B-2 and shadow fading from Table B-3 in TR 36.777 | UMi-AV, UMa-AV, RMa-AV Pathloss models from Table B-2 and shadow fading from Table B-3 in TR 36.777 | The free space path loss (FSPL) model from equation 6.6-2 and shadow fading for LOS in Tables 6.6.2-1, 6.6.2-2, 6.6.2.-3 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 |
| High-UAV | The basic path loss model (including clutter loss and shadow fading) models from equation 6.6-4 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 | FSPL model from equation 6.6-2 and shadow fading for LOS in Tables 6.6.2-1, 6.6.2-2, 6.6.2.-3 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 | FSPL model from equation 6.6-2 and shadow fading for LOS in Tables 6.6.2-1, 6.6.2-2, 6.6.2.-3 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 |
| Note: The corresponding height values for each node for each scenario (UMi-AV, UMa-AV and RMa-AV) is defined in Table 1. |  |  |  |

Proposal 12: Use the following table to define pathloss and shadow fading for Case 7 between a normal UE and an aerial UE for each scenario (UMi-AV, UMa-AV, RMa-AV).

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| normal UE | UMi, UMa, RMa Pathloss models and shadow fading from Table 7.4.1-1 in TR 38.901 | UMi-AV, UMa-AV, RMa-AV Pathloss models from Table B-2 and shadow fading from Table B-3 in TR 36.777 | The basic path loss model (including clutter loss and shadow fading) models from equation 6.6-4 in TR 38.811 for dense urban, urban and suburban and rural scenarios. Note: Use S band models for FR1 and Ka-band models for FR2 |
| Note: The corresponding height values for each node for each scenario (UMi-AV, UMa-AV and RMa-AV) is defined in Table 1. |  |  |  |

Background channel

Proposal 13: Regarding combination of target and background channel, support Option 2 Alt1 “Power normalization on both target channel and background channel”

Proposal 14: Adopt a scaling parameter which defines a relative power difference between the background channel and target channel

Environment object

Proposal 15: When the EO type-2 is modeled in the target channel, adopt Option B, “Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model”

Proposal 16: An unintended target is a blockage factor in the target channel and further study whether Blockage model A or Blockage model B should be implemented

Power threshold for dropping concatenated links

Observation 6: For option 0 to generate indirect paths of NLOS ray+ NLOS ray in the target channel, we need to normalize ray power by number of rays per cluster(M).

Proposal 17: Normalize rays in Option 0 by a scaling factor (e.g.,  1/M where M is the number of rays in target-Rx link) to avoid power inflation through concatenation

Observation 7: For Indoor Open Office scenario, nearly 40% and 3% of paths are dropped with the path dropping threshold of -25dB and -50dB, respectively.

Observation 8: For Indoor Factory scenario, nearly 45% and 4% of paths are dropped with the path dropping threshold of -25dB and -50dB, respectively.

Proposal 18: Do not adopt path dropping after concatenation

Modeling micro-Doppler

Proposal 19: Adopt micro-Doppler functions for humans and UAVs provided in Table 5.

Table 5: Micro-Doppler functions for micro-motions of humans and UAVs.

| Type of target | Formula of micro-Doppler function φmicro,p(t) |
| --- | --- |
| Human respiration/heartbeat | φmicro,p(t)=R0+rpDsin2πfrtmax is the initial location offset. is the location vector of human. is the maximum amplitude of respiration/heartbeat. is the frequency of respiration/heartbeat. |
| Human walking/running | φmicro,p(t)=R0+rpp=1NpDpsin2πfpt is the initial location offset. is the location vector of human. is the number of human segments. is the maximum amplitude of motion of pth segment. is the frequency of motion of pth segment. |
| UAV propeller rotation | φmicro,k,p(t)=Rj+ruL2cosΩjt+ϕj+k2πNB j=1,⋯,NR;k=0,⋯,NB-1 For kth rotation blade on the jth rotor of UAV is the initial location offset of jth rotor and is the initial phase of jth rotor. is the location vector of UAV is the the distance between the center of blade and the center of the rotation and is the frequency of jth blade rotation. |

Sensing area

Proposal 20: Define a sensing coverage area for the ISAC channel model where the area includes only possible points for which the UE or network can localize an object within a specific accuracy requirement.

# Reference

- Chairman’s notes, RAN1#120b, Wuhan, China, Apr., 2025.

- Chairman’s notes, RAN1#118b, Hefei, China, Oct., 2024.

- Chairman’s notes, RAN1#120, Athens, Greece, Feb., 2025.

- 3GPP TR 38.901 V17.1.0 “Study on channel model for frequencies from 0.5 to 100 GHz” (2023-12).

- 3GPP TR 36.777, “Study on Enhanced LTE Support for Aerial Vehicles, ” v 15.0.0., Dec., 2017.

- 3GPP TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks”, v15.4.0, Sept., 2020.

- Chairman’s notes, RAN1#116, Athens, Greece, Feb., 2024.

- Chairman’s notes, RAN1#119, Orland, USA, Dec., 2024.

- ETSI Draft GR ISC 002: "Integrated Sensing and Communications (ISAC); Channel Modeling, Measurements and Evaluation Methodology".

- 3GPP TR 38.859, “Study on expanded and improved NR positioning; (Release 18)”

- J. Lou et al., "A Unified Channel Model for Both Communication and Sensing in Integrated Sensing and Communication Systems," VTC2023-Fall, Hong Kong, Hong Kong, 2023, pp. 1-6, doi: 10.1109/VTC2023-Fall60731.2023.10333766.

# Appendix

## Appendix – A (Simulation setup for multipoint target modelling)

Scenario description

| Parameter | Values |
| --- | --- |
| frequency | 3.5 GHz |
| Bandwidth | 100 MHz |
| Total gNB TX power, dBm | 23 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |
| gNB antenna height | 3 m |
| gNB location | (10, 30, 3) |
| Scenario Layout | Indoor Factory |
| Object Shape | Human |
| Object Locations (m) | - Location of the human target in Scenario #1 - (14, 20, 3) - Location of the human target in Scenario #2 - (20, 20, 3) - Location of the human target in Scenario #3 - (60, 20, 3) |
| Relative coordinates of each body part (relative to the reference point) (m) | - Lower belly(reference) - (0, 0, 0) - Belly - (0 0 0.20) - Chest (right side) - (0, -0.20, 0.25) - Chest (left side) - (0, 0.20, 0.25) - Neck - (0,0, 0.5) - Head - (0 0 0.9) - Legs – (0, 0, -0.25) - Right hand - (0 -0.8572 -0.162) - Left hand - (0 0.7529 -0.529) |
| UE location | (5.86, 20, 3) |
| UE mobility | static |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = 0.5λ |
| UE height, m | 3 m (fixed) |
| Minimum distance between gNB and Object | 2 m |
| Minimum distance between UE and Object | 2 m |
| LOS condition for Sensing Link | gNB to Object link – forced LOS Object to UE link – forced LOS |

## Appendix – B (Simulation setup for effect of different power threshold for NLOS-NLOS path analysis)

Scenario description

| Parameter | Values |
| --- | --- |
| frequency | 3.5 GHz |
| Bandwidth | 100 MHz |
| Total gNB TX power, dBm | 24 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |
| gNB antenna radiation pattern | Isotropic |
| gNB location | (10, 30, 3) for Indoor Open office scenario and (10,30,8) for InF-SH scenario |
| Channel model | Indoor Open Office, Indoor Factory(SH) Number of clusters for Tx-target and target-Rx link = 20 |
| Number of Objects | 10 |
| Object Location | Within 20 m radius of gNB location (minimum distance between gNB-target is 10 m) |
| Object Shape | Human |
| Object RCS | Fixed value (0 dBsm) |
| UE mobility | static |
| UE antenna radiation pattern | Isotropic |
| UE height, m | Fixed – 3 m |
| LOS condition for Sensing Link | gNB to Object link – forced LOS Object to UE link – forced LOS |
| LOS condition for Communication Link | According to probability distribution specified in 3GPP TR 38.901 |
| Note 1: According to 3GPP TR 38.802 Note 3: According to 3GPP TR 38.901 |  |

## Appendix – C (Simulation setup for sensing coverage area)

Scenario description

| Parameter | Values |
| --- | --- |
| frequency | 3.5 GHz |
| Bandwidth | 100 MHz |
| Total gNB TX power, dBm | 23 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |
| gNB antenna height | 3 m |
| gNB location | (10, 30, 3) |
| Scenario Layout | Indoor Factory |
| Object Shape | Square plate (3m*3m) |
| UE location | (110, 30, 3) |
| UE mobility | static |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = 0.5λ |
| UE height, m | 3 m (fixed) |
| Minimum distance between gNB and Object | 2 m |
| Minimum distance between UE and Object | 1 m |
| LOS condition for Sensing Link | gNB to Object link – forced LOS Object to UE link – forced LOS |