3GPP TSG RAN WG1 #121	R1-2504087

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	9.3.1

Source: 	Fujitsu

Title:	Discussion on SBFD TX/RX/measurement procedures

Document for:		Discussion and decision

Introduction

The Rel-19 work item on evolution of NR duplex operation includes a list of objectives as in the WID[1], out of which the following objectives should be discussed under agenda item 9.3.1.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

In this contribution, we will share our views on indication of frequency and time location of SBFD subbands, and transmissions and receptions in SBFD symbols and/or non-SBFD symbols for SBFD-aware UEs.

Discussion

SBFD configuration

Regarding UE-specific configuration, in previous discussions, the most controversial part is whether UE-specific GB is needed. In our understanding, this depends on RAN4 study. If it is needed, RAN4 can define the candidate sizes. Then the cell-specific configuration can guarantee that the cell-specific GB is not smaller than any UE-specific GB. Therefore, it is not necessary to support UE-specific configuration.

Proposal 1: Do not support UE-specific configuration of SBFD subbands.

Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details

SBFD symbols are configured based on referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon. For TDD UL-DL configuration, the UE needs to determine the DL/UL/F symbols for DL/UL BWPs based on the DL/UL/F symbols configured with respect to referenceSubcarrierSpacing. Similarly, for SBFD configuration, the UE also needs to determine the SBFD/non-SBFD symbols for DL/UL BWPs based on the configuration with respect to referenceSubcarrierSpacing. The corresponding rules should be discussed.

As clarified in spec, referenceSubcarrierSpacing is smaller or equal to the SCS of any DL/UL BWP. For a DL/UL BWP with NCP, a symbol of the DL/UL BWP would overlap with only one SBFD symbol or only one non-SBFD symbol for referenceSubcarrierSpacing. So, it is obvious that a symbol overlaps with only SBFD/non-SBFD symbol of the reference SCS should also be an SBFD/non-SBFD symbol. However, For a DL/UL BWP with ECP, a symbol may simultaneously overlap with an SBFD symbol and a non-SBFD symbol for referenceSubcarrierSpacing, as shown in Figure 1. In this case, whether the symbol for the DL/UL BWP should be an SBFD symbol or non-SBFD symbol needs to be clarified.

Figure 1. BWP symbol overlapping with an SBFD symbol and a non-SBFD symbol for referenceSubcarrierSpacing

The following options can be considered.

- Option 1: the symbol is an SBFD symbol.

- Option 2: the symbol is a non-SBFD symbol.

- Option 3: the symbol is a gap between SBFD symbols and non-SBFD symbols.

Proposal 2: UE determines SBFD/non-SBFD symbols for active DL/UL BWPs with NCP or ECP based on the SBFD/non-SBFD symbols configured with respect to referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon. For the active DL/UL BWP:

- a symbol overlapping with only SBFD symbol or only non-SBFD symbol for referenceSubcarrierSpacing is considered as an SBFD symbol or a non-SBFD symbol, respectively.

- a symbol overlapping with an SBFD symbol and a non-SBFD symbol for referenceSubcarrierSpacing is considered as (down-select from the following options):

  - Option 1: an SBFD symbol.

  - Option 2: a non-SBFD symbol.

  - Option 3: a gap between SBFD symbols and non-SBFD symbols.

Separate power control

Agreement For a single TRP scenario, support separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Decide in RAN1#118 whether separate UL power control for FR2 is supported based on UL spatial relation Info framework and/or unified TCI state framework. Agreement For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols - FFS: Details of the separate power control parameter Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

According to the agreements, separate power control for PRACH/PUSCH/PUCCH/SRS in SBFD symbols and non-SBFD symbols is supported, with FFS on details of separate power control parameters.

For all the UL signals/channels, the transmitting power is limited by the UE configured maximum output power PCMAX,f,c. For example, for a transmission occasion, the UE could configure a maximum output power within bounds that are determined based on NW configured maximum power, UE power class and MPR etc. To avoid severe UE-UE CLI in SBFD symbols, it is preferable to implement additional limitations/reductions to the maximum output power for SBFD symbols. To support that, an additional parameter can be used to determine maximum output power for SBFD symbols, and this parameter value can be configured by the gNB.

Proposal 3: Support separate maximum output power for SBFD symbols and non-SBFD symbols.

- An additional parameter is used to determine the maximum output power for SBFD symbols.

The introduction of dedicated P0AlphaSets for SBFD symbols enables separate closed-loop power control configurations for SBFD and non-SBFD symbols. However, the mechanism for supporting separate closed-loop power control through TPC commands in DCI remains unspecified.

A DCI may schedule UL transmissions across SBFD symbols and non-SBFD symbols in same or different slots. To support separate closed-loop power control for transmissions in SBFD symbols and transmissions in non-SBFD symbols scheduled by the same DCI, two TPC commands should be provided in the DCI.

Proposal 4: To support per symbol-type closed-loop power control for PUSCH transmissions or PUCCH transmissions scheduled by a DCI format, two TPC commands can be included in the DCI format.

- When the DCI format includes two TPC commands, each TPC command applies for each closed-loop power control respectively.

In the legacy, the UE determines transmission power per transmission occasion which is defined as below. For PUSCH repetition Type B, a PUSCH transmission occasion refers to a nominal repetition. That is, the UE would determine the transmission power per nominal repetition.

A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214].

In SBFD operation, if the UE is configured with separate power control parameters for SBFD symbols and non-SBFD symbols, when a nominal repetition spans both SBFD symbols and non-SBFD symbols, the applicable power control parameters become ambiguous. On the other hand, according to the agreement below, each actual repetition would be either in SBFD symbols or in non-SBFD symbols. To maintain consistent implementation of separate power control for different symbol types, the UE should determine transmission power individually for each actual repetition, instead of for each nominal repetition.

Agreement For PUSCH repetition type B, adopt Option 1. - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

Proposal 5: In SBFD operation, for PUSCH repetition Type B, the UE determines transmission power per actual repetition (instead of nominal repetition in the legacy).

Another issue is related to PHR. Since separate power control for SBFD symbols and non-SBFD symbols is supported, separate PHR for SBFD symbols and non-SBFD symbols is needed. In previous releases, twoPHRmode and corresponding MAC CEs which can contain two PHRs per cell were introduced. However, those legacy MAC CEs are designed for m-TRP scenario and cannot be reused for SBFD operation. To support separate PHR for SBFD symbols and non-SBFD symbols, twoPHRmode can be reused, and new MAC CEs should be introduced.

Proposal 6: Support separate PHRs for SBFD symbols and non-SBFD symbols.

- Resue twoPHRmode and introduce new MAC CEs to support the separate PHRs.

PDSCH/PUSCH

HARQ process ID allocation

In the legacy, for multi-PUSCH scheduled by a single DCI and multi-PUSCH configured grant, PUSCH(s) not transmitted due to collision with legacy TDD UL-DL configurations or SSB is not assigned with an HARQ process ID. For multi-PDSCH scheduled by a single DCI, PDSCH(s) not received due to collision with legacy TDD UL-DL configurations is not assigned with an HARQ process ID. The corresponding detailed rules can be found in the spec below.

For multi-PUSCH scheduled by a single DCI: When the UE is scheduled with multiple PUSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied if nrofHARQ-ProcessesForPUSCH is provided, or with modulo operation of 16 applied, otherwise. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

For multi-PUSCH configured grant: When nrofSlotsInCG-Period is configured for Type 1 configured grant or Type 2 configured grant, HARQ process ID for the first configured PUSCH grant and each subsequent valid configured PUSCH grant within a periodicity of the configuration is determined as in clause 5.4.1 of [10, TS 38.321], where a valid configured PUSCH grant is the one not colliding with the DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, and not colliding with a symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst as described in clause 11.1 of [6, TS 38.213].

For multi-PDSCH scheduled by a single DCI: A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1, 1_2, 1_3, 4_0, 4_1, or 4_2 decode the corresponding PDSCHs as indicated by that DCI. When the UE is scheduled with multiple PDSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, HARQ process ID is then incremented by 1 for each subsequent PDSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPDSCH applied if nrofHARQ-ProcessesForPDSCH is provided, or with modulo operation of nrofHARQ-ProcessesForPDSCH-v1700 applied if or nrofHARQ-ProcessesForPDSCH-v1700 is provided, or with modulo operation of 8 applied, otherwise. HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided.

However, in SBFD operation, the rules to determine the PUSCH(s) not transmitted and PDSCH(s) not received are different from the legacy. Therefore, the corresponding rules for HARQ process ID allocation should be updated.

For multi-PUSCH scheduled by a single DCI and multi-PUSCH configured grant, in the case of Configuration 1, when the valid symbol type is SBFD symbols, if link direction indication is not provided or supported for SBFD symbols, only PUSCH(s) within SBFD symbols and not overlapping with SSB would be transmitted. In Figure 2 (assuming all PUSCHs not overlapping with SSB), only PUSCH2 should be allocated with an HARQ process ID.

Figure 2: Configuration 1, SBFD symbols as valid symbol type

When the valid symbol type is non-SBFD symbols, only PUSCH(s) within non-SBFD symbols not indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated and not overlapping with SSB would be transmitted. In Figure 3 (assuming all PUSCHs not overlapping with SSB), only PUSCH4 should be allocated with an HARQ process ID.

Figure 3: Configuration 1, non-SBFD symbols as valid symbol type

In the case of Configuration 2, if link direction indication is not provided or supported for SBFD symbols, only PUSCH(s) within SBFD symbols or within non-SBFD symbols not indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated and not overlapping with SSB would be transmitted. In Figure 4 (assuming all PUSCHs not overlapping with SSB), only PUSCH2 and PUSCH4 should be allocated with a HARQ process ID, respectively.

Figure 4: Configuration 2

Proposal 7: To determine HARQ process ID for multi-PUSCH scheduled by a single DCI or multi-PUSCH configured grant,

- In Configuration 1,

  - If the valid symbol type is SBFD symbols, HARQ process ID is not allocated to a PUSCH overlapping with a non-SBFD symbol or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

  - If the valid symbol type is non-SBFD symbols, HARQ process ID is not allocated to a PUSCH overlapping with an SBFD symbol, or a non-SBFD symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

- In Configuration 2,

  - HARQ process ID is not allocated to a PUSCH overlapping with a non-SBFD symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, and is not allocated to a PUSCH across SBFD symbols and non-SBFD symbols.

Similarly, for multi-PDSCH scheduled by a single DCI, in the case of Configuration 1, when the valid symbol type is SBFD symbols, if link direction indication is not provided or supported for SBFD symbols, only PDSCH(s) within SBFD symbols would be transmitted. When the valid symbol type is non-SBFD symbols, only PDSCH(s) within non-SBFD symbols not indicated as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated would be transmitted. In the case of Configuration 2, if link direction indication is not provided or supported for SBFD symbols, only PDSCH(s) within SBFD symbols or within non-SBFD symbols not indicated as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated would be transmitted.

Proposal 8: To determine HARQ process ID for multi-PDSCH scheduled by a single DCI,

- In Configuration 1,

  - If the valid symbol type is SBFD symbols, HARQ process ID is not allocated to a PDSCH overlapping with a non-SBFD symbol.

  - If the valid symbol type is non-SBFD symbols, HARQ process ID is not allocated to a PDSCH overlapping with an SBFD symbol, or a non-SBFD symbol that is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

- In Configuration 2,

  - HARQ process ID is not allocated to a PDSCH overlapping with a non-SBFD symbol that is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and is not allocated to a PDSCH across SBFD symbols and non-SBFD symbols.

multiple SPS PDSCHs in a slot

In the legacy, more than one SPS configuration can be activated. However, subject to UE capabilities, the UE may only be able to receive up to one/two PDSCHs in a slot. If there is more than one SPS PDSCH in a slot, the UE needs to do selection. The procedure is defined as below.

If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE.

For SBFD operation, in case of Configuration 1, the rule should be enhanced to resolve collision with invalid symbol type. Besides, the impacts of tdd-UL-DL-ConfigurationDedicated should be considered. For example, if tdd-UL-DL-ConfigurationDedicated is not applicable to SBFD symbols but applicable non-SBFD symbols, for PDSCH in non-SBFD symbols, tdd-UL-DL-ConfigurationDedicated should be ignored for determining whether there exists collision, while for PDSCH in non-SBFD symbols, it should be considered.

Proposal 9: For PDSCH selection in case of multiple PDSCHs in a slot, the UE determines if more than one SPS PDSCH are in a slot after excluding SPS PDSCH(s) according to the following rules:

- In Configuration 1,

  - If the valid symbol type is SBFD symbols, excluding a PDSCH overlapping with a non-SBFD symbol.

  - If the valid symbol type is non-SBFD symbols, excluding a PDSCH overlapping with an SBFD symbol, or a non-SBFD symbol that is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

- In Configuration 2,

  - Excluding a PDSCH overlapping with a non-SBFD symbol that is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or a PDSCH across SBFD symbols and non-SBFD symbols.

PUSCH repetition type B

In general, to determine the actual repetitions for PUSCH repetition Type B, there are 2 steps.

- Step 1: Determine invalid symbol(s) for PUSCH repetition Type B transmission.

- Step 2: Determine actual repetitions according to the invalid symbol(s) and slot boundary.

The corresponding detailed rules can be found in the spec below.

For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission. - For a reduced capability half-duplex UE in paired spectrum, symbols that do not start or end at least or , respectively, from the last or first symbol of an SS/PBCH block with index indicated by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or by NonCellDefiningSSB, or by ssb-PositionsInBurst in SSB-MTC-AdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are considered as invalid symbol(s) for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, if numberOfInvalidSymbolsForDL-UL-Switching is configured, numberOfInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated as downlink in each consecutive set of all symbols that are indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberOfInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon. - For operation with shared spectrum channel access with semi-static channel occupancy, symbols in an idle duration associated with a periodic channel occupancy as described in Clause 4.3.1.1 of [16, TS 37.213], or in an idle duration in a period associated with an initiated channel occupancy as described in Clause 4.3.2. of [16, TS 37.213] are considered as invalid symbol(s) for PUSCH repetition Type B transmission. - The UE may be configured with the higher layer parameter invalidSymbolPattern, which provides a symbol level bitmap spanning one or two slots (higher layer parameter symbols given by invalidSymbolPattern). A bit value equal to 1 in the symbol level bitmap symbols indicates that the corresponding symbol is an invalid symbol for PUSCH repetition Type B transmission. The UE may be additionally configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by invalidSymbolPattern), where each bit of periodicityAndPattern corresponds to a unit equal to a duration of the symbol level bitmap symbols, and a bit value equal to 1 indicates that the symbol level bitmap symbols is present in the unit. The periodicityAndPattern can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40 msec. The first symbol of periodicityAndPattern every 40 msec/P periods is a first symbol in frame 𝑛𝑓 mod 4 = 0, where P is the duration of periodicityAndPattern-r16 in units of msec. When periodicityAndPattern is not configured, for a symbol level bitmap spanning two slots, the bits of the first and second slots correspond respectively to even and odd slots of a radio frame, and for a symbol level bitmap spanning one slot, the bits of the slot correspond to every slot of a radio frame. If invalidSymbolPattern is configured, when the UE applies the invalid symbol pattern is determined as follows: - if the PUSCH is scheduled by DCI format 0_1, or corresponds to a Type 2 configured grant activated by DCI format 0_1, and if invalidSymbolPatternIndicatorDCI-0-1 is configured, - if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern; - otherwise, the UE does not apply the invalid symbol pattern; - if the PUSCH is scheduled by DCI format 0_2, or corresponds to a Type 2 configured grant activated by DCI format 0_2, and if invalidSymbolPatternIndicatorDCI-0-2 is configured, - if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern; - otherwise, the UE does not apply the invalid symbol pattern; - otherwise, the UE applies the invalid symbol pattern. - If the UE - is configured with multiple serving cells within a cell group and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, - a symbol indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, or by NonCellDefiningSSB, or by ssb-PositionsInBurst in SSB-MTC-AdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting is considered as an invalid symbol for PUSCH repetition Type B transmission in - any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA and - a symbol is considered as an invalid symbol in another cell among the set of serving cell(s) provided with directionalCollisionHandling-r16 for PUSCH repetition Type B transmission with Type 1 or Type 2 configured grant except for the first Type 2 PUSCH transmission (including all repetitions) after activation if the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on the reference cell as defined in Clause 11.1 of [6, TS 38.213], or the UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on the reference cell in the symbol. For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of all potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot. An actual repetition with a single symbol is omitted except for the case of L=1. An actual repetition is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213], and Clause 5.34.3 of [10, TS 38.321]. The UE shall repeat the TB across actual repetitions. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2, where N=1.

In the previous meeting, the agreements below were achieved. However, how to determine the invalid symbol(s) is yet specified.

Agreement For PUSCH repetition type B, adopt Option 1. - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

In our view, to determine the invalid symbols, at least the conditions related to downlink indication by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated should be revised.

Proposal 10: For PUSCH repetition Type B,

- A non-SBFD symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol.

- If numberOfInvalidSymbolsForDL-UL-Switching is configured, numberOfInvalidSymbolsForDL-UL-Switching symbol(s) after the last non-SBFD symbol that is indicated as downlink in each consecutive set of all symbols that are indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as invalid symbol(s).

UCI multiplexing

Since the interference environments for SBFD symbols and non-SBFD symbols are different, it can be considered to support separate β offset values for UCI multiplexing on PUSCHs transmitted on SBFD symbols and non-SBFD symbols. β is used to determine the number of UCI REs and balance the performance between UCI and PUSCH. Separate β offset values can better adapt to the interference environments for SBFD symbols and non-SBFD symbols. For example, a single β is configured for Type 1 configured grant in legacy, which may not be flexible enough when Type 1 configured grant PUSCH is across SBFD symbols and non-SBFD symbols. Therefore, separate β offset values can be used for Type 1 configured grant PUSCH across SBFD symbols and non-SBFD symbols.

Proposal 11: For UCI multiplexing on PUSCH, separate β offset values can be supported to determine the number of UCI REs for SBFD symbols and non-SBFD symbols.

SRS

Each SRS resource set with usage set to 'nonCodebook' is associated with a CSI-RS. This CSI-RS is used to determine the precoder of the SRS. To ensure accurate precoder derivation, it is preferred that the CSI-RS shares the same symbol type (SBFD/non-SBFD) as the target SRS resource set and is measured only in the symbol type.

Proposal 12: For each SRS resource set with usage set to 'nonCodebook', the associated CSI-RS shares the same symbol type as the SRS resource set.

Carrier aggregation

In the RAN1#120 meeting, the following agreement was achieved for SBFD in CA.

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

Regarding the FFS, in our understanding, ‘half-duplex CA case’ refers to directional collision handling between reference and other cell(s) for half-duplex operation in TDD CA. In the legacy, if the UE is configured with directional handling, it should determine a reference cell as below.

the UE determines a reference cell for a symbol as an active cell with the smallest cell index among - the configured multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - the cells of each band respectively if the UE is capable of simultaneous transmission and reception by simultaneousRxTxInterBandCA for the configured multiple serving cells, where the symbol is configured as - downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol.

With that understanding, if the SBFD cell is determined as the reference cell, how to determine the link direction of the symbol needs to be enhanced because of the new symbol types, i.e. SBFD symbols and non-SBFD symbols, of the reference cell. The spirit of the current rule should be followed.

Proposal 13: For half-duplex CA, if the SBFD cell is determined as the reference cell of a symbol, the symbol is configured as

- downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the symbol is not an SBFD symbol

- uplink, if the symbol is a flexible or SBFD symbol and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol

- downlink, if the symbol is a flexible or SBFD symbol and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol.

Conclusion

According to the discussions above, we have the following proposals.

SBFD configuration

Proposal 1: Do not support UE-specific configuration of SBFD subbands.

Proposal 2: UE determines SBFD/non-SBFD symbols for active DL/UL BWPs with NCP or ECP based on the SBFD/non-SBFD symbols configured with respect to referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon. For the active DL/UL BWP:

- a symbol overlapping with only SBFD symbol or only non-SBFD symbol for referenceSubcarrierSpacing is considered as an SBFD symbol or a non-SBFD symbol, respectively.

- a symbol overlapping with an SBFD symbol and a non-SBFD symbol for referenceSubcarrierSpacing is considered as (down-select from the following options):

  - Option 1: an SBFD symbol.

  - Option 2: a non-SBFD symbol.

  - Option 3: a gap between SBFD symbols and non-SBFD symbols.

Separate power control

Proposal 3: Support separate maximum output power for SBFD symbols and non-SBFD symbols.

- An additional parameter is used to determine the maximum output power for SBFD symbols.

Proposal 4: To support per symbol-type closed-loop power control for PUSCH transmissions or PUCCH transmissions scheduled by a DCI format, two TPC commands can be included in the DCI format.

- When the DCI format includes two TPC commands, each TPC command applies for each closed-loop power control respectively.

Proposal 5: In SBFD operation, for PUSCH repetition Type B, the UE determines transmission power per actual repetition (instead of nominal repetition in the legacy).

Proposal 6: Support separate PHRs for SBFD symbols and non-SBFD symbols.

- Resue twoPHRmode and introduce new MAC CEs to support the separate PHRs.

PDSCH/PUSCH

Proposal 7: To determine HARQ process ID for multi-PUSCH scheduled by a single DCI or multi-PUSCH configured grant,

- In Configuration 1,

  - If the valid symbol type is SBFD symbols, HARQ process ID is not allocated to a PUSCH overlapping with a non-SBFD symbol or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

  - If the valid symbol type is non-SBFD symbols, HARQ process ID is not allocated to a PUSCH overlapping with an SBFD symbol, or a non-SBFD symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

- In Configuration 2,

  - HARQ process ID is not allocated to a PUSCH overlapping with a non-SBFD symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, and is not allocated to a PUSCH across SBFD symbols and non-SBFD symbols.

Proposal 8: To determine HARQ process ID for multi-PDSCH scheduled by a single DCI,

- In Configuration 1,

  - If the valid symbol type is SBFD symbols, HARQ process ID is not allocated to a PDSCH overlapping with a non-SBFD symbol.

  - If the valid symbol type is non-SBFD symbols, HARQ process ID is not allocated to a PDSCH overlapping with an SBFD symbol, or a non-SBFD symbol that is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

- In Configuration 2,

  - HARQ process ID is not allocated to a PDSCH overlapping with a non-SBFD symbol that is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and is not allocated to a PDSCH across SBFD symbols and non-SBFD symbols.

Proposal 9: For PDSCH selection in case of multiple PDSCHs in a slot, the UE determines if more than one SPS PDSCH are in a slot after excluding SPS PDSCH(s) according to the following rules:

- In Configuration 1,

  - If the valid symbol type is SBFD symbols, excluding a PDSCH overlapping with a non-SBFD symbol.

  - If the valid symbol type is non-SBFD symbols, excluding a PDSCH overlapping with an SBFD symbol, or a non-SBFD symbol that is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

- In Configuration 2,

  - Excluding a PDSCH overlapping with a non-SBFD symbol that is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or a PDSCH across SBFD symbols and non-SBFD symbols.

Proposal 10: For PUSCH repetition Type B,

- A non-SBFD symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol.

- If numberOfInvalidSymbolsForDL-UL-Switching is configured, numberOfInvalidSymbolsForDL-UL-Switching symbol(s) after the last non-SBFD symbol that is indicated as downlink in each consecutive set of all symbols that are indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as invalid symbol(s).

Proposal 11: For UCI multiplexing on PUSCH, separate β offset values can be supported to determine the number of UCI REs for SBFD symbols and non-SBFD symbols.

SRS

Proposal 12: For each SRS resource set with usage set to 'nonCodebook', the associated CSI-RS shares the same symbol type as the SRS resource set.

CA

Proposal 13: For half-duplex CA, if the SBFD cell is determined as the reference cell of a symbol, the symbol is configured as

- downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the symbol is not an SBFD symbol

- uplink, if the symbol is a flexible or SBFD symbol and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol

- downlink, if the symbol is a flexible or SBFD symbol and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol.

- References

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.