3GPP TSG RAN WG1 Meeting #121						          					    R1-2504113

Malta, MT, May 19th – 23th, 2025

Agenda item:		9.1.2

Source:	Nokia

Title:	AI/ML for Positioning Accuracy Enhancement

Document for:		Discussion and Decision

# Introduction

The RAN #102 meeting approved the Rel-19 WI on AI/ML for NR Air Interface [1], based on AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air [2] and revised in RAN#105 and RAN#107. In this contribution, we discuss the enhancements related to AI/ML for positioning, work item objectives related to AI/ML positioning sub-use cases, latest agreements, observations, and conclusions done in previous meetings (See Appendix A).

We organize this technical document in sections highlighting our main proposals and observations related to Case 1, Case 3a, and Case 3b as per the guidelines in [3]. In Section 2, we address the main aspects of performance monitoring. In Section 3 and Section 4, we provide relevant aspects to be considered for ensuring consistency and data collection details, respectively. Section 5 addresses aspects related to inference operation. Section 6 addresses details of the life cycle management (LCM) framework. In addition, Section 7 lists higher layer parameters impacting other WGs. Finally, in Section 8, observations and proposals are summarized.

# Performance monitoring

In this contribution focuses on open topics that RAN1 should address in this last work item meeting. For example, resolving the FFS for Option B related to monitoring for Case 1. Another topic that requires attention is the specification impact of label-free monitoring and triggering conditions for monitoring Case 1. Finally, the monitoring Case 3a is also discussed based on some RAN3 agreements.

## Label-based monitoring aspects for Case 1

In the previous meetings, the following agreements were achieved between companies to address aspects related to label-based monitoring.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

In addition, in RAN1#119 companies agreed to support Option A for Case 1 monitoring

Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

One FFS was solved in RAN1#120bis with the following agreement.

Agreement For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

In the main agreement, the FFS for Option B requires a solution from RAN1 perspective. As stated in the agreement, in Option B, the LMF performs monitoring metric calculation with the following details:

Option B-1: LMF does not have the UE's location information readily available. Hence, it needs to first obtain the ground truth label information of the target UE by performing the positioning of the target UE using a legacy positioning method. But, as discussed above, this compromises the quality of the ground truth label particularly in e.g., NLOS scenarios where the legacy positioning performance is proven to be unreliable. Hence, the monitoring results derived from such unreliable ground truth label may be misleading and therefore, this is not a desirable option.

Option B-2: The PRU measurements and the associated PRU ground truth labels are available at the LMF and are used for the monitoring purpose. The LMF provides to UE with the PRU measurements, and in response, the UE to reports inference result based on the PRU measurement. Because the LMF has the accurate PRU ground truth label, which is of high quality, the performance results are not compromised. Hence, Option B-2 a desirable option.

Based on the above discussion, Option B-1 can reuse legacy procedures and signaling for estimating the ground truth information without any involvement of PRUs. However, they raise concern on the quality of the generated labels and the validity of any monitoring results derived from them. On the other side, Option B-2 offers more reliable approach for monitoring thanks to the involvement of PRU.

For model performance monitoring of AI/ML positioning Case 1, “FFS: Option B” in RAN1#119 agreement is resolved by: For model performance monitoring of AI/ML positioning Case 1, supports Option B-2

For Case 1, the label-based monitoring may involve PRU, UE, and LMF entities. In this regard, Table 1 summarizes possible combinations between entities to derive the monitoring metrics and entities doing the decision based on the monitoring outcome. There are multiple options for the decision entity to make the final decision based on the monitoring output. In general, for label-based monitoring the metric calculation may be done on the UE-side and/or on the LMF-side.

Table 1 - Combination of scenarios between entities deriving monitoring metric for Case 1 label-based monitoring Option B and entities determining monitoring decision. The green cells show our preferred option.

| Performance monitoring option | Entity generating Part A data for performance monitoring | Entity deriving monitoring metric/outcome | Potential entity making LCM decision based on monitoring outcome | Expected specification impact |
| --- | --- | --- | --- | --- |
| Option B-1 | UE | LMF | LMF | Target UE reports at least inference outcome to LMF |
| Option B-2 | PRU | LMF | LMF | LMF delivers Part A data collected from PRU to target UE, and target UE reports inference outcome to LMF |

## Label-free monitoring aspects for Case 1

Label-free monitoring does not have a dependency on the availability of ground truth. So, it may be based on buffered inference output, drifting monitoring, and confidence level associated with the inference output.

To support the different variants of label-free monitoring it is expected that the target UE may enhance the monitoring metric by receiving assistance information from the LMF based on surrounding UEs. To enable such type of features for monitoring, the LMF may assist by collecting the monitoring metrics from UEs and sending such information to target UE, as a result, the label-free monitoring is enhanced.

For label-free monitoring there is a set of specification impacts which are listed in the following:

- The signalization of monitoring outcome to LMF,

- specific settings for the label-free performance monitoring method, which may require additional assistance from the LMF.

- The UE may request assistance data from the LMF to gather analytics of label-free monitoring metrics from other surrounding UEs to enhance the label-free monitoring.

Label-free monitoring for Case 1 has similar signalization requirements than label-based monitoring, the unique difference is that no message is required to deliver information relative to the ground truth.

For model performance monitoring of AI/ML positioning Case 1, supports label-free based monitoring for Case 1.

For Case 1, the label-free monitoring may involve UE, and LMF entities. Table 2 summarizes possible combinations between entities to derive the monitoring metrics and entities doing the decision based on the monitoring outcome. There are multiple options for the decision entity to make the final decision based on the monitoring output. In general, for label-free monitoring the metric calculation is done by the UE. However, the initial UE monitoring outcome may be enhanced by receiving assistance from the LMF.

Table 2 – Options for deriving monitoring metric for Case 1 label-free monitoring and entities determining monitoring decision. The green cells show our preferences between different options.

| Performance monitoring option | Entity generating Part A data for performance monitoring | Entity deriving monitoring metric/outcome | Potential entity making LCM decision based on monitoring outcome | Expected specification impact |
| --- | --- | --- | --- | --- |
| Target UE performs monitoring metric calculation | UE | UE | LMF | UE receives assistance from LMF for monitoring metric calculation, e.g., statistical function parameters configuration, UE reports monitoring metric to LMF |
| LMF performs monitoring metric calculation | UE | LMF | LMF | UE reports necessary data to LMF for monitoring, e.g., inference input statistics |

For both, label-based and label-free performance monitoring, the following common scenarios are identified:

- For the case where the monitoring metric is derived in the LMF-side, the reporting of measurements from the UE to LMF is required and it should be done prioritizing/using new IEs in LPP.

- For the cases when UE is the entity deriving the performance monitoring metric, UE must report to LMF (i) the derived monitoring metric to determine the monitoring outcome or (ii) the monitoring outcome. Both approaches may enable an life cycle management (LCM) decision based on performance monitoring outcome at the LMF side.

For model performance monitoring of AI/ML positioning Case 1, the target UE reports monitoring outcome at least when the LMF request performance monitoring.

## Monitoring triggering for Case 1

For both, label-based and label-free performance monitoring, various conditions may trigger the need for performance monitoring at the UE such as certain areas or time in the network impacting channel conditions, availability of TRPs, network load, etc. Since LMF assisted by gNB can have a better knowledge of such conditions in the network and their impact on the performance, e.g., based on past positioning QoS of different UEs, or network-side additional conditions related to specific Associated-ID information (TRPs location or Beam information), the LMF may configure the UE with certain conditions to trigger performance monitoring proactively. Otherwise, without any information, the UE may trigger monitoring unnecessarily, which results in additional signaling and processing overhead. Conversely, if UE avoids triggering of monitoring to save processing and/or signaling effort, this might result in degraded performance under considerably changing radio conditions. Therefore, LMF can let the UE to timely trigger monitoring and report its outcome to enable efficient LCM decisions.

For Case 1, LMF may configure UE with certain conditions to trigger performance monitoring. These conditions may include availability of TRPs, channel conditions, network load, and network-side additional conditions (e.g. Associated-ID information).

## Monitoring aspects for Case 3a

For Case 3a, the following agreement was done in RAN1#118.

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

For Case 3a monitoring, RAN3 got the following agreements in and RAN3#126 respectively.

Working Assumption (RAN3#125-bis) WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

Agreement (RAN3#126) Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

The decision done by RAN3 indicates that the NG-RAN (gNB) is independent of the LMF to perform monitoring metric calculation, which may be interpreted as NG-RAN is independent on executing monitoring decisions without any LMF dependency. In other words, the monitoring for Case 3a is up to NG-RAN (gNB) implementation.

(Conclusion) Monitoring for Case 3a is up to NG-RAN implementation and no specification impact is expected from RAN1 perspective.

# Ensuring consistency between training and inference

Based on the current agreements in agenda item 9.1.3.3 [2] clause 4.2.3, the following potential approaches may be considered to ensure consistency between training and inference: consistency based on explicit assistance data, implicit assistance data (Associated-ID), and consistency assisted by monitoring (UE/LMF performance assessment).

RAN 1 to consider the following approaches to ensure consistency between training and inference regarding NW-side additional conditions: (i) consistency based on explicit assistance data; (ii) based on implicit assistance data enabled by Associated-ID; and (iii) consistency assisted by monitoring.

As identified in the study item, the generalization is a concern that requires to be addressed in AIML positioning. To fill this gap, generic and specific solutions are presented.

Generic solutions are enabled by:

- Consistency based on explicit assistance data, and

- Consistency based on a generic definition of Associated-ID.

In addition, specific solutions focusing on Info #7 of legacy DL-TDOA assistance data, such as:

- Consistency based on explicit information without providing details of TRP location and Beam information

- Content of Associated-ID message and area granularity.

Complementing our contribution, to facilitate the discussion between companies, different options are proposed to the definition of Associated-ID. In addition, other aspects related to network-side additional conditions are discussed at the end of this subsection.

## Generic solutions to ensure consistency

### Consistency based on explicit assistance data

In the current discussion, consistency based on assistance data may be enabled by explicit information shared from LMF to UE. For instance, in RAN1#119, an agreement indicates that majority of assistance data used by legacy DL-TDOA are exposed explicitly. For example, PRS-configuration may be disclosed for consistency checking but this assistance data may not have an exact match between training data collection and inference, so it may be required to quantify the matching information by using a delta parameter and based on this delta, the UE may assess and select a suitable model. In addition, UE can share the preferred consistency delta information with the network in order to assist in reducing the gap between training and inference conditions (e.g. changing PRS configuration).

In Case 1, for consistency between training and inference, when explicit information is provided by assistance data, a delta indicating the variation of the assistance data is used to assess the consistency.

As complementary solution, RAN1 may consider other methods such as defining context scenarios for the inference operation, where context scenarios provide some high-level information or requirements related to the datasets used for training (in other words, high-level info related to NW-side additional conditions). This sort of information may be exchanged between the LMF and the UE prior to the inference. As UE-sided models are selected by the UE, the LMF may assist indicating the specific context scenario for the inference. Overall, how to map NW-additional conditions to some sort of high-level information that considers a context scenario is required to be defined to enable this sort of scheme. For Case 1, specific context scenarios based on areas with specific channel characteristics or TRP patterns may be mapped to fixed geographical information as TRPs transmitting DL PRS during inference.

For Case 1, RAN1 to consider LMF assistance related to validity area as specified in legacy and include additional granularity for consistency purposes.

### Consistency based on Associated-ID

To ensure consistency between training and inference, the data collection at the UE may follow specific data categorization based on a unique global identifier, which is composed at least by:

- the global cell ID (GCI), which is already available in the legacy specification, and

- Associated-ID.

Overall, the network identifiers must be capable of representing (a set of) NW-side additional conditions that is applicable in the training phase and refer to the same identifiers in the inference phase (when the corresponding NW assumptions match with the NW-side additional conditions). Here, the Associated-ID(s) are unique identifiers for specific network vendor, which may be used to differentiate the same or different sets of NW-side additional conditions applicable for AI/ML positioning.

For Case 1, to ensure consistency between training and inference related to NW-additional conditions, assistance data (e.g. PRS configuration, GCI) used for data collection (training) or inference shall be associated with the Associated-ID(s).

For consistency between training and inference in Case 1, if the Associated-ID is introduced as implicit indication for Info #7, the UE shall know what to assume for Info #7 when it receives an Associated-ID. As explicit information for TRPs is not provided, the Associated-ID may represent a group of TRPs, the UE needs to assume same physical properties for the TRPs e.g., number of TRPs, order of TRPs, locations of TRPs.

Similarly, in the case of DL-AoD positioning (TS 38.305), the UE position is estimated based on DL-PRS-RSRP and/or DL-PRS-RSRPP measurements taken at the UE of downlink radio signals from multiple NR TRPs, along with knowledge of the spatial information of the downlink radio signals and geographical coordinates of the TRPs. The UE performs these measurements using assistance data received from the LMF. The parameters used in the assistance data that may be transferred from LMF to UE for DL-AoD positioning are listed in Table 8.11.2.1.0-1 (TS 38.305). In Case UE needs to have at least Info#16, Info#17 from LMF to maintain the consistency between training and inference, the LMF could adopt the same approach as DL-TDOA to provide Info#16 and Info#17 implicit or explicit using the Associated-ID as highlighted in Alt1 and Alt 2 in the following agreement.

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs).

|  | Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TdoA as applicable |
| --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |
| 3 | DL-PRS configuration of candidate NR TRPs |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |
| 9 | PRS-only TP indication |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |
| 11 | LOS/NLOS indicators |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |
| 13 | Validity Area of the Assistance Data |
| 14 | PRU measurements together with the location information of the PRU |
| 15 | Data facilitating the integrity results determination of the calculated location |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |
| 17 | Expected Angle Assistance information |
| 18 | PRS priority list |

The Associated-ID as part of assistance data can assist the consistency as long as the meaning of Associated-ID is same for the UE regardless of the time. Therefore, Associated-ID is the most appropriate solution from the above alternatives to address the consistency issues without compromising the privacy of TRPs actual locations and beam information.

RAN1 down scope Alt-#3 and Alt#4 and only consider Alt#1 and Alt#2 (inclusion of Associated-ID) for both DL-TDOA and DL-AoD as part of assistance data for consistency checking in Case 1.

## Specific solutions to ensure consistency

### Consistency based on explicit information without providing details of TRP location and beam Information

Another mechanism to ensure consistency between training and inference may be enabled by exposing assistance data information by using partially explicit information. So, no generic container is used to disclose network-side additional conditions (e.g. Associated-ID) and there is not any explicit information exposed to UE. However, valuable information is disclosed to target UE without infringing proprietary information that NW vendors avoid disclosing.

For consistency purposes, TRP location information expressed in real numbers representing the exact location (e.g. axis values of x and y location) is not necessary. Instead, the delta of the new TRP location compared to the original location is an important indication of NW-side additional condition that may assist UEs.

Another useful assistance data for consistency is the beam information. However, the exact angle and azimuth values are not necessary when generic quantization of the beam information is disclosed from the LMF to UE. Instead, the set of quantized beam information from different UEs may facilitate the identification of specific areas that are valid for certain models deployed in the target UE.

For consistency, RAN1 to consider a new assistance data from LMF to UE by exposing semi-explicit information for TRP location and beam information. Each assistance data related to Info#7 and Info #16, #17 generates a new assistance data to be exposed explicitly from LMF to UE.

### Content of Associated-ID message and area granularity

One bottle neck raised in the latest meetings on Associated-ID was the lack of detail information on the content of the Associated-ID message for AI/ML positioning. In the generic approach, the content may consider aspects related to the set of network-side additional condition. However, for positioning, it is expected that at least Info #7 (TRP location) may be exposed implicitly by using an specific definition of the content of Associated-ID relative to Info #7.

The granularity of the Associated-ID is limited to the Cell-ID (see agreements/working assumption in agenda item 9.1.3.3 [2] clause 4.2.3,). However, for positioning, it could be beneficial if the granularity is extended to the validity area (more than a unique Cell-ID) which is already supported by the specification or if the granularity is extended to a set of few Cell-IDs or set of TRPs. All these approaches may be considered in the discussion of Associated-ID for Positioning. 

For the case that the granularity is based on the validity area, to avoid ambiguity in usage of the Associated-ID, the message should be composed at least by:

- Statistical information on changes in the TRP locations in the validity area

- Level of changes in the TRP locations in the validity area

- Indications that any TRP in the validity area that is muted.

For the case Associated-ID represents a validity area, the content of Associated-ID message for Info#7 is composed at least by: statistical information of changes in TRP location, level of changes of TRP location, and additional indication of TRPs muted during the generation of the Associated-ID. The message may use a bitmap of 8 bits length.

For the case that the granularity is a subset of the validity area, which means that the Associated-ID represents a set of Cell-IDs (non all Cell-IDs composing a validity area) or TRPs, the message should be composed at least by:

- Statistical information on changes in the TRP locations in the validity area

- Level of changes in the TRP locations in the validity area

For the case Associated-ID represents a set of TRPs or Cell-IDs, the content of Associated-ID message should be composed at least by: statistical information of changes in TRP location and level of changes of TRP location. The message content may use a bitmap of 8 bits length.

### Generation of ID representing the Associated-ID

As stated before, one solution to represent the generation of IDs representing the Associated-ID is up to network vendors. However, RAN1 may define the format of such IDs. It may be relevant for cases where the granularity of the Area is not limited to one certain validity area. So, a format X.Y.Z of a target area indicates: the validity area where the target area is part of (represented by X), the target area or a set of TRPs in the validity area (represented by Y), and the number of TRPs used to define the target area (represented by Z). Such format may enhance the granularity details of the Associated-ID disclosed to target UE for consistency purposes.

RAN1 to consider a format X.Y.Z to represent the ID of each Associated-ID to indicate the granularity of the target area that is represented by the Associated-ID where X may represent the validity area where the target area is located, Y represent the target area, and Z indicates the number of TRPs that are part of the target area.

In our previous proposals, it is assumed that every network vendor should generate the ID representing the Associated-ID. This solution may imply that the UE vendors should be able to differentiate by implementation the source of each Associated-ID received during inference data collection or inference.

Another approach is that RAN1 may standardize the ID of the Associated-ID to generalize the solution of consistency without the dependency of identifying the network vendor generating the Associated-ID. To enable this solution, the ID should be a standardized range of integer numbers and each integer number should be mapped to different characteristics of TRP location or TRP beam information. For instance, ID=1 indicates that there is no change on the set of TRPs, ID=2 indicates that there is a very low rate of changes of TRPs, ID=3 indicates that there is a high rate of changes of TRPs.

For consistency purposes, the Associated-ID representation is unique across network vendors, using a standardized range of integer values representing the TRP location or beam information changes.

### UE-Assisted generation of Associated-ID

All scenarios exposed before considering that the NW vendors generate by implementation the implicit information (Info #7 and Info #16) that is delivered to target UE for consistency purposes. In all of these solutions, the selected TRPs that are part of one specific validity area, or a unique Cell-ID, or few Cell-IDs may not be useful in practical execution of training data collection and inference. In addition, it may not be preferable to use all TRPs mapped to certain validity area or set of Cell-IDs. So, one feasible and efficient strategy to define the scope of Associated-ID may be based on the assistance provided by UE to LMF, it is enabled by providing a set of preferred TRPs/Cell-IDs that may be used to generate the Associated-ID message, in other words, the target UE assists LMF to define the Area that will be represented by the Associated-ID. Optionally, the LMF may suggest a pool of set of TRPs/Cell-IDs, and based on this set of TRPs/Cell-IDs, the UE may select by implementation the best set of TRPs/Cell-IDs. At the end, the LMF receives assistance from UE, and LMF has the final decision of what set of TRPs/Cell-IDs may be used.

For consistency purposes, the target UE assists LMF on selecting the preferred set of TRPs/Cell-IDs, and LMF generates the Associated-ID message based on the suggested set of TRPs/Cell-IDs.

## Associated-ID definition for AIML positioning

As per the RAN1#119 agreement, in Alternative 1 and 2, LMF may implicitly provide Info #7 to UEs through means such as an Associated-ID. This approach helps UEs determine if Info #7 remains consistent between training and inference without revealing the actual location coordinates of TRPs. However, the definition of the Associated-ID and the method by which LMF can implicitly indicate Info #7 to UEs is still missing.

One way to assist UEs for evaluating consistency with respect to Info #7 can be to provide a time information for which Info#7 remains changed (or alternatively, unchanged). To illustrate, LMF may indicate the last time (e.g., via time stamp or by using an ID that represents the time information) when Info #7 was changed. Alternatively, LMF may indicate the expected duration until which Info#7 will remain unchanged, such as based on the network deployment conditions.

With such information, UEs can check whether TRP locations have been changed between training and inference, by comparing the time information of their training dataset, and the time information provided by LMF in the assistance data.

For consistency checking in Case 1, Associated-ID or implicit Info#7 represents a time information for which TRP locations remains changed (or alternatively, unchanged), such as via a timestamp or an ID representing the last time when the location of TRP(s) changed.

Nevertheless, if the Associated-ID or implicit indication lacks any location details of TRP, the accuracy of positioning may decline, depending on the positioning algorithm or AI/ML models used by UEs. For instance, UEs might train AI/ML models that incorporate TRP location information alongside DL PRS measurements from these TRPs, potentially achieving better accuracy. Additionally, UEs might use TRP location information to select the TRPs that offer the best geometric dilution of precision (GDOP), thereby enhancing positioning accuracy. For example, if the network configures 16 TRPs for DL PRS transmission, the UE could choose the optimal four TRPs expected to provide the best GDOP by avoiding TRPs that are co-linear relative to the UE. In such cases, the absence of TRP location information could reduce the quality of inference, specifically the accuracy of UE location estimation, thus degrading the positioning QoS.

To assist the UEs for consistency and selecting best TRPs for inference, while not revealing the actual TRP locations in the network, one way is to provide an Associated-ID or implicit Info#7 that represents relative location information of the TRP(s), which could be either: a) with respect to a specific reference location unknown to the UEs; or b) relative locations of the TRPs with respect to each other.

For example, for (a), LMF can indicate [Associated-ID for TRP#1: delta-Latitude= +100, delta-Longitude=-200 (in milliarcseconds); Associated-ID for TRP#2 = delta-Latitude= +200, delta-Longitude = +150], where delta is with respect to a location not revealed to the UE.

To illustrate the option (b), LMF can provide distance D between the TRPs together with the angle A of the vector between the TRPs (with respect to a reference axis), e.g., in the form of a triangular matrix.

This way, UEs can benefit from relative location information of TRPs, e.g., for consistency checking, model training, TRP selection for inference, etc., whereas the actual geographical coordinates of the TRPs remain unrevealed.

Associated-ID or implicit Info#7 represents relative location information of the TRP(s), either: a) with respect to a specific reference location unknown to the UEs; or b) relative locations of the TRPs with respect to each other. Such information can benefit UEs for consistency checking, model training, TRP selection for inference, etc., while not revealing the actual geographical coordinates of the TRPs.

## Other aspects related to network-side additional condition

Assigning an Associated-ID for every possible combination of network conditions and broadcasting it each time the conditions change is impractical. This approach would lead to considerable signaling overhead and necessitate all UEs to continuously track the Associated-ID related to network conditions. Additionally, since UEs may independently gather training data and train models, the network lacks awareness of when UEs collect training data or the specific network conditions they prefer during the data collection or inference phases.

To overcome these challenges, network may offer notifications for changes to certain network-side conditions (including any Associated-IDs) that can be requested by UEs in a subscription manner. That is, each UE may request notifications from network that are indicative of any changes (or no changes) to certain network-side conditions that the UE is interested in, together with indicating any time period that UE wishes to receive such notifications.

UE may request notifications from the network that are indicative of any changes (or no changes) to certain network-side conditions that the UE is interested in.

In order to perform consistency checking between training and inference, Info#7 related to TRP geographical coordinates can be used implicitly through Associated-ID. So far, it is assumed that all the TRPs involved in the training/inference process are actually considered as for the processing of Info#7 for consistency checking.

However, we propose to limit the set of TRPs involved in consistency checking because the following reasons:

- Improve consistency checking procedure: through pointing only important differences in the conditions between training and inference by focusing only on the sub-set of most relevant / important TRPs (with regards to their contribution to the ML model accuracy)

- Reduce reporting overhead: through sharing information only for a preferred subset of TRPs instead of the whole list of TRPs involved in training / inference

Thus, the LMF would identify the most important TRPs and consider only this sub-set during consistency checking process, which will then bring it to #K TRPs (with K < N for example K= 5 and N is the total number of TRPs in a certain validity area involved in training data collection/inference.

For AI/ML based positioning Case 1, assistance information for consistency checking can include Info#7 for selected subset TRPs from the set used for training/inference.

# Data collection

Regarding data collection, this contribution emphasizes on topics that may generate specification impact. For example, RAN1 should solve some FFSs on the time stamp information, mapping between Part A and Part B, ground-truth data collection for Case 1.

## Time stamp information

The time stamp associated with the AI/ML training data (measurement and/or label) is to be captured and reported by the data generating entity to the LMF. This is particularly relevant for example in scenarios where the measurement data (Part A component of the training data) collection entity and label data (Part B component of the training data) generation entities are different.

To capture and report time stamp of the AI/ML training data via legacy means, currently the LPP supports reporting either SFN information or the UTC time information from the UE to the LMF. Further, NRPPa supports optional reporting of time in seconds relative to 00:00:00 on 1 January 1900 (Relative Time 1900) along with mandatory SFN information reporting from the gNB to the LMF.

The SFN information carries NR time stamp which can include not only the SFN number but also the slot number, and hence it can capture the time resolution in milliseconds (ms). However, since the SFN number ranges from 0 to 1023, any data collection and reporting of the collected data beyond 10.24 ms (SFN cycle length) results in SFN ambiguity. Hence, the SFN information alone is not sufficient to capture the time stamp information for the data collection

On the other hand, usage of UTC time can avoid the time information ambiguity. However, the ms level granularity cannot be achieved with the UTC time of format YYMMDDhhmmssZ. Hence, UTC time in milliseconds needs to be adopted if the UTC time information is to be used. Note that, the reporting of UTC time information, in particular at ms-level of precision, increases the signaling overhead. Hence, the always reporting of UTC time information is not desirable. A similar analysis between gNB/TRP and LMF indicates that the same applies to Relative Time 1900 reporting over NRPPa.

To this end, the SFN information associated with the training data can be always reported to capture the time information. And this is complemented with the UTC time information over LPP and Relative Time 1900 over NRPPa only when there is a change in SFN cycle to resolve the SFN ambiguity.

To resolve the SFN time info ambiguity with reduced signaling overhead, the SFN time info for a channel measurement and/or a label ground-truth can be complemented with the UTC time info only when there is a change in SFN cycle between two consecutive channel measurements and/or labels in the report.

For reporting of UTC time with SFN time info as time stamp of channel measurements and label ground-truth is considered in RAN1 as configurable.

## Mapping between part A and part B

For training data collection, different entities generating Part A and Part B are agreed in RAN1#117.

Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed

The latest agreement from RAN1#120 discuss about timestamp information is reported with the part A and part B, and the second agreement how the Part A and Part B are mapped together.

Agreement For AI/ML based positioning, - When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed. - When label is to be reported/sent, time stamp of label is the time instance for which the label is valid. Agreement - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above

The FFS indicates that the association between Part A and Part B may be further discussed. In our view, the association is directly related to data collection for Case 3a, where the channel measurement (Part A) is generated by the NG-RAN (gNB) and the ground-truth label is generated by at least the LMF (Part B). Note that LMF may in turn obtain Part B from the UEs.

When Part A and Part B are generated by different entities, the time stamp information may not be sufficient for the paring both parts for the following reasons.

- First, based on the current network conditions such as load, resource usage etc., it is possible that the NW may use different positioning methods (e.g., DL based) or different time instances for the collection of Part A and Part B.

- Secondly, Case 3a is UL-based where the gNB is the entity collecting the Part A and requesting the label information from LMF. But, when two UEs perform the UL transmission at the same time, it introduces ambiguity during the mapping of Part A and Part B.

To overcome the above challenges, in addition to the time stamp, the legacy UL SRS configuration may be utilized where SRS resource identifier act as the mapping information between Part A and Part B.

The mapping information may be done in the LMF or in the gNB but including a signalization that ensure UE privacy not indicating any explicitly UE ID.

For Case 3a, when Part A and Part B are generated by different entities, in addition to timestamp, legacy UL SRS information (e.g., SRS resource ID) is used for pairing between a Part A entry and a Part B entry. The mapping may be done in LMF or gNB but always ensuring UE privacy information.

Another approach to complement the association between Part A and Part B for Case 3a is to generate a temporal unique identifier at LMF side for data collection purposes without exposing any UE privacy information, so, the unique identifier may differ from any global or local UE ID. This identifier is used across UEs and gNBs during the data collection for mapping Part A and Part B. One important remark is that enabling the unique identifier ensures the mapping, however, increases the signalization overhead, so, it may be an optional approach to enhance data collection for Case 3a in cases where time stamp introduces ambiguity (e.g., several UEs generating samples and labels simultaneously).

RAN1 to introduce a new temporal unique identifier defined by LMF and shared across target UE and gNB for data collection purposes to enhance the mapping between Part A and Part B at least for Case 3a.

Addressing the FFS related to the duration of Part B, a validity duration can be estimated based on the UE mobility status.

In fact, for an immobile UE, the validity duration of either Part A or Part B should be longer compared to the case where the UE is mobile (e.g. pedestrian or in a car).

The necessary configuration to select the validity duration based on the UE mobility situation can be provided by the LMF and can account for the method selected by the UE to determine its mobility (e.g. RSRP variation, number of handovers).

For training data collection of AI/ML based positioning 3a/3b, pairing between a Part A entry and a Part B entry requires the time stamp information as well as pairing validity duration (depended on UE mobility and reported by UE following LMF configuration).

## Ground truth data collection for Case 1

UE may use a variety of methods to generate ground truth such as based on non-NR, NR RAT-dependent, or NR RAT-independent positioning methods. For RAT-dependent methods, LMF needs to be involved as per the legacy positioning framework. To generate a ground truth label (or its approximation) associated with its inference output (UE location estimation), UE may send a positioning request to LMF to get its location information.

Nevertheless, since Case 1 relies on DL measurements, if LMF utilizes the same DL-based methods to estimate the target UE’s position, which is intended to be used as ground truth generation, then the estimation would not be reliable in case of imperfections, e.g., heavy NLOS links, UE measurement inaccuracy, timing error or estimation errors. Thus, it is important if UE can indicate a suitable method (that is possibly non-DL based method) or LMF determines a suitable method (that is possibly non-DL based method) to generate ground truth for UE-side models matching one specific quality target.

Similarly, multiple methods can be used to estimate UE’s position, among one of which could be selected according to different criteria, e.g., with respect to confidence of estimation or relative difference between the estimations using different methods (e.g., RAT, non-RAT based etc.). For this, the LMF may indicate UE, positioning method(s) together with necessary criteria to select an estimation to be used as ground truth.

For data collection by the target UE in Case 1, UE may request ground-truth label (Part B) from LMF, the request includes desired positioning method(s) (e.g., non-NR and/or NR RAT-dependent and/or NR RAT-independent) with necessary criteria (e.g., required confidence value) .

# Inference operation

The inference operation discussed in this section follows three important aspects: AI/ML inference input, AI/ML inference output, and signaling and procedures for inference.

## AI/ML model inference input for Case 3b

In RAN1#120bis, the following conclusion was agreed between companies

Conclusion For channel measurement type (A) (i.e., path-based measurement), it is not necessary to introduce enhancements on the number of reported paths in Rel-19.

So, one consequence may be to reuse the reporting scheme used in legacy to deliver measurements from NG-RAN to LMF. One way to address this issue is by reducing the signaling overhead involved in the reporting of measurements used by LMF as input for Case 3b. These alternatives are:

- Window based approach.

- Differential quantization approach.

### Window based approach

For LMF-side model (Case 3b) one crucial aspect is how the gNB reports the measurement to the LMF while avoiding overhead. In this regard, due to clustered nature of the dominant taps, the overhead can further be reduced by choosing multiple windows. By doing so, the overhead is reduced significantly compared to the existing schemes. Furthermore, the power information observed over each window may be estimated if needed to determine which cluster has the dominant component. Details of the reporting are disclosed in Section 5.5.1 of [4].

For Case 3b inference input, depending on the channel observation, gNB can determine/select the required reporting configuration to enable the reporting windowing scheme without compromising the DL/UL measurements content.

### Differential quantization approach

For any data collection purposes, LMF may ask channel measurements to NG-RAN (inference Case 3b) by reporting radio measurements. In this case, one alternative is the differential quantization, which is detailed in Section 5.5.2 of [4].

For Case 3b inference input, to reduce the signaling overhead of reporting measurements from UE (Case 1) and NG-RAN (Case 3b) to LMF, NG-RAN may use a reporting scheme based on differential quantization indicating at least the number of RSRPP messages for carrying the measurement report.

## AI/ML model inference output for Case 3a

The output of the AI/ML model inference in Case 3a is a positioning-related intermediate feature. In this regard, RAN1 has agreed on the following two major intermediate features: LOS/NLOS indicator and timing information and the reporting of both may reuse legacy mechanisms.

Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

On direct consequence of the above agreement is that the intermediate feature can be reported to LMF via reusing existing procedures in NRPPa. However, in RAN1#119, several proposals were done during the meeting to distinguish between legacy methods and Rel-19 AIML. In our view, there is no benefit for LMF if such distinguishing is enabled. In addition, gNB has better information (e.g. channel characteristics) to do that type of assessment compared to LMF.

In line with the previous statement, from RAN1 perspective, gNB is the entity hosting the AIML model, so the performance monitoring assists the selection between legacy method and Rel-19 AIML, which is up to implementation, as indicated in Figure 1. So, the intermediate feature reported by gNB must be transparent to any indication, so LMF should expect always the best suitable intermediate feature without any further detail or related signalization.

For measurement report of AI/ML assisted positioning Case 3a, all reported intermediate feature (timing information or LoS/NLoS indication) from gNB to LMF does not require to distinguish whether the information is (i) a legacy intermediate feature or (ii) AI/ML Case 3a intermediate feature.

Figure 1 - Illustration of the impact of monitoring for Case 3a. As the monitoring is also up to gNB, there is no necessity to differentiate the reporting to LMF, the selection is up to gNB implementation.

In RAN1#119, the final summary of specification support for positioning accuracy enhancement discussion includes the following table highlighting potential reasons to differentiate AI/ML and Legacy reporting for Case 3a

There is only the need to differentiate the measurement generated by AI/ML vs legacy, so that the LMF can use the received reports accordingly. The differentiation can be achieved by adding a “made by AI” tag in the measurement report. The possible combinations of associated measurement and LOS/NLOS indicator Associated measurement (e.g., timing). Absent; Legacy measurement*; Timing measurement generated by AI/ML. LOS/NLOS indicator; Absent; N/A; Support (Existing specification); Support in Rel-19. Generated by legacy methods; Not supported (Existing specification); Support (Existing specification); Support in Rel-19. Generated by AI/ML; Not supported in Rel-19; Support in Rel-19; Support in Rel-19. * When the associated legacy measurement is reported, it is not limited to timing measurements. It can be timing measurement or angle measurement.

Note that NRPPa signalling procedures allow the reporting of measurements together with a number of optional IEs that include LoS/NLoS information, measurement quality, etc. Since the IEs are optional, different combinations of those IEs can be used to support a broad combination of scenarios, including the combinations listed in the table above.

(Conclusion) It is not RAN1 scope to discuss RAN3 mechanisms for reporting different combinations available in NRPPa.

## Signalling and procedures for inference

### Inference for Case 3a

The inference operation at gNB would be necessary when LMF determines to utilize an UL positioning method upon an LCS location request. Thus, LMF could request one or more gNBs by utilizing legacy NRPPa messages. The gNB selects to use non-AI/ML or AI/ML approach.

To perform the desired inference operation, the target UE needs to be configured with UL SRS transmission. While the transmission of the UE could be configured by its serving gNB, the characteristics of the configuration (e.g., bandwidth, periodicity, etc.) could be determined as it is done in legacy.

(Conclusion) For Case 3a, no indication is required for AI/ML output inference to LMF

### Inference for Case 3b

The positioning measurement, potential preprocessing, measurements reporting by NG-RAN (gNB/TRP) would be necessary when LMF determines to utilize an UL positioning method, respectively, upon an LCS location request. Thus, LMF could request the target UE and one or more gNB/TRPs that were determined to be involved in the positioning inference, by requesting positioning measurements by using the legacy framework.

For Case 3b (LMF side model), for inference LMF may request reporting of UL measurements from gNB via legacy mechanisms defined in NRPPa.

# LCM framework

In RAN1#118bis, the following proposal was discussed offline between companies:

Proposal 7.2.3-1 Rel-19 support functionality-based LCM for AI/ML positioning Case 1. - To prepare for inference operation, the target device reports the applicable functionalities by sending a ProvideCapabilities message to the LMF. LMF can send a RequestCapabilities message to inquire on the capability of the target device.

Further, the RAN2 agreements in this aspect includes the following:

Agreements (RAN2#126) - The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. - wait for RAN1 for associate ID discussion - At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements

Agreements (RAN2#127bis) 1: The following procedures for LCM for UE sided model for AI positioning Case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

Agreements (RAN2#128) 1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered Agreements (RAN2#129) 1: Introduce AI/ML positioning Case 1 as a new positioning method. 2: Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR. 3: FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed. FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation, 4: The content of error cause is discussed while drafting stage3 CRs. 5: As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list. 6: UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed.

The AI/ML positioning Case 1 method relies on the UE capability in supporting AI/ML operation for positioning. When UE has multiple AI/ML models, the positioning method can be realized by using different UE capabilities corresponding to the different underlying models that UE may have.

Note that it is expected that the UE may have (or have access to) multiple models which may be associated with different network-side (additional-)conditions. That is, depending on the network-side (additional-)conditions, the UE may select one model over the other. Hence, the network-side (additional-)condition information is provided to the UE (cf. Step 3).

Since the models at the UE are linked to the network-side (additional-)condition information, for a given network-side (additional-)condition, the UE may have only a subset of its model(s) available. In other words, only a subset of the models may be applicable for the given network-side (additional-)condition. Consequently, only a certain subset of the supported UE capabilities becomes applicable for the given network-side (additional-)condition. This needs to be indicated to the LMF so that LMF can set up a suitable configuration for the AI/ML operation (cf. Step 4). Here, for a given set of models for a UE, the applicable condition(s) is a subset of the supported UE capabilities that is associated with the applicable model(s) linked to the given network-side (additional-)condition information. Hence, for a given set of models, while the capability of the UE does not change dynamically and remains static, the applicability varies based on the network-side (additional-)condition information.

In Case 1, for a given set of models for the UE, the supported UE capabilities of the UE remain static, but their applicability varies depending on the network-side (additional) condition(s) (cf. applicability indication of step 4 in the RAN2 agreement for LCM for UE sided model).

Clearly, the configurations that is used for Case 1 to determine the position of the UE (i.e., configuration for performing inference) are based on the UE capabilities. In other words, the so-called inference configuration is determined considering the UE capability. Therefore, the UE capability directly impacts the inference configuration (i.e., selection of the configuration parameters) and hence needs to be reported to LMF (cf. Step 2) so that the configuration chosen by the LMF for AI/ML positioning is compatible with the UE capability.

Furthermore, when UE can offer multiple capabilities (based on the multiple underlying models), there may be multiple LPP configuration possibilities for Case 1. That is, the same AI/ML-based positioning method can be configured differently based on the reported capability.

In order to perform inference in Case 1, the necessary configuration for performing inference would indicate the requested location information with any QoS requirements (e.g., accuracy, response time, etc.), requested measurements (if any), reporting configuration (e.g., reporting periodicity), in addition to the assistance data such as containing the DL PRS configuration and any network-side (additional) conditions.

For Case 1, the so-called “inference configuration” indicated to UE corresponds to the requested location information with any QoS requirements (e.g., accuracy, response time, etc.), requested measurements (if any), reporting configuration (e.g., reporting periodicity), in addition to the assistance data such as containing the DL PRS configuration.

The reporting of applicability information to the LMF may impact the inference configuration setting. However, to avoid additional specification impact, no reconfiguration is expected after the inference configuration was set by the LMF and delivered to the UE.

For Case 1, no inference reconfiguration is expected based on dynamic applicability report from the UE after UE receives the inference configuration.

# Higher layer parameters for AI/ML positioning

A set of higher layer parameters are identified based on the current Rel. 19 WI agreements, details of the preliminary list of parameters are shared in Appendix B.

Send an LS to RAN2/RAN3 to indicate the updated set of higher layer parameters for AI/ML positioning after RAN1#121 (check Appendix B).

# Conclusion

In this contribution, we discuss details of AI/ML for positioning enhancement use case and have the following observations and proposals:

## Observations

Observation 1: In Case 1, for a given set of models for the UE, the supported UE capabilities of the UE remain static, but their applicability varies depending on the network-side (additional) condition(s) (cf. applicability indication of step 4 in the RAN2 agreement for LCM for UE sided model).

## Proposals

Proposal 1: For model performance monitoring of AI/ML positioning Case 1, “FFS: Option B” in RAN1#119 agreement is resolved by: For model performance monitoring of AI/ML positioning Case 1, supports Option B-2

Proposal 2: For model performance monitoring of AI/ML positioning Case 1, supports label-free based monitoring for Case 1.

Proposal 3: For model performance monitoring of AI/ML positioning Case 1, the target UE reports monitoring outcome at least when the LMF request performance monitoring.

Proposal 4: For Case 1, LMF may configure UE with certain conditions to trigger performance monitoring. These conditions may include availability of TRPs, channel conditions, network load, and network-side additional conditions (e.g. Associated-ID information).

Proposal 5: (Conclusion) Monitoring for Case 3a is up to NG-RAN implementation and no specification impact is expected from RAN1 perspective.

Proposal 6: RAN 1 to consider the following approaches to ensure consistency between training and inference regarding NW-side additional conditions: (i) consistency based on explicit assistance data; (ii) based on implicit assistance data enabled by Associated-ID; and (iii) consistency assisted by monitoring.

Proposal 7: In Case 1, for consistency between training and inference, when explicit information is provided by assistance data, a delta indicating the variation of the assistance data is used to assess the consistency.

Proposal 8: For Case 1, RAN1 to consider LMF assistance related to validity area as specified in legacy and include additional granularity for consistency purposes.

Proposal 9: For Case 1, to ensure consistency between training and inference related to NW-additional conditions, assistance data (e.g. PRS configuration, GCI) used for data collection (training) or inference shall be associated with the Associated-ID(s).

Proposal 10: RAN1 down scope Alt-#3 and Alt#4 and only consider Alt#1 and Alt#2 (inclusion of Associated-ID) for both DL-TDOA and DL-AoD as part of assistance data for consistency checking in Case 1.

Proposal 11: For consistency, RAN1 to consider a new assistance data from LMF to UE by exposing semi-explicit information for TRP location and beam information. Each assistance data related to Info#7 and Info #16, #17 generates a new assistance data to be exposed explicitly from LMF to UE.

Proposal 12: For the case Associated-ID represents a validity area, the content of Associated-ID message for Info#7 is composed at least by: statistical information of changes in TRP location, level of changes of TRP location, and additional indication of TRPs muted during the generation of the Associated-ID. The message may use a bitmap of 8 bits length.

Proposal 13: For the case Associated-ID represents a set of TRPs or Cell-IDs, the content of Associated-ID message should be composed at least by: statistical information of changes in TRP location and level of changes of TRP location. The message content may use a bitmap of 8 bits length.

Proposal 14: RAN1 to consider a format X.Y.Z to represent the ID of each Associated-ID to indicate the granularity of the target area that is represented by the Associated-ID where X may represent the validity area where the target area is located, Y represent the target area, and Z indicates the number of TRPs that are part of the target area.

Proposal 15: For consistency purposes, the Associated-ID representation is unique across network vendors, using a standardized range of integer values representing the TRP location or beam information changes.

Proposal 16: For consistency purposes, the target UE assists LMF on selecting the preferred set of TRPs/Cell-IDs, and LMF generates the Associated-ID message based on the suggested set of TRPs/Cell-IDs.

Proposal 17: For consistency checking in Case 1, Associated-ID or implicit Info#7 represents a time information for which TRP locations remains changed (or alternatively, unchanged), such as via a timestamp or an ID representing the last time when the location of TRP(s) changed.

Proposal 18: Associated-ID or implicit Info#7 represents relative location information of the TRP(s), either: a) with respect to a specific reference location unknown to the UEs; or b) relative locations of the TRPs with respect to each other. Such information can benefit UEs for consistency checking, model training, TRP selection for inference, etc., while not revealing the actual geographical coordinates of the TRPs.

Proposal 19: UE may request notifications from the network that are indicative of any changes (or no changes) to certain network-side conditions that the UE is interested in.

Proposal 20: For AI/ML based positioning Case 1, assistance information for consistency checking can include Info#7 for selected subset TRPs from the set used for training/inference.

Proposal 21: For reporting of UTC time with SFN time info as time stamp of channel measurements and label ground-truth is considered in RAN1 as configurable.

Proposal 22: For Case 3a, when Part A and Part B are generated by different entities, in addition to timestamp, legacy UL SRS information (e.g., SRS resource ID) is used for pairing between a Part A entry and a Part B entry. The mapping may be done in LMF or gNB but always ensuring UE privacy information.

Proposal 23: RAN1 to introduce a new temporal unique identifier defined by LMF and shared across target UE and gNB for data collection purposes to enhance the mapping between Part A and Part B at least for Case 3a.

Proposal 24: For training data collection of AI/ML based positioning 3a/3b, pairing between a Part A entry and a Part B entry requires the time stamp information as well as pairing validity duration (depended on UE mobility and reported by UE following LMF configuration).

Proposal 25: For data collection by the target UE in Case 1, UE may request ground-truth label (Part B) from LMF, the request includes desired positioning method(s) (e.g., non-NR and/or NR RAT-dependent and/or NR RAT-independent) with necessary criteria (e.g., required confidence value) .

Proposal 26: For Case 3b inference input, depending on the channel observation, gNB can determine/select the required reporting configuration to enable the reporting windowing scheme without compromising the DL/UL measurements content.

Proposal 27: For Case 3b inference input, to reduce the signaling overhead of reporting measurements from UE (Case 1) and NG-RAN (Case 3b) to LMF, NG-RAN may use a reporting scheme based on differential quantization indicating at least the number of RSRPP messages for carrying the measurement report.

Proposal 28: For measurement report of AI/ML assisted positioning Case 3a, all reported intermediate feature (timing information or LoS/NLoS indication) from gNB to LMF does not require to distinguish whether the information is (i) a legacy intermediate feature or (ii) AI/ML Case 3a intermediate feature.

Proposal 29: (Conclusion) It is not RAN1 scope to discuss RAN3 mechanisms for reporting different combinations available in NRPPa.

Proposal 30: (Conclusion) For Case 3a, no indication is required for AI/ML output inference to LMF

Proposal 31: For Case 3b (LMF side model), for inference LMF may request reporting of UL measurements from gNB via legacy mechanisms defined in NRPPa.

Proposal 32: For Case 1, the so-called “inference configuration” indicated to UE corresponds to the  requested location information with any QoS requirements (e.g., accuracy, response time, etc.), requested measurements (if any), reporting configuration (e.g., reporting periodicity), in addition to the assistance data such as containing the DL PRS configuration.

Proposal 33: For Case 1, no inference reconfiguration is expected based on dynamic applicability report from the UE after UE receives the inference configuration.

Proposal 34: Send an LS to RAN2/RAN3 to indicate the updated set of higher layer parameters for AI/ML positioning after RAN1#121 (check Appendix B).

# References

| [1] | Qualcomm (Moderator), “RP-234039 - New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP TSG RAN Meeting #102, Edinburgh, 2023. |
| --- | --- |
| [2] | Qualcomm (Moderator), “3GPP TR 38.843 - Study on Artificial Intelligence (AI)/Machine Learning (ML) Rel. 18,” 2023. |
| [3] | Qualcomm (Moderator), “RP-250792 - Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP TSG RAN Meeting #107, Incheon, 2025. |
| [4] | Nokia, “R1, 2402997 - AI/ML for Positioning Accuracy Enhancement , 3GPP TSG RAN WG1 #116-bis,” Changsha, China, 2024. |
| [5] | 3GPP, “TS 37.355 - LTE Positioning Protocol (LPP),” 2024. |
| [6] | 3GPP, “TS 38.215 - NR Physical layer measurements,” 2024. |
| [7] | 3GPP, “TS 38.214 - NR Physical layer procedures for data,” 3GPP, 2024. |

# Appendix A

The agreements reached on th [3]e agenda item 9.1.2 related to AI/ML for Positioning Accuracy Enhancement are indicated in each 3GPP RAN1 meeting.

Agreements reached in 3GPP RAN1#116 meeting

Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE. Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement - For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF: - timing information; - paired timing information and power information. Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model. Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

Agreements reached in 3GPP RAN1#116-bis meeting

Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Conclusion - It is out of RAN1 scope to decide whether/how synthetic data (i.e., not direct physical data) and related entities are used in AI/ML based positioning. In RAN1 discussion, data (e.g., measurement data, label data) refer to physical data, not synthetic data. Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 2a, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A.NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Agreements reached in 3GPP RAN1#117 meeting

Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed Agreement Draft LS R1-2405577 is endorsed in principle by adding the latest agreements made in this meeting and adding “agreements” to “Note: the working assumptions above are based on RAN1 understanding for RAN work item (NR_AIML_air).” Agreement Final LS R1-2405578 is endorsed.

Agreements reached in 3GPP RAN1#118 meeting

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications Agreement For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB. Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by UE, the Nt’ selected samples are expected to be selected based on a rule. - FFS: the rule - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Agreement For Rel-19 AI/ML based positioning Case 2b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameters values of Nt, Nt', to the UE via LPP, subject to UE capability. - FFS: k

Agreements reached in 3GPP RAN1#118bis meeting

Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information Conclusion For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB. Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. Agreement For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data. Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable. Information; UE-based DL-TdoA; UE-based DL-AoD. 1; Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement. 2; Timing relative to the serving (reference) TRP of candidate NR TRPs. 3; DL-PRS configuration of candidate NR TRPs. 4; Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation. 5; SSB information of the TRPs (the time/frequency occupancy of SSBs). 6; Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs). 8; Fine Timing relative to the serving (reference) TRP of candidate NR TRPs. 9; PRS-only TP indication. 10; The association information of DL-PRS resources with TRP Tx TEG ID. 11; LOS/NLOS indicators. 12; On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation. 13; Validity Area of the Assistance Data. 14; PRU measurements together with the location information of the PRU. 15; Data facilitating the integrity results determination of the calculated location. 16; TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP). 17; Expected Angle Assistance information. 18; PRS priority list. [1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0 [2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

Agreements reached in 3GPP RAN1#119 meeting

Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements Agreement For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs).

Agreements reached in 3GPP RAN1#120 meeting

Conclusion For AI/ML based positioning Case 3a, regarding the time stamp in a measurement report from gNB to LMF, - Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0...5} Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting: - Nt' = {8, 16, 24} Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label. Agreement For AI/ML based positioning, - When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed. - When label is to be reported/sent, time stamp of label is the time instance for which the label is valid. Agreement Send Reply LS R1-2501522 to SA2 about Case 2b with the content of Section 2 of R1-2501521 by revising “RAN1 would continue work and may provide additional input in the future to SA2 for Case 2b if any new agreement reached in RAN1.” To “RAN1 may provide additional input in the future to SA2 for Case 2b if any new agreement reached in RAN1.” Agreement Send Reply LS R1-2501524 to SA2 about Case 3b with the content of Section 3 of R1-2501521 by adding all the agreement made in this meeting for Case 3b. Agreement Final LS R1-2501523 is endorsed. Final LS R1-2501525 is endorsed. Agreement For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only. Conclusion For Rel-19 AI/ML based positioning Case 3b, - If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type. - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested. - It is up to RAN3 to decide the signaling details. - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement. Agreement RAN1 send an LS to RAN3 and RAN2 to inform them above agreements and conclusions. Agreement - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above. LS 1627/1628

Agreements reached in 3GPP RAN1#120bis meeting

Conclusion - The offset (used in the agreement made in RAN1#119) refers to the duration (in samples) between the reference time and the starting time of the list of Nt consecutive values. Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: whether transmit offset from gNB to LMF” in RAN1#119 agreement is resolved by: - No offset is transmitted from gNB to LMF Conclusion For channel measurement type (A) (i.e., path-based measurement), it is not necessary to introduce enhancements on the number of reported paths in Rel-19. Agreement For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method. Agreement For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

# Appendix B

Based on the current RAN1 agreements a preliminary list of high-level parameters for AIML positioning are indicated in Table 3.

Table 3 - Higher layer parameters for AIML positioning impacting other WGs.

| Sub-feature | RAN2/RAN3 parent IE | Parameter name in the spec. | New of existing? | Description | Value range | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| AI/ML Positioning Case 3b | [RAN3] TRP Measurement Request Item; Measured Result Item | [kX, e.g., X=6] Timing Reporting Granularity Factor | Existing | Value (0..5) corresponds to (k0..k5) in TS 38.133 | {0...5} | Relevant agreement: Agreement in RAN1#119 For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, • The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. • The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. o The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. • The timing information is defined relative to a reference time, same as the path-based measurement. • The Nt’ selected time domain channel measurement values are expected to be those with the highest power. • The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. • LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: o Nt'<=24. FFS: Nt' values. o Nt = {32, 64, 128} o FFS: k o The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement • FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified Agreement in RAN1#118 For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0...5} |
| AI/ML Positioning Case 3b | [RAN3] TRP Measurement Request Item; | [nt-r19] | New | This parameter is indicating the number of consecutive channel response values. | {32, 64, 128} | Relevant agreement: Agreement in RAN1#119 For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, • The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. • The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. o The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. • The timing information is defined relative to a reference time, same as the path-based measurement. • The Nt’ selected time domain channel measurement values are expected to be those with the highest power. • The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. • LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: o Nt'<=24. FFS: Nt' values. o Nt = {32, 64, 128} o FFS: k o The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement • FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified. Agreement in RAN1#118 For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. |
| AI/ML Positioning Case 3b | [RAN3] TRP Measurement Request Item; | [ntPrime-r19] | New | This parameter is used to indicate number of the estimated channel response values in time domain that are selected from a list of Nt consecutive channel response values | {8, 16 24} | Relevant agreement: Agreement in RAN1#119 For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, • The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. • The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. o The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. • The timing information is defined relative to a reference time, same as the path-based measurement. • The Nt’ selected time domain channel measurement values are expected to be those with the highest power. • The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. • LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: o Nt'<=24. FFS: Nt' values. o Nt = {32, 64, 128} o FFS: k o The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement • FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified Agreement in RAN1#118 For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting: - Nt' = {8, 16, 24} |
| UE-based positioning Case 1 | FFS for RAN2 | MonitoringOutcome | New | An indication that the target UE cannot perform the Case 1 positioning method | FFS for RAN2 | Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: • Option A. The target UE side performs monitoring metric calculation. o The target UE may signal the monitoring outcome to the LMF. o FFS: content of monitoring outcome • FFS: Option B Agreement For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: • the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method. |
| UE-based positioning Case 1 | FFS for RAN2 | MonitoringRequest | New | An indication that the LMF request Monitoring to Target UE for Case 1 positioning method | FFS for RAN2 | Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: • Option A. The target UE side performs monitoring metric calculation. o The target UE may signal the monitoring outcome to the LMF. o FFS: content of monitoring outcome • FFS: Option B Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. |
| UE-based positioning Case 1 | FFS for RAN2 | Assistance Data collection of ground truth-label (It is up to RAN2 to decide which existing IE to use) | New | LMF sends target UE its location info | FFS for RAN3 | LMF sends the ground truth label (i.e., location info) to UE. It is up to RAN2 to decide whether the ground truth label is sent to UE via LPP or other protocol. Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label. |