3GPP TSG RAN WG1 #121		      	R1-2504153

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.12.2

Source: 	ETRI

Title: 	Discussion on low band carrier aggregation via switching

Document for:	Discussion/Decision

Introduction

In RAN1#120bis, the following agreements were reached regarding the semi-static configuration, including the switching pattern, switching gap, and other related topics [1].

- For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases, - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell. - SCS 15KHz on both carriers. - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. - For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. - The pattern starts from the beginning of SFN 0 of PCell. - Restriction of maximum X switch(es) within Y slot(s). - FFS: detailed value and relation of X and Y. - FFS: other restriction. - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, down-select one of the following solutions: - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier. - Alt 3: Switching gap is always assumed at the SDL carrier. - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms. - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed). - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL. - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA. - FFS: whether it has specification impact.

Additionally, based on the Feature Lead Summary from RAN1#120bis, the following topics are expected to be discussed at RAN#121 [2].

- Semi-static pattern configuration including switching gap - Down selection of the location of switching gap for switching from FDD carrier to SDL carrier among 3 alternatives (always FDD, always SDL, configurable) - Restriction of maximum X switch(es) within Y slot(s), FFS: detailed value and relation of X and Y - FFS other restrictions (e.g., maximum number of consecutive slots configured as SDL, etc.) - Assumptions/restrictions on important RS, e.g., SSB, TRS, etc. - How to avoid collision with switching gap, e.g., NW implementation (UE is not expected) or UE to drop, etc. - From moderator, to justify the configuration of switching gap, NW should avoid collision - UE processing timeline - Any processing timeline relaxation is needed, if so, which process and how to relax (considering UL Tx switching as reference point, but keep in mind that we have both DL and UL switch and this agenda is only for semi-static pattern) - Only candidate scenario that PRACH may require UE to have additional switch - Whether to handle, if yes, how to handle - Remaining collision handling with FDD or SDL duration - Whether to handle, if yes, which scenarios and how. Please make proposal more concrete so that it can be specified using the existing specification reference as much as possible without creating new terminology - HARQ-ACK codebook handling (Type I, SPS, etc.) - For SCell activation/deactivation - Assume RAN4 will handle it based on the current progress of RAN4 - For SCell dormancy - Low priority and most likely no time. SCell dormancy does not completely shut down the UE operation on SCell. - Other issues

In this contribution, we discuss the details of semi-static pattern configuration and the associated collision management for the switching gap.

Discussion

- 2.1 Semi-static pattern configuration including switching gap

The location of the switching gap for FDD to SDL transition needs to be clearly defined to ensure robust operation in low band NR carrier aggregation via switching. The switching pattern is configured semi-statically via RRC and operates on a slot basis, with switching gaps intended to isolate carrier transitions such that the UE operates only on one carrier at any given time.

Previously, three options were previously considered: switching gap always configured at the switch-from carrier (i.e., FDD), switching gap always configured at the SDL carrier, and switching gap configured either at FDD or SDL based on network configuration. Placing the switching gap at the switch-from carrier (FDD) has been identified as a consistent and efficient solution. This option aligns well with the slot-based design framework and supports a clear timing reference using the downlink timing of the PCell. It allows the transition from FDD to SDL to occur smoothly without interfering with ongoing uplink transmissions or reference signal measurements. It simplifies both network-side scheduling logic and UE-side processing, promoting robust and predictable behavior across vendors. Additionally, this approach allows preemptive exclusion of the final FDD slot from scheduling certain signals, such as CSI-RS or PUCCH, minimizing the risk of collision. Placing the switching gap at the SDL carrier may offer some benefits in terms of maintaining continuous use of the final slot on the FDD carrier, particularly in scenarios where the FDD resource is heavily loaded. However, this configuration may introduce challenges for uplink timing alignment and could complicate scheduling due to the shift in timing reference to the SDL side. Allowing the network to configure the switching gap on either carrier may provide flexibility under varied deployment or traffic conditions. However, it can increase signaling complexity and introduce implementation inconsistencies, particularly in multi-vendor scenarios. This may reduce predictability in switching behavior and place additional processing requirements on the UE.

Although placing the switching gap at the FDD carrier may lead to some scheduling constraints in the final FDD slot, these can be mitigated by treating the slot as a protected or restricted region in scheduling policies. For example, the gNB may avoid scheduling PDSCH or UL grants in that slot and exclude RS transmissions to reduce conflict. Careful RRC configuration of the switching pattern may further protect uplink timing and ensure signal integrity.

Proposal 1: The switching gap for FDD to SDL transition can be configured at the switch-from carrier (i.e., FDD).

To complement the decision on switching gap placement, the frequency of switching between FDD and SDL carriers is another important aspect of semi-static configuration. Frequent switching within a configured bitmap duration can impact RS reception, HARQ operations, and uplink timing alignment. Therefore, switching patterns should maintain moderate frequency to support stable system performance. A switching pattern is semi-statically defined over a bitmap, such as 40 slots, where each bit maps to a carrier. Each transition between FDD and SDL is a switching event, and each carrier duration is expected to last for a reasonable number of consecutive slots.

Switching too frequently—such as every 1 or 2 slots—can disrupt reference signal reception (e.g., SSB, TRS, CSI-RS), which often relies on multi-slot coherence to ensure reliable measurement and decoding. This may lead to reduced channel estimation accuracy and degraded beam management. In the HARQ process, frequent switching can misalign data transmission and corresponding ACK/NACK reception windows, potentially stalling the retransmission cycle or causing decoding failures. In addition, uplink timing advance (TA) may become unstable, as consistent uplink timing references require uninterrupted slot continuity. These issues may also increase UE processing complexity and power consumption, while limiting gNB scheduling flexibility, particularly for latency-sensitive or bursty traffic. For example, limiting switching occasions to 8 or fewer within a 40-slot bitmap yields an average carrier phase length of 5 or more slots. This configuration aligns with typical RS periodicity, HARQ timing windows, and uplink TA stability, while allowing implementation flexibility.

Proposal 2: Switching between FDD and SDL carriers may be configured to ensure that each carrier duration continues for a reasonable number of consecutive slots.

- 2.2 Collision management for switching gap

Semi-static switching configurations between FDD and SDL carriers require careful attention to switching gap collisions. The switching gap is a designated period during which the UE does not operate on either carrier to enable safe RF switching and timing alignment. To ensure seamless operation without transmission or reception conflicts, it is essential to manage the risk of collisions between scheduled signals and the switching gap. For dynamic or scheduled transmissions such as DCI-triggered PDSCH, PUSCH, HARQ-ACK, SR, CSI feedback, and SRS triggered by MAC-CE or DCI, the network can fully control the scheduling timing. In these cases, the network should prevent any scheduling of these signals on a carrier that is inactive due to a switching gap. This ensures that the UE is not required to take any additional action, such as dropping or rescheduling, thereby simplifying UE behavior and maintaining deterministic system operation. However, for periodic or semi-static resources, including periodic CSI-RS, semi-persistent CSI-RS, SPS-PDSCH, CG-PUSCH, periodic SRS, and SR configured on PUCCH, there may be instances where the scheduled signal coincides with a switching gap. Because these resources are not dynamically scheduled each time, the network may not be able to avoid such overlap in every case. In these specific and limited cases, the UE may drop the reception or transmission of the signal that overlaps with the switching gap.

The scope of such UE drop behavior must be strictly confined to a clearly defined set of signals to avoid ambiguity and to ensure consistent implementation across different vendors. For example, the UE is expected to drop a signal only if it falls into typical categories such as: periodic or semi-persistent CSI-RS configured via CSI-ReportConfig, periodic or semi-persistent SRS, SPS-PDSCH, CG-PUSCH of type 1 or type 2, SR configured on PUCCH, or a transmission utilizing repetition or multiple-slot allocation such as TBoMS.

Allowing the UE to drop signals outside this defined list would introduce uncertainty and potential performance inconsistency. Therefore, any other signals or channels, especially those triggered dynamically, must be managed exclusively by the network to avoid collision. This approach supports a clear separation of responsibilities between the network and the UE, promotes predictable switching behavior, and preserves the reliability of physical and MAC layer operations.

Proposal 3: Collisions between the switching gap and physical signals or channels can be avoided by the network through scheduling. The UE may drop a signal only if it overlaps with the switching gap and belongs to a predefined list of periodic or semi-static resources.

Conclusions

In this contribution, we provided our view on the semi-static pattern configuration and collision handling for the switching gap, and proposed the following.

Proposal 1: The switching gap for FDD to SDL transition can be configured at the switch-from carrier (i.e., FDD).

Proposal 2: Switching between FDD and SDL carriers may be configured to ensure that each carrier duration continues for a reasonable number of consecutive slots.

Proposal 3: Collisions between the switching gap and physical signals or channels can be avoided by the network through scheduling. The UE may drop a signal only if it overlaps with the switching gap and belongs to a predefined list of periodic or semi-static resources.

References

- Chairman’s notes, RAN1#120bis, April 7 - 11, 2025.

- R1-2503110, “FL summary #4 of Low band carrier aggregation via switching”, April 7 - 11, 2025.