3GPP TSG RAN WG1 #122                                                                R1-2505167

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	10.3.2

Source:	Spreadtrum, UNISOC

Title:	Discussion on study of air interface for Device 2b/C

Document for:	Discussion and decision

Introduction

The WID of enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices was approved in RAN#108 as follows [1]:

A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19. Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage. …

In this contribution, we will discuss the air interface design for active device (Device 2b and Device C).

Discussion

Architectures of Device 2b and C

In Rel-19 SI [2], we studied the architecture of Device 2b with RF-ED, IF-ED, and ZIF-ED receiver, as shown in Figure 1 and 2. According to the Rel-20 A-IoT SID, Device 2b and Device C have the similar device architecture and have IF-ED/ZIF-ED receiver. In our view, Device 2b and Device C can share the same architecture. Compared with Device 2b, the most significant difference is that Device C has a higher power consumption. Accordingly, some active components can adopt the higher power consumption ones, and the performance can be better, i.e., PA, LNA, etc.

In short, the architectures (with IF-ED/ZIF-ED receiver) achieved in Rel-19 SI for Device 2b could be the baseline for the architecture of Rel-20 redefined Device 2b and Device C.

Proposal 1: The architectures (IF-ED or ZIF-ED receiver) achieved in Rel-19 SI for Device 2b could be the baseline for the architecture of Rel-20 redefined Device 2b and Device C.

Observation 1: Compared with Device 2b, the most significant difference is that Device C will have a higher power consumption, and some active components can be adopted with the higher power consumption for better performance.

Figure 1: Architecture of device 2b with IF-ED receiver in Rel-19

Figure 2: Architecture of device 2b with ZIF receiver in Rel-19

Physical channel design enhancement

For Rel-20 A-IoT, the physical channel design should be rethought for supporting Device 2b and Device C and meet the requirements for outdoor scenarios.

R2D transmission:

In Rel-20, the basic physical layer design for R2D in Rel-19, including waveform, modulation, line coding, and CRC, can be reused for active device. Considering active device has higher performance of SFO calibration as studied in Rel-19 SI for device 2b, it could support larger M value, e.g., 32, for supporting larger R2D data rate. However, if larger M value(s) is/are supported, the CP handling method for M = 24 will not work, and should be further considered.

Proposal 2: The design for the R2D physical channel in Rel-19 for Device 1, including waveform, modulation, line coding, and CRC, can be reused for active device in Rel-20.

In Rel-20, the coverage of active devices in outdoor scenarios will be larger than that of Device 1 studied in Rel-19. We have to evaluate whether R2D is the bottleneck for coverage if we fully reuse the design of the Rel-19 physical layer design of R2D. According to the output of evaluation of 10.3.1, RAN1 decides whether to support R2D repetition.

Proposal 3: According to the output of evaluation of 10.3.1, RAN1 decides whether to support R2D repetition.

FDMed R2D transmission was proposed in Rel-19 SI by some companies to save the inventory time. However, the inventory time is more limited to the D2R FDM rather than R2D FDM. Besides, FDMed R2D will increase the implementation complexity of the device, affect the coverage (the power splitting between the parallel R2D channels), and require additional interference suppression to deal with the intermodulation between the parallel R2D transmissions. Hence, FDMed R2D transmission should not be supported.

Observation 2: FDMed R2D transmission will increase the implementation complexity of the device, affect the coverage (the power splitting between the parallel R2D channels), and require additional interference suppression to deal with the intermodulation between the parallel R2D transmissions

Proposal 4: FDMed R2D transmission should not be supported in Rel-20 only if the sufficient justification has been justified.

D2R transmission:

For D2R transmission, the single-tone RF waveform is generated internally by active device. Other physical layer design in Rel-19 A-IoT, e.g., modulation, CRC, FEC, multiplex, and repetition, can be reused without any enhancement, unless the Rel-19 design can not meet the target requirements.

Specifically, regarding modulation, due to the different modulation architecture compared with backscatter modulation, the active device can filter out the high-order odd harmonics with an LPF. It means active device can support more D2R FDMA frequency points compared with Device 1. Considering 1SB OOK/BPSK modulation will bring additional device implementation complexity, we should only support 2SB modulation for active devices.

Proposal 5: The single-tone RF waveform of active device is generated internally.

Observation 3: Compared with device 1, active device has LPF in Tx which can filter out the high-order odd harmonics, and can support more D2R FDMA frequency points within the same D2R system bandwidth.

Proposal 6: The designs for the D2R physical channel in Rel-19 for Device 1, e.g., modulation(2SB), CRC, FEC, and repetition, can be reused in Rel-20 for active device, unless the Rel-19 designs can not meet the target requirements.

In Table 1, we summarize the physical channel design in Rel-19 and our consideration for Rel-20.

Table 1: Physical channel design

|  |  | Rel-19 Device 1 | Rel-20 active device |
| --- | --- | --- | --- |
| R2D | Waveform | OFDM-based | OFDM-based |
|  | Modulation | OOK-4 M = {2, 6, 12, 24} | OOK-4 M = {2, 6, 12, 24, [32]} |
|  | Line coding | Manchester | Manchester |
|  | CRC | CRC-6 and CRC-16 | CRC-6 and CRC-16 |
|  | Repetition | Not support | Support, if needed |
|  | Multiplex | TDM | TDM |
| D2R | Waveform | Outside carrier wave | independent signal generation |
|  | Modulation | OOK, BPSK (2SB) | OOK, BPSK (2SB) |
|  | CRC | CRC-6, CRC-16 | CRC-6, CRC-16 |
|  | Multiplex | TDM, FDM | TDM, FDM |
|  | Repetition | Yes | Yes |
|  | FEC | TBCC | TBCC |
|  | SFS | support | Support multiple D2R frequency point |

R2D and D2R frame structure:

For R2D, one transmission includes R-TAS, PRDCH, postamble, and possible padding. For D2R, one transmission includes preamble, PDRCH, and possible midamble. In Rel-20 for the design for active device, the frame structure of R2D and D2R achieved in Rel-19 for Device 1 could be the baseline. Additional enhancement could also be considered, i.e., necessary enhancement for supporting DOA traffic and CFO calibration which will be discussed in Sec 2.4 and 2.5.

Proposal 7: Rel-19 R2D and D2R frame structure for Device 1 could be the baseline for active device in Rel-20. Additional necessary enhancement for supporting DOA and CFO calibration should also be considered.

Random access

In Rel-20, the active device will support inventory, command, sensor, and positioning. For inventory and command, the design in Rel-19 for Device 1 can be the baseline for the design for active device.

Proposal 8: Rel-19 design for indoor inventory/command of device 1 can be the baseline for outdoor inventory/command design for active device.

Multiplexing / multiple access

TDM for Msg1 and Msg 3:

In Rel-19, RAN1 support up to 2 time domain resources for Msg1 D2R transmission, and 1 time domain resources for Msg3 transmission. For active device, the limitation for supporting multiple TDM D2R transmission is the available time. The available time of active device is very short, as shown in Table 2. Obviously, the number of TDM resources for Msg1 and Msg3 should be limited, and only one time domain resource for Msg1 and Msg 3 transmission should be the baseline.

Observation 4: The sustainable time of active device is the challenge for supporting multiple TDM resources for Msg1 and Msg3.

Proposal 9: Only one time domain resource for Msg1/Msg3 transmission should be the baseline.

Table 2: Sustainable time of ambient IoT device

| Parameters |  |  |  | Device 1 |  | Active device |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Capacitance (uF) | Vlow (V) | Vhigh (V) | Consumed Energy (uJ) | Power Consumption (mW) | Discharging Time (s) | Power Consumption (mW) | Discharging Time (s) |
| 1 | 0 | 2 | 2 | 0.001 | 2 | 0.600 | 0.0033 |
| 10 | 0 | 2 | 20 | 0.001 | 20 | 0.600 | 0.0333 |
| 1 | 1 | 2 | 1.5 | 0.001 | 1.5 | 0.600 | 0.0025 |
| 10 | 1 | 2 | 15 | 0.001 | 15 | 0.600 | 0.025 |
| 10 | 0 | 2 | 20 |  |  | 1 | 0.02 |
| 10 | 1 | 2 | 15 |  |  | 1 | 0.015 |
| 10 | 0 | 2 | 20 |  |  | 10 | 0.002 |
| 10 | 1 | 2 | 15 |  |  | 10 | 0.0015 |

FDM for Msg1 and Msg3:

As analysis in Sec 2.2, due to the modulation architecture, active device can support more D2R transmission frequency domain resources. Hence, we can support more frequency domain resources for Msg1 and Msg3 transmission.

Proposal 10: More frequency domain resources for Msg1 and Msg3 transmission could be supported by active device.

Timing

In Rel-19, we defined Toffset1, Toffset2, Toffset3, and Toffset4, for the timing in CBRA and CFRA. Figure X illustrates the details.

Figure 3: Timing relationship

In Rel-20, the above parameters could be reused for active device for supporting inventory/command. Besides, because active device has better performance for SFO calibration, Toffset2 can be set smaller than that in Rel-19. In Rel-19 for SFO = 105ppm, Toffset2 = 0.25Toffset1 + 1.25Tmsg1. For the active device, 103ppm could be assumed and Toffset2 = 0.002Toffset1 + 1.002Tmsg1. Hence, the value of Toffset2 for active device could be 0.248 (Toffset1 + Tmsg1) less than that for device 1.

Observation 5: Since active device has better performance for SFO calibration, Toffset2 for active device can be 0.248 (Toffset1 + Tmsg1) less than that for Device 1.

Proposal 11: The parameters of timing defined in Rel-19, e.g., Toffset1, Toffset2, Toffset3, and Toffset4, should be reused for supporting inventory/command for active device.

DOA traffic supporting

In Rel-19 SI, RAN1 studied that, for DO-DTT and DT traffic types, the D2R resource(s) for D2R transmission is/are indicated in a R2D transmission, but this is not applicable at least for the first D2R transmission for DO-A traffic. Besides, we can also find that, if there is no R2D sync signal used for the device synchronization, device can not perform SFO and CFO calibration, and this affect the performance of D2R transmission.

Obviously, it is up to RAN2 decision how to determine/indicate the D2R resource(s) for the first D2R transmission for DO-A traffic.

Proposal 12: It is up to RAN2 decision how to determine/indicate the D2R resource(s) for the first D2R transmission for DO-A traffic.

For maintain the synchronization of active device, the periodic sync signal should be introduced. The frame structure of periodic sync signal could be the same as the R2D frame structure designed in Rel-19, including R-TAS (SIP + CAP), PRDCH, and postamble, as shown in Figure 4. The design of R-TAS and postamble can be the same as that in Rel-19. Besides, PRDCH may not exist when the sync signal is only used for synchronization. If RAN2 decides that the PRDCH in sync signal is used for carry the indication information for D2R resource(s) for the first D2R transmission for DO-A traffic, this PRDCH may be needed.

Figure 4: frame structure of R2D sync signal

Proposal 13: New periodic sync signal should be introduced in Rel-20 for supporting the synchronization of active device for DO-A traffic.

It is worth noting that, the active device will support DO-DTT, DT, and DOA traffic in Rel-20. In order to avoid duplication works, the design for inventory/command and DOA for outdoor scenarios should be fully applicable to indoor scenarios.

Proposal 14: The design for inventory / command and DOA for outdoor scenarios should be fully applicable to indoor scenarios for active device.

CFO calibration signal

For active devices, the RF carrier wave is generated internally. Limited to the power consumption budget, the accuracy of the clock used for the carrier wave is not very high, i.e., CFO = 103 - 104 ppm. Assuming that the active device operates in Band n8 (DL: 880-915MHz, UL: 925-960MHz), the carrier frequency offset can be up to ~ 9 MHz. A 9MHz single side guard band will be introduced which can not be acceptable. Hence, a signal/part for CFO calibration should be provided to synchronize / calibrate the device clock for LO for carrier frequency to achieve the accuracy after clock sync /calibration at device side.

For earning a better calibration performance, CFO calibration signal/part is better a RF single-tone sine wave. Active devices will use analog circuits, e.g., PLL or FLL, to achieve frequency tracking and locking.

Regarding the calibration signal/part, it could be a signal with fully “1” modulated in baseband and be a part of existing frame structure, e.g., attached to the end of current R2D frame structure, or be a separate R2D signal. In Figure 5, we propose several possible frame structure with CFO calibration signal/part.

Figure 5: R2D transmission with CFO calibration signal/part

Proposal 15: Additional calibration signal/part used for CFO calibration is considered, which can be

- Option 1: a part of any R2D transmission for CFO calibration, or

- Option 2: a separate R2D transmission with calibration signal for CFO calibration.

For Device 2b and Device C, the common calibration signal/part should be design, i.e., no device type specific calibration signal/part introduced.

Proposal 16: The addition calibration signal/part should be applied to Device 2b / Device C.

Besides, for different topologies, the R2D is transmitted in different band. For topology 1, e.g., D4T1 and D1T1, the R2D transmission with CFO calibration signal/part is transmitted in DL band. It means the CFO calibration signal/part and target D2R carrier wave are in different band. The device will receive this signal/part and leverage the frequency divider and frequency multiplier for calibrating the D2R carrier wave. For topology 2, e.g., D2T2, the R2D transmission with CFO calibration signal/part is transmitted in the DL band. It means that the CFO calibration signal/part and the target D2R carrier wave are in same band. The device will receive this signal/part and calibrate the D2R carrier wave directly.

Proposal 17: For T1 topology, e.g., D4T1 and D1T1, the calibration signal/part is transmitted in DL band. The device will receive this signal/part and leverage the frequency divider and frequency multiplier for calibrating the D2R carrier wave.

Proposal 18: For T2 topology, i.e., D2T2, the calibration signal/part is transmitted in UL band. The device will receive this signal/part and calibrate the D2R carrier wave directly.

Summary

In the contribution, we provides our view on the study of air interface for Device 2b/C, and propose that,

Proposal 1: The architectures (IF-ED or ZIF-ED receiver) achieved in Rel-19 SI for Device 2b could be the baseline for the architecture of Rel-20 redefined Device 2b and Device C.

Observation 1: Compared with Device 2b, the most significant difference is that Device C will have a higher power consumption, and some active components can be adopted with the higher power consumption for better performance.

Proposal 2: The design for the R2D physical channel in Rel-19 for Device 1, including waveform, modulation, line coding, and CRC, can be reused for active device in Rel-20.

Proposal 3: According to the output of evaluation of 10.3.1, RAN1 decides whether to support R2D repetition.

Observation 2: FDMed R2D transmission will increase the implementation complexity of the device, affect the coverage (the power splitting between the parallel R2D channels), and require additional interference suppression to deal with the intermodulation between the parallel R2D transmissions

Proposal 4: FDMed R2D transmission should not be supported in Rel-20 only if the sufficient justification has been justified.

Proposal 5: The single-tone RF waveform of active device is generated internally.

Observation 3: Compared with device 1, active device has LPF in Tx which can filter out the high-order odd harmonics, and can support more D2R FDMA frequency points within the same D2R system bandwidth.

Proposal 6: The designs for the D2R physical channel in Rel-19 for Device 1, e.g., modulation(2SB), CRC, FEC, and repetition, can be reused in Rel-20 for active device, unless the Rel-19 designs can not meet the target requirements.

Proposal 7: Rel-19 R2D and D2R frame structure for Device 1 could be the baseline for active device in Rel-20. Additional necessary enhancement for supporting DOA and CFO calibration should also be considered.

Proposal 8: Rel-19 design for indoor inventory/command of device 1 can be the baseline for outdoor inventory/command design for active device.

Observation 4: the sustainable time of active device is the challenge for supporting multiple TDM resources for Msg1 and Msg3.

Proposal 9: Only one time domain resource for Msg1/Msg3 transmission should be the baseline.

Proposal 10: More frequency domain resources for Msg1 and Msg3 transmission could be supported by active device.

Observation 5: since active device has better performance for SFO calibration, Toffset2 for active device can be 0.248 (Toffset1 + Tmsg1) less than that for Device 1.

Proposal 11: the parameters of timing defined in Rel-19, e.g., Toffset1, Toffset2, Toffset3, and Toffset4, should be reused for supporting inventory/command for active device.

Proposal 12: it is up to RAN2 decision how to determine/indicate the D2R resource(s) for the first D2R transmission for DO-A traffic.

Proposal 13: New periodic sync signal should be introduced in Rel-20 for supporting the synchronization of active device for DO-A traffic.

Proposal 14: the design for inventory / command and DOA for outdoor scenarios should be fully applicable to indoor scenarios for active device.

Proposal 15: additional calibration signal/part used for CFO calibration is considered, which can be

- Option 1: a part of any R2D transmission for CFO calibration, or

- Option 2: a separate R2D transmission with calibration signal for CFO calibration.

Proposal 16: the addition calibration signal/part should be applied to Device 2b / Device C.

Proposal 17: For T1 topology, e.g., D4T1 and D1T1, the calibration signal/part is transmitted in DL band. The device will receive this signal/part and leverage the frequency divider and frequency multiplier for calibrating the D2R carrier wave.

Proposal 18: For T2 topology, i.e., D2T2, the calibration signal/part is transmitted in UL band. The device will receive this signal/part and calibrate the D2R carrier wave directly.

Reference

- RP-251840, Rel-20 Ambient IoT outdoor SID, June 9-13, 2025

- TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR