3GPP TSG RAN WG1 #122	                                  	      R1-2505455

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	10.2.1

Source:	Xiaomi

Title:	Discussion on the improvement of SRS capacity and coverage

Document for:	Discussion and Decision

Introduction

The Rel-20 WID on NR MIMO Phase 6 is approved [1], which includes the following objective:

- On enhancing UL capacity and coverage, specify the following enhancements:

  - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification:

    - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding

      - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible.

    - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set

      - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

SRS capacity and coverage has always been the bottleneck for the system especially for the cell edge uses’ experience. Especially for TDD systems, it is highly dependent on SRS for the estimation of UL CSI and DL CSI to achieve a good performance. In this contribution, we present our views on the enhancements for further SRS capacity and coverage.

Discussion

SRS enhancements for the partial-frequency sounding

In Rel-17, RPFS has been introduced to increase SRS capacity and coverage with higher power density. However, when a UE supports SRS frequency hopping with RPFS, only a single frequency-domain starting position is possible within each SRS frequency hop as shown in Figure 1-a, which is quite challenging for channel interpolation. Due to this limitation, multiple frequency-domain starting positions in respective time-domain symbols within each SRS frequency hop can be enhanced to facilitate the channel estimation accuracy and diversity gain with channel interpolation as shown in Figure 1-b.

- R17 RPFS                                                                   b) Enhanced RPFS

Figure 1. single and multiple frequency-domain starting positions for RPFS

Multiple frequency-domain starting positions within each SRS frequency hop can be considered as a method of a symbol-level frequency hopping for RPFS within each SRS hop. To enable this kind of frequency hopping scheme, how to derive the multiple frequency-domain starting positions within each SRS hop and how to apply to each symbol need to be solved.

In the current spec [2], the frequency-domain starting position for RPFS is derived from kF  by

noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF

Where the starting frequency index for RPFS  kF∈0,1,…,PF-1  is configured by the higher-layer parameter StartRBIndex and khop is given corresponding to the value of  PF   and enabled by the higher-layer parameter enableStartRBHopping.

Multiple frequency-domain starting positions for the RPFS symbol-level frequency hopping can be derived considering the current configuration of kF  appied to all the SRS hops during one SRS frequency period. A unified formular considering both single and multiple frequency-domain starting positions is preferred.

The frequecy offset for the sub-hop within each SRS hop can be defined as ks, and noffsetRPFS  is defined by

noffsetRPFS=NscRBmSRS,BSRSkF+khop+ks(m)mod PFPF

ks(m)∈0,1,…,PF-1 , m∈0,1,…,M-1 where M is the number of sub-hops within an SRS hop, and noffsetRPFS can be applied to each symbol within the sub-hop.

The frequency offset ks(m) can be enabled by a RRC parameter to support the symbol-level RPFS frequency hopping within each SRS hop, if configured as disabled, ks(m)=0. This feature can be configured based on UE capability reporting.

Proposal 1: A unified formular supporting both single and multiple frequency-domain starting positions is preferred, and noffsetRPFS can be applied to each symbol of the symbol-level sub-hop within an SRS hop.

Proposal 2: The RPFS frequecy offset for the sub-hop within each SRS hop can be defined as ks, and noffsetRPFS  is defined by

noffsetRPFS=NscRBmSRS,BSRSkF+khop+ks(m)mod PFPF

ks(m) ∈0,1,…,PF-1, m∈0,1,…,M-1 where M is the number of sub-hops within an SRS hop

Proposal 3: A RRC parameter can be introduced to enable the frequency offset ks to support the symbol-level RPFS frequency hopping within each SRS transmission, if it is configured as disabled, ks(m)=0，m∈0,1,…,M-1.

The frequency offset ks can be network configured or pre-defined at least according to PF and applied to different granularity of the time-domain resources for each sub-hop within an SRS hop.  Two options of SRS transmission methods can be considered.

- Option 1: Multiple frequency-domain starting positions can be applied directly to s symbol(s). According to current spec, s applies to 1 when 1/2/3/4/8 SRS ports are configured while s =2 only if the higher-layer parameter nrofSRS-Ports-n8 equals ports8tdm. The number of sub-hops M within an SRS hop can be derived from M=R/s, R is the RepetitionFactor configured. Two examples are illustrated in Figure 2 and Figure 3 with frequency hopping parameters. We can learn that in this method SRS can sound as many RB sets as possible in one frequency period which would benefit the CSI performance as the first priority.

Figure 2. enhanced RPFS pattern 1 (R=4, s=1, ks(m) ∈{0,2,1,3}, kF=0)

Figure 3. enhanced RPFS pattern 2 (R=5, s=1, ks(m) ∈{0,2,1,3}, kF=0)

- Option 2: Also, multiple frequency-domain starting positions can be applied for different SRS hopping bundles, where multiple number of X symbols are bundled corresponding to the same sub-hop within an SRS hop. SRS sub-hopping bundle size X or the number of sub-hops M can be defined. With this method, SRS transmission can both benefit from repetitions for larger coverage and from sounding different RB sets for better performance, and the tradeoff is up to gNB scheduling. The number of sub-hops M is the integer part of R/X. And for cases if R/X is not an integer, how to deal with the orphan symbols may need to be further considered or only integer values for R/X are allowed. Two examples are illustrated in Figure 4 and Figure 5 with frequency bundles.

Figure 4. enhanced RPFS pattern 3 (R=4, s=1,  ks(m) ∈{0,2,1,3}, kF=0, X=2)

Figure 5. enhanced RPFS pattern 4 (R=5, s=1,  ks(m) ∈{0,2,1,3}, kF=0, X=2)

Proposal 4: The frequency offset ks can be network configured or pre-defined at least according to PF .

Proposal 5: The granularity of the time-domain resources within an SRS hop can be s symbol(s) defined in current spec or a hopping bundle defined as multiple number of symbols bundled corresponding to the same sub-hop within an SRS hop.

Proposal 6: When hopping bundle is configred for the symbol-level frequency hopping within an SRS hop, whether and how to deal with the orphan symbol(s) if the number of sub-hops R/X is not an integer needs to be further considered.

SRS enhancements for cross-slot transmission

In real TDD deployments, SRS configuration is mainly provided in S slot where typically only two or four symbols can be used. Hence, the utilization of more contiguous symbols for SRS configuration via supporting SRS transmission across two adjacent slots (S and U slots) can improve SRS coverage and provide additional flexibility.

To enable a cross-slot SRS transmission within a SRS resource, how to define the time domain resource allocation (TDRA) is the issue need to be solved.

In current spec [3], SRS resource should be allocated within one slot, and the SRS resource set should be transmitted within one slot too. Firstly, to support cross-slot SRS transmission, it should be allowed to remove the restriction for the time-domain resource allocation for a SRS resource, but the allocation of number of slots should be no more than two slots. And same restriction can also be relaxed for a SRS resource set. Cross-slot SRS transmission within an SRS resource can be enabled by gNB configuration based on UE capability reporting. It can be configured in an SRS resource level or a SRS resource set level.

Proposal 7: To support cross-slot SRS transmission, an SRS resource or a SRS resource set is allowed to be transmitted within slot or across two slots.

The time-domain resource allocation for a cross-slot SRS resource can be illustrated in Figure 6 below, where NsymbSRS is the number of consecutive OFDM symbols given by the field nrofSymbols for the SRS resource, X is the number of consecutive OFDM symbols of the SRS resource contained in S slot or the first slot, Y is the number of consecutive OFDM symbols in U slot or the second slot, the l0 is the starting position in the time domain calculated based on a configured offset loffset .

Figure 6. TDRA for a cross-slot SRS resource (S slot + U slot)

The parameter l0 for cross-slot SRS resource needs to be defined according to [2]. There can be different methods to define the TDRA parameters for this new use case. To minimize the spec impact, we utilize the current configurations as much as possible, the network can still configure NsymbSRS and loffset .

- Different from the current spec, for NsymbSRS, the value range of the parameter should be extended to NsymbSRS∈{1, 2, 3, …,27}, while NsymbSRS∈{2,4,6，…,26} only if the higher-layer parameter nrofSRS-Ports-n8 equals ports8tdm. Thus, we can further define the value range of the RepetitionFactor (R) as  R∈{1, 2, 3, …,13}.

- For the offset loffset∈0,1,…,13 currently is defined as the offset of counting symbols backwards from the end of the slot.  For the cross-slot use case, the end of the slot may be considered as the last symbol in the S slot or the last symbol in the U slot corresponding to the configured slot offset. Either way is feasible for the indication.

To align with the legacy definition as much as possible, we propose the following,

- Redefine the loffset as the offset counting symbols backwards from the end of the S slot boundary for the indication of the SRS starting position inside the S slot when the value range of loffset is defined as loffset∈0,1,…,13. And the parameters from the above figure can be derived accordingly, i.e., X=loffset +1, Y= Ns -X= Ns - loffset -1. The parameter l0 for SRS resource transmitted starting within S slot can be maintained as legacy, which is l0=Nsymbslot-1-loffset .

- Redefine the loffset as the offset counting symbols forward from the end of the S slot boundary for the indication of the SRS starting position inside the U slot when the value range of loffset is defined as loffset∈-1,-2,…,-14.  The parameter l0 for SRS resource transmitted starting within U slot can be maintained as legacy, which is l0=-1-loffset .

Proposal 8: Reuse the legacy RRC configurations to support cross-slot SRS transmission, and to support the cross-slot SRS transmission we propose the following,

- Redefine the loffset as the offset counting symbols backwards from the end of the S slot boundary for the indication of the SRS starting position inside the S slot when the value range of loffset is defined as loffset∈0,1,…,13. The parameter l0 for SRS resource transmitted starting within S slot can be maintained as legacy, which is l0=Nsymbslot-1-loffset .

- Redefine the loffset as the offset counting symbols forward from the end of the S slot boundary for the indication of the SRS starting position inside the U slot when the value range of loffset is defined as loffset∈-1,-2,…,-14.  The parameter l0 for SRS resource transmitted starting within U slot can be maintained as legacy, which is l0=-1-loffset .

Proposal 9: The value range NsymbSRSof needs to be extended to NsymbSRS∈{1, 2, 3, …,27} to support the cross-slot SRS transmission while NsymbSRS∈{2,4,6，…,26} only if the higher-layer parameter nrofSRS-Ports-n8 equals ports8tdm.

Proposal 10: Redefine the value range of RepetitionFactor (R) as  R∈{1, 2, 3, …,13}.

Another issue is there may be impacts on the TDRA for PUSCH when cross-slot SRS transmission is configured. S is the starting symbol S relative to the start of the slot for PUSCH mapping type A defined in Table 6.1.2.1-1 [3], which is in current spec always set to 0. For the case of cross-slot SRS transmission, S represents the updated starting symbol index.

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A (repetition Type A only) | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} for repetition Type A, {1,…,27} for repetition Type B | {0,…, 11} | {1,…,12} | {1,…,12} for repetition Type A, {1,…,23} for repetition Type B |

To resolve the issue, two possibilities can be considered.

- Alt.1: From the above TDRA parameters, the starting symbol index S can be derived from S =Y = Ns - loffset -1.

- Alt.2: When cross-slot SRS is configured, PUSCH mapping Type A is not configured.

For the above alternatives, we prefer to support only PUSCH mapping type B to avoid more spec impacts.

Proposal 11: When cross-slot SRS transmission is configured, suggest to support only PUSCH mapping Type B for PUSCH transmission.

Conclusion

In this contribution, we provide several text proposals based on the draft specs as below,

SRS for RPFS:

Proposal 1: A unified formular supporting both single and multiple frequency-domain starting positions is preferred, and noffsetRPFS can be applied to each symbol of the symbol-level sub-hop within an SRS hop.

Proposal 2: The RPFS frequecy offset for the sub-hop within each SRS hop can be defined as ks, and noffsetRPFS  is defined by

noffsetRPFS=NscRBmSRS,BSRSkF+khop+ks(m)mod PFPF

ks(m) ∈0,1,…,PF-1, m∈0,1,…,M-1 where M is the number of sub-hops within an SRS hop

Proposal 3: A RRC parameter can be introduced to enable the frequency offset ks to support the symbol-level RPFS frequency hopping within each SRS transmission, if it is configured as disabled, ks(m)=0，m∈0,1,…,M-1.

Proposal 4: The frequency offset ks can be network configured or pre-defined at least according to PF .

Proposal 5: The granularity of the time-domain resources within an SRS hop can be s symbol(s) defined in current spec or a hopping bundle defined as multiple number of symbols bundled corresponding to the same sub-hop within an SRS hop.

Proposal 6: When hopping bundle is configred for the symbol-level frequency hopping within an SRS hop, whether and how to deal with the orphan symbol(s) if the number of sub-hops R/X is not an integer needs to be further considered.

SRS for cross-slot transmission:

Proposal 7: To support cross-slot SRS transmission, a SRS resource or a SRS resource set is allowed to be transmitted within slot or across two slots.

Proposal 8: Reuse the legacy RRC configurations to support cross-slot SRS transmission, and to support the cross-slot SRS transmission we propose the following,

- Redefine the loffset as the offset counting symbols backwards from the end of the S slot boundary for the indication of the SRS starting position inside the S slot when the value range of loffset is defined as loffset∈0,1,…,13. The parameter l0 for SRS resource transmitted starting within S slot can be maintained as legacy, which is l0=Nsymbslot-1-loffset .

- Redefine the loffset as the offset counting symbols forward from the end of the S slot boundary for the indication of the SRS starting position inside the U slot when the value range of loffset is defined as loffset∈-1,-2,…,-14.  The parameter l0 for SRS resource transmitted starting within U slot can be maintained as legacy, which is l0=-1-loffset .

Proposal 9: The value range NsymbSRSof needs to be extended to NsymbSRS∈{1, 2, 3, …,27} to support the cross-slot SRS transmission while NsymbSRS∈{2,4,6，…,26} only if the higher-layer parameter nrofSRS-Ports-n8 equals ports8tdm.

Proposal 10: Redefine the value range of RepetitionFactor (R) as  R∈{1, 2, 3, …,13}.

Proposal 11: When cross-slot SRS transmission is configured, suggest to support only PUSCH mapping Type B for PUSCH transmission.

References

- RP-251856, New WID: NR MIMO Phase 6, Samsung, RAN#108, June 2025

- TS38.211, v19.0.0

- TS38.214, v19.0.0