3GPP TSG RAN WG1 #122		R1-2506000

Bengaluru, India, Aug 25th – 29th, 2025

Source:	HONOR

Title:	Views on numerology and frame structure design for 6GR

Agenda Item:	11.3.2

Document for:	Discussion and Decision

- Introduction

6G new radio study item has been approved in the RAN plenary #108 meeting [1]. One of the objectives is to strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. In this contribution, numerologies and frame structure are discussed to support diverse services and diverse device types for 6G new radio.

- Numerologies

The following design criteria has been considered for numerologies design in 5G:

- Diverse services and key performance requirements including eMBB, mMTC and URLLC.

- Support for diverse types of devices such as handhold UE, redcap and IoT devices.

- Support for wide ranges of spectrums, including FR1, FR2.

6G aims to support the similar various services and diverse types of devices using the spectrums ranges up to 52.6GHz, therefore 5G numerology design should be used as a starting point.

Proposal 1: Use numerologies in 5G NR as a starting point.

  - SCS

Flexible SCS have been defined in 5G as the below table. Besides the SCSs showed in the below table, 480k and 960k also be defined for FR2-2. As the 6G study will address frequency ranges up to 52.6GHz, 480k and 960k will not be discussed in the contribution.

| SCS | Supported for SSB | Supported for SI and Initial Access | Supported for data |
| --- | --- | --- | --- |
| 15kHz | Yes | Yes | Yes |
| 30kHz | Yes | Yes | Yes |
| 60kHz | No | Yes | Yes |
| 120kHz | Yes | Yes | Yes |
| 240kHz | Yes | No | No |

Table 1: SCS supported in 5G below frequency 52.6GHz

In addition to the above design criteria, 5G SCS design also considers factors such as coverage, mobility, latency and phase noise. However, the SCS design of 5G is too complicated. The UE needs to first blindly detect the SCS of the SSB based on the frequency band, and then determine the SCS for initial access based on the MIB configuration.

Based on the SID, one of the objectives of 6G architecture should be to avoid excessive configurations and minimizing multiple options for the same functionality directly addresses real-world concerns about implementation complexity, energy consumption, and UE burden.

The supported SCS, at least for the SSB and initial access, could be considered to be simplified.

Proposal 2: Study whether/how to reduce supported SCS for SSB and initial access, i.e., only a single SCS per band/sub-FR/carrier type/etc. such as 15 kHz SCS for FR1 FDD, 30 kHz SCS for FR1 TDD, 120 kHz SCS for FR2-1 and FFS for frequency range between FR1 and FR2-1.

Furthermore, for the data channel, current 5G support the following SCS:

- SCS for bands below 1G: 15kHz, 30kHz;

- SCS for bands between 1G and 7G: 15kHz, 30kHz, 60kHz;

- SCS for band 24G~52.6GHz: 60kHz,120kHz;

6G study represents an opportunity to revisit the foundations of data channel SCS design. In the study, whether to keep or down select the SCSs should be determined based on specified assessment from new services.

Proposal 3: FFS down select for SCS in frequency bands for data channel.

  - CP design

To mitigate inter-symbol interference (ISI) and preserve subcarrier orthogonality, a Cyclic Prefix (CP) was introduced in OFDM systems. The design criterion mandates that the CP duration must exceed the maximum multipath delay within the cell coverage. Additionally, since the CP degrades the spectral efficiency, minimizing its duration is critical for communication. However, for sensing applications which will be supported in 6G, longer CP enable larger sensing ranges. This creates a trade-off between communication and sensing requirements.

Observation 1: Inherent conflict exists between communication and sensing requirements for CP duration.

In NR systems, CP duration decreases with increasing subcarrier spacing (SCS) to maintain approximately constant CP overhead. For typical sensing scenarios, such as UAV sensing defined in TR 22.837, the required sensing range is 500–1000m [2]. When the SCS is 30 kHz, the CP duration is 2.34μs and the sensing range is about 351m. For 60 kHz SCS with a normal CP (duration:1.17μs), this range reduces to 175m.

Observation 2: The CP duration of NR system may not meet sensing coverage requirements.

In 5G, extended CP is only defined for 60kHz SCS, enabling sensing range up to 525 meters. Extended CP for other SCS could be considered for sensing services. If sensing symbols use extended CP, other symbols within the same slot must follow this configuration, which could degrade spectral efficiency to some extent. However, this solution is straightforward and maintains compatibility with existing OFDM frame structures.

Proposal 4: Consider to support CP design within a unified framework to simultaneously address communication and sensing requirements.

- Frame structure

The following aspects should be considered for 6GR frame structure design.

- Migration from 5G to 6G;

- Support for various services including sensing and NTN services;

- Support larger carrier bandwidth;

- Configuration method;

Migration from 5G to 6G

Among the 5G to 6G migration architectures, there is majority support of the following one in which a 5G/6G dual mode gNB share the same re-farming band using MRSS. To support this MRSS architecture, it should be considered that the 6G frame structure configuration align with 5G in a certain time/frequency granularity.

Figure 1: Migration from 5G to 6G using MRSS

Proposal 5: To support MRSS migration from 5G to 6G, consider the 6G frame structure configuration align with 5G system in a certain time/frequency granularity.

Support for various services including sensing and NTN services

Except the traditional services, 6G needs to consider to support new services such as ISAC and NTN. ISAC and NTN services may need more time interval for sensing or UL/DL transition. 6G frame structure design should consider the requirement of this kind of new services.

Proposal 6: 6G frame structure design should consider new services such as sensing and NTN.

Support larger carrier bandwidth

When designing the 5G frame structure, the initial consideration did not include flexibility in the frequency domain, which subsequently led to complexity in introducing SBFD. In the future, 6G will support larger carrier bandwidths, such as carrier bandwidths above 200M. The flexibility of the frame structure in the frequency domain becomes more necessary.

Proposal 7: The design of the 6G frame structure needs to consider flexibility in the frequency domain.

Configuration method

In 5G, the frame structure can be configured via RRC and DCI as below:

- Cell specific RRC configuration;

- UE specific RRC configuration;

- DCI based dynamic configuration;

Cell specific and UE specific RRC configuration method can be reused for 6G. Regarding to DCI based dynamic configuration, it is a bit complicated and has not been well deployed. In 6G, whether to use this method should be determined based on specified assessment from new services.

Proposal 8: Cell and UE specific RRC configuration for frame structure can be reused for 6G and FFS whether to use DCI based dynamic configuration based on specified assessment from new services.

- Conclusions

In this contribution, we provide our views on numerologies and frame structure design considerations for 6G. The following observations and proposals are given:

Proposal 1: Use numerologies in 5G NR as a starting point.

Proposal 2: Study whether/how to reduce supported SCS for SSB and initial access, i.e., only a single SCS per band/sub-FR/carrier type/etc. such as 15 kHz SCS for FR1 FDD, 30 kHz SCS for FR1 TDD, 120 kHz SCS for FR2-1 and FFS for frequency range between FR1 and FR2-1.

Proposal 3: FFS down select for SCS in frequency bands for data channel.

Proposal 4: Consider to support CP design within a unified framework to simultaneously address communication and sensing requirements.

Proposal 5: To support MRSS migration from 5G to 6G, consider the 6G frame structure configuration align with 5G system in a certain time/frequency granularity.

Proposal 6: 6G frame structure design should consider new services such as sensing and NTN.

Proposal 7: The design of the 6G frame structure needs to consider flexibility in the frequency domain.

Proposal 8: Cell and UE specific RRC configuration for frame structure can be reused for 6G and FFS whether to use DCI based dynamic configuration based on specified assessment from new services.

Observation 1: Inherent conflict exists between communication and sensing requirements for CP duration.

Observation 2: The CP duration of NR system may not meet sensing coverage requirements.

# References

- RP-251881, New SID: Study on 6G Radio, RANP#108, June, 2025.

- Research on the requirements and application scenarios of 6G sensing, IMT-2030(6G) Promotion Group, Nov. 2023.