3GPP TSG RAN WG1 #122	R1-2506281

Bengaluru, India, Aug 25th – 29th, 2025

Source:	NTT DOCOMO, INC.

Title:	Maintenance of support of RedCap and eRedCap UEs in FR1-NTN

Agenda Item:	8.11.2

Document for:	Discussion and Decision

- Introduction

At the RAN1#121 meeting, it was concluded that R19 NR-NTN WI was completed. In this contribution, maintenance of (e)RedCap support in FR1-NTN will be discussed and several proposals will be submitted.

- Discussion

  - UE behavior when UL drop occurs

Even though dropping behavior of either DL or UL has been defined for Case 3 and Case 4, the existing several features to be used for (e)RedCap UEs will not work due to the dropping behavior. One important note is, handling these problems by gNB implementation, e.g., collision avoidance by gNB or blind decoding at gNB, is not reasonable. Collision avoidance by gNB implementation is not aligned with why overlap handling for Case 3 and for Case 4 is to be introduced.

    - DMRS bundling

If DL reception is prioritized over UL transmission, a possible situation is that some of PUSCH repetitions are dropped. In TN, gNB knows the dropping slots and thus it is expected that gNB and UE have the same understanding of actual TDWs for the PUSCH repetitions when DMRS bundling is applied. However, this is not the case in NTN as abovementioned. gNB does not know whether or when a nominal TDW is segmented into multiple actual TDWs as illustrated below. It will be impossible for gNB to decode the PUSCH by using the divided actual TDWs.

Fig. 1: Why DMRS bundling does not work in Case 4.

For the solution, the last window and the next window can also be dropped together with the dropping slots. The wasted UL transmissions should be cancelled as the divided actual TDWs will not be used effectively by gNB.

Proposal 1:

- Adopt the following TP for TS 38.214.

| Reason for change: | When UL drop due to DL/UL overlap occurs is unclear at gNB side, and in this case, actual TDW of DMRS bundling will not be aligned between gNB and UE. |
| --- | --- |
| Summary of change: | All repetitions within a nominal TDW is dropped when UL drop due to DL/UL overlap occurs within the nominal TDW occurs. |
| Consequences if not approved: | Channel estimation at gNB side for DMRS bundling could not work accurately and thereby PUSCH decoding is failed. |

### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS <Unchanged parts omitted> - For reduced capability half-duplex UEs, - a dropping or cancellation of a PUSCH or PUCCH transmission according to clause 17.2 of [6, TS 38.213] or - an overlapping of the gap between two consecutive PUSCH or two consecutive PUCCH transmissions and any symbol of downlink reception or downlink monitoring For a NTN cell, if a nominal TDW is divided into more than one actual TDWs due to the event for reduced capability half-duplex UEs, transmissions within the actual TDWs are dropped. The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1, 0_2 or 0_3, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to frequency hopping, or in response to the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or in response to the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to any event not triggered by DCI or MAC-CE. The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1, 0_2 or 0_3, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to an event triggered by MAC-CE, subject to UE capability. of dmrs-BundlingRestart [13, TS 38.306] and when pusch-WindowRestart or pucch-WindowRestart is enabled. <Unchanged parts omitted>

    - UL slot counting for repetitions

In the same situation assumed at the last section, with TN, gNB knows the dropping slots and thus it is expected that gNB and UE have the same understanding regarding which slots are not used and how many subsequent slots are including repetitions instead of the dropping slots. However, this is not the case in NTN. gNB does not know these aspects.

For example, when UE-A is scheduled with 8 PUSCH repetitions from slot n, the transmissions may be completed at slot n+7 or may be completed at slot n+9 due to drop of some repetitions overlapped with DL. gNB does not know which case occurs, and thus gNB must skip scheduling any other UE for subsequent slots that are potentially used for the repetitions, such as slots n+8 and n+9. This wasted situation should be avoided. It is noted that this problem becomes larger when DL repetitions are typically applied as discussed in R19 DL coverage enhancement.

Fig. 2: Why PUSCH repetitions does not work efficiently

Proposal 2:

- Adopt the following TP for TS 38.214.

| Reason for change: | When UL drop due to DL/UL overlap occurs is unclear at gNB side, and in this case, which slot is used for PUSCH repetition type A will not be aligned between gNB and UE. |
| --- | --- |
| Summary of change: | PUSCH slot counting includes the slot where UL drop due to DL/UL overlap occurs, i.e., slot without UL TX is also counted as PUSCH slot. |
| Consequences if not approved: | Unused and wasted resources are required to avoid potential overlap with other TX/RX, or resource overlap could occur. |

#### 6.1.2.1 Resource allocation in time domain <Unchanged parts omitted> For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or 0_3 for paired spectrum only, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, or 0_3 for paired spectrum only, based on the TDRA information field value in the DCI format 0_1, 0_2 or 0_3. - For the case of a reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3 when AvailableSlotCounting is enabled and K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1, 0_2 or 0_3, based on the TDRA information field value in the DCI format 0_1, 0_2 or 0_3. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot does not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. A slot is counted if transmission at the slot is dropped due to the case of a reduced capability half-duplex UE. <Unchanged parts omitted>

- Conclusion

In this contribution, we discussed support of RedCap/eRedCap UEs in FR1-NTN. Observations/Proposals are summarized as following:

Proposal 1:

- Adopt the following TP for TS 38.214.

| Reason for change: | When UL drop due to DL/UL overlap occurs is unclear at gNB side, and in this case, actual TDW of DMRS bundling will not be aligned between gNB and UE. |
| --- | --- |
| Summary of change: | All repetitions within a nominal TDW is dropped when UL drop due to DL/UL overlap occurs within the nominal TDW occurs. |
| Consequences if not approved: | Channel estimation at gNB side for DMRS bundling could not work accurately and thereby PUSCH decoding is failed. |

### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS <Unchanged parts omitted> - For reduced capability half-duplex UEs, - a dropping or cancellation of a PUSCH or PUCCH transmission according to clause 17.2 of [6, TS 38.213] or - an overlapping of the gap between two consecutive PUSCH or two consecutive PUCCH transmissions and any symbol of downlink reception or downlink monitoring For a NTN cell, if a nominal TDW is divided into more than one actual TDWs due to the event for reduced capability half-duplex UEs, transmissions within the actual TDWs are dropped. The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1, 0_2 or 0_3, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to frequency hopping, or in response to the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or in response to the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to any event not triggered by DCI or MAC-CE. The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1, 0_2 or 0_3, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to an event triggered by MAC-CE, subject to UE capability. of dmrs-BundlingRestart [13, TS 38.306] and when pusch-WindowRestart or pucch-WindowRestart is enabled. <Unchanged parts omitted>

Proposal 2:

- Adopt the following TP for TS 38.214.

| Reason for change: | When UL drop due to DL/UL overlap occurs is unclear at gNB side, and in this case, which slot is used for PUSCH repetition type A will not be aligned between gNB and UE. |
| --- | --- |
| Summary of change: | PUSCH slot counting includes the slot where UL drop due to DL/UL overlap occurs, i.e., slot without UL TX is also counted as PUSCH slot. |
| Consequences if not approved: | Unused and wasted resources are required to avoid potential overlap with other TX/RX, or resource overlap could occur. |

#### 6.1.2.1 Resource allocation in time domain <Unchanged parts omitted> For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or 0_3 for paired spectrum only, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, or 0_3 for paired spectrum only, based on the TDRA information field value in the DCI format 0_1, 0_2 or 0_3. - For the case of a reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3 when AvailableSlotCounting is enabled and K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1, 0_2 or 0_3, based on the TDRA information field value in the DCI format 0_1, 0_2 or 0_3. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot does not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. A slot is counted if transmission at the slot is dropped due to the case of a reduced capability half-duplex UE. <Unchanged parts omitted>