3GPP TSG RAN-WG1 Meeting #122			R1-2506459

Bengaluru, India, 25-29 Aug, 2025

Source: 	Moderator (Huawei)

Title:	FL summary #2 for Ambient IoT: “10.3.1 Evaluations”

Document for:	Discussion and decision

Agenda item:	10.3.1

Introduction

The outcome for the study of Ambient IoT in Rel-19 for indoor scenarios can be referred in TR38.769 [1]. The outcome for the RAN1 normative work of Ambient IoT in Rel-19 can be referred in TS38.291 [2]. For the study of Ambient IoT in Rel-20 for outdoor scenarios, a set of objectives have been approved and updated in the SID [3]. According to the chair’s agenda and contributions [4-26] submitted to this meeting from companies, this feature lead summary is organized to discuss the evaluation assumptions and results for Ambient IoT study in Rel-20. Please companies indicate your contact list for this agenda into the table in the end of Section 1.

The draft FL proposals will be found in each section with the following way of naming:

Proposal 3.1a-v1: XXX … - ‘3.1’ is the section number in this document - ‘a’ is marked as the serial of proposals under this section, e.g. different proposals in this section will be numbered as {a, b, c, …} - ‘v1’ is the version of a proposal, and will be numbered as {v1, v2, v3, …} to update

Please follow the guidelines for offline discussions and inputs to FL summary

- Proposals/questions marked as [open] are candidates to be discussed in this meeting.

- Once a proposal/question marked as [closed] then no need further discussion in this meeting.

- Proposals/questions marked as [H] are assumed to be in high priority for the upcoming discussion round in this meeting.

- Proposals/questions marked as [M] are assumed to be in medium priority for the upcoming discussion round in this meeting.

- Please kindly provide your views by 20:00 Monday local time for the 1st round discussion. Early inputs before 13:00 Monday local time (before 1st online) are highly appreciated.

Please companies indicate your contact lists for this agenda as follows:

| Company | Contact person | Email |
| --- | --- | --- |
| Huawei, HiSilicon | Zhe Jin (Feature lead) | jinzhe.jinzhe@huawei.com |
| Huawei, HiSilicon | Xiang Mi | shawn.mixiang@huawei.com |
| Huawei, HiSilicon | Sarun Selvanesan | sarun.selvanesan@huawei.com |
| NTT DOCOMO | Mayuko Okano | mayuko.okano.ca@nttdocomo.com |
| NTT DOCOMO | Sun Weiqi | sunwq@docomolabs-beijing.com.cn |
| NTT DOCOMO | Hiroki Harada | hiroki.harada.sv@nttdocomo.com |
| Tejas Networks | Sushmita Ghosh | sushmitag@tejasnetworks.com |
| SONY | Martin Beale | martin.beale@sony.com |
| Samsung | Miao Zhou | miao.zhou@samsung.com |
| Samsung | Hyemin Choe | hams.choe@samsung.com |
| LG Electronics | Jay kim | jayhyung.kim@lge.com |
| LG Electronics | Hosin Lee | hosin.lee@lge.com |
| vivo | Kai WU | wukai5g@vivo.com |
| vivo | Dongru Li | dongru.li@vivo.com |
| vivo | Zhipeng Lin | zhipeng.lin@vivo.com |
| Xiaomi | Teng Ma | mateng5@xiaomi.com |
| Xiaomi | Huan Sun | sunhuan7@xiaomi.com |

Related proposals

| Source | Observations/Proposals |
| --- | --- |
| FUTUREWEI | Observation 1: The active A-IoT devices (Device 2b and Device C) have channel selecting capability, which can enable the devices to support FDM on the R2D link and FDMA on the D2R link. Observation 2: With CFO being reduced to 10s to 100s ppm range after carrier frequency offset calibration, the device’s SFO should be reduced to the same ppm range. Observation 3. In addition to RF harvesting, other forms of energy harvesting are supported in active A-IoT devices. Observation 4: It is expected that both Budget-Alt1 and Budget-Alt2 are used in evaluations. Observation 5: The delay spread values are 129 ns for UMi and 363ns for UMa. Observation 6: The nominal delay spread for FR1 is 100 ns. Proposal 1: In calculating SINR the noise and interference bandwidth should be the same as the signal bandwidth when using Budget-Alt2 to calculate receiver sensitivity. Proposal 2: For link budget evaluation, adopt UMi and UMa for pathloss evaluation. Proposal 3: For link level simulations, select a delay spread value of 100 ns. Proposal 4: For link level simulations, adopt TDL-B as the channel model. |
| Spreadtrum, UNISOC | Proposal 1: The pathloss model of UMa-NLOS in TS 38.901 is adopted for coverage evaluation. Proposal 2: For evaluation purpose, - the Tx power of A-IoT BS reader is assumed as 46dBm. - the Tx power of Device 2b and Device C is be assumed as -10dBm and 0 dBm, respectively. Proposal 3: The link budget parameters and values agreed in R19 SI should be considered as the starting point for link budget calculation in R20 SI. Proposal 4: TDL-C channel model is used for outdoor simulation assumption. Proposal 5: The simulation assumption of initial SFO and CFO agreed in R19 is reused for device 2b/C. |
| Huawei, HiSilicon | Observation 1: The energy source used by the device for energy harvesting and the size of the capacitor for energy storage have an impact on the operation time of the device. Proposal 1: For the study of outdoor deployment scenario in Rel-20, Table 1 is adopted as deployment scenario assumptions. Proposal 2: For the study of outdoor deployment scenario in Rel-20, the maximum distance between reader and device is calculated as 2*(ISD/3). Proposal 3: For the study of outdoor deployment scenario in Rel-20, RAN1 uses the pathloss model formula from TR 38.901 to derive the coverage distance based on the link budget. Proposal 4: Adopt the link budget template in Table 3 with updated items and values for the Rel-20 Ambient IoT study. Proposal 5: The “Penetration margin (dB)” is introduced in the link budget, and the value is 26.25 dB for UMa NLOS pathloss model, and 17.46 dB or 25 dB for RMa NLOS pathloss model. Proposal 6: For the study of outdoor deployment scenario in Rel-20, the channel model for link level simulation is TDL-C, with an RMS delay spread of 300 ns. Proposal 7: For the study of outdoor deployment scenario in Rel-20, reference data rate can be 0.1~1 kbps for both R2D and D2R coverage evaluation. Proposal 8: For the study of outdoor deployment scenario in Rel-20, the CFO and SFO values for device 2b/C can be set as follows - For device 2b (as per TR38.769) - The initial SFO can randomly select a value from the range of [0.1 ~ 1] *104 ppm, and the initial CFO can be set as 1000 ppm. - The SFO after clock sync / calibration at device side is 103 ppm, and CFO after clock sync / calibration at device side is 100 ppm. - For device C - The initial SFO and initial CFO can be set as 50 ppm. - The SFO and the CFO after clock sync / calibration at device side can be set as 10 ppm. Proposal 9: Link level simulation is evaluated according to the assumptions in Table 4. Proposal 10: Companies are suggested to report the evaluation results based on Table 5. Proposal 11: Companies to report assumptions on the energy source and capacitor size for energy harvesting/storage used to obtain the reported data rates. Our assumptions are as follows: - Energy source: solar power/light - Capacitor size: ~200 mF |
| Ericsson | - Observation 1: For Rel-20 A-IoT outdoor devices, the coverage target should reflect the differentiation from 6G LPWA, consider the SID guidelines and the mentioned distance range of 50 to 500 m. - Observation 2: As a starting point and before considering any coverage enhancement technique, the initial evaluations should consist in performing an analysis (e.g., link-budget) assuming the higher transmitting power available for devices C to determine if the coverage target can be fulfilled. - Observation 3: For maximum transmit power for coverage evaluation of device C in sensor applications, where the transmission power budget is lower than that in other use cases, at least small values such as 0 dBm or -3 dBm can be considered to evaluate whether the coverage targets can still be met. Proposal 1: The Rel-19 SI A-IoT link budget methodology and template, Table 4.3.2-1 in 38.769, can be taken as starting point and adapted accordingly for evaluating coverage of active A-IoT devices in outdoor scenarios. Proposal 2: The so-called Budget-Alt2 can be used for calculating receiver sensitivity and coverage evaluation for both reader-to-device (R2D) and device-to-reader (D2R) links. Proposal 3: For coverage evaluation of active devices in D4T1 scenario, RAN1 should consider UMa NLOS pathloss models. Proposal 4: For pathloss models UMa NLoS, the shadow fading margins can be considered as 6 dB. Proposal 5: Consider the following parameters for the reader: transmit power spectrum density of 36 (dBm/MHz), TX/Rx antenna gain of 8 dBi, and vertical height of 25 m. Proposal 6: Consider the following maximum transmit power for coverage evaluation of device C: -3 dBm, 0 dBm +5 dBm. Proposal 7: Use the adapted version of Table 4.3.2-1 from 3GPP TR 38.769, shown in Table 1, to evaluate the coverage of active A-IoT devices in outdoor scenarios (the needed updates are marked by red color). Proposal 8: Consider TDL‑C channel with a delay spread of 300 ns to link-level evaluations. Proposal 9: Use the adapted version of Table 4.2.2-1 from 3GPP TR 38.769, shown in Table 2, to evaluate the BLER performance of active A-IoT devices in outdoor scenarios (the updates are marked by red). |
| CATT | Observation 1: For R2D link, there will be no error floor only when M=2. Observation 2: For the R2D link in TDL-C channel: - The values of required SNR at BLER=10% when M=2, 6, 12, 24 are 4.2 dB, 9.7 dB, 12.7 dB, 18.2 dB respectively. - The values of required SNR at BLER=1% when M=2 is 16.3 dB. Observation 3: The values of required SNR at BLER=10% and 1% for the D2R link in TDL-C channel are -5.8 dB and -4.7 dB respectively. Observation 4: For device 2b with R2D data rate 7.5kbps and D2R data rate 5kbps, the coverage distance can achieve: - 232.8m when the target BLER is 10% - 218.2m when the target BLER is 1% Observation 5: For device C with R2D data rate 7.5kbps and D2R data rate 5kbps, the coverage distance can achieve: - 756.5m when the target BLER is 10% - 709.0m when the target BLER is 1% Proposal 1: For the coverage evaluation in Rel-20, the link budget template defined in Rel-19 should be used as baseline. Proposal 2: UMa NLOS and [RMa LOS] can be used as the pathloss model for the Rel-20 A-IoT study of outdoor deployment scenarios. Proposal 3: Both of the hardware capabilities of A-IoT device implementation and the value derived from the Budget-Alt2 need to be considered in the determination of receiver sensitivity for A-IoT devices. Proposal 4: Use the link budget template in Annex A to evaluate the coverage performance for device 2b and device C in outdoor scenario. Proposal 5: The following channel models can be considered in LLS for Rel-20 A-IoT: - TDL-C, 300ns, 3km/h - [TDL-E, 30ns, 3km/h] Proposal 6: Reference data rate higher than 100kbps can be considered in the LLS for Rel-20 A-IoT. Proposal 7: Use the assumptions in Annex B for the LLS for device 2b and device C. |
| vivo | Observation 1: The cell edge data rate, derived according to a certain coverage range, is not equivalent to user-experience data rate actually needed in AIoT use cases. - For example, if the cell edge data rate is ~5kbps assuming X = [500] meters coverage, while the user experienced data rate required in AIoT use case is typically less than 1kbps, the achievable coverage for Rel-20 AIoT is higher than X = [500] meters. Observation 2: BPSK and FSK are constant-envelope waveforms that enable much higher PA efficiency and transmitter efficiency compared with that for OFDM waveform widely used in cellular networks. Observation 3: initial clock accuracy is about 103ppm and residual CFO of 30ppm can be achieved after calibration. Observation 4: 500m coverage cannot be always achieved in UMa scenario and in RMa scenario based on Rel-19 signal channel design with minimum data rate with M=2 for R2D and ~5kbps for D2R. Observation 5: Remarkable coverage improvement can be achieved with enhancement solutions on top of Rel-19 air interface design. Observation 6: For AIoT-SSB signal/channel, 500m in UMa and 1000m in RMa scenario can be achieved respectively if 2 block level repetitions are used. Proposal 1: 500 meters can be assumed as starting point to determine cell edge data rate for UMa and RMa scenario - Different data rates can be evaluated to determine cell edge data rate, and to determine actual coverage can be achieved under user-experience data requirements, e.g., <1kbps. Proposal 2: For coverage evaluation for outdoor and active device, reuse evaluation methodologies used for device 2b for indoor scenario in Rel-19, with necessary changes, e.g., BS/AIoT device Tx power, channel model, and etc., applicable to outdoor scenario. Proposal 3: IF envelop detector or ZIF envelop detector should be assumed for receiver of active device - R2D receiver sensitivity is derived using budget-Alt2, i.e., receiver sensitivity is calculated based on required SNR obtained from LLS results. Proposal 4: For evaluation purpose for Rel-20 AIoT, 3.84Msps sampling rate at device can be considered as starting point. Proposal 5: For evaluation purpose for Rel-20 AIoT, 1~3 RB channel bandwidth can be considered as starting point for both D2R and R2D. Proposal 6：For evaluation purpose, 46dBm/43dBm Tx power and 8dBi antenna gain can be assumed for outdoor BS. Proposal 7：For evaluation purpose, - -10 dBm Tx power is assumed for device 2b; - 4 dBm Tx power is assumed for device C. Proposal 8: UMa-NLOS/RMa-NLOS can be considered as a pathloss model for outdoor, and shadow fading margin is 6dB and 8dB for UMa and RMa, respectively. Proposal 9: 10 km/h device speed is assumed in LLS. Proposal 10: TDL-C with 300ns for NLOS channel can be used in LLS for outdoor scenario. Proposal 11: Following coverage enhancement solutions on top of Rel-19 air interface design can be considered in coverage evaluation for Rel-20 AIoT - Repetition and/or FEC can be considered for R2D - Modulation schemes robust to CFO and enhancements on repetition/FEC, can be considered for D2R. Proposal 12: Evaluate coverage of periodic sync signal, aka., AIoT SSB considering large initial CFO. Proposal 13: Evaluate multiple data rates, and achievable cell edge data rate can be determined as the highest data rate which can achieve coverage better than selected maximum distance between Reader and Device e.g., 500 meters. Proposal 14: TR captures a conclusion that the reported data rates can be supported by energy sources with high energy conversion efficiency, e.g., solar, vibration, thermal, etc., with sufficient energy storage, which is up to implementation. - Reporting of detailed assumptions of energy source/storage corresponding to the data rate is not needed. Proposal 15: Adopt the Table 4 in R1-2505410 as evaluation assumptions for coexistence. - Detailed cases for co-existence evaluation, e.g., NR/AIoT aggressor AIoT/NR victim, etc., is up to RAN4 discussion. |
| Nokia | Observation 1: For the representative use cases rUC5, rUC6, and rUC8, more power budget for longer range communications and active data transmission capability is necessary like Device 2b and Device C. Observation 2: The only evaluation scenario in scope is “D4T1-C” (Deployment scenario 4 with topology 1 with no external carrier wave) which can be characterized as follows (following the format in Table 4.2.1-1 of TR 38.769): Scenario; CW Inside/outside topology; Diagram of the scenario; Description of the scenario; Device; CW spectrum; D2R spectrum; R2D spectrum. D4T1-C; No CW; No CW Node.; Device 2b, C; N/A; UL; DL. Observation 3: According to the SID, device 2b and device C use IF-ED or ZIF receiver. Proposal 1: RAN1 conclude that Device 2b and Device C are necessary and applicable to support the representative use cases rUC5, rUC6, and rUC8. Proposal 2: Select UMa (urban macro) as scenario for evaluation. Proposal 3: Select UMa-NLOS as pathloss model for link budget calculation. Proposal 4: Select TDL-C with 300 ns delay spread as channel model for link level simulation. Proposal 5: Consider 3 km/h and 10 km/h for device velocity. Proposal 6: Adopt initial SFO 104 ppm for device 2b and device C. Proposal 7: Adopt 800 bits as additional message size for LLS. Proposal 8: Use the link budget template (Table 4.3.2-1 of TR 38.769) agreed in Rel-19 as starting point. Proposal 9: Adopt Budget-Alt2 to derive the R2D receiver sensitivity. Proposal 10: Adopt 3-sector base station antenna pattern with 8 dBi. Proposal 11: Adopt base station antenna height of 25 m. Proposal 12: Adopt base station TX power of 46 dBm. |
| Xiaomi | Observation 1: The KPIs for sensor data collection in TS 22.369 can be applied as references when designing and determining the evaluation assumptions of Rel-20 A-IoT. Observation 2: The co-existence simulation methodology warrants re-examination to accommodate certain novel circumstances within the DT41-C scenario. Observation 3: The assumptions for coexistence simulation can be adaptively adjusted by referring to the existing simulation assumptions in Rel-19. Proposal 1: For device 2b in D4T1, the applicable maximum distance target value is 500m. For device C in D4T1, the applicable maximum distance value can reuse 500m or can be defined separately. Proposal 2: The procedures for sensor use case can be defined by the following steps: - Step 1: R2D synchronization signal transmission. Reader transmits synchronization signal to be received by A-IoT device(s). - Step 2: A-IoT random access. Synchronized A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure. - Step 3: Energy harvesting. A-IoT device(s) perform energy harvesting until the stored power is higher than the pre-defined threshold. - Step 4: Possible D2R data transmission. A-IoT device(s) perform the sensing result(s) transmission(s). Proposal 3: The definition of latency of sensor use case can be refined as the time interval between the time that R2D synchronization signal transmission and time that D2R data transmission completed or successfully received by reader. Proposal 4: For the purpose of potential evaluation, D4T1 can be added to the Table 4.2.1-1 of Evaluation scenarios in TR 38.769. Table 4.2.1-1: Evaluation scenarios (update) Scenario; CW Inside/outside topology; Diagram of the scenario; Description of the scenario; Device 1/2a/2b/C; CW spectrum; D2R spectrum; R2D spectrum. D4T1-C; No CW; No CW Node.; Device 2b/C; N/A; UL; At least DL. Proposal 5: In Rel-20 A-IoT for evaluation assumptions of both R2D and D2R, the channel model can consider: - D4T1: UMa NLOS with TDL-A and UMa LOS with TDL-D as baseline. - D4T1: RMa and SMa can be optionally considered. Proposal 6: In Rel-20 A-IoT, for evaluation assumption, the message size can additionally consider about 800 bits for outdoor sensor. Proposal 7: In Rel-20 A-IoT, for evaluation assumption, R2D FEC with 1/3 CC can be optionally considered in addition to “No FEC”, in order to achieve larger coverage. Proposal 8: For Rel-20 A-IoT, the coverage evaluation assumptions for Link-level simulation in TR 38.769 can be updated to support new deployment and active device with the following Table as reference. Proposal 9: The UMa, RMa (optional) and SMa (optional) scenario in TR 38.901 could be considered as the fundamental assumptions for the purpose of formulating the assumptions regarding the layout of D4T1 in Rel-20 A-IoT. Proposal 10: In Rel-20 A-IoT, for evaluation purpose, the following Link budget template and coverage calculation are defined for outdoor scenario and device 2b/C. Proposal 11: At least case 7 should be considered for coexistence evaluation in Rel-20 A-IoT. Proposal 12: The coexistence cases for Rel-20 A-IoT can be down selected from the definitions provided in TR 38.769 as show in the following Table. Proposal 13: The distribution of NR UE can be either within the coverage area of the A-IoT reader or solely within the coverage area of the NR BS, without overlapping with the area where the A-IoT devices are situated. Proposal 14: RAN1 needs to clarify whether the transmission power of the A-IoT reader is independent of the transmission power of the NR BS. Proposal 15: RAN1 needs to clarify whether to study the method for interference suppression to provide reasonable assumptions for the coexistence simulation work. |
| China Telecom | Proposal 1: LLS-based link budget methodology for A-IoT in indoor scenarios can be reused for A-IoT in outdoor scenarios for coverage evaluation methodology. Proposal 2: For an outdoor evaluation scenario, - For R2D/D2R link, - Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements. - Step 2: Obtain the receiver sensitivity derived by the required SINR which is given by LLS results. - Step 3: Obtain the coverage performance for link R2D/D2R based on the receiver sensitivity from step 2 and link budget template. Proposal 3: For outdoor scenarios, MPL and distance is used as performance evaluation metric for link budget calculation, where the distance is derived from MPL and corresponding pathloss model. Proposal 4: The following pathloss models for outdoor scenarios are used in the coverage evaluation. - TDL-C channel model for NLOS component - TDL-D channel model for LOS component Proposal 5: Regarding the carrier frequency, at least 900MHz and 2GHz need to be considered for FDD spectrum for outdoor scenarios. Proposal 6: Support the following BS layouts for UMa and RMa scenarios. Proposal 7: For LLS evaluation in outdoor scenarios, the total transmit power of BS can be configured as 36dBm as a starting point. Proposal 8: The Tx antenna gain of BS for A-IoT in outdoor scenarios should be considered. - The detailed values can be FFS. Proposal 9: For outdoor scenarios, link performance can be evaluated by predefining reference data rates and then deriving the achievable coverage range. - The detailed values of reference data rate can be FFS. Proposal 10: Coherent receiver is recommended in D2R transmission in outdoor A-IoT scenarios. |
| Samsung | Observation 1: For D2R link level simulation with payload size of 20 bits and 96 bits, after adding CFO=100 ppm with drift rate of 0.1ppm/s, both payload size could reach target BLER of 10% and 1%. The performance degradation for payload size of 20 bits are negligible, and for payload size of 96 bits there are 1dB degradation at BLER of 10% and ~3dB degradation at BLER of 1%. Proposal 1: Study applicable maximum distance target values for device 2b and device C, respectively, within the range of 50-500m, considering - Define separate target values of Device 2b and C, study the performance gap considering at least Tx power difference and R2D receiver sensitivity - Target value of device 2b is larger than 50m to achieve improvement from Rel-19 - Target value of device C can start with typical 3GPP outdoor UE coverage Proposal 2: The definition of latency in Rel-19 is reused for Rel-20 evaluation at least for DO-DTT and DT traffic types. - Study if new definition of latency are needed for DO-A traffic type. Proposal 3: On the basis of evaluation assumptions in Rel-19 SI, study at least the following assumptions for D4T1 scenarios: - Channel model - BS related assumptions, e.g. total transmit power, antenna gain, number of antenna elements, number of TXRUs - Layouts Proposal 4: On the basis of evaluation assumptions in Rel-19 SI, study at least the following refinements according to Rel-19 outcomes: - Block structure, e.g. X-ambles, CRC, FEC, repetitions - Reference data rate - Transmission bandwidth for R2D and D2R - Modulation and line encoding Proposal 5: On the basis of evaluation assumptions in Rel-19 SI, study at least the following assumptions according to Device 2b/C capabilities: - D2R transmit power - R2D receiver aspects, e.g. receiver noise figure, receiver sensitivity - ED bandwidth for R2D reception, transmission bandwidth and frequency resource allocation methods for D2R transmission - CFO aspects, e.g. CFO values, feasible assumptions on reader handling of CFO estimation and correction, D2R FDM related assumptions and restrictions, assumptions on phase offset, etc. Moreover, SFO aspects to reflect chip length drift - Assumptions related to self-generated D2R signal, e.g. waveform, power control Proposal 6: The layouts for coexistence evaluation can be up to RAN4. Proposal 7: Study the feasibility and appropriate assumptions for R2D narrowband filtering for coverage and coexistence evaluations. |
| ZTE | - Proposal 1: For coverage evaluation, the cell-edge data rate is investigated and the following methodology can be considered: - Step 1: Obtain the MPL based on the maximum coverage distance via pathloss model; - Step 2: Derive the required SNR from link budget according to the MPL; - Step 3: Determine the data rate corresponding to the required SNR via link-level simulations. - Proposal 2: Clarify the coverage target of outdoor deployment. The coverage distance of 500 m is assumed as the starting point and larger coverage distance can be studied to support rural use cases. - Proposal 3: Both in-band and standalone deployments should be considered in A-IoT coverage evaluation. - Proposal 4: For pathloss model, UMa NLOS is recommended as mandatory and the RMa NLOS model can be treated as optional. - Proposal 5: An additional penetration margin of 20 dB should be accounted in link budget based on O2I Car penetration loss model in 38.901. - Proposal 6: The link budget template in Table 2 can be considered as a starting point for A-IoT coverage evaluation. - Proposal 7: Coverage enhancements for both R2D and D2R transmissions should be studied for Device 2b/C to support the outdoor scenario. - Proposal 8: For energy storage, the maximum PRDCH reception time and the maximum PDRCH transmission time that Device 2b/C supports can be assessed and the above assessment methodology can be considered as a starting point. - Proposal 9: Device (un)availability solutions need be studied for Device 2b/C for the following cases: - Device (un)availability prior to receiving the target paging message. - Device (un)availability during the random access procedure. - Device (un)availability after the random access procedure. - Proposal 10: For the coexistence between Rel-20 A-IoT and NR/LTE, the cases in Table 3 and 4 can be considered and the details are determined by RAN4. |
| Transsion | Proposal 1: The maximum distance for each device type can be determined based on evaluation results of the link budget. Proposal 2 It is recommended to use the Budget-Alt2 method for calculating receiver sensitivity in the link budget template. Proposal 3 For Deployment Scenario 4, it is recommended to use the UMa (Urban Macro) and UMi (Urban Micro) channel models defined in TS 38.901 as the basis for pathloss modeling. |
| OPPO | - Proposal 1: The UMa scenario is considered for A-IoT outdoor scenarios. - Proposal 2: A-IoT devices drop uniformly distributed over the horizontal area in D4T1. - Proposal 3: D4T1-C is defined for A-IoT outdoor scenarios for the purpose of evaluation. - Proposal 4: The UMa NLOS model is considered as pathloss model in coverage evaluation for D4T1. - Proposal 5: The total Tx power of macro-BS is assumed to be 46 dBm. - Proposal 6: The total Tx power of Device C is assumed to be -3 dBm. - Proposal 7: The Tx antenna gain of BS for outdoor is assumed to be 8 dBi. - Proposal 8: The shadow fading margin is assumed to be 7.8 dB for the UMa NLOS model. Proposal 9: The link budget template for Device 2b can be reused for Device C, with the update of the total Tx power only. Proposal 10: For D4T1, TDL-C channel model with delay spread of 300 ns is considered for link-level simulation. Proposal 11: The LLS assumptions of Device 2b can be reused for Device C. Proposal 12: Consider -30dBm as RF energy harvesting sensitivity in Rel-20 A-IoT outdoor study. Proposal 13: Consider energy storage of A-IoT device <=200μJ in Rel-20 A-IoT outdoor study. |
| Tejas | Observation 1: To evaluate the pathloss model for outdoor Ambient IoT, the NLOS pathloss model for outdoor urban macro BS and urban micro BS are considered from TR 38.901. It has been observed that urban macro BS provides better coverage than urban micro BS. Observation 2: The coverage enhancement depends on multiple factors such as, the receiver sensitivity of device and reader, transmit power of device and reader, communication bandwidth, BS/reader and device height, etc. Observation 3: Considering urban macro BS and UE height of 5 m, the coverage values of Device 2b for different device transmit power are, 55m for -20dBm, 75m for -15dBm, and 103m for -10dBm device transmit power. Observation 4: Considering urban macro BS and UE height of 5 m, the coverage values of Device C for different device transmit power are, 157m for -3dBm, 188m for 0dBm, 225m for 3dBm, and 253m for 5dBm device transmit power. Observation 5: It has been observed that the 3D distance from reader to device increases with the device antenna height. Proposal 1: A suitable transmit power of device and Reader (base station) can be chosen to achieve the maximum target coverage which is the minimum value of the coverage achieved for the physical channels PRDCH, PDRCH for monostatic scenario. Proposal 2: A suitable and feasible device antenna height of 5-20m can be chosen to achieve the coverage target. |
| LGE | Proposal 1: Support at least 500m as the maximum target distance to support outdoor scenarios for Rel-20 Ambient IoT Proposal 2: Support 38 dBm as the basic evaluation assumption for maximum Tx power of base station for outdoor scenarios Proposal 3: At least assume following values for maximum Tx power of active devices for Rel-20 Ambient IoT study: - 5 dBm for device C - -10 dBm for device 2b Proposal 4: For Rel-20 Ambient IoT study, reuse as much as possible the link budget parameters for IF/ZIF receiver assumed in Rel-19 Proposal 5: MPL can be used for link budget calculation Proposal 6: UMa scenario as described in TR 38.901 can be assumed for path-loss model Proposal 7: TDL-C channel can be used as the mandatory link level simulation channel model Proposal 8: For LLS, parameters not specific to deployment scenarios (e.g. indoor/outdoor scenario) can be reused for Rel-20 evaluation assumptions |
| Apple | Proposal 1: Following LLS assumptions can be considered as the starting point for Rel-20 Ambient IoT Study on active devices for outdoor scenario DS4: Table 1: LLS assumptions for DS4 Proposal 2: Following link budget template can be considered as the starting point for Rel-20 Ambient IoT Study on active devices for outdoor scenario DS4: Table 2: Link budget template for DS4 Proposal 3: In terms of design targets, coverage study should aim to determine separate coverage range for device 2b and device C (within the recommended range of 50-500m) - For determining the respective coverage range comparison between device 2b and device c, same air interface design assumed as baseline and those can be feasible within device’s complexity and power requirements Proposal 4: Following deployment assumption can be considered as the starting point for Rel-20 Ambient IoT Study on active devices for outdoor scenario DS4: Table 3: DS4 deployment assumptions |
| NEC | Observation 1: For device with maximum transmit power higher than or equal to -10dBm, R2D would be the bottleneck for pathloss calculation. Observation 2: For device with maximum transmit power as -20dBm, D2R could be bottleneck only when 46dBm maximum transmit power adopted by gNB. Proposal 1: For outdoor channel model, UMi-street and UMa can be used for evaluation, while RMa and SMa may not be suitable due to large intersite distance. Proposal 2: For evaluation purpose, R2D would be the bottleneck of pathloss when transmit power of active device is higher or equal to -10dBm. Proposal 3: To meet the maximum distance of 50-500m, at least 83dB pathloss should be provided, which means the maximum transmit power of gNB should be higher than or equal to 38dBm, and device should use FEC and repetition when adopting -20dBm transmit power. |
| CMCC | Proposal 1: The MPL and coverage distance is used as performance metric for link-budget calculation. Proposal 2: For R2D link and D2R link in the coverage evaluation, Budget-Alt2 is used for device 2b and device C. Proposal 3: For an evaluation scenario, the coverage evaluation methodology is based on the following steps. - For R2D link or D2R link, - Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements - Step 2: Obtain the receiver sensitivity using Budget-Alt2 (receiver sensitivity is derived by required SINR which is given by LLS results). - Step 3: Obtain the coverage performance based on the receiver sensitivity from step 2 and link budget template. Proposal 4: For energy harvesting of device 2b and device C, various power sources provided by external environment other than RF energy harvesting can be considered. Proposal 5: Link budget for communications between reader and device can be calculated respectively as below， - MPLR2D = Transmitter Tx power – Device receive sensitivity + Transmitter antenna gain + Receiver antenna gain – shadowing fading margin – polarization loss - MPLD2R = Device Tx power – Receiver sensitivity+ Transmitter antenna gain + Receiver antenna gain – shadowing fading margin – polarization loss Proposal 6: The following pathloss model can be used in the coverage evaluation of Ambient IoT - For urban scenarios, UMA_A NLOS model can be used. - For rural scenarios, RMA_A NLOS model can be used. Proposal 7: For outdoor A-IoT BS readers, total Tx power of 38 dBm/46 dBm and Tx antenna gain of 15dBi can be considered for coverage evaluation. Proposal 8: The item of receiver interference density can be added in the link budget template considering the impact of outdoor cell interference, the candidate values can be reported by companies. Proposal 9: Adopt the link budget template in Table 2.1.1-1 for link budget evaluation in Rel-20 Ambient IoT. Proposal 10: For antenna configuration, 1Tx/1Rx for Ambient IoT devices is considered. Proposal 11: For link level performance evaluation, the following channel models are assumed, - Chanel models TDL-C as in TR 38.901, assuming a delay spread of 300ns and speed of 3km/h. Proposal 12: The following parameters of sampling frequency are considered in the evaluations, Parameter; Values. Sampling Frequency; Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial CFO for device 2b/C: 100 ppm (M) 200 ppm (O) 1000 ppm (O, only as initial CFO) Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. CFO after calibration for device 2b/C: 10 ppm - Note: Above assumptions are for LLS evaluation purpose only. Proposal 13: The BB LPF reported by companies in assumptions for LLS can be referred as receiver bandwidth for calculation of noise power for R2D in link budget template. Proposal 14: Adopt the LLS assumptions in Table 2.1.2-1 for coverage evaluation in Rel-20 Ambient IoT. |
| Sony | Observation 1 – Assuming Urban Marco pathloss model, the maximum distance 50-500 m is equivalent to LOS pathloss of 64.6-96 dB and NLOS of 79-118 dB. Observation 2 – With 5dBm Tx power, a link budget of 115dB can be achieved and the cell edge coverage can be realized. Observation 3: Device 2b may run without recovery periods on small batteries even though recovery period may be needed for devices relying on energy harvesting. Observation 4: Recovery periods may be needed for Device C when it performs its transmission when operating on constrained batteries, depending on device power consumption. Proposal 1 – RAN1 to use link budget template of Device 2b in the TR 38.769 when calculating link budget for device types of 2b and C. Proposal 2 – RAN1 to update/include the following parameters to address deployment scenario 4 and Device type C - Tx power for both BS and Device types, - Noise figure for Device types 2b and C, - Channel model to represent outdoor Macro, - CFO and SFO. Proposal 3 – For comparing applicability of Device 2b and Device C, RAN1 to include latency and spectral efficiency as performance metrics in addition to data rate at the cell edge. Proposal 4: Consider including energy harvesting and battery characteristics when choosing possible output power levels and system parameters. Proposal 5: For traffic type of DO-A, study feasibility of using code-based non-orthogonal multiple access in D2R. |
| InterDigital | Observation 1: Using the Rel-19 PRDCH design with wider receive BW (e.g., 2 MHz) may not be sufficient to achieve coverage at link distances of 500 m. Observation 2: Using the Rel-19 PDRCH design may achieve coverage at 500 m for device C, but not for Device 2b. Proposal 1: Use the evaluation assumptions from Table 1 as a starting point for R2D link level simulation. Proposal 2: Use the evaluation assumptions from Table 2 as a starting point for D2R link level simulation. Proposal 3: Study methods to improve R2D coverage such as channel coding, repetition. Proposal 4: Larger number of repetitions and lower code rate should be supported to improve D2R coverage if it is concluded that coverage enhancement is necessary. Proposal 5: Develop a revised link budget for Device 2b and Device C in Rel-20 based on the evaluation assumptions in Table 6. |
| Qualcomm | Observation 1: The A-IoT use cases with target coverage of 500m (i.e., use cases requiring large cells, wide area outdoor) have overlap that for LPWA, e.g., smart grid operation, gas meter, water meter, large smart farm sensors, infrastructure monitoring, etc. Observation 2: Low D2R data rate case - Device 2b - Device 2b coverage is D2R limited. - Max coverage of Device 2b (-20dBm, D2R 7bkps) is 61.8m. - Max coverage of Device 2b (-10dBm, D2R 7bkps) is 118.6m. - Device C - Device C (-3dBm) coverage is D2R limited. - Max coverage of Device C (-3dBm, D2R 7/4bkps) is 409.5m. - Device C (5dBm) coverage is R2D limited. - Max coverage of Device C (+5dBm, D2R 7/4bkps) is 690m. Observation 3: Higher data rate case - Device 2b - Device 2b link is D2R limited. - Max coverage of Device 2b (-20dBm, 42bkps) is 54.6m. - Max coverage of Device 2b (-10dBm, 42bkps) is 104.8m. - Device C - Device C (-3dBm) coverage is D2R limited. - Max coverage of Device C (-3dBm, 42/4bkps) is 361.7m. - Device C (5dBm) coverage is R2D limited. - Max coverage of Device C (+5dBm, 42/4bkps) is 609.6m. Observation 4: For active device, long tx/rx duration should be avoided due to higher power consumption. Observation 5: Higher data rate can reduce energy consumption by reducing tx/rx duration. Observation 6: Rel-20 devices should support a higher data rate to energy storage requirement. Observation 7: In outdoor (TDL-C, 300ns), the BLER performance of PRDCH of large M = 24, 32 degrade significantly due to long delay spread in outdoor environment compared indoor (TDL-A, 30ns). Observation 8: Overlaid sequence could provide diversity gain for the performance of PRDCH with M values <=12. Observation 9: For PRDCH detection, the energy detection method provides the significant gain of a few dB to more than 10dB compared to edge detection method. Observation 10: For SFO=1000ppm, the CP handling Method 1-1 works reasonably well for all M values up to M=32. Proposal 1: For coverage and coexistence evaluation, RAN1 to study at least two target use cases in outdoor scenarios in terms of coverage. - Target 1:50-100m (small cell/local area outdoor – small smart farm sensors, temperature/humidity monitoring sensor, outdoor inventory for small business, small size infrastructure monitoring) for device 2b - Target 2: 400-500m (large cell, wide area outdoor – smart grid operation, gas meter, water meter, large smart farm sensors, infrastructure monitoring) for device C Proposal 2: At least for evaluation purposes, RAN1 to investigate the following active device architectures with ZIF/IF receivers with further details regarding their potential effects on system design - Whether Tx RF filter / Rx RF filter is necessary or not for outdoors - Sampling clock speed faster than that for Device 1, e.g., 5.76 MHz or 11.52MHz - The accuracy assumption for Rx/Tx LO for generating carrier frequency at 900MHz and absence/presence of crystal - Whether I/Q branches are needed or not depending on the modulation scheme, waveform, etc. - Noise figure for 2b/C - Feasibility of power amplifier for 2b/C Proposal 3: For coverage evaluation purpose, consider the outdoor scenario D4T1-C. Proposal 4: For [1D]-R2D Number of Tx antenna/element TxRU/tx chain modelled in LLS, consider (M,N,P,Mg,Ng) = (4,2,2,1,1) or (2,4,2,1,1) with two TxRUs. Proposal 5: For coverage evaluation purpose, assume total R2D BS tx power of [44, 49]dBm with PSD limit of 36+X dBm/MHz and where X is a power boosting factor. X=0 for NR signal and X=6dB for A-IoT signal. Proposal 6: For [1F] R2D transmission bandwidth, consider 180kHz ~ 4.32MHz. For D2R transmission bandwidth, consider 15kHz as starting point. Proposal 7: For [1G]-R2D BS Tx antenna gain consider 17dBi (based on 16 elements, 2 Tx RU). For [1G]-D2R device tx antenna gain, consider -3dBi to capture practical value. Proposal 8: For [1N]-R2D Cable loss, consider 3dB considering outdoor BS. Proposal 9: For [2B]-R2D Bandwidth used for the evaluated channel, companies to report rx bandwidth considering tx signal bandwidth, Rx LO uncertainty, and BB LPF bandwidth. Proposal 10: For [2B]-D2R Bandwidth used for the evaluated channel, companies to report rx bandwidth considering, tx signal bandwidth (with 1SB/2SB assumption), and Tx LO uncertainty. Proposal 11: For [2D]-R2D Receiver Noise Figure, consider 18dB for device 2b and 15dB for device C. For [2D]-D2R Receiver Noise Figure, consider 5dB. Proposal 12: For [2L]-R2D/D2R Receiver sensitivity, consider Budget-Alt2 method only. Proposal 13: For [3A] R2D/D2R Shadow fading margin, consider 7.82dB for UMi and 6dB for UMa. Proposal 14: For [0d] Channel model, for NLOS consider TDL-C(M) and TDL-A(O). For LOS, consider TDL-D. Proposal 15: For [0d]-R2D/D2R Delay spread, consider following delay spread values for channel models. - 300ns(M) and 150ns(O) for TDL-C(M) - 150ns(M) and 30ns(O) for TDL-A(O) - 150ns(M) and 30ns(O) for TDL-D(O) Proposal 16: For [0f]-R2D/D2R Device velocity of 3km/h. Proposal 17: For [0h1] BS number of antenna elements, consider (M,N,P,Mg,Ng) = (4,2,2,1,1) or (2,4,2,1,1) with 2 TxRUs. Proposal 18: For [0m] Reference data rate, consider 7, 14, 42, 84, and 168kbps as a starting point. Proposal 19: For [0q-a] Sampling frequency offset, consider following options. - SFO before calibration: 10^3 - 10^4 ppm (M) - SFO after calibration: 10^3 ppm (M), 10^2 ppm (O) Proposal 20: For [0q2] Carrier frequency offset, consider following options. - CFO before calibration: 10^2 ~ 10^3 ppm (M), 10 - 100 ppm (O) - CFO after calibration: 100ppm (M), 10ppm(O) Proposal 21: For [1b] ED bandwidth, companies to report considered ED bandwidth considering tx signal BW and LO uncertainty. Proposal 22: For [1d] Waveform, OOK-4 or DFT-spread-OFDM could be baseline. Proposal 23: For [1e] Modulation, consider OOK as baseline. Other options are not precluded. Proposal 24: For [1j] Detection/decoding method for line coding, for line coding decode, if exist, RAN1 to assume “Energy detection” method (comparing energy of first chip and that of the second chip) as baseline. Companies to report schemes, if different scheme is considered. Proposal 25: For [2a1] Transmission bandwidth, support 15kHz, 30kHz, …, 1.44MHz, 2.88MHz for 1SB waveform. For 2SB waveform two times of 1SB signal is assumed. Proposal 26: RAN4 performs coexistence study considering outdoor multi-cell environment with inter-cell interference. |
| NTT DOCOMO | Proposal 1: For LLS assumptions for coverage evaluation in Rel-20, Rel-19 LLS assumptions for device 2b in Table 4.2.2-1 of TR38.769 should be the baseline. Proposal 2: For coverage evaluation in Rel-20, UMa NLOS pathloss model should be assumed for both R2D/D2R. Proposal 3: For LLS assumptions for coverage evaluation in Rel-20, TDL-C with 300ns delay spread should be assumed for both R2D/D2R. Proposal 4: For LLS assumptions for coverage evaluation in Rel-20, 1kbps and 5-7kbps can be kept as mandatory value for reference data rate. Proposal 5: For LLS assumptions for coverage evaluation in Rel-20, discuss whether the assumption on SFO for device C should be the same as device 2b or can be smaller. Proposal 6: For LLS assumptions for coverage evaluation in Rel-20, OOK-4 should be assumed for R2D modulation. M value can be 2 or 1 as baseline and other values are up to company’s report. Proposal 7: For LLS assumptions for coverage evaluation in Rel-20, Manchester coding specified in Rel-19 should be assumed for R2D line coding scheme. Proposal 8: For LLS assumptions for coverage evaluation in Rel-20, neither channel coding nor repetition should be assumed for R2D as baseline. Proposal 9: For LLS assumptions for coverage evaluation in Rel-20, Manchester coding or no line coding with square wave modulation as specified in Rel-19 should be assumed for D2R. Proposal 10: For LLS assumptions for coverage evaluation in Rel-20, TBCC specified in Rel-19 should be assumed for D2R as baseline. Proposal 11: For LLS assumptions for coverage evaluation in Rel-20, whether to apply block-level repetition for D2R can be up to company depending on the corresponding reference data rate. Proposal 12: For LLS assumptions for coverage evaluation in Rel-20, updates in red in Table 1 should be applied to Table 4.2.2-1 in TR38.769. Proposal 13: For link budget template for coverage evaluation in Rel-20, Rel-19 link budget for device 2b in Table 4.3.2-1 of TR38.769 should be the baseline. Proposal 14: For link budget template for coverage evaluation in Rel-20, total transmission power for BS in R2D should be 33dBm. Proposal 15: For link budget template for coverage evaluation in Rel-20, total transmission power for A-IoT device in D2R should be - -20dBm and -10dBm for device 2b - -3dBm and 5dBm for device C Proposal 16: For link budget template for coverage evaluation in Rel-20, transmission antenna gain for BS in R2D should be [17] dBi assuming 2 Tx antenna elements / TxRU and 8dBi antenna element gain. Proposal 17: For link budget template for coverage evaluation in Rel-20, receiver sensitivity for both R2D/D2R should be based on Budget-Alt2 in TR38.769. Proposal 18: For link budget template for coverage evaluation in Rel-20, shadow fading margin for both R2D and D2R can be 6 dB or 7dB if O2I scenario. Proposal 19: For link budget template for coverage evaluation in Rel-20, updates in red in Table 2 should be applied to Table 4.3.2-1 in TR38.769. |
| IIT Kanpur | Proposal 1: Use the link budget template defined in Rel-19 as the baseline for coverage evaluation in Rel-20. Proposal 2: Use at least UMa NLOS and RMa LOS pathloss models for the Rel-20 A-IoT study of outdoor deployment scenarios. Proposal 3: For studying the outdoor deployment scenarios, update to the Rel-19 link budget should be made to incorporate the evaluation assumptions for outdoor deployment including deployment layout, UE distribution, and additional BS-related parameters such as totasl Tx power, Tx/Rx antenna gain, number of antenna elements, and number of TXRUs. Proposal 4: Study the applicable maximum distance target values for Device 2b and Device C separately, within the range of 50–500 m, while considering the transmit power difference and receiver design aspects. Proposal 5: Use the Rel-19 link budget alt 2 for coverage evaluation in Rel-20 of the active device types with IF/ZIF receiver. Proposal 6: For the Rel-20 study on active device types, reuse the link budget parameters for IF/ZIF receivers assumed in Rel-19. Proposal 7: For link-level performance evaluation, use the TDL-C (DS-300 ns) and TDL-D (DS-100ns) channel profiles as defined in TR 38.901. |

Discussion

Based on inputs from papers, companies are mainly focusing on evaluation assumptions for this meeting. Most companies express views in their papers that the link budget template and LLS table in Rel-19 can be used as starting point for Rel-20 study with adapting to active device and outdoor scenarios. FL plans to discuss link budget and LLS assumptions in this way:

- Highlight in red texts on the potential update of items/values in link budget template table and LLS assumptions table for companies to see things to be discussed. (See in the end of section 3.3 and 3.4)

- Pick those things in red texts to be discussed and make corresponding individual FL proposals in section 3.3 and 3.4

- Once agreements achieved after discussion on those individual FL proposal, later in this week, FL will try to update the link budget template table and LLS assumptions table accordingly and ask if RAN1 could agree updated tables for further evaluation work.

In addition, FL has the following observations:

- For general aspects of evaluation, companies are mainly talking about the followings which can be found in section 3.1,

  - Rx sensitivity alternatives

  - Evaluation methodology

  - Applicable maximum distance

  - Others

- For necessary further assumptions of deployment scenarios for coverage and coexistence evaluations, companies are mainly talking about the followings which can be found in section 3.2,

  - Pathloss model

  - Deployment assumptions

  - Others

- For link budget template, some companies give detail updates on items and values in their papers. Companies are mainly talking about updates on the followings which can be found in section 3.3,

  - BS and Device Tx power

  - BS and Device antenna gain

  - BS and Device Rx noise figure

  - Bandwidth assumption

  - Interference density

  - Penetration margin/loss

- For LLS table, some companies give detail updates on items and values in their papers. Companies are mainly talking about updates on the followings which can be found in section 3.4,

  - Channel model and delay spread for outdoor

  - Payload size

  - Reference data rate

  - Device SFO/CFO assumptions

  - R2D assumption on FEC/repetition

- For energy harvesting/storage, companies are mainly talking about the followings which can be found in section 3.5,

  - Energy source

  - Energy storage

General

### >>Rx sensitivity alternatives

For Rx sensitivity Budget-Alt1 and Budget-Alt2 defined in R19, FL observes the followings

- 10 companies [Huawei, Ericsson, CATT, vivo, Nokia, Transsion, CMCC, Qualcomm, DCM] propose to use Budget-Alt2 for the Rel-20 study.

- 2 company [FUTUREWEI, Xiaomi] mentioned both Budget-Alt1 and Budget-Alt2.

Given that, FL propose the followings:

### [Closed][H] Proposal 3.1a-v1:

Proposal: For Rel-20 study, Budget-Alt2 (as per TR38.769) is used to derive Rx sensitivity in the evaluation for both R2D and D2R.

| Company | Views |
| --- | --- |
| CATT | Support Budget-Alt2. But the hardware capabilities of A-IoT device implementation should also be considered. |
| Spreadtrum | Support. |
| FUTUREWEI | Ok to the proposal and suggest to use Budget-Alt1 as optional |
| Ericsson | We support the proposal. |
| NEC | Support. |
| LGE | Okay with the proposal. |
| SONY | Support. |
| Samsung | OK |
| ZTE, Sanechips | OK |
| Xiaomi | OK |
| FL1 | This was agreed during Monday online thus closed. |
| QC | Support |

### >>Evaluation methodology

FL observes the followings,

- Most companies reuse Rel-19 methodology to do LLS for obtaining required SINR under certain reference data rate assumption, then derives Rx sensitivity according to that required SINR and do link budget calculation. FL copy the following from section 7.1 of TR 38.769

- On the other hand, 2 companies [vivo, ZTE] propose different methodology than Rel-19 which is to get required SNR from link budget by MPL/maximum distance first, then do LLS to determine data rate according to that required SNR.

Thus, FL would like to check views from companies which option is preferred from the followings.

### [Open][H] Proposal 3.1b-v1:

Proposal: For Rel-20 study, RAN1 to decide which of the followings is used to perform coverage evaluation for R2D and D2R:

- Option 1 (as Rel-19):

  - Step 1: Obtain the required SINR by link-level simulations under different reference data rates

  - Step 2: Obtain the receiver sensitivity based on the required SINR if Budget-Alt2 is used

  - Step 3: Obtain the MPL/distance based on the receiver sensitivity and link budget

- Option 2

  - Step 1: Obtain the MPL based on the distance via pathloss model

  - Step 2: Derive the required SINR from link budget according to the MPL

  - Step 3: Determine the data rate corresponding to the required SINR via link-level simulations.

| Company | Views |
| --- | --- |
| CATT | Prefer option 1. The target distance should be determined based on the evaluation results to ensure the rationality of the value, rather than being pre-set. On the other hand, from the perspective of LLS implementation, it’s hard to obtain the link performance that exactly matches the given SNR value. Additional evaluation error may be introduced by option 2. |
| Spreadtrum | Prefer option 1. Coverage should be calculated under different assumptions of reference data rates. Option 2 is more like evaluating the achievable cell edge data rate. These two options are different evaluations. |
| Ericsson | We support the methodology used in Rel-19: option 1. |
| DOCOMO | We support option 1. In general, we think achievable coverage highly depends on device complexity and data rate, thus the target on coverage as well as device complexity and data rate need to be discussed together. Then, we suggest evaluating coverage with different target data rate and target device complexity. |
| Tejas Networks | We prefer Option 1. Coverage should be calculated from MPL under different assumptions of reference data rate. |
| SONY | We are OK with either option. We think they will lead to the same conclusions. |
| Samsung | We prefer Option 1. |
| vivo | Fine with Option 1 |
| ZTE, Sanechips | Option 2 is more in line with the SI description that “ study outcomes should include achievable cell edge data rate assuming Maximum distance between Reader and Device of 50-500 m”. In Option 2, based on the required SNR, LLS can attempt different data rate configurations to obtain an approximate SNR at the target BLER (e.g., 1%). |
| CMCC | For the purpose of coverage evaluation, we think Option 1 should be used. The SID clearly says that we need to evaluate coverage, and the methodology should be Option 1. Intention of Option 2 is not clear. For Option 2, it first assumes a design targets and backwards obtains the cell edge data rate. However, at the current stage, we don’t even know what is the coverage distance. |
| Xiaomi | Rel-19 steps on methodology work well, and there is no necessary to change it in Rel-20. Option 1 should be kept and supported. |
| Huawei, HiSilicon | Support Option 1, this is aligned with existing TR. RAN1 can consider multiple reference data rate, and companies can simulate and report corresponding distance for each data rate. In Step 3 of Option 2, it seems multiple simulations need to be performed to get the corresponding data rate, i.e., Option 2 include running Option 1 multiple times. The simulation workload of Option 2 is higher. |
| QC | The current Rel-19 link budget excel sheet could be used for both options. Among the two, we prefer to use Option 1 since we can start from at least some feasible parameter values (e.g., SINRs, data rates). Then, the output (distance) will be also feasible accordingly. For Option 2, RAN1 first needs to determine target distance(s). Then, in step 2 and 3, resulting required SINR and required data rate may or may not be feasible, i.e., it is not clear whether the resulting SINR and data rate are achievable. |
| IDCC | We prefer Option 1. |

### >>Applicable maximum distance

Several companies mentioned applicable maximum distance or coverage target, the views are diverse and FL summarize the followings:

- Co-site outdoor BS coverage (ISD based): Huawei

- Typical 3GPP outdoor UE coverage: Samsung (Device C)

- 500m: vivo (starting point), Xiaomi, ZTE (starting point), LGE (at least 500m)

- >500m: Xiaomi (Device C), ZTE

- >50m: Samsung (Device 2b)

- 50-100m: Qualcomm (Device 2b)

- 400-500m: Qualcomm (Device C)

- Separate coverage range for Device 2b and C: Samsung, Qualcomm, Apple

- The maximum distance for each device type can be determined based on evaluation results of the link budget: Transsion

From above, FL observes some companies think Device 2b and Device C should have separate applicable maximum distance target within 500m. Some companies think at least Device C may have applicable maximum distance larger than 500m, or achieve typical 3GPP outdoor UE coverage or co-site deployment should be considered as ISD based maximum distance between reader and device.

### [Open][M] Question 3.1c-v1:

Question: For Rel-20 study, please companies share views on the coverage target in the following table.

- Q1: For outdoor coverage, is the applicable maximum distance target within 50-500m or could it be larger than 500m (e.g. consider co-site deployment and typical 3GPP UE outdoor coverage)?

- Q2: Should the applicable maximum distance target be investigated after seeing the evaluation results?

| Company | Views |
| --- | --- |
| CATT | The applicable maximum distance target can be investigated after seeing the evaluation results. |
| Ericsson | Reply to Q1: target should be within 50–500 m.50m-500m. Reply to Q2: Yes (considering that the applicable maximum distance target should be within 50m-500m according to SID) |
| NEC | Q1: It may require sources other than TR 38.848 to support coverage beyond 500m. |
| SONY | Q1. The goal should be to support operation of Ambient IoT devices with existing cellular infrastructure, hence the maximum distance should be 500m. The study should determine the data rates that can be supported at ranges between 50m-500m and device transmit powers between -3dBm and +5dBm,a according to the SID: - study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range Q2. The SID doesn’t task RAN1 to come up with a maximum distance target. It tasks us to investigate the data rates that can be achieved as a function of R2D distance and device output power. |
| Samsung | Q1: We are open to study target range > 500m to satisfy practical range of outdoor BS deployment. If evaluation result shows it hard to achieve, we can further consider to fallback to <= 500m. Q2: Yes |
| vivo | Q1: First, the SID did not ask us to determine a ‘coverage target’. Only ask RAN1 to select a coverage value to determine cell edge data rate. So any value selected between 50-500 meters can not be regarded as design target for R20 Ambient IoT. Q2: Maximum distance achievable can be investigated after seeing the evaluation results. If the cell edge data rate is [5] kbps for 500 meters after evaluation, while the required data rate for AIoT use cases is less than 1kbps according to TS22.369, the actual coverage can be achieved would be much higher than X=500 meters. In this case, conclusions like cell edge data rate is [5] kbps for 500 meters is less meaningful outcome for study item. |
| ZTE, Sanechips | Q1: Accoding to the ISD in practical macro cell deployment, the A-IoT outdoor coverage target should be assumed as 500 m. And larger coverage targets also can be considered for rural case. Q2: Yes, the applicable maximum maximum distance can be further refined based on the evaluation results. |
| CMCC | Q1: From operator perspective, we strongly prefer to consider the co-site deployment and therefore eventually the coverage can exceed 500 m. Q2: Yes. In addition, we think that such questions and related discussion can be postponed. In this meeting, we need to prioritize the discussion on the evaluation assumptions for coverage evaluation. |
| Xiaomi | Q1: Target of >500m can be determined based on study and evaluation/calculation during SI phase. Similar as the procedure and steps in Rel-19, the final target coverage distance can be determined along with the evaluation results. Q2: Yes. |
| Huawei, HiSilicon | Q1: Yes. The SID clearly says “Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs.” This reflects the requirement in real deployment, i.e., to reduce the cost of deploying A-IoT. In real deployment, ISD varies in urban, sub-urban, rural, and cell range depends on ISD. Therefore, achieving 500m does not mean the co-site deployment requirement can be met in all cases. RAN1 needs to consider commercial deployment requirement. Q2: Yes. It is not urgent to discuss the coverage target in Aug. In Aug, RAN1 needs to spend more time trying to finalize evaluation assumptions/parameters/etc. as much as possible, so that companies can start doing evaluations after Aug meeting. It’s good to check companies’ views on this question, but we think no proposal about this question is needed in Aug. |
| QC | Q1: Current SID says “Maximum distance between Reader and Device of 50-500 m”. Beyond 500m is out of current scope according to SID. Q2: RAN1 should consider detailed use cases first and distance targets should come from that. |
| IDCC | Q1. We are open to discuss. Q2. Yes. |

### >>Others

FL observes that some companies mentioned to discuss assumptions on active device with ZIF/IF receivers (e.g filter etc.), FL plans to discuss them together with parameters in link budget or LLS table where it will be used in the evaluation.

FL observes 3 companies [Nokia, Xiaomi, OPPO] suggest for Rel-20 study RAN1 could define evaluation scenario based on the update of evaluation scenarios defined in section 4.2.1 of TR38.769. FL understand the proposal from companies are straight forward according to the SID. Thus, FL plans to focus on evaluation assumptions in terms link budget and LLS which would be fundamental for evaluation work.

FL observes that most companies are talking about coverage evaluation, among them 3 companies [CMCC, CTC, LGE] propose MPL/distance is the metric for coverage evaluation in link budget calculation. FL understand MPL and distance is already in the TR and no body propose to change, thus seems no need to re-agree.

In addition, FL noticed that 3 companies [Xiaomi, Samsung, Sony] discuss latency definition while 1 company [Sony] discuss spectral efficiency.

Deployment scenarios

### >>Pathloss model

FL observes companies propose the following pathloss models for outdoor deployment evaluation:

- UMa: 18 companies [FUTUREWEI, Spreadtrum, Huawei, Ericsson, CATT, vivo, Nokia, Xiaomi, CTC, ZTE, Transsion, OPPO, Apple, NEC, CMCC, InterDigital, DCM, IITK]

  - 12 companies indicate NLOS while 1 company indicates LOS

- UMi: 3 companes [FUTUREWEI, Transsion, NEC]

- RMa: 8 companies [Huawei, CATT, vivo, Xiaomi, CTC, ZTE, CMCC, IITK]

  - 5 companies indicate NLOS while 3 company indicate LOS

- SMa: 1 company [Xiaomi]

Based on above, FL propose the followings

### [Open][H] Proposal 3.2a-v1:

Proposal: For Rel-20 study, the following path-loss model is used in the coverage evaluation:

- Option 1: UMa NLOS defined in TR38.901

- Option 2: RMa NLOS defined in TR38.901

| Company | Views |
| --- | --- |
| CATT | Support UMa NLOS for the use cases like smart manhole cover safety monitoring and elderly health care, RMa LOS for agriculture related use cases with sparse scatters. |
| Spreadtrum | Support option1. The SID assumes that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. According to TS38.901, the pathloss model should adopt UMa-NLOS. |
| FUTUREWEI | Option 1 |
| Ericsson | We support Option 1. |
| NEC | Support Option 1. Since the inter-site distance is 1732m which is larger than twice of 500m, RMa may not be suitable |
| DOCOMO | We prefer to make option 1 mandatory to minimize the evaluation workload. |
| LGE | Support Option 1 as mandatory and Option 2 can be reported optionally by companies. |
| FL | FL would like to clarify this proposal is made based on picking 2 most interests out of 4 scenarios discussed in company’s paper. FL’s intention was not asking for down-selection when writing them as options. Supporting both is also on the table, please continue share your views. |
| Tejas Networks | We support Option 1 as mandatory and option 2 as optional |
| SONY | Option 1 is OK. Option 2 can also be studied. |
| Samsung | OK with the two options. If down-select is needed, we prefer option 1. |
| vivo | Support both |
| ZTE, Sanechips | Support both. |
| CMCC | Support. Both options are typical outdoor deployments and pathloss models and can be supported. |
| Xiaomi | Both path-loss model of UMa and RMa can be studied and evaluated in Rel-20 for outdoor scenario. Besides, NLOS and LOS should be considered during evaluation since both can reflect the practical use cases. |
| Huawei, HiSilicon | Both options need to be supported. One typical use case of Ambient IoT is logistic, where the devices will move from urban to rural. So both urban and rural are typical scenarios. The workload increase of supporting both UMa and RMa is very marginal compared with supporting only one of them. Because the link level simulation (LLS) does not depend on UMa and RMa, so companies only need to do LLS once, and then use excel to get the result for UMa and RMa. |
| FL1 | Based on the feedback, FL update to Proposal 3.2a-v2 for Tuesday morning offline discussion, see in section 4.2 |
| QC | Thanks for the input and FL’s efforts for discussion. QC proposed UMa and UMi. We prefer to include UMa and UMi. This was not captured in the summary. We propose following update in the proposal. Proposal: For Rel-20 study, the following path-loss model is used in the coverage evaluation: - Option 1: UMi NLOS defined in TR38.901 - Option 2: UMa NLOS defined in TR38.901 We think RMa is similar to UMa and will not give meaningful difference. |
| IDCC | We are ok with both with Uma higher priority. |

### >>Deployment assumptions

FL observes companies talking in their papers on the followings:

- Companies indicate BS and Device antenna height which will be used in path-loss calculation.

- Many companies indicate outdoor BS layout and ISD in their papers for the evaluation while 1 company [Samsung] propose the layouts for coexistence evaluation can be up to RAN4.

### [Open][H] Proposal 3.2b-v1:

Proposal: For Rel-20 study, the antenna height for BS and Device used for evaluation purpose are as follows:

- BS antenna height

  - UMa-NLOS: 25m

  - RMa-NLOS: 35m

- Device antenna height

  - 1.5m

| Company | Views |
| --- | --- |
| CATT | OK |
| Spreadtrum | Support. |
| NEC | Support. |
| DOCOMO | Ok. |
| LGE | Support the proposal. |
| Tejas Networks | Support |
| Samsung | OK |
| vivo | OK |
| ZTE, Sanechips | OK |
| CMCC | Support |
| Xiaomi | OK |
| Huawei, HiSilicon | ok |
| FL1 | See updated Proposal 3.2b-v2 with editorial change in section 4.2 |
| QC | We propose following update in the current FL proposal. - BS antenna height - UMi-NLOS: 10m - UMa-NLOS: 25m - Device antenna height - 1.5m |

FL understands as Samsung pointed out, in the TR38.769 RAN4 already have assumptions on outdoor BS layout when did Rel-19 coexistence work with indoor A-IoT without RAN1 inputs at that time. For the study of Rel-20, RAN1 may take that into consideration when discuss the deployment assumption. FL collect the inputs into the table and further check views from companies.

With above, FL propose the followings and please companies to share your views

### [Open][H] Proposal 3.2c-v1:

Proposal: For Rel-20 study, the following layout assumptions is used for evaluation purpose:

- Hexagonal grid, 19 macro sites, 3 sectors per site

- ISD is considered as follows

  - UMa NLOS: [200m or 500m], 750m

  - RMa NLOS: [1732 m or 5000m]

| Company | Views |
| --- | --- |
| vivo | Support ISD = 750 meters for UMA. Prefer ISD=1732 meters for Rural. For ISD=5000meters, it is more challenge compared with 1732, we should first evaluate whether this is achievable before we use this for co-existence evaluation. |
| ZTE, Sanechips | It is preferred that - ISD is considered as follows - UMa NLOS: 500m, 750m - RMa NLOS: 1732 m |
| CMCC | Support 750 m for UMa and 1732 m for RMa. |
| Huawei, HiSilicon | Support UMa NLOS 750m and RMa NLOS 1732m. UMa 750m has already been adopted by RAN4 in R19 AIoT TR 38769 as below. |
| FL | See in section 4.2, updated proposal 3.2c-v2 according to the feedback. |
| QC | We propose following update to the current proposal. • Hexagonal grid, 19 macro sites, 3 sectors per site • UMi NLOS: 200m • UMa NLOS: 500m |

### >>Others

A few companies talked coexistence in their papers, FL have the following observations:

- Coexistence cases

  - 1 company [vivo] propose that detailed cases for co-existence evaluation, e.g., NR/AIoT aggressor AIoT/NR victim, etc., is up to RAN4 discussion.

  - 2 companies [Xiaomi, ZTE] propose coexistence cases in their paper.

- Inter-cell interference

  - 1 company [Qualcomm] propose that RAN4 performs coexistence study considering outdoor multi-cell environment with inter-cell interference.

  - 1 company [Xiaomi] propose that RAN1 needs to clarify whether to study the method for interference suppression to provide reasonable assumptions for the coexistence simulation work.

  - 1 company [FUTUREWEI] mentioned with the RF LO and IF filter, an active A-IoT device can also makes it possible coexistence with other 5G/NR system(s) on downlink.

  - 1 company [Samsung] propose to study the feasibility and appropriate assumptions for R2D narrowband filtering for coverage and coexistence evaluations.

FL understands the coexistence cases were not determined in RAN1 and up to RAN4. For the second point FL plans to discuss narrowband filtering for Rx bandwidth in link budget and LLS assumptions.

Link budget

Companies express views in their papers that the link budget template used in Rel-19 (as per TR 38.769) can be used as starting point for Rel-20 study with adapting to active device and outdoor scenarios. FL plans to discuss those key adapting parts from inputs first, then try to formulate the link budget template table later in this meeting for company further checking the update.

### >>BS and Device Tx power

For BS Tx power, FL observes the followings from inputs

- 49dBm: Qualcomm

- 46dBm: Spreadtrum, CATT, vivo, Nokia, Xiaomi, OPPO, CMCC

- 44dBm: Qualcomm

- 43dBm: vivo, ZTE (for standalone deployment), Apple

- 38dBm: Huawei, Xiaomi, LGE, NEC(>=38dBm), CMCC

- 36dBm/MHz: Ericsson

- 36+XdBm/MHz where X is 6dB for A-IoT signal and 0dB for NR: Qualcomm

- 33dBm: Xiaomi, ZTE (for in-band deployment), InterDigital, DCM

- 24dBm: Xiaomi, InterDigital

- Xiaomi propose that RAN1 needs to clarify whether the transmission power of the A-IoT reader is independent of the transmission power of the NR BS.

FL notices that:

- The values companies provided in wide range and some companies mentioned in-band and standalone case may have different BS Tx power.

- Most companies propose Tx power in dBm while 2 companies propose power spectral density dBm/MHz

  - Note that for 36dBm/MHz which contains maximum 4PRBs and approximately equivalent to ~30dBm/1PRB and ~46dBm/50PRBs

  - Note that for 36+6dBm/MHz power boosting which contains maximum 4PRBs and approximately equivalent to ~36dBm/1PRB and ~46dBm/10PRBs

Thus, based on above, FL propose the followings with understanding at least two values are needed for in-band and standalone deployment respectively. FL would like companies to share views which value(s) to pick.

### [Open][H] Proposal 3.3a-v1:

Proposal: For Rel-20 study, BS Tx power assumption for evaluation purpose is from the followings:

- For standalone: [46, or 43] dBm

- For in-band:

  - Option 1: [38, or 33] dBm

  - Option 2: (36+X) dBm/MHz

    - X = [0 or 6] dB

| Company | Views |
| --- | --- |
| Ericsson | We support the proposal without X dB power boosting in Option 2. |
| DOCOMO | We agree with the direction, i.e., one value assuming standalone deployment and one value assuming in-band to NR deployment. For in-band, we support option 1 and support 33dBm or 36dBm can be acceptable. We tend to agree that total transmission power should be derived based on PSD limitation and assumed BW for R2D transmission especially for outdoor scenario. If we go with option 2, we support 33dBm/MHz or can be ok with 36dBm/MHz. Power boosting if applicable should be limited to standalone deployment which does not have any other transmission outside the A-IoT band in the same band. |
| LGE | Support the proposal. |
| Tejas Networks | We support this proposal. |
| SONY | Support proposal. For standalone, 46dBm or 43dBm are OK. For in-band, we should consider a PSD limitation. We would support 36dBm / MHz. We don’t need to consider power boosting. |
| Samsung | OK with the proposal. For in-band cases, we prefer option 1. |
| vivo | Refer to NB-IoT in TR 36.802, BS max Tx power is 46 dBm for in-band and 43 dBm for standalone. We can reuse these assumptions. |
| ZTE, Sanechips | We prefer 43 dBm for standalone and 33 dBm or Option 2 for in-band |
| CMCC | We think for A-IoT in standalone bands, the total Tx power of A-IoT BS readers can be assumed as 46dBm, for in-band, we prefer Option 1 with 38dBm is assumed for TX power of reader. |
| Xiaomi | This proposal can be discussed as the starting point. For standalone, 46 dBm can be supported, and for in-band, Opiton 1 is preferred. |
| Huawei, HiSilicon | Ok with proposal. Support 46dBm for SA, and 38dBm for IB. Option 1 is simpler and preferred. |

For Device Tx power, FL observes the followings from inputs

- Device 2b

  - -10dBm: Spreadtrum, CATT, vivo, Xiaomi, LGE, Apple, CMCC, DCM

  - -20dBm: CATT, Xiaomi, Apple, CMCC, DCM

- Device C

  - 5dBm: Ericsson, Xiaomi, LGE, Apple, CMCC, DCM

  - 4dBm: vivo

  - 0dBm: Spreadtrum, Ericsson, CATT, Apple, CMCC

  - 0-5dBm: Huawei

  - [-3, 5]dBm: ZTE

  - -3dBm: Ericsson, Xiaomi, OPPO, Apple, DCM

Thus, FL propose the followings based on the inputs.

### [Open][H] Proposal 3.3b-v1:

Proposal: For Rel-20 study, Device Tx power assumption for evaluation purpose is from the followings:

- Device 2b: {-10, -20} dBm

- Device C: {-3, 0, 5} dBm

| Company | Views |
| --- | --- |
| CATT | OK. |
| Spreadtrum | Support. |
| FUTUREWEI | Ok to the proposal |
| Ericsson | We support the proposal. |
| NEC | Ok. |
| DOCOMO | Support. |
| LGE | Support the proposal. |
| Tejas Networks | Support |
| SONY | Support |
| Samsung | OK |
| vivo | Clarify that, are we going to down select among these values? or evaluate all of these? Regarding device Tx power for device C, -3dBm Tx power is too low. If we assume 10mW peak power consumption, and Tx efficiency 5%, which is too low, and even worse than the transmission efficiency of OFDM transmitter. Considering we are using BPSK/MSK with constant envelop, the transmission efficiency should be much higher than traditional OFDM transmitter. Hence, -3dBm Tx power should be removed. And based on our survey 5dBm transmission power can be achieved at 10mw power consumption [Ref]. [Ref] Arun Paidimarri, Phillip M. Nadeau, et. “A 2.4GHz Multi-Channel FBAR-based Transmitter with an Integrated Pulse-Shaping Power Amplifier”, Institute of Electrical and Electronics Engineers |
| ZTE, Sanechips | OK |
| CMCC | For Device C, we think that -3 dBm is too low. From coverage evaluation perspective, we prefer to consider larger transmission power candidates. |
| Xiaomi | OK |
| Huawei, HiSilicon | Suggest to add 3dBm for device C as well. This can give RAN1 more information for further discussions. |
| FL1 | Based on above feedback, FL update to Proposal 3.3b-v2 in section 4.2 |

### >>BS and Device antenna gain

For BS antenna gain, FL observes the followings:

- 17dBi: Huawei, Apple, Qualcomm (based on 16 elements, 2 Tx RU), DCM (2*8)

- 15dBi: CMCC

- 8dBi: Ericsson, CATT, vivo, Nokia, OPPO

- 6dBi: Interdigital

FL understands that the main difference between large value and small value may be from BS antenna gain model. As per Qualcomm and DoCoMo mentioned the proposed 17dBi gain including 16 elements (i.e. 8 elements per Tx RU). It will similar as those proposed 8dBi but need to set 8 elements per Tx RU in link level simulation. FL understands in that sense, they should be same in principle but results in two different options to do the evaluation. Thus, FL propose the followings:

### [Open][H] Proposal 3.3c-v1:

Proposal: For Rel-20 study, BS antenna gain assumption in link budget for evaluation purpose is from the followings:

- Option 1: 17dBi

  - This assumes in the LLS table (as per TR38.769), the values in “[0h1] Number of antenna elements” and “[0h2] the Number of TXRUs” would be same (as Rel-19)

- Option 2: 8dBi

  - This assumes in the LLS table (as per TR38.769), the value in “[0h1] Number of antenna elements” would be 8 times of the value in “[0h2] the Number of TXRUs”

| Company | Views |
| --- | --- |
| DOCOMO | Support option 1. Based on the coverage evaluation in previous releases for outdoor scenario for NR, around 17dBi was assumed as antenna gain with assumption on 16 antenna elements and 2 TxRUs for FR1 FDD band such as 700MHz and we think this value is feasible for antenna gain for Macro BS assumption. |
| ZTE, Sanechips | It needs to be clarified whether the BS antenna gain uses a fixed value of 17 dBi or 8 dBi, or is calculated based on (gain of antenna element) + 10 log (Number of antenna elements / Number of TXRUs). If the latter calculation method is adopted, the link budget template should be updated accordingly. |
| CMCC | Option 1 |
| Huawei, HiSilicon | Support Option 1. As FL explained, technically Option 1 and 2 do not have much difference. But Option 1 is simpler. |

For Device antenna gain, FL observes most companies remain to use 0 dBi in their papers while 1 company Qualcomm propose -3dBi instead.

- 0dBi: Huawei, FUTUREWEI, Spreadtrum, Ericsson, CATT, Xiaomi, ZTE, Tejas, CMCC, InterDigital, Apple

- -3dBi: Qualcomm

Thus, FL propose the following,

### [Open][H] Proposal 3.3d-v1:

Proposal: For Rel-20 study, Device antenna gain assumption in link budget for evaluation purpose is 0dBi for both Device 2b and C.

| Company | Views |
| --- | --- |
| CATT | Support . |
| Spreadtrum | Support. |
| FUTUREWEI | 0 dBi |
| Ericsson | We support the proposal. |
| DOCOMO | Ok. |
| LGE | Support the proposal. |
| Tejas Networks | Support |
| Samsung | OK |
| vivo | Support |
| ZTE, Sanechips | OK |
| CMCC | Support. |
| Xiaomi | OK |
| Huawei, HiSilicon | ok |

### >>BS and Device Noise figure

For BS noise figure, FL observes that most companies use 5dB in their papers while 1 company [Huawei] uses 3dB. Thus, FL propose the following,

### [Open][H] Proposal 3.3e-v1:

Proposal: For Rel-20 study, BS noise figure assumption in link budget for evaluation purpose is 5dB.

| Company | Views |
| --- | --- |
| CATT | OK. |
| Ericsson | We support the proposal. |
| DOCOMO | Ok. |
| LGE | Support the proposal. |
| Tejas Networks | OK |
| Samsung | OK |
| ZTE, Sanechips | OK |
| CMCC | Support |
| Xiaomi | OK |
| Huawei, HiSilicon | We can be ok with this for progress. |

For Device noise figure, FL observes that some companies use 15dB for both Device 2b and C. Some companies think the noise figure for Device 2b can be larger than 15dB (although in TR 38.769, Device 2 is assumed to be 15dB). Some companies think the noise figure for Device C can be better than Device 2b. Based on the values mentioned from papers, FL propose the following,

### [Open][H] Proposal 3.3f-v1:

Proposal: For Rel-20 study, Device noise figure assumption in link budget for evaluation purpose is from the followings:

- Option 1:

  - 15dB for both Device 2b and Device C

- Option 2:

  - Device 2b: [15, 18, 15-20, or 40] dB

  - Device C: [10-15, 15, or 30] dB

| Company | Views |
| --- | --- |
| CATT | The difference of the hardware architectures for device 2b and device C may need to be clarified before we determine the value of device noise figure. |
| Spreadtrum | Support option1. According to R20 SID, it assumes similar device architectures for Device 2b and Device C. Therefore, the noise figure assumption for device 2b and Device C should be same. |
| FUTUREWEI | Option 2 |
| NEC | Support Option 1, unless the description of device 2b and C in SID is updated accordingly. |
| DOCOMO | Open to discuss. |
| LGE | Same view with CATT. Clarification is needed between device 2b and device C. |
| SONY | We prefer option 2. The RX architecture for device 2b and device C are likely to be different (based on available power consumption). We hence think that the noise figure for device C can be significantly less than for device 2b. |
| vivo | Similar view as CATT. |
| ZTE, Sanechips | The noise figure is mainly depending on the LNA design. We did not see the obvious difference, then in this case, option 1 based on R19 study could be used. |
| CMCC | Option 1. In general, we think that the main difference for device 2b/C are on the transmission power, and for the other parts they are basically the same, so prefer to use the same noise figure assumptions for both devices. |
| Huawei, HiSilicon | Generally fine with Option 2 direction. SID says “Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C.”. Device C can have higher power and higher cost than Device 2b, so Device C should be better than Device 2b. |

### >>Bandwidth assumption

FL understand the following bandwidth will be used in evaluation for link budget and LLS:

- R2D Tx bandwidth

- D2R Tx bandwidth

- R2D Rx bandwidth

- D2R Rx bandwidth

For R2D Tx bandwidth, FL observes the following values discussed in papers:

- 180kHz: Huawei, FUTUREWEI, Spreadtrum, vivo, Tejas, CMCC, InterDigital, Ericsson, Xiaomi, ZTE, CATT, Qualcomm, Apple

- 360kHz (O): Ericsson, vivo, ZTE

- 540kHz (O): vivo, ZTE

- M*180kHz: CATT

- 180kHz ~ 4.32MHz: Qualcomm

- 1.08MHz: Ericsson, Xiaomi

Thus, FL propose the followings

### [Open][H] Proposal 3.3g-v1:

Proposal: For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 180kHz as starting point

  - Other values of multiple 180kHz are not precluded and up to company report

| Company | Views |
| --- | --- |
| CATT | For M=12 and M=24, the minimum bandwidth is larger than 180kHz, as agreed in Rel-19 A-IoT. The BLER performance of R2D link may degrade in R20 due to larger delay spread and more severe multi-path fading. So 180kHz may be not enough. Integer multiples of 180kHz can be used as starting point. |
| Spreadtrum | Support. |
| FUTUREWEI | Ok to the proposal |
| NEC | Support. |
| DOCOMO | Ok. |
| LGE | Support the proposal. Other values can be reported by companies regarding the M value used for LLS. |
| Tejas Networks | Support |
| SONY | We envisage higher bandwidths than 180kHz, given the number of devices that would be supported in an outdoor scenario. Is the 180kHz bandwidth the R2D channel bandwidth or the system bandwidth? We assume that transmissions to multiple dev2c / devC devices can be frequency division multiplexed. |
| Samsung | OK |
| vivo | OK |
| ZTE, Sanechips | OK |
| CMCC | Support. |
| XIaomi | OK |
| Huawei, HiSilicon | ok |
| FL1 | @Sony, FL understands 180kHz is transmission BW, not system BW in the case you mentioned. |

For R2D Rx bandwidth, FL observes the following values discussed in papers:

- Many companies use 180kHz as ED bandwidth or BB LPF bandwidth

- CMCC propose the Rx BW the BB LPF reported by companies in assumptions for LLS can be referred as receiver bandwidth for calculation of noise power for R2D in link budget template.

- Qualcomm propose the for [2B]-R2D Bandwidth used for the evaluated channel, companies to report rx bandwidth considering tx signal bandwidth, Rx LO uncertainty, and BB LPF bandwidth.

FL understands as Qualcomm mentioned, due the Rx LO uncertainty of active device, ZIF/IF receiver filter BW will be impact by e.g. CFO ppm assumption. Thus, FL propose

### [Open][H] Proposal 3.3h-v1:

Proposal: For Rel-20 study, R2D receiver bandwidth assumption in link budget and LLS for evaluation purpose is up to company report with considering:

- R2D transmission bandwidth

- Device Rx LO uncertainty (e.g. CFO ppm assumption)

- Note: ED bandwidth and BB LPF bandwidth in LLS are assumed to be same as R2D receiver bandwidth in link budget for active device with ZIF/IF receiver

| Company | Views |
| --- | --- |
| CATT | OK |
| Spreadtrum | Support. |
| FUTUREWEI | Ok to the proposal. Up to the companies to report |
| LGE | Support the proposal. |
| SONY | OK. However, it is FFS whether the device architecture supports ED (devC could support a receiver with better performance than ED). |
| Samsung | OK |
| vivo | For the note,’ ED bandwidth and BB LPF bandwidth in LLS are assumed to be same’ While in TR38.769, it assumes that for BB LPF - [X]-order Butterworth/RC filter with cutoff frequency at half of R2D transmission bandwidth. We wonder why the assumption is not reused. |
| ZTE, Sanechips | OK |
| CMCC | Generally support to report by companies. Similar comment as vivo, the BB LPF cutoff frequency is half of R2D transmission BW, not the same. |
| Huawei, HiSilicon | ok |
| FL1 | @vivo,@CMCC, FL understands cutoff frequency is half of R2D transmission BW which means BB LPF BW = 2*cutoff frequency = R2D transmission BW. And considering CFO impact, the cutoff frequency will be larger than half of R2D transmission BW. |

For D2R Tx bandwidth, FL observes the followings discussed in papers:

- Many companies use same assumption as Rel-19 as per TR 38.769

- 1 company Huawei propose to use 10kHz instead of 15kHz

- 1 company Ericsson propose 180kHz (M) and 15kHz (O)

- 1 company Qualcomm propose to use values 15kHz, 30kHz, … 2.88MHz corresponding to Rel-19 D2R chip/bit duration and change to SSB bandwidth.

FL propose the following,

### [Open][H] Proposal 3.3i-v1:

Proposal: For Rel-20 study, D2R transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 15kHz as starting point

  - Other values are not precluded and up to company report

  - Companies to report which of DSB or SSB modulation is assumed for D2R transmission bandwidth

| Company | Views |
| --- | --- |
| CATT | D2R transmission bandwidth depends on the modulation scheme and bit duration. Both of the two factors has multiple candidates. So taking only one value as starting point may be too limited. |
| Spreadtrum | Support. |
| FUTUREWEI | Ok to the proposal. |
| DOCOMO | Ok. |
| LGE | Support the proposal. |
| Samsung | OK |
| vivo | OK |
| ZTE, Sanechips | OK |
| CMCC | Support. |
| Xiaomi | OK |
| Huawei, HiSilicon | We can be ok with this for progress. |
| FL1 | @CATT, here is just BW, and may corresponding to chip/bit duration. The proposal decoupled with modulation scheme |

For D2R Rx bandwidth, it refers to LLS and describe as company to report. Thus, FL think may be only need to clarify the Rx bandwidth needs to consider Tx LO uncertainty compared to Rel-19. Thus, FL will not make an individual proposal here and the update can be found in LLS table update.

### >>Interference density

4 companies [Huawei, CMCC, Xiaomi, Qualcomm] discussed interference/multi-cell interference while among them 2 companies [Huawei, CMCC] propose to add a row/item into link budget template as receiver interference density which is similar as TR38.830 or TR38.910 for outdoor coverage evaluation. In addition, CMCC propose values of 0dB or XdB up to company report (FL understands this would require further clarify from CMCC the meaning of proposed value since the unit of interference density is dBm/Hz rather relative dB). Huawei propose to reuse values from TR 37.910 which is -169.3dBm/Hz for R2D and -165.7dBm/Hz for D2R.

Thus, FL propose the followings

### [Open][H] Proposal 3.3j-v1:

Proposal: For Rel-20 study, add a new item of “Receiver interference density (dBm/Hz)” into the link budget template and the value of X dBm/Hz for evaluation purpose is from the followings:

- For R2D, X = {-169.3, other value(s)}

- For D2R, X = {-165.7, other value(s)}

| Company | Views |
| --- | --- |
| ZTE, Sanechips | OK |
| CMCC | We are fine with the proposal. |
| Huawei, HiSilicon | Support. R19 only considers indoor, which is limited area, so that inter-cell interference can be resolved/alleviated by implementation, e.g., round robin inventory. R20 is outdoor, it’s almost impossible to avoid inter-cell interference in outdoor scenarios. Thus, this item needs to be added. Otherwise, RAN1 evaluation only considers noise and is too optimistic and cannot reflect real deployment scenarios. FYI: TR 38830 already have “Receiver interference density” as below. The values in the proposal comes from TR 37910 and can be found from excel in screenshot below. |
| FL1 | See update to Proposal 3.3j-v2 with editorial change in section 4.2 |

### >>Penetration margin

4 companies [Huawei, Xiaomi, ZTE, Apple] propose to consider penetration loss/margin for outdoor scenario coverage evaluation and hence propose to add a new item to link budget template. Among of them, 1 company Xiaomi propose company to report with justification and other 3 companies propose details values. ZTE and Apple propose 0 or 20 dB while Huawei propose 26.25 for UMa NLOS, 17.46 or 25 for RMa NLOS. Note that companies indicate these values mentioned in their papers are based on TR38.901.

Thus, to simplify the discussion, FL propose the following based on the collection of values provided from companies.

### [Open][H] Proposal 3.3k-v1:

Proposal: For Rel-20 study, add a new item of “Penetration margin (dB)” into link budget template and the value of X dB for evaluation purpose is from the followings:

- For both R2D and D2R

  - X = {0, 15, 20, 25} dB

| Company | Views |
| --- | --- |
| Spreadtrum | It’s not clear for the actual scenario and use case from the existing deployment to consider penetration margin. |
| DOCOMO | We think it depends on whether to consider, e.g., O2I scenario or not. If Rel-20 targets only device outdoor scenario, we think such penetration loss is not necessary to be considered. |
| SONY | We don’t need to consider penetration margin. AIoT devices are not expected to work inside metal containers. |
| ZTE, Sanechips | If we only consider O2O case, that is to say, the device could move by itself. Instead, in our understanding, the A-IOT device would be carried in the vehicle, which actually introduce the penetration loss. Therefore, 20dB could be assumed as baseline according to TR38.901 and others value could be further discussed. |
| CMCC | We understand this proposal from a different perspective. In the SID, it clearly says co-site deployment of NR macro BS, and from our perspective, we strongly expect that A-IoT outdoor deployment can have an aligned coverage, which means that the MCL should be 144 dB. We can consider such kind of design target in RAN1. But since from Rel-19, we are using MPL and coverage distance as the performance metric, an alternative way is to consider a more realistic assumption for outdoor scenario, and we think that penetration loss can be included as one of the realistic parameters, where for wide area tracking, it is highly possible that the device 2b/C are deployed in a car. |
| Xiaomi | We are OK to discuss about penetration margin, but the first step is to discuss and make it clear that what exact use case and scenario should be considered in Rel-20 outdoor, and what kind of penetration loss should be considered when devices are inside buildings or cars. |
| Huawei, HiSilicon | Support. The value of penetration loss depends on use case. One typical use case of Ambient IoT is logistic, where the devices will probably be inside a container, e.g., crate, box, transport vehicle, etc. Thus, in real deployment, there will be penetration loss even in some outdoor scenarios. In some other use case, e.g., sensor with good condition, maybe it’s true the penetration loss can be 0dB. If RAN1 simply consider 0dB penetration loss for all cases, the evaluations are too optimistic and cannot reflect real and diverse deployment scenarios. Having this item and allowing multiple values can give companies flexibility to simulate different use cases. FYI: “Penetration margin (dB)” already exists in TR 38830 as below (coverage SI TR). |

### >>Link budget template

For information at this time, link budget template table is assumed to be updated as follows according to the discussion in above sections,

### [Open][M] Proposal 3.3l-v1:

Proposal: For Rel-20 study, “Link budget template” is expected to be as followings,

Link budget template for Rel-20 study

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | Deployment scenario 4 with topology 1 with no external carrier wave | Deployment scenario 4 with topology 1 with no external carrier wave |
| [0B] | Device 2b/C | Device 2b/C | Device 2b/C |
| [0C] | Center frequency (MHz) | 900MHz (M), 2GHz (O) | 900MHz (M), 2GHz (O) |
| [0D] | Pathloss model | To be discussed | To be discussed |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | For BS: 2(M) or 4(O) antenna elements for 0.9 GHz | 1 |
| [1E] | Total Tx Power (dBm) | To be discussed | To be discussed |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180kHz(M), 360kHz(O), 1.08MHz(O) To be discussed | Refer to LLS table [2a1] |
| [1G] | Tx antenna gain (dBi) | To be discussed | For A-IoT device, 0dBi |
| [1J] | Ambient IoT on-object antenna penalty | N/A | 0.9dB or 4.7dB |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | For BS, X dB, X <=3 to be reported by companies with justification provided in row 5A | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth To be discussed | Refer to LLS table [2a3] |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | For IF/ZIF receiver 15dB, Device 2 Other values can be optionally used and reported by companies To be discussed | For BS as reader: 5dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [[2E1]] | [Receiver interference density (dBm/Hz)] | [To be discussed] | [To be discussed] |
| [2F] | Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0.9 dB or 4.7 dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt1/ Budget-Alt2 (see note1) To be discussed | Budget-Alt2 |
| [2L] | Receiver Sensitivity (dBm) | To be discussed | To be discussed |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | To be discussed | To be discussed |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB | 0 dB |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| [[3E]] | [Penetration margin (dB)] | [To be discussed] | [To be discussed] |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

FL notes: Calculated details in Note 1 will be updated later accordingly. Above those cells which contain red highlighted texts will be discussed in section 3.3 with individual proposals and that cell will be updated once corresponding agreement achieved. Please companies kindly indicate whether you are fine or have comments with the remaining cells which only have black texts.

| Company | Views |
| --- | --- |
| CATT | In Rel-20 A-IoT, outdoor macro BS is considered as the reader, which is different from the assumption of indoor BS in Rel-19. So [1D] Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS for BS also needs to be discussed. |
| CMCC | We think some tables should be further discussed: [1J] and [2H]: Rel-20 device 2b/C has an entirely different format from Rel-19 device 1. For Device 1, its antenna is directly patched to a surface and therefore on-object antenna penalty is assumed. For Device 2b/C, it is more like a module and the antenna are not directly patched to a surface and therefore no need to consider this on-object penalty. At least for these two tables, 0 dB should be added as one alternative. |
| FL1 | @CATT, see your proposal in paper, will check with other companies when discuss the BS antenna gain. @CMCC, FL will add [0dB] when update the table later and ask for companies to further check. |

LLS

Companies express views in their papers that the LLS assumption table used in Rel-19 (as per TR 38.769) can be used as starting point for Rel-20 study with adapting to active device and outdoor scenarios. FL plans to discuss those key adapting parts from inputs first, then try to formulate the LLS assumption table later in this meeting for company further checking the update.

### >>Channel model and delay spread

For channel model in link level simulation, FL observes the followings:

- TDL-A: InterDigital, Qualcomm (O)

- TDL-B: FUTUREWEI

- TDL-C: Spreadtrum, Huawei, Ericsson, CATT, vivo, Nokia, CTC (for NLOS), OPPO, LGE, CMCC, Qualcomm (M), DCM, IITK

- TDL-D: CTC (for LOS), Qualcomm (for LOS), IITK

- TDL-E: CATT

For delay spread in link level simulation, FL observes the followings:

- 100ns: FUTUREWEI, IITK (TDL-D)

- 150ns: Qualcomm (TDL-A/D)

- 300ns: Huawei, Ericsson, CATT (TDL-C), vivo, Nokia, OPPO, CMCC, Qualcomm, DCM, IITK (TDL-C)

- 30ns: CATT(TDL-E), InterDigital (TDL-A), Qualcomm (Option for TDL-A/D)

Based the inputs, FL propose the following,

### [Closed][H] Proposal 3.4a-v1:

Proposal: For Rel-20 study, channel model TDL-C with an RMS delay spread of 300 ns delay spread is used for link level simulation.

| Company | Views |
| --- | --- |
| CATT | Support |
| Spreadtrum | Support. |
| FUTUREWEI | Ok to the proposal. Suggest to use TDL-B/100 ns as optional |
| DOCOMO | Support. |
| ZTE, Sanechips | Ok |
| Xiaomio | LOS should also be considered in Rel-20 A-IoT. |
| FL1 | This was agreed during Monday online with restriction for NLOS |

### >>Reference data rate

FL observes the followings companies used in their papers:

- Reuse Rel-19: Spreadtrum, Xiaomi, CMCC, NTT DOCOMO, CATT, InterDigital

- 0.1-1kbps: Huawei

- 5-7kbps: Ericsson, DoCoMo

- 50kbps: Ericsson

- >100kbps: CATT

- 7, 14, 42, 84, and 168kbps: Qualcomm

- 1kbps-640kbps: FUTUREWEI

FL copies existing values from TR38.769 here

FL propose the followings,

### [Open][H] Proposal 3.4b-v1:

Proposal: For Rel-20 study, reference data rate in LLS assumption for evaluation purpose is from the followings:

- Reuse the values in Rel-19 (as per TR 38.769) as starting point

  - FFS whether to add additional larger value(s)

| Company | Views |
| --- | --- |
| CATT | OK |
| Spreadtrum | Support. |
| FUTUREWEI | Ok to the proposal. |
| NEC | Support |
| DOCOMO | Support. |
| SONY | We are OK to use the TR38.769 data rates as a baseline. We are also OK to support other data rates, where these additional values don’t need to be larger: they could be smaller. The SID asks RAN1 to study the cell edge data rates as a function of range and TX power. Those data rates are not necessarily larger than in TR38.769 |
| Samsung | OK |
| LGE | Support. |
| vivo | OK |
| ZTE, Sanechips | OK |
| CMCC | Support |
| Xiaomi | OK |
| Huawei, HiSilicon | ok |

### >>Message size

2 companies [Nokia, Xiaomi] propose to add 800 bits in addition to the 3 values of 20 bits, 96 bits and 400 bits in current TR 38.769.

### [Open][H] Proposal 3.4c-v1:

Proposal: For Rel-20 study, message size in LLS assumption for evaluation purpose is from the followings:

- Reuse the values in Rel-19 (as per TR 38.769) as starting point

  - FFS whether needs further update to add additional value of 800 bits.

| Company | Views |
| --- | --- |
| Spreadtrum | Support. |
| NEC | Support. |
| Samsung | OK |
| LGE | Support |
| ZTE, Sanechips | OK |
| CMCC | Support |
| Xiaomi | Support |
| Huawei, HiSilicon | ok |

### >>Device CFO/SFO

For CFO and SFO assumption for LLS assumption, FL observes the followings

  - Device 2b:

    - Initial SFO:

      - R19: Spreadtrum, Huawei, CATT, Qualcomm

      - 10^4 ppm: Nokia

    - After SFO:

      - R19: Spreadtrum, Huawei, Ericsson, Qualcomm

      - 10s-100s ppm: FUTUREWEI

    - Initial CFO

      - R19: Spreadtrum, Huawei, CMCC

      - 200 ppm: Apple (O), CMCC (O)

      - 100 ppm: Apple (M), CMCC (M)

      - 10^2-10^3 ppm: Qualcomm (M)

      - 10-100 ppm: Qualcomm (O)

    - After CFO

      - R19: Spreadtrum, Huawei

      - 10s-100s ppm: FUTUREWEI

      - 10 ppm: CMCC, Qualcomm (O)

      - 100 ppm: Qualcomm (M)

  - Device C:

    - Initial SFO

      - R19: Spreadtrum, Qualcomm

      - 10^4 ppm: Nokia

      - 10^3 ppm: CATT

      - 50ppm: Huawei

    - After SFO

      - R19: Spreadtrum, Qualcomm

      - 10s-100s ppm: FUTUREWEI

      - 10 ppm: Huawei

      - 100 or 1000 ppm: Ericsson

    - Initial CFO

      - R19: Spreadtrum,

      - 200 ppm: Apple (O), CMCC (O)

      - 100 ppm: Apple (M), CMCC (M)

      - 50ppm: Huawei

      - 80 ppm: CATT

      - 10^2-10^3 ppm: Qualcomm (M)

      - 10-100 ppm: Qualcomm (O)

    - After CFO

      - R19: Spreadtrum

      - 10s-100s ppm: FUTUREWEI

      - 10 ppm: Huawei, CMCC

      - 10 ppm: CMCC, Qualcomm (O)

      - 100 ppm: Qualcomm (M)

Based on the inputs, FL propose the followings,

### [Open][H] Proposal 3.4d-v1:

Proposal: For Rel-20 study, SFO and CFO in LLS assumption for evaluation purpose is from the followings:

- Option 1: Device C is better than Device 2b

| Option 1 | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | 10^3~10^4 ppm | [50, 80, 10~100] ppm |
| SFO after clock sync/calibration | 10^3 ppm | [10] ppm |
| Initial CFO | 10^2~10^3 ppm | [50, 80, 10~100] ppm |
| CFO after clock sync/calibration | 100 ppm | [10] ppm |

- Option 2-1: Device C and Device 2b are same (with large ppm)

| Option 2-1 | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | 10^3~10^4 ppm | 10^3~10^4 ppm |
| SFO after clock sync/calibration | 10^3 ppm | 10^3 ppm |
| Initial CFO | 10^2~10^3 ppm | 10^2~10^3 ppm |
| CFO after clock sync/calibration | 100 ppm | 100 ppm |

- Option 2-2: Device C and Device 2b are same (with small ppm)

| Option 2-2 | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | [50, 80, 10~100] ppm | [50, 80, 10~100] ppm |
| SFO after clock sync/calibration | [10] ppm | [10] ppm |
| Initial CFO | [50, 80, 10~100] ppm | [50, 80, 10~100] ppm |
| CFO after clock sync/calibration | [10] ppm | [10] ppm |

| Company | Views |
| --- | --- |
| CATT | Support option 1. The power consumption of device C is about ten times larger than that of device 2b. So for device C, clocks with higher accuracy and power consumption can be used and smaller SFO and CFO can be achieved. |
| Spreadtrum | We support option 2-1. According to R20 SID, it assumes similar device architectures for Device 2b and Device C. Therefore, the SFO and CFO for device 2b and Device C should be same and the values discussed in R19 for device 2b are reused. |
| FUTUREWEI | Option 1 with following changes: Device 2b initial CFO/SFO 10^3 ppm and 100 ppm after calibration. Device C initial SFO/CFO 100 ppm and 10 ppm after calibration. |
| Ericsson | We support the proposal. |
| NEC | Option 2-1 seems reasonable, unless SID can capture the difference. |
| DOCOMO | Given that the clock accuracy is based on peak power consumption, we support option 1. |
| SONY | Option 1. Device C can support more accurate clocks. We think device type C can support a lower CFO after calibration than 10ppm. |
| Samsung | OK with the proposal. We support Option 1 and option 2-2, and prefer option 1 between these two. |
| LGE | Support Option 1. |
| vivo | First, in our understanding, the only difference between device 2b and device C is PA output power, and the clock assumptions for device 2b and device C are same. Second, SFO and CFO value before and after calibration are same, since a Clock can be used for different purposes. Besides, CFO calibration to 10 ppm may be feasible in some cases. But, when device is 500 meters (or even longer distance) away from base station, whether it is still feasible to achieve 10 ppm CFO calibration accuracy should be well justified. - Option 2-3: Device C and Device 2b are same (with small ppm) Option 2-2; Device 2b and device C. Initial SFO; 10^3~10^4ppm. SFO after clock sync/calibration; [30] ppm. Initial CFO; 10^3~10^4ppm. CFO after clock sync/calibration; [30] ppm. |
| ZTE, Sanechips | Prefer Option 1 with the minor modifications: Option 1; Device 2b; Device C. Initial SFO; 10^3~10^4 ppm; [50, 80, 10~100] ppm. SFO after clock sync/calibration; 10^23 ppm; [10] ppm. Initial CFO; 10^2~10^3 ppm; [50, 80, 10~100] ppm. CFO after clock sync/calibration; 10s~100 ppm; [10] ppm. |
| CMCC | As we commented for Proposal 3.3f that the main difference for Device 2b/C are the transmission power, from this perspective, the assumptions on SFO/ CFO should not vary too much for the two kinds of devices. In addition, for the detailed assumptions on SFO and CFO, although different values are captured in the TR 38.769 for device 2b, that is essentially considering clocks for different purposes. However, in practical implementation, we highly doubt that a device 2b or C will use different reference clock for different purposes, therefore, same assumptions can be assumed for SFO and CFO for the device. |
| Huawei, HiSilicon | Option 1. Device C has higher power than device 2b, thus Device can achieve better SFO/CFO than device 2b. This principle is also reflected in the TR, where device 2b has better SFO/CFO than device 1. Option 2-1 is unreasonable, since Device C definitely can have better SFO/CFO. Option 2-2 is also unreasonable, since technically it’s almost infeasible for device 2b to reach such small SFO/CFO. |
| FL1 | Based on feedback, FL updates in Proposal 3.4d-v2, see in section 4.2 |

### >>FEC and repetition

For R2D FEC, some companies e.g. Ericsson, DoCoMo propose no FEC as baseline, some other companies e.g. CMCC and Huawei propose companies to report FEC. FL propose the followings,

### [Open][H] Proposal 3.4e-v1:

Proposal: For Rel-20 study, R2D FEC in LLS assumption for evaluation purpose is from the followings:

- Company to report no FEC or using FEC in their evaluations

| Company | Views |
| --- | --- |
| CATT | OK with the proposal. |
| Spreadtrum | Support. |
| FUTUREWEI | Ok to the proposal. |
| Ericsson | We believe the baseline assumption should be no FEC, and the decision on whether FEC needs to be considered can be postponed until after agreements are reached in 10.3.2. |
| DOCOMO | Ok. |
| LGE | Support the proposal. |
| vivo | Support. Besides, some further details, the bit collection scheme, can also be reported when FEC is used. |
| ZTE, Sanechip | OK with the proposal. It should be encouraged that company could provide some assumptions for FEC if it is used. |
| CMCC | OK with the proposal, although we think that considering the large coverage for outdoor, using FEC should be the baseline. |
| Xiaomi | OK |
| Huawei, HiSilicon | Ok. |

For R2D repetition, some companies e.g. Ericsson, DoCoMo propose no repetition as baseline, some other companies e.g. vivo, Huawei propose to use repetition. Ericsson propose to add a new row for R2D repetition. FL understand in Rel-19 D2R already support repetition but missing in LLS table. FL propose the followings,

### [Open][H] Proposal 3.4f-v1:

Proposal: For Rel-20 study, add new item of “Repetition” for both R2D and D2R into LLS assumptions for evaluation purpose

- Company to report no repetition or using repetition in their evaluations

| Company | Views |
| --- | --- |
| CATT | OK in general. The details e.g. repetition number, bit/block/chip level repetition should also be reported. |
| Spreadtrum | Support. |
| FUTUREWEI | Ok to the proposal. |
| Ericsson | We believe the baseline assumption should be no repetition, and the decision on whether repetition needs to be considered/ number of repetitions can be postponed until after agreements are reached in 10.3.2. |
| DOCOMO | Ok. |
| Tejas Networks | Support |
| Samsung | OK |
| LGE | Support the proposal. |
| vivo | Support |
| ZTE, Sanechips | OK |
| CMCC | OK with the proposal. |
| Xiaomi | OK |
| Huawei, HiSilicon | Support. We assume companies will probably simulate the case with different number of repetitions (maybe including no repetition). Explicitly adding such item can make RAN1 evaluations and discussions clearer. “no repetition” is also allowed by adding such item. |

### >>LLS assumptions table

For information at this time, LLS assumption table is assumed to be updated as follows according to the discussion in above sections,

### [Open][M] Proposal 3.4g-v1:

Proposal: For Rel-20 study, “Coverage evaluation assumptions for link-level simulation” is expected to be as followings,

Coverage evaluation assumptions for link-level simulation for Rel-20 study

| No. | Parameters |  | Assumptions |
| --- | --- | --- | --- |
| R2D/D2R common parameters |  |  |  |
| [0a] | Carrier frequency |  | Refer to link budget template |
| [0b] | SCS |  | 15 kHz as baseline |
| [0c] | Block structure |  | Blocks as agreed in Rel-19 as starting point, other blocks reported by companies |
| [0d] | Channel model |  | To be discussed |
| [0e] | Delay spread |  | To be discussed |
| [0f] | Device velocity |  | 3 km/h |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| [0h1] | BS | Number of antenna elements | 2 or 4 |
| [0h2] |  | Number of TXRUs | 2 or 4 |
| [0m] | Reference data rate |  | 1 kbps (M) 5 - 7 kbps (M) 48 - 60 kbps (O) 0.1 kbps for message size of 20 bits or 96 bits (O) Other data rates can be reported by companies Note 1: Companies to report the exact data rate. Note 2: The exact data rate is close to the values listed above. Note 3: The exact data rate is calculated by dividing the total message size (excluding CRC) by the total transmission time including applicable overheads(e.g., CRC, pre/mid/post-ambles if present). Note 4: The exact data rate may be related to coding scheme, repetition and etc. Note 5: All data rates considered are for evaluation purpose only To be discussed |
| [0n] | Message size |  | {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size To be discussed |
| [0p] | BLER target |  | 1%, 10% |
| [0q] | Sampling frequency |  | Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): (M) Randomly select a value from the range of [0.1 ~ 1] *10^4 ppm for device 2, (M) Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1, (O) Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 2, Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution Note: Above values are only for sampling purpose. Note: Above assumptions are only for LLS evaluation purpose only for R2D and D2R. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. Note: SFO after clock calibration can be applied to Fe. CFO for device 2b: 100 ppm (M) 200 ppm (O) 1000 ppm (O, only as initial CFO) Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. Note: Above assumptions are for LLS evaluation purpose only To be discussed |
| [0r] | Device 2b/C |  | ZIF/IF-ED |
| R2D specific parameters |  |  |  |
| [1a] | Transmission bandwidth |  | 180 kHz as baseline. Other larger values are not precluded. |
| [1b] | ED bandwidth |  | The ED bandwidth is the bandwidth for calculating the noise/interference (if any) power: For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, [180] kHz for IF/ZIF receiver. Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth. To be discussed |
| [1c] | BB LPF |  | [X]-order Butterworth/RC filter with cutoff frequency at half of R2D transmission bandwidth. Companies to report X = {3, 5}. To be discussed |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator |
| [1e] | Modulation |  | OOK-4 Companies to report M chips per OFDM symbol |
| [1f] | Line code |  | Manchester |
| [1g] | FEC |  | No FEC as baseline To be discussed |
| [1h] | ADC bit width |  | 4-bit |
| [1j] | Detection/decoding method for Line code |  | Companies to report |
| [[1k]] | [Repetition] |  | [To be discussed] |
| D2R specific parameters |  |  |  |
| [2a1] | Transmission bandwidth |  | [2a1]-Alt1 (M): DSB X kHz is considered for D2R transmission bandwidth. The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz [2a1]-Alt2: SSB X kHz is considered for D2R transmission bandwidth. The value is for one sideband, i.e., the total transmission bandwidth for SSB is X kHz. Not applicable for at least device 1 X = {[15 (M)], [180 (O)]}, other values are not precluded and reported by companies To be discussed |
| [2a2] | Chip rate |  | Companies to report |
| [2a3] | Receiver bandwidth |  | D2R receiver bandwidth is the bandwidth used at the reader side to filter out the D2R signals for calculating noise and interference (if any) power. Assume the receiver matches the transmitter's modulation, i.e., to receiver uses SSB when transmitter uses SSB, receiver uses DSB when transmitter uses DSB. Companies to report the value, and further down-selection of the values and DSB/SSB is not precluded. |
| [2b] | Waveform (CW) |  | RF LO |
| [2d] | Modulation |  | Companies to report modulation, e.g., OOK, BPSK |
| [2e] | Line code |  | Companies to report, e.g., Manchester encoding, no line coding |
| [2g] | FEC |  | Companies to report, e.g., CC, No FEC |
| [2h] | ADC bit width |  | Companies to report, e.g., 11-bit, 14-bit |
| [2j] | D2R receiver |  | Companies to report, e.g., coherent receiver / non-coherent receiver |
| [[2k]] | [Repetition] |  | [To be discussed] |
|  | Other assumptions |  |  |
| [3a] | Other assumptions |  | To be reported by company |
| [3b] | Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target. |  |  |

FL notes: Above those cells which contain red highlighted texts will be discussed in section 3.4 with individual proposals and that cell will be updated once corresponding agreement achieved. Please companies kindly indicate whether you are fine or have comments with the remaining cells which only have black texts.

| Company | Views |
| --- | --- |
| CATT | [0h1] Number of antenna elements for BS also needs to be discussed, as our view in Proposal 3.3l. |
| vivo | For [0f] Device velocity Device velocity is up to 10 km/h for outdoor scenario as per TR 22.840. Thus, 10 km/h can be assumed for link level simulation assumption. |
| ZTE, Sanechips | We are not sure about [2b] whether we still need the CW waveform. |
| CMCC | [0p] We prefer to discuss use 10% as the BLER target. Considering the very large coverage targets for outdoor, 1% BLER may be too stringent. Also, the Rel-20 A-IoT system can consider designs such as HARQ-ACK and re-transmissions and a BLER of 1% seems not as fundamental as Rel-19. |
| FL1 | FL noticed these and will try to discuss them when update the table. |

Energy harvesting/storage

7 out of [23] companies discuss energy harvesting/storage in their papers. FL observes the followings main interests on assumptions:

- Energy sources

  - RF energy source: 2 companies [OPPO, Qualcomm]

  - Other energy source than RF: 5 company [FUTUREWEI, Huawei, vivo, CMCC, Qualcomm]

    - e.g., solar/light, vibration, thermal, etc.

- Energy storage

  - In addition to capacitor, 1 company [Sony] also discuss small rechargeable battery.

  - Companies discuss energy storage size in different volumes

    - e.g. tens of uF, <=200μJ, ~200mF

  - 1 company [ZTE] propose that for energy storage, the maximum PRDCH reception time and the maximum PDRCH transmission time that Device 2b/C supports can be assessed and the above assessment methodology can be considered as a starting point.

  - 1 company [Qualcomm] discuss data rate and energy consumption/storage for coverage evaluation.

  - 1 company [vivo] propose that TR captures a conclusion that the reported data rates can be supported by energy sources with high energy conversion efficiency, e.g., solar, vibration, thermal, etc., with sufficient energy storage, which is up to implementation.

Based on above inputs, FL propose the followings,

### [Open][M] Proposal 3.5a-v1:

Proposal: For Rel-20 study, RAN1 assumes companies to report energy source/storage assumptions with their evaluations.

| Company | Views |
| --- | --- |
| FUTUREWEI | Ok to the proposal. |
| Huawei, HiSilicon | We are supportive of this proposal. The motivation to consider these assumptions for energy harvesting/storage comes from their impact on the reported cell edge data rate. This would depend on the energy source for energy harvesting and the size of the capacitor for energy storage, and assumptions on these parameters can be reported. We support to leave it up to company reports because such energy harvesting/storage assumptions can vary drastically, for which it would be difficult to obtain consensus on. |
| Tejas Networks | Support |
| SONY | Support. Agree with Huawei. |
| ZTE, Sanechips | Support |

Evaluation results

FL observes some companies have preliminary results as early inputs in their paper for this meeting. FL will not process evaluation results at this moment. Once evaluation assumptions are going to be determined, FL will kick off results collection for companies to fill into and update their results in future meetings.

Offline/Online proposals

Online - Monday afternoon

Offline - Tuesday morning

### [Open][H] Proposal 3.1b-v1:

Proposal: For Rel-20 study, RAN1 to decide which of the followings is used to perform coverage evaluation for R2D and D2R:

- Option 1 (as Rel-19):

  - Step 1: Obtain the required SINR by link-level simulations under different reference data rates

  - Step 2: Obtain the receiver sensitivity based on the required SINR if Budget-Alt2 is used

  - Step 3: Obtain the MPL/distance based on the receiver sensitivity and link budget

- Option 2

  - Step 1: Obtain the MPL based on the distance via pathloss model

  - Step 2: Derive the required SINR from link budget according to the MPL

  - Step 3: Determine the data rate corresponding to the required SINR via link-level simulations.

### [Open][H] Proposal 3.2a-v2:

Proposal: For Rel-20 study, the following path-loss models are used in the coverage evaluation:

- UMa NLOS defined in TR38.901

  - ISD is considered as: [500m], 750m

- [RMa NLOS defined in TR38.901]

  - [ISD is considered as: 1732m]

- [UMi NLOS defined in TR38.901]

  - [ISD is considered as: 200m]

- [Note: Company can also report to use RMa LOS defined in TR38.901]

### [Open][H] Proposal 3.2b-v2:

Proposal: For Rel-20 study, the antenna height for BS and Device used for evaluation purpose are as follows:

- BS antenna height

  - UMa: 25m

  - [RMa: 35m]

- Device antenna height

  - 1.5m

### [Open][H] Proposal 3.2c-v2:

Proposal: For Rel-20 study, the following layout assumptions is used for evaluation purpose:

- Hexagonal grid, 19 macro sites, 3 sectors per site

- ISD is considered as follows

  - UMa NLOS: [500m], 750m

  - [UMi NLOS: 200m]

  - [RMa NLOS: 1732m]

### [Open][H] Proposal 3.3a-v1:

Proposal: For Rel-20 study, BS Tx power assumption for coverage purpose is from the followings:

- For standalone: [46, or 43] dBm

- For in-band:

  - Option 1: [38, 33] dBm over R2D transmission BW

    - ZTE, vivo, Xiaomi, CMCC, DCM, Samsung, Huawei, FUTUREWEI, LGE, Nokia

  - Option 2: (36+X) dBm/MHz

    - X = [0 or 6] dB

    - DCM, Ericsson, Qualcomm

### [Open][H] Proposal 3.3b-v2:

Proposal: For Rel-20 study, Device Tx power assumption for evaluation purpose is from the followings:

- Device 2b: {-10, -20} dBm

- Device C: {0, 5} dBm

Note: Companies can also report other value(s) used in their evaluations.

### [Open][H] Proposal 3.3c-v1:

Proposal: For Rel-20 study, BS antenna gain assumption in link budget for evaluation purpose is from the followings:

- Option 1: 17dBi

  - This assumes in the LLS table (as per TR38.769), the values in “[0h1] Number of antenna elements” and “[0h2] the Number of TXRUs” would be same (as Rel-19)

- Option 2: 8dBi

  - This assumes in the LLS table (as per TR38.769), the value in “[0h1] Number of antenna elements” would be 8 times of the value in “[0h2] the Number of TXRUs”

### [Open][H] Proposal 3.3d-v1:

Proposal: For Rel-20 study, Device antenna gain assumption in link budget for evaluation purpose is 0dBi for both Device 2b and C.

### [Open][H] Proposal 3.3e-v1:

Proposal: For Rel-20 study, BS noise figure assumption in link budget for evaluation purpose is 5dB.

### [Open][H] Proposal 3.3f-v1:

Proposal: For Rel-20 study, Device noise figure assumption in link budget for evaluation purpose is from the followings:

- Option 1: CMCC, vivo, InterDigital, Qualcomm, Nokia, ZTE

  - 15dB for both Device 2b and Device C

- Option 2: Sony, FUTUREWEI, Huawei, Qualcomm, Nokia, ZTE, Wiliot, LGE

  - Device 2b: [15] dB

  - Device C: [13] dB

  - Note: Wiliot suggest 40dB for Device 2b

### [Open][H] Proposal 3.3g-v1:

Proposal: For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 180kHz as starting point

  - Other values e.g. 360kHz, 540kHz of multiple 180kHz are not precluded and up to company report

### [Open][H] Proposal 3.3h-v1:

Proposal: For Rel-20 study, R2D receiver bandwidth assumption in link budget and LLS for evaluation purpose is up to company report with considering:

- R2D transmission bandwidth

- Device Rx LO uncertainty (e.g. CFO ppm assumption)

- Note: ED bandwidth and BB LPF bandwidth in LLS are assumed to be same as R2D receiver bandwidth in link budget for active device with ZIF/IF receiver

### [Open][H] Proposal 3.3i-v1:

Proposal: For Rel-20 study, D2R transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 15kHz as starting point

  - Other values are not precluded and up to company report

  - Companies to report which of DSB or SSB modulation is assumed for D2R transmission bandwidth

### [Open][H] Proposal 3.3j-v2:

Proposal: For Rel-20 study, add a new item of “Receiver interference density (dBm/Hz)” into the link budget template and the value of X dBm/Hz for evaluation purpose is from the followings:

- For R2D, X = -169.3

- For D2R, X = -165.7

Note: Other values are not precluded and up to company report

### [Open][H] Proposal 3.3k-v1:

Proposal: For Rel-20 study, add a new item of “Penetration margin (dB)” into link budget template and the value of X dB for evaluation purpose is from the followings:

- For both R2D and D2R

  - X = {0, 15, 20, 25} dB

### [Open][H] Proposal 3.4b-v1:

Proposal: For Rel-20 study, reference data rate in LLS assumption for evaluation purpose is from the followings:

- Reuse the values in Rel-19 (as per TR 38.769) as starting point

  - FFS whether to add additional larger value(s)

### [Open][H] Proposal 3.4c-v1:

Proposal: For Rel-20 study, message size in LLS assumption for evaluation purpose is from the followings:

- Reuse the values in Rel-19 (as per TR 38.769) as starting point

  - FFS whether needs further update to add additional value of 800 bits.

### [Open][H] Proposal 3.4d-v2:

Proposal: For Rel-20 study, SFO and CFO in LLS assumption for evaluation purpose is from the followings to be down-select:

- Option 1: Device C is better than Device 2b

| Option 1 | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | 10^3~10^4 ppm | [50, 80, 10~100] ppm |
| SFO after clock sync/calibration | [10^2, 10^3] ppm | [10] ppm |
| Initial CFO | 10^2~10^3 ppm | [50, 80, 10~100] ppm |
| CFO after clock sync/calibration | [10s~100, 100] ppm | [10] ppm |

- Option 2-1: Device C and Device 2b are same (with large ppm)

| Option 2-1 | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | 10^3~10^4 ppm | 10^3~10^4 ppm |
| SFO after clock sync/calibration | 10^3 ppm | 10^3 ppm |
| Initial CFO | 10^2~10^3 ppm | 10^2~10^3 ppm |
| CFO after clock sync/calibration | 100 ppm | 100 ppm |

- Option 2-2: Device C and Device 2b are same (with small ppm)

| Option 2-2 | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | [50, 80, 10~100, 10^3~10^4] ppm | [50, 80, 10~100, 10^3~10^4] ppm |
| SFO after clock sync/calibration | [10, 30] ppm | [10, 30] ppm |
| Initial CFO | [50, 80, 10~100, 10^3~10^4] ppm | [50, 80, 10~100, 10^3~10^4] ppm |
| CFO after clock sync/calibration | [10, 30] ppm | [10, 30] ppm |

### [Open][H] Proposal 3.4e-v1:

Proposal: For Rel-20 study, R2D FEC in LLS assumption for evaluation purpose is from the followings:

- Company to report no FEC or using FEC in their evaluations

### [Open][H] Proposal 3.4f-v1:

Proposal: For Rel-20 study, add new item of “Repetition” for both R2D and D2R into LLS assumptions for evaluation purpose

- Company to report no repetition or using repetition in their evaluations

Online - Tuesday morning

### [Open][H] Proposal 3.1b-v1:

Proposal: For Rel-20 study, RAN1 to decide which of the followings is used to perform coverage evaluation for R2D and D2R:

- Option 1 (as Rel-19):

  - Step 1: Obtain the required SINR by link-level simulations under different reference data rates

  - Step 2: Obtain the receiver sensitivity based on the required SINR if Budget-Alt2 is used

  - Step 3: Obtain the MPL/distance based on the receiver sensitivity and link budget

- Option 2

  - Step 1: Obtain the MPL based on the distance via pathloss model

  - Step 2: Derive the required SINR from link budget according to the MPL

  - Step 3: Determine the data rate corresponding to the required SINR via link-level simulations.

### [Open][H] Proposal 3.2a-v2:

Proposal: For Rel-20 study, the following path-loss models are used in the coverage evaluation:

- UMa NLOS defined in TR38.901

  - ISD is considered as: [500m], 750m

- [RMa NLOS defined in TR38.901]

  - [ISD is considered as: 1732m]

- [UMi NLOS defined in TR38.901]

  - [ISD is considered as: 200m]

- [Note: Company can also report to use RMa LOS defined in TR38.901]

### [Open][H] Proposal 3.2b-v2:

Proposal: For Rel-20 study, the antenna height for BS and Device used for evaluation purpose are as follows:

- BS antenna height

  - UMa: 25m

  - [RMa: 35m]

- Device antenna height

  - 1.5m

### [Open][H] Proposal 3.2c-v2:

Proposal: For Rel-20 study, the following layout assumptions is used for evaluation purpose:

- Hexagonal grid, 19 macro sites, 3 sectors per site

- ISD is considered as follows

  - UMa NLOS: [500m], 750m

  - [UMi NLOS: 200m]

  - [RMa NLOS: 1732m]

### [Open][H] Proposal 3.3a-v1:

Proposal: For Rel-20 study, BS Tx power assumption for coverage purpose is from the followings:

- For standalone: [46, or 43] dBm

- For in-band:

  - Option 1: [38, 33] dBm over R2D transmission BW

    - ZTE, vivo, Xiaomi, CMCC, DCM, Samsung, Huawei, FUTUREWEI, LGE, Nokia

  - Option 2: (36+X) dBm/MHz

    - X = [0 or 6] dB

    - DCM, Ericsson, Qualcomm

### [Open][H] Proposal 3.3b-v2:

Proposal: For Rel-20 study, Device Tx power assumption for evaluation purpose is from the followings:

- Device 2b: {-10, -20} dBm

- Device C: {0, 5} dBm

Note: Companies can also report other value(s) used in their evaluations.

### [Open][H] Proposal 3.3c-v1:

Proposal: For Rel-20 study, BS antenna gain assumption in link budget for evaluation purpose is from the followings:

- Option 1: 17dBi

  - This assumes in the LLS table (as per TR38.769), the values in “[0h1] Number of antenna elements” and “[0h2] the Number of TXRUs” would be same (as Rel-19)

- Option 2: 8dBi

  - This assumes in the LLS table (as per TR38.769), the value in “[0h1] Number of antenna elements” would be 8 times of the value in “[0h2] the Number of TXRUs”

### [Open][H] Proposal 3.3d-v1:

Proposal: For Rel-20 study, Device antenna gain assumption in link budget for evaluation purpose is 0dBi for both Device 2b and C.

### [Open][H] Proposal 3.3e-v1:

Proposal: For Rel-20 study, BS noise figure assumption in link budget for evaluation purpose is 5dB.

### [Open][H] Proposal 3.3f-v1:

Proposal: For Rel-20 study, Device noise figure assumption in link budget for evaluation purpose is from the followings:

- Option 1: CMCC, vivo, InterDigital, Qualcomm, Nokia, ZTE

  - 15dB for both Device 2b and Device C

- Option 2: Sony, FUTUREWEI, Huawei, Qualcomm, Nokia, ZTE, Wiliot, LGE

  - Device 2b: [15] dB

  - Device C: [13] dB

  - Note: Wiliot suggest 40dB for Device 2b

### [Open][H] Proposal 3.3g-v1:

Proposal: For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 180kHz as starting point

  - Other values e.g. 360kHz, 540kHz of multiple 180kHz are not precluded and up to company report

### [Open][H] Proposal 3.3h-v1:

Proposal: For Rel-20 study, R2D receiver bandwidth assumption in link budget and LLS for evaluation purpose is up to company report with considering:

- R2D transmission bandwidth

- Device Rx LO uncertainty (e.g. CFO ppm assumption)

- Note: ED bandwidth and BB LPF bandwidth in LLS are assumed to be same as R2D receiver bandwidth in link budget for active device with ZIF/IF receiver

### [Open][H] Proposal 3.3i-v1:

Proposal: For Rel-20 study, D2R transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 15kHz as starting point

  - Other values are not precluded and up to company report

  - Companies to report which of DSB or SSB modulation is assumed for D2R transmission bandwidth

RAN1 agreements achieved during Rel-20 SI

- RAN1#122 (Bengaluru)

Reference

- TR 38.769	Study on solutions for ambient IoT (Internet of Things)

- TS 38.291		Ambient IoT Physical layer

- RP-251884	New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

- R1-2505154	Discussion on Evaluation Methodology for R20 A-IoT	FUTUREWEI

- R1-2505166	Discussion on evaluation assumptions and results for Ambient IoT	Spreadtrum, UNISOC

- R1-2505212	Evaluation for active A-IoT device in outdoor scenario	Huawei, HiSilicon

- R1-2505246	Evaluations	Ericsson

- R1-2505304	Evaluation methodology for A-IoT outdoor deployment scenarios	CATT

- R1-2505410	Evaluation methodologies, assumptions and results for R20 AIoT	vivo

- R1-2505423	Initial views on Evaluation Methodology for Rel-20 AIoT	Nokia

- R1-2505457	Discussion on evaluation methodology for Ambient IoT in NR outdoor for active devices	Xiaomi

- R1-2505513	Discussion on evaluations for Ambient IoT	China Telecom

- R1-2505578	Evaluation methodology and assumptions for Rel-20 Ambient IoT	Samsung

- R1-2505603	Discussion on evaluation of active Ambient IoT device	ZTE Corporation, Sanechips

- R1-2505615	Discussion on evaluation for Ambient IoT	Transsion Holdings

- R1-2505751	Discussion on EVM for R20 A-IoT	OPPO

- R1-2505793	Evaluation assumptions for outdoor Ambient IoT 	Tejas Network Limited

- R1-2505852	Evaluations for Rel-20 Ambient IoT SI	LG Electronics

- R1-2505907	On Rel-20 Ambient IoT evaluation assumptions	Apple

- R1-2505923	Evaluations for Ambient IoT	NEC

- R1-2506091	Discussion on evaluation assumptions	CMCC

- R1-2506112	Ambient IoT evaluations for outdoor active devices	Sony

- R1-2506158	Evaluations for Active Device in Ambient IoT	InterDigital, Inc.

- R1-2506212	Evaluations	Qualcomm Incorporated

- R1-2506299	Study on evaluations for Ambient IoT outdoor for active device	NTT DOCOMO, INC.

- R1-2506373	Discussion on evaluation methodology for Rel-20 AIoT	IIT Kanpur