3GPP TSG-RAN WG1#122	R1-2506545

Bengaluru, India, Aug 25-29, 2025

Agenda Item:	8.5.3

Source:	Moderator (Ericsson)

Title:	Summary #2 of AI 8.5.3 for R19 NES

Document for:	Discussion

# Introduction

This is the summary for AI 8.5.3 on the Maintenance for adaptation of common signalling for NES based on the views expressed by companies in the contributions listed in the Appendix A and providing topics and proposals for discussion/agreement.

# Adaptation of PRACH

## RAN2 LS

### Discussion point 2.1-1 (RAN2 LS on RA-RNTI)

RAN2 LS [18] mentions the following

RAN1 impact for this LS was discussed in below tdocs

- [1],[5],[6],[13],[19],[20](Alt-2) – No change to RAN1 specifications. Send reply LS to RAN2

  - [5],[6],[13],[20](Alt-2) – request RAN2 to update f_id equation in 38.321 sub clause 5.1.3

  - [19] – gentle suggestion for how RAN2 might re-interpret f_id based on RAN1 agreements

- Change to RAN1 specifications

  - [9] – Update nRA calculation in 38.211 sub clause 6.3.3.2

  - [20](Alt-1) – Update clause 6.3.3.2 to describe relationship between nRA and f_id

| Company | Comment (if any) |
| --- | --- |
| FL | Please provide your view on how to handle RA-RNTI calculation considering the RAN2 LS in [18]. Q1. Is there need to update 38.211 as proposed in [9]? Q2. Should RAN1 send reply LS to RAN2 as proposed in [1],[5],[6],[13]? If sending reply LS should RAN1 request RAN2 to update the f_id calculation as proposed in [5],[6],[13]? |
| Google | Q1. No Q2. No |
| Xiaomi | Q1. No Q2. No |
| Samsung | Q1: No Q2: Yes |
| Qualcomm | Q1: No Q2: Yes but up to RAN2 on how it can be captured. |
| Sharp | Q1. No Q2. Yes. |
| ZTE Corporation, Sanechips | Q1. No Q2. No |
| CATT | Q1: No Q2: Yes |
| Ericsson1 | Q1. No |
| Apple | Q1: We understand that the f_id was not in RAN1 spec previously and actually there is missing connection between nRA in RAN1 spec and f_id in RAN2 spec. We think the missing part can be added in the TP and we also provide a version of TP for discussion (R1-2505871 in our contribution under AI5) TP #1 for TS 38.211 Section 6.3.3.2 Mapping to physical resources <unchanged parts omitted> Random access preambles can only be transmitted in the frequency resources given by either the higher-layer parameter msg1-FrequencyStart or msgA-RO-FrequencyStart if configured as described in clause 8.1 of [5 TS 38.213]. The PRACH frequency resources nRA∈0,1,…,M-1, where M equals the higher-layer parameter msg1-FDM or msgA-RO-FDM if configured, are numbered in increasing order within the initial uplink bandwidth part during initial access, starting from the lowest frequency. Otherwise, nRA are numbered in increasing order within the active uplink bandwidth part, starting from the lowest frequency. For PRACH resources provide by RACH-ConfigCommon or RACH-ConfigDedicated, nRAvalue is the same as f_id defined in clause 5.1.3 of [TS38.321]. For PRACH resources configured provided by addl-RACH-Config-Adaptation, nRA plus the number provided by msg1-FrequencyStart in the same RACH-ConfigCommon as addl-RACH-Config-Adaptationdetermines determines the value of f_id in clause 5.1.3 of [TS38.321] <unchanged parts omitted> Q2: Yes, if no RAN1 spec is to be changed. |

Round 1 Summary:

- On Q1, majority of companies prefer not to update RAN1 specs

- On Q2, a slight majority of companies think a reply LS to RAN2 is needed, however it is not clear from the responses whether or not RAN1 should explicitly recommend what change to the equation for f_id is needed

Based on the responses so far, the moderator makes the following proposals:

#### [Closed] Proposal 2.1-1 (Offline consensus)

- Send LS to RAN2

  - FFS: wording

- Conclude in RAN1 that there is no change to RAN1 specs, e.g., no change to nRA calculation in 38.211

#### [Closed] Discussion on LS reply to RAN2

Discuss the following draft LS text from vivo [19]:

1. Overall Description:

RAN1 thanks RAN2 for providing the LS on RA-RNTI for PRACH adaptation. From RAN1 perspective, there is no impact on the determination of f_id since f_id is not defined in RAN1 specification. Besides, RAN1 understands that additional PRACH resource can be partially overlapping with legacy PARCH resources in time domain and/or frequency domain, where an additional RO is invalid if it is overlapped with legacy valid RO in both time and frequency domain. RAN1 related agreements which could be related to how f_id is re-interpreted for calculating RA-RNTI are provided below for reference:

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping

  - Note: the overlapped RO for legacy resource is not impacted

  - FFS: Clarification on configuration of legacy ROs

Agreement

- Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported

  - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs)

  - FFS: When there is no configuration of Msg1-FDM

- Separate configuration of number of SSB per RO is supported

2. Actions:

RAN1 kindly requests RAN2 to take the above into account for further work.

| Company | Comment (if any) |
| --- | --- |
| FL | Please provide your view on the above draft LS reply from vivo. If you would like changes to the wording, please provide complete text here. |
| Samsung | Some suggestions on the content. 1. Overall Description: RAN1 thanks RAN2 for providing the LS on RA-RNTI for PRACH adaptation. From RAN1 perspective, there is no proper place to capture the impact on impact on the determination of f_id since f_id is not defined in RAN1 specification. The nRA in RAN1 specification is index for physical signal generation and cannot be directly used for f_id. So RAN1 kindly requests RAN2 to capture the impact on f_id of in TS38.321. Besides, RAN1 understands that additional PRACH resource can be partially overlapping with legacy PARCH resources in time domain and/or frequency domain, where an additional RO is invalid if it is overlapped with legacy valid RO in both time and frequency domain. RAN1 related agreements which could be related to how f_id is re-interpreted for calculating RA-RNTI are provided below for reference: Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported |
| Sony | We agree the text from Samsung (as above). |
| vivo | We support the text of our LS reply. We don’t agree to remove the related agreement as Samsung suggests. This is related to how f_id is re-interpretated since additional RO can be partially overlapping with legacy ROs as the following figure shows: If following RAN2 LS, i.e., RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy Ros, f_id of additional RO will exceed 8 that is not existing in current legacy RA-RNTI calculation equation. Considering this, RAN1 agreements related with f_id reinterpretation should be aware to RAN2 to determine a correct f_id re-interpretation mechanism. |
| LG | We support the content suggested by Samsung. |
| Apple | We support the updates from Samsung. |
| ETRI | We support the version updated by Samsung. In our understanding, UE will transmit PRACH on a valid RO, so invalid ROs (due to overlap) need not be taken into account in the RA-RNTI calculation. |
| Fujitsu | We support the version updates by Samsung. |
| Nokia/NSB | For RAN2 better understanding of our RAN1 agreements, we could copy-pasted the early RAN1 agreements in the LS as suggested by vivo. It can be easier for RAN2 colleagues for further work on this aspects. |
| ZTE Corporation, Sanechips | Vivo’s suggestion is preferred. It’s beneficial for better RAN2 understanding. |
| Samsung2 | Based on the offline discussion with vivo, we think the understanding of valid RO should be counted for f_id is shared as always. We also think there is no harm to include the previous agreement. So revision is added as following in blue. 1. Overall Description: RAN1 thanks RAN2 for providing the LS on RA-RNTI for PRACH adaptation. From RAN1 perspective, there is no proper place to capture the impact on impact on the determination of f_id since f_id is not defined in RAN1 specification. The nRA in RAN1 specification is index for physical signal generation and cannot be directly used for f_id. So RAN1 kindly requests RAN2 to capture the impact on f_id of valid RO in TS38.321. Besides, RAN1 understands that additional PRACH resource can be partially overlapping with legacy PARCH resources in time domain and/or frequency domain, where an additional RO is invalid if it is overlapped with legacy valid RO in both time and frequency domain. RAN1 related agreements which could be related to how f_id is re-interpreted for calculating RA-RNTI are provided below for reference: Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported |
| Xiaomi | We agree the text from Samsung. |
| Moderator | There seems to be quite good support for Samsung’s revision with inclusion of the RAN1 agreements. The moderator proposes some small editorial updates. Please provide your view on this update. If you have concerns, please provide wording changes to be constructive using the following as a baseline. 1. Overall Description: RAN1 thanks RAN2 for providing the LS on RA-RNTI for PRACH adaptation. From RAN1 perspective, there is no proper place in RAN1 specifications to capture the impact on f_id since f_id is not defined in RAN1 specifications. The index nRA in RAN1 specification is an index for physical signal generation and cannot be directly used for determination of f_id . So RAN1 kindly requests RAN2 to capture the impact on f_id of valid RO in TS38.321 based on the following RAN1 agreements: Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported 2. Actions: RAN1 kindly requests RAN2 to take the above into account for further work. |
| Moderator 2 | It seems like the above wording is stable. The full draft LS is uploaded to a sub-folder “LS Reply to RAN2”. We can do a final check in the offline this morning (Thursday). |
| Offline consensus | 1. Overall Description: RAN1 thanks RAN2 for providing the LS on RA-RNTI for PRACH adaptation. From RAN1 perspective, there is no proper place in RAN1 specifications to capture the impact on f_id since f_id is not defined in RAN1 specifications. The index nRA in RAN1 specification is an index for physical signal generation and cannot be directly used for determination of f_id . RAN1 kindly requests RAN2 to take the above into account, as well as the below RAN1 agreements, with regards to the impact on f_id of valid RO in TS38.321: Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported 2. Actions: RAN1 kindly requests RAN2 to take the above into account for further work. |

## Proposed Corrections (TPs)

### Discussion point 2.2-1 (new RRC parameters)

[4] proposes the following

[5] proposes an alternate approach to handle totalNumberOfRA-Preambles for additional PRACH resources  (fix it to 64 as proposed in TP1 of [5])

| Company | Comment (if any) |
| --- | --- |
| FL | Q1. Please provide your view on whether to introduce the additional RRC parameters proposed in [4] (Proposal 2) Q2. If additional RRC parameters are not introduced, please provide your view on whether TP1 of [5] is required or not. |
| Google | Q1. We think the power control parameters for additional PRACH resources can be configured separately. Given the periodicity for additional PRACH resource and legacy PRACH is different, the opportunity for retransmission of PRACH could be different. |
| Xiaomi | Q1. We don’t see the motivation to provide separate parameters for legacy RACH and additional RACH. Q2. It is gNB implementation issue. We believe gNB can find a proper way out. Indeed, a fixed value put a restriction on additional RO configuration. |
| ZTE Corporation, Sanechips | Open to discuss and agree with Google that the power control parameters for additional PRACH resources can be configured separately. Because the requirement, e.g., the maximum transmission times for UL WUS and for random access may be different. |
| CATT | Q1: We think separate RRC parameters can be introduced for additional PRACH. |
| Apple | Q1. yes Q2. similar view as Xiaomi. |
| LG | Q1. We believe that additional parameters are unnecessary because gNB can handle them. Q2. Similar view with Xiaomi. |

Round 1 summary

- On Q1, majority of companies who responded think separate RRC parameters can be discussed, but there is not a common view on which separate parameters should be adopted. In [4] the proposed parameters are for the number of preambles. Two companies recommend having separate parameters for power control.

- Based on the responses so far, it is clear that more discussion is needed to align on which (if any) separate parameters are needed.

#### [Closed] Proposal 2.2-1

Amongst the following parameters in RACH-ConfigCommon

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- preambleTransMax

- powerRampingStep

- ra-ResponseWindow

- ra-Msg3SizeGroupA

- messagePowerOffsetGroupB

- ra-ContentionResolutionTimer

- rsrp-ThresholdSSB

- rsrp-ThresholdSSB-SUL

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

- numberOfRA-PreamblesGroupA

- totalNumberOfRA-Preambles

Discuss which, if any, are provided separately for additional PRACH resources and legacy PRACH resources.

Alt-1: None of the parameters are provided separately; all are common

Alt-2: The following two parameters are provided separately; the rest are common

- numberOfRA-PreamblesGroupA

- totalNumberOfRA-Preambles

Alt-3: The following power-control related parameters are provided separately; the rest are common

- preambleReceivedTargetPower

- preambleTransMax

- powerRampingStep

In the on-line discussion, there was not consensus to introduce new RRC parameters for the additional PRACH resources meaning that Alt-1 is the default. However, there was some desire to not cut off discussion in RAN2. Hence the moderator makes the following proposal for a RAN1 conclusion.

#### [Closed] Proposed Conclusion 2.2-1a

Amongst the following parameters in RACH-ConfigCommon for legacy PRACH resources

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- preambleTransMax

- powerRampingStep

- ra-ResponseWindow

- ra-Msg3SizeGroupA

- messagePowerOffsetGroupB

- ra-ContentionResolutionTimer

- rsrp-ThresholdSSB

- rsrp-ThresholdSSB-SUL

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

- numberOfRA-PreamblesGroupA

- totalNumberOfRA-Preambles

There is no consensus in RAN1 to provide any of these parameters separately for additional PRACH resources.

| Company | Comment (if any) |
| --- | --- |
| FL | Please provide your view on the above proposal |
| Samsung | We are ok with this, but we also think there is no need to have any conclusion or not. |
| LG | Support |
| Apple | We still think the following at least numberOfRA-PreamblesGroupA need to be separately configured. The parameter configures the number of preambles for group A per SSB. We already agreed that ssb-perRACH-OccasionAndCB-PreamblesPerSSB is separately configured for additional ROs. Then it may results in different total number of preambles per SSB. If numberOfRA-PreamblesGroupA is not configured accordingly, it may result in no preambles for group B. |
| ETRI | Ok with the proposed conclusion. |
| Fujitsu | We are fine with the conclusion. |
| Xiaomi | Ok with FL’s proposal. |
| Ericsson2 | OK with FL proposal |
| Moderator | All companies except one are okay with the conclusion. The moderator suggests we go with Conclusion 2.2-1a. |

### Discussion point 2.2-2 (additional PRACH availability for PDCCH order)

Following was agreed in RAN1#120

Agreement (from RAN1#120)

For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

- FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s)

- FFS: Details on how to reuse existing bit for the 1-bit indication

[5],[6],[8],[9],[11] propose corrections to 38.213 to clarify UE behaviour for availability of the PRACH resources indicated by PDCCH ordered PRACH.

| Company | Comment (if any) |
| --- | --- |
| FL | Q1. Please indicate whether UE behaviour for availability of the PRACH resources indicated by PDCCH ordered PRACH needs to be clarified in 38.213 Q2. If clarification is needed, please indicate your preferred approach - TP3 in [5] proposes new RRC parameter. A validity timer specific for PDCCH order - TP in [6] proposes to reuse the validity timer used for CBRA - TPs in [8], [11] propose that the PRACH occasions are available for the triggered PRACH associated with the PDCCH order |
| Google | OK to further clarify the UE behaviour. We can discuss the TPs directly. |
| Xiaomi | We prefer TP in [6]. The reason for validity is to guarantee gNB and UE has common understanding on PRACH adaptation. It is hold for both CBRA and CFRA. Hence, the validity timer used for CBRA can be directly applied to CFRA. |
| Samsung | Q1: yes Q2: as proponent for TP in [6], the motivation for that is to reuse the existing parameter as much as possible instead of introducing new parameter. |
| ZTE Corporation, Sanechips | Prefer TP in [6]. |
| CATT | Q1:Yes Q2: We prefer TPs in [8] since it follows the principle of the legacy PDCCH order. |
| Ericsson1 | Q1. Yes Q2. The approach given by TPs in [8], [11] are aligned with the agreement. Other approaches are not. |
| Nokia/NSB | To our view, TP in [6] seems work without the technical issue. |
| LG | We think TP in [6] is not align with the agreement. Therefore, we support TPs in [8], [11] to clarify the UE behaviour. |
| DCM | Q1: Yes Q2: TPs in [8] is ok to us |

Round 1 summary

- All responses so far indicate that UE behavior for availability of the PRACH resources indicated by PDCCH ordered PRACH should be clarified in 38.213

- Several companies prefer TP in [6]

- Two companies prefer [8] or [11] with one company stating that these are aligned with the agreement whereas [6] is not.

#### [Closed] Proposal 2.2-2

- Down select to one of the following TPs:

  - Alt-1 (from [6])

TS 38.213 (V19.0.0)

8.1 Random access preamble

================= text omitted===============================

Valid PRACH occasions associated with addl-RACH-Config-Adaptation, and additionally in association periods indicated by prach-SubsetMask-Index-Adaptation, if provided, are indicated as available for PRACH transmission based on an indication in a DCI format 1_0 with CRC scrambled by a P-RNTI or a C-RNTI [5, TS 38.212]. For indication by DCI format 1_0 with CRC scrambled by the P-RNTI, or for indication by DCI format 1_0 with CRC scrambled by the C-RNTI and the value of the random access preamble index field is zero, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the P-RNTI. For indication by DCI format 1_0 with CRC scrambled by the C-RNTI and the value of the random access preamble index field is not zero, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the C-RNTI.

================= text omitted===============================

  - Alt-2 (from [8])

Clause 8.1

--- omit unrelated text----

Valid PRACH occasions associated with addl-RACH-Config-Adaptation, and additionally associated with prach-SubsetMask-Index-Adaptation, if provided, are activated for PRACH transmission based on an indication in a DCI format 1_0 with CRC scrambled by a P-RNTI [or a C-RNTI] [5, TS 38.212]. For activation by DCI format 1_0 with CRC scrambled by the P-RNTI, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the P-RNTI. For activation by a DCI format 1_0 with CRC scrambled by the C-RNTI, the PRACH occasions are available until the UE performs the first PRACH transmission triggered by the DCI.

---- omit unrelated text ----

  - Alt-3 (from [11])

8.1	Random access preamble

<Omit unchanged text>

Valid PRACH occasions associated with addl-RACH-Config-Adaptation, and additionally in association periods indicated by prach-SubsetMask-Index-Adaptation, if provided, are indicated as available for PRACH transmission based on an indication in a DCI format 1_0 with CRC scrambled by a P-RNTI or a C-RNTI [5, TS 38.212]. For indication by DCI format 1_0 with CRC scrambled by the P-RNTI, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the P-RNTI. For indication by DCI format 1_0 with CRC scrambled by the C-RNTI, the PRACH occasions are available for the triggered PRACH associated with the PDCCH providing the DCI format 1_0 with CRC scrambled by the C-RNTI.

<Omit unchanged text>

--------------------------------------------End TP----------------------------------------------------------------------------------------

While there is some support for the TP in [6], two companies have pointed out that this TP is not in-line with the above RAN1 agreement Hence, the moderator recommends limiting further discussion to the TPs in [8] and [11].

#### [Closed] Proposal 2.2-2a

- Down select to one of the following TPs:

  - Alt-2 (from [8])

Clause 8.1

--- omit unrelated text----

Valid PRACH occasions associated with addl-RACH-Config-Adaptation, and additionally associated with prach-SubsetMask-Index-Adaptation, if provided, are activated for PRACH transmission based on an indication in a DCI format 1_0 with CRC scrambled by a P-RNTI [or a C-RNTI] [5, TS 38.212]. For activation by DCI format 1_0 with CRC scrambled by the P-RNTI, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the P-RNTI. For activation by a DCI format 1_0 with CRC scrambled by the C-RNTI, the PRACH occasions are available until the UE performs the first PRACH transmission triggered by the DCI.

---- omit unrelated text ----

  - Alt-3 (from [11])

8.1	Random access preamble

<Omit unchanged text>

Valid PRACH occasions associated with addl-RACH-Config-Adaptation, and additionally in association periods indicated by prach-SubsetMask-Index-Adaptation, if provided, are indicated as available for PRACH transmission based on an indication in a DCI format 1_0 with CRC scrambled by a P-RNTI or a C-RNTI [5, TS 38.212]. For indication by DCI format 1_0 with CRC scrambled by the P-RNTI, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the P-RNTI. For indication by DCI format 1_0 with CRC scrambled by the C-RNTI, the PRACH occasions are available for the triggered PRACH associated with the PDCCH providing the DCI format 1_0 with CRC scrambled by the C-RNTI.

<Omit unchanged text>

--------------------------------------------End TP----------------------------------------------------------------------------------------

| Company | Comment (if any) |
| --- | --- |
| FL | Please provide your view on Proposal 2.2-2a |
| Samsung | As proponent of [6], May I ask which part of our TP is aligned with the agreement? Besides, For both TP in Alt-2/3 is not realy working. “the PRACH occasions are available until the UE performs the first PRACH transmission triggered by the DCI” is not clear and not aligned with the PDCCH order PRACH, in either CBRA or CFRA, the resource can be last several rounds since the UE can do retransmission. “the PRACH occasions are available for the triggered PRACH associated with the PDCCH providing the DCI format 1_0 with CRC scrambled by the C-RNTI”, is also not clear and giving no control of resource availability. So we point out, there is difference if UE is triggred with CBRA, or CFRA. For CBRA, there should be no difference for normal CBRA, and for CFRA, it’s only matter we need new RRC parameter for the duration or not, and we prefer to reuse the legacy paramer since it’s CR phase. |
| LG | First of all, we support a unified solution that can be applied consistently to both CBRA and CFRA without introducing new RRC parameter, rather than different interpretations of availability. Our understanding of the Alt-2 we support is that when availability is indicated by the PDCCH order, additional PRACH resources are used only for the first PRACH transmission, and legacy PRACH resources are used thereafter (i.e., for retransmissions). |
| ETRI | We think Samsung’s TP is clearer than other alternatives. |
| Fujitsu | We support Alt 3. In our view, the limitation in Alt 2 that additional PRACH resource are only available for the first PRACH transmission does not align with the agreement. |
| Ericsson2 | We prefer Alt 3 as text in TP is aligned with agreement (i.e., blue highlighted part below) Agreement (from RAN1#120) For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) - FFS: Details on how to reuse existing bit for the 1-bit indication Also OK with Alt2 as it is in same direction as Alt 3. Using validity-DurationForAddlRACHAdaptation for PDCCH order case (as proposed in Alt 1) is not aligned with above agreement which clearly says “indicate whether the additional PRACH resource(s) is available for the triggered PRACH”. |
| Moderator | There are still divergent views: - Some companies still support TP in [6], but there is still concern that this is not in line with the agreement - There is split support between the TPs in [8] and [11], but now one company comments that Alt-2 is also not in-line with the agreement The moderator recommends that we discuss further in the offline today (Thursday) about finding a solution that is in-line with the agreement. |

### [Closed] Discussion point 2.2-3 (Short message indicator table in 38.212)

[3],[7],[9] propose updates to Table 7.3.1.2.1-1: Short Message indicator in 38.212 and text related to it. The cited reason for change is that, without the proposed correction, below agreement is not correctly reflected in 38.212. [16] proposes update to Short message indicator fields description in 38.21 and Table 7.3.1.2.1-5: PRACH resource indicator of 38.212 to address same issue.

Agreement (from RAN1# 121)

Additional PRACH availability indication can be carried by a DCI 1_0 with P-RNTI with Short Messages Indicator set to 00, 01,10,11.

Note: Above is already reflected in the endorsed editor CR 38.212

| Company | Comment (if any) |
| --- | --- |
| FL | Q1. Please indicate whether the updates proposed in TP2 of [3] or TP1 of [7] or Proposal 5 of [9] or Proposal 3 of [16] are needed. If yes, please indicate which formulation of these TPs is preferred. |
| Google | We are open to discuss the TPs |
| Samsung | No. |
| Qualcomm | We think the current spec is clear. |
| Sharp | Regarding this issue, RAN1 should clarify whether the PRACH adaptation indication is one content of the short messages or not, although it is included in Short Messages field. At least, current running CR on TS38.331 assumes the indication is one content of the short messages. If we follow the running CR description (i.e. PRACH adaptation indication is short message), we need some clarification on the table since current table describes 01 does not include short messages. Otherwise (i.e. PRACH adaptation indication is separated from short messages), the intention of the above agreement is to handle the PRACH adaptation indication independently from other contents in paging DCI. Therefore, UEs can read the 1-bit indication without referring to the Short Message Indicator field. Based on this interpretation, the Short Messages Indicator field (including Table 7.3.1.2.1-1) does not require additional description, as it is not relevant to the 1-bit indication. Additionally, we understand that there is the intention to process the indication at the physical layer. Therefore, in TS38.212, the description of the indication should ensure that the corresponding operations can be completed at the physical layer. In order to reflect the intention, the description of the Short Messages field in Proposal 3 of [16] is necessary. |
| ZTE Corporation, Sanechips | The modification for Table 7.3.1.2.1-1: Short Message indicator in 38.212 should be supported and the wording can be discussed. |
| CATT | As a proponent, we think the TP is needed. Since according to the table if the Short Messages Indicator set to ‘00’, all the bits in the DCI are reserved and the UE can ignore the entire DCI. If the Short Messages Indicator set to ‘01’, only scheduling information for Paging and TRS availability indication are required to be decoded, there is no description on the indication of PRACH availability in the Short Message Indicator table. |
| Apple | We think the Proposal is needed. |
| LG | As we mentioned in our contribution, since UE ignores all the bits carried in DCI format 1_0 when Short Message Indicator is set to ‘00’, Table 7.3.1.2.1-1 should be modified to specify the UE behaviour when addl-RACH-Config-Adaptation is configured to achieve indicating the availability of additional PRACH resources. In addition, TS 38.331 should be referred for UE to know which bit in DCI format 1_0 indicates the availability of additional PRACH resources. Therefore, the proposal 5 of [9] is needed. |
| DCM | We think current spec works and no need to change further. Our undestanding is that the PRACH adaptation indication is indicated through short message, which is carried from higher layer. When we look at short message indicator, 00/01 do not carry short message meaning, gNB will not use those indicators when gNB want to indicate PRACH adaptation. |
| Moderator | So far, 6 companies think that a spec change is needed, wheres 3 companies think that the current spec works without change. Hence, there is not consensus on whether or not this issue is an essential correction. DCM observes that if if the gNB needs to indicate PRACH adaptation, it can set the short message indicator to 10 or 11 and avoid 00/11 which points in the direction that the spec is not broken without a change. The moderator asks for further comments taking into account DCMs observation. |
| Apple | In the last meeting, there was argument to not limit gNB behavior in setting the short message indicator value. We agreed and should folllow the agreement. |
| ETRI | We think the spec update is needed. The agreement states that additional PRACH availability indication should be applicable for all the values of Short Messages Indicator. |
| Fujitsu | We share the same view as DCM that the system can still work with the current spec. It is worthy noting that the current TS 38.212 has the following description: ============================= - Short Messages - 8 bits, according to Clause 6.5 of [9, TS38.331]. If only the scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, all the bits in this bit field are reserved, except the bit indicating the availability of the PRACH resource configured by addl-RACH-Config-Adaptation according to Clause 6.5 of [9, TS38.331]. ============================= Thus, if the network want to indicate availability of additional PRACH resources, it can set the short message indicator to a value other than 00, such as 01, 10 or 11. In conclusion, we don’t think it is an essential correction. |
| Xiaomi | We agree with DCM that the current specification works and nothing broken. |
| Sharp | We believe that the motivation behind the RAN1 agreement for Short Messages Indicator is that the UE always checks the 5th bit in the Short Messages field without checking the Short Messages Indicator field, if addl-RACH-Config-Adaptation is configured. Therefore, to avoid any contradictions, this behavior should be clearly described in TS 38.212 as follows. “If addl-RACH-Config-Adaptation is configured, 5th bit in Short Messages field is present for the availability of the PRACH resource configured by addl-RACH-Config-Adaptation irrespective of the value of Short Messages Indicator, otherwise, the 5th bit is reserved.” By such description, the UE can ignore Short Message Indicator field description for the case of PRACH adaptation, and no update is needed for the table. #We would still like to know whether companies assume that the PRACH adaptation indication is treated as a short message, since this may affect the description related to short messages. |
| Moderator | There are divergent views on whether a spec change is needed or the issue can be handled by gNB implementation. The moderator suggests to further discuss in the offline (Thursday) if time permits to see if companies can converge. |

### [Closed] Discussion point 2.2-4 (rounding for Mask index to AP mapping)

[9],[10] discuss correction/clarification to Table 8.1-0: Mapping of mask index to association periods per Kmask association pattern periods in 38.213. The cited reason for change is that the current text is unclear for the case when the total number of Association Periods (APs) in the K_mask association pattern periods is not a integer multiple of 2/4/8/16..

| Company | Comment (if any) |
| --- | --- |
| FL | Q1. Please indicate if clarification is needed for the issue raised in proposal 4 of [9] and Proposal 1 of [10]. Q2. If clarification is needed, please indicate if TP provided in proposal 4 of [9] is acceptable. |
| Google | We are open to discuss the TPs |
| Samsung | This issue can be up to gNB configuration to handle without specification work. But we are open to hear more views. |
| CATT | It can up to gNB configuration to avoid such issue. |
| Apple | We think the two options result in the same results, i.e. put limitation on NW to ensure that the number of APs within the K_APP APPS can be divided by 2/4/8. However, if we go with the option1, we prefer to have conclusion to be captured. Option 1: NW ensures that within the K_APP APPs, the number of APs can be divided by 2 or 4 or 8, according to the selected mask; Option 2: The actual usable APPs are ceiled/floored accordingly. |
| LG | We share the same view with Apple but we prefer to modify the each entry in Table 8.1-0 in TS 38.213 to ensure an integer number of APs to apply the PRACH mask. |
| DCM | If there is a limitation on the combination between K_APP and the mask index, we are fine to go with LGE's proposal or option 2 from Apple |
| Moderator | There is a split of views: some companies think that the issue can be solved by gNB implementation, while others prefer a spec change. Please continue discussion, taking into account the comments that the issue can be solved by gNB implementation. |
| ETRI | We prefer to update the specification. Either option is okay for us. |
| Fujitsu | We support updating the specification. Either LG’s proposal or Apple’s option 2 is acceptable to us. |
| Xiaomi | We also share the view that it can be properly handled by gNB implementation. |
| Moderator | There are divergent views on whether a spec change is needed or the issue can be handled by gNB implementation. The moderator suggests to further discuss in the offline (Thursday) if time permits to see if companies can converge. |

### [Closed] Discussion point 2.2-5 (additional PRACH for SBFD)

[2] proposes to configure two additional PRACH resource sets for NES, where one is not applicable to the SBFD symbols/slots and the other one is applicable to the SBFD symbols/slots, with regard to UEs that support NES only and UEs that support both SBFD and NES.

| Company | Comment (if any) |
| --- | --- |
| FL | Q1. Please provide your view on Proposal 2 of [2]. Note that this proposal has RAN2 impact as additional RRC signalling needs to be introduced. |
| Google | We think clarification of additional PRACH when SBFD is enabled is necessary. |
| Samsung | At the CR stage, we think it’s better to introduce new mixing of different features. So RAN1 can conclude this configuration issue can be left to gNB. |
| Nokia/NSB | Considering we are now in the maintenance phase, to our view, there is no support of SBFD with additional PRACH resources in Rel19 NES. |
| LG | We share the same view with Samsung and Nokia. |
| Moderator | There is clear view that since we are in maintenance phase, it is not warranted to address the issue raised in [2]. The moderator recommends closing discussion on this issue. |
| Xiaomi | Agree with moderator’s guidance. |

## Other

TBU

# Adaptation of SSB in time domain

## Proposed Corrections (TPs)

### [Close] Discussion point 3.1-1 (clarifications to other UE procedures when using SSB periodicity adaptation)

Several contributions propose clarifications to other UE procedures in 38.213 and 38.214 for case when SSB periodicity adaptation is configured.

- [1],[9],[14],[15] propose clarifications to PDCCH monitoring (TP to Section 10, TS 38.213 given in [1],[14],[15]),

- [1],[9],[14],[15] propose clarifications to PDSCH resource mapping and rate matching (TPs for Section 5.1.4, TS 38.214 given in [1],[14],[15])

- [14] proposes clarifications to determination of SS/PBCH block transmission pattern for UL transmission (TP for section 11.1 of 38.213)

- [15] proposes clarifications to

  - PUSCH Resource Allocation in Time Domain (TPs to section 6.1.2.1, section 6.1.2.3 of 38.214)

  - Determination of HARQ-process ID for multiple PUSCH scheduled with DCI (TP to section 6.1. of 38.214)

  - UE procedure for deferring HARQ-ACK for SPS PDSCH (TP to section 9.2.5.4 of 38.213)

  - PUCCH repletion procedure (TP to section 9.2.6 of 38.213)

  - UE procedure for UTO-UCI reporting ((TP to section 9.3.1 of 38.213))

| Company | Comment (if any) |
| --- | --- |
| FL | Q1. Please provide your view on whether above clarifications are needed. FL intention is to first check high-level view (i.e., clarifications needed vs. current text in the specifications is sufficient). Once the high-level principle is established, detailed TPs can be discussed in next round. |
| Google | OK to discuss the TPs |
| Samsung | We agree with the UE behaviour, but fail to see a need for spec change. Current specifications are generally covering all cases for resource allocation based on SSB. |
| Qualcomm | The TPs should be discussed. The R15/16/17/18 spec is defined with respect to always-on SSB with fixed periodicity. The varied SSB periodicity due to DCI-based adaptation impacts the legacy UE procedures. For example, an SSB before the DCI indication may not impact the procedure but it would impact the procedure after the indication. The nature of clarification is similar to what we agreed last meeting for impact of OD-SSB on RO validation and SSB-RO mapping |
| Nokia, NSB | Clarifications are needed |
| LG | We think those clarifications are needed |
| Moderator | Based on the limited responses so far, 4 companies feel clarification is needed and one does not. Certainly discussion can continue; however, the moderator doubts that it will converge with the limited time available in this meeting. |
| Xiaomi | We share the view clarifications are needed. |
| Ericsson2 | We do not follow why the spec should be duplicated like it was done for OD-SSB. As indicated by Samsung, for the case of SSB periodicity adaptation, current spec text is sufficient to cover intended behaviour. |
| Moderator | The moderator recommends closing discussion on this issue for this meeting due to limited time. |

### [Close] Discussion point 3.1-2 (clarifications to valid RO determination)

[1],[9],[12],[15] discuss clarifications to valid RO determination when SSB periodicity adaptation is configured together with PRACH adaptation using additional PRACH resources. [1],[12] indicate preference for clarifying that the RO-SSB mapping and PRACH validity are based only periodicity provided via ‘legacy signaling’.

| Company | Comment (if any) |
| --- | --- |
| FL | Q1. Please provide your view on Proposal 3 of [1] and Proposal 1 of [12]. Q2. Please provide your view on Proposal 4 of [1] and Proposal 8 of [15]. It would be good if proponents could also provide relevant TPs to speed up discussion. |
| Google | OK to discuss, and we can reuse the same principle for OD-SSB |
| Samsung | Similar to above, we agree with the UE behaviour, but fail to see a spec change needed. |
| Qualcomm | This should be clarified in the spec. The nature is similar to what we agreed last meeting for impact of OD-SSB on RO validation and SSB-RO mapping |
| CATT | OK to discuss. |
| Nokia, NSB | We support that PRACH validation and SSB-RO mapping are based on periodicity provided via legacy signalling. The issue for discussion is if additional RO dropping rule needs to be introduced to ensure that current N_gap rule is not violated when SSB periodicity is adapted. The other option is that it is up to network implementation to guarantee that N_gap rule is not violated. |
| LG | We also think the spec change is needed as same principle in OD-SSB. |
| Moderator | Based on the limited responses so far, 5 companies feel clarification is needed and one does not. Certainly discussion can continue; however, the moderator doubts that it will converge with the limited time available in this meeting, especially since there do not appear to be TPs available. |
| Xiaomi | OK to discuss. |
| Moderator | The moderator recommends closing discussion on this issue for this meeting due to limited time. |

## Other

# Conclusion

TBU

# Appendix A (Contributions)

| 1 | R1-2505138 | Adaptation of common signal/channel transmissions | Nokia, Nokia Shanghai Bell |
| --- | --- | --- | --- |
| 2 | R1-2505259 | Adaptation of Common Signals | Google |
| 3 | R1-2505329 | Maintenance on adaptation of common signal/channel transmissions | CATT |
| 4 | R1-2505357 | Maintenance on common channel/signal adaptation for eNES | Huawei, HiSilicon |
| 5 | R1-2505380 | Maintenance on adaptation of common signal/channel transmissions | vivo |
| 6 | R1-2505544 | Remaining issues on adaptation of common signal/channel transmissions | Samsung |
| 7 | R1-2505597 | Discussion on common signal channel for NES | ZTE Corporation, Sanechips |
| 8 | R1-2505706 | Remaining issue for common signal/channel transmission | OPPO |
| 9 | R1-2505844 | Remaining issues on adaptation of common signal/channel transmissions | LG Electronics |
| 10 | R1-2505879 | Remaining issues on adaptation of common signal/channel for NES enhancements | Apple |
| 11 | R1-2505993 | Maintenance for R19 NES adaptation of common signal/channel transmissions | Ericsson |
| 12 | R1-2506035 | Adaptation of common signal/channel transmissions | MediaTek Inc. |
| 13 | R1-2506056 | Remaining issues on SSB and PRACH time domain adaptations | ETRI |
| 14 | R1-2506080 | Maintenance of adaptation of common signal/channel transmission | CMCC |
| 15 | R1-2506182 | Adaptation of common channel transmissions | Qualcomm Incorporated |
| 16 | R1-2506251 | Maintenance on adaptation of common signal/channel transmissions | Sharp |
| 17 | R1-2505117 | LS reply on DCI-based PRACH adaptation | Ericsson |
| 18 | R1-2505118 | LS on RA-RNTI for PRACH adaptation | Nokia |
| 19 | R1-2505362 | Draft reply LS on RA-RNTI for PRACH adaptation | vivo |
| 20 | R1-2505871 | Discussion on RAN2 LS on RA-RNTI for PRACH adaptation | Apple |

# List of RAN1 agreements

## RAN1#116

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

Agreement

For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study

- Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

- For the additional PRACH resources,

  - Adaptation of PRACH resource periodicity/PRACH occasion

  - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle

  - Adaptation based on extending cell DRX operation for PRACH

  - Concentrating ROs in time domain

- Other options are not precluded

Agreement

For adaptation of paging,

- Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings

- Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

Agreement

For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following:

- Applicability to UE in idle/inactive and/or connected mode

- Applicability to PCell and/or SCell(s)

Agreement

For the adaptation mechanisms of SSB in time-domain, study further following mechanisms:

- Adaptation mechanism indicated or configured by gNB without UE trigger

- Adaptation triggered by UE (if any)

FFS: Details of associated signaling/indication/configuration

Agreement

For the adaptation mechanisms of PRACH in time-domain

- Support at least PRACH adaptation provided by gNB without UE trigger

  - FFS: PRACH adaptation with UE trigger

  - Note: UE trigger means UE requests adaptation of PRACH

- Study at least the following,

  - Dynamic signaling and/or semi-static signaling of PRACH adaptation

  - Adaptation of PRACH transmission according to certain condition

  - Applicability to idle/inactive and/or connected mode UEs

  - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

## RAN1#116bis

Agreement

For indication of adaptation of SSB in time-domain,

- Support at least SSB adaptation provided by gNB without UE trigger

Agreement

For adaptation of PRACH in time-domain, support at least the following:

- Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

  - Configuration of additional PRACH resources is provided by semi-static signalling

    - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs

  - FFS: adaptation mechanism for additional PRACH resources

  - Note: No change to the existing PRACH configuration tables in 38.211

Agreement

For adaptation of PRACH in time-domain, support the following:

- SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any)

  - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs

  - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources

  - FFS: SSB-RO mapping for the additional PRACH resources

Agreement

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode

Agreement

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Agreement

For adaptation of PRACH in spatial domain,

- Study possibility of scenarios with non-uniform distribution of UEs in different beams

    - Note 6: Companies are encouraged to provide details on how they map UEs to different beams

- Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any),

    - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation

      - 20ms SSB period

      - 30kHz SCS, DDDSU TDD pattern

      - Setting A: SIB1 period (20ms/40ms/160ms)

      - Setting B1: Cell load (Empty/low/medium)

      - Setting B2: Traffic model

      - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

      - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

      - Setting E1: PRACH configurations

        - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs

      - Case A1-1: PRACH configuration #5 (20ms)

      - Case A1-2: PRACH configuration #17 (10ms)

      - Case A2-1: PRACH configuration #0 (160ms)

        - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs

      - Case B1: PRACH configuration #17 (10ms)

      - Case B2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed time domain adaptation mechanism

        - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration

      - Case C1: PRACH configuration #17 (10ms)

      - Case C2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs

      - Setting F: Cat 1/Cat 2 BS as defined in TR38.864

      - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C

      - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1

      - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc

      - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded

      - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report)

    - Other frameworks for network energy evaluation are not precluded, e.g. including for FR2

## RAN1#117

Agreement

For the study of adaptation of PRACH in spatial domain, following network energy savings gains were reported by sources based on the evaluation framework agreed in RAN1#116bis:

- Two sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs A1-1, zero load [R1-2404409, R1-2405107]

  - -4% ~ -45%

- Seven sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1/A1-2, zero load [R1-2404225, R1-2404185, R1-2404334, R1-2404123, R1-2404562, R1-2405107, R1-2405163]

  - 0% ~ 31%

  - Note: Five sources assumed that case B1 has same PRACH resources as case A1-2. Remaining two sources evaluated only A1-2.

  - Note: Three sources showed NES gains 0% ~ 10% [R1-2404225, R1-2404185, R1-2404334]

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 1.0%~8.8%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes.

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -48.41%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, CAT1 BS power model, for case C1 vs A1-2, zero load [R1-2404626]

  - 4.59%~38.04%

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Four sources showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1/A1-2, zero load [R1-2404562, R1-2404225, R1-2403943, R1-2404626]

  - 0% ~ 3.5%

  - Note: Three sources assumed that case B1 has same PRACH resources as case A1-2. One source evaluated only A1-2.

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 0%~0.2%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -1.19%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Two sources showed following NES gain for TDD, CAT1 or CAT2 BS power model, case C2 vs B2, zero load [R1-2403943, R1-2405107]

  - Less than 0.2%

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs A1-2 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 13.7%/8.7%/4.9%/2.6% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs B1 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 8.03%/5.1%/3.06%/1.74% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, C1 vs B1/A1-2, different loads [R1-2404562]

  - 16%/4.78% for light/medium cell load for CAT1 BS power model

  - 0.65%/0.29% for light/medium cell load for CAT2 BS power model

- One source showed following NES gain for TDD, C1 vs B1, different loads [R1-2404626]

  - -18.57%~0%/-2.52%~0% for low /medium cell load for CAT1 BS power model

  - -0.81%~0%/-0.42%~0% for low /medium cell load for CAT2 BS power model

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, C1 vs A1-2, different loads [R1-2404626]

  - 3.67%~19.88%/2.29%~5.22% for low /medium cell load for CAT1 BS power model

  - 0.67%~1.75%/0.39%~0.91% for low /medium cell load for CAT2 BS power model

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed NES gain for FDD, C1 vs B1, zero load [R1-2404464]

  - 1.4%~7% for CAT1 BS power model

  - 0%~0.3% for CAT2 BS power model

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed NES gain for FR2, CAT1 BS power model, spatial domain adaptation of PRACH configuration index 75 vs a time domain adaptation of PRACH configuration index 75, zero load [R1-2405163]

  - 4%~7%

- Note 1: About possibility of scenarios with non-uniform distribution of UEs in different beams

  - Several companies indicated (and three companies showed data/analysis) that there can be scenarios with non-uniform distribution of UEs in different beams.

  - Several companies mentioned that for non-uniform UE distribution, it can be addressed by gNB implementation e.g. by adjusting SSB beamwidth, etc. Several companies also mentioned that it is not clear how gNB can predict the distribution of UEs in different beams, especially for Idle/Inactive UEs.

- Note 2: Most sources that showed the NES gains (if any) for adaptation of PRACH in spatial domain compared to A1-2/B1 observed that the gain would be due to reduction in the number of overall ROs in time domain in their evaluations. Most of these companies only accounted for ROs in time domain.

- Note 3: The evaluation results assumed the non-uniform distribution of UE is static during the evaluation time period.

Conclusion

There is no consensus in RAN1 on the support of PRACH adaptation in spatial domain

Agreement

For adaptation of PRACH in time-domain, support at least the following case(s)

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains

- FFS: whether additional conditions are needed to support the above cases

- FFS: Additional case whether full/partial overlap in both time and frequency is allowed

- Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources

Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For adaptation of SSB in time-domain, Option 1 is supported

- Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

- Note: Using Option 2 to realize Option 1 is not precluded

  - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active]

    - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities

- FFS: Details including applicable scenarios

- FFS: Support of Cell DTX for connected mode UEs for SSB

Agreement

For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least:

- a PRACH configuration index

- FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources

Study further the following

- When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource,

  - Additional parameter(s) for determining the additional PRACH resources e.g.

    - Scaled/adjusted PRACH configuration period

    - Additional timing offset

    - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Muting/masking ROs

- When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource

  - Additional mechanisms (if any) for determining the additional PRACH resources e.g.

    - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

- Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

Agreement

For the adaptation mechanism for additional PRACH resources, study further the following:

- Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation

- Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

  - Strive to re-use existing DCI format(s)

- Option 3: Adaptation of PRACH transmission according to predefined condition(s)

  - FFS: details

- Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation

  - Strive to re-use existing DCI format(s)

- Option 5: Enhanced cell DRX

## RAN1#118

Agreement

For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 1-1: Scaled/adjusted PRACH configuration period

    - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Opt 1-3: Muting/masking ROs

    - Opt 1-4: additional timing offset(s)

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources,

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

    - Opt 2-2: Additional timing offset(s)

- FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

- FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset)

Agreement

Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

RAN1#117 Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For SSB-RO mapping rule for additional PRACH resources for Case 2.

- Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping

Agreement

For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE),

- At least DCI based adaptation is supported. No introduction of new DCI format.

Agreement

For adaptation mechanism(s) of SSB in time-domain,

- For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

  - FFS: Adaptation for SSB that is not CD-SSB is supported (A2)

  - FFS: Adaptation for SSB not on sync raster is supported (A3)

- For Rel-19 NES-capable UE’s SCell

  - Adaptation of SSB configured for the SCell is supported for the following cases

    - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell

    - Adaptation for SSB that is not CD-SSB on sync raster (B2’)

    - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Agreement

For DCI-based adaptation for additional PRACH resources,

- Select from the following DCI format(s) to carry the adaptation indication.

  - DCI format 1_0

  - DCI format 2_7

  - DCI format 2_9

- FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options

- Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period

- Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period

- Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

## RAN1#118bis

Agreement

For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources.

Agreement

For adaptation of PRACH in time-domain, support both of the following

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

- FFS: Additional details

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

Agreement

For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

- FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

- Discuss further following options for signaling

- Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

  - msg1-FrequencyStart at least for 4-step RACH

  - FFS: other applicable legacy frequency domain parameter(s)

- Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

  - Note: Offset to legacy frequency domain parameter(s) is a new parameter

  - FFS: applicable legacy frequency domain parameter(s)

Conclusion

There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).

- No spec impact

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

## RAN1#119

Agreement

Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?):

- RAN1 agreed that at least SSB burst periodicity is adapted.

- There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options.

Agreement

Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?):

- The frequency location is same before and after SSB adaptation.

Agreement

Reply to Q3(What is the spatial relation before and after SSB adaptation?):

- There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation.

- At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

Further update to be made based on RAN1#119 progress if any.

Agreement

At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16]

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

    - Alt 2-6: Network configured time period

Conclusion

There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

Working Assumption

For DCI-based adaptation for additional PRACH resources,

- Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI

- Option 1: Use reserved bits in the DCI format

  - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format

- Option 2: Use Bits 5-8 within the Short Message (from upper layers)

  - Note: Availability should be confirmed by checking with RAN2.

- Option 3: Use bits available for both Option 1 and Option 2

FFS: Payload size for adaptation for additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - [DCI payload size = 1 bit]

  - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources

  - FFS: details

Agreement

Confirm the following working assumption from RAN1#118bis.

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

## RAN1#120

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping

  - Note: the overlapped RO for legacy resource is not impacted

  - FFS: Clarification on configuration of legacy ROs

Conclusion

There is no RAN1 consensus to support the following in Rel-19

- New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

- New UE trigger

- Adapting the transmitted number of SSBs within a SSB burst

- Adaptation of the time domain positions of SSBs within a burst

Conclusion

There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell

Agreement

For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell

- UE is configured with SSB burst periodicity using legacy signalling for the SCell

- UE is configured with X additional SSB burst periodicity for the SCell

  - FFS: Value of X

- SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity

  - FFS: Whether there is specification impact

  - Note: This does not impact the discussion on OD-SSB

- For switching the periodicity, down-select between

  - (MAC-CE)

    - MAC-CE details for SSB burst periodicity adaptation is up to RAN2.

  - (DCI)

    - Alt 1-3: DCI based signalling is used

Agreement

For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

- FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s)

- FFS: Details on how to reuse existing bit for the 1-bit indication

Agreement

For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration

- FFS: Validity time duration for availability is configured by higher layer signaling or predefined

- FFS: Location of the reference point defined in the specification

- FFS: Value/granularity of the validity time duration.

- FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources

- The mask is applicable at unit of

  - Alt 1: PRACH association period

  - Alt 2: PRACH association pattern period

  - Alt 3: SFN level

- The PRACH association period is determined based on valid additional ROs only.

- The mask is applied after valid RO determination and SSB-RO mapping.

- Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask.

- This is applicable at least for adaptation for DCI 1_0 with P-RNTI

- The DCI does not indicate PRACH mask selection

- FFS: how the mask is identified

  - Option 1: The PRACH mask is from a PRACH mask table

    - Pre-defined table with N=[4 or 8 or 16] rows

    - The semi-static signalling indicates a PRACH mask index

  - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

Agreement

- Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported

  - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs)

  - FFS: When there is no configuration of Msg1-FDM

- Separate configuration of number of SSB per RO is supported

Agreement

Study the following options for the reference point (for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI in a PF) for RRC idle/inactive mode UE and RRC connected mode UE,

- Option 1: SFN of the first PF from the next I-DRX cycle

- Option 2: SFN of the first PF from the current I-DRX cycle

- Option 3: From the first frame of the first PRACH association period after UE receives the DCI

- Option 4: From the first frame of the current SI modification period

- Option 5: From the first frame of the next SI modification period

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

## RAN1#120bis

Agreement

For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI,

- Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources

  - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon

Agreement

For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI

- Use one bit from the Bits 5-8 within the Short Message field (from upper layers)

- Send LS to RAN2 to confirm the use of this bit.

Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs

Agreement

For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI

- the validity duration is configured via higher layer signalling.

Agreement

For DCI-based adaptation for additional PRACH resources, PRACH mask that identifies the subset of the additional PRACH resources is applicable at unit of

- PRACH association period

- This PRACH mask applies to every [configured] K SSB RO association pattern period(s)

Agreement

For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell,

- The DCI is scrambled a new RNTI,

  - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell

- the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter

- the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell

Agreement

For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell.

- Xmax=2

Agreement

Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported

- CB-PreamblesPerSSB

Agreement

LS on DCI-based PRACH adaptation endorsed with the ACTION part modified compared to draft LS in R1-2503085 as follows:

- ACTION: RAN1 respectfully asks RAN2 to confirm whether the use of above bit is feasible.

Final LS in R1-2503086.

Agreement

For adaptation of PRACH in time-domain, for a connected mode UE,

- One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources.

Working Assumption

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213].

- FFS: how to determine the first [possible] SSB burst

Agreement

For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is given by:

- Option 1-2:  Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods)

  - For K: one from up to four candidate values {2,4,8, [1 or 16]}

| Mask index | Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP) |
| --- | --- |
| 0 | 1st half of the APs in K APPs |
| 1 | 1st quarter of the APs in K APPs |
| 2 | 1st eighth of the APs in K APPs |
| 3 | 1st sixteenth of the APs in K APPs |

Agreement

- For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI

- The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is

  - (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..})